���������������������

���������������������    Quarterdeck Office Systems

���������������������    150 Pico Boulevard

���������������������    Santa Monica, CA  90405

���������������������

���������������������

February 10, 1995

Background

==========

Certain Phoenix BIOS revisions do not mark their Advanced Power Management

(APM) BIOS code as ROM.  This code is usually located between E800-EFFF.  As

QEMM loads it maps High RAM or the EMS Page Frame to these addresses because

they are not marked as ROM and therefore look unused.  When QEMM does this,

the APM code gets overwritten with the Page Frame or with High RAM, into which

QEMM's LOADHI may place a program.  Any subsequent APM calls will either lock

the machine or generate an exception because they try to execute invalid code

-- the code of the program that happens to be at that address, rather than the

appropriate BIOS code.

This will be a problem for any memory manager, including DOS' EMM386.EXE, that

uses this address space.  Quarterdeck and Phoenix are, as of this writing,

working co-operatively for a complete solution to this problem.  In the

meantime, QEMM users may take advantage of the following temporary remedies.

Symptoms of the Problem

=======================

One of the surest ways to detect this problem is to start Quarterdeck's

Manifest, which as it starts checks for APM's presence.  If Manifest crashes

immediately with an Exception #6 or an Exception #13, there is a very good

chance that the problem above is the cause of the crash.

Fixing the Problem When APM is Enabled

======================================

If APM is enabled, the region of address space that it uses must be EXCLUDEd

via a parameter on the QEMM line.  To find the appropriate EXCLUDE, run

Manifest, and note the segment address at which the Exception occurs.  This is

the number before the colon in a line that will look something like this:

Exception #6 at E900:9146

You numbers may differ, but it is the number before the colon that is

significant.  In our example above, E900 is segment number, and will be the

base of the EXCLUDE.  The necessary EXCLUDE should be 4K.  Using your own

segment value, add a 4K EXCLUDE to the QEMM line.  For example,

DEVICE=C:\QEMM\QEMM386.SYS RAM <your other parameters> X=E900:4K

Since by adding this EXCLUDE you have made a change to your memory layout, you

should run QEMM's OPTIMIZE program.  APM will now work properly, at the

expense of a small amount of High RAM and some fragmentation of your High RAM

regions.

Fixing the Problem When APM is Disabled

=======================================

E800FIX.COM, which should be included with this file, works around this

problem when APM is disabled.  The benefit to disabling APM in this case is to

save 4K of High RAM, and to prevent the fragmentation of High RAM.

Copy E800FIX.COM (which should accompany this file) to your QEMM directory.

Then add the following line to your AUTOEXEC.BAT:

C:\QEMM\E800FIX.COM

This is all you'll need to do.  E800FIX will take a very small amount of

memory indeed.  If at any time you wish to unload E800FIX.COM while it is

resident, typing E800FIX a second time will unload it from memory.

If neither of these solutions works on your system, Quarterdeck would

like to hear about it.

**************************************************************************

* This technical note and the program which accompnies it may be copied  *

*   and distributed freely as long as they are distributed in their      *

*             entirety and are not distributed for profit.               *

*           Copyright (C) 1995 by Quarterdeck Office Systems             *

************************* E N D   O F   F I L E **************************