===========================================================================

=   Diamond Computer Systems, Inc.                                        =

=   SpeedStar 24X Windows Driver Disk                                     =

=                                                                         =

=   README.TXT                                                     1994   =

===========================================================================

This diskette contains the Windows 3.1 drivers version 2.04.  These

drivers correct most of the outstanding Windows 3.1 driver issues.

Manual Installation Procedures

______________________________

The four .EXE files contain all the driver files.

SHARED.EXE     contains file compatible with both 3.0 and 3.1 releases.

WIN31DRV.EXE   contains Windows 3.1 specific files.

WININST.EXE    contains Windows Install files.

Windows 3.1:

------------

1. This example will use C:\SS24X\WIN31DRV as the destination directory

for the drivers.  The Install application WILL NOT run properly

unless the Windows drivers are located in the <Path>\WIN31DRV

sub-directory.

2. Create the new directory if it does not already exist.

(Example:       MD C:\SS24X <Enter>)

MD C:\SS24X\WIN31DRV <Enter>)

3. Extract the WIN31DRV.EXE file to the new directory.

(Example:  A:WIN31DRV.EXE C:\SS24X\WIN31DRV <Enter>)

4. Extract the SHARED.EXE file to the new directory.

(Example:  A:SHARED.EXE C:\SS24X\WIN31DRV <Enter>)

5. Extract the WININST.EXE file to the parent directory.

(Example:  A:WININST.EXE C:\SS24X <Enter>)

6. Verify operation of your Windows 3.1 installation in standard VGA

mode (640x480x16) before continuing installation.

7. From the Windows Program Manager, choose:

File

Run

C:\SS24X\INSTALL <Enter>

8. This will start the SpeedStar 24X INSTALL application and allow

you to select the desired resolution and colors.

9. If you the INSTALL program cannot find the desired driver, or if

you choose to INSTALL NEW, the INSTALL program will prompt you to

enter the path where the Windows 3.1 drivers for your SpeedStar 24X

can be found. At this prompt type:

C:\SS24X\WIN31DRV

10. The DIAMOND icon will be copied to the Control Panel inside

Windows.