READ

THIS

BEFORE

PRINTING!!!!

This document has been formatted in a special way. Virtually all dot

matrix printers have a condensed mode which prints 132 characters

across a standard 8 1/2 inch page.  When this file is printed out in

condensed mode, the resulting printed pages can be cut down to 5 1/2 X

8 1/2 inches.  The cut pages will fit nicely in the back of your

DOS manual for storage.

Typically, you can turn on this mode by sending a special control

sequence to the printer from BASIC.  For example, you can turn on the

condensed mode of the IBM/Epson printer with the BASIC statement:

LPRINT chr$(15).  If your printer has such a condensed mode, turn it

on now, before printing the rest of this document.

(tm)

CHASM

Cheap Assembler

for the IBM Personal Computer

User's Manual

(c) 1985 by David Whitman

Version 4.07

David Whitman

P.O. Box 1157

North Wales, PA 19454

(215) 641-7522 (days)

(215) 234-4084 (evenings)

Table of Contents

Why CHASM?..................................................1

What can CHASM do?..........................................2

What WON'T it do?...........................................3

System Requirements.........................................4

Advanced and Subset CHASM...................................5

Modifying CHASM's I/O Defaults..............................6

Syntax.....................................................11

Labels.....................................................13

Operands...................................................16

Operand Expressions........................................22

Resolution of Ambiguities..................................25

Pseudo-Operations..........................................29

Macros.....................................................38

Structures.................................................48

8087 Support...............................................52

Outside the Program Segment................................54

Running CHASM..............................................55

Error and Diagnostic Messages..............................59

Execution of Assembled Programs............................64

Notes for Those Upgrading to This Version of CHASM.........75

Miscellaneous and A Word From Our Sponsor..................78

=============================================================

Appendix A: 8088 Mnemonic List.............................83

Appendix B: 8087 Mnemonic List.............................85

Appendix C: Differences Between CHASM and TOA..............86

Appendix D: Description of Files...........................90

Appendix E: Bug Reporting Procedure........................91

Appendix F: Using CHASM on "Compatible" Systems............92

Advanced Version Order Form................................94

1

>>Why CHASM?<<

Why go to the trouble to write an assembler, when one already

exists?  The IBM Macro Assembler is a very powerful software tool

available off the shelf.  It supports features such as macros,

multiple segments, and linking to external procedures.

Unfortunately, the cost of all this power is complexity. The

Macro Assembler is so complicated that IBM warns beginners it is

only suitable for "experienced assembly language programmers".

For most users, this sophistication is more of a hindrance than

an aid.  Even when writing short, simple programs, the user is

saddled with a set of confusing pseudo-ops only appropriate for

large, multi-segment programs.  Producing a fast loading

executable file requires running three separate programs (MASM,

LINK and EXE2BIN) before you can get down to testing.

The macro assembler is totally unsuitable for use with BASIC.

Although it is *possible* to produce machine language BASIC

subroutines with the Macro Assembler, the process is incredibly

convoluted and confusing.

To top it all off, the Macro Assembler costs an overpriced $125.

CHASM is, I hope, a more reasonable compromise between power and

accessibility.  CHASM is simple to use and understand.  Unlike

the Macro Assembler, CHASM doesn't require a second LINK step to

produce a working program.  CHASM also produces fast loading

programs without the use of the utility EXE2BIN.

CHASM supports several different simple mechanisms for getting

machine language routines into BASIC and Turbo Pascal, the two

most popular languages for the IBM Personal Computer.

Finally, the suggested payment for CHASM is a modest $40.

A Note for Beginners:

Before going on, you might find it useful to print and read the

file PRIMER.DOC, included on your CHASM disk.  PRIMER is a gentle

introduction to assembly language, which will teach you some of

the vocabulary and key concepts you will need to start out with.

2

>>What can CHASM do?<<

CHASM is a tool for translating assembly language into the native

machine language of the 8088 microprocessor.  Using CHASM, you

can write down easy to remember "mnemonics" which are then

converted into the relatively incomprehensible series of ones and

zeros that your PC prefers to work with.

In addition to simple mnemonic translation (such as provided by

the mini-assembler in DEBUG), CHASM provides a great many

"convenience" features which make writing machine language much

easier.

CHASM allows you to define labels for branching, rather than

requiring you to figure out offsets or addresses to jump to.

CHASM also lets you give symbolic names to any constants or

memory locations you use, to make your program easier to

understand.

You can instruct CHASM to make your file "BLOADable" so that

BASIC can load it as a subroutine.   A utility is also provided

to convert machine language into BASIC "DATA" statements, so that

BASIC can "POKE" routines into memory.  Similarly, for Turbo

Pascal CHASM can produce external procedures and functions, or a

file of Turbo "INLINE" statements.

CHASM has intelligent error and diagnostic messages which guide

you in correcting mistakes and ambiguities in your program.  A

nicely formatted listing is produced during assembly, to help

during debugging.

In general, CHASM is designed to eliminate much of the confusion

and dirty work involved in writing machine language for the IBM

PC.

Using CHASM, you can produce:

1. Lightning fast "stand-alone" programs.

2. Machine language subroutines for BASIC programs, both

interpreted and compiled.

3. Machine language procedures and functions for Turbo Pascal.

3

>>What WON'T it do?<<

In the interest of simplicity, CHASM has the following

restrictions:

1. Multiple segment definitions are not allowed. CHASM assumes

that your entire program fits in one segment, that the cs, ds,

and es registers all point to this same segment, and that the

ss register points to a valid stack area.  An equivalent

statement is that CHASM produces COM files, not EXE files.

2. Linking to Microsoft languages is not supported.  You can't

use CHASM to produce object modules for use with IBM/Microsoft

Pascal or FORTRAN.

4

>>System Requirements<<

Minimum system requirements to use CHASM are:

IBM PC or true compatible (must emulate IBM BIOS)

128K of memory            (192K for IBM PCjr)

1 disk drive

80 column display

DOS 2.0 or later.

Note the DOS 2 requirement.  To provide *true* DOS 2 support, it

was necessary to give up DOS 1 compatibility.  If you're still

using DOS 1, you'll need to upgrade to DOS 2.0 or later to use

CHASM.

Adding more memory will allow you to assemble larger programs.

CHASM can take advantage of all available memory, up to a

megabyte.

CHASM will run faster if your source files and object files are

on a hard disk or RAM disk.

If you have a non-IBM computer, please read about the /VIDEO

switch in "Modifying CHASM's I/O Defaults" and also the appendix

titled "Using CHASM on Compatible Systems"

5

>>Advanced and Subset CHASM<<

CHASM is available in two flavors, Advanced and Subset.

The two versions vary in their capabilities and method of

distribution.

The subset version is the budget release.  It may be freely

copied by individuals as "user-supported" software, and is

available from user groups and bulletin boards across the

country.  Every time the subset runs, it prints a banner page

suggesting a payment of $40 to the author.  As its name

suggests, Subset CHASM does not support all the features of

Advanced CHASM.

Advanced CHASM is the deluxe release.  It runs twice as fast, and

has a number of features not supported by the subset:

macros

conditional assembly

8087 support

include files

operand expressions

structures

Advanced CHASM is only available directly from Whitman Software.

Users who make the suggested $40 payment receive an upgrade to

Advanced CHASM.  Details, and an order blank are given at the

end of this document.

Throughout this document, features supported only by the

advanced version will be marked as follows:

==>Advanced version only.

Attempts to use these features in the subset version will

generally result in error messages, with the advanced feature

otherwise being ignored; however, unpredictable behavior could

result in some instances.

6

>>MODIFYING CHASM'S I/O DEFAULTS<<

CHASM supports the use of a configuration file, which can be used

to override some of CHASM's I/O defaults.  If you are willing to

accept CHASM's defaults, you may skip this section - on most

systems, CHASM will work perfectly well without the file

described below.

The configuration process involves supplying a text file named

CHASM.CFG on your default drive.  You can create this file with

any text editor or word processor.

You have some freedom in where you put the configuration file.

If CHASM.CFG is not found in the current directory, CHASM will

also try the following paths:

\CHASM\CHASM.CFG

\CHASM.CFG

The file should contain a series of text "switches" of the

following form:

/SWITCH, xx [, yy, zz,...]

Where "/SWITCH" is a reserved word, listed below, and xx, yy, zz

are numbers.  The brackets around yy and zz indicate that these

numbers are optional - don't put brackets in CHASM.CFG.

Each switch should start a new line, and the switch and each of

the numbers should separated by one or more blanks or commas.

The following switches are implemented:

/VIDEO      Video access method.  With this switch, you can

control the method CHASM uses to write data on your

video screen.  Three settings are currently

supported:

/VIDEO 0  Direct write to screen memory.

/VIDEO 1  Direct write to screen memory, but only

during the horizontal retrace interval.

/VIDEO 2  Write using BIOS calls.

7

/VIDEO      (continued) In general, the lower a number you

specify for /VIDEO, the faster CHASM will run.

/VIDEO 0 is the fastest mode, and is intended for use

with either the IBM monochrome display adapter or the

EGA adapter.  You can use /VIDEO 0 with the CGA

adapter, but some annoying "snow" and flicker may

result.

/VIDEO 1 is intended for use with the CGA adapter.

It is almost as fast as /VIDEO 0, and the snow is

eliminated.  /VIDEO 1 is CHASM's default mode.

/VIDEO 2 is intended for non-IBM systems that emulate

the IBM BIOS, but have significantly different screen

hardware.  If you have trouble running CHASM on a

"compatible" system, try setting /VIDEO 2.  Use of

/VIDEO 2 will slow CHASM down significantly.

/FG         Foreground color.  Users with color monitors may

select a foreground color from the following list:

0    Black               8    Gray

1    Blue                9    Light Blue

2    Green              10    Light Green

3    Cyan               11    Light Cyan

4    Red                12    Light Red

5    Magenta            13    Light Magenta

6    Brown              14    Yellow

7    White              15    High Intensity White

Example: (Magenta)

/FG 5

/BG         Background color.  Selections 0-7 above are

available. Example: (Cyan)

/BG, 3

8

/132        Printer 132 column mode.  The numbers following this

switch are the ASCII codes for the characters which

cause your printer to go into condensed mode.  You

may specify as many characters as you like.  If you

don't provide this switch, CHASM will truncate source

lines in listings to avoid going over 80 columns. You

can also include characters to activate any special

features of your printer you want CHASM to use during

printing.  Example: (for IBM printer)

/132, 15

/80         Printer 80 column mode.  Similar to /132, but the

numbers represent the characters to return your

printer to normal.  Include the codes for any

characters you want CHASM to send to your printer

before returning you to DOS.  The following example

turns off condensed mode and causes a Form Feed on

the IBM printer:

/80, 18, 12

/FF         Formfeed.  A value of zero tells CHASM that your

printer doesn't recognize ASCII 12 as a formfeed

command.  Any other value, and CHASM assumes that

formfeeds will be recognized.  The default is no

formfeed character.

If /FF is off, CHASM will simulate formfeeds with a

series of linefeeds.  However, most printers respond

quicker to a formfeed than to multiple line feeds, so

set /FF on if possible.  Example: (on)

/FF 1

/PAGELEN    CHASM assumes that there are 66 lines to each printed

page.  If you use different sized paper, enter the

number of lines per page after this switch.

Example: (European standard)

/PAGELEN 72

9

/LINES      By default, CHASM will print 58 lines on each printed

page, then skip over the perforation to the next

page.  You can change this number to suit your paper

and personal taste.  Example:

/LINES 50

/BEEP       Enables/Disables audible warning when errors are

discovered in your source program.  A value of zero

turns off beeping, anything else turns it on. The

default is on.  Example: (off)

/BEEP 0

/DWIDTH    When listing to a disk file, CHASM normally truncates

listing lines to 80 columns to prevent wrap-around

when viewing the file.  In some instances, such as

disk-based print spooling with DOS's PRINT utility,

you may wish to override this truncation.  You can

enter a new truncation limit after this switch.

Example: (128 column lines)

/DWIDTH 128

/DPAGE     For easier scanning, listings sent to disk files do

not normally contain page breaks.   If you want to

produce printable listing files, you can turn on page

breaks by setting /DPAGE to any value other than

zero.  Example: (page breaks on)

/DPAGE 1

10

/PATH     Path Strategy.  Affects the way CHASM constructs the

default file names for list and object files.  If

/PATH is set to 0 (the default), any drive or path

specified on the source file name will be included in

the default file names.  If /PATH is set to anything

else, path and drive info are removed, thus putting

the default list and object files in the current

directory of the logged drive.  The following table

shows how this works:

/PATH | Source filename   | Default object file

------|-------------------|-------------------------

0  | a:\chasm\test.asm |   a:\chasm\test.com

1  | a:\chasm\test.asm |   test.com

/PATH only affects how the default file names are

constructed - CHASM won't edit filenames that you

explicitly type in.

/TIMER     Enables/Disables reporting of total assembly time. A

value of 0 turns off timing, anything else turns it

on.  The default is off.  This switch is used

internally at Whitman Software to benchmark new

versions of CHASM.

A sample CHASM.CFG file, suitable for use with the IBM dot matrix

printer, is included on your CHASM distribution disk.

11

>>Syntax<<

CHASM accepts a standard DOS text file for input.  In this

context, "standard DOS text file" means:

1. Lines are terminated with a CR/LF pair.

2. The end of the file is marked with an EOF marker (cntl-Z).

Virtually every word processor or editor produces files which

meet these criteria.  Some word processors have to be in a

special mode to produce standard DOS files.  (For example, in

WordStar, you have to be in "non-document" mode.) The user's

manual for your editor should inform you if any special mode

needs to be activated.

Some users have reported problems with the user-supported word

processor PC-Write.  Under certain circumstances, older

versions of PC-Write can produce lines ending with just a LF

(violation of rule 1) or files which aren't terminated with an

EOF marker (violation of rule 2).  If you use PC-Write, make sure

you have the latest version.

Lines may be any combination of upper and lower case characters.

CHASM does not distinguish between the two cases: everything

except single quoted strings are automatically converted to upper

case during the parsing process.  Thus, BUFFER, Buffer, buffer,

and bUFFer all refer to the same symbol.

The characters  blank ( ), comma (,), single quote (')

semi-colon (;) and TAB are reserved, and have special meaning to

CHASM (see below).

Each line must be no more than 80 characters long and have the

following format:

Label Operation Operand(s) ;comment

The different fields of an input line are separated by the

delimiters blank ( ), comma (,) or TAB.  Any number of any

delimiter may be used to separate fields.

12

Explanation of Fields:

Label: A label is a string of characters, beginning in column 1.

Depending on the operation field, the label might represent a

program location for branching, a memory location, or a

numeric constant (see the section titled "Labels" for more on

this topic). To ensure that CHASM can distinguish between

labels and numeric constants, the first character of a label

must *not* be a number (0-9), plus sign (+) or minus sign(-).

Anything beginning in column 1, except a comment, is

considered a label.

Operation: Either a pseudo-op (see the section with the same

name) or an instruction mnemonic as defined in "The 8086

Book" by Rector and Alexy.  A list of acceptable mnemonics is

given in Appendix A.

Note 1: Except as modified below,"The 8086 Book" is the

definitive reference for use with CHASM.

Note 2: There are several ways to resolve some ambiguities in

8086 assembly language.  Please read page 3-285 of The 8086

Book, and "Resolution of Ambiguities" in this document.

Operand(s): A list of one or more operands, as defined in

the section titled "Operands", separated by delimiters.

Comment: Any string of characters, beginning with a semicolon

(;). Anything to the right of a semicolon will be ignored by

CHASM.

Note that except for the case of an operation which requires

operands, or the EQU pseudo-op which requires a label, all of the

fields are optional.  The fields MUST appear in the order shown.

13

>>Labels<<

The symbol in the label field of an input line can be interpreted

in four different ways by CHASM:

1. A program location which may be branched to.

2. A memory location for data storage.

3. An equated symbol, which takes the place of a numeric

constant.

4. A macro name, which takes the place of a series of

frequently used source code lines.

The default interpretation of a symbol is a program location for

branching.  This default is modified by the presence of one of

the following pseudo-ops in the instruction field:

DB, DM, DS, or DW:

Normal:  The symbol is a memory location.

In a Structure: The symbol is a numeric constant.

EQU:

Normal:  The symbol is a numeric constant.

Memory Option:  The symbol is a memory location.

MACRO:

The symbol is a macro name.

A given symbol may have only ONE of the above interpretations!

Attempts to branch into a memory location or an equated symbol

will result in error messages.  Similarly, CHASM will not allow

you to treat program code as a data area.  Examples:

TEXT DB  'Hit any key when ready'  ;memory location

MOV  AL,TEXT                  ;ok

JMP  TEXT                     ;wrong!

LOOP MOV  AX,CX                    ;program location

JMP  LOOP                     ;ok

MOV  AX, LOOP                 ;wrong!

14

If for some arcane reason you *need* to branch into a data area,

you can fool CHASM by placing a label on an otherwise blank line,

immediately before the data area.  Example:

JNZ NPU

RET

NPU                                ;dummy label for jump

DB  D8H, 00H                 ;an 8087 instruction

If you have a masochistic urge to crash your system by writing

self-modifying code, there are at least two ways you can defeat

CHASM's injunction against using program code as a data area.

The first way is to use DS to declare zero bytes of storage

immediately before the code you want to access.  A label on the

null DS will have the same offset as the immediately following

code.  Example:

MOV   JUNK1, 9090         ;change endless loop to NOP

JUNK1  DS    0

JUNK   JMP  JUNK

A sneakier approach is to load the OFFSET of a program

location into a register, then use the register for indirect

addressing. Using the optional displacement field, you can even

address the middle of an instruction.  Examples:

MOV   BX, OFFSET(CALL)

MOVB  1[BX], 00H          ;change interrupt number in code

CALL   INT   0

In general, I cannot recommend trying to get around CHASM's type

restrictions.  If you find yourself in a situation where it

seems necessary to fool CHASM, there's probably a safer, more

direct way to legally program what you're trying to accomplish.

Labels can be up to 80 characters long, but only the first 15

characters are significant.  For example, CHASM considers the

following labels identical:

VERYLONGLABELOVER15CHARACTERS

VERYLONGLABELOVER

15

To avoid ambiguity, a given string can legally appear in the

label field of only ONE statement.  If the same string appears on

more than one instruction, all instances after the first will

receive an error message.  Remember that labels are only

significant to 15 characters, and if the first 15 characters are

identical, an error will occur.

TWO   EQU  '2'      ;first use is ok

TWO   PROC FAR      ;wrong! symbol already defined

CHASM has a number of reserved strings which are "predefined" in

the symbol table, and will generate an error message if used as

labels.  All the register names are reserved, as are the indirect

address mode combinations.  The only other reserved strings are

the words "NEAR" and "FAR", and the symbol "$". Examples:

AX   MOV  AX, DX      ;wrong! (register name)

[DI] ADD  AX, BX      ;wrong! (indirect address)

FAR  CALL GETINPUT    ;wrong! (reserved word)

$    SUB  AX, DX      ;wrong! (reserved word)

16

>>Operands<<

The following operand types are allowed.

1. Immediate data: A number, stored as part of the program's

object code.  Immediate data are classified as either byte,

expressible as an 8 bit binary integer; or word, expressible

as a 16 bit binary integer.  If context requires it, CHASM

will left-pad byte values with zeroes to convert them to word

values.  Attempts to use a word value where only a byte will

fit will cause an error message to be printed.

Immediate data may be represented in 9 ways:

A. An optionally signed decimal number in the range -32768

to 32767.  Examples:

MOV AL, 21

MOV BX, -6300

B. A series of up to 4 hex digits, followed by the letter

H.  The first digit must be non-alphabetic, i.e. in the

range 0-9, to allow CHASM to distinguish between numbers

and symbols. If necessary, a leading zero, which does

not count in the four allowed digits, may be added to

fulfill the non-alphabetic condition.

If the RADIX pseudo-op has been used to change the

default number base to 16, the final H can optionally be

omitted.  Examples:

ADD   CX, 0B123H   ;leading zero required

RADIX 16           ;change default number base

ADD   DL, 12       ;same as 12H

C.  A series of up to 16 binary digits, followed by the

letter B.  Examples:

MASK   EQU   00000111B

MOV   AL, 10000000B

17

D. A symbol representing types A, B or C above, defined

using the EQU pseudo-op.  Examples:

MASK EQU 10H

MAX  EQU 1000

AND  CL,MASK

SUB  AX,MAX

E. The offset of a label or storage location returned by

the OFFSET operator.  OFFSET always returns a word

value. OFFSET is used to get the address of a named

memory location, rather than its contents.  Example:

MOV DI,OFFSET(BUFFER)

BUFFER DS  0FFH

F. The ASCII value of a printable character, represented by

the character enclosed in single quotes (').  Thus, the

following lines will generate the same object code:

MOV AL,41H  ;ascii code for 'A'

MOV AL,'A'

G. When producing Turbo Pascal INLINE code, the function

TURBO(x) assembles as 16 bit immediate data.

H. ==>Advanced version only:

Labels within structures become immediate operands whose

values equal their offset within the structure.  See the

section titled "Structures" for more detail and

examples.

I. ==>Advanced version only:

The length of a structure, returned either by the LENGTH

operator, or simply the structure's name.  See the

section titled "Structures" for more detail and

examples.

J. An expression that evaluates out to type Immediate.

Examples:

MOV AX, (3+2)*5

MOV DX, MEMLOC1-MEMLOC2

See the "Operand Expressions" section for more details.

18

2. Register Operands

A. An 8 bit 8088 register from the following list:

AH    AL

BH    BL

CH    CL

DH    DL

B. A 16 bit 8088 register from the following list:

AX   BX   CX   DX   SP   BP   SI   DI

C. An 8088 segment register from the following list:

CS   SS   DS   ES

D. An 8087 stack register from the following list:

ST ST(1) ST(2) ST(3) ST(4) ST(5) ST(6) ST(7)

Note: ST can also be referenced as ST(0).

3. Memory Operands: The contents of a memory location addressed

by one of the following methods.  Note that none of the memory

addressing options specifies the *size* of the operand.  8088

instructions have word or byte sized operands, and 8087

instructions can have word, short, long or temporary real

operands.  See the section titled "Resolution of Ambiguities"

for more on this topic.

All memory operands can optionally be preceded with a segment

override to specify the segment register to be used in

accessing memory.  The override takes the form of a segment

register, followed by a colon, followed by the memory operand.

No intervening delimiters are allowed.  Examples:

MOV AX, ES:[80H]   ;80H into the extra segment

MOV CS:[DI], SI     ;indirect with DI in the code segment

Segment overrides are also discussed in the section titled

"Resolution of Ambiguities".

19

A. Direct Memory Address.

1. A number or other immediate operand, enclosed in

brackets, indicating an offset into the data segment.

Example:

BUFFER EQU 5A5AH

MOV BH, [BUFFER]

MOV [80H], DI

MOV AX, CS:[TURBO(I)]

2. A symbol, defined to be a variable (i.e. a named memory

location) using the EQU pseudo-op.  Example:

FCB EQU [80H]

MOV DI,FCB

3. A symbol, defined to be a variable by its use on a

storage defining pseudo-op.  Examples:

MOV AX,FLAG

MOV DATE,BX

FLAG DS 1

DATE DB 31

4. The special symbol '$'.  $ returns an address of value

equal to the current setting of CHASM's location

counter.  This address can be used as either a memory

location or a program location for branching.  Used by

itself, $ has little utility, but it is a very powerful

tool when used in expressions.  Example:

MOV AX, $+4    ;get the byte after this instruction

5. ===> Advanced version only:

An expression that evaluates out to type address.

Example:

MOV AX, buffer+3

BUFFER DS 20

See the "Operand Expressions" section for more details.

20

B. Indirect Memory Address:  The address of the operand is the

sum of the contents of the indicated register(s) and a

displacement.  The register, or sum of registers, are

enclosed in square brackets: []

The displacement is optional, and takes the form of an

immediate operand, placed without intervening delimiters to

the left of the first bracket.

Immediate data written as decimal numbers, with values

between 127 and -128 generate a signed 8 bit offset.

Values outside this range, or those expressed in some

other manner generate 16 bit offsets.

The following indirect modes are provided:

1. Indirect through a base register (BX or BP).  Examples:

ENTRYLENGTH EQU 6

MOV AX, ENTRYLENGTH[BP]

MOV DL, -2[BX]

MOV CX, TURBO(COUNT)[BP]

MOV 9A9AH[BX], AX

2. Indirect through an index register (DI or SI).

Examples:

MOV [DI], CX

MOV CX, -5[SI]

3. Indirect through the sum of one base register and one

index register.  Examples:

MOV [BP+DI], SP      ;note that no spaces are

MOV BX, 10H[BX+SI]   ;allowed within the

MOV CL, [BP+SI]      ;brackets.

MOV DH, -2[BX+DI]

21

4. Labels

A label on most instructions may be used as an operand for

call and jump instructions.  See the section titled "Labels"

for more information.  Examples:

START    PROC NEAR

CALL GETINPUT

JMPS START

ENDP

GETINPUT PROC NEAR

5. Strings

A string is any sequence of characters (including delimiters)

surrounded by single quotes (').  If you want to use the

single quote character inside a string, put two of them

in a row for each one you want in the string. Example:

DB 'This is a ''string'' with embedded quotes'

22

>>Operand Expressions<<

CHASM can perform arithmetic calculations at assemble time, to

help you generate memory addresses and immediate data constants.

The following operand types can participate in operand

expressions:

immediate data

program locations

data storage locations

Actually, for the purpose of expression evaluation, CHASM treats

both program locations and data storage locations identically.

Thus, really only two types of operands are allowed in

expressions: immediate and address.

Symbols standing for immediate operands must have been defined

prior to use in an expression, or you may get Phase Errors (see

the error message section for a discussion on this topic).  You

should put all your EQU and STRUC pseudo-ops at the beginning of

your program, before any machine instructions.

The following arithmetic operators are available:

+  addition

-  subtraction

*  multiplication

/  integer division

Note that / performs *integer* division.  5/2 evaluates to 2.

The normal rules of precedence apply.  To force evaluation

contrary to normal precedence, you can use parenthesis: ().

MOV AX  2+5*3   ;means mov ax, 17 because * outranks +

MOV AX, (2+5)*3 ;means mov ax, 21

No delimiters can appear within an expression.  If you separate

the parts of an expression with delimiters, CHASM will try to

interpret each part as a separate operand.

MOV [BX], BUFFER + 3    ;WRONG!!

MOV [BX], BUFFER+3      ;ok

23

The type of an operand expression is determined by the individual

operand types that are combined within it.  In evaluating each

operation in an expression, the result type follows this rule:

If the two operand types are the same, the result type is

IMMEDIATE.

If the two operand types are different, the result type is

an ADDRESS.

In expressions, both program locations and data storage locations

are considered identical.  An expression which evaluates to type

address can be used in either a MOV or a JMP.

The result type of a complicated expression is determined by

replacing individual binary operations with their result, and

evaluating the (simpler) expression thus formed, following the

above rules.

Here's a program fragment with some expressions, and a discussion

of their types and significance:

EXPSAMPLE PROC  NEAR

MOV   AX, 3+5         ;immediate, calculated value

MOV   DX, STRING-END  ;immediate, length of string

MOV   STRING+5, 'A'   ;address, fifth byte of string

JMP   EXPSAMPLE+100H  ;address, used as program loc.

JMP   STRING+100H     ;valid address syntax, but silly

STRING    DB    'MESSaGE'       ;data area

END                             ;marks position of end of string

The special symbol '$' returns the current value of CHASM's

location counter for use in expressions.  $ is of type address.

Thus, $-1 is the address of the byte immediately prior to the

instruction currently being assembled.

24

The rules for typing expressions were set up to produce the

"most useful" result type, taking a guess as to why one would

want to do a given calculation.  If the type isn't what you have

in mind, you can coerce CHASM using square brackets or the

OFFSET function:

MOV AX, 3+5              ;immediate

MOV AX, [3+5]            ;coerced to address

MOV DX, $+4              ;address

MOV DX, OFFSET($+4)      ;coerced to immediate

Under certain circumstances, CHASM can get confused when you use

OFFSET or LENGTH functions in expressions.  The problem occurs if

the expression starts with a function, and ends with a close

parenthesis:

MOV AX, OFFSET(BUFFER)*(3+2)    ;will be misinterpreted

CHASM will try to interpret this as the offset of "BUFFER)*(3+2",

and you'll get the error message "Illegal or undefined argument

for OFFSET".  The solution is to enclose the whole works in

parenthesis, to force CHASM to recognize it as an expression:

MOV AX, (OFFSET(BUFFER)*(3+2))  ;correct

25

>>Resolution of Ambiguities<<

The language defined in "The 8086 Book" contains a number of

ambiguities which must be resolved by an assembler.  This is

discussed throughout the book, but pages 3-285 and 3-286

specifically cover this topic.  CHASM's solutions of these

problems are discussed in this section.

A. 8088 Memory references:

When one specifies the address of a memory location, it is

unclear how large an operand is being referenced.  An operand

might be a byte, or a word.

1. If a register is present as an operand, it is assumed that

the memory operand matches the register in size.  An

exception to this rule are the shift and rotate

instructions, where the CL register is used as a counter,

and has nothing to do with the size of the other operand.

Examples:

MOV MASK, AX  ;mask is a word

MOV DH, [BX]  ;BX points to a byte

NEG [SI]      ;error, operand of unknown size

SHR FLAG, CL  ;error, flag is of unknown size

2. If no register is present, (or if the only register is CL

being used as a counter) the size of the memory operand is

specified by adding the suffix "B" or "W" to the

instruction mnemonic.  Examples:

NEGB [SI]        ;SI points to a byte

SHRW FLAG, CL    ;flag is a word

MOVW MASK, 0AH   ;mask is a word

MOVB MASK, 0AH   ;mask is a byte

MOVW MASK, 9A9AH ;must specify size even though

;immediate operand implies word

26

B. 8087 Memory References:

All real and integer 8087 memory references are ambiguous as

to the operand size.  Integer operands could be word, short,

or long.  Reals can be short, long or temporary real.  As with

8088 memory references, you specify the size using a suffix:

W: word

S: short

L: long

T: temporary real

For more details and examples, see the section on 8087

support.

C. Indirect Branching.

The 8088 supports two flavors of indirect branching: intra, and

intersegment.  A register is set to point at a memory location

which contains a new value for the program counter, and in the

case of intersegment branching, a new value for the CS register

as well.

The syntax of "The 8086 Book" does not specify which flavor of

branch is being invoked.  CHASM adds the suffixes "N" (for near,

or intrasegment) and "F" (for far, or intersegment) to the

indirect CALL and JMP mnemonics.  Examples:

CALLN [BX]    ;intrasegment call

JMPF  [DI]    ;intersegment jump

JMP   [BP]    ;error, unspecified flavor

D. Long and Short Jumps

Two types of relative jumps are supported by the 8088: short

(specified by a signed 8 bit displacement) and long (specified by

a 16 bit displacement).  Both are implemented in CHASM as a jump

to a label.

The short jump is specified by mnemonic JMPS. Since one of the

displacement bits is used to indicate direction, only seven are

left to express the magnitude of jump.  JMPS (and similarly, all

the jump on condition instructions) is thus limited to branching

to labels within a range of -128 to +127 bytes.

27

CHASM reserves mnemonic JMP for the long jump.  JMP may be used

to jump anywhere within the program segment, but the object code

generated is less compact than that from JMPS.

Examples:

START PROC NEAR

JMPS END     ;short jump

JMP  START   ;long jump

END   ENDP

E. Instruction Prefixes.

The 8088 supports three instruction prefixes:

1. SEG: segment override. An alternate segment register is

specified for a reference to memory.

2. REP, REPE,REPNE,REPZ,REPNZ: repeat. A string primitive is

repeated until a condition is met.

3. LOCK: Turns on the LOCK signal. Only useful in

multi-processor situations.

SEG is implemented as a modifier attached to a memory operand.

If you want to override the default segment register for a memory

access, precede the memory operand with the segment register

followed by a colon.  Examples:

MOV AX, ES:FLAG     ;flag is in the extra segment

MOV CS:[100H], DX   ;offset 100H in the code segment

28

The other prefixes are implemented as separate instructions. They

appear on a separate line, immediately before the instruction

which they modify.  For compatibility with earlier versions of

CHASM, you can also specify segment overrides on a separate line,

using the mnemonic SEG. Examples:

REP

MOVSB           ;move bytes until CX decremented to 0

SEG SS

MOV AX,BUFFER   ;buffer is in the stack segment

LOCK

MOV [8FFH], AX  ;lock bus to ensure data is transferred

;before other processors try to access it

Note for 8087 users:  You may get unexpected results using the

separate instruction form of SEG on 8087 instructions.  Use

the operand modifier form for segment overrides on 8087

instructions.

29

>>Pseudo-Operations<<

The following pseudo-ops are implemented:

A. BSAVE: Generate object code in BSAVE format.

Instructs CHASM to build a header in the format of BASIC's

BSAVE command.  The resulting object code file may be BLOADed

by BASIC programs.  No operands are required, and the

pseudo-op may appear anywhere within the source code.

Example:

ORG    0     ;no psp

SUBRT PROC   FAR   ;subroutine for BASIC program

BSAVE        ;make BLOADable object file

B. COUNT ...ENDC:  Count bytes.

COUNT was available in earlier versions of CHASM. With the

addition of operand expression support, COUNT became obsolete,

and was eliminated to make room for new features.

Existing programs using COUNT can be modified as in the

following example.  The length of the string is calculated by

subtracting two labels, one just before the string, one just

after it:

MESSAGE    COUNT

MSG_TXT    DB   'This utility requires DOS 2.0!' beep cr lf

ENDC

MOV CX, LENGTH(MESSAGE)

becomes:

MSG_TXT    DB   'This utility requires DOS 2.0!' beep cr lf

MSG_END

MOV   CX, MSG_END-MSG_TXT

30

C. DB: Declare Bytes

Memory locations are filled with values from the operand list.

Any number of operands may appear, but all must fit on one

line. Acceptable operands are immediate data, or strings

enclosed in single quotes (').  DB interprets strings as a

series of ASCII bytes.

If a label appears, it is redefined as a memory location, and

the data area may be referred to using the label, rather than

an address. Examples:

MOV AX,MASK

MASK  DB  00H,01H

STG   DB  'A string operand'

CHASM generates an error message ("Data too large") if word

operands (such as OFFSETs, or numbers greater than 255) are

found in the DB operand list. DW should be used for declaring

words.

D. DM: Declare Multiple Bytes

Like COUNT, DM became obsolete when operand expressions became

available, and has been eliminated to make room for new

features.  Existing programs using DM can be modified as

follows:

DM  500, ENTRYLENGTH

becomes:

DS  500*ENTRYLENGTH

31

E. DS: Declare Storage

Used to declare large blocks of identically initialized

storage.  The first operand is required, a number specifying

how many bytes are declared.  If a second operand in the form

of a number 0-FFH appears, the locations will all be

initialized to this value.  If the second operand is not

present, locations are initialized to 0.   As with DB, any

label is redefined as a memory location.  To save space, the

object code does not appear on the listing.  Examples:

DS 10         ;10 locs initialized to 0

DS 100H,1AH   ;256 locs initialized to 1AH

F. DW: Declare Words

Used to unambiguously assign a word of storage for each item

in the operand list.  Any number of immediate operands may

appear, but all must fit on one line.  As with DB, any label

is redefined as a memory location. Example:

DW 0012H, FFFFH       ;four bytes declared

G. EJECT: Begin New Print Page

When listing is enabled, causes CHASM to move to the top of

the next listing page.  Normally has no effect on listings

sent to the screen or to disk, although you can enable EJECTs

in disk listings with CHASM's /DPAGE configuration switch.

H. EJECTIF: Conditional Page Break

Requires one immediate operand.  If listing is enabled

and fewer than that many lines are left on the current page of

the listing, CHASM will move to the top of the next page.

If you put an appropriate EJECTIF at the beginning of each

procedure or section of your programs, CHASM will keep them in

one piece.  Like EJECT, EJECTIF normally has no effect on

listings to the screen or disk.  You can enable EJECTIF for

disk listings with CHASM's /DPAGE configuration switch.

Example:

EJECTIF 20    ;following procedure is 20 lines long

32

I. ENDP: End of Procedure

See PROC (below) for details.

J. ENDSTRUC: End of Structure

==>Advanced version only.  See STRUC (below) for details.

K. EQU: Equate

Used to equate a symbolic name with a number. The symbol may

then be used anywhere the number would be used.  Use of

symbols makes programs more understandable, and simplifies

modification.

An alternate form of EQU encloses the number in square

brackets: []. The symbol is then interpreted as a memory

location, and may be used as an address for memory access.

This version is provided to allow symbolic reference to

locations outside the program segment. Examples:

MOFFSET    EQU 0B000H

MONOCHROME EQU [0000H]

Warning: Difficult to debug errors may result from using a

======>  symbol prior to its being defined by EQU.  You are

strongly urged to group all your equates together at

the beginning of programs, before any other

instructions. See "Phase Error" in the Error Message

section.

L. INLINE: Generate Turbo Pascal inline statements

Instructs CHASM to output object code in the form of a text

file, suitable for including in Turbo Pascal inline

statements.  The resulting object file is not directly

executable, but with minimal editing, can be compiled by Turbo

Pascal as inline data.

INLINE can appear anywhere in your source file, and requires

no operands.

See the "Execution of Assembled Programs" section of this

document for examples, and more details.

33

M. IFXX... [ELSE]... ENDIF: Conditional Assembly

===> Advanced version only.

CHASM's conditional assembly pseudo-ops can be used to cause

macros to expand differently according to the parameters

supplied on the invocation.  Many different IF pseudo-ops are

provided, to allow testing for (in)equality, relative size,

and (non)existence of parameters.  You can also test whether a

parameter is a register, and if so, what type.

For more details and examples, see the Macro section of this

document.

N. INCLUDE: Include file

==>Advanced version only.

INCLUDE requires one string operand, a filename enclosed in

single quotes.  If desired, you can specify a drive and/or a

path as part of the filename.

The contents of the specified file are logically inserted into

the source file at the point where the INCLUDE appears.

INCLUDEs cannot be nested: an error message will be printed if

the specified file itself contains an INCLUDE.  Example:

INCLUDE '\ASM\STDIO.HDR'    ;bring in standard library

O. LIST: Enable listing output

Output to the list device is enabled, presumably after a

NOLIST was encountered.  No operands required.

34

P. MACRO ...ENDM: Macro Definition

==> Advanced version only.

Declares a macro.  MACRO requires no operands, and signals

CHASM that the following lines constitute a macro definition,

and are to be stored for later use, rather than assembled in

place.  A label is required on the MACRO statement to name the

defined macro.

ENDM terminates the macro definition, and requires no

operands.

For more information and examples, see the section titled

"Macros".

Q. NOLIST: Disable listing output

Normal output to the list device is disabled.  Error messages

are listed as usual.  No operands required.

R. ORG: Origin

Allows direct manipulation of the location counter during

assembly.  By default, CHASM assembles code to start at offset

100H, thus leaving room for the program segment prefix

normally built by COMMAND or DEBUG.  In situations where no

PSP is provided, such as routines to be called from BASIC, you

should override this default with ORG, or incorrect assembly

may result.

ORG requires one operand, a number between 0 and FFFFH, which

represents the new setting of CHASM's location counter.

Although the location counter may be reset anywhere within a

program, generally this pseudo-op should be used before any

machine executable instructions for meaningful results.

Example:

ORG 0   ;Code will be assembled for starting

;offset of 0

35

S. PROC ...ENDP: Procedure Definition

Declares a procedure.  One operand is required on PROC, either

the word NEAR, or the word FAR.  This pseudo-op warns CHASM

whether to assemble returns as intra (near) or intersegment

(far).  Procedures called from within the program being

assembled should be declared NEAR.  Generally, all others

should be FAR.  ENDP terminates the procedure, and requires no

operands.  If a RET is encountered outside of a declared

procedure, an error occurs.  Procedures may be nested, up to

10 deep.  Example:

MAIN  PROC  FAR

...

...      ;body of procedure

ENDP

T. RADIX:  Default Number Base

CHASM's default radix is 10, meaning that numbers are assumed

to be in base 10 unless they end in "B" or "H".  The RADIX

pseudo-op allows you to change this default.  Allowed RADIX

values are 16 and 10.  Setting RADIX 16 allows you to specify

hex numbers without the trailing "H".

Note that when RADIX 16 is in effect, there is no way to

specify numbers in base 1 (binary) or base 10 (decimal).  To

write in either of these bases, shift back to RADIX 10.  For

example:

RADIX 16

mov   ax, 1B     ;means 1B in hexadecimal

mov   ax, 20     ;means 20 in hexadecimal

mov   ax, 30H    ;trailing "H" allowed, but not necessary

RADIX 10

mov   ax, 1B     ;means 1 in binary

mov   ax, 20     ;means 20 in decimal

mov   ax, 30H    ;means 30 in hexadecimal

36

U. STRUC ...ENDSTRUC: Structure Definition

==> Advanced version only.

Declares a structure.  STRUC requires no operands, and signals

CHASM that the following lines constitute a structure

template, and not actual storage declaration.  If a label

appears on the STRUC, the label is equated with the length of

the structure.

ENDSTRUC terminates the structure definition, and requires no

operands.

Inside the structure, storage defining pseudo-ops behave

somewhat differently.  See the section titled "Structures" for

more information. Example:

DIRENTRY STRUC      ;disk directory entry

NAME     DS    8

EXT      DS    3

ATRIB    DS    1

RESERVED DS   10

TIME     DS    2

DATE     DS    2

START    DS    2

SIZE     DS    4

ENDSTRUC

V. WAITON / WAITOFF: Toggle Automatic WAIT Assembly

CHASM normally assembles WAIT instructions before most 8087

instructions.  After a WAITOFF pseudo-op is encountered, CHASM

will not add WAITs.  This allows you to let the 8088 and 8087

run in parallel for greater speed, putting in WAITs manually

where synchronization is important.  WAITON turns automatic

WAIT assembly back on.

W. CHKJMP / NOCHKJMP

Following a CHKJMP pseudo-op, CHASM will check each JMP

instruction to see if it could have been coded as JMPS (to

produce tighter code).  JMP instructions with displacements

smaller than 128 bytes will be flagged with a diagnostic

message ("Could Use JMPS").  NOCHKJMP turns off JMP checking.

37

X. = : Assignment to Assembler Variable

CHASM supports the use of "assembler variables", which can be

dynamically redefined throughout your program.  The assignment

operator "=" acts similarly to pseudo-op EQU.  A label on a

"=" line will be defined as equivalent to the operand, until

it is redefined by another assignment statement.  Unlike EQU,

with "=" you can assign any valid operand type (except string)

to the assembler variable. These assignments do *NOT* result

in object code generation - they simply redefine a symbol to

have different meanings to CHASM in different parts of your

program.  For example:

x    =   [80]

mov ax, x      ;means mov ax, [80]

x    =   bx

mov ax, x      ;x  redefined, means mov ax, bx

x    =   100H

mov ax, x      ;redefined again, means mov ax, 100H

38

>>Macros<<

==>Advanced version only.

Macros are an advanced feature.  Beginners may wish to skip this

section until they become more experienced with CHASM and

assembly language programming.

A. Introduction

Macros are a shorthand way of writing frequently used

sections of code.  Using macros, you can write a code fragment

once, then any time you want to use it, just reference it by

name.  Once you define a macro and give it a name, anywhere

CHASM sees the name will be automatically "expanded" into the

previously defined code.

For example, suppose you were writing a large program, and at

the beginning of each subroutine you pushed all the general

purpose registers onto the stack to save their contents.

Every subroutine would start out something like this:

push ax       ;save register contents

push bx

push cx

push dx

Before long, you'd get pretty sick of writing the same thing

for each subroutine.  Macros are a way to put some of the

boring, repetitive nature of assembly language into the hands

of the assembler, freeing you up for the more creative

aspects.  Here's how you define a CHASM macro to take the

place of this code fragment:

savestate  macro

push ax           ;save general purpose registers

push bx

push cx

push dx

endm

39

Note the MACRO and ENDM statements.  These signal to CHASM

that the enclosed code is a macro definition to be saved for

later use, rather than code to be assembled at this point in

your program.  The MACRO statement also gives a name

(SAVESTATE) to the macro, which will take the place of the

stored code.

Now, at the beginning of each subroutine you can just write

"SAVESTATE" when you want to push the general purpose

registers. Here's an example:

get_input  proc  near

savestate

...

...

endp

Given the previous macro definition, the above example is

*exactly* equivalent to:

get_input  proc  near

push ax

push bx

push cx

push dx

...

...

endp

The only difference is that less busy work is required on your

part.

Macros are NOT subroutines!  Subroutines are coded once, then

called within your program at run time.  Macros are expanded

in-line at assembly time, and the code is inserted into your

program at the invocation point.  If you invoke the macro 20

times in your program, you'll end up with 20 copies of the

macro code.

40

Although this does waste some memory space, you save execution

time by eliminating the subroutine call and return process.

At a minimum, it takes 32 machine cycles to call a subroutine.

The four instruction macro given above requires 40 cycles to

execute.  If it were coded as a subroutine, the time to

execute it would almost *double*.  Macros trade off space for

speed.

B. Macro Parameters

CHASM's macros allow 9 user defined parameters, which are

evaluated at expansion time.  The parameters work just like

those in DOS batch files.  Here's an example of a macro with

one parameter.  PRINT calls DOS function 9 to print a string

on the console.  A parameter is used to specify the name of

the string to be printed:

print    macro

mov  ah, 09H          ;specify print string function

mov  dx, offset(%1)   ;point to string

int  21H              ;call DOS

endm

Given this definition, when CHASM sees a line like:

print title

The following code gets inserted in its place:

mov  ah, 09H

mov  dx, offset(title)

int  21H

Note how the "%1" in the macro definition got replaced by

"title" which was put as an operand on the invocation of the

macro.  You can put up to 9 operands onto a macro invocation,

and they will be substituted for the dummy parameters %1

through %9.

41

If you put a label on a macro invocation, two things happen.

As usual, the label represents a program location for

branching, with offset equal to the beginning of the expanded

macro.  You can branch into the expanded macro by using the

label as an operand on a jump or call instruction.  In

addition, if the special dummy parameter "%0" is used in the

macro definition, it is replaced with the label text when the

macro is expanded.  This feature is provided to facilitate

writing loops within macros.

If you fail to provide an operand for any dummy parameter used

in the macro definition, CHASM substitutes a null string of

length zero.  To leave one parameter blank, but provide

replacements for parameters with higher numbers, use the

special operand "%B" for the one you want blank.  For example:

def_mem  macro

%1       db  %2, %3

endm

def_mem twobytes, 128, FFH

def_mem %B, 'Hit any key when ready...'

Expands to:

twobytes  db 128, FFH

db 'Hit any key when ready...'

C. Internal Labels

A potential problem exists if you put a label on a statement

within a macro.  The first time the macro gets invoked,

everything works fine.  However, remember that a given label

can only be used once.  The second time you invoke the macro,

the line with the label will get a "Duplicate definition"

error message.

CHASM offers "internal labels" for use within macros.  When

expanding macros, CHASM replaces these internal labels with a

unique text, different for each invocation of the macro.

42

Macro internal labels are of the form:

%Lx

The "%L" signals CHASM that you want an internal label.  The

"x" can be any character that isn't a delimiter.  By using

different characters, you can define a many different labels

in a each macro.

Within the macro, you use the internal label symbol just like

a normal label.  For example:

INTLAB MACRO

%LA    MOV AX, DX

JMPS %LA

ENDM

Each time this macro is invoked, all occurrences of "%LA" will

be replaced with a different text.  (The replacement text will

be of the form "%LAnnnn" where "nnnn" is a number.)

Using the symbol table dump on your listing, you can figure

out what text CHASM used in any given invocation.  DON'T try

to use this information to branch into the macro invocation

from the outside.  Editing your source file can cause CHASM to

use a different substitution text the next time you assemble.

Internal labels are intended for macro INTERNAL use only.

D. Macro Nesting

Macro invocations can be nested, up to 10 deep.  Invocations

are maintained on a stack, and each invocation has it's own

set of parameters and internal labels.

CHASM does not check for recursive macro calls.  If you call a

macro from within itself, it's quite possible to get caught in

an endless loop.  This can also happen indirectly, where a

macro invokes another macro, which ends up calling the first.

If you get caught, you can escape by hitting Ctrl-Break.

Experienced programmers can use macro recursion along with

expressions and conditional assembly to create some really

elegant macros.  However, this is definitely not for beginners

or the faint of heart.  Enter at your own risk.

43

E. Conditional Macro Expansion

This advanced feature allows you to write general purpose

macros which expand in different ways based on the parameters

used on the invocation.

For example, suppose you wanted to write a macro to do

operating system calls.  Many of the DOS functions just load

the function number into AH, then call DOS with an interrupt.

Here's a macro to do this:

doscall  macro

mov ah, %1

int 21H

endm

Given the above definition, "DOSCALL 1" will expand into a

call to DOS function #1, keyboard input.

Unfortunately, not all DOS calls are quite so simple.  For

example, the call for printing a string to the console

requires that the offset of the string be loaded into DX.

Different system calls are going to require somewhat different

handling.  One approach would be to write separate macros for

each function.  However, with 87 different functions in DOS 2,

this starts to get unwieldy.

A more general approach is to incorporate some "intelligence"

into the macro, and let it expand differently for different

DOS functions.  Here's a slightly more general macro, which

loads DX whenever the "print string" function is requested:

doscall  macro

mov ah, %1

ife %1 9

mov dx, offset(%2)

endif

int 21H

endm

44

"IFE" is short for "If Equal".  The "IFE" and "ENDIF" bracket

a line of macro code which will be included during expansion

only if the "IFE" statement is satisfied.  In this example,

the MOV DX statement will be included only if the first

parameter is equal to 9.  The enclosed line is indented in

this example to help show the structure of the macro, but

indentation is not required.

Given the above definition, "DOSCALL 1" expands to only two

lines:

mov ah, 1

int 21H

However, "DOSCALL 9, STRING" expands to three lines:

mov ah, 9

mov dx, offset(string)

int 21H

CHASM supports twelve different conditional test statements.

They are:

IFE       "if equal"

IFNE      "if not equal"

IFGT      "if greater than"

IFGE      "if greater than or equal"

IFLE      "if less than or equal"

IFLT      "if less than"

IFB       "if blank"

IFNB      "if not blank"

IFREG     "if a register"

IFREG8    "if an 8 bit register"

IFREG16   "if a 16 bit register"

IFSEGREG  "if a segment register"

Most of the conditionals take two operands, and perform a

comparison.  The operands are evaluated, and their values are

compared according to the condition being tested.  Strings are

compared in the normal alphabetical order sense.  Examples:

IFE   15, 0FH       ;is true

IFGT  'ABCD' 'EFGH' ;is false

IFLE  20H, 128      ;is true

45

IFB and IFNB require only one operand, which should be a dummy

parameter.  These conditionals test whether the parameter was

left "blank" or if a replacement was provided on the

invocation.

The IFREG conditionals take one operand.  They return true if

the operand is a register in the following sets:

IFREG:    any register

IFREG8:   AH, AL, BH, BL, CH, CL, DH, DL

IFREG16:  AX, BX, CX, DX, DI, SI, BP, SP

IFSEGREG: CS, DS, ES, SS

You can place as many lines as you like between an IF

statement and the ENDIF.  All will be included if the

condition tested is true, and none will be included if it's

false.

An optional "ELSE" construct is also provided.  Statements

after the ELSE get included only if the tested condition is

false. CHASM's conditional assembly syntax can be summarized

as follows:

IF.....            ;if statement with parameter(s)

s1                  ;statements included if true

s2                  ;    "         "      "   "

ELSE               ;end of "true" option, begin "false"

s3                  ;statements included if false

s4                  ;    "         "      "   "

ENDIF              ;end of conditional assembly

IF statements can be nested, up to 10 deep.  During nesting,

CHASM assumes that ELSEs are paired with the latest unfinished

IF statement.  For example:

example macro

ife %1, 1

db 'Outer IF is true'

ifb %2

'Inner IF is true'

else

'Where do I belong?'

endif

endif

46

"EXAMPLE 1, 5" expands to:

db 'Outer IF is true'

db 'Where do I belong?'

But "EXAMPLE 2" produces no expansion at all.

This is easiest to follow if you use "structured" indenting

when writing the macro, as in the example above. Remember,

however, that CHASM ignores indenting and follows the

"latest unfinished" rule in figuring out which IF gets the

ELSE.  Don't fool yourself with improper indentation.

F. Final Notes:

The conditional assembly pseudo-ops are not restricted to use

only in macros, although there aren't that many useful

non-macro applications.

CHASM won't recursively expand parameters.  For example:

RECUR MACRO

XOR %1, %2

ENDM

RECUR AX, %1

expands to:

XOR AX, %1   ;actual expansion

NOT to:

XOR AX, AX   ;won't happen

Like equates and structures, macro definitions should all be

placed at the beginning of your programs, before the macro

gets invoked.  If you invoke a macro before it's defined, a

phase error will occur.

47

Macro definitions are stored and expanded blindly by CHASM,

with no syntax checking.  Dummy parameter symbols can appear

in any of the input line fields.  Any errors in a macro

definition will only become evident when CHASM expands the

macro and attempts to assemble the result.

In keeping with the "shorthand" philosophy of macros, the

results of macro expansion don't normally appear on the

listing.  If you want to see the expansion for debugging

purposes, insert a LIST pseudo-op as the first line of the

macro definition.

Once you start writing macros, you might find it useful to

gather them together into a single file.  You can then pull

this file into the beginning of all your programs with an

INCLUDE pseudo-op.  Macros which don't get invoked won't add

anything to the object code produced, and you'll save yourself

the trouble of typing in the ones you *do* need.  If you put a

NOLIST at the beginning and a LIST at the end of your INCLUDE

file, you don't even have to look at the definitions on your

listings.

48

>>Structures<<

==>Advanced version only.

Structures are an advanced feature.  Beginners may wish to skip

this section until they become more experienced with CHASM and

assembly language programming.

CHASM's structure capability allows you to generate a "template"

with which to organize repetitive data structures.  As an example

of a repetitive data structure, consider a phone list.  Each

entry in the list has three components:

Name    - 20 characters

Address - 50 characters

Phone   - 10 characters

Each entry thus uses a total of 80 bytes of storage.  To declare

a list with 500 entries, you would declare 80 x 500 = 4000 bytes:

PHONELIST  DS  4000   ;500 entries @ 80 bytes/entry

That's easy enough, but now what's the offset of the 346th

address field?  This is starting to get confusing, and time

consuming to figure out.

Furthermore, hard-coding numbers (like that 4000, above) into a

program is never a good idea.  What's going to happen when you

decide to add a zip code field, or make more room in the name

field because you just met Alexandria Zbrievskivich?  You'd have

to go through and change by hand each number which depended on

the actual layout of your data.  Murphy's law guarantees that

it'll take somewhere between "several" and "too many" assemblies

to find them all.

Structures allow you to set up a symbolic template which makes it

much easier to manage structured data like this phone list.  By

using symbols defined in the structure, rather than bare numbers,

a change in data structure doesn't mean a frantic search

throughout your entire program to make corrections.  If you

change the structure definition, the symbols take on new values

automatically.

49

Here's what the phone list structure looks like:

LISTENTRY  STRUC

NAME       DS 20

ADDRESS    DS 50

PHONE      DS 10

ENDSTRUC

Note the STRUC and ENDSTRUC statements.  They mark the beginning

and end of the structure, and give it a name (LISTENTRY).

Within the structure are storage defining pseudo-ops.  DS is used

in this example, but DB and DW could also be used.  ORG can be

used within a structure, but any 8088 instruction will result in

an diagnostic message and termination of the structure

definition.

Inside a structure, the storage defining pseudo-ops

behave somewhat differently than normal.  No actual storage is

set aside, but CHASM keeps track of how much space would normally

be declared.

Labels on pseudo-ops get assigned values equal to their offset

within the *structure*, not within the program as a whole.  Also,

CHASM will treat the labels as immediate operands, rather than

memory locations.  The result of the structure given above is to

generate three immediate operands, with the following values:

NAME    =  0

ADDRESS = 20

PHONE   = 70

CHASM does one other piece of useful book-keeping during

structure definitions.  If a label appears on the STRUC

pseudo-op, it gets treated as an immediate operand, whose value

is equal to the total length of the structure.

You can either use the STRUC label directly as an operand, or if

you like "pretty" code, use the LENGTH operator on it.  In this

example, both LISTENTRY and LENGTH(LISTENTRY) are immediate

operands of value 80.

(Inside note: LENGTH is a null operator, provided mainly for

aesthetic reasons.  Using LENGTH will often make your code more

readable, but is equivalent to using just the label itself.)

50

Like equates and macros, structures should be placed at the

beginning of your programs before any machine instructions,

otherwise phase errors can occur.  If you *do* embed structures

inside your program, you can eliminate any phase errors by using

the LENGTH function to reference any of the immediate operands

generated by the embedded structure.

The immediate operands generated during a structure definition

can be very useful in writing your program.  Following the phone

list example, here's a better way to declare storage for the

list:

NUMENTRYS  EQU 500

PHONELIST  DS  LISTENTRY*NUMENTRYS ;500 entries of length

;defined by the structure.

Now if you add another field to the structure, PHONELIST will

automatically increase in size.

The 8088's indirect addressing modes, coupled with structures,

make a very powerful combination for accessing structured data in

memory.  Suppose AX contains the number of the entry you want to

work on.  You can calculate the address of the entry as follows:

MOV BX, LENGTH(LISTENTRY)    ;length per entry

MUL AX, BX                   ;times entry number

ADD AX, OFFSET(PHONELIST)    ;plus the starting offset

MOV BX, AX                   ;BX <== frame pointer

BX is now a "frame pointer" - it points to the beginning of the

desired entry.  You can access the various parts of the entry

using the optional displacement field in the indirect address.

For example, here's how you would store the letter 'A' into the

first byte of the address field:

MOV  ADDRESS[BX], 'A'

As another example, the following line reads the third letter of

the name into the AL register:

MOV  AL, NAME+3[BX]

51

The first entry in a structure almost invariably has value 0.

CHASM "optimizes" indirect addresses with structure-generated

constants equal to zero, generating the no-offset form of the

address.  In this example:

MOV AX, NAME[BX]

actually assembles as MOV AX, [BX] rather than MOV AX, 0[BX]

The resulting code is more compact, and runs faster.

The more complicated indirect modes can be used to scan through

or point within the fields.  The following program fragment gets

the fourth digit in the phone number:

MOV DI, 4             ;specify 4th digit

MOV AL, PHONE[BX+DI] ;read it into AL

Often times, you'll want to process entries sequentially.  To

move to the next entry, you just add the length of the entry to

the frame pointer:

ADD   BX, LENGTH(LISTENTRY)    ;point to next entry

The preceding examples should give you a feel for what you can

do with structures, but do not exhaust all the possibilities.

Experiment with this feature, and many of your programs will be

both more readable and more easily modified.

52

>>8087 Support<<

===> Advanced version only.

In addition to the normal 8088 instructions, CHASM's Advanced

version supports all the 8087 mnemonics.  Look in the appendices

for a list available instructions.

Several books describing the functions of the 8087 instructions

are available.  CHASM's 8087 support is based on Appendix D of

Intel's 8087 Support Library, and the book "Assembly Language

Programming for the IBM Personal Computer" by David J. Bradley

(Prentice-Hall, 1984).

ALL the 8087 instructions that reference memory for a real or

integer quantity are ambiguous as to the size of operand.

Integers can be Word (2 bytes), Short (4 bytes) , or Long (8

bytes).  Reals can be Short (4 bytes), Long (8 bytes) or

Temporary Real (10 bytes).

As with 8088 memory references, CHASM resolves ambiguities using

a suffix.  The following suffixes can be added to mnemonics to

specify operand size:

W: word

S: short

L: long

T: temporary real

Here are some examples of using suffixes:

FADD   [200H]    ;wrong! ambiguous memory reference

FADDT  [200H]    ;add 10 byte temporary real

FILD   CS:[DI]   ;wrong! ambiguous memory reference

FILDW  CS:[DI]   ;load 2 byte integer

CHASM automatically generates a WAIT instructions prior to most

8087 instructions.  The only exceptions are the "No Wait"

instructions with a "N" as the mnemonic's second letter.

Examples:

FYL2X   ;automatically preceded by WAIT

FNCLEX  ;no wait form of FCLEX

53

If you'd prefer to manually add WAITs only where needed to

synchronize critical instructions, the WAITOFF pseudo-op disables

CHASM's automatic WAIT assembly.  Using fewer WAITs allows the

8088 and 8087 to run in parallel more often, giving better

performance.  Make sure you synchronize with a WAIT before

allowing the 8088 to access a location being modified by the

8087.  You can turn CHASM's automatic WAIT assembly back on using

a WAITON pseudo-op.

You must use the operand modifier form for segment overrides on

8087 instructions.  A separate SEG instruction will modify

CHASM's automatically generated WAIT instruction, and won't

affect the intended 8087 instruction.  Example:

SEG CS

FLDS [100H]     ;wrong!

FLDS CS:[100H]  ;correct

54

>>Outside the Program Segment<<

As mentioned previously, CHASM does not support multiple segment

definitions.  Provision is made for limited access outside of the

program segment, however.

A. Memory References:

To access memory outside the program segment, you move a new

segment address into the DS register, then address using

offsets in the new segment.  The memory option of the EQU

pseudo-op allows you to give a variable name to offsets in

other segments. For example, to access DOS's equipment flag:

BIOS_DATA   EQU  40H

EQUIP_FLAG  EQU  [0010H]

MOV  AX,BIOS_DATA  ;can't move immed. to DS

MOV  DS,AX

MOV  AX,EQUIP_FLAG ;get bios equipment flag

B. Code Branching:

CHASM supports 4 instructions for branching outside the

program segment.

1. Direct CALL and JMP

New values for the PC and CS registers are included in the

instruction as two immediate operands.  Example:

BIOS         EQU  0F000H            ;RAM bios segment

DISKETTE_IO  EQU  0EC59H            ;disk handler

JMP  DISKETTE_IO,BIOS

2. Indirect CALLF and JMPF

Four consecutive bytes in memory are initialized with new

values for the PC and CS registers.  The CALLF or JMPF then

references the address of the new values.  Example:

BIOS        EQU   0F000H           ;RAM bios segment

PRINTER_IO  EQU   0EFD2H           ;printer routine

MOV   [DI],PRINTER_IO

MOV   2[DI],BIOS

CALLF [DI]

55

>>Running CHASM<<

A. Prompt Mode

From DOS, type:

CHASM

If you're using the Subset version, a hello screen is printed,

followed by the message:

Hit Esc to exit, anything else to continue...

Advanced CHASM skips the commercial message.

You're now presented with a series of prompts:

Source code file name? [.asm]

Type in the name of the file which contains your program.  If

you don't include an extension for the filename, CHASM

assumes it to be .ASM.  If CHASM is unable to find the file,

it will give you the option of naming another file, or

returning to DOS.  Note that anywhere CHASM expects a

filename, you can optionally include a drive and/or path.

Assuming your file is present, CHASM prompts:

Direct listing to Printer (P), Screen (S), or Disk (D)?

[nul:]

Respond with either an upper or lower case letter.  If you

just press enter, no listing will be produced.  If you select

"D", CHASM will prompt:

Name for listing file? [fname.lst]

Type in a name for the listing file.  If you just press ENTER,

the name defaults to that of your source file, with an

extension of .LST.

56

Listing Notes:

1. The setting of the /PATH configuration switch controls

the exact form of the default list file name.

2. Regardless of where the listing is sent, error messages

are always echoed to the screen.

3. Suppressing the listing will result in faster assembly.

The final prompt is:

Name for object file? [fname.com]

Type in a name for the assembled program.  If you just press

ENTER, the name defaults to that of your source file, with an

extension of .COM.

Note: The setting of the /PATH configuration switch controls

the exact form of the default object file name.

CHASM now assembles your program.  A status line is maintained

on the screen, showing how many lines have been processed,

along with how many errors have been discovered.  CHASM makes

two passes over your source file, outputting the listing and

object code on the second pass.  You can pause assembly at any

time by hitting Cntl-S (or just S).  Hitting any key then

resumes assembly. You may abort assembly and return to DOS at

any time by hitting Esc, Ctrl-C or Ctrl-Break.

At the end of the second pass, a final summary of the assembly

process is printed.  It will look something like:

0 Error(s) detected

0 Diagnostic(s) offered

954 (3BAH) Bytes of object code generated

This information should be self-explanatory.  The number of

bytes is given in both decimal and hex format.

57

If labels appeared in your program, a dump of the symbol table

will follow.  This lists each user-defined symbol, along with

its value (in hex). The symbols are printed in alphabetical

order.  Each value is preceded by a one-letter code, which

tells the symbol's type:

P: a program location

M: a memory location for data

I: immediate data

Upon exit, CHASM sets the system variable ERRORLEVEL to

(surprise!) the total number of errors discovered in your

source file. If you run CHASM from a batch file, you can use

this feature to automatically invoke your text editor if

errors were discovered.

B. Expert Mode:

This mode allows you to specify all i/o information on the

command line which invokes CHASM.  The syntax is:

CHASM sourcefile [p|s|d|/] [listfile|/] [objectfile]

Items within brackets ([]) are optional. You may select *one*

of any list of items separated by a bar (|).

Basically, you just include on the command line all your

responses to the normal prompts.  Each response must be

separated from the others by either a space or comma.

If you don't specify the list device/file or the object file,

they default to NUL: and sourcename.COM respectively.  To

represent a carriage return (to specify a default choice, but

allow modifying a later response) use the character slash (/).

58

Expert mode examples:

1. Source file is EXAMPLE.ASM, no listing, object file

EXAMPLE.COM:

CHASM example

2. Source file is SDIR.ASM, list to printer, object file

SDIR.COM:

CHASM sdir p

3. Source file is MYFILE.PRG, list to disk file MYFILE.LST,

object file SUBR.COM:

CHASM myfile.prg d / subr.com

59

>>Error and Diagnostic Messages<<

Error messages generated on pass one appear on the listing before

any source code is printed, and mention the line number to which

they refer.  The majority of messages occur during pass two, and

will appear in the listing immediately prior to the line which

caused the message.  Unless the listing itself is going to the

screen, messages and the source line which generated them will be

echoed there.

Add Leading Zero to Hex Constant: Diagnostic.  The unknown

symbol could be interpreted as a hexadecimal number if a

leading zero was added.

CHASM Internal Error: XX PC: YYYY or

CHASM I/O Error: XX PC: YYYY

Sigh. You just discovered a problem in CHASM itself.  Please

contact Whitman Software, following the procedure in the

appendix on Bug Reporting.

Could Use JMPS: Diagnostic.  The specified label requires an

offset of less than 128 bytes; specifying the short jump would

result in more compact code.  The assembled code is correct,

however.

Conditional Nested Too Deeply:  IF statements can only be nested

10 deep.

Data too Large:  You are attempting to use a word of immediate

data where only a byte is allowed.

DM out of range:  The product of the DM's operands is either

negative, or greater than 32767.

Duplicate Definition of XXX in (linenum): Pass 1 error.  An

attempt was made to define a symbol already present in the

symbol table.

ELSE without IF: An ELSE was encountered, but no corresponding

conditional pseudo-op was found.

ENDIF without IF: An ENDIF was encountered, but no corresponding

conditional pseudo-op was found.

ENDM without MACRO:  An ENDM was encountered, but no

corresponding MACRO was found.

60

ENDP without PROC: An ENDP was encountered, but no corresponding

PROC was found.

ENDSTRUC without STRUC: An ENDSTRUC was encountered, but no

corresponding STRUC was found.

EQU Without Label: No symbol was found to equate with the

operand.

File not found: XXX in (linenum).  Pass one error.  CHASM was

unable to find the file XXX, specified in the INCLUDE

pseudo-op.

Heap Full: Too many XXX.  Usually a Pass one error.

You've run out of memory for the symbol and macro tables.  You

shouldn't see this message unless you have only 128K and are

assembling a very large program.

Illegal Label: XXX in (linenum). Pass one error.  The symbol

XXX begins in column one and has as its first character a

number, or a plus or minus sign.  Alternatively, you tried to

use a reserved word or symbol as a label.

Illegal Operation for Structure - ENDSTRUC Implied: Diagnostic.

The current line is within a structure, and is not a storage

defining pseudo-op.  CHASM generates an ENDSTRUC, which

terminates the structure definition, then assembles the line

normally.

Illegal or Undefined Argument for LENGTH:  The argument for the

LENGTH  function was not present in the symbol table as an

immediate operand on pass 2.

Illegal or Undefined Argument for OFFSET: The argument for the

OFFSET function was not present in the symbol table as a near

label or memory location on pass 2.

Missing ENDM:  The end of the input file was encountered, and at

least one MACRO had not been terminated by an ENDM.

Missing ENDP: The end of the input file was encountered, and at

least one PROC had not been terminated by an ENDP.

Missing ENDSTRUC: The end of the input file was encountered, and

at least one STRUC had not been terminated by an ENDSTRUC.

61

Multiple Segment Overrides are Illegal:  Diagnostic.  You have

specified more than one segment override on this instruction.

The first override is used, and the other(s) ignored.

Nested INCLUDE: An INCLUDE was encountered in an INCLUDEd file.

The INCLUDE pseudo-op cannot be nested.

Nested Structure: A STRUC was encountered inside a structure.

Structures cannot be nested.

No Name For Macro:  Diagnostic.  The macro statement did not have

a label.  CHASM is unable to give a name to the macro, and you

will be unable to reference it.

Operands Not Compatible:  The size of the two operands does not

match.

Phase Error: A label or memory location is found to have

different values on pass 1 and pass 2.  A difficult to debug

error: generally the problem is not caused by the statement

which received the error message. The problem is caused by an

improper statement before this one, but after any other labels

(otherwise *they* would have received the error message).

When phase errors are discovered, CHASM prints this message,

then resynchronizes the location counter to match the offset

calculated on pass one.  If further phase errors are reported,

the line responsable for each subsequent error will be located

between two Phase Error messages, but after any unflagged

labels.

There are four documented ways to generate phase errors.

1. A previous instruction used a symbolic immediate operand

prior to the symbol's definition.

2. A previous instruction made improper use of a forward

referenced label, either an attempt to branch into a data

area, or to access a code area as if it was data.

3. The label on the flagged statement is defined more than

once in the program.

4. A previous instruction invoked a macro prior to its

definition.

62

Whitman Software would appreciate hearing about any other

situations which cause the Phase Error message to appear.

Parameter Too Large for Expansion: Diagnostic. Replacement of the

dummy parameter would cause the macro line to exceed CHASM's

internal 255 character limit for manipulating strings.

Generally this message will be accompanied by the "Source Line

Truncated" message, warning that a line has exceeded the

allowed 80 columns.

Procedures Nested Too Deeply: Procedures may be

nested no more than 10 deep.

Source Line Truncated:  The length of the input line exceeded 80

characters.

Specify Word or Byte Operation: Diagnostic.  CHASM suggests that

the Syntax Error might be resolved by adding the suffix "B" or

"W" to the instruction mnemonic.  Most, but not all, ambiguous

memory references are flagged with this diagnostic.

Syntax Error: (OP) (DEST), (SOURCE).  CHASM was unable to find a

version of the instruction (OP) which allows the operands

(DEST) and (SOURCE).  Either the instruction doesn't exist, or

it is an inappropriate choice for the given operands.  The (OP)

(Dest), (Source) is a reconstruction of your source line based

on how CHASM parsed it.  A comparison of the reconstruction and

your original source code will sometimes help pinpoint the

error.

Syntax Error messages are followed by two diagnostics which

spell out in words CHASM's best guess about the operands.

Again, a comparison between CHASM's guesses and what you

really meant can help find the problem.

Too Far For Short Jump: The displacement to the specified label

is not in the range -128 to +127.

Undefined Operand for EQU: Any operands on an EQU statement must

have been previously defined.

Undefined Symbol XXX: The symbol XXX was used as an operand, but

never appeared as a label, and is not a predefined symbol.

63

Unrecognized Operand XXX: XXX is used in the DB or DW operand

list, but is not a valid immediate operand. (or string, in the

case of DB).

64

>>Execution of Assembled Programs<<

A. Object code format

The object code file produced by CHASM is in the form of a

memory image, exactly as will be present in your computer at

run time.  No link step is required.  Provided that the segment

registers are set correctly, the architecture of the 8088

guarantees that code is self-relocating, and will run correctly

loaded anywhere in memory.  Storing a program as an exact image

of memory at run time is called the COM format by IBM.

This COM format is *not* that produced by the IBM assembler.

The output of the IBM assembler is in the form of an "object

module" suitable for input to the linker.  The object module

is not directly executable, but must first be "filtered"

through the linker.  This adds an extra step to the process of

producing a working program, but gives you the option of

combining multiple object modules into one program.  The

resulting linked program is *still* not a memory image, but

has a header which is used to perform relocation during

loading.  This linked program plus header is called the EXE

format by IBM.

B. Running Assembled Programs From DOS

DOS provides a loader for running machine language programs.

To run a program, you merely type its name, without the

extension.  This is what you're doing every time you use a DOS

external command such as FORMAT or CHKDSK.  In fact, the COM

format is named after "external COMmand".

When DOS loads a program, it examines the file extension to

determine what format the file is in, either COM or EXE.  This

is why CHASM defaults to using the extension .COM for your

object file.  If you plan to run the program from DOS, don't

change the extension.

For COM programs, DOS builds a 255 byte long "program segment

prefix" and sets the segment registers to point to this PSP.

The contents of the file are then loaded verbatim right after

the PSP, at offset hex 100 in the segment defined by the

segment registers.  As soon as loading is complete, your

program is executed starting with the instruction at hex 100.

65

Although you can totally ignore the PSP, you should read pages

E-3 through E-11 of the DOS manual to see what DOS puts there

for you.  It turns out there are some real goodies which your

program might want to use.

When your program is done, it must transfer control back to

DOS, otherwise the 8088 will continue to fetch what it

believes are instructions from whatever garbage or bit-hash

happens to follow your program in memory.  The easiest way to

return to DOS is to execute the instruction:

INT 20H

This is the vectored interrupt reserved by DOS for program

termination.

While we're on the topic of vectored interrupts, you would be

well rewarded to study both the DOS Technical Reference and

Hardware Technical Reference Manuals to find out what happens

when you execute some of the other interrupts.  Some very

useful functions, such as file handling and screen i/o, are

available at the machine language level through this

mechanism.  Information on interrupts is also available in

Peter Norton's book "Programmer's Guide to the IBM PC", which

is cheaper than buying both of IBM's reference manuals, and

also more readable.

Looking at things the other way, by changing the interrupt

vector for a given function to point to your own code, you can

override the way DOS or the BIOS does something, and do it

your way.  DOS even provides a method (via interrupt 27H) by

which your new code can be grafted onto DOS, and not be

overwritten by other programs.

C. Debugging Assembled Programs

IBM provides an excellent utility with DOS, called DEBUG.COM.

By specifying your program's name as a parameter when invoking

DEBUG, you can observe your program execute with DEBUG's trace

and other functions.  To debug your program, from DOS type:

DEBUG progname.COM

66

DEBUG builds a PSP and loads your program just like DOS does,

but you have the added power of the debugging commands to

monitor your program while it runs.  See chapter 6 of the DOS

manual for more details about using DEBUG.

On the topic of debugging, I can recommend most highly a

program called TRACE86, from Morgan Computing (10400 N.

Central Expressway, Suite 210, Dallas, TX 75231).  The program

replaces DEBUG, and although rather steeply priced, makes the

IBM debugger look silly.  I've been using TRACE86 for some

time now, and wouldn't be without it.

D. Using Assembled Programs in BASIC

To incorporate a machine language subroutine in a BASIC

program, write it in assembly language, then assemble it with

CHASM.  You should read page C-7 of the BASIC manual for some

conventions to use in writing your subroutine.  In particular,

note that you must declare the routine to CHASM as a FAR

procedure using the PROC pseudo-op, and that the last

instruction of the routine should be a RET.

Unlike programs which are run directly from DOS, your routine

will not be preceded by a program segment prefix.  You should

prevent CHASM from leaving room for a PSP by putting an ORG 0

pseudo-op at the beginning of your routine. If you don't

include the ORG, memory references will not be assembled

correctly.  Example:

ORG  0    ;no psp

SUBR PROC FAR  ;far procedure

...       ;body of subroutine

RET

ENDP

CHASM supports two methods for getting assembled routines into

BASIC programs.  The methods differ in whether the routine is

included in the BASIC program file, or in a separate file.

A utility program called COM2DATA is provided for including

machine language within BASIC program files.  The program is

distributed in source code form (file COM2DATA.ASM) and must

be assembled with CHASM prior to use.  The program functions

as a DOS 2 filter, reading a COM file in from the standard

input, and writing a series of BASIC DATA statements to the

standard output.

67

COM2DATA's syntax is as follows:

COM2DATA [<infile] [>outfile] [linenum]

You specify the input and output files as with any DOS 2

filter.  The linenum parameter sets the starting line number

used on the BASIC code produced.  If you don't specify

linenum, it defaults to 1000.

If you MERGE the file of DATA statements into your BASIC

program, the program can then READ the data and POKE it into

memory.  An example program to do this is given on page C-6 of

the BASIC manual.  An alternative approach would be to store

the routine in a string variable, which could later be located

with the VARPTR function.

If you would prefer to keep your machine language subroutine

in a separate file, include a BSAVE pseudo-op somewhere within

your assembly language source code.  CHASM will build a header

on the object code produced, which will mimic that built by

BASIC's BSAVE command.  The resulting file may be BLOADed by

BASIC to any location in memory.

You transfer control to your routine with either the USR

function, or the CALL statement.  Syntax for these statements

can be found in the BASIC manual.

68

E. Using Assembled Programs with Turbo Pascal:

CHASM and Turbo Pascal work splendidly together, complementing

each other's strong points.  You can use CHASM to provide new

functions you wish Turbo had, or to fine tune a critical

procedure for optimum speed.   CHASM itself is written in a

combination of Turbo Pascal and CHASM.

CHASM supports two techniques for producing machine language

code for Turbo Pascal: external procedures or functions, and

Turbo INLINE code.

1. External Procedures and Functions:

Turbo loads external procedures and functions within the same

segment as the rest of your Pascal program.  You have no

control of the exact load location (more on this later), but

on the other hand, you don't have to worry about setting aside

a special location for your procedures (as in BASIC).  Since

your external procedure is loaded in the same segment as the

Pascal code, it should be declared NEAR to CHASM:

EXTERNAL PROC NEAR

...           ;body of procedure

...

ENDP

Turbo passes parameters to your procedure/function via the

stack.  To work effectively, a good grasp of the stack

structure is critical.  Read the Turbo manual for information

on internal data formats and parameter passing, to see just

what to expect on the stack.  Also, remember that the stack

grows down from the top of memory.

If you're going to access the stack in your procedure, the

first thing you should do is set up BP as a stack pointer.

Since Turbo also uses BP, you have to save the current value

first.  The obvious place to save it is on the stack...

PUSH BP           ;save old BP

MOV  BP, SP        ;and set up to indirectly address stack

69

You can now access the parameters on the stack using offsets

off the BP register.  Note that since you PUSHed BP,

everything is 2 bytes deeper onto the stack than what Turbo

originally sent you.

Here's an example.  Suppose you declare the following external

function in Turbo:

function Sum(x,y: int): int;

external 'sum.com';

After you've pushed BP, here's how the stack looks:

stack contents           indirect address

----------------------------------------------

<value of parameter y>        6[BP]

<value of parameter x>        4[BP]

<return address to Turbo>     2[BP]

<old BP value>                 [BP]

The indirect addresses go up two at a time, since each item on

the stack is a word (two bytes) long. You can access the

parameters using their indirect addresses. Here's the code for

an external function to add two integer parameters:

SUM  PROC NEAR

PUSH BP          ;save old BP

MOV  BP, SP      ;set up stack pointer

MOV  AX,  4[BP]  ;get parameter x

MOV  ADD, 6[BP]  ;add parameter y

;leave sum in AX to return to Turbo

POP  BP          ;restore old BP for Turbo

RET  4           ;clear params off stack

ENDP

A more elegant way to access the parameters is by using a

CHASM structure to define their offsets on the stack:

STACK    STRUC

OLDBP    DW   0000H

RETADDR  DW   0000H

XPARAM   DW   0000H

YPARAM   DW   0000H

ENDSTRUC

70

With this structure added to the above example, you could

access the parameters like this:

MOV  AX, XPARAM[BP]    ;get parameter X

Functions return scalar results by having the value in AX upon

return.  The function in the above example saves some time by

calculating the value in AX in the first place.  Upon exit,

the function POPs BP, to restore the value Turbo was using.

Note the RET 4 in the example.  This returns to Turbo, while

simultaneously POPing (and discarding) 4 bytes off the stack.

This clears off the two parameters which Turbo passed.  If

there were three parameters, you'd use a RET 6; if none, a

simple RET would do. When Turbo receives control, it assumes

that you've cleaned up the stack by removing all parameters.

If you don't do this properly, a run-time error, or even a

system crash will result.

(Typical scenario:  Turbo tries to return from one of its own

subroutines, thinking the top of the stack has the return

address.  Unfortunately, the top is really an integer

parameter, left over from your external function.  Turbo's

RET sends control into some random area of memory, and boom -

the system crashes.)

It's easy to get confused about the exact contents of the

stack.  If your procedure doesn't seem to be working right,

the first thing to suspect is Turbo and you have different

ideas about what's where on the stack.  A DEBUG session can

usually straighten things out.

Boolean functions constitute a special case which is poorly

documented in the Turbo Pascal manual.  Boolean functions must

return their result in two ways:

1. by setting the zero flag (Z = false, NZ = true)

2. and by returning either 0 (false) or 1 (true) in AX

The first return method is assumed by Turbo if you use the

function in a conditional statement, the second if you assign

the value of the function to a variable.  You need to return

the result both ways to cover all possible uses of your

function.

71

When external functions are called, in addition to the normal

parameters, Turbo passes something called the  "function

result" on the stack.  When the result is a scalar type, this

seems to be intended just as a local work area for you to use,

since Turbo ignores any value you store there.  Like a

parameter, the function result must be POPed off the stack

when your return to Turbo. Unlike scalar parameters (which

always occupy a word of stack memory, even if they are defined

as byte), the function result is the exact length of the

result type.  Thus for a boolean function, you have to POP off

one extra byte above and beyond those for clearing off the

parameters.

A problem of addressability can crop up if your external

procedure tries to maintain its own local variables and/or

constants.  The problem is that you have no way of knowing

just where Turbo is going to load your procedure within the

shared segment.  As such, the address CHASM calculates for any

memory locations are going to be offset from their real values

by some unknown constant, the offset of the procedure within

the shared segment.  This is called a relocation problem.

Fortunately, there's a way around this problem, but it

requires using a trick.  Your program has to figure out, *at

run time*, just where it's located in memory.  If you could

find out the offset of any known point in your procedure,

you'd "have your bearings" so to speak, and could go on.

The trick is as follows.  The 8088 CALL instruction pushes the

address of the next instruction onto the stack, then branches

to the location given in the CALL.  By performing a dummy

CALL, then stealing the value off the stack, we have the

location of a known spot in the procedure.   By subtracting

the offset within the procedure of that known location, we get

a pointer to the beginning of the procedure which can be used

to access everything else.  Here's an example:

72

LOCAL  PROC NEAR

PUSH BP                     ;set up to access stack

MOV  BP, SP                 ; ditto

CALL DUMMY                  ;establish addressability

DUMMY  POP  BX                     ;  "           "

SUB  BX, OFFSET(DUMMY)

MOV  AX, 4[BP]                ;get parameter

SEG  CS                       ;offset relative to CS

ADD  OFFSET(TOTAL)[BX], AX    ;maintain running total

POP  BP

RET  2

TOTAL  DW   0000H

ENDP

We use indirect addressing here.  After the funny business

with the CALL, POP, SUB sequence, BX has a pointer to the

beginning of the procedure.  Using indirect addressing, we

take that pointer and add in the offset of the memory location

we want to access.  In this example we're using a local

variable to maintain a running total of the parameter which

gets passed.

Note the SEG CS just before the ADD which accesses the

location TOTAL.  Since we found our bearings by stealing the

address of a program instruction, our offset is known relative

to the CS register, NOT the DS which is normally used to

access data.  The SEG CS forces the 8088 to calculate the

address using CS rather than the default DS register.  Every

time you access a memory location within your procedure, you

*MUST* do it relative to CS by using a segment override.

Turbo requires that you preserve the values of the following

registers:

BP, CS, DS, SS

If you want to use these registers in your routine, the

easiest way to preserve them is to PUSH them onto the stack at

the beginning of your routine, then POP them just before

returning.  As near as I can tell, you can safely trash the

other registers.

73

2. INLINE Code

An alternative method of incorporating code into Turbo Pascal

is through Turbo's inline statement.  The inline statement is

intended for short routines or patches where you just give

Turbo a list of numbers representing the code.  In addition to

numbers, you can also include variable names in the inline

list.  Turbo replaces them with their offsets at compile time.

CHASM's INLINE pseudo-op is provided to facilitate producing

Turbo inline code.  If you put an INLINE pseudo-op in your

CHASM source file, rather than producing a normal object code

file, CHASM produces a text file formatted to include in a

Turbo inline statement.  For example:

inline        ;shift to inline mode

mov  ah, 0FH  ;call bios for video mode

int  10H      ;  ditto

produces the following object file:

{    inline       ;shift to inline mode    }

$B4/$0F/      {    mov ah, 0FH  ;call bios for video mode}

$CD/$10/      {    int 10H      ;  ditto                 }

Object code is output in text form, as hex constants.  A

comment with the source code for each line is also generated.

The INLINE pseudo-op can appear anywhere in your source file,

and it requires no operands.

Object files produced from source files with an INLINE

pseudo-op are NOT executable!  They contain text suitable for

inclusion in Turbo Pascal inline statements.  It's probably a

good idea to override CHASM's default name for the object

file, and specify something with an extension other than COM

to prevent DOS from trying to run the program.

Your inline code must preserve the BP, SP, DS and SS

registers. If you need to modify these registers, you should

PUSH the ones you need at the beginning of your code, and POP

them at the end.

74

Turbo's inline statement allows you to insert variable names

in with the list of numbers.  At compile time, Turbo will

replace the name with the 16 bit offset of the variable in its

native segment.  (See the Turbo Pascal manual for a discussion

of internal data formats and the native segment of variables.)

CHASM supports this capability with the TURBO() function.

CHASM treats TURBO() as 16 bit immediate data during assembly.

However, in place of the two bytes of data, CHASM outputs the

function argument literally for Turbo to evaluate:

inline

mov   bl, turbo(flag)[bp]     ;local variable "flag"

mov   ax, cs:[turbo(maxcode)] ;typed constant "maxcode"

lds   si, turbo(y)[bp]        ;pointer to var parameter "y"

produces:

{      inline                        }

$8A/$9E/FLAG/       {      mov   bx, turbo(flag)[bp]     }

$2E/$A1/MAXCODE/    {      mov   ax, cs:[turbo(maxcode)] }

$C5/$B6/Y/          {      lds   si, turbo(y)[bp]        }

75

>>Notes for Those Upgrading to This Version of CHASM<<

CHASM is not yet carved in stone - improvements and corrections

are made fairly frequently, based on both my own experience in

using the program, and the comments of outside users.  This

section summarizes the changes which have been made since version

1.2 was released.  Changes followed with an asterisk (*) denote

modifications which could invalidate programs written under

earlier versions of CHASM.

Version   Notes

4.07      Speed enhancement.  Intersegment JMP and CALL fixed.

Expressions now allowed in the subset.

4.06      Speed enhancement.  Turbo Pascal INLINE facility.

/VIDEO configuration switch.  New conditionals: IFREG,

IFREG8, IFREG16, IFSEGREG.  New pseudo-ops: INLINE,

RADIX, CHKJMP, NOCHKJMP.  New function: TURBO(). Single

quotes now allowed in strings (use two).  Obsolete

COUNT, ENDC and DM pseudo-ops no longer supported. (*)

4.05      Speed enhancement.  Characters now work in expressions.

EJECT and EJECTIF appear *before* page breaks.  /DPAGE

configuration switch.  Parameter collection on macro

invocations now halts before comments.  SHL can now

also be written as SAL.

4.04      RET with displacement is now assembled correctly.

Parser bug fixed which crashed on strings with '/'.

4.03      Assembler variables.  Operand size incompatibility now

reported. FSUB now assembles correctly.  Intermittent

bug in expressions with forward references fixed.

Indirect memory references with offsets and segment

overides now assemble correctly.  Only the first

occurrence of a phase error is now reported.

4.02      DS storage assembly is now turned off during structure

definitions.  Structures assemble faster. Intermittent

problem with macro parameter expansion cleared up.

4.01      8087 support.  WAITON and WAITOFF pseudo-ops. Internal

labels for macros.  Corrects bug which caused crash on

certain syntax errors.

76

4.00      Total rewrite in Turbo Pascal.  Subset replaces

interpreted version as free distribution release.

Faster assembly.  Symbol and macro tables now use all

available memory.  Nested macros permitted.  Operand

expressions. $ now returns location counter value.  New

syntax for segment overrides.  Conditional pseudo-ops

now evaluate operands.  Improved error and diagnostic

messages.  DOS 2 or later now required (*). DOS 2 path

support for files. Setting of DOS 2 ERRORLEVEL. EJECTIF

pseudo-op added. New configuration switches: /DWIDTH,

/FF /TIMER, /PATH. CHASM.CFG must now have only one

switch per line (*).

3.15      Macros added. New pseudo-ops: MACRO, ENDM, IFE, IFNE,

IFB, IFNB, ELSE, ENDIF.

3.14      Interpreted version frozen at version 2.13, further

changes apply only to compiled version.  Memory

requirement raised to 128K.  INCLUDE, STRUC, ENDSTRUC,

DM, DW, LIST, NOLIST, COUNT and ENDC pseudo-ops added.

Alternate mnemonics for the jump on condition

instructions, and alternate syntax for the DIV and MUL

instructions.  Binary numbers added.

2.13      Assembly can now be aborted with the Esc key. Negative

decimal numbers are working again.  Input lines now

limited to 80 characters, and labels must begin with a

non-numeral. (*)

2.12      Listings can now be suppressed.  Error messages echoed to

the console on non-screen listings. Expert mode added.

2.11      Pagination improved.  Listings now time stamped. OFFSETs

and word values now allowed in DB operand list.

2.10      Equated symbols allowed in the DB operand list.  Status

line improved.

2.09      The first digit of hexadecimal constants must now be in

the range 0-9. A leading zero is permitted on four digit

hex constants, to allow fulfilling this condition. (*)

2.08      Configuration process expanded.  CHASM now skips over

perforations on printed listings.  EJECT pseudo-op added.

2.07      Oops.  Configuration file now works as advertised.

77

2.06      CHASM now supports reverse long jumps.

2.05      Compiled version released.  BSAVE pseudo-op.

Configuration process simplified.

2.04      TABs are now expanded and replaced with blanks, for

compatibility with IBM text editors.

2.03      Two bugs corrected.  The first bug involved incorrect

assembly of indirect memory references which used a

displacement in the range 128-255.  The second caused a

program crash if a hex number longer than 4 digits was in

found in the input file.

2.01      COM2DATA utility added.

2.00      Corrected a bug in the DS and DB pseudo-ops which caused

the last label in a program to be redefined as a memory

location.  Also, the TAB character was added as a new

delimiter, and PRIMER.DOC was added to the CHASM package.

1.9       The short jump is now represented with mnemonic JMPS, for

compatibility with DEBUG version 1.1. (*)

1.8       The operand type "character" was added as a new way to

represent immediate data.

1.7       The DS operator now works for blocks larger than 255

bytes.  Also, the OFFSET function now works properly in

the displacement field of an indirect memory reference.

1.6       A revision of this document.  Some sections were improved

slightly, and in response to user requests, a section on

execution of assembled programs was added.

1.5       Corrected an error which generated the message "Data too

Long" if the value FFH was used as 8 bit immediate data.

1.4       User interface improved.  CHASM now traps some common

input errors such as misspelling a file name, or

forgetting to turn on your printer.

1.3       A speed enhancement.  Version 1.3 benchmarks about 5

times faster than version 1.2.

78

>>Miscellaneous and A Word From Our Sponsor...<<

A. Programming Notes:

1. CHASM is written in a combination of Turbo Pascal and

CHASM.  This is less incestuous than it sounds: the

program was written in Turbo, then profiled to single

out the critical routines.   The rate determining sections

were rewritten in optimized assembly language, and

assembled with the original Turbo Pascal version of CHASM.

These routines were then incorporated as Turbo external

procedures and functions in a new version of CHASM.

The speed enhancements possible by "helping out" Turbo with

CHASM can be quite dramatic.  Replacing four rate limiting

routines out of the over two hundred routines in CHASM gave

almost a four-fold speed increase!

CHASM's source code is available to registered users by

sending a formatted disk and stamped return mailer.  If you

make any improvements, I'd like to hear about them for

possible inclusion in future releases.

Please note that although you can modify CHASM for your own

use, under NO CIRCUMSTANCES may you distribute modified or

translated versions, either in the public domain or for

profit.

B. Red Tape and Legal Nonsense:

1. Disclaimer:

CHASM is distributed as is, with no guarantee that it will

work correctly in all situations.  In no event will the

Author be liable for any damages, including lost profits,

lost savings or other incidental or consequential damages

arising out of the use of or inability to use these programs,

even if the Author has been advised of the possibility of

such damages, or for any claim by any other party.

Despite the somewhat imposing statement above, it *is* my

intention to fix any bugs which are brought to my attention.

See the appendix on Bug Reporting for more details.

79

2. Copyright Information:

The entire CHASM distribution package, consisting of the

main program, documentation files, and various data and

utility files, is copyright (c) 1983, 1984, 1985 and 1986

by David Whitman.  The author reserves the exclusive right

to distribute this package, or any part thereof, for

profit. The name "CHASM (tm)", applied to a microcomputer

assembler program, is a trademark of David Whitman.

CHASM's Subset version and various subsidiary files may be

copied freely by individuals for evaluation purposes.  It

is expected that those who find the package useful will

make a contribution directly to the author of the program.

The Subset version identifies itself by displaying a banner

page giving the author's address and inviting free copying.

ONLY VERSIONS DISPLAYING THIS BANNER PAGE MAY BE COPIED.

CHASM's Advanced version is only available to registered

users who have made the $40 suggested payment.  Registered

users may copy the program for backup purposes, but must

restrict use of the program to either one user or one CPU,

at their option.

CHASM's source code is made available for educational

purposes and to allow users to customize for their own

personal use.  Under NO CIRCUMSTANCES may modified versions

or translations into other computer languages be

distributed, either in the public domain or for profit.

User groups and clubs are authorized to distribute CHASM's

Subset version under the following conditions:

1.  No charge is made for the software or documentation.  A

nominal distribution fee may be charged, provided that

it is no more than $8 total.

2.  Recipients are to be informed of the user-supported

software concept, and encouraged to support it with

their donations.

3.  The program and documentation are not modified in ANY

way, and are distributed together.

80

Interested manufacturers are invited to contact Whitman

Software to discuss licensing CHASM for bundling with

MS-DOS based computer systems.

Distribution of CHASM outside the United States is through

licensed distributors, on a royalty basis.  Interested

distributors are invited to contact Whitman Software.

3. Royalty Information:

No royalties are required to distribute programs produced

using CHASM.  However, if you send me a copy of any major

program you have produced using CHASM, I'll give you a free

page of advertising in this document.

4. Educational Discount:

Substantial discounts are available for multi-CPU licenses

of CHASM's Advanced version to educational institutions.

Contact Whitman Software for details.

C. An Offer You Can't Refuse.

CHASM is User-Supported software, distributed under a

modification of the FREEWARE (tm) marketing scheme developed by

the late Andrew Fluegelman, whose inspiration and efforts are

gratefully acknowledged.

Anyone may obtain a free copy of CHASM's Subset version by

sending a blank, formatted diskette to the author. An

addressed, postage-paid return mailer must accompany the disk

(no exceptions, please).

A copy of the program, with documentation, will be sent by

return mail.  The program will carry a notice suggesting a

payment to the program's author.  Making the payment

is totally voluntary on the part of the user.  Regardless of

whether a payment is made, the user is encouraged to

share the program with others.  Payment for use is

discretionary on the part of each subsequent user.

81

The underlying philosophy here is based on three principles:

First, that the value and utility of software is best assessed

by the user on his/her own system.  Only after using a

program can one really determine whether it serves personal

applications, needs, and tastes.

Second, that the creation of independent personal computer

software can and should be supported by those who benefit

from its use.  Remember the Tanstaafl principal:  There

Ain't No Such Thing as a Free Lunch.

Finally, that copying and networking of programs should be

encouraged, rather than restricted.  The ease with which

software can be distributed outside traditional commercial

channels reflects the strength, rather than the weakness,

of electronic information.

If you like this software, please help support it.  Your

support can take three forms:

1. Become a registered user.  The suggested payment for

registration is $40.

2. Suggestions, comments, and bug reports.  Your comments will

be taken seriously - user feedback was responsible for most

of the changes listed in CHASM's revision history.

3. Spread the word.  Make copies of the Subset for friends.

Write the editor of your favorite computer magazine.

Astronomical advertising costs are one big reason that

commercial software is so overpriced.  To continue offering

CHASM this way, I need your help in letting other people

know about CHASM.

82

Those who make the $40 payment to become registered users

receive the following benefits:

1. An upgrade to the Advanced version of the program. The

Advanced version executes twice as fast as the Subset and

supports macros, conditional assembly and other features.

An order form for the Advanced version is given at the end

of this manual.

2. User support, by phone or mail.  Support is only

available to registered users.  Phone numbers and the

address for help are given below.

3. Notices announcing the release of significant upgrades.

CHASM is copyrighted, and users are requested NOT to make copies

of the Advanced version other than for their own use.  I am

strongly opposed to copy protection, and would regret being

forced to protect CHASM.  Please recognize the amount of time and

money which went into producing CHASM, and respect the wishes of

the author.

David Whitman

P.O. Box 1157

North Wales, PA 19454

(215) 641-7522 (days)

(215) 234-4084 (evenings)

83

Appendix A: 8088 Mnemonic List

This appendix lists the mnemonics which CHASM will recognize,

grouped roughly by function.  Consult "The 8086 Book" for

definitions of these instructions, and for the operands each will

accept.  Mnemonics marked with an asterisk (*) will accept a 'B'

or 'W' suffix for ambiguous memory references.

Arithmetic:

AAA      AAD       AAM       AAS       ADC*      ADD*      CBW

CWD      CMP*      CMPS*     DAA       DAS       DEC*      DIV*

IDIV*    IMUL*     INC*      MUL*      NEG*      SBB*      SUB*

Data Movement:

LAHF     LDS       LEA       LES       LODS*     MOV*     MOVS*

POP      POPF      PUSH      PUSHF     SAHF      XCHG     XLAT

Logical:

AND*     NOT*      OR*       TEST*     XOR*

String Primitives:

CMPS*    LODS*     MOVS*     SCAS*     STOS*

Instruction Prefixes:

LOCK     REP       REPE      REPNE     REPNZ     REPZ      SEG

Program Counter Control: (unconditional)

CALL     CALLN     CALLF     JMP       JMPF      JMPN      JMPS

RET

Program Counter Control: (conditional)

JA       JAE      JB      JBE      JC        JCXZ       JE

JG       JGE      JL      JLE      JNA       JNAE       JNB

JNBE     JNC      JNE     JNG      JNGE      JNL        JNLE

JNO      JNO      JNP     JNS      JNZ       JO         JP

JPE      JPO      JS      JZ       LOOP      LOOPE      LOOPNE

LOOPNZ   LOOPZ

84

Processor Control:

CLC      CLD       CLI       CMC       HLT       NOP       STC

STD      STI       WAIT

I/O:

IN       OUT

Interrupt:

INT      INTO      IRET

Rotate and Shift:

RCL*     RCR*      ROL*      ROR*      SAL*      SAR*      SHL*

SHR*

85

Appendix B: 8087 Mnemonic List

===> Advanced version only.

This appendix lists the 8087 mnemonics recognized by CHASM's

Advanced version, grouped roughly by function.

Arithmetic:

FADD     FADDP    FCHS     FDIV     FDIVP     FDIVR    FDIVRP

FIADD    FIDIV    FIDIVR   FIMUL    FISUB     FISUBR   FMUL

FMULP    FPREM    FSUB     FSUBP    FSUBR     FSUBRP

Mathematical Functions:

F2XM1    FABS     FPATAN   FPTAN    FRNDINT   FSCALE   FSQRT

FXTRACT  FYL2X    FYL2XP1

Data Movement:

FBLD     FBSTP    FILD     FIST     FISTP     FLD      FLD1

FLDL2E   FLDL2T   FLDLG2   FLDLN2   FLDPI     FLDZ     FST

FSTP     FXCH

Comparison:

FCOM     FCOMP    FCOMPP   FICOM    FICOMP    FTST      FXAM

Processor Control:

FCLEX    FDECSTP  FDISI    FENI     FFREE     FINCSTP   FINIT

FNCLEX   FNDISI   FNENI    FNINIT   FNOP      WAIT

Processor Status:

FLDCW    FLDENV   FNSAVE   FNSTCW   FNSTENV   FNSTSW    FRSTOR

FSAVE    FSTCW    FSTENV   FSTSW

86

Appendix C: Differences Between CHASM and That Other Assembler

Virtually all magazine articles about assembly language

programming on the IBM PC assume that the reader is using That

Other Assembler - you know, the outrageously priced one.  This

appendix will try to summarize the differences between the two

programs.  Please note that I do not own a copy of That Other

Assembler, and therefore this section is not complete, nor even

guaranteed to be correct.  I continue to work on this section,

and anyone with more experience is invited to make additions or

corrections, so this section will continually improve with time.

A. General Differences

The biggest difference is philosophical.  The IBM assembler was

designed for use by professional assembly language programmers,

to write operating systems and other huge projects.  This is

reflected in the large size and relative complexity of the macro

assembler.

On the other hand, CHASM was designed for use by beginners, to

write relatively short programs.  This was done by leaving out

some of the power offered by IBM's assembler, in exchange for

simplicity and ease of use.  The main simplification involved

producing object code in the COM format, rather than the EXE

format chosen by IBM. There are two main consequences of this

choice:

1. You can't link routines assembled by CHASM to

Microsoft languages. (Although you *can* include them in

Turbo Pascal or BASIC programs.)

2. Your program has to fit in one 64K segment.  If (shudder!)

you want to write a 256K assembly language program, you're

out of luck.

Like Pascal, the IBM assembler is a strongly typed language.  By

requiring you to specify the *type* of each memory location you

will access in your program, the IBM assembler generally knows

what size of memory operand you want.  If you don't like the

declared size, you have to override the default with the PTR

operator.  Thus, to loading the AL register from a location

declared word is a syntax error, unless you specify BYTE PTR

before the address.

87

In analogy to the C language, CHASM is weakly typed. CHASM is

perfectly happy extracting a byte from where you originally set

aside a word - CHASM can't tell the difference. In most cases,

CHASM can tell what size you want from context:  for example, if

you're using a word register, it *must* be a word memory access. On

the other hand, for any access to memory which doesn't have a

register as the other operand, you must add either a 'B' or a 'W'

to the instruction mnemonic used by IBM.

B. Miscellaneous Differences:

1. Short Jumps:

IBM uses the SHORT keyword, CHASM uses an 'S' suffix.

Example:

JMP   SHORT label    ;ibm

JMPS  label          ;chasm

2. Offset Function:

Where IBM precedes an operand with the keyword OFFSET,

CHASM has a *function* called OFFSET. CHASM requires

parentheses around the operand. Example:

MOV   AX, OFFSET FCB  ;ibm

MOV   AX, OFFSET(FCB) ;chasm

3. Declaring Storage:

A.  If you don't care what value a memory location is

initialized to, the IBM assembler allows you to specify

'?' as its contents.  In CHASM, if you don't care what

value the variable is initialized to, just put down a

zero.  Example:

DB   ?      ;ibm

DB   0      ;chasm

88

B.  The IBM assembler allows the keyword DUP as an operand

in storage declaring pseudo-ops.  This means to repeat

the definition as many times as the number just before

the DUP.  Example:

DW   3 DUP(?)  ;ibm

DW   0, 0, 0   ;chasm

4. ASSUME Pseudo-op:

IBM's ASSUME pseudo-op tells the assembler where the segment

registers will be pointing.  CHASM always assumes that the CS,

DS and SS registers point to the beginning of the code

segment, and that the SS register has been set up to point to

a valid stack area.  If you find an ASSUME pseudo-op with

different assumptions for the CS, DS and ES registers you'll

have to figure out the addresses for memory references in the DS

and ES segments yourself.

5. Segment Pseudo-op:

This pseudo-op is used to set up multiple segments in the IBM

assembler.  Since CHASM only allows one segment, there is no

equivalent pseudo-op.  If there is only one segment definition

in an IBM assembler program, everything is fine, just leave

the pseudo-op out for CHASM.

Often times the SEGMENT pseudo-op is used to provide

addressing of an area in the BIOS, or perhaps the interrupt

vector table at the beginning of memory.  For example, if a

program needed to get at the BIOS data area, in the IBM

assembler you would define a dummy segment with the same

structure as that in the BIOS listing in Technical Reference:

DATA          SEGMENT AT 40H

RS232_BASE    DW    4 DUP(?)

PRINTER_BASE  DW    4 DUP(?)

EQUIP_FLAG    DW    ?

MFG_TST       DB    ?

MEMORY_SIZE   DW    ?

IO_RAM_SIZE   DW    ?

All this is really accomplishing is giving a name to some

memory locations which are outside the actual program being

written.

89

In CHASM, you can simulate the dummy segment using a

structure.  This will generate a series of immediate operands

whose values correspond to the offsets of the labels in the

dummy segment.  You can then reference the locations by

enclosing the label names in square brackets, to coerce from

type immediate to type address.

DUMMY         STRUC            ;simulate dummy segment

RS232_BASE    DW    0, 0, 0, 0

PRINTER_BASE  DW    0, 0, 0, 0

EQUIP_FLAG    DW    0

MFG_TST       DB    0

MEMORY_SIZE   DW    0

IO_RAM_SIZE   DW    0

ENDSTRUC

MOV AX, [EQUIP_FLAG]

6. Labels:

The macro assembler indicates a local label by appending a

colon (:).  The colon does not become part of the label, and

is not included when referencing the label.  CHASM's labels

are all global, and although they may end with a colon, the

colon will become part of the label itself, and must then be

used when referencing the label.  Example:

a2:   mov ax,cx      ;ibm

jmp a2         ; "

a2:   mov ax,cx      ;chasm

jmp a2:        ;  "

CHASM does provide support for local labels in macros.  See the

discussion on internal labels in the macro section of this

document.

7. Entry Point:

The macro assembler allows you to specify the point within

your program where execution will begin.  A label is put

on the entry point, then to indicate entry, the same label is

placed on the "END" pseudo-op.  Since COM programs must always

start at offset 100H, CHASM doesn't allow setting an entry

point, or use the END pseudo-op.

90

Appendix D: Description of Files

Your CHASM distribution disk contains a number of files.  This

appendix will give a brief statement of the purpose of each.

FILE           DESCRIPTION

----------------------------------------------------------------

CHASM.CFG       Sample configuration file, for IBM printer.

CHASM.COM       The CHASM program (either subset or advanced)

CHASMS.COM      This file may appear on the disk of advanced

version users.  It is the current subset version,

that can be shared with others.  Please rename it

CHASM.COM on their disk, to avoid confusion.

CHASM.DOC       This document.

EXAMPLE.ASM     Sample source file.

COM2DATA.ASM    Source code for COM2DATA filter.

COM2DATA.DOC    Documentation for COM2DATA.

FREEWARE.DOC    References to other User Supported programs.

PRIMER.DOC      Simple introduction to assembly language.

Occasionally, various other sample source files for CHASM will be

distributed.  These files will have extension ASM, and will be

accompanied by a corresponding DOC file.

91

Appendix E: Bug Reporting Procedure

Although each version of CHASM is tested extensively prior to

release, any program of this magnitude is bound to contain a few

bugs.  It is the intention of Whitman Software to correct any

genuine problem which is reported.

If you think you have found a bug in CHASM, please take the time

to report it for correction.  Although any report is helpful,

correction of the problem will be easiest if you provide the

following:

1. The version of CHASM you are using.  Your problem may have

been fixed already!

2. A brief description of what you believe the problem to be.

3. A printed listing of a source file which manifests the

problem.

* DON'T send a 5,000 line program which has one

manifestation of the bug!  Isolate the problem area, or

write a short sample routine that demonstrates the bug.

Unlike normal commercial software, where corrections are saved up

for a major revision, bugs in CHASM are fixed as soon as reported,

with a new version released almost immediately (which is why there

are so many versions in CHASM's revision history).

=====> BONUS <======

If you send a copy of your problem source file on disk, it will

be returned with either a new, corrected version of CHASM, or

with an explanation of what you were doing wrong to *think* you'd

found a bug.

92

Appendix F: Using CHASM on "Compatible" Systems

CHASM was written specifically for the IBM PC, but should

function normally on true "compatibles".  This appendix is a new

section to summarize compatibility data for various systems.

Since CHASM version 4 is a totally new program, little

compatibility data is currently available.  If you are using (or

are unable to use...) CHASM on a non-IBM computer, please write

with your experiences.  Does CHASM work correctly on your system?

Are there specific problem areas?  Can they be worked around?

If you are using a non-IBM system, I strongly recommend that at

least to start out, you include the line:

/VIDEO 2

in your CHASM.CFG file (see the section on "Modifying CHASM's I/O

Defaults" for a discussion of the CHASM.CFG file).  This will

force CHASM to use BIOS calls to access your video screen.  By

default, CHASM writes directly to the screen hardware for maximum

speed.  Without this line, if your hardware is not *strictly* IBM

compatible, CHASM's output could be invisible, or your system

could even hang up and require re-booting.

The following systems are reported to run CHASM version 4

successfully:

AT&T 6300

Chameleon

Columbia 1600-1

Compaq, Compaq DeskPro, Compaq Plus

Corona PC-2

Heath 151

IBM PC, XT, AT, 3270 PC, 3270 PC/G

ITT XTRA

Kaypro 16

Leading Edge

Mega XT

PC Designs FD-1

Sanyo 555

Superior PC

Tandy 1000, 2000

Televideo 1605

Zenith 150

93

The following systems have one or more problems:

================================================================

Tandy 1200HD - CHASM crashes on some systems.  The problem seems

to be related to the exact amount of free memory available on the

system.  You can prevent the crash by slightly changing the

amount of free memory, to either a higher or lower number.

Probably the easiest way to do this is to change the number of

disk buffers, or load an extra device driver such as ANSII.SYS.

Call Whitman Software if you have trouble.

================================================================

================================================================

IBM PCjr. - Several users report getting as far as the source file

prompt, at which point the program crashes.  Several other users

(invariably with more than 128K of memory) report that CHASM works

fine.  The PCjr uses part of user memory for the screen buffer, and

PCjr users probably need 192K to run CHASM.

================================================================

94

********ADVANCED VERSION ORDER FORM********

Please add me to the list of registered CHASM users, and send me

an upgrade to Advanced CHASM.  I understand that CHASM is

copyrighted, and agree not to distribute any unauthorized copies

of this Advanced version.

Note that version 4 requires DOS 2 (or later)

and 128K of memory. (192K for PCjr)

Computer Model: ____________________________________

Diskette format:            Total Memory: _______K

__ single sided/9 sector

__ doubled sided/9 sector

Check one:

___ I enclose a check for $40

___ I am a past customer.  The enclosed check brings my

total payment up to $40.

Where did you hear about CHASM? ________________________________

Name:    _______________________________________________________

Address: _______________________________________________________

City, State, Zip: ______________________________________________

================================================================

Send order form and check to:

Whitman Software

P.O. Box 1157

North Wales, PA 19454

95

==============PRINTER ENHANCEMENT===================

Michael Hoyt, of Soft and Friendly Software, has produced a set

of printer enhancement programs using CHASM, which is sold under

the name Prowriter Utilities.  The package supports the following

printers:

NEC 8023A-C

Prowriter I  (C. Itoh 8510)

Prowriter II (C. Itoh 1550)

The package contains three programs:

PRINT_CHARACTERS

PRINT_SCREEN

PRINT_SET

Once PRINT_CHARACTERS is run, it attaches itself to DOS, and

makes your printer have exactly the same character set as your

video monitor.  The conversion is very professionally done.

Particularly impressive are the line drawing characters, which

actually form connected lines, both horizontally and vertically.

As if this wasn't enough, PRINT_CHARACTERS adds italics

capability as well.  The italics make very effective emphasis in

documents and letters, and look really good.

PRINT_SCREEN is a graphics screen dump, activated by the normal

Shift/PrtSc sequence.  Several options are available which trade

off speed and print quality.  Since I have the mono card, I

haven't tried PRINT_SCREEN, but Michael sent me a sample printout

which looked quite nice.

PRINT_SET is a menu-driven program to turn on and off the various

special printing modes supported by these printers.  A simple but

effective program.

I've been using this package with my NEC 8023 for a few months

now, and I like them quite a bit.  To get a copy, send $35 to:

Soft and Friendly

RR 2 Box 65

Solsberry, IN 47459

96

================= NEW PRODUCT ==================

If you use the IBM/Microsoft BASIC compiler, chances are your

programs are bigger and slower than they have to be.  If all

unreferenced line numbers are removed from your source program,

and the /N switch is used, BASCOM will "optimize" your program.

The result is tighter, more efficient code.

NUMZAP is a utility which carefully scans your source file, and

deletes all the non-essential line numbers.  Performing this task

by hand would be prohibitively time consuming and you'd probably

introduce errors into your program in the process.  NUMZAP will

do the job in minutes, 100% error free.

The old BASIC version of CHASM was passed through NUMZAP, and the

resulting compiled code shrank by a factor of 10% (!).  That 10%

reduction could make the difference between your program running

in 64K, or having users with minimal systems get "Out of Memory"

messages just before your program crashes.

An added advantage to using NUMZAP is that bigger programs can be

compiled.  You may not be aware that there is a limit on the size

of program which the compiler can handle.  BASCOM uses up space

remembering the offset of each line number in your program. If

you have too many numbered lines, BASCOM will run out of room and

you'll get a unending series of "TC" (Too Complex) error

messages.  By eliminating the unneeded line numbers, you give

BASCOM more elbow room.  The free space available to compile

CHASM increased 27% (!) after using NUMZAP.

NUMZAP is available under the standard FREEWARE deal - just send

a formatted disk and self-addressed, stamped return mailer to:

David Whitman

P.O. Box 1157

North Wales, PA 19454

Be sure to specify that you are interested in NUMZAP.  If you

like the program, a donation of $15 is suggested.

#################