This is a discussion on memory errors and fixes I participated in on

the Midwest's best BBS, EXEC-PC, (414)964-6330.

t: ALL

s: PARITY ERROR SCHTUFF

cc: TOM PETERS

cc: ROBERT SCRIP

I would like to know if there is a way to determine which ram chip is bad

based on the error message that is displayed when a NMI error occurs.

I have read a specific example of calculating this, but am not quite sure

how to apply that specific example to a more general case.  Any help?

At least two people (including me) would really like to know a GENERAL

FORMULA.  Also, how does one tell which is the B0 chip, which is the parity

chip, which is the B7 chip and so on?

---------------

c: HARDWARE  #975  05-03-88  11:17

f: TOM PETERS (LEADER)

t: JIM NICKEL

s: R: PARITY ERROR SCHTUFF  Reply to #969

Well, its hard to determine generally what chip is bad, given a message

from your bios saying you have bad memory. But it can be done.

Memory is arranged in an array 9 x n. More about n later. Usually the

physical layout of the board gives good clues about which given 9 chips

form a "column" or word. If you can turn the board so you have multiple

coulumns of 9 chips, it wil be easier.

NOTE: Some memory boards have no parity bit, so substitute 8 for 9.

Now, n. Each column represents 16, 64, or 256k. Nobody uses 4116 rams

anymore, so I'll address (sorry about the pun) only 64k (4164) and 256k

(41256) chips. Run CHKDSK if you can and get the figure for BYTES TOTAL

MEMORY. Or if you know how much memory should be in there, use that figure.

To convert the BYTES TOTAL MEMORY figure to k, divide it by 1024. Example:

mine says 655360 BYTES TOTAL MEMORY, divide by 1024 gives 640k.

Now, try to look at the arrangement of chips on the board and get it to

make sense to you in terms of enough 9 chip columns to make up the ammount

of memory you think you have. I have 3 columns of 41256 chips, which would

seem to be 768k, but DOS ignores anything above 640k. You could also have

10 columns of 64k chips (4164's) altho I doubt it.

Often you will find (in a 640k setup) 2 columns of 256k chips (that's 512)

plus two columns of 64k chips (total: 640k). You can mix them as long as

the larger chips come first.

Now, which chip went bad? Really, the most common answer is: The one with

the bent pin, or the one that's partialy popped out of its socket, or the

one with a staple draped over its connections. Or the one I just installed

backwards. When we get chips installed right and no physical damage occurs

to them, they tend to work right for quite a while. So look for the

obvious first.

To find a chip that's gone bad, you need two pieces of information:

1. At what address was the failure?

2. What bit pattern was written and what was read back? (and what is the

XOR of the two?)

The address is nearly always given by error messages. Its usually a hex

number, e.g. 40000h. Convert it to decimal, 262144, and divide by 1024, to

get a value in k: 256. So, somewhere between the 256k mark and the end of

the size of that column of chips, there is a bad one.

If you had 64k chips at that address, you'd be in row 4 (numbering from 1).

Right? 256k/64k=4 rows. Since they are 64k chips, any error from 40000h to

50000h is in row 4.

If they were 256k chips, an error at the 256k mark would have to be in the

second column.

Hex addresses and conversions:

64k    10000h

128k   20000h

192k   30000h

256k   40000h

320k   50000h

384k   60000h

448k   70000h

512    80000h

576k   90000h

640k   A0000h

704k   B0000h

768k   C0000h

Now, maybe your bios gives paragraph:offset form of the address. It would

look like 4000:0000 or something. That's easy to convert to a linear

address, just take the paragraph part, add a zero, and add the offset in.

You have to know how to add in hexadecimal. E.g. 4030:0020

40300

0020 +

-----

40320     < --- Linear address. Convert it to k with the table above.

Another example:

2020:0C80, 2020 plus a zero is 20200:

20200

0C80 +

-----

20E80     < -- linear address. If you have to do a lot of this, go out

right now and get a casio calculator that does hex/dec conversions and

arithmetic. Or easier to use is the TI LCD programmer II calculator. We sel

a lot of them to programmers.

I'll continue in another message with help in pinpointing the chip that's

bad. Good luck,

Thomas M. Peters,

Stevenson's Office Equipment

4517 N. Oakland Ave

Milwaukee, WI   53211   414-964-6330 (voice)

#############################################