WHATPORT.COM

Version 3.5

February 14, 1990

For detecting installed serial ports, type of mouse, and IRQ conflicts

Copyright 1989, 1990 Life Sciences Editorial Services

1236 River Bay Road

Annapolis, Maryland 21401

AUTHOR: Peter Petrakis (CIS 76555,1175)

IMPROVEMENTS IN THIS VERSION:

Version 3.5 of WHATPORT.COM corrects a defect that prevented

earlier versions from detecting COM3 and COM4 on PS/2 computers.

DESCRIPTION:

This program detects what serial ports (COM1-COM4) are installed

in an MSDOS or PCDOS system.  The type of mouse, if installed,

is also shown (serial, bus, InPort, PS/2, HP) together with the

interrupt request (IRQ) line it's using (unless it's a PS/2).

The program warns of potential IRQ line conflicts between the

mouse and serial ports.  In the case of PS/2 computers, where

there should be no possibility of conflict between the mouse

and a serial port, version 3.5 only reports the presence of the

PS/2 mouse and the serial ports that it finds installed.

PRINCIPLE:

Because PCs have only two IRQ lines for serial communications,

COM3 must share COM1's IRQ line and COM4 must share COM2's

IRQ line.  Unfortunately, two devices using the same IRQ

line can't be active at the same time.  COM3 can't be used for

a printer or modem if a serial mouse is active on COM1, for

example.  Likewise, COM4 can't be used if a serial mouse is

active on COM2.  In other words, if you have four COM

ports and a mouse is attached to one of them, you do not

have three COM ports available for other devices, you only

have two.  The COM port that shares the IRQ line being

used by the mouse is not usable.  WHATPORT detects and

reports such conflicts.

The following table shows what ports are available for other

devices when a serial port mouse is present on a multiple-port

DOS system:

Serial port mouse on     Ports available for other devices

_____________________    _________________________________

COM1 or COM3 (IRQ4)          COM2 or COM4 (IRQ3)

COM2 or COM4 (IRQ3)          COM1 or COM3 (IRQ4)

Conflicts with serial ports are also possible in the case of a

bus mouse.  For example, if a bus mouse is configured to use

IRQ3, there will be a conflict with both COM2 and COM4.  If it

is configured to use IRQ4, there will be a conflict with both

COM1 and COM3.  WHATPORT also detects and reports these conditions

so a different IRQ can be selected for the bus mouse.

HOW TO USE THE PROGRAM:

Just run it and read the results.  Adjustment for color or

monochrome display is automatic in this version, but monochrome

can be forced by putting /m on the command line.

DISTRIBUTION:

WHATPORT.COM and this document are provided free by

Life Sciences Editorial Services, Annapolis, Maryland.  They

may be copied and distributed promiscuously provided no

money is charged for them, they are not modified, and they

are not separated.  Although the program has been extensively

tested, Peter Petrakis makes no warranty concerning its

usefulness on all systems and he accepts no responsibility or

liability for any mishap resulting from its use.