MSYST

A User Terminal Program

For use with TNC's or Modems

By WA8BXN

(C) Copyright 1993 Hub Computers, Inc.

This program and documentation is shareware. If you find it useful please

send $5 (or more!) to support further development of this program and the

MSYS BBS system. Distribution of this program will be through electronic

means (the Cleveland Hamnet telephone BBS, for example). You may also

provide copies of the program and this documentation file to other

amateur radio operators interested in using it provided no changes are

made to this documentation file nor the program. Make out your check to and

send to:

Michael Pechura

10809 Beechwood Drive

Chesterland, OH 44026

Introduction

------------

This program was written primarily to allow use of the YAPP protocol

provided by MSYS for file (binary or ASCII) uploading and downloading

using either modem or tnc. It includes additional functions that make

it usable as a simple terminal program for general use. It can be used

with most TNC's that can be used with a terminal emulator program as

well as most modems. Basic features include

* YAPP protocol support

* ASCII file capture

* ASCII file transmission

* 800 line scroll back buffer

* DOS Shell to run other programs without exiting MSYST

* Interrupt driven serial I/O with 16,000 byte input buffer

* Support for serial ports at any address and Int 0-7

Installation

------------

MSYST automatically will create a configuration file the first time you

run it. It will ask you questions whose answers will be stored in the

file MSYST.DEF. If you ever want to change any of your selections, all

you have to do is delete MSYST.DEF and run MSYST to go through the

configuration procedure again. The following options are configurable:

* Serial port address

* Interrupt number

* Serial port speed

* Text background color

* Text foreground color

* CTS support (Turn it on unless it causes program to hang)

Usage

-----

For the most part what ever you type at the keyboard is sent to your

modem or TNC. The exceptions to this are the following keys:

F1 - Gives brief help information

F2 - Receive YAPP file after you tell BBS to send the file

F3 - Send YAPP file after you tell BBS to receive the file

F4 - Capture what appears on screen to a file

F5 - Send an ASCII file as if typed

F6 - End capture

F8 - Send password reply using PASSWORD.RMT file

F9 - DOS Shell (use exit command to return to MSYST)

Alt/X - Exit MSYST

Arrow keys, Page Up or Down, Home, and End enter scroll back mode.

To exit this mode, press Escape. F1 key (only F key that works when in

scroll back mode) will give help summary. Alt/X will exit scroll mode and

the program as well. While in scroll back mode incoming data is not

displayed but is accumulated in the 16,000 byte input buffer. When you

exit scroll back mode it will be displayed.

Using YAPP Protocol to transfer files

-------------------------------------

Using YAPP is particularly easy when you are using a modem connection to

MSYS. To download a file, type YD and the filename you want MSYS to send.

Then press F2 and give the name you want the file to have on your system.

You must complete this within 30 seconds of sending the YD command to MSYS.

Next sit back and watch the file status be updated as the file is received.

To upload a file using a modem, type YU and the name the file will have

on the MSYS system. When MSYS says its ok to send your file (note that

you have to be specifically enabled to upload files) press F3. Give the

name of the file you want to send and wait for it to be sent.

Using a TNC complicates things a bit because you have to switch your TNC

into transparent mode for the actual data transfer. To receive a file,

do the following:

Type YD followed by the name of the file on MSYS you want

When MSYS says to get ready to receive the file, type ^C to

get back into your TNC's command mode

Type the command for your tnc to get it into transparent mode

(Usually TRANS)

Press F2 and give the name you want the file to have on your system

When the file transfer is complete, excape transparent mode

(usually by typing ^C^C^C) to get back to your

TNC's command mode.

Now type the command to get your TNC in normal conversation

mode (usually CONV or K)

To send a file using a TNC, do the following:

Type YU and the name the file is to have on the MSYS system

When MSYS says its ok to send the file, type ^C to get to

the TNC command mode.

Get the TNC into transparent mode (see above)

Press F3 and enter the name of the file on your system

When the file transfer is complete, follow the instructions above

If there is enough interest and support for this program it will be

improved in the future. Possible enhancements could include VGA support,

compressed data transmission and "smart" interaction with MSYS. And of

course an improved documentation file. At this time I mainly want to

provide a way to use the YAPP additions to MSYS over the phone (most

of the other YAPP programs automate getting a TNC into transparent mode

and thus can't easily be used with modems) and to test the waters to

see what interest and support there may be for this program. Enjoy!