Copyright 1993 by U.S. Robotics, Inc. 8100 North McCormick Boulevard

Skokie, Illinois  60076-2999

SPORTSTER 14,400 and

SPORTSTER 14,400 FAX

CCITT V.32 bis with V.42 bis

14,400 to 300 BPS MODEM

COMMAND

SUMMARY****************************************************

A	Manual Answer: go off hook in answer mode.

Pressing any key aborts the operation.

A/	Re-execute the last command; use to redial. Does

not take AT or <Enter>.

Any key	Abort off-hook dial/answer operation and hang up.

AT	Required command prefix, except with A/ and +++.

Use alone to test for OK result code.

Bn	U.S./CCITT Answer sequence.

B0	CCITT answer sequence--Default

B1	U.S. answer tone

Dn	Dial the specified phone number. Include any of

the following options.

P	Pulse (rotary) dial--Default

T	Tone dial

,	(Comma) Two-second pause

;	(Semicolon) Return to Command mode after

dialing

"	(Quotation mark) Dial the letters that follow

!	(Exclamation point) Flash the switch hook

@	Dial, wait for quiet answer, and continue

W	Dial, wait for another dial tone, and

continue

R	Reverse frequencies

DSn	Dial the phone number string stored in NVRAM at

position n.

En	Command mode local echo. See DIP switch 4.

E0	Echo OFF

E1	Modem displays keyboard commands

Fn	Online local echo.

F0	Echo ON

F1	Echo OFF--Default

Hn	ON/OFF hook control.

H0	Hang up (go on hook)

H1	Go off hook

In	Information display.

I0	Three-digit product code

I1	Results of ROM checksum

I2	Results of RAM checksum

I3	Reserved

I4	Display current modem settings

I5	Display nonvolatile memory (NVRAM) settings

I6	Reserved

I7	Return product configuration

Mn		Speaker (Audio Monitor) operation.

M0	Speaker always OFF

M1	Speaker ON until CONNECT--Default

M2	Speaker always ON

M3	Speaker ON after dial, through CONNECT

On		Return online.

O0	Return online

O1	Return online and retrain

Qn		Result Codes displayed/suppressed. See DIP switch

3.

Q0	Display result codes

Q1	Quiet mode; no result codes

Q2	Result codes only in Originate mode

Sr=n		Set register r to n.

Sr?		Show contents of S-Register r.

Vn		Verbal/numeric result codes. See DIP switch 2.

V0	Numeric codes

V1	Verbal codes

Xn		Result code set. Default is X1.

Xn Setting

Result Code             X0	X1	X2	X3	X4

-------------------------------------------------0/OK                    +	+	+	+	+

1/CONNECT               +	+	+	+	+

2/RING                  +	+	+	+	+

3/NO ERROR              +	+	+	+	+

4/ERROR                 +	+	+	+	+

5/CONNECT	+	+	+	+

6/NO DIAL TONE			+		+

7/BUSY				+	+

8/NO ANSWER				+	+

10/CONNECT 2400	+	+	+	+

13/CONNECT 9600	+	+	+	+

18/CONNECT 4800	+	+	+	+

20/CONNECT 7200	+	+	+	+

21/CONNECT 12000	+	+	+	+

25/CONNECT 14400	+	+	+	+

--------------------------------------------------Functions

Xn Setting Result Code                 X0   X1   X2   X3   X4 -------------------------------------------------Adaptive Dialing			    +    +    +

Wait for 2nd Dial Tone (W)				    +    +

Wait for Answer @				    +    +

Fast Dial			    +	    +

Z		Reset to software defaults.

&An		Enable/disable ARQ result codes.

&A0  ARQ result codes disabled

&A1  ARQ result codes enabled--Default &A2  V.32 modulation indicator added

&A3  Protocol indicators added--LAPM/MNP/NONE (error control) and V42BIS/MNP5 (data compression)

&Bn	Modem's serial port rate.

&B0  Variable, follows connection rate--Default &B1  Fixed serial port rate

&B2  Fixed in ARQ mode, variable in non-ARQ mode

&Cn	Carrier Detect (CD) signal.

C0	CD override

&C1  Normal CD operations

&Dn	Data Terminal Ready operations.

&D0  DTR override

&D1  Reserved

&D2  Normal DTR operations

&F	Load the read-only (non-programmable) factory

settings.

&Hn	Transmit Data flow control.

&H0  Flow control disabled--Default

&H1  Hardware flow control, Clear to Send (CTS) &H2  Software flow control, XON/XOFF

&H3  Hardware and software control

&In	Received Data software flow control.

&I0		Flow control disabled--Default

&I1		Your XON/XOFF signals go to your modem and

to the remote system

&I2		Your XON/XOFF signals go to your modem only

&I3-4 Hewlett Packard: Host mode (&I3), Terminal mode (&I4)

&I5		ARQ mode--same as &I2; non-ARQ mode--look

for incoming XON/XOFF

&Kn	Data compression enabled/disabled.

&K0  Data compression disabled

&K1  Auto enable/disable--Default &K2  Data compression enabled &K3  MNP5 compression disabled

&Mn	Normal or Error Control (ARQ) mode (1200 bps and

higher).

&M0		Normal mode, error control disabled

&M1-3 Reserved

&M4		Normal/ARQ--Default

&M5		ARQ mode only

&Nn	Data rate, phone line interface.

&N0  Variable rate--Default

&N1  300 bps

&N2  1200 bps

&N3  2400 bps

&N4  4800 bps

&N5  7200 bps

&N6  9600 bps

&N7  12,000 bps

&N8  14,400 bps

&Rn	Received Data hardware flow control, Request to

Send (RTS).

&R0  Reserved

&R1  The modem ignores RTS--Default

&R2  Received Data to computer only on RTS

&Sn		Data Set Ready (DSR) operations.

&S0  DSR override; always ON--Default

&S1  Modem controls DSR

&Tn		Test modes. See Appendix F for testing

instructions.

&T0  End testing

&T1  Analog Loopback

&T2  Reserved

&T3  Local Digital Loopback

&T4  Enable Remote Digital Loopback

&T5  Deny Remote Digital Loopback--Default &T6  Initiate Remote Digital Loopback

&T7  Remote Digital with self test and error detector

&T8  Analog Loopback with self test and error detector

&W	Write current configuration to NVRAM.

&Yn  Break Handling.

&Y0  Destructive, but don't send Break

&Y1  Destructive, expedited--Default &Y2  Nondestructive, expedited

&Y3  Nondestructive, unexpedited

&Zn=s	Write phone number string s to NVRAM at position

n.

+++  Escape to Online-command mode or, if S14=1, hang up and escape to Command mode.

DIP SWITCHES**************************************************** *

Factory

Switch	Setting   Function

------------------------------------------------------------

1	UP	Data Terminal Ready (DTR) Operations

UP	Normal DTR operations: computer or

terminal must provide DTR signal for modem to accept commands; dropping DTR terminates a call

DOWN DTR always ON (Override)

2	UP	Verbal/Numeric Result Codes

UP	Verbal (word) results

DOWN Numeric results

3	DOWN Result Code Display

UP	Results suppressed (Quiet mode)

DOWN Results enabled

4	UP	Command Mode Local Echo

UP	Keyboard commands displayed

DOWN Display of commands suppressed

5	DOWN Auto Answer

UP	Modem answers on first ring

DOWN Auto answer suppressed

6    UP	Carrier Detect (CD) Operations

UP	Modem sends CD signal when it connects

with another modem, drops CD on disconnect DOWN CD always ON (Override)

7    UP	Power-on and ATZ Reset Defaults

UP	Load NVRAM defaults

DOWN Load factory (ROM) defaults

8    DOWN "AT" Command Set Recognition

UP	Recognition disabled (Dumb Mode)

DOWN Recognition enabled (Smart Mode)

SREGISTERS*************************************************** *****

Register* Default	     Function

-----------------------------------------------------------------

S0	See DIP	     Sets the number of rings on which

Switch 5  to answer in Auto Answer mode. When DIP switch 5 is UP, the modem

answers on the first ring; S0=1 has the same effect. When DIP switch 5 is DOWN (factory setting), Auto Answer is disabled; S0=0 (modem answers on 0 rings, or never) has the same effect.

S1	0		Counts and stores the number of

rings from an incoming call.

S2	43		Stores the ASCII decimal code for

the escape code character. Default character is "+". A value of 128-255 disables the escape code.

S3	13		Stores the ASCII decimal code for

the Carriage Return character. Valid range is 0-127.

S4	10		Stores the ASCII decimal code for

the Line Feed character. Valid range is 0-127.

S5	8		Stores the ASCII decimal code for

the Backspace character. A value of 128-255 disables the Backspace key's delete function.

S6	2		Sets the number of seconds the modem

waits before dialing, the minimum value for which is 2. If set to X2 or X4, the modem dials as soon as

it detects a dial tone (fast dials).

S7	60		Sets the number of seconds the modem

waits for a carrier. May be set for much longer duration if, for example, the modem is originating an international connection.

S8	2	Sets the duration, in seconds, for

the comma pause option.

S9	6	Sets the required duration, in tenths

of a second, of the remote modem's carrier signal before recognition by the Sportster.

S10	7	Sets the duration, in tenths of a

second, that the modem waits after loss of carrier before hanging up. If set to 255, the modem does not hang up when it loses carrier.

S11	70	Sets the duration and spacing, in

milliseconds, of dialed Touch-Tones.

S12	50	Sets the duration, in fiftieths of

a second (20 msec. units), of the guard time for the +++ escape code sequence.

S13	0	S13=64 disables MNP level 3 (used

for testing Level 2).

S14	0	If set to 1, the modem hangs up on

receipt of the escape code, returns to Command mode and sends the NO CARRIER result code.

S15	0	Bit-mapped. See Appendix B.

S16	0	S16=2 used for Dial Test, factory

testing.

S18	0	Test timer, disabled when S18 is

set to 0. Used to set the duration, in seconds, of modem diagnostics, before the modem automatically times out and terminates the test.

S19	0	Sets the duration, in minutes, for

the Inactivity Timer, disabled when S19=0.

S21	10	Sets the length of Breaks, in

10-millisecond units, that the modem sends to the computer during MNP calls.

S22	17	Stores the ASCII decimal code for

the XON character. Valid range is 0-127.

S23	19	Stores the ASCII decimal code for

the XOFF character. Valid range is 0-127.

S25	5	Sets the duration, in hundredths of

a second, that DTR must be dropped for the modem to respond.

S27	0	Bit-mapped. See Appendix B.

S28	8	Sets the duration, in tenths of a

second,of the V.21/V.23 fallback delay.

S34		6		Bit-mapped. See Appendix B.

S38		0		Sets an optional delay, in seconds,

before a forced hang-up and clearing of the Transmit buffer when DTR drops during an ARQ call. Has no effect if ATH is used to terminate a call.

*Gaps in the numeric sequence indicate reserved registers.

RS-232C INTERFACE PIN DEFINITIONS************************************

Signal

DB-25	DB-9 Circuit     Function          Source

------------------------------------------------------------

1	-	AA		Chassis Ground    Both

2	3	BA		Transmitted Data  Terminal

3	2	BB		Received Data          Modem

4	7	CA		Request to Send   Terminal

5	8	CB		Clear to Send          Modem

6	6	CC		Data Set Ready    Modem

7	5	AB		Signal Ground          Both

8	1	CF		Carrier Detect    Modem

12	-	SCF		Speed Indicate    Modem

20	4	CD		Data Terinal Ready     Terminal

22	9	CE		Ring Indicate          Modem

DEFAULT SETTINGS**************************************************** *

The following table lists the defaults stored in nonvolatile memory

(NVRAM) when the modem is shipped.

Setting		Function

----------------------------------------------------------

B0		CCITT answer sequence

F1		Online local echo disabled

M1		Speaker ON during dial through CONNECT

P		Pulse dialing type

X1		All result codes except No Dial Tone and Busy

&A1		ARQ result codes enabled

&B0		Computer/modem rate follows connection rate

&G0		Guard tone disabled

&H0		Transmit Data flow control disabled

&I0		Received Data software flow control disabled

&K1		Data compression auto enabled/disabled

&M4		Normal/ARQ mode

&N0		Variable connection rates

&P0		U.S. & Canada pulse dial ratio

&R1		Modem ignores Request to Send (RTS)

&S0		Data Set Ready (DSR) override

&T5	Deny remote digital loopback

&Y1	Breaks, computer to modem, destructive and

expedited

&Zn=s		Stored phone number s at position n--

Blank

S2=43		ASCII decimal escape code character

S3=13		ASCII decimal Carriage Return

S4=10		ASCII decimal Line Feed

S5=8	ASCII decimal Backspace

S6=2	Dial wait time, sec.

S7=60		Carrier wait time, sec.

S8=2	Dial command pause (,) time, sec.

S9=6	Carrier recognition wait time, 1/10th sec.

S10=7		Carrier loss wait time, 1/10th sec.

S11=70		Touch-tone duration/spacing, msec.

S12=50		Escape code sequence guard time, 1/50th

sec.

S19=0		Inactivity timer disabled

S21=10		Received Break length, 1/100th sec.

S22=17		ASCII decimal XON character

S23=19		ASCII decimal XOFF character

S28=8		V.21/V.23 fallback delay, 1/10th sec.

S38=0		Disconnect wait time, sec.

19.2K bps Bit rate

7 bits		Word Length

Even	Parity