PrintCAD 1.06

resident ADI/HP-GL pen plotter emulator

for use as a printer driver

with MS-DOS CAD packages

including AutoCAD and Microstation

(C) Copyright Ravitz Software Inc. 1991

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

p-1

Contents -------------------------------------------------------------

Introduction ..................................................... 2

License and Registration                                           3

What Is New ...................................................... 3

Installation                                                       4

Operation ........................................................ 4

Suggested Startup .BAT Files                                       5

Syntax ........................................................... 6

Overview of Options                                                7

/A - Location Option ............................................. 7

/C - Pen Color Option                                              8

/D - Output Destination Option ................................... 8

/F - Output Format Option                                          9

/J, /K - Printer Code Prefix, Suffix Options .................... 10

/L - Page Layout Option                                           11

/M - Magnification Option ....................................... 12

/O - Origin and Orientation Option                                12

/P - Intercept Mask Option ...................................... 12

/R - Internal Resolution Option                                   13

/S - Pen Shading Option ......................................... 13

/T - Temporary File Option                                        13

/W - Pen Width Option ........................................... 14

/X - Control Option                                               14

/Z - Chord Angle and Enhanced Font Option ....................... 14

Large Paper                                                       15

Canon BJ Printers ............................................... 16

Canon LBP Printers                                                16

Epson 9 Pin Printers ............................................ 16

Epson, NEC, Toshiba 24 Pin Printers                               16

HP LaserJet, DeskJet ............................................ 16

HP PaintJet                                                       16

IBM LaserPrinter ................................................ 17

Kodak Diconix Color 4                                             17

PostScript Printers ............................................. 17

Compatibility and Technical Information                           18

User Support .................................................... 19

p-2

Introduction ---------------------------------------------------------

PrintCAD is a pen plotter emulator that acts as a printer driver for

most CAD packages.  It  lets you print  drawings directly  from  CAD

programs,  via  the  CAD  program's  pen  plotter  driver  -  either

AutoCAD's ADI plotter driver or an HP-GL driver.

PrintCAD works with AutoCAD, AutoSketch, Microstation, Generic CADD,

Draft Choice, PC-Draft-CAD, and other CAD programs. It will print on

Epson  compatible  9  and  24 pin printers,  NEC and Toshiba 24  pin

printers, HP  LaserJet,  DeskJet,  and PaintJet,  Canon  LBP  and BJ

printers, IBM LaserPrinter, Kodak Color 4, and PostScript printers.

PrintCAD outperforms most  native CAD printer  drivers and  provides

additional  flexibility, such as pen width, shading, and color (with

color  mixing), and  paper  size,  orientation,  magnification,  and

positioning.  PrintCAD  uses the best graphics modes  available  for

each printer that it supports.

PrintCAD is a swapping resident program, using 5K of base memory and

368K of  EMS or  XMS memory.  Either  368K of  EMS or 368K of XMS is

required. PrintCAD automatically loads high if a 5K XMS upper memory

block is available.

PrintCAD has been  tested  in a  variety  of environments.  However,

because it is a swapping  resident program, unusual  situations  may

cause  it to  hang the system.  Please  test it  thoroughly on  your

system  before using it with  unsaved data.  PrintCAD  has  not been

tested under Microsoft Windows, and you should not expect it to work

in this environment.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to  make  sure  that  the  shareware

principle  works   for  you.   If  you   are  unable  to  resolve  a

shareware-related  problem  with  an ASP  member  by  contacting the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an  ASP member, but does not

provide technical support for members' products. Please write to the

ASP  Ombudsman at  545  Grover Road,  Muskegon, MI 49442  or send  a

CompuServe message via CompuServe Mail to ASP Ombudsman 70007,3536.

p-3

License and Registration ---------------------------------------------

PrintCAD is  provided  as is.  There are  no warranties expressed or

implied.  PrintCAD  is  a  swapping resident  program that may  have

problems with Microsoft Windows and unusual DOS enviroments.

PrintCAD is distributed as shareware.  You may use  PrintCAD without

charge on a trial basis to determine its suitability for you. If you

continue to use  it  after  your evaluation,  a $39 registration  is

requested.  This  registration  covers  use by  a  single person (on

multiple computers)  or installation  on  a single  computer (may be

used by a group of people).  There is no  cost to  use a later  1.**

version after registering any 1.** version.

You may distribute PrintCAD as a stand alone product if you keep the

entire   package  together,  unchanged,   including   this   license

explanation, and do not charge more than $10. You may not distribute

PrintCAD as part of another product.

To  register your copy of PrintCAD, send your name, address, and $39

check, and specify PrintCAD 1.06, to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

You will be sent  a  current disk  (specify  3.5  or 5.25).  If  you

previously registered PrintCAD,  you  may send $10 to get  a current

disk.

It would  be  helpful  if  you  note  your  CAD  package,  operating

environment, and PrintCAD startup parameters.

For users outside the USA, please make checks payable in US dollars,

and  except for Canada, please use  a check or money order from a US

bank if possible.  And don't forget to  include your country in your

address.

For users who register through a  business, don't  forget to include

your name.

What is New ----------------------------------------------------------

version 1.06 - /XA option provides compatibility with PC-Draft-CAD

version 1.04, 1.05 - updated to handle DOS 5.0 and DESQview better

version 1.01..1.03 - fixes problems with /FN, /FS

The  latest  version  of  PrintCAD is  available  from  the  support

bulletin board at 606-268-0577 (1200/2400,N,8,1 24 hours/day, XMODEM

or YMODEM (1K XMODEM) download), packed in .ZIP format.

p-4

Installation ---------------------------------------------------------

The PrintCAD package includes these files:

PCAD.EXE  program          PCADDV.EXE  special loader for DESQview

PCAD.DOC  documentation    README.*    quick information

To install PrintCAD copy the distribution files to your working disk

or directory.

PCAD.EXE  is  a stand  alone executable  file.  There  are no  extra

printer driver files or overlays.

PCAD.DOC is this manual. You can print it with "COPY PCAD.DOC PRN".

PCADDV.EXE is a loader for setting up PCAD underneath DESQview.

The  README.*  files are not needed to  run  PrintCAD, but they must

accompany the rest of the files if you distribute the package.

Operation ------------------------------------------------------------

When PrintCAD  is run  for the first  time,  it installs itself  and

swaps most  of  its  code  to EMS (expanded memory) or XMS (extended

memory). Subsequent runs look for the resident copy and send control

information to it, updating the parameters.

When resident, PrintCAD intercepts calls to DOS that the CAD package

makes to output HP-GL.  This data is used  to drive the printer.  To

make this work, you  must tell  PrintCAD how to recognize that HP-GL

data is being output from your CAD pacakage.  Choose a filename mask

that you can match  with your CAD package, such as *.PLT.  Set  this

with the /P option (/P*.PLT).  Inside your CAD package, output HP-GL

(HP 7475, HP 7440/17440, or HP 7550) to a file name that matches the

mask. PrintCAD intercepts the data and prints the picture.

For  AutoCAD and AutoSketch, use ADI  for the filename mask (/PADI).

Configure  AutoCAD for an ADI plotter on interrupt hex 78 - up to 15

pens, and  1016  dots/inch.  Then all output to this driver goes  to

PrintCAD via interrupt 78.  You can also use PrintCAD with AutoCAD's

HP-GL driver - setup is the same as noted in the previous paragraph.

After making PrintCAD resident, you can verify that it is functional

by copying an HP-GL file to "pcad" with the DOS COPY command.

PrintCAD  causes  a high beep after swapping in its code and  a  low

beep before swapping it out, to let you know that it is processing.

PrintCAD can be used with DESQview (DV). If you load PrintCAD inside

a  DV window, it will only be visible from that  window.  No special

steps are  needed for this.  If you load PrintCAD before running DV,

there may be problems, depending on your configuration.  If you have

problems, instead of running PCAD to  install PrintCAD, run  PCADDV.

This loads  PrintCAD at  the top of lower memory, preventing it from

overwriting DV code.  You must have a 5K XMS upper memory block  for

PCADDV to work.  Do not use  the /XL option  with PCADDV  as it will

cause PCAD's resident block to split lower memory.

p-5

Suggested Startup .BAT Files -----------------------------------------

Below are  some  sample one line .BAT files  for various situations.

Such .BAT files  may  be used  to  install  PrintCAD  or  change its

parameters.  You should not install PrintCAD from under a DOS shell,

but you can change parameters in this situation.

AutoCAD or AutoSketch, DeskJet or LaserJet IIP/III, 1/1 prints

PCAD /padi/fi

Microstation, Canon BJ330, 1/1 print, 11x17 paper

PCAD /p*.000/f7/l10,16,.25,.25/j27 67 0 17

AutoCAD or AutoSketch, LaserJet II, legal paper, best fit, center

PCAD /padi/mf1/a/fl/j27 38 108 51 65

PC-Draft-CAD, DeskJet or LaserJet IIP/III, 1/1 prints

PCAD /xa/p*.plt/fi

(/XA and/or /XL may be needed for use with PC-Draft-CAD.)

p-6

Syntax ---------------------------------------------------------------

The calling syntax of PrintCAD is:

PRINTCAD /option1 /option2 ...

Spaces between parameters are not required.

Except for the /X and  /P options, if a given option is entered more

than once, only the last entry is used.  And if an option is invalid

(but not incorrect), it is ignored. Only one /X and one /P option is

allowed.

If you do  not enter any  parameters, PrintCAD  displays  its option

reminder screen.

Below, [  and ] enclose optional suboptions and | separates mutually

exclusive suboptions. The options and suboptions may be in any order

except  that  multiple  numeric suboptions are  ordered  and numeric

suboptions must follow character suboptions where the characters may

be digits.  "c"  refers to  a suboption character,  "x" and  "y"  to

suboption  numbers,  "i"  and "j" to suboption  integers, "b"  to  a

suboption byte, and "f" to a DOS file or device name.

For numbers (x, y), a decimal point  is allowed but is not required,

and scientific notation is not allowed.  "-" is allowed but not "+".

For  integer  input (i, j),  only  base ten  integers from -32768 to

32767 are allowed. "+" is not allowed. A byte (b) must be a base ten

integer from 0 to  255 or a hexadecimal  number prefixed with $ from

$00 to $FF.  Further range checking is done for most options to flag

unreasonable input.  If an option  has multiple  numeric suboptions,

these may be separated with commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

p-7

Overview of Options --------------------------------------------------

PrintCAD's options fall into several categories.  The first is input

and output choices.  The input  option (/Pmask) lets you  choose  an

intercept filename mask, or you may select ADI interrupt 78 by using

"adi".  "pcad" is always intercepted. The output options are /F, the

output format (printer model), and /D, the destination port, device,

or file.

The  second  category is  the page layout, which is all contained in

the /L option.  This  includes  the  size and  location of the print

window (which defines the  hard clip  limits of  the plot), and  the

option to box the window.

The third category defines  how the  plot  looks  within  the  print

window.  /M,  the  magnification,  sets  the size  of  the plot.  /A

determines the location of the plot within the print window. /O sets

the  origin  and  orientation, and  is used to rotate or  mirror the

plot.

The  fourth category defines the pens, /W for the width,  /C for the

color, and /S for the shade.

And the last  category determines how PrintCAD processes the  plot.

This  includes the temporary file /T, the default and minimum chord

angle /Z, the internal  resolution /R, and /J  and /K which let you

set up and reset the printer for special situations.

/A - Location Option -------------------------------------------------

/A[O][x,y] - location - default /AO0,0

The A option determines the location of the plot in the print window

by specifying a point on the plot that is to be aligned with a point

in the print window.  You can specify the  plot alignment point with

x,y  in   inches   from  the   plot's   origin  (before   PrintCAD's

magnification is applied), or let it default to the plot center. For

the print window alignment point, you can default  to the  center or

specify the origin with  O.  By default the plot 0,0 is aligned with

the print window origin.

/A      puts the center of the plot at the center of the print

/AO0,0  puts the plot 0,0 at the print origin

p-8

/C - Pen Color Option ------------------------------------------------

/Cc.. - color - default /CK

The C option specifies  the color  of  each of the  eight pens.  The

colors are B for blue, C for cyan,  G for green, K (or L) for black,

M for magenta, R for red, W for white, and Y for yellow. Unspecified

pens use the last selected color (/CRGB is the same  as /CRGBBBBBB).

If  the output format does not allow color or color is suppressed in

the /F option then every color except white is converted to black.

Additional colors can be created with shade mixing. These colors use

shading patterns  to  mix  the  eight pure colors.  The format for a

mixed color is  .??  where each ?  represents any of  the eight pure

colors.  The first  color is printed  with the pen's shading pattern

(/S option)  and the  second  is printed  with  the  inverse shading

pattern.  If  the shade value is  0 (solid) it is replaced by 2 (one

pel checkerboard) which produces the most uniform mixed colors.

The mixed colors need line widths of at least 2 (4 for 2 pel shading

patterns) to look good, and they may  have visual artifacts on edges

near 45 degree angles.  Multipass dot  matrix formats may not handle

color mixing very well.

Only  a few of the printers that PrintCAD supports can handle color.

These are marked with ~ in the printer list (see /F option). Shading

may be used to emulate color on black only printers (see /S option).

/CK         sets pen 1..8 to black

/CKR.MB.KY  sets pen 1    to black

pen 2    to red

pen 3    to magenta on blue

pen 4..8 to black on yellow

/D - Output Destination Option ---------------------------------------

/D[+]f - destination - default /D1

The D option specifies the output printer port, device, or file. The

+ suboption  causes files to be appended rather than overwritten (it

has  no effect on printer port or device output).  1, 2, or 3 causes

PrintCAD to use  the BIOS  printer routines, bypassing DOS.  This is

generally preferred  to using  the DOS printer  devices (LPT1, LPT2,

LPT3).

If your  printer  is attached to  a  serial  port,  specify  /DCOM1,

/DCOM2, etc.  PrintCAD cannot set up the port.  This is usually done

in the AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

/D2          sends output to the second parallel port via BIOS

/DLPT3       sends output to the DOS device LPT3

/D+TEMP.PRN  appends output to file TEMP.PRN

p-9

/F - Output Format Option --------------------------------------------

/Fc[+|*][!][~][x,y] - output format - default /FN~

The F  option specifies the output format.  +, and *  are resolution

modifiers,  !  selects  a compression mode driver, and  ~  turns off

color processing on color printers.  +, *, !, and ~ are allowed even

where they have no effect.

You may  enter  the horizontal and vertical resolution of the output

device in dots/inch.  This overrides PrintCAD's normal output device

resolution, but does not affect  the  structure of the output format

(+, and * generally  do  alter the output structure).  These numbers

must follow the format character.

In addition to letting you choose any resolution for a bit map, this

feature  lets you compensate for print size  error.  For example,  a

360x360 dpi printer consistently under indexes so that a 6 inch high

box comes out 5.95 inches.  Using 360,363 (6/5.95*360 = 363) removes

the error.

Drivers marked below with ! have a compression mode driver, selected

with "!".  This may be  very useful, but it depends on your printer.

These  drivers compress the  output data by breaking  bit map strips

with printhead positioning commands.  This  saves data  transmission

time, but some  printers spend more  time positioning the  printhead

than is saved by not sending the data.

Drivers marked below with ~  are  color devices.  Using ~ suppresses

color processing, substituting black for all colors except white.

/F4     IBM LaserPrinter (PPDS mode) ...................... 300x300

/F7     Canon BJ (130E, 10E, 300, 330 - Proprinter mode)    360x360

/F9  !~ NEC 24 pin printers ............................... 180x180

/F9+ !~ NEC 24 pin printers (2 pass)                        360x180

/F9* !~ NEC 24 pin printers (4 pass) ...................... 360x360

/FD     Canon LBP (ISO mode)                                300x300

/FI     HP LaserJet 2P/3, DeskJet ......................... 300x300

/FK   ~ Kodak Diconix Color 4                               192x192

/FL     HP LaserJet compatible printers ................... 300x300

/FN   ~ Epson 9 pin compatible printers                     120x72

/FN+  ~ Epson 9 pin compatible printers (3 pass) .......... 120x216

/FN*  ~ Epson 9 pin compatible printers (6 pass)            240x216

/FO  !~ Toshiba 24 pin printers ........................... 180x180

/FO+ !~ Toshiba 24 pin printers (2 pass)                    360x180

/FO* !~ Toshiba 24 pin printers (4 pass) .................. 360x360

/FP   ~ HP PaintJet                                         180x180

/FS   ~ PostScript printers .............................. 1016x1016

/FT  !~ Epson 24 pin compatible printers                    180x180

/FT+ !~ Epson 24 pin compatible printers (2 pass) ......... 360x180

/FT* !~ Epson 24 pin compatible printers (4 pass)           360x360

p-10

/J, /K - Printer Code Prefix, Suffix Options -------------------------

/J[b,b,..] - printer code prefix - default /J

/K[b,b,..] - printer code suffix - default /K

/J lets  you send codes to the printer before PrintCAD's usual  data

is sent and /K lets  you send  codes after PrintCAD's usual  data is

sent.  This  lets  you  set  up and reset  the  printer  for special

situations. These are not needed in the normal use of PrintGL.

The  codes  are  specified  with  byte  values  (0..255  or $0..$FF)

separated with blanks or commas.  There is  a limit of 64 characters

for any option, so very long  sequences must  be handled outside  of

PrintCAD.

/J27 79   sets skip perforation off  to allow plots  larger than one

page on most continuous form printers

/K10 10   sends two line feeds after the plot

p-11

/L - Page Layout Option ----------------------------------------------

/L[B][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the print window edge using pen 8.

The first optional x and  y are the print window width and height in

inches rounded to  .01.  These define  the hard  clip limits.  These

values are not checked against  your printer's capabilities.  If you

specify a print  window  that exceeds the output  device limits, the

results are  unknown.  You may need to reset your printer's margins,

maximum line  length, or  page length to get large plots  (see Large

Paper). The size needed for a given width and height exceeds x and y

by 8 pels to allow for line widths.

The  second optional x and y are the left and top margins,  measured

from the printer's left margin and current  vertical position to the

print  window.  The top margin  is  modified  to match the printer's

native indexing,  but  is  accurate  to  .02 inch.  The  margins for

PostScript are measured from the device's 0,0 (left, bottom).

Each printer has a default page layout.  The default print window is

7.54x10.14.  This corresponds to HP 7440 A size  paper.  The margins

are printer specific to compensate for different paper handling.  In

evaluating user  options, PrintCAD  processes each  option and  when

finished reprocesses the default page layout for the chosen  printer

and  then reprocesses the last user /L option.  This procedure gives

good default values so that your /L option can change the letter  or

size suboptions and still have good margins.

/LB7.5,3,.5,0  makes  the  print window 7.5 inches wide and 3 inches

high,  with  a .5 inch left margin and no top margin,

and boxes the print window

/L7.54,10.14   sets window equivalent to HP 7440 with A  paper

/L7.54,10.74   sets window equivalent to HP 7440 with A4 paper

/L7.84,10.20   sets window equivalent to HP 7475 with A  paper

/L7.60,10.88   sets window equivalent to HP 7475 with A4 paper

/L10.20,16.38  sets window equivalent to HP 7475 with B  paper

/L10.88,15.90  sets window equivalent to HP 7475 with A3 paper

p-12

/M - Magnification Option --------------------------------------------

/M[F]x - magnification - default /M1

The M option specifies  the magnification.  F fits the  plot to  the

print window size.  The default, /M1, gives the same size plot as an

HP 7475 plotter.

/MF.9  fits the plot to the print window and shrinks it 10%

/M1    prints at the same size as an HP 7475

/M.5   prints at half the size of an HP 7475

/O - Origin and Orientation Option -----------------------------------

/O[1|2|3|4[L]] - origin, orientation - default /O1

The O  option sets the printer's origin and orientation.  The origin

is one  of  four  corners numbered 1..4 for upper  left, lower left,

lower right, and upper  right.  The  L  suboption  specifies  a left

handed coordinate system (this should never be needed).

Here are pictures of PrintCAD's view of the output media compared to

an HP 7475's view, to help with choosing a /O option.

HP 7475                                    PrintCAD

A size             B size

y��������Ŀ      0,0��������Ŀy          ul������Ŀur

�        �         �        �            1�      �4

�        �         �        �             �      �

0,0����������x        �        �             �      �

�        �             �      �

�        �           ll��������lr

�        �            2        3

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

/P - Intercept Mask Option -------------------------------------------

/PADI|mask - intercept mask

/PADI sets up PrintCAD for AutoCAD.  /Pmask  lets you  choose a  DOS

file mask as PrintCAD's intercept mask.  The mask should not include

disk  or directory information  - PrintCAD ignores this  information

when looking at file names.

/Padi    sets up PrintCAD to take data from AutoCAD interrupt 78.

/P*.plt  sets up PrintCAD to intercept data written to *.plt

p-13

/R - Internal Resolution Option --------------------------------------

/Ri - internal resolution - default /R1016

/R sets the internal grid resolution  in dots/inch.  It is  normally

1016 and rarely needs  to be changed.  To fit very large plots  into

the   -32768..32767  coordinate  limit,  use   /R508.   Setting  the

resolution to a multiple  of the output device  resolution times the

magnification may yield a small improvement in print quality.

/S - Pen Shading Option ----------------------------------------------

/Sc.. - pen shading patterns - default /S0

/S  assigns a shading pattern to each pen.  Unspecified pens use the

last specified shade.  The shade values are 0..8.  0  is the default

solid line.  1..4 are 1 dot patterns (75%, 50%, 25%, 12.5%) and need

line widths of at least three to give good looking  lines.  5..8 are

2x2 dot patterns  (75%, 50%, 25%,  12.5%) and need line widths of at

least  five to give good looking  lines.  Some printers may wash out

shading because they have  a dot size  that is large relative to the

dot spacing.  This is often the  case  with  dot  matrix printers in

multipass modes.

1 ��������         2 ��������          3 � � � �          4 �   �

��������           ��������            � � � �              �   �

��������           ��������            � � � �            �   �

��������           ��������            � � � �              �   �

5 ��������         6 ��  ��            7 ��  ��           8 ��

��  ��               ��  ��

��������           ��  ��              ��  ��                 ��

��  ��               ��  ��

/S023  sets pen 1    to solid

pen 2    to 1 pel checkerboard

pen 3..8 to 1 pel dots

/T - Temporary File Option -------------------------------------------

/T[+]f - temporary file - default /TPRINTGL.TMP

PrintCAD  uses  a temporary file when it does not have enough memory

to handle  a plotfile.  You can use /T to put it on a RAM  disk  for

extra speed.  You can force the use of a temporary file, if the plot

has over  1024 data points, with +.  If you use a RAM  disk this may

speed printing.

p-14

/W - Pen Width Option ------------------------------------------------

/Wc.. - pen widths - default /WA

The W option assigns pen line widths. Each width may be 1..9, .10 ..

.16 pels or A..I for 2..10 units of 4/1016 inch (.1 mm).  A..I gives

a  device  independent width, but it is an approximation, limited by

the device technology. Unassigned pens use the last assigned width.

For  nonsquare  matrix  printers,  PrintCAD  uses  "pens"  that  are

optimized for an x/y ratio of .67, 1, or 1.5. The widths are figured

in the direction of  higher resolution, and are  not accurate in the

other direction.

Two  notes  apply  to  lines  of over  eight  pels  width.  They are

distorted  at  the  print window  edge to  fit within the  four  pel

border. And they require additional processing time.

/WA      sets pen 1..8 to 0.2 mm

/WB2.16  sets pen 1    to 0.3 mm

pen 2    to 2 pels

pen 3..8 to 16 pels

/X - Control Option --------------------------------------------------

/XQ - queries PrintCAD installation

/XR - removes PrintCAD if allowed

/X[X][L][A] - controls PrintCAD's setup

/XQ  checks to see  if  PrintCAD is installed.  /XR removes PrintCAD

from memory, if allowed. /XQ and /XR disallow all other options.

The  other suboptions control PrintCAD's setup,  and  are  effective

only during installation.  /XX forces the  use  of XMS for  swapping

(instead of EMS).  /XL forces PrintCAD to load low (instead of using

an XMS UMB).  /XA sets an alternate handle. These may be used in any

combination. Each one may or may not improve compatibility.

/XL  sets up PrintCAD in low memory

/Z - Chord Angle Option ----------------------------------------------

/Z[E]i,j - default, min chord angle, enhanced font - default /Z10,5

The  Z  option  specifies the default  and  minimum chord angles for

circles and arcs. The HP 7475 defaults to a chord angle of 5 with no

minimum. But using chord angles of less than 10 returns little print

quality  improvement and takes a lot  of  processing  time.  So  the

default Z option is /Z10,5. For better emulation use /Z5,2.

The  suboption E  says to use the enhanced font,  which  doubles the

number of chords on curves in text, improving the  curve smoothness.

This is only effective on character heights above .5 cm. And for CAD

packages  (including  AutoCAD)  that  use  vectors  to  create  text

(instead of the HP-GL label command), this option is ineffective.

/Z5,2  sets the default chord angle to 5 degrees, the minimum to 2

p-15

Large Paper ----------------------------------------------------------

PrintCAD can handle printing on  paper larger than the usual 8.5x11,

but  this  takes  some  extra  work.  First  you  need  to  increase

PrintCAD's print window to cover the large paper.  This is done with

the /L option.  The print window plus the  margins  (/L) must fit on

the paper.

And then  you must make sure that your printer recognizes the larger

paper  height.  For  continuous forms  paper this  is  best  done by

setting  skip perforation to  off, either by  printer switch or with

PrintCAD prefix codes.  For Canon, Epson, and NEC line printers (dot

matrix, ink jet, thermal) use /J27 79.

For cut sheet paper, on Canon, Epson, and NEC line printers use /J27

67 0 n, where n is the page length in inches.

For page printers and HP-PCL line  printers, legal size paper may be

specified with  these printer code prefixes: HP-PCL printers /J27 38

108  51 65 (/K27  38  108  50 65 to  reset  to 11  inch  paper), IBM

LaserPrinter  /J27  67 0  14  (/K27  67 0 11  to  reset), Canon  LBP

printers /J27 91 51 50 59 59 112 (/K27 91 51 48 59 59 112 to reset).

To  get  a 10x16 print window on 11x17 paper with a Canon, Epson, or

NEC line printer, use these options: /L10 16 .4 .25 /J27  67  0  17.

You  will  probably  need  to adjust  the  margins (.4 .25) for best

centering.

p-16

Canon BJ Printers ----------------------------------------------------

/F7 supports  the BJ130E, BJ10E, BJ300, and BJ330 in Proprinter mode

at 360 dpi. The BJ130 is not supported.

These  printers have an  image  density option  (switch  1-9 on  the

BJ130, 1-4 on  the BJ300).  Low density eliminates every second pel.

To get the full 360 dpi resolution, use high density.

Canon LBP Printers ---------------------------------------------------

/FD  works with any Canon LBP printer in ISO mode.  It takes up to 1

meg of memory to print a full page of graphics.

You may need  to set the printer memory mode with /J 27  59 27 91 50

38 122.

If the printer is in Diablo mode, PrintCAD switches to ISO mode.  To

switch back to Diablo mode on completion, use /K 27 58.

Epson 9 Pin Printers -------------------------------------------------

/FN covers a wide range  of  9  pin dot matrix printers.  You need a

cyan/magenta/yellow ribbon  to  get  color  prints.  /FN+ uses 1/216

indexing and /FN* uses 1/216 indexing and 1/240 graphics.

Epson, NEC, Toshiba 24 Pin Printers ----------------------------------

/FT  covers all  Epson  LQ  printers  and many  compatible  printers

(Panasonic, etc.).  /F9  works with NEC  24 pin printers.  /FO works

with Toshiba 24 pin printers.  /FO resets the  line feed distance to

1/6 inch on completion.

For  all of these drivers, you need a cyan/magenta/yellow ribbon  to

get color prints.  The + and * drivers use 1/360  graphics mode, and

the * drivers use 1/360 (1/120 for /FO*) indexing and reset the line

feed  distance to  1/6 inch on  completion.  A clean paper path with

equal tension on each side is needed for good 360x360 graphics.

HP LaserJet, DeskJet -------------------------------------------------

/FL works with any LaserJet in portrait mode.  Portrait mode  may be

set with  /J 27 38 108 48 79 or you can reset the printer with /J 27

69.

Full  page plots take up to 1 meg of memory  (white area reduces the

required memory).

Use /FI for DeskJets.  For LaserJet  2P and 3* with at least 1.5 meg

of memory, use /FI.

HP PaintJet ----------------------------------------------------------

/FP drives PaintJet printers. PaintJet printers are sold with either

a serial or parallel interface. The parallel interface is faster.

p-17

IBM LaserPrinter -----------------------------------------------------

/F4 works with the  LaserPrinter in PPDS (native) mode.  It takes up

to 1 meg of memory to print a full page of graphics.

Kodak Diconix Color 4 ------------------------------------------------

/FK drives the Color 4.

PostScript Printers --------------------------------------------------

/FS outputs PostScript.  All  characters, arcs, fill, and clips  are

converted to simple vectors.

PrintCAD's pen width parameters are assumed to be in units of 4/1016

inch.  Color and shading  parameters are mapped into gray  levels or

RGB color levels.

p-18

Compatibility and Technical Information ------------------------------

PrintCAD will run on any IBM compatible MS-DOS computer with 368K of

EMS (3.0 or above) or 368K of XMS and a base  memory region of 272K.

Because  it is a  swapping resident program, it may have problems in

some environments.  PrintCAD can handle DOS extenders that use VCPI,

but DPMI based programs (MS Windows) may cause problems.

PrintCAD is  a TSR that takes over interrupt hex  21 and watches for

DOS 2.0+ file management open functions.  If the filename on an open

command  matches either "pcad"  or the  mask specified with  the  /X

option,  then file handle dec  99  (or dec  15 if  /XA  is  used) is

returned and PrintCAD is awakened. It then responds to write, device

information, close functions, seek, and set date functions.

When awake, PrintCAD stores  HP-GL commands (from the write command)

until  its buffer  limit (16K)  is  reached and then it swaps in its

272K memory space, processes the data, and swaps the original memory

back.

PrintCAD was  written  by  Cary  Ravitz and compiled  with Borland's

Turbo Pascal 6.0.

p-19

User Support ---------------------------------------------------------

The  quickest  way to  resolve  problems with PrintCAD is to use the

support bulletin board.  The Ravitz Software support  bulletin board

is at:

606-268-0577  1200/2400,N,8,1  24 hours/day.

To ask a question (or make a comment or suggestion), first prepare a

file with the details of your question. Please be very specific, and

include  any related information  on  your  hardware,  software, and

operating environment.  Then call the bulletin board  and choose the

upload question  option.  XMODEM file  transfer protocol is required

for this.  Your question will be assigned  a number (such as 1015) -

remember this number. An answer file (with the assigned number) will

be posted, usually within 24 hours.

To  get an  answer,  call the  bulletin  board, choose the  download

option, then the answer option, and then enter  the  answer  number.

XMODEM is also required for downloading answers.

All question and answer files are public.

You may send questions to Compuserve - 70431,32  Cary Ravitz.

You may send questions by mail to:

Ravitz Software Inc.

P.O. Box 25068

Lexington, KY  40524-5068

USA

Please send  a diskette  (360K, 1.2M, 720K,  or 1.44M, 720K is best)

with your question  so that example  files or newer  versions of the

software may  be  included  with the  response.  Return  postage  is

appreciated.