SIC v5.6a additional notes:

Adapters and displays:

Using the SCREEN <expression> statement:

Screen mode summary:

The following tables briefly summarize each of the screen modes. The color

adapters referred to are:

Monochrome Display and Printer Adapter (MDPA)

Color Graphics Adapter (CGA)

Enhanced Graphics Adapter (EGA)

Video Graphics Array (VGA)

Multicolor Graphics Array (MCGA)

The Hercules Graphics Card, Graphics Card Plus, Olivette Graphics Card, and

InColor adapters for monochrome monitors are not supported.

Many screen modes support more than one combination of rows and columns on the

screen. See the WIDTH Statement for more information about changing the number

of rows and columns on the display.

The graphics screen modes reset the window to full screen. Returning the screen

mode to SCREEN 0 restores the window to text mode.

MDPA screen modes:

The Monochrome Display and Printer Adapter (MDPA) must be used with a

monochrome display. Only SCREEN 0, text mode, can be used with the MDPA.

Rows and   Color        Display

Mode   columns    attributes   colors   Resolution

----   --------   --------     -------  ----------

0      80x25      16           3        720x350

CGA screen modes:

The Color Graphics Adapter (CGA) and Color Display are typically paired. This

combination permits running text-mode programs, and both medium-resolution and

high-resolution graphics programs.

Rows and   Color

Mode  columns    attributes   Resolution

----  --------   --------     ----------

0     40x25      16           320x200

0     80x25      16           640x200

1     40x25      4            320x200

2     80x25      2            640x200

EGA screen modes:

The Enhanced Graphics Adapter (EGA) may be used with either the Color Display

or the Enhanced Color Display. In modes 0, 1, 2, 7, and 8, these pairings

produce similar results.

Mode 9 takes full advantage of the capabilities of the Enhanced Color

Display. Mode 9 provides for the highest resolution possible for the

EGA/Enhanced Color Display configuration. Programs written for this

mode will not work for any other hardware configuration except the VGA.

Following are the screen modes that can be used with an EGA:

Rows and Color       Display

Mode  columns  attributes  colors  Resolution

----  -------- ----------  ------- ----------

0     40x25    16          16      320x200

40x25    16          64      320x350

40x43    16          64      320x350

80x25    16          16      640x200

80x25    16          64      640x350

80x25    16          16      640x200

80x25    16           3      720x350

80x43    16          64      640x350

80x43    16           3      720x350

1     40x25     4          16      320x200

2     80x25     2          16      640x200

7     40x25    16          16      320x200

8     80x25    16          16      640x200

9     80x25     4          64      640x350

80x43     4          64      640x350

80x25    16          64      640x350

80x43    16          64      640x350

10     80x25     4           9      640x350

80x43     4           9      640x350

EGA and VGA Adapters with Monochrome Display:

Only the EGA and VGA can be paired with the Monochrome display to run

programs in screen mode 10. This mode can be used to display monochrome

graphics at a very high resolution with the optional effects of blinking

and high intensity.

The following two tables summarize the color attributes, display colors,

and effects for screen mode 10 used with a monochrome display.

Color attributes:

SCREEN 10, Monochrome display:

Color attribute    Default effect

---------------    --------------------

0                  Off

1                  On, normal intensity

2                  Blink

3                  On, high intensity

Display color values:

SCREEN 10, Monochrome display:

Display color      Effect

---------------    --------------------

0                  Off

1                  Blink, off to on

2                  Blink, off to high intensity

3                  Blink, on to off

4                  On

5                  Blink, on to high intensity

6                  Blink, high intensity to off

7                  Blink, high intensity to on

8                  High intensity

VGA screen modes:

The Video Graphics Array (VGA) adapter offers significantly enhanced

text and graphics in all modes.

Rows and  Color

Mode  columns   attributes  Colors  Resolution

----  --------  ----------  ------  ----------

0     40x25     16          64      360x400

40x43     16          64      320x350

40x50     16          64      320x400

80x25     16          64      720x400

80x43     16          64      640x350

80x43     16           3      720x350

80x50     16          64      640x400

80x50     16           3      720x400

1     40x25      4          16      320x200

2     80x25      2          16      640x200

7     40x25     16          16      320x200

8     80x25     16          16      640x200

9     80x25     16          64      640x350

80x43     16          64      640x350

10     80x25      4           9      640x350

80x43      4           9      640x350

11     80x30      2         256      640x480

80x60      2         256      640x480

12     80x30     16         256      640x480

80x60     16         256      640x480

13     40x25    256         256      320x200

MCGA screen modes:

The Multicolor Graphics Array (MCGA) combines the modes of the CGA with

the very high resolution and 256K color modes of the VGA to provide enhanced

text and graphics in all modes.

Rows and  Color       Display

Mode  columns   attributes  colors   Resolution

----  --------  ----------  -------  ----------

0    40x25      16                  320x400

80x25      16                  640x400

1    40x25       4                  320x200

2    80x25       2                  640x200

11    80x30       2          256K    640x480

80x60       2          256K    640x480

13    40x25     256          256K    320x200

The MCGA uses the same display color values as the VGA.

-end-