The SuperVGA Kit

----------------

Version 5.1

5 February 1995

Copyright (C) 1992-95 SciTech Software.

All Rights Reserved.

The SuperVGA Kit is a full Software Development Kit for working with SuperVGA

video cards that have a VESA VBE compliant Video BIOS. This SDK also includes

full support for the new VBE 2.0 32 bit protected mode routines and 32 bit

linear framebuffer modes. It is also intended to function as a Software

Development Kit for developers wishing to take advantage of the VBE 2.0

features of UniVBE and eventually the new VBE/AF Accelerator Functions

that will be provided by UniVBE.

*NEW* WinDirect Technology!

---------------------------

The SuperVGA Kit also includes SciTech Software's WinDirect technology for

writing full screen Windows applications using VBE services. WinDirect

allows Windows 3.1 applications to shut down the GDI and obtain direct

access to the hardware. Then you can start any VBE video mode that you

desire for maximum performance (like UniVBE's new 320x200/240/400 8 bit

modes or perhaps 320x240 15/16 bit modes) with full hardware double buffering

and linear framebuffer support. Not only that, but when VBE/AF is complete

you will be able to write full screen VBE apps that include full hardware

acceleration!

While an application is running WinDirect fullscreen, all normal Window's

services are still active, such as Audio, Networking, CD-ROM, MultiMedia,

Memory Management and Resource Management. In fact GDI is still available

and you can even do your rendering using GDI into a WinG offscreen buffer

and then simply copy that to the VBE video display!

WinDirect is also fully compatible with Microsoft's forthcoming Windows '95

operating system. WinDirect uses the standard Video for Windows function

calls to shut down GDI and restore it back to life, so it does not rely

on any special display drivers VxD's to be installed. In fact WinDirect

does not even need Video for Windows to be installed - all that is required

is that DISPDIB.DLL be shipped with the resulting application.

*NEW* Hybrid 16/32 bit technology!

----------------------------------

The SuperVGA Kit also includes SciTech Software's new Hybrid 16/32 bit

technology. This allows normal 16 bit Windows apps to directly call any

32 bit assembler just like any normal 16 bit function. However because the

code is marked as USE32, you can use the full 32 bit string instructions

to directly access HUGE memory buffers in a linear fashion. You can use

this for writing *really* fast WinG code, or you can use the examples

in the SuperVGA Kit and write full screen linear framebuffer code that

runs directly as a 16 bit Windows App! Calling a hybrid 32 bit function

from a 16 bit Windows app incurs no extra overhead compared to calling

any other 16 bit far function.

The VESAVBE.C module contains all the code necessary to set up and call

the VBE 2.0 32 bit protected mode bank switching, palette setting and

double buffering routines directly from your 16 bit code for absolute

maximum performance.

Using the new Hybrid 16/32 bit technology will allow developers to start

builing 32 bit enabled applications while continuing to use current 16

bit Windows development tools. If you wish to eventually move to full

Win32, you can do so simply by re-compiling and re-assembling your code.

Hence you can start to include full 32 bit performance under Windows 3.1

without the overhead of Win32s. The only drawback is that currently

all hybrid 32 bit code must be written in assembler (it gets called

with a 16 bit stack frame not a 32 bit stack frame).

Using it in your own programs

-----------------------------

The SuperVGA test programs all call upon a single C library to access the

SuperVGA using the VESA VBE programming interface. You can use this

same library to provide SuperVGA support in your own applications, or

you can take the source code for the library as a starting point and

expanding on it from there. For tinkering with VBE 2.0 applications using

SVGA.LIB directly is a great starting point.

If you wish to develop your own graphics library or use VBE 2.0 in your own

code, we *highly* recommend you link in and use the VESAVBE.C module as your

main interface to VBE 2.0. None of the code in VESAVBE.C is speed critical,

and this modules takes care of fixing minor bugs in slightly varying VBE 1.2

implementations to make the high level interface consistent. This module

also takes care of isolating protected mode applications from the need

to meddle with pointers to real mode memory locations by mapping all such

data into the default data segment automatically.

Assembling the source files

---------------------------

All of the assembler source files have to be assembled with Borland's TASM,

since they are written using TASM's IDEAL mode. If you do not have TASM,

you can simply link with the pre-assembled object files provided.

Test programs

-------------

The library now has only one single test program, that presents a set of

menus for testing all of the available video modes that the library

supports (and your hardware supports).

Note that the test program requires a VESA VBE compliant SuperVGA in

order to run, so if your video card does not have the VESA VBE in the BIOS,

then you will need to install the Universal VESA VBE before running it.

PM/Lite library

---------------

The SuperVGA Kit uses the PM/Lite library, which should also have come

as pre-compiled library files in the same archive. If you wish to compile

it up for a different extender, you will need to obtain the source code

archive (see the file INSTALL.ME for details).

Global Variables Reference

--------------------------

int     VBEVersion;         VBE Version number (in BCD)

int     maxx,maxy;          Maximum device coordinate values (res-1)

long    maxcolor,defcolor;  Maximum and default color values

int     maxpage;            Maximum video page number (numpages-1)

int     bytesperline;       Bytes in a logical scanline

int     bitsperpixel;       Current pixel depth

int     bytesperpixel;      Bytes in a pixel (if > 1)

int     memory;             Video memory available in Kb

short   modeList[];         List of available video modes (USE THIS!)

char    OEMString[];        OEM string from VBE

int     capabilities;       Capabilities bits from VBE

int     curBank;            Current read/write bank

The following globals define the pixel format information, and is used by the

rgbColor() routine to build the correct pixel for the current display mode.

You can however use it yourself to build scanline information in the desired

format if you need to optimise these routines for speed.

uchar   redMask,greenMask,blueMask;  Color channel mask values

int     redPos,greenPos,bluePos;     Starting position of color channel bits

int     redAdjust,greenAdjust;       Shift factors need to adjust to correct

int     blueAdjust;                     postition in pixel image

SVGA.LIB Function Reference

---------------------------

Include file: "svga.h"

int SV_init(void);

Detects if a VESA VBE compliant SuperVGA is out there, and

initialises the library if one is. Returns the packed BCD version

number for the VBE interface detected, or 0 if none is present. For

VBE 1.2 this is 0x102, for VBE 2.0 this is 0x200.

bool SV_setMode(int mode);

Set the specified video mode, given the internal VBE mode number.

DONT pass old style VBE mode numbers to this routine (ie: 0x101

for 640x480x256), although these *may* still work, the VBE 2.0

method is to search through the list of available video modes

for the one that has the desired resolution and color depth. This

will allow you code to work with all custom video resolutions

provided by different OEM VBE drivers (like UniVBE/Pro). Have a

look at the code in the HELLOVBE.C file that demonstrates how

to start any video mode given a user specified resolution.

To set a VBE 2.0 linear framebuffer mode, or the flag vbeLinearBuffer

with the video mode.

*DONT* attempt to set a linear framebuffer mode unless it is available.

You can determine this by checking that the vbeMdLinear flag is set

in the modeAtttibutes field of the VBE_modeInfo block for this video

mode.

This routine also sets up all the drawing function vectors to

point to the correct routines for the specified video mode.

Returns FALSE is the mode could not be set correctly.

void SV_restoreMode(void);

Restores the previous video mode active befor the setSuperVGAMode

routine was called. Also correctly restores the VGA 50 line mode

if it was previously active.

int SV_getModeName(char *buf,VBE_modeInfo *mi,ushort mode,bool useLinear);

Builds a ASCII string representing the video mode given a valid

VBE_modeInfo block. If the useLinear flags is TRUE and a linear

buffer mode is available, this function returns the linear

framebuffer version of the video mode number.

bool SV_set8BitDAC(void);

This function enables the 8 bit wide palette if available and

returns TRUE. If an 8 bit wide palette is not available, this

routine returns FALSE and the palette width is unchanged.

bool SV_set6BitDAC(void);

This function restores the default VGA style 6 bit palette.

ulong SV_rgbColor(uchar r,uchar g,uchar b);

Packs a set of RGB values into a color value for passing to the

primitive drawing routines that is appropriate for the current

video mode. This routine is intended to work with RGB video modes

such as the 15, 16, 24 and 32 bits per pixel modes (in 8 bit modes

it will packed it into a simple 2:3:2 style pixel, so you can set

up your own pseudo RGB palette if you so desire).

You should use this routine to convert all color values to ensure

that they work correctly on the different types of direct color

video modes available.

bool SV_setBytesPerLine(int bytes);

Sets the scanline length to a specified bytes per line value.

If this succeeds, this function updates the internal variables in

the library for drawing with the new logical scanline width. If

this function fails it returns false.

This function is available only under VBE 2.0 and above.

bool SV_setPixelsPerLine(int xMax)

Sets the scanline length to a specified number of pixels. If this

succeeds, this function updates the internal variables in the

library for drawing with the new logical scanline width. If this

function fails it returns false.

This function is available only under VBE 1.2 and above.

void SV_setPalette(int start,int num,VBE_palette *pal,int maxProg);

This function set the specified palette entries, by either

directly programming the VGA hardware (for VBE 1.2 and below) or

by calling the VBE 2.0 palette interface routines. This routine

avoids 'snow' effects on older systems by only programming

'maxProg' values per vertical retrace interval. If you set maxProg

to 256, all values will be programmed at once and the palette

set will be synced to a vertical retrace. If you set maxProg to

-1, all values will be set at once and the routine will *not*

wait for a vertical retrace before setting the values. For systems

that cause snow, a good value of maxProg is about 100-120.

This routine is *fast* and will provide the fastest method of

programming the palette that will work in all systems. Because

of the way that palette values are programmed, color values will

not be dropped on systems that have slower IO response, so you

should *always* use this routine rather than programming the

palette yourself. If the controller is and you program the

palette yourself (as opposed to having the VBE 2.0 do it) your

code will most likely product no result and may well hang the

machine waiting for a VGA retrace that never occurs.

The buffer is expected to be in the correct format, which will

be an array of 'palette' structures. If the palette is currently

in the default 6 bit VGA mode, this routine expects the values to

be 6 bits wide. If it is in an 8 bit palette mode, then the values

should be 8 bits wide (and hence you will need to do the

conversion yourself).

Note that the VBE_palette structure is the same as the Windows

RGBQUAD palette structure, which is *not* the same as the normal

BIOS structure.

void SV_setActivePage(int page);

Sets the currently active video page for output. Used to implement

double buffering for smooth animation.

void SV_setVisualPage(int page);

Sets the currently visible display page. Used to implement

double buffering for smooth animation.

void SV_setBank(int bank);

'C' callable bank switch routine to set the current read/write bank

to the specified value. Assembler functions dont call this routine

bit call the register level version in _svgasdk.asm.

void SV_putPixel(int x,int y,ulong color);

Plots a pixel at the specified (x,y) location in the specified

color. The color value MUST be in the correct format for the

current video mode (use rgbColor() to pack HiColor and TrueColor

RGB values). The pixel is plotted on the currently active

display page (which may possibly be hidden from view).

void SV_clear(ulong color);

Clears the currently active display page to the specified color.

The color value MUST be in the correct format for the current video

mode (use rgbColor() to pack HiColor and TrueColor RGB values).

void SV_line(int x1,int y1,int x2,int y2,ulong color);

Draws a line from the point (x1,y1) to (x2,y2) in the specified

color. The color value MUST be in the correct format for the

current video mode (use rgbColor() to pack HiColor and TrueColor

RGB values). The line is drawn on the currently active

display page (which may possibly be hidden from view).

void SV_writeText(int x,int y,char *str,ulong color);

Writes the text sting at the location x,y in the standard VGA

8x16 VGA font, and the specified color. The background between

the text is not erased.

VESAVBE.C module reference

--------------------------

int VBE_detect(VBE_vgaInfo *vgaInfo);

Detects if a VESA VBE is out there and functioning correctly. If we

detect a VBE interface we return the VBE_vgaInfo block returned by

the VBE and the VBE version number. This functon also internally

translates all strings and the videoModeTable from real mode

memory into the local data segment so you can directly access it

from normal C code.

bool VBE_getModeInfo(int mode,VBE_modeInfo *modeInfo);

Returns the video mode information for the specified VBE internal

video mode number. The mode number must be valid, or this routine

will return FALSE. Check the modeList[] array which contains the

list of available video modes that you can use.

You should carefully examine this mode information block to find

out whether the video mode supports a linear framebuffer mode and

if so where the buffer is located in physical memory. Note that

not *all* video modes are compatible with the linear framebuffer

access, so you should *always* check this block to see if the

particular mode you are using supports a linear framebuffer.

For double buffering you *must* use the NumberOfImagePages value

to determine how many hardware buffers you have available. For

VBE 2.0 these values will always be correct, but for VBE 1.2

VESAVBE fudges it which may be incorrect in some cases.

long VBE_getPageSize(VBE_modeInfo *modeInfo);

Computes the page size in bytes for the specified mode information

block, rounded up to the appropriate boundary (8k, 16k, 32k or 64k).

Pages >= 64k in size are always rounded to the nearest 64k boundary

(so the start of a page is always bank aligned). This page size

values should be used for computing the starting locations of all

the hardware video pages for performing double or multiple buffering

using the VBE.

bool VBE_setVideoMode(int mode);

Set the specified video mode, given the internal VBE mode number.

DONT pass old style VBE mode numbers to this routine (ie: 0x101

for 640x480x256), although these *may* still work, the VBE 2.0

method is to search through the list of available video modes

for the one that has the desired resolution and color depth. This

will allow you code to work with all custom video resolutions

provided by different OEM VBE drivers (like UniVBE/Pro). Have a

look at the code in the HELLOVBE.C file that demonstrates how

to start any video mode given a user specified resolution.

To set a VBE 2.0 linear framebuffer mode, or the flag vbeLinearBuffer

with the video mode.

*DONT* attempt to set a linear framebuffer mode unless it is available.

You can determine this by checking that the vbeMdLinear flag is set

in the modeAtttibutes field of the VBE_modeInfo block for this video

mode.

Returns FALSE is the mode could not be set correctly.

int VBE_getVideoMode(void);

Returns the currently active VBE or VGA video mode number.

int VBE_getStateSize(int flags);

Returns the size of the SuperVGA State block required to save

the current state of the video hardware. Similar to the standard

VGA routine, but extended to include the SuperVGA hardware state.

bool VBE_saveState(int flags,void *buffer);

Saves the SuperVGA state into the pre-allocated buffer.

bool VBE_restoreState(int flags,void *buffer);

Restores the SuperVGA state from the buffer.

bool VBE_setBank(int window,int bank);

Call the VBE to set the current bank value for the specified window.

This routine performs this using an Int 10h, so you should *not* use

it from high performance code!!

int VBE_getBank(int window);

Calls the VBE to return the current bank for the specified window.

bool VBE_setPixelsPerLine(int pixelsPerLine,int *newBytes,int *newPixels,

int *maxScanlines);

Attempts to set the logical scanline length to a specified number

of pixels. If this routine succeeds, the new logical scanline

information is returned in 'newBytes', 'newPixels' and 'maxScanlines'.

Note that the actual value programmed may *not* be exactly what

was requested, by may be slighly larger due to hardware limitations.

*NOTE* VBE 1.2+ only!

bool VBE_setBytesPerLine(int bytesPerLine,int *newBytes,int *newPixels,

int *maxScanlines);

Attempts to set the logical scanline length to a specified bytes

per scanline value. If this routine succeeds, the new logical scanline

information is returned in 'newBytes', 'newPixels' and 'maxScanlines'.

Note that the actual value programmed may *not* be exactly what

was requested, by may be slightly larger due to hardware limitations.

*NOTE* VBE 2.0+ only!

bool VBE_getScanlineLength(int *bytesPerLine,int *pixelsPerLine,

int *maxScanlines);

Obtains the current scaline length in bytes per line and pixels

per line. This routine also returns the maximum number of scanlines

for the current video mode, and defines the maximum accessible

memory location in video memory.

*NOTE* In some video modes, even though the controller have may

2Mb or more video memory, only 1Mb or 512Kb is accessible

to the application. In this case this function *must* be

used to determine just how far into the framebuffer the

virtual screen extends.

For double buffering you *must* use the NumberOfImagePages

value to determine how many buffers you have available.

*NOTE* VBE 1.2+ only!

bool VBE_getMaxScanlineLength(int *maxBytes,int *maxPixels);

Obtains the maximum possible bytes per scanline and pixels per

scanline supported by the hardware in a particular video mode.

You should call this routine to determine just how wide you can

make the virtual buffer for virtual scrolling applications.

*NOTE* VBE 2.0+ only!

bool VBE_setDisplayStart(int x,int y,bool waitVRT);

Sets the CRTC display starting address to the specified value. You

can use this routine to implement hardware scrolling. You can

also use this function to perform double buffering - see the

SV_setVisualPage() routine in _SVGASDK.ASM for sample code.

If the waitVRT flag is false, the routine will not wait for

a vertical retrace before programming the CRTC starting address,

otherwise the routine will sync to a vertical retrace. Under

VBE 1.2 it is not guaranteed what the behaviour will be (some

wait and some dont).

Returns FALSE if the value could not be set.

*NOTE* This routine always used Int 10h, so for maximum performance

under protected mode you should use the VBE 2.0 32 bit

version.

bool VBE_getDisplayStart(int *x,int *y);

Returns the current display starting address in pixels.

bool VBE_setDACWidth(int width);

Attempts to set the DAC width to the specified number of bits.

Generally only 6 or 8 bits are acceptable values. If this

function fails, it returns false.

int VBE_getDACWidth(void);

Returns the current DAC width.

bool VBE_setPalette(int start,int num,VBE_palette *pal,bool waitVRT);

Sets the specified block of palette entires as fast a possible.

If the waitVRT flag is true, the routine will sync to a vertical

retrace before programming the values. However this routine

will *always* program all palette entries one after another, so

if you need to check for snow you will need to interleave calls

to this code with smaller blocks of registers to program per

retrace. See the SV_setPalette() for examples of this.

The buffer is expected to be in the correct format, which will

be an array of 'palette' structures. If the palette is currently

in the default 6 bit VGA mode, this routine expects the values to

be 6 bits wide. If it is in an 8 bit palette mode, then the values

should be 8 bits wide (and hence you will need to do the

conversion yourself).

Note that the VBE_palette structure is the same as the Windows

RGBQUAD palette structure, which is *not* the same as the normal

BIOS structure.

*NOTE* This routine always uses Int 10h, so for maximum performance

under protected mode you should use the VBE 2.0 32 bit

routine. In fact for protected mode code this routine

will be *slow* as it needs to copy the buffer into a real

mode address for every function call.

ushort VBE_getLinearSelector(VBE_modeInfo *modeInfo);

Utility function to create a selector to the linear framebuffer

for the specified video mode. The selector is created only

once (the linear framebuffer location *never* moves after bootup)

and is cached for all other functions calls. The selector

is always created with a limit of 8Mb, regardless of how much

physical memory is really present.

void *VBE_getSetBank(void);

Returns a pointer to the relocated 32 bit protected mode bank

switching routine.

Under 32 bit protected mode this routine is a 32 bit near function.

Under 16 bit Windows this routine returns a 16:16 far pointer with

the selector set to a USE32 code segment so that this function

can be called directly from 16 bit code. Internally the VESAVBE

module creates a small thunk to call the 32 bit near function

and return to the 16 bit far calling function.

This function can only be called from assembler and takes the

following arguments in registers:

BX := window number

DX := bank number (in granularity units)

void *VBE_getSetDisplayStart(void);

Returns a pointer to the relocated 32 bit protected mode CRTC

start address routine.

Under 32 bit protected mode this routine is a 32 bit near function.

Under 16 bit Windows this routine returns a 16:16 far pointer with

the selector set to a USE32 code segment so that this function

can be called directly from 16 bit code. Internally the VESAVBE

module creates a small thunk to call the 32 bit near function

and return to the 16 bit far calling function.

This function can only be called from assembler. The arguments

to this routine are completely different to VBE function 4F07h,

and takes the value to be programmed into the CRTC start address

registers as a linear starting address, not as an (X,Y) pixel

coordinate. In 4 bit video modes this value is equal to the byte

address in video memory for the starting address. In 8+ bit modes

this value is equal to the linear byte address in video memory

divided by 4 (historically this is related to the fact that the

CRTC register are programmed on a plane boundary in chain-4 video

modes). The actual value programmed may be rounded down by the

hardware if required (lots of systems align to an 8 byte or even

16 byte boundary).

This function takes the following arguments in registers:

DX := High byte of Linear CRTC starting address (see above)

CX := Low byte of Linear CRTC starting address

BX := 00h - Set display start

80h - Set display start and wait for retrace

*NOTE* There is no provision for setting the starting address to

an *exact* X coordinate pixel offset. Lots of hardware cannot

handle this, and the PEL panning register is not available

in many video chips.

void *VBE_getSetPalette(void);

Returns a pointer to the relocated 32 bit protected mode palette

programming routine.

Under 32 bit protected mode this routine is a 32 bit near function.

Under 16 bit Windows this routine returns a 16:16 far pointer with

the selector set to a USE32 code segment so that this function

can be called directly from 16 bit code. Internally the VESAVBE

module creates a small thunk to call the 32 bit near function

and return to the 16 bit far calling function.

This function can only be called from assembler and takes the

following arguments in registers:

BL  := 00h - Set palette data

80h - Set palette data and wait for retrace

CX  := Number of palette entries to program

DX  := Starting palette entry

ES:EDI := Pointer to block of palette date in correct format

void VBE_freePMCode(void);

Function to free the 32 bit PM code block used to relocate the

32 bit code into the applications own code space. This function

*must* be called after the video mode is returned to text mode

to ensure that the appropriate routines will be relocated the

next time a video mode is started.

<*** END OF DOCUMENT ***>