SPINDOWN Version 1.2 - This is the fourth public release of SPINDOWN.

http://www.ncf.carleton.ca/~aa571

SPINDOWN - What is it?

------------------

SPINDOWN is a DOS command line utility to set the spindown or timeout

values for ATAPI and SCSI CDROM drives.  It now supports SCSI drives

through the ASPI drivers, ATAPI drives through the ASPI drivers in

Windows 95, ATAPI drivers through the ATASPI drivers, and most other

ATAPI drives through the native mode interface.

I wrote this utility for two reasons.  The first was that it seemed

in the 'net community that there was no fix for the problem, and

second was because of my new Toshiba drive.  When the 5302 drive

first came out it had (and still does) a nasty habit of going into

a sleep mode after 32 seconds.  To many people this caused problems

when you had to wait for 5-10 seconds every time the drive had

to spin back up to speed.

Toshiba has released a new driver for their drives which allows you

to set the spindown in the command line.  It only seems to work for

some people, and also has trouble under Windows '95 on some machines.

It also only allows you to set the spindown time on boot up, so if

you want to change it, you have to reboot your machine.

This utility does not necessarily have to be run on a Toshiba.  It

should run on any ATAPI or SCSI CDROM drive.  The Toshiba drives seem to

have the shortest SPINDOWN time of any drives on the market.

First Warning!

PLEASE NOTE:  Changing the default values for the timeout may cause

problems.  The following is a quote directly from the ATAPI spec.

"NOTE: Higher values in this parameter may have an adverse effect on the

drive MTBF, in some implementations."

Second Warning!

Make sure you only specify the IO/IRQ/Device of CDROM drives.  SPINDOWN

has no way of knowing if you have specified a CDROM drive or a hard

disk.  The hard disk should not recognize the ATAPI commands I send

to it, so there shouldn't be a problem.  However, please be careful.

How do I use it?

-----------------

Spindown has a very simple command line interface.  It talks to ATAPI

drives under DOS through my device driver.  It talks to SCSI drives

under DOS and Windows through the ASPI driver.   It talks to ATAPI

drives under Windows 95 through the ASPI driver.

Valid timeout values are 1, 2, 4, 8, 16, 32, 60, 120, 240, 480, 960,

and 1920.  Any values entered other than these will be rounded down

to the next legal value.  These legal values are set down in the

ATAPI spec.  It seems that Toshiba may not necessarily use

the exact values in the spec, so the timeouts may vary a little.

When using ATAPI and DOS, you must give the program the IO address,

the IRQ and the device number of the drive you are trying to change.

As well you give it the new timeout value in seconds.

Valid IO addresses are 1f0, 17f, 1e0i 1e8, and 168.

Valid IRQs are 10, 11, 12, 13, 14, and 15

Valid Device numbers are 0 for Master and 1 for Slave.

NOTE: Running this program in a DOS window under Windows may give

unpredictable results.  SPINDOWN checks to see if it is running

under Windows and will warn you.

When using the ASPI mode for SCSI drives under DOS or Windows, or ASPI

mode for ATAPI drives under Windows 95, you must give the program

the Adapter #, the SCSI ID and the Logical Unit Number(LUN) of the

drive.  Valid SCSI IDs are 0 to 7. Valid LUNs are 0 to 7.  Normally

there is only one adapter in a machine, so the Adapter # is usually 0.

Included is a program called SCSIPING.  It will scan your machine and

list all the CDROMs found and their ADAPTER:SCSI_ID:LUN values.

When running under a DOS window under Win95, some machines may support

using ATAPI drives through the ASPI interface.  However, bugs in

Microsoft's drivers prevent this from working on some machines. I have

worked a long time to get this working on a lot of machines, but I

can't guarantee it will work on all machines.  It has taken a significant

amount of effort to get around many of the problems in the device drivers

written by Microsoft.  It appears to be worth the trouble so that you

can have much more control over the drives.

To use the ASPI interface to the ATAPI drives in a DOS window under

Windows 95, you have to use the /95ATAPI option.

As of version 1.2, you can also use SPINDOWN with ATAPI drives through

the ATASPI drivers.  These drivers are for DOS and Win95 and usually

come as part of "PowerIDE" from Future Domain when you buy their

IDE controller.  However, as far as I can tell, the drivers will

work on any IDE controller, not just the ones from Future Domain.

Using the ATASPI drivers is probably the best way to use SPINDOWN and

ATAPI drives.  It means that I have the most control over the

drive and can bypass all the normal device drivers.  In this way

I can be sure that the device drivers are not at fault when SPINDOWN

doesn't work.

If you have a Future Domain IDE controller and PowerIDE, you are

all set.  You just have to add the device drivers in config.sys

and autoexec.bat. (see below my example)

If you don't have a Future Domain product, you will have to find

the ATASPI drivers elsewhere.  You might want to have a look

at Mediavision's FTP site (ftp.mediavis.com), or their web site

(www.mediavis.com), where they keep update driver files for

many of their products.  You might find update drivers for your

MV600, which runs on a Future Domain controller.

The two lines needed in my config.sys file are:

device=c:\ataspi\ataspi16.sys /c:170,15,376h /v

device=c:\ataspi\fdatacd.sys /d:mscd000

The line needed in my autoexec.bat file is:

c:\ataspi\mscdex /d:mscd000 /v /e

If you use the /ATASPI or /95ATAPI options, SPINDOWN will also check to

see if your drive supports changing the spindown times.  SPINDOWN can now

directly query the drive itself to see if it supports this.

The following is the command line usage:

Usage: SPINDOWN TIMEOUT <mode> <options>

TIMEOUT - Specify the new drive spindown time in seconds

/ASPI    ADAPTER:ID:LUN - Use ASPI drivers to talk to SCSI drive

/ATASPI  ADAPTER:ID     - Use ATASPI drivers to talk to ATAPI drive

/95ATAPI ADAPTER:ID     - Use Win 95 ASPI drivers to talk to ATAPI drive

/ATAPI   IO:IRQ:DEVICE  - Use native mode ATA/ATAPI drivers

IRQ    - Specify the IRQ of the IDE controller

DEVICE - Specify the Device number of the IDE drive

IO     - Specify the IO address of the IDE controller

ADAPTER- Specify the ASPI adapter number

ID     - Specify the ASPI device ID number

LUN    - Specify the ASPI device logical unit number

---------Examples to set spindown time to 2 minutes:

DOS only ATAPI drive at IO=170, IRQ=15, Device#=0

spindown 120 /atapi 170:15:0

DOS or Windows ASPI driver with SCSI drive at Adapter=0, ID=2, LUN=0

spindown 120 /aspi 0:2:0

Windows '95 ATAPI drive at Adapter=0, ID=0

spindown 120 /95atapi 0:2

DOS ATASPI driver with ATAPI drive at Adapter=0, ID=0

spindown 120 /95atapi 0:2

-----------------------------------------------------------------------

The following is the output of the program in various configurations:

For DOS only, ATAPI drive at IO=170, IRQ =15, Device=0, Timeout=2 minutes

spindown 120 /atapi 170:15:0

SPINDOWN 1.0a

Copyright (c) 1996 - Computall Services.

Running DOS version 6.22

Setting inactivity timer to 2 minutes

IO port: 0x170, IRQ: 15, Device: 0

Checking for controller at 0x170 ... Found controller at 0x170

Downloading 0x0c bytes...

Downloading remaining 0x04 bytes...

Done

-----------------------------------------------------------------------

For DOS/Windows, SCSI drive at Adapter=0, SCSI ID=2, LUN=0, Timeout=2 minutes

spindown 120 /aspi 0:2:0 /scsi

SPINDOWN 1.0a

Copyright (c) 1996 - Computall Services.

Running DOS version 6.22

Setting inactivity timer to 2 minutes

Adapter: 0 ID: 2, LUN: 0

ASPI Manager id: SYMBIOS LOGIC  , adapter id: SYM53C8XX

vendor_id:TOSHIBA , product_id:CD-ROM XM-3401TA, revision:0283

Spindown time changed successfully...

Done

-----------------------------------------------------------------------

For Windows 95 only, ATAPI drive at Adapter=0, SCSI ID=0, LUN=0, Timeout=2 minutes

spindown 120 /aspi 0:0:0 /atapi

SPINDOWN 1.0a

Copyright (c) 1996 - Computall Services.

Running DOS version 7.0

Running Windows 4.0

Setting inactivity timer to 2 minutes

Adapter: 0 ID: 0, LUN: 0

ASPI Manager id: ASPI for Windows, adapter id: ESDI_506

vendor_id:TOSHIBA , product_id:CD-ROM XM-5302TA, revision:1095

Spindown time changed successfully...

Done

-----------------------------------------------------------------------

Copying and Registration (does he mean this costs $$$?)

-------------------------------------------------------

I am allowing full freedom to copy this program.

It IS a fully functional version.

It is NOT Crippleware!

There is NOT a pro version available for an unreasonable sum of money.

If you don't agree with the shareware policy, then you can:

a) use the program anyway and violate the copyright, and as such

you will probably end up rotting in HE double hockey stick

b) don't use the program at all

The registration fee for using SPINDOWN is now $15 US.

I won't be asking for a fee every time a new version comes out.  This

fee will give you a lifetime (as long as I bug fix and add features)

of free updates via the Internet.  I will also notify registered users

of any new versions for a year.

My mail address is:

Jim McLaughlin

337 Nautica Ct.

Indian Harbour Beach, FL

32937

If you have any questions, my email address is jmclaugh@nortel.ca, but RTFM

before you start sending me questions.  I also have a home page which

will always have pointers to the most recent versions of all my programs,

and many, many pointers to other CDROM related sites and driver

archives:

http://www.ncf.carleton.ca/~aa571

You might want to check out some of my other programs: (ATAINF is

useful to tell if SPINDOWN has indeed set your new values correctly)

CDDA

CDDA is a DOS PC based utility that allows the user to extract digitally

perfect copies of samples from audio CDs.  It also allows the user

to extract MPEG streams from CDi Digital Video CDs, and VideoCDs, and

XA frames from CD-XA CDs.

ATAINF

ATAINF is a program I wrote to find the parameters and capabilities

of ATA and ATAPI drives.  ATA drives are more commonly known as

IDE or EIDE hard drives, and ATAPI are more commonly known as IDE

versions of CDROM drives.  There were only a couple of programs

out there that would dump some of the paramters of IDE hard drives,

but they had no support for CDROM and most of the time they did

not follow the ATA specs.

-----------------------------------------------------------------------

I would also like to try and keep all the files together and unchanged

in the archive, so if you are passing this around don't change the files.

In this distribution the files contained are:

SPINDOWN.EXE - the program itself

SPINDOWN.DOC - this document file

SCSIPING.EXE - a program to scan ASPI for CDROM drives

If you are one of those Shareware houses, I don't want to see this program

available for $6.99 plus shipping and handling.  I don't like seeing

the average Joe getting burned for a 10 cent disk, 1 cent label and 2 cents

worth of labour to make the disk, especially when he can get 600+ Meg

of really good stuff from something like the Simtel CD for $25

(Thanks to Keith for keeping Simtel running, I just wish it would stay

still for good).

How to get hold of me for help.

-------------------------------

If you want to make suggestions or want to send compliments you can contact

me at the address below.  I might even listen to a complaint or two.

If you would like assistance in solving problems, please include a

dump from the.  You should be able to run the program like this and

get the dump in a text file:

SPINDOWN 120 /ATAPI 170:15:0  >dumpfile.txt

I have real difficulty in tracking down problems without the

dumps.  In the past the majority of people asking for help just send

along a short note telling me my program doesn't work, and what

they remember the error messages saying.  This just doesn't cut it.

I have to have the dumps to help.

The bottom line - RTFM before writing me.  Also read the alt.cd-rom

FAQ.  many of your questions can be answered by reading.

EMAIL: jmclaugh@nortel.ca

Changes in 1.2 from 1.0a

--------------------------

- much better support for ATAPI drives with either ATASPI drivers or

ASPI drivers under Win95

- cleaned up the SCSI code to work on more drives

Changes in 1.0a from 0.9a

--------------------------

- added support for ASPI

- added support for SCSI drives

- added support for ATAPI drives under Windows '95

- a large number of people were having problems with version 0.9 and

were not happy.  I think I have all of their problems fixed in this

version.  I have completely rewritten my dos device driver, and

harmonized it with my device drivers for CDDA and ATAINF.  This has

been quite well tested, and should work much better.

- a number of messages have been added when errors occur to help me

debug

- a number of changes have been made to the code to make up for problems

with most Toshiba ATAPI drives that are not completely compliant with

the ATAPI spec. (Is it that tough for them to make their drives

follow the spec?  Other vendors like Sony don't seem to have this problem)

Known bugs:

- none at this time, but I imagine this will change within a few hours

of the software being released