GO R8A version 1.2 (C)

Receiver Controller

for the Drake (R) R8A

(C) Copyright Joe Fahrenholtz 1997

pg. 1

Contents

pg.

Introduction                               1

Installation                               3

Operation                                  5

GO R8A Commands                            9

Channel Data Entry                        15

Useful Tips                               16

User Support                              17

Revision History                          17

License and Registration                  18

Introduction

GO R8A is  a  DOS  based  program  for controlling the Drake (R) R8A

communications  receiver.  It  was  developed  using an R8A receiver

which  has  R8A.8  firmware  installed.  GO R8A  is  not intended to

compete  with other Windows based packages which replace the radio's

scan  functions.  Rather,  it  is intended to complement the radio's

built-in  capabilities.  In  addition,  it  runs  on 386 and 486 PCs

which should should be inexpensive and relatively easy for the radio

hobbyist to acquire.  GO R8A may run on a 286 or earlier PC.  GO R8A

may  also  support  an  R8B  (see  the  Installation section of this

document for options set in file GOR8A.CFG).

It  would  be  best  if you are familiar with the radio's functions.

This  program  can make using your radio a little more user friendly

if  you don't mind typing commands.  Most of the functions available

at the radio's front panel are available via the keyboard.

There are complementing functions that can be quite useful, such as:

BACKUP -

Copies  the  memory  (440  channels  for the R8A, 1000

channels   for  the  R8B,  with  all  available  mode,

bandwidth,  gain,  agc,  antenna,  frequency,  station

name, noise blanker, and display info)  from the radio

to an editable ASCII data file.

STORE MEM -

Copies  the information in an editable ASCII data file

(such as that created by the BACKUP command above)  to

the  radio's memory.  This can be done either with the

file  created with the BACKUP command  (to restore its

memory) or with a smaller file, for example to load 10

new stations into one bank of the radio's memory.

pg. 2

Introduction (cont'd)

GO -

Searches  the  station  names stored in the radio (the

entire  7  character  string or any portion of it) and

goes to the first channel with a matching string.

NOTE:  BACKUP  must  be used prior to GO to generate a

current backup memory file

SCAN A-B SPECIAL -

SCAN FREQ SPECIAL -

SCAN LIST SPECIAL -

Specialized  scan  functions.  Try them, you might like

them. I developed them for my own use. However they are

not as fast as the radio's built in scanning functions.

SCAN A-B SPECIAL  features user specified scan step and

delay  and  a  "go  to  frequency"  function. SCAN FREQ

SPECIAL and SCAN LIST SPECIAL have specified scan delay

and  string  search  functions and utilize user created

ASCII   files.    All   three   scan   functions   have

pause/scan/up/down/reverse single key commands.

CHANNEL DATA -

Simplifies  input  of  channel data,  all on one screen

with defaults that can speed the process.

The  program  features  both "long" (easier to remember) and "short"

(faster to input) commands for most functions.

One  feature  of  GO R8A  is the fact that the data files needed for

memory  maintenance  and  PC  based  scanning are editable from your

favorite  DOS  (or  Windows- during file maintenance) text editor or

even your word processor if it can import and export ASCII files.

pg. 3

Installation

The GO R8A package includes these files:

EXAMPLE.LST  sample file for SCAN LIST SPECIAL command

EXAMPLE.FRK  sample file for SCAN FREQ SPECIAL command (kHz)

EXAMPLE.FRM  sample file for SCAN FREQ SPECIAL command (MHz)

GOR8A.EXE  program executable file

GOR8A.CFG  configuration file

GOR8A12.DOC  documentation file (this document)

SUNITS.R8A  sample S-meter calibration file

To  install  GO R8A, copy the distribution files to your working disk

or directory.

GOR8A.EXE  is  a standalone  executable  file.  There are no required

driver files. GOR8A12.DOC is this manual. "COPY GOR8A12.DOC PRN" will

print it from the DOS command prompt.

This  program was developed using a DOS based programming language on

a  16 MHz  386SX PC  with  640 x 480  VGA  video.  The user should be

somewhat  familiar  with DOS and should have DOS available on his PC.

Running  in  a  DOS  window  in  the  Windows operating system is NOT

recommended.  The program may run on a 286 (or earlier) PC,  but this

has not been tested as of this writing.

This  program  requires DOS.  If your computer uses one of the recent

Windows  operating  systems  that  does  not  require  DOS  (such  as

Windows 95  or  NT),  you will need to create a bootable A: drive DOS

diskette  (refer  to  the DOS "SYS" command in a DOS manual)  and you

will  have  to  use the bootable diskette to boot up on your A: drive

when  you  want  to  run GO R8A.  The bootable diskette is not needed

under Windows 3.0 or 3.1, as they require the presence of DOS on your

computer.

GO R8A uses a configuration file,  GOR8A.CFG, to set runtime options.

As bundled, GOR8A.CFG looks like this:

COM1

-04.00 HOURS UTC OFFSET

R8A

COLOR

LOCAL

Using  the initial GOR8A.CFG file,  GO R8A uses serial port COM1.  An

ASCII  text  editor  can  be  used  to  select any desired port.  For

example, to use port COM2, the first line should read:

COM2

The COM port must be set at 9600 baud, no parity, 8 data bits, 1 stop

bit. Refer to a DOS manual or DOS on-line help under the MODE command

for  information  on  setting  these  parameters.   The  DOS  command

MODE COM1:96,N,8,1 sets up COM1 for GO R8A on my PC under DOS 6.0.

pg. 4

Installation (cont'd)

GOR8A.CFG  should  also  be  edited  to  set  your  UTC  (Coordinated

Universal Time)  offset.  This  is  the  number that must be added to

(positive)  or subtracted from (negative) UTC to get your local time.

In  the  above  initial  file,  -4.00  hours  reflects summer for the

Eastern  United  States.  Any number  (+/- or decimal such as +xx.50)

can  be  entered.  GO R8A  sets  both  local  time and UTC (using the

offset) of the radio at the same time via the SET TIME function.

In order to support the new R8B,  GOR8A.CFG may be useful in allowing

access to storing/backing up channels 440-999. Edit the third line of

GOR8A.CFG to:

R8B

Since I have an R8A, I can't predict the results. As of this writing,

this feature has not been tested with an R8B. If you have an R8A, the

third line should read R8A.

Your  computer  must  have 640 x 480 VGA video support to run GO R8A.

The  fourth  line  of  GOR8A.CFG  can  be  used  to  choose color vs.

monochrome. If you want monochrome VGA, edit the fourth line to:

MONO

The fifth line of GOR8A.CFG indicates whether your PC clock is set to

local time or UTC. If it is set to UTC, edit the fifth line to:

UTC

The  cable from the PC serial port to the RS232 connector on the back

of the radio should:

1) Have the proper end connectors- DB-9 male at the radio end and the

proper no. of pins/gender at the PC end.

2) Be  a  shielded cable to minimize the pickup of computer generated

RF noise.

Refer  to  the  "RS232 Interface" section of the manual supplied with

your  receiver  for  additional  important info on selecting a cable.

Your  local  Radio Shack  or  other  electronics  parts  store can be

helpful in selecting a suitable cable.

pg. 5

Operation

At  the  DOS  command  prompt  at the working disk or directory, type

GOR8A  <enter>.  The  program  will turn the radio on,  then read and

display current radio settings. If this does not happen, be sure that

power  is  being supplied to the radio.  A command prompt will appear

below  the display.  Refer to the R8A Commands section of this manual

for  command  line options.  Most of the radio's pushbutton functions

are  available  from  the  command line.  Some enhanced functions are

added  via this program.  The radio's analog controls (the notch/tone

knobs,  passband offset/squelch knobs,  and volume/rf gain knobs) are

not controllable from the PC.  An S-meter is not displayed on the PC,

but  an  S-meter  reading  is  available  via  the SIG command at the

R8A COMMAND> prompt.

I  would  recommend  that  the first command sent to the radio is the

BACKUP  command.  This  will create a file called R8BACKUP.SAV, which

contains  all  the  available  information  in  your  radio's  memory

channels.  The  time required for BACKUP may vary. On my 386SX PC, it

takes  approximately  60  seconds. It may also be a good idea to copy

this  file to another backup file of your own naming since subsequent

BACKUP  commands  may  overwrite  the  R8BACKUP.SAV file. Examine the

file with your favorite text editor to verify that it is complete and

to become familiar with the file format. The R8BACKUP.SAV file can be

used  as the input file for the STORE MEM command in order to restore

the  radio's memory.  Modification of this file can be used for large

or small scale changes of radio memory.

One  feature  of  GO R8A  is  the fact that the data files needed for

memory  maintenance  and  PC  based  scanning  are editable from your

favorite  DOS  (or Windows-  during  file maintenance) text editor or

even  your  word  processor  if it can import and export ASCII files.

The structures of the editable ASCII data files are as follows:

EXAMPLE.LST:

CH,HZ,SYNC,MEMLOCK,NBLANK,AGC,GAIN,NOTCH,ANT,MODE, BW,  FREQMHZ,   NAME

10, K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,   .55000,WKRC

11, K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,   .70000,WLW

12, K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,   .74000,WNOP

13, K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,   .91000,WPFB

14, K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,  1.16000,WKYN

CH,HZ,SYNC,MEMLOCK,NBLANK,AGC,GAIN,NOTCH,ANT,MODE, BW,  FREQMHZ,   NAME

20, K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,  2.50000,WWV-250

21, K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,  3.33000,CHU-333

The  same  format  (above)  is used for STORE MEM (.BSM) input files,

the  R8BACKUP.SAV  output file which results from the BACKUP command,

and the files output by READ MEM.

pg. 6

Operation (cont'd)

Permissible data for the EXAMPLE.LST (.BSM, .SAV) file fields are:

CH-        0 to 439  for  the  R8A,  0 to 999  for the R8B,  integer,

channel number

For BACKUP (.SAV) and STORE MEM (.BSM) files:

Channel  numbers  can  start at any number,  and will be

sorted  in  ascending order by the program.  Also, it is

permissible to skip (gap) channels (refer to the example

file).

IMPORTANT NOTE: Gap (undefined) channels are erased when

using STORE MEM.

IMPORTANT NOTE:   Any   duplicate   channels   will   be

eliminated  by  the  program  when  it sorts by channel

number.

For SCAN LIST SPECIAL (.LST) files:

The channel number is optional,  but the comma after the

field is mandatory.

HZ-        K (kHz) OR M (MHz),  selects  units  of  frequency display

SYNC-      ON or OFF, synchronous detection

MEMLOCK-   ON or OFF,  effective  for  STORE MEM  only-  not for SCAN

LIST SPECIAL. MEMLOCK ON  skips the given channel during

receiver built-in scanning only (via the SCAN command).

NBLANK-    OFF ,W (wide), or N (narrow), noise blanker control

AGC-       OFF, F (fast), or S (slow), agc control

GAIN-      ATTN (attenuator), PRE (preamp), or OFF

BW-        0.5, 1.8, 2.3, 4.0, or  6.0,  IF  filter  bandwidth  (kHz)

NOTE: The BW data must be 3 characters,  the 2nd character

must be a decimal point (.)

NOTCH-     ON or OFF

ANT-       1, 2, or VHF, selects antenna

MODE-      AM, CW, RTTY, FM, LSB, or USB

FREQMHZ-   0.1 to 30.0, frequency (MHz only)

IMPORTANT NOTE:  Frequency must be MHz,  the HZ field only

controls the PC display and radio display of units.

NAME-      For BACKUP (.SAV) and STORE MEM (.BSM) files:

7 characters maximum, each character A thru Z, 0 thru 9,

(space), or - (dash)

For SCAN LIST SPECIAL (.LST) files:

10  characters  maximum, any PC displayable character is

permissible

NOTE: The  NAME  field  is optional,  but the comma before

the field is mandatory.

NOTE: If there is any possibility that you may want to use

your  SCAN LIST SPECIAL  (.LST)  files  as input to STORE

MEM,  you  should  probably  hold  to the same NAME field

restrictions as for BACKUP and STORE MEM.

Note: Comma delimiters are REQUIRED between the above data fields.

pg. 7

Operation (cont'd)

EXAMPLE.FRK:

FREQKHZ    NAME

550        WKRC

700        WLW

740        WNOP

910        WPFB

1160       WKYN

Permissible data for the EXAMPLE.FRK fields are as follows:

FREQKHZ-   100 to 30000, frequency (kHz)

NOTE:  FREQKHZ  in  the first line controls the PC screen

display  and  radio display of units and tells the PC to

expect  kHz  frequency data in ALL remaining lines.  The

rule  here  is  that the first line (FREQKHZ    NAME) is

optional, but if present, all subsequent lines must have

kHz frequency data.  If the first line (FREQKHZ    NAME)

is  not  present,  the  program  will prompt the user to

input units.

NAME-      50  characters  maximum,  any PC displayable character is

permissible.

NOTE: This field is optional.

Note:   Comma or space delimiters are NOT REQUIRED between the above

data fields.

pg. 8

Operation (cont'd)

EXAMPLE.FRM:

FREQMHZ    NAME

.55        WKRC

.7         WLW

.74        WNOP

.91        WPFB

1.16       WKYN

Permissible data for the above EXAMPLE.FRM

fields are as follows:

FREQMHZ-   0.1 to 30.0, frequency (MHz)

NOTE: FREQMHZ  in  the  first line controls the PC screen

display  and  radio display of units and tells the PC to

expect  kHz  frequency data in ALL remaining lines.  The

rule  here  is  that the first line (FREQMHZ    NAME) is

optional, but if present, all subsequent lines must have

kHz frequency data.  If the first line (FREQMHZ    NAME)

is  not  present,  the  program  will prompt the user to

input units.

NAME-      50  characters  maximum,  any PC displayable character is

permissible.

NOTE: This field is optional.

Note:   Comma or space delimiters are NOT REQUIRED between the above

data fields.

Note  that  in the EXAMPLE.LST,  EXAMPLE.FRK,  and EXAMPLE.FRM files,

there  are  lines  of  data  which contain field explanations.  These

field  explanation  lines  can be copied to any position in the files

to make reading and editing easier. However,  they are optional,  and

need  not  be present.  Placement of blanks within a line of data and

number  of decimal places for frequencies are generally not critical,

but can be helpful in making files readable.

As of this writing,  the program has not been tested on an radio with

a VHF converter installed. I can not predict the results of operation

of this program with a VHF converter.

IMPORTANT NOTE: To avoid impaired scanning action,  it is recommended

that your scanning programming avoids rapid switching of the antenna

select relay and preamp/attenuator gain relay.

See  the Channel Data Entry and Useful Tips sections of this document

and  the  manual  supplied  with  your radio for additional operating

information.

pg. 9

GO R8A Commands

*BASIC RADIO CONTROLS*

"AGC FAST","AF"

"AGC SLOW","AS"

"AGC OFF", "AO"

"ANT 1",   "A1", antenna 1

"ANT 2",   "A2", antenna 2

"ANT VHF", "AC", VHF antenna

"BW 0.5",  "W0", IF filter bandwidth 0.5 kHz

"BW 1.8",  "W1", IF filter bandwidth 1.8 kHz

"BW 2.3",  "W2", IF filter bandwidth 2.3 kHz

"BW 4.0",  "W4", IF filter bandwidth 4.0 kHz

"BW 6.0",  "W6", IF filter bandwidth 6.0 kHz

"Fnnn.nnnn","Fnnnnn.nn","Fnnnnnnn", go to frequency nnn...

"USB",     "M1"

"LSB",     "M2"

"RTTY",    "M3"

"CW",      "M4"

"FM",      "M5"

"AM",      "M6"

"BLANK OFF","BF", noise blanker off

"BLANK N",  "BN", noise blanker narrow

"BLANK W",  "BW", noise blanker wide

"NOTCH OFF","NF"

"NOTCH ON", "NO"

"POWER OFF","PF"

"POWER ON", "PO"

"PRE",      "PRE ON",  "G+", preamp on

"ATTN",     "ATTN ON", "G-", attenuator on

"PRE OFF",  "ATTN OFF","G0", no gain, PRE/ATTN off

".XX" ,    "ST0", step 2 decimal places

".X" ,     "ST1", step 1 decimal place

"." ,      "ST2", step 0 decimal places

"STEP",    "ST",  cycle through step settings

"SYNC OFF","SF",  synchronous detector off

"SYNC ON", "SO",  synchronous detector on

"UP",    "U",   tune up

"DOWN",  "D",   tune down

"VFO A", "VA",  select VFO A

"VFO B", "VB",  select VFO B

"A=B",   "AEB", set VFO A = VFO B

pg. 10

GO R8A Commands (cont'd)

*MEMORY MANAGEMENT*

"READ MEM",  "BR"

block read memory channels from radio to ASCII PC file, prompts for

lower and upper channel numbers and output file name

"STORE MEM", "BS"

block store memory channels to radio from ASCII PC file,

prompts for name of file

IMPORTANT NOTE:  Channel  number  gaps  are  permissible in the PC

file to be stored.  HOWEVER, GAP (UNDEFINED) CHANNELS ARE ERASED.

This  is  consistant  with the structure of the R8BACKUP.SAV file

created  by the BACKUP command, which has gaps at blank channels.

"ECnnn", erase channel nnn

"MEM TUNE OFF", "CTF"

"MEM TUNE ON",  "CTO"

"NAME CH",  "NC", name channel, prompts for channel name,

7  characters  maximum,  characters A thru Z,  0 thru 9,  (space),

or - (dash)

NOTE: In order to name (or rename) a channel, the radio must be in

Memory Mode  (MEM  or  MEM TUNE  must  be  displayed  next to the

channel  number  on  the display).  Entering a channel number via

Cnnn will put the radio in Memory Mode.

"NAME ON",  "NA", display name

"NAME OFF", "NI", display frequency

"Cnnn", go  to  channel  nnn,  "C"  (no  trailing  numbers)  goes to

previously used channel

"STORE CH", "PR", store channel, prompts for channel number

"BACKUP",  user  can  back up radio memory channels to ASCII PC file

R8BACKUP.SAV,  memory  contents can be restored to the radio using

R8BACKUP.SAV as the input file for STORE MEM

"CHANNEL DATA", "CHD",  prompts  for  all inputs needed to specify a

channel,  with  useful  defaults,  significantly easier than using

individual line commands. Hit <enter> to accept the defaults shown

"GO", "GO ccccccc", ccc... = up to 7 characters maximum, search for a

memory channel by name, a portion of a name can be used, the search

can  be continued sequentially through memory,  hit <enter> to exit

NOTE: "BACKUP"  must be run prior to using this command so that the

PC memory backup file is current

pg. 11

GO R8A Commands (cont'd)

*INFORMATION RETRIEVAL*

"ID",  identify radio firmware

"REPORT", "RA", <enter>, report all radio settings

"REPORT LIST", "RSL",      report scan list,   see

also "SLxxxxxx..." command under *SCAN FUNCTIONS*

"SIG", "RSS", report signal strength

*SCAN FUNCTIONS*

"SCAN MEM", "S1"

"SCAN LIST","S2"

"SCAN A-B", "S3"

"SCAN SEEK","S4"

"SCAN TIME","S5"

"SCAN CARR","S6"

"SCAN", "SCAN START", "SCO",  enters scan mode,  locks out

radio front panel keys, the following commands are allowed:

+---------------------------------+

|       R8A *SCAN* COMMANDS       |

|                                 |

|   "STEP 0",      "ST0"          |

|   "STEP 1",      "ST1"          |

|   "STEP 2",      "ST2"          |

|   "STEP 3",      "ST3"          |

|   "STEP",        "ST"           |

|                                 |

|   "SYNC OFF",    "SF"           |

|   "SYNC ON",     "SO"           |

|   "UP",          "U"            |

|   "DOWN",        "D"            |

|                                 |

|   "SCAN MEM",    "S1"           |

|   "SCAN LIST",   "S2"           |

|   "SCAN A-B",    "S3"           |

|   "SCAN SEEK",   "S4"           |

|   "SCAN TIME",   "S5"           |

|   "SCAN CARR",   "S6"           |

|                                 |

|   "START","SCAN","SCO","<enter>"|

|   "STOP","SCF",  "<enter>"      |

|   "END",         "EXIT"         |

+---------------------------------+

"VFO LIMIT A", "SA", transfers current VFO to scan limit A

"VFO LIMIT B", "SB", transfers current VFO to scan limit B

pg. 12

GO R8A Commands (cont'd)

*SCAN FUNCTIONS* (cont'd)

"SLxxxxxx...", enters lists (banks) to scan,

where xx is bank 00 to bank 43 (up to 10 banks) for the R8A,

or xx is bank 00 to bank 99 (up to 20 banks) for the R8B

Example: "SL00010203" enters banks 00, 01, 02, and 03

Example: "SL" (no trailing numbers) clears the scan list

See also "RSL" command under *INFORMATION RETRIEVAL*

"SCAN A-B SPECIAL", "SCNAB", user can set frequency limits, step, and

delay.   This  function  does  not  use  or  modify  radio  memory.

User is prompted for:

SCAN START FREQUENCY

SCAN END FREQUENCY

SCAN STEP

SCAN DELAY (SEC)

During scanning, user can key:

<spacebar>  - pause/scan toggle

R  - reverse scan direction

U  - scan up

D  - scan down

G  - go to frequency,

prompts for frequency

C  - change scan parameters,

prompts for new parameters

E  - exit scan A-B special

"SCAN FREQ SPECIAL", "SCNF",  user  can scan the frequencies in a user

created file, user can specify frequency and name. A file containing

"new"  frequencies  can  be  merged  with the scan data file.  As an

option,  duplicate  frequencies  can be deleted via a sort function.

A  string  search  option  can  find  frequencies  and station names

quickly.  Any  number  of frequencies can be scanned.  This function

does not use or modify radio memory.

During scanning, user can key:

<spacebar>  - pause/scan toggle

R  - reverse scan direction

U  - scan up

D  - scan down

G  - go to string, searches frequency and name for any length

string, prompts for string

C  - change scan delay

E  - exit scan freq special

pg. 13

GO R8A Commands (cont'd)

*SCAN FUNCTIONS* (cont'd)

"SCAN LIST SPECIAL", "SCNL",  user  can scan the frequencies in a user

created file,  user can specify frequency display units K<Hz>/M<Hz>,

synchronous   detection   ON/OFF,    noise   blanker   ON/OFF,   agc

OFF/F<ast>/S<low>, gain PRE/ATTN/OFF, notch ON/OFF, antenna 1/2/VHF,

mode AM/CW/USB/LSB/RTTY/FM,  IF filter bandwidth 0.5/1.8/2.3/4.0/6.0

kHz,  frequency (MHz) and name.  A file containing "new" frequencies

can  be  merged  with  the scan data file.  As an option,  duplicate

frequencies  can  be  deleted via a sort function.  A string  search

option can find frequencies and station names quickly. Any number of

frequencies  can  be  scanned.  This function does not use or modify

radio memory.

During scanning, user can key:

<spacebar>  - pause/scan toggle

R  - reverse scan direction

U  - scan up

D  - scan down

G  - go to string, searches frequency and name for any length

string, prompts for string

C  - change scan delay

E  - exit scan list special

*CLOCK AND TIMER FUNCTIONS*

"TIMER 1 OFF",     "TD1"

"TIMER 2 OFF",     "TD2"

"TIMER 1 ON",      "TE1"

"TIMER 2 ON",      "TE2"

"TIMER RELAY OFF", "TF"

"TIMER RELAY ON",  "TO"

"RELAY BREAK OFF", "TSF", do not activate relay on squelch break

"RELAY BREAK ON",  "TSO", activate relay on squelch break

"TL", report local time

"TLhhmmss", set local time, hhmmss=hourminutesecond

"TU", report UTC

"TUhhmmss", set UTC,        hhmmss=hourminutesecond

"T1F", report timer 1 off time

"T1O", report timer 1 on time

"T2F", report timer 2 off time

"T2O", report timer 2 on time

"T1Fhhmm", set timer 1 off time, hhmm=hourminute

"T1Ohhmm", set timer 1 on time,  hhmm=hourminute

"T2Fhhmm", set timer 2 off time, hhmm=hourminute

"T2Ohhmm", set timer 2 on time,  hhmm=hourminute

pg. 14

GO R8A Commands (cont'd)

*CLOCK AND TIMER FUNCTIONS* (cont'd)

"SET TIME", "TIME",  sets  radio local and UTC time,  time can also be

set for PC, time is input in 24 hr. HH:MM:SS format.

NOTE:  UTC  offset  value  and PC clock LOCAL/UTC value in GOR8A.CFG

file  must  be  correct for proper SET TIME function.  Refer to the

discussion  of  file  GOR8A.CFG in the Installation section of this

manual.

User can choose:

D  - display PC time and current UTC offset

X  - transfer time from PC to radio

B  - set time for both radio and PC

R  - set time for radio only

E  - exit

*MISCELLANEOUS*

"LOCK OFF", "UNLOCK", "LF"

"LOCK ON",  "LOCK",   "LO"

"STEP 10",  "SSU", set broadcast band channel step to 10 kHz

"STEP 9",   "SSE", set broadcast band channel step to 9 kHz

"MHZ", "SM", display MHz

"KHZ", "SK", display kHz

"DISPLAY FREQ",   "FR"

"DISPLAY TIME",   "TI"

"BRIGHT", "LAMP", "LB", select display brightness

"SHELL",  temporarily  transfers control to DOS,  can be used for file

management or to check for available scan data files via the DOS DIR

command. Type EXIT at the DOS command prompt to return to GO R8A.

"END", "EXIT"

pg. 15

Channel Data Entry

There are several ways to enter channel data.

Perhaps the easiest way to add a few channels is to use CHANNEL DATA,

which  is  a single screen method of entering channels one at a time.

All storable channel data may be specified.

Another channel entry method is to create or edit a file for use with

the  STORE MEM command.  Refer to the GO R8A commands section of this

manual for details. Note that gap (undefined) channels in the file to

be stored are erased with STORE MEM. For example:

CH,HZ,SYNC,MEMLOCK,NBLANK,AGC,GAIN,NOTCH,ANT,MODE, BW,  FREQMHZ,   NAME

0 , K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,   .55000,WKRC

8 , K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,  0.91000,WPFB

9 , K, ON ,    OFF,   OFF,F  ,PRE ,  OFF,1  ,AM  ,6.0,  1.16000,WKYN

This  file will ERASE channels 1 thru 7 in the radio's memory if used

with STORE MEM.  However,  when using SCAN LIST SPECIAL with the same

file,  no  channels  are  erased  because no information is stored in

radio  memory with SCAN LIST SPECIAL.  If you want to preserve nearby

channels  and add or change others using STORE MEM,  it would perhaps

be  best to edit a portion of the current R8BACKUP.SAV file,  leaving

nearby channels intact.

Finally,  it  is  possible  to  use individual line commands to enter

channels.  First, set frequency, agc, antenna, etc. via line commands

or  the  radio  control  panel.  Then  use Memory Management commands

STORE CH, C and NAME CH to store and name the channel.

pg. 16

Useful Tips

It  is  advisable  to use file suffixes which identify the file type.

For  example,  I use a .BSM suffix on STORE MEM files,  .LST for SCAN

LIST SPECIAL files,  .FRK (kHz) and  .FRM (MHz) for SCAN FREQ SPECIAL

files.

When  sorting a large file in SCAN FREQ SPECIAL or SCAN LIST SPECIAL,

be advised that the bubble sort that is performed can take some time.

Included  with  the  program  is  a file named SUNITS.R8A.  This is a

sample  data  file which contains "raw" S-unit data for my particular

receiver.  You  can  use  this  data file to calibrate GO R8A to your

receiver's S-meter using the following procedure.

1) Tune your radio to a very strong AM station in your area.

2) Send the AGC OFF command to the radio.

3) Use the rf gain knob to adjust the S-meter to an S-1 level.

4) Send  the  command RSSS (note- three S's) to the radio.  The radio

will  report  the  signal  strength  in  S-units just like the RSS

command, and will also display "raw" signal strength data.  Record

the raw signal strength.

5) Repeat steps 3) and 4) for S-2, S-3, S-4, S-5, S-6, S-7, S-8, S-9,

+10, +20, +30, +40, +50, and +60.

6) Edit the data file per your raw signal strength readings.

The SUNITS.R8A file contains 15 pairs of numbers.  Each line contains

a  number  corresponding  to  the  S-level,  a comma,  and raw signal

strength number corresponding to that S-level.  The sample SUNITS.R8A

file (for my receiver) looks like this:

1,00

2,01

3,15

4,29

5,43

6,50

7,75

8,90

9,107

10,130

20,147

30,181

40,208

50,219

60,229

GO R8A uses this file,  if present,  to calculate the signal strength

when it receives raw signal strength data from the radio. If the file

is  not  present,  a  built in set of data is used.  Both the example

SUNITS.R8A  data  file and the built-in data may give S-unit readings

which differ slightly from your receiver's analog S-meter.

pg. 17

User Support

Send questions, comments, or suggested improvements by mail to:

Joe Fahrenholtz

3711 Mt. Carmel Rd.

Cincinnati, Ohio 45244-1617

USA

Return   postage   or   International  Reply  Coupons   (IRC's)   are

appreciated.

I will try to get back to you on any questions ASAP.

A  word  about the author.  I am a shortwave listener (SWL hobbyist).

Currently,  I  enjoy  longwave  beacon  DXing and casual listening to

international shortwave broadcasts.

Revision History

GO R8A Version 1.0: Initial Release

GO R8A Version 1.1:

Added R8B Support, Bug fix in  SCAN LIST SPECIAL  (frequency  field

characters  were lost during sort),  added SHELL out to DOS,  added

CHDATA station data input method,  added TIME command, added the GO

command  and G<o to>  string  search functions to SCAN LIST SPECIAL

and SCAN FREQ SPECIAL.

GO R8A Version 1.2:

All  inputs made case insensitive,  converted SCAN LIST SPECIAL and

SCAN FREQ SPECIAL  to  "all on one screen"  input,  added  enhanced

parsing  of input files to SCAN LIST SPECIAL and SCAN FREQ SPECIAL,

eliminated  "."  entry of leading spaces in station names in CHDATA

and NC functions.

pg. 18

License and Registration

GO R8A   is   provided  as-is.  There  are  no  expressed  or implied

warranties.

GO R8A  is  copyrighted  by Joe Fahrenholtz. You may copy GO R8A only

for use under this license agreement.

GO R8A is distributed as shareware. You may use GO R8A without charge

on  a  trial  basis  to  determine its  suitability  for  you. If you

continue  to  use  it  after  your  evaluation,  a $15 (United States

dollars- USD) registration is requested. This registration covers use

by  a  single  person  (on  multiple  computers) or installation on a

single  computer (may be used by a group of people). There is no cost

to  use  a later 1.* version (if available) after registering any 1.*

version.  I  will  notify  registered  users of GO R8A version 1.* of

releases up to and including version 2.0 (if any).

+--------------------------------------------------------------------+

|Anyone may distribute GO R8A as a stand alone product if the  entire|

|package is distributed together, unchanged, including  this  license|

|explanation, at a  charge  not  to  exceed  $10 (USD).              |

+--------------------------------------------------------------------+

|                           GO R8A Users                             |

| Registration is NOT INCLUDED in any distribution fee that you pay. |

+--------------------------------------------------------------------+

There is a fee as follows for requests for GO R8A via diskette:

$ 6.00 (USD) United States

$ 9.00 (USD) Canada or Mexico

$12.00 (USD) All other countries

If  you  have  the software, there is no need for a diskette request.

Software  is  offered  on  diskette  for people with no Internet/BBS

access.  Registration  is  NOT INCLUDED  in the diskette request fee.

To  register your copy of GO R8A, send your name, address, and check

or money order (U.S. dollars only please) to:

Joe Fahrenholtz

3711 Mt. Carmel Rd.

Cincinnati, Ohio 45244-1617

USA

For  users  outside the United States and Canada, please use a check

from  a  U.S.  or Canadian bank. Equivalent payment in International

Reply Coupons  (or  IRCs- convertible to postage) is acceptable, but

not preferred. Be sure to include your country in your address.

Registration response and user feedback will determine whether there

are  future  versions  of GO R8A. I should note that GO R8A is fully

functional as bundled with this documentation and that this document

file is current, complete documentation for GO R8A version 1.2.