;

;

;-- 22-Dec-88 <-Latest Change ------------------------------------------------

;

;  This file is intended to serve two purposes:

;

;  1) To provide a reference to the myriad of ESCAPE codes used in the

;     Hewlett Packard printer control language (PCL) for the various

;     models of HP Laser Printers.

;

;  2) As a data file for a series of utilities that operate on PCL files

;     to display the contents in an intelligable manner and provide some

;     filtering and editing capabilities. (Utilities are in the works!)

;

;  The primary source of information for this file has been the HP LaserJet

;  Series II Technical Reference Manual and other various sources such as the

;  file ESC_CODE.ARC by S. Noh (SYSOP Form Feed LaserJet BBS (210) 869-1327)

;  found on many bulletin boards.

;

;  The list as presented is sorted on Functional Area, then by Escape code

;  sequence.  You can easily adjust this by passing the file through the

;  DOS or other SORT utility of your choice.

;

;  To enable this document to stay up to date, I encourage anybody with

;  knowledge greater than mine to add any codes they know of to the list,

;  but to retain the format provided as this will be expected by the PCL

;  utilities when they are done. Where the availability on a printer model

;  is unknown, a ? appears in the availability column. For those codes

;  that have a user defined numeric field, the comment ends with "= #'.

;  This will cause the PCL display utility to fill in the value when

;  analyzing the PCL file.

;

;  The original file name that I have distributed is called PCL100.ARC. In

;  order to keep some track of the state of updates to the file I suggest

;  that if you upload it without any changes to another board, you retain

;  the same name,  if you contribute some additional information, add one

;  to the file name (i.e. PCL101.ARC).  A history section is provided at

;  the end of the file for you to record your name and any additional info

;  you think may be useful to others on this topic (optional). Just start

;  line with a semicolon (;) to indicate that it is a comment.

;

;  Lindsey S. Mundy | EXEC-PC Board    (414) 964-5160 Bob Mahoney SYSOP

;  5913 View Lane,  |    or

;  Edina, MN 55436  | Terrapin Station (612) 623-0152 Howard Eckman SYSOP

;

;-----------------------------------------------------------------------------

;

;

;

;-----------------------------------------------------------------------------

;  General ESC code format:

;

;  {esc} p s nnn t   where

;

;    p - the primary   field   Normally a symbol (* & etc.)

;    s - the secondary field   Normally a lower case letter

;  nnn - numeric value field   Optional variable width numeric

;    t - terminator    field   Normally a upper case letter (plus @)

;

;

;             /-----2-- Available on LaserJet Series II   \

;             |/----D-- Available on LaserJet Series IID  | x = not available

;             ||/---P-- Available on LaserJet Plus        >

;             |||/--5-- Available on LaserJet 500         | ? = unknown

;             ||||/-L-- Available on LaserJet             /

;             |||||

;             |||||

;-------------|||||------------------------------------------------------------

;p s nnn t  | ||||| | Functional Area       | Detailed Description

;------------------------------------------------------------------------------

& d     @  | 2???? | Auto-underlining------- Disable

& d     D  | x???? | Auto-underlining------- Enable

& d   0 D  | 2???? | Auto-underlining------- Enable Fixed

& d   3 D  | 2???? | Auto-underlining------- Enable Floating

=  | 2???? | Cursor Position-------- Half Line Feed

& a   # C  | 2???? | Cursor Position-------- Horizontal Column = #

& a   # H  | 2???? | Cursor Position-------- Horizontal Decipoint = #

& a   # R  | 2???? | Cursor Position-------- Vertical Row = #

& a   # V  | 2???? | Cursor Position-------- Vertical Decipoints = #

& f   0 S  | 2???? | Cursor Position-------- Push Position

& f   1 S  | 2???? | Cursor Position-------- Pop  Position

* p   # X  | 2???? | Cursor Position-------- Horizontal Position: Dots = #

* p   # Y  | 2???? | Cursor Position-------- Vertical   Position: Dots = #

Y  | 2???? | Display Functions Mode- Enable

Z  | 2???? | Display Functions Mode- Disable

* c   # D  | 2???? | Font Management-------- Assign Font ID = #

* c   0 F  | 2???? | Font Management-------- Delete All Fonts

* c   1 F  | 2???? | Font Management-------- Delete All Temporary Fonts

* c   2 F  | 2???? | Font Management-------- Delete Last Font ID Specified

* c   4 F  | 2???? | Font Management-------- Make Font Temporary

* c   5 F  | 2???? | Font Management-------- Make Font Permanent

& k   0 S  | 2???? | Font Pitch------------- 10.00 (Primary & Secondary)

& k   2 S  | 2???? | Font Pitch------------- 16.66 (Primary & Secondary)

(     3 @  | 2???? | Font Pitch------------- Default Font = Primary

)     3 @  | 2???? | Font Pitch------------- Default Font = Secondary

(     # X  | 2???? | Font Selection--------- Primary   Font ID = #

)     # X  | 2???? | Font Selection--------- Secondary Font ID = #

& k   0 G  | 2???? | Line Term-------------- CR=CR,    LF=LF,    FF=FF

& k   1 G  | 2???? | Line Term-------------- CR=CR+LF, LF=LF,    FF=FF

& k   2 G  | 2???? | Line Term-------------- CR=CR,    LF=CR+LF, FF=CR+FF

& k   3 G  | 2???? | Line Term-------------- CR=CR+LF, LF=CR+LF, FF=CR+FF

& f   0 X  | 2???? | Macro Control---------- Start Macro Definition

;-----------------------------------------------------------------------------

;

;

;------------------------------------------------------------------------------

;p s nnn t  | 2DP5L | Functional Area       | Detailed Description

;------------------------------------------------------------------------------

& f   1 X  | 2???? | Macro Control---------- Stop  Macro Definition

& f   2 X  | 2???? | Macro Control---------- Execute Macro

& f   3 X  | 2???? | Macro Control---------- Call Macro

& f   4 X  | 2???? | Macro Control---------- Enable Overlay

& f   5 X  | 2???? | Macro Control---------- Disable Overlay

& f   6 X  | 2???? | Macro Control---------- Delete Macros

& f   7 X  | 2???? | Macro Control---------- Delete All Temp. Macros

& f   8 X  | 2???? | Macro Control---------- Delete Macro ID

& f   9 X  | 2???? | Macro Control---------- Make Temporary

& f  10 X  | 2???? | Macro Control---------- Make Permanent

& f   # Y  | 2???? | Macro Control---------- Macro ID = #

9  | 2???? | Margin Control--------- Clear Horizontal Margins

& a   # L  | 2???? | Margin Control--------- Left  Margin Column = #

& a   # M  | 2???? | Margin Control--------- Right Margin Column = #

& l   # E  | 2???? | Margin Control--------- Top Margin  Lines = #

& l   # F  | 2???? | Margin Control--------- Text Lenght Lines = #

& k   # H  | 2???? | Motion Control--------- Horizontal Motion (1/20") HMI = #

& l   # C  | 2???? | Motion Control--------- Vertical Spacing (1/48) VMI = #

& l   1 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 1

& l   2 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 2

& l   3 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 3

& l   4 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 4

& l   6 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 6

& l   8 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 8

& l  12 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 12

& l  16 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 16

& l  24 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 24

& l  48 D  | 2???? | Motion Control--------- Vertical Spacing Lines = 48

& l   1 A  | 2???? | Page Size Select------- Executive

& l   2 A  | 2???? | Page Size Select------- Letter

& l   3 A  | 2???? | Page Size Select------- Legal

& l  26 A  | 2???? | Page Size Select------- European A4

& l  80 A  | 2???? | Page Size Select------- Monarch

& l  81 A  | 2???? | Page Size Select------- Commercial 10

& l  90 A  | 2???? | Page Size Select------- International DL

& l  91 A  | 2???? | Page Size Select------- International C5

& l   0 O  | 2???? | Page Size Select------- Portrait  Orientation

& l   1 O  | 2???? | Page Size Select------- Landscape Orientation

& l   # P  | 2???? | Page Size Select------- Length Lines = #

& l   # X  | 2???? | Page Size Select------- Number of Copies = #

& l   0 H  | 2???? | Paper Input Control---- Eject Page

& l   1 H  | 2???? | Paper Input Control---- Feed from Tray

& l   2 H  | 2???? | Paper Input Control---- Manual Feed

& l   3 H  | 2???? | Paper Input Control---- Manual Envelope Feed

& l   0 L  | 2???? | Perforation Skip------- Disable

& l   1 L  | 2???? | Perforation Skip------- Enable

( s   # H  | 2???? | Primary Font Pitch----- Characters / Inch (CPI) = #

( s   # V  | 2???? | Primary Point Size----- Points = #

( s   0 P  | 2???? | Primary Spacing-------- Fixed

( s   1 P  | 2???? | Primary Spacing-------- Proportional

( s   0 B  | 2???? | Primary Stroke Weight-- Medium

( s   3 B  | 2???? | Primary Stroke Weight-- Bold

;-----------------------------------------------------------------------------

;

;

;------------------------------------------------------------------------------

;p s nnn t  | 2DP5L | Functional Area       | Detailed Description

;------------------------------------------------------------------------------

( s   0 S  | 2???? | Primary Style---------- Upright

( s   1 S  | 2???? | Primary Style---------- Italic

(     0 A  | 2???? | Primary Symbol Set----- HP Math 7

(     0 B  | 2???? | Primary Symbol Set----- HP Linedraw

(     0 D  | 2???? | Primary Symbol Set----- ISO 60: Norwegian 1

(     1 D  | 2???? | Primary Symbol Set----- ISO 61: Norwegian 2

(     0 E  | 2???? | Primary Symbol Set----- HP Roman Extension

(     1 E  | 2???? | Primary Symbol Set----- ISO  4: United Kingdom

(     0 F  | 2???? | Primary Symbol Set----- ISO 25: French

(     1 F  | 2???? | Primary Symbol Set----- ISO 69: French

(     0 G  | 2???? | Primary Symbol Set----- HP German

(     1 G  | 2???? | Primary Symbol Set----- ISO 21: German

(     8 G  | 2???? | Primary Symbol Set----- HP Greek8

(     0 I  | 2???? | Primary Symbol Set----- ISO 15: Italian

(     0 K  | 2???? | Primary Symbol Set----- ISO 14: JIS ASCII

(     1 K  | 2???? | Primary Symbol Set----- HP Katakana

(     2 K  | 2???? | Primary Symbol Set----- ISO 57: Chinese

(     8 K  | x???? | Primary Symbol Set----- Kana-8

(     0 M  | 2???? | Primary Symbol Set----- HP Math 7

(     1 M  | 2???? | Primary Symbol Set----- Technical

(     8 M  | 2???? | Primary Symbol Set----- HP Math8

(     0 N  | 2???? | Primary Symbol Set----- ISO100: ECMA-94 (Latin 1)

(     0 O  | 2???? | Primary Symbol Set----- OCR-A

(     1 O  | 2???? | Primary Symbol Set----- OCR-B

(     0 S  | 2???? | Primary Symbol Set----- ISO 11: Swedish

(     1 S  | 2???? | Primary Symbol Set----- HP Spanish

(     2 S  | 2???? | Primary Symbol Set----- ISO 17: Spanish

(     3 S  | 2???? | Primary Symbol Set----- ISO 10: Swedish

(     4 S  | 2???? | Primary Symbol Set----- ISO 16: Portuguese

(     5 S  | 2???? | Primary Symbol Set----- ISO 84: Portuguese

(     6 S  | 2???? | Primary Symbol Set----- ISO 85: Spanish

(     0 U  | 2???? | Primary Symbol Set----- ISO  6: ASCII

(     1 U  | 2???? | Primary Symbol Set----- HP Legal

(     2 U  | 2???? | Primary Symbol Set----- ISO  2: IRV

(     7 U  | 2???? | Primary Symbol Set----- OEM-1

(     8 U  | 2???? | Primary Symbol Set----- HP Roman-8

(     9 U  | x???? | Primary Symbol Set----- ANSI

(    10 U  | 2???? | Primary Symbol Set----- PC-8

(    11 U  | 2???? | Primary Symbol Set----- PC-8 (D/N)

(    15 U  | 2???? | Primary Symbol Set----- HP Pi Font

( s   0 T  | 2???? | Primary Typeface------- Lineprinter

( s   3 T  | 2???? | Primary Typeface------- Courier

( s   4 T  | 2???? | Primary Typeface------- Helvetica

( s   5 T  | 2???? | Primary Typeface------- Times Roman

( s   6 T  | 2???? | Primary Typeface------- Letter Gothic

( s   8 T  | 2???? | Primary Typeface------- Prestige

( s  11 T  | 2???? | Primary Typeface------- Presentation

( s  17 T  | 2???? | Primary Typeface------- Optima

( s  18 T  | 2???? | Primary Typeface------- ITC Garamond

( s  19 T  | 2???? | Primary Typeface------- Cooper Black

( s  20 T  | 2???? | Primary Typeface------- Coronet Bold

( s  21 T  | 2???? | Primary Typeface------- Broadway

( s  22 T  | 2???? | Primary Typeface------- Bodini Condensed

;-----------------------------------------------------------------------------

;

;

;------------------------------------------------------------------------------

;p s nnn t  | 2DP5L | Functional Area       | Detailed Description

;------------------------------------------------------------------------------

( s  23 T  | 2???? | Primary Typeface------- Century Schoolbook

( s  24 T  | 2???? | Primary Typeface------- University Roman

E  | 2???? | Printer Control-------- RESET

* b   # W  | 2???? | Raster Graphics-------- Transfer Data: Rows = #

* r   0 A  | 2???? | Raster Graphics-------- Start - Left Graphic Margin

* r   1 A  | 2???? | Raster Graphics-------- Start - Current Cursor Position

* r     B  | 2???? | Raster Graphics-------- End Graphics

* t  75 R  | 2???? | Raster Graphics-------- Resolution  75 Dots/Inch

* t 100 R  | 2???? | Raster Graphics-------- Resolution 100 Dots/Inch

* t 150 R  | 2???? | Raster Graphics-------- Resolution 150 Dots/Inch

* t 300 R  | 2???? | Raster Graphics-------- Resolution 300 Dots/Inch

* c   # A  | 2???? | Rectangular Area Fill-- Rectangle Width:  Dots = #

* c   # B  | 2???? | Rectangular Area Fill-- Rectangle Height: Dots = #

* c   1 G  | 2???? | Rectangular Area Fill-- Fill Pattern    1 Horizontal Line

* c   2 G  | 2???? | Rectangular Area Fill-- Fill Pattern   2% Gray / 2 Vert Lines

* c   3 G  | 2???? | Rectangular Area Fill-- Fill Pattern    3 Diagonal Lines

* c   4 G  | 2???? | Rectangular Area Fill-- Fill Pattern    4 Diagonal Lines

* c   5 G  | 2???? | Rectangular Area Fill-- Fill Pattern    5 Square Grid

* c   6 G  | 2???? | Rectangular Area Fill-- Fill Pattern    6 Diagonal Grid

* c  10 G  | 2???? | Rectangular Area Fill-- Fill Pattern  10% Gray

* c  15 G  | 2???? | Rectangular Area Fill-- Fill Pattern  15% Gray

* c  30 G  | 2???? | Rectangular Area Fill-- Fill Pattern  30% Gray

* c  45 G  | 2???? | Rectangular Area Fill-- Fill Pattern  45% Gray

* c  70 G  | 2???? | Rectangular Area Fill-- Fill Pattern  70% Gray

* c  90 G  | 2???? | Rectangular Area Fill-- Fill Pattern  90% Gray

* c 100 G  | 2???? | Rectangular Area Fill-- Fill Pattern 100% Gray

* c   # H  | 2???? | Rectangular Area Fill-- Rectangle Width:  Decipoints = #

* c   0 P  | 2???? | Rectangular Area Fill-- Select Fill = Rule

* c   2 P  | 2???? | Rectangular Area Fill-- Select Fill = Gray Scale

* c   3 P  | 2???? | Rectangular Area Fill-- Select Fill = HP Pattern

* c   # V  | 2???? | Rectangular Area Fill-- Rectangle Height: Decipoints = #

) s   # H  | x???? | Secondary Font Pitch--- Characters/Inch (CPI) = #

) s   # V  | x???? | Secondary Point Size--- Point = #

) s   0 P  | x???? | Secondary Spacing------ Fixed

) s   1 P  | x???? | Secondary Spacing------ Proportional

) s   0 B  | x???? | Secondary Stroke Wt.--- Meduim

) s   3 B  | x???? | Secondary Stroke Wt.--- Bold

) s   0 S  | x???? | Secondary Style-------- Upright

) s   1 S  | x???? | Secondary Style-------- Italic

)     0 A  | x???? | Secondary Symbol Set--- HP Math 7

)     0 B  | x???? | Secondary Symbol Set--- HP Linedraw

)     0 D  | x???? | Secondary Symbol Set--- ISO 60: Norwegian 1

)     1 D  | x???? | Secondary Symbol Set--- ISO 61: Norwegian 2

)     0 E  | x???? | Secondary Symbol Set--- HP Roman Extension

)     1 E  | x???? | Secondary Symbol Set--- ISO  4: United Kingdom

)     0 F  | x???? | Secondary Symbol Set--- ISO 25: French

)     1 F  | x???? | Secondary Symbol Set--- ISO 69: French

)     0 G  | x???? | Secondary Symbol Set--- HP German

)     1 G  | x???? | Secondary Symbol Set--- ISO 21: German

)     8 G  | x???? | Secondary Symbol Set--- HP Greek8

)     0 I  | x???? | Secondary Symbol Set--- ISO 15: Italian

)     0 K  | x???? | Secondary Symbol Set--- ISO 14: JIS ASCII

)     1 K  | x???? | Secondary Symbol Set--- HP Katakana

;-----------------------------------------------------------------------------

;

;

;------------------------------------------------------------------------------

;p s nnn t  | 2DP5L | Functional Area       | Detailed Description

;------------------------------------------------------------------------------

)     2 K  | x???? | Secondary Symbol Set--- ISO 57: Chinese

)     8 K  | x???? | Secondary Symbol Set--- Kana-8

)     0 M  | x???? | Secondary Symbol Set--- HP Math 7

)     1 M  | x???? | Secondary Symbol Set--- Technical

)     8 M  | x???? | Secondary Symbol Set--- HP Math8

)     0 N  | x???? | Secondary Symbol Set--- ISO100: ECMA-94 (Latin 1)

)     0 O  | x???? | Secondary Symbol Set--- OCR-A

)     1 O  | x???? | Secondary Symbol Set--- OCR-B

)     0 S  | x???? | Secondary Symbol Set--- ISO 11: Swedish

)     1 S  | x???? | Secondary Symbol Set--- HP Spanish

)     2 S  | x???? | Secondary Symbol Set--- ISO 17: Spanish

)     3 S  | x???? | Secondary Symbol Set--- ISO 10: Swedish

)     4 S  | x???? | Secondary Symbol Set--- ISO 16: Portuguese

)     5 S  | x???? | Secondary Symbol Set--- ISO 84: Portuguese

)     6 S  | x???? | Secondary Symbol Set--- ISO 85: Spanish

)     0 U  | x???? | Secondary Symbol Set--- ISO  6: ASCII

)     1 U  | x???? | Secondary Symbol Set--- HP Legal

)     2 U  | x???? | Secondary Symbol Set--- ISO  2: IRV

)     7 U  | x???? | Secondary Symbol Set--- OEM-1

)     8 U  | x???? | Secondary Symbol Set--- HP Roman-8

)     9 U  | x???? | Secondary Symbol Set--- ANSI

)    10 U  | x???? | Secondary Symbol Set--- PC-8

)    11 U  | x???? | Secondary Symbol Set--- PC-8 (D/N)

)    15 U  | x???? | Secondary Symbol Set--- HP Pi Font

) s   0 T  | x???? | Secondary Typeface----- Lineprinter

) s   3 T  | x???? | Secondary Typeface----- Courier

) s   4 T  | x???? | Secondary Typeface----- Helvetica

) s   5 T  | x???? | Secondary Typeface----- Times Roman

) s   6 T  | x???? | Secondary Typeface----- Letter Gothic

) s   8 T  | x???? | Secondary Typeface----- Prestige

) s  11 T  | x???? | Secondary Typeface----- Presentation

) s  17 T  | x???? | Secondary Typeface----- Optima

) s  18 T  | x???? | Secondary Typeface----- ITC Garamond

) s  19 T  | x???? | Secondary Typeface----- Cooper Black

) s  20 T  | x???? | Secondary Typeface----- Coronet Bold

) s  21 T  | x???? | Secondary Typeface----- Broadway

) s  22 T  | x???? | Secondary Typeface----- Bodini Condensed

) s  23 T  | x???? | Secondary Typeface----- Century Schoolbook

) s  24 T  | x???? | Secondary Typeface----- University Roman

( s   # W  | 2???? | Soft Font Creation----- Download Characters: Bytes = #

) s   # W  | 2???? | Soft Font Creation----- Create Font Header: Bytes = #

* c   # E  | 2???? | Soft Font Creation----- Character Code (Decimal) ASCII = #

& p   # X  | 2???? | Transparent Print Data- Bytes = #

& s   0 C  | 2???? | Wrap-around Mode------- Enable

& s   1 C  | 2???? | Wrap-around Mode------- Disable

;-----------------------------------------------------------------------------

;

;

;------------------------------------------------------------------------------

;

;  History of PCLxxx.ARC

;  ---------------------

;

;  Original release 22 December 1988.   L.S.Mundy.

;

;  Please add any additions or corrections to this file and redistribute

;  using the naming convention specified above through the Bulletin Board

;  network of your choice.   Enjoy!.

;

;

;

;

;

;

;

;

;

;

;

;

;-----------------------------------------------------------------------------