Time Crafter Shareware

Copyright (c) 1994 Aurora Systems Research

All rights Reserved

OVERVIEW

Time Crafter is a Shareware product and is available to you

free of charge for a period of 30 days.  If you find this

product useful PLEASE register it.  This entitles you to a free

copy of the latest version of Time Crafter without the

annoying about box, notification of updates, and encourages

the author to continue producing new windows engineering

shareware.

Time Crafter is a timing diagram documentation tool.  A

timing diagram is used by engineers and technicians to

document the way a circuit or system operates or should

operate.  This type of documentation is crucial to good design

and debugging but up to now one could only use paper and

pencil (with a good eraser) or an expensive CAD package

costing $1000 or more to produce these diagrams on a PC.

It was our idea to produce an inexpensive ($15 bucks is a

movie and popcorn for two) program to do timing diagrams

so that every one who needs it can have a copy on their PC.

Try it out for 30 days we think you will find it useful.

ASR is planning to release two more versions of this program

adding features until Time Crafter is full featured CAD tool.

Time Crafter Shareware consists of four files:

TIMEINST.DOC   		Installation and User's Guide (This File)

TIMEORDR.DOC		Registration Order Form

TIMECRFT.EXE		Time Crafter Program

TIMECRFT.TC		Sample Timing Diagram

Time Crafter is easy to install.  Just copy the Time Crafter

files into thier own subdirectory (i. e. C:\TIME).  Then install

Time Crafter into a group menu of the Program Manger.  Just choose

'New' of the 'File' menu in Program Manager and enter the program

path and name under the 'Command line' entry.

To register just send $15 to :

Aurora Systems Research

PO BOX 593

Long Beach Ms 39560

For a full users manual with tutorial plus the lastest version

of Time Crafter send in $30 (See the file TIMEORDR.DOC for more

details).

Print the form in the TIMEORDR.DOC either from notepad

or DOS and send it in.  Be sure to include your name and

return address if you want to receive the diskette with Time

Crafter Professional.

USING TIME CRAFTER

There are basically three types of objects that the user can

create.

1.	Clock - This is a periodic signal such as the clock that

runs the computer you are now looking at.  It has a name,

a basic frequency of oscillation and a period.

The period of a clock is = 1/clock frequency and updates

when you change either the period or frequency field.

The offset is the time to the first clock rising edge from

zero.

The rising and falling jitter describes areas where it is not

known if the clocks edge has risen or fallen.

Duty Cycle describes what percentage of the time the

signal is high.

You can select either KHz, MHz, or GHz for the

frequency field which also selects either microseconds

(us), nanoseconds (ns) or picoseconds (ps) as the units

for the other fields.

2.	Signal - This is user defined signal.  When you first place

a signal you are prompted for the initial value for the

signal.  It can be one of the following.

High - Signal is at a logical high state usually +5V.

Low - Signal is at a logical low state usually 0V.

Tri-State - Signal is floating and is in a high impedance

state.

Valid - There is valid data on a bus or group of lines.

Invalid - Invalid data exists on a bus or group of lines.

The user can then add edges to the signal (100 edges max)

by clicking the left mouse button at the time he want's the

edge to occur at the same horizontal position. (Just move

horizontally left from the last edge and hold the left mouse

button down to move the edge.  Release the left button to

place the edge).

The user can move a placed edge by clicking and holding

the left button near the edge he want's to move and moving

the mouse left or right.

The user can change the value of an edge by single

clicking the right button on an edge.  Subsequent right

clicks will toggle through the five types of edges/signals.

3.	Text -  When text is selected a text dialog box will appear

prompting the user for the text to be displayed.  If no text

is entered or the delete button is pressed the item will be

deleted.  To Move text just click and hold the left mouse

button on the text.  If you now move the mouse the text

moves with you.  You can move text to any position using

this technique.  Double clicking on the text allows you to

edit or delete it.

To Create one of these items you can either click on it's menu

bar button, or select insert from the signal menu.  The signal

menu can also be used to delete, move or edit existing

signals.  Just select the signal by clicking on the left mouse

button over the signal's name and chose the corresponding

command from the signal menu.  Double clicking the left

button on a signal's name allows you to edit a signal.

To see more or less signal you can zoom in or zoom out using

the menu bar buttons or the zoom in or out command of the

view menu.  Zooming in or out changes the amount of

information displayed on the screen and amount displayed

when printing.

The user can use the File menu to save or load timing

diagrams.  Load the sample timing diagram TIME.TC to get an

idea of the Time Crafter's capabilites. The user can also

print using the File menu.  Note that the printout only spans

one page and uses the current zoom scale factor and start time

when printing.  If you want to print a certain time range be

sure to set you starting time (use the horizontal scroll bar on

the bottom of the window) and the zoom factor before printing.

There are some limitations to version 1.0.  Most applications

probably will not be affected by these.  Some limitations will be

corrected/updated in version 2.0.

Time Crafter 1.0 Limitations

1.	You can only have 10 clocks.

2.	You can only have 10 Signals.

3.	You can only have 100 edges in a signal.

4.	You can only have 20 Text Strings

5.	You can't scroll signals on and off the screen vertically.

6.      You can't copy to the clipboard (unless you use the print

screen function)

The next version of time crafter will have the following:

Version 2.0 (~second quarter 94)

1.	More clocks, signals and text.

2.	Vertical scroll for signals.

3.	User selectable fonts for text.

4.	Formatted Clipboard save.

5.	Slanted rise and fall line option.

Version 3.0(~first quarter 95)

1.      Constraints

2.      Help Index

SHAREWARE INFO

Time Crafter Shareware is copyrighted and released as SHAREWARE.

Operators of bullentin boards are encouraged to post Time Crafter

Shareware for downloading by their users.  It's also Ok to share

a copy of Time Crafter Shareware with a friend down the hall. ASR

only asks that those copying the program not delete, modify, or

separate the program file, documentation files or copyright notice.

These files are listed at the beginning of this document. No other

person or business (other than Aurora Systems Research) may accept

payment or royalties on the program.  Distributors of user-supported

shareware libraries or public domain programs must obtain written

permission to distribute copies of Time Crafter.

LIMITED WARRANTY

THIS PROGRAM AND THE ACCOMPANYING DOCUMENTATION IS PROVIDED "AS IS"

WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESSED OR IMPLIED,

INCLUDING BUT NOT LIMITED TO THE IMPLIED WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT

TO DEFECTS IN THE PROGRAM OR DOCUMENTATION.  AURORA SYSTEMS RESEARCH

ASSUMES NO RISK AS TO QUALITY AND PERFORMANCE OF THE PROGRAM.  SHOULD

THE PROGRAM PROVE DEFECTIVE, YOU ASSUME THE ENTIRE COST OF ANY LOSS OF

PROFIT OR ANY OTHER COMMERCIAL DAMAGE.  SOME STATES DO NOT ALLOW THE

EXCLUSION OF IMPLIED WARRANTIES SO THE ABOVE EXCLUSION MAY NOT APPLY

TO YOU.  THIS STATEMENT SHALL BE CONSTRUED, INTERPRETED, AND GOVERNED

BY THE LAWS OF THE STATE OF MISSISSIPPI.