NET-ADR

================

A program to manage and observe ethernetaddresses

Christel Veltrup

Forschungszentrum Informationstechnik GmbH

Sankt Augustin

September 1993

------------------------------------------------------------------------------

NET-ADR is a program that helps managing a list of ethernetaddresses by

watching and observing the traffic in order to hold the list up to date and

to find any changes.

It belongs to the concept of Net-Recording. Net-Recording means to store

and present all information that might be essential for controlling the

network behavior.

One of it's tasks is to detect and mark new stations on the network.

The program NET-ADR will help finding out those new stations and their

names by filtering information from the packet headers.

The list created by NET-ADR includes:

- Ethernetaddress

- Protocols used

- Name of the ethernetstation

- protocol specific information like

IP-address when IP is used,

IPX-address or IPX-server name for the IPX-Protocol,

full AppleTalk name when AppleTalk Protocol is used

With these informations it is possible to find out the following:

- a computer uses new or different protocols

- a different IP-address

- a new name

While watching the frames on the net, the program gathers statistics on

how often each of the known protocols is seen and the program can search

for IP-addresses multiple used by different computers.

Furthermore NET-ADR includes a program that sends ARPs (Address Resolve

Protocol) to specified IP-addresses and allows you to compare the actual used

IP-address to the information from the addresslist created by the other parts

of the program.

Requirements:

- IBM compatible (286 or higher) system,

DOS operating system,

- RS232 or parallel interface connected to an Ethernet Box or

an ethernet board with packet driver interface.

The Ethernet Box is used as an ethernet statistic probe for Net-Recording.

It gathers all statistical data and measures load, errors and collisions

by hardware.

The Ethernet Box is a useful hardware tool for network managemant but here

in the context of NET-ADR it is only used to receive ethernet frames.

If you don't have an Ethernet Box you can run the program NET-ADR with

a packet driver. The packet driver interface must support extended

functionality to be set into promiscous mode and its interruot vector

has to be sited between numbers 60h and 80h.

The programs listed below are necessary to run NET-ADR correctly:

net-adr.exe    the main program

adrsamm.exe    gather addresses

adr_upd.exe    saving new addresses to the list

ip_check.exe   searching for IP-addresses used by different computers

arp.bat        send ARPs to a number of IP-addresses and

compare results of ARPs to the address list

vndir.exe      view a file

mov-000.exe    numbering files

eak-term.exe   terminal program for the Ethernet Box

rpsort.com     a sort program

How to use NET-ADR ?

I hope the program is self-explaining but nevertheless a few words:

When the program NET-ADR is started (by typing 'net-adr' on the commandline),

first of all you have to select the port or the packetdriver.

COM (1 or 2) means a serial port, LPT (1 or 2) the parallel port and PktDrvr

the packetdriver.

You can specify the port on the commandline as well, just type COM (1 or 2),

LPT or PKT.

The Main Menu comes up if the selected port was initialized succesfully.

To use the ARP-Test program, you have to edit the file 'arp.bat' before and

change the IP-address that's written after 'arptest' to your own address.

A short explanation of the menu items follows:

* Address *

- Collect Adresses

Reads frames in order to find as many different ethernet-addresses as

possible. Pressing any key stops it.

The addresses that were read are compared to those which are already stored

in the addresslist so only completely new addresses or addresses with

any difference to the list are written to a file.

There are two files that are created each time the program reads any frames.

First there is adr_diff.XXX (where XXX is an number), this file contains all

addresses which have a known name. The other file created is adr_unb.XXX.

All completely unknown addresses are written to this file.

The files that were created most recently are those with the highest numbers

following the name.

- Collect with Request

The same as above with the only difference that an AppleTalk Name Binding

Protocol request block is sent every 20 seconds in order to get some more

names of computers using AppleTalk.

The request-frame is set by default to get all Appletalk-frames with

type = NBP (Name Binding Protocol) no matter what type or zone they have.

Possible types that may be seen are BroadCast, AFPServer, LaserWriter and many

others.

You can change these settings by typing anything else there.

For example you may want to get only names that are used together with

BroadCasts, or the names of LaserWriters or only names that belong to a

special AppleTalk zone.

Of course it makes no sense to write any unknown or silly names there, but I

think that's obvious.

Which settings should be used to get the best answers or whatever, I don't

know, you have to try it out.

If you use an Ethernet Box, it may occur that changing the request-frame

failes. In this case it is possible that the Ethernet Box requires a password

afterwords. You can use the program eak-term to reset the password. The

program NET-ADR uses 'h' as password, so you can reset the password by typing

'h' as old password and setting new password to nothing.

- Update Addresslist

The addresses that were written to 'adr_diff.XXX' are read and displayed on

the screen, so you can decide whether they shall be written to the addresslist

or not.

Apart from writing or not writing the new informations there is the

possibility to edit the old entry.

This is how you can update the name or the IP-address, or you can delete a

whole entry by typing 'N' for each protocol.

When you're through with all the addresses you can delete the files

'adr_diff.*' and 'adr_unb.*'.

* View *

- Differences to addresslist   = adr_diff.*

- Unknown addresses            = adr_unb.*

- Addresslist                  = adressen.all

- Multiple used IP-addresses   = ip_diff

* Others *

- Protocols

Displays the statistics of protocols that was seen when 'Collect Addresses'

was called the last time.

- Check IP

Searching for IP-addresses that are used by more than one computer.

If there are any, they are displayed and written to the file 'ip_diff.'

- ARP - Test

The program asks for two IP-Adresses (in the form e.g. 1.2.3.4) which specify

a start and an end address. Every single IP-address between those two is then

used to send an ARP.

The number of IP-Addresses between start and end address must be less than

2048.

The bottom line tells how many unanswered ARPs are still there.

The sending and receiving of ARP's stops either if all ARPs are answered or a

key is pressed.

The results of sending the ARPs are then compared to the mapping of

IP-addresses to ethernetaddresses from the addresslist (adressen.all) and the

file 'arp-ip.dat' is created to save the addresses.

- List ARP-Replies

View the file arp-ip.dat.

The file is sorted by IP-addresses. It includes the IP-address, the

ethernetaddress and the result of comparing to the addresslist.

(no entry)    means there is no information for this ethernetaddress in the list.

(no IP-entry) means to this ethernetaddress there is only information of other

protocols in the list.

(IP-Address)  e.g. 01020304 that's the address which is stored as the IP-

address belonging to this ethernetaddress

* Terminal *

This program can only be called when you're connected via COM or LPT to an

Ethernet Box. If you use an Ethernet Box you should be familiar with this

program so I don't describe it here.

NET-ADR uses the following files:

adressen.sdf:

The file 'adressen.sdf' is a product of the program NET-CONTROL which is

a tool for collecting and controlling data in an Ethernet LAN.

It is used by NET-ADR to find the name belonging to an ethernetaddress.

If this file doesn't exist, you can write it, or run the program

without it. It should work correctly, I hope.

An entry in this file (one line) has to be of the following form:

ethernetaddress(12 characters)  blank   name(15 characters) ...

Any other information following the name will be ignored by the program.

The file has to be sorted by ethernetaddresses in ascending order!

The filename 'adressen.sdf' can be overwritten by typing another filename

on the commandline when starting NET-ADR.

net-adr (port or PKT) [your file]

adressen.set:

The file 'adressen.set' includes the mapping of name of the protocol to a

number between 1 and 9.

The programm NET-ADR recognizes the following protocols:

Name    typefield (Bytes 13-14 of the ethernetframe)

IP       0800

Dec      60xx

Apple    809B  or

80F3  or

length (< 0600 (hex)) followed by "AAAA"

Novell   8137  or

8138  or

length (s.o.) followed by "FFFF"

LanMngr  length (s.o.) followed by "F0F0" oder "F0F1"

GH       4748  or

564E

If this file does not exist when you run the program an initial screen is

displayed where you have to do this mapping. It will then be written to the

file 'adressen.set'.

Apart from the protocols mentioned above you can give a number for routers and

for the rest that means all protocols that the program doesn't know.

The numbers have to be between 1 and 9 and you can give the same number to

more than one protocol if you want to.

adressen.all:

'adressen.all' is the list of addresses and protocols created and used by the

program. If the file does not exist it will be created the first time you call

'Update addresslist'.

It contains

- ethernetaddress

- name

- the protocols used by this computer

- special protocol information like

* IP-address when IP is used,

* IPX-address or IPX-server name for the IPX-Protocol,

* full AppleTalk name when AppleTalk Name Binding Protocol is used

The different protocols are represented by digits (see 'adressen.set' for more

explanation).

Example:

02608C8C87D6           fiery   3 Fiery:LaserWriter:*

02608CA3EB33         novell2   4 00000000_02608CA3EB33_0455

080005034022          sphinx   1 811A0929

080020113B0C             zsv   1 811A086E

080020113B0C             zsv   3 Zsv:AFPServer:BackboneET

08002011BBB2           jumbo   1 811A0864

08002011BBB2           jumbo   3 jumbo:UNIX/CAP:BI_admin

Here 1 stands for IP, 3 for AppleTalk Protocol and 4 is IPX-protocol

The file 'adressen.all' will not be updated each time you call 'Collect

addresses'. Only if you select 'Update Addresslist' from the Main Menu you can

write new addresses to this file.

router.adr:

If the file 'router.adr' does not exist it will be created by the program (if

needed).

It contains all addresses and names of routers. There can be up to 40

addresses in this file.

What's so special about routers? Hm, here the only important thing is that a

router can use many different IP-addresses and that's ok. So if you don't want

a message every time the router uses another address you have to put its

ethernetaddress into the file.

This can be done by the program using 'Update Addresslist' and typing

'y' for router.

Output-files generated by NET-ADR:

adr_diff.* and adr_unb.*:

When the program stops collecting adresses (maximum number of addresses is 250

or you can stop ist by pressing any key), all addresses that were read are

compared to the information that's already stored in 'adressen.all'.

Only those addresses, that

- use a different protocol

or

- use a new IP-address, IPX-address or different name (only AppleTalk)

or

- are not found in 'adressen.all'

or

- are not found in 'adressen.sdf' (means, there's no name known)

are written to the files 'adr_diff' or 'adr_unb'.

The addresses that are completely unknown (no name found) are written to

'adr_unb' all others to 'adr_diff'.

protok.tmp :

This file contains the statistic which you can see by calling 'Protocols' from

the Main Menu.

Everytime 'Collect' is called this file is updated.

ip_diff :

NET-ADR creates this file when the item 'Check IP' was called from the Main

Menu. It includes all ethernetaddresses and names of computers using the same

IP-addresses.

------------------------------------------------------------------------------

The program NET-ADR is free to use for anybody who likes to try it.

I don't know if it's really free from bugs, so if you find out any or if you

have comments ore something else, please email me:

Christel.Veltrup@gmd.de

Christel Veltrup

February '94

------------------------------------------------------------------------------

For more information on the Ethernet Box and the program NET-CONTROL

please contact

RzK D.Koepke

Bitzenstr. 11

D-53567 Asbach Ww

Tel. 49 2683 4547

Fax  49 2683 4537

------------------------------------------------------------------------------

Packet Drivers (the crynwr packet driver collection) are available via ftp

at oak.oakland.edu, file pktd11.zip in the directory pub/msdos/pktdrv

#