NAME

odipkt - translate Open Data-Link Interface to Packet Driver

interface

SYNOPSIS

odipkt [ logical_board [ vector ] ]

DESCRIPTION

Odipkt.com provides a Packet Driver API over  Novell's  Open

Data-Link  Interface,  allowing  a single network adapter to

service IPX, tcp/ip, and other  protocol  stacks  which  use

packet  drivers.   Odipkt.com  understands  Ethernet,  Token

Ring, and ARCnet  frame  types.   Ethernet  and  Token  Ring

frames  are  passed unchanged to packet driver applications,

but ARCnet frames are internally mapped to Ethernet.   Thus,

packet driver applications do not need specific knowledge of

ARCnet in order to operate in such environments.

The logical_board argument indicates which  logical  adapter

odipkt  should  bind  to.   Under  ODI, there is one logical

adapter per frame type.   There  may  be  multiple  physical

adapters and multiple frame types per physical adapter.  The

logical adapter number starts at zero and increases  by  one

for each frame type.  For example, if a system contained two

physical adapters with two frame types each, there would  be

four  logical  adapters  numbered  0-3.  The frame types are

specified in the NET.CFG file which must be in  the  current

directory  (or  on the APPEND path) when the ODI drivers are

loaded.

Typically, the frame type used by default by  Netware  on  a

given  adapter  will  not  be  appropriate  for tcp/ip.  The

correct frame type  for  tcp/ip  on  Token  Ring  is  TOKEN-

RING_SNAP  and the correct frame type for tcp/ip on Ethernet

is almost always ETHERNET_II.  Assuming an existing  instal-

lation  with  one  adapter running the default Netware frame

type, it will usually be necessary to define a second  frame

type  (as  listed above) and specify the logical_board argu-

ment as 1.  An exception to this rule is ARCnet  which  uses

only  one  frame type.  Below is a NET.CFG entry for a 3c501

using the default address and  interrupt.   In  many  cases,

this  is the only entry required in NET.CFG.  Other Ethernet

adapters would use similar configurations:

Link driver 3c501

envelope type ETHERNET_802.3

envelope type ETHERNET_II

Given the above entry, the logical board argument  of odipkt

would be 1.  Token Ring uses different frame types:

Link driver token

envelope type TOKEN-RING

envelope type TOKEN-RING_SNAP

Again, the logical_board should be specified as 1  for  this

example.  ARCnet installations require no frame-type defini-

tions and should use a logical_board of 0.

Vector is the packet driver interrupt at  which  to  provide

service.  The default is 105 and the parameter is decimal.

Odipkt.com should be loaded after lsl.com and  the  adapter-

specific  driver(s).   It  must, of course, be loaded before

the packet driver applications which use  it.   The  Netware

shell  and  ipxodi.com  can  be  loaded  before or after the

odipkt.com program.

AUTHOR

Dan Lanciani

185 Atlantic Road

Gloucester, MA 01930