Sinclair ZX Spectrum Emulator 'Z80' v2.01  -  3/5/93  -  by G.A. Lunter

��������������������������������������������������������������������

Z80 - Frequently Asked Questions

1 - How to print the diagram of the tape-interface

as shown in DIAGRAMS.Z80, or make other screendumps.

2 - How to LLIST and LPRINT to a printer

3 - How to build the tape-interface

4 - What to do if the Zandbergen joystick doesn't work correctly

5 - How to get other emulators or spectrum programs

6 - How to contact the author (new email adress)

7 - Manic Miner and Jetset Willy run too fast!

8 - How to save a .SCR snapshot to tape or .TAP file

��������������������������������������������������������������������

ad 1. How to print a screendump

You can't use COPY (the 48K version at least) to make a screendump.

COPY is intended only for the original ZX-printer. The design of the

ZX-printer is completely different from printers nowadays. However,

there are two other ways to make screendumps.

When the Interface 1 appeared, it was possible to attach a serial

printer to the RS232 port of the Interface 1. The emulator can

redirect the RS232 to the serial port of the PC, a file or a

parallel (printer) port.

The SamRam can make a screendump on a dot-matrix printer. It is

programmed for a Star SG-10. If you have a different printer, there

is two other ways to print screendumps. See below.

Example 1: (Screendump for Star SG-10 compatible matrix printer)

Redirect the Spectrum Interface 1 RS232 output to LPT1 by

pressing F4, O, L, 1.

Load a .Z80 file, and wait for the picture you want to print

to appear.

Switch to SamRam mode by pressing F10, H, 3, CTRL-ENTER, and

generate an NMI by pressing X, N.

Press P, then 1 for printing the screen.

Example 2: (Screendump for EPSON compatible matrix printer)

Redirect the RS232 output to LPT1 by pressing F4, O, L, 1.

Load a .Z80 file, and wait for the picture you want to print

to appear.

Save it as a .SCR screendump by pressing F10, X, S, name

Switch to Spectrum 128K mode (F9, 5, ENTER)

Choose '128K BASIC' from the menu, and type PAUSE 0:COPY,

and press ENTER once.

Press F10, X, L, name, to load the screen snapshot back into

memory.

Go back to the emulator (ESC, ESC) and press any key to start

printing.

Example 3: (Screendump for all other printers)

Follow the steps from example 2 up to the saving of the .SCR

screen snapshot.

Leave the emulator.

Run CONVERT on the .SCR file to generate a .GIF or a .PCX file,

for example

CONVERT b g screen.scr screen.gif,   or

CONVERT b x screen.scr screen.pcx

Find another program that can print the .GIF or .PCX file

(Windows for instance can handle .PCX files)

��������������������������������������������������������������������

ad 2. How to LLIST and LPRINT to a printer

As told above, you have to redirect the output of the Interface 1

RS232. In the emulator, press F4, O, L, 1.  Like on the real spectrum

with Interface 1, if you want to print to the Interface 1 RS232, you

have to open the #3 channel.  In Spectrum Basic, type: OPEN #3,"b" or

OPEN #3,"t" for basic listings (LLIST).  (Press SHIFT-ALT-4 for

OPEN #)

In this way you can print anything as long as your spectrum program

supports printing to the Interface 1 RS232 and your model printer. For

example, it is perfecty possible to print a textfile on your printer

with Tasword 3, or make a printout with Art Studio.

�������������������������������������������������������������������

ad 3. How to build the tape-interface.

Two different diagrams for the tape-interface are shown in

DIAGRAMS.Z80, along with a partlist.  The second diagram is much

easier to build than the first diagram, and works as good and

sometimes even better.  Some parallel ports don't work correctly with

the first diagram. The easier, second diagram should work in all

cases.

It is not possible to order the tape interface through the author of

the spectrum emulator. If you are unable to build one yourself, try

asking a friend. The diagram is very simple and should not be to hard

to build, not even for an unexperienced person.

�������������������������������������������������������������������

ad 4. What to do if the Zandbergen joystick doesn't work correctly

On a fully implemented pc-joystick port, two analog joysticks can be

attached (on one plug!).  A number of multi-io cards can handle only

one analog joystick.  However, the Zandbergen joystick needs a fully

implemented joystick port. If only two directions (out of four) work,

your joystick port is not usable.  Use an analog pc-joystick instead.

The joystick port of a Sounblaster is fully implemented and ok to

use.

If you have more than one joystick port in your pc, you have to

disable one.

�������������������������������������������������������������������

ad 5. How to get other emulators or spectrum programs

You can tranfer your own spectrum programs to the pc by using the

tape-interface.  I'm sorry, but I can't send you disks filled with

spectrum programs.  If you want other spectrum emulators (for pc,

atari, amiga), programs that emulate other computers or more spectrum

stuff, call the Tatort BBS:

Tatort BBS Groningen

+31-50-264840

v22, v22bis, v32, v32bis, MNP2-5, v42, v42bis (300-14400)

At the time of writing 24 hours a day, but subject to change.  Please

try calling between 22:00 and 9:00 first.

Also, if you have internet access, SIMTEL 20 mirrors have a directory

emulators (nic.funet.fi, wuarchive.wustl.edu)

�������������������������������������������������������������������

ad 6. How to contact the author.

You can contact Gerton Lunter by email or normal post:

Internet email: gerton@rcondw.rug.nl

Post adress:

Gerton Lunter

P.O. Box 2535

9704 CM  Groningen

The Netherlands,

or leave a message on the bbs.

�������������������������������������������������������������������

ad 7. Manic Miner and Jetset Willy run too fast!

True. Most of the time these games are either producing sound or

executing an LDIR instruction to update the screen. Since this

instruction can be emulated with a single REP MOVSB instruction of

the 8086, emulation is suddenly very fast. Without sound, Manic

Miner and Jetset Willy were two of very few games that were

actually playable on a 8 MHz PC, and of course I don't want to slow

down the emulator on slow machines!

If you turn the LDIR emulation switch on, the LDIR instruction will

behave somewhat differently, important for well-protected programs.

As a side effect, it will then also run at the right speed.

�������������������������������������������������������������������

ad 8. How to save a .SCR snapshot to tape or .TAP file

First make the appropriate choises in the tape menu, F7.  Then type

SAVE "name" SCREEN$.  Press ENTER, and then press F10 in the one second

gap between header and data block.  When saving to tape you can easily

see it; when saving to disk, after pressing ENTER give the emulator a

split second to save the header to disk before pressing F10.

Now load the .SCR snapshot into memory, and continue saving.