SimpTerm - A Simple Amateur Radio

Communications Program

Jim Lynch

October 27, 1993

Contents

1  Introduction                                               2

2  Quick start                                                3

3  Display description                                        4

3.1  Receive window  : : : : : : : : : : : : : : : : : : : 4

3.2  Status line : : : : : : : : : : : : : : : : : : : : : 4

3.3  Transmit window  : : : : : : : : : : : : : : : : : :  4

4  The Initialization file                                    5

4.1  Keyboard assignments  : : : : : : : : : : : : : : : : 5

4.1.1  Keyboard macros : : : : : : : : : : : : : : :  5

4.1.2  Assigning files to function keys  : : : : : :  7

4.1.3  Assigning functions to function keys  : : : :  7

4.2  Definition variables  : : : : : : : : : : : : : : : :10

4.3  Color Configuration   : : : : : : : : : : : : : : : :18

4.4  Command line options  : : : : : : : : : : : : : : : :18

5  Station logbook                                           19

6  Selective calling feature                                 20

7  License                                                   21

8  Trouble shooting                                          21

9  Error messages                                            21

10 Final notes                                               23

1

11 Bugs                                                      23

12 Futures                                                   23

1    Introduction

SimpTerm is a simple terminal program designed to be used with

almost any tnc or tu on the market. Features of SimpTerm are:

o Split window operation.

o Macro key definitions.

o User customizable Help screen

o Most of the non-ascii keys can be used as function keys

o Optional scroll back feature on the receive window and trans-

mit windows

o Simple status display in the middle of the screen

o Capturing of data to a disk file

o Access DOS commands without dropping communications

connection

o Control of the com port definitions from command line, init

file and keyboard.

o Works on 8088 as well as 80486 and everything in-between.

o Status line

o Small enough to work well on resource tight platforms, like

laptops.

o Selcal functions, limited unattended operation.

o Times can be in GMT or local time.

o A station logging function.

o User selectable color scheme

o Function keys and control keys can be assigned to a macro

string, cause a file to be uploaded or call a function within

the program.

2

There are loads of programs that are tailored to specific tnc's and

there are dozens of good terminal programs that can control any

tnc in a "dumb" (that's a technical term) manner. This program is

an attempt to provide the average ham with something in-between.

It has a lot of features, but not so many as to make use of the pro-

gram or the tnc hard.  It requires you to know the commands of

your tnc, but also gives you the flexibility that only direct commu-

nications with a tnc can provide.

Statistics:

Author:   Jim Lynch

Call: K4GVO

Address:   128 Carrollwood Dr. Fayetteville, GA 30214

internet address:   jwl@cray.com

packet address:   k4gvo@wgabbs.ga.usa or check the white pages

If you write, please enclose a sase.  If you don't I reserve the

right not to answer.

SimpTerm was developed with:

o Turbo C 2.0

o C/Windows Toolchest, from Mix Software

It was developed on a Toshiba T1000SE, but runs on my 386sx

with a VGA color monitor just fine.

2    Quick  start

Edit the simp.ini file and change all references from K0QWG to

your call. Make sure the line that starts with "COM" looks ok for

your setup. If you have any problems understanding that line, see

the discussion on the COM directive later in this document.

Now execute the program. You will see an intro screen followed

shortly by a message instructing you to hit any key.  Do so and

you should see the screen split into two windows with a status line

between them. If you press enter, you will should see the prompt

from your TNC appear in the top window.  If you don't, then a

number of things could be wrong. See the trouble shooting section

of the manual for additional help.

3

Now press the F1 key.  You should see a new window appear

listing some of the function keys that are currently assigned to

functions and what those functions do. Press any key to close the

help window.  Press the F3 key.  You should have a screen that

displays small windows of different colors with a number assigned

to each. Also you should see a menu near the bottom center of the

screen.

The menu is used to change the colors of the receive, transmit,

status and logging screens.  See the section on configuration for

details.  Press ESC to return the normal display if you like the

default colors.

Enter your tnc commands as you would normally do with a

dumb terminal.  If you have the echo turned on, you will see the

commands as you type on both screens.

Take a look at the simp.ini file and read the section on the

initialization file details. You should be able to customize this file

fairly easily.

3    Display  description

There are three displays on the main screen. These are the trans-

mit, receive and status windows.

3.1    Receive window

This is the top window. Data coming back from the RS-232 port

will be displayed here. That is usually the output from the other

station. If you have the echo turned on you will also see the com-

mands that you typed.

3.2    Status line

The screen is divided into two separate windows with a status line

between them.  This status line contains information about the

current configuration of SimpTerm, the status of the selcal features

and other interesting information.

3.3    Transmit window

This is the bottom window.  It displays what you are typing and

some of the error messages reported by SimpTerm.

4

4    The  Initialization  file

The initialize file (init file) is used to load the keyboard macros,

assign keys to files,  specify initial com port settings,  specify a

"preload" file for customizing your tnc each time you start SimpTerm

and to define a number of other options. The default init file name

is simp.ini.  This file must be in the directory from which you

started SimpTerm.

4.1    Keyboard assignments

Keys can be assigned to macros, functions or be used to initiate

file uploads.

A full listing of the keys that can be assigned are described in

the keys.doc file found in this distribution. They take the form of:

KEY_{modifier_}keyname

The modifier may or may not be necessary, hence the curly braces,

meaning the stuff inside the braces isn't necessary, but might be

important. The modifier is one of the following:

SHIFT    (or SHFT in one case) designates the key must be de-

pressed with the shift key held down.

CTRL    designates the key must be depressed with the control key

held down.

ALT   designates the key must be depressed with the alt key held

down.

A few examples of valid keys are:

o KEY_ALT-X

o KEY_CTRL_P

o KEY_F10

o KEY_SHFT_TAB (this is the only SHFT, the rest are SHIFT)

o KEY_SHIFT_F4

4.1.1   Keyboard macros

Keyboard macros are strings that can be assigned to many of the

keys on your keyboard. This lets you assign frequently used char-

acter strings to a single key to save keystrokes. For instance, you

5

might want to assign your call to the F1 key and your name to the

F2 key and your QTH to the F3 key.  That way when you were

talking to someone (with your fingers, that is) you would just type

"Hi, this is <F1>, <F2> in <F3>" and your computer would fill

in the right stuff. <F1> in the above example means that you hit

the F1 key, not that you typed the string "<F1>". Bear with me

you old timers, I want to make sure the newcomers don't give up

on us.

To define a macro you need to place the key name (yes, it must

be in upper case) in column one, followed by a space, followed by

the macro replacement string.

A simple example based on the earlier discussion would be:

o KEY_F1 K4GVO

o KEY_F2 Jim

o KEY_F3 Fayetteville, GA

Now when you punch <F1> <F2> <F3>

K4GVO JIM Fayetteville, GA

will get sent to the tnc.  I hope you were connected to someone,

'cause sending that garbage to the tnc in command mode won't be

very interesting.

The maximum length of a string that can be assigned to a key

is 511 characters.  You can continue the string on the next line

by placing a backslash (\) character at the very end of a line and

continuing with the definition on the next line.  Be careful not to

add a space after the \ because you won't be able to see it and the

program will think the next line is a new macro definition.  Note

that there will be no line breaks sent at the end of the line.

Example:

KEY_ALT_C cq cq cq cq cq cq de k4gvo $"Backslash$

cq cq cq cq cq cq de k4gvo $"Backslash$

cq cq cq de k4gvo k

This is an example of a complete definition. Note that the first

two lines have the backslash but the last one doesn't. This would

be ok for a cw call, but you probably want to be able to insert a

new line occasionally for rtty, amtor or packet work.

6

You can insert any control character, including a carriage return

(that is what you normally send when you hit the enter key). Let's

try the above example with the addition of a carriage return.

Example:

KEY_ALT_C cq cq cq cq cq cq de k4gvo^M$"Backslash$

cq cq cq cq cq cq de k4gvo^M$"Backslash$

cq cq cq de k4gvo k^M

Now we have a reasonably constructed macro that won't offend the

station on the other end.

A control character is formed by preceding a letter with a caret

as shown above.  The control characters ctrl-a through ctrl-z are

formed as ^A through ^Z. If you want to include the caret as a

character to be send, use a double caret (^^).  Since there is no

such thing as a control-caret, the program will know to send a single

caret.  You can also specify other special characters and strings.

See the section on selective call reply strings for an expanded set

of options within the macro string.

4.1.2   Assigning files to function keys

A function key can be assigned to a file by preceding the key name

with a ampersand (@). The name of the file follows the key name.

Hence:

@KEY_ALT_U mydefs.txt

@KEY_HOME c:$"Backslash$bob$"Backslash$mydefs.txt

are valid definitions. When the specified key is depressed, the file

is uploaded to the tnc. This means that the contents of the file are

sent to the tnc.  This is handy for defining a set of parameters to

customize your tnc for a specific mode of operation. You could use

the macros to do this, but they are limited to only 511 characters.

Another use for this is the "brag tape" (the word tape left over from

when we used paper tape on the old *eletype machines). You could

put a description of your station, location, personal information on

a file and by hitting a single key, you can send it to the other

station.

4.1.3   Assigning functions to function keys

That sounds like a redundant statement doesn't it? It's not really.

There are six functions available in version 1.0 that can be assigned

to function keys. These are:

7

quit        exit the program. Not a bad function to have.

togcap      toggle logging of the incoming data.

help        display the help screen

dos         jump to dos

status      print status of selcal

togsel      toggle looking for selcal strings

logit       brings up a data entry form for the station log

config      configure the colors of the various parts of the

screen

reset       reset the communications port

rforward    scroll the receive window forward one page

xforward    scroll the transmit window forward one page

rforward1   scroll the receive window forward one line

xforward1   scroll the transmit window forward one line

rback       scroll the receive window back one page

xback       scroll the transmit window back one page

rback1      scroll the receive window back one line

xback1      scroll the transmit window back one line

baudup      increment the communications baud rate

bauddown    decrement the communications baud rate

rlarger     make the receive window one line larger

rsmaller    make the receive window one line smaller

macro       assign a macro to a key on the fly

upload      upload a file, prompts for name

confcom     bring up a menu for com parameter selection

The togcap function will either ask for the name of a capture

file to start saving data to or close that file if capturing had already

been started by a previous call to togcap. The default capture file

name is simp.cap. You may override that name by typing in a new

name over top of the simp.cap entry. While you are capturing, the

name of the capture file is displayed on the status line. When you

are not capturing, the status lines displays "Capture off".

The help function is a rather simple one. It just opens a window

on the screen and dumps the contents of the help file into the

window.  If the help file is too long, the top part of the screen is

8

scrolled off into never never land. Using a text editor of your choice

you may edit this file as you see fit. The default help file is named

simp.hlp, but you can tell SimpTerm to use another file via the

HELP customization option in the init file. See the init file section

for details.

The dos function brings up a second copy of the command

processor.  This allows you to execute commands just as if you

had exited the program, but the program is still in memory and

waiting for you to finish. After you have finished with dos and want

to return to SimpTerm, just enter "exit". That will bring you back

to SimpTerm where you left off.  The communications buffer can

hold 2048 characters. That is about one screenful. If you stay away

too long, you may lose data. Be careful.

There are 8 scrolling functions. These are:

rforward, rforward1, rback, rback1, xforward, xforward1,

xback xback1

The rforward and xforward functions cause the receive or trans-

mit window to scroll forward one page. The rforward1 and xfor-

ward1 functions cause the receive or transmit screens to scroll

forward one line.

The rback, rback1, xback and xback1 functions perform the

reverse operations respectively.

The baudup function will increase the baud rate one step. It

will cycle through the following baud rates, 300, 1200 2400, 4800

and 9600.  If you must use other rates, you will have to set them

via the COM input line in the init file.

The bauddown function will decrease the baud rate one step.

It will cycle through the same baud rates as described in the pre-

vious paragraph.

The rsmaller function will reduce the size of the receive window

by one line. This has the effect of moving the status line up one line

and increasing the size of the transmit window by one line.  The

rlarger function has the reverse effect. You cannot reduce either

window below 3 lines high.

The macro function is used to define a macro and assign it to

a key without leaving the program. You are first asked to press a

key. This is the key to which the macro will be defined. I suggest

you don't use a printable key, like a letter or a number, but that is

a legal key. You can use this to assign macros to keys that aren't

on the list of valid keys as long as the program recognizes that a

key has been pressed.

9

The upload function lets you upload any file to the tnc. You

will be prompted for the file name.

The confcom function brings up a menu screen with radio

buttons to use to configure the com port.  You may find it isn't

worth much unless you have a mouse.

To assign a function key to one of these functions, precede the

definition by an exclamation point (!).

E. g.

!KEY_ALT_X quit

!KEY_CTRL_L togcap

!KEY_ALT_H help

!KEY_F10 togsel

!KEY_ALT_S status

!KEY_ALT_J dos (Jump to dos)

The simp.ini file included in the distribution assigns the page-

down and pageup keys to the rforward and rback functions respec-

tively. Also the up arrow and down arrow keys are assigned to the

rback1 and rforward1 functions.

Just in case you don't have the quit function assigned to a key,

the END key has been permanently bound to the quit function.

So you don't accidently hit it and drop out of the program, the

end key must be pressed twice in succession to end the program.

You can't assign the KEY_END key to anything else. The program

won't complain if you try, but it just won't work the way you think

it should.

There are a few other keys that are special.

KEY_BKSP and KEY_CTRL_H

are intra-line edit functions and cannot be reassigned.

KEY_ENTER

can't be reassigned either, obviously.

4.2    Definition variables

You can set a number of configuration options by defining variables

in the initialization file. This section discusses them.

Comment lines

10

If the program finds a # (pound) sign in the first column of a line

in the init file, it will stop processing that line. This is a way that

you can comment your init file. Also any of the options described

below that have a fixed format will allow comments following all

of the permitted arguments. If an option has a variable number of

arguments, you can't put a comment on that line.

E. g.

# this is a valid comment

mid 15 # this is ok also.

COM port definition option

The COM port parameters must be set to match your tnc. If you

don't know what speed, word size, etc.  that your tnc is using,

consult the manual that came with the tnc.  A lot of the packet

tnc's around have an autobaud detect feature.  When you power

them up for the first time they wait until you type an asterisk and

then they can determine the operating parameters you are using.

If you have both a COM option line in your init file and a

command line com option then the command line option will be

used. This way the command line can be used to override the init

file if desired.

If you don't tell SimpTerm what parameters to use it will default

to:

use COM1

2400 bits per second (baud)

8 data bits

No parity

1 stop bit.

If you insert the line,

COM1 4800,7,E,1

it will change the COM1 port to work with:

4800 bits per second (baud)

7 data bits

even parity

1 stop bit.

See the section on command line options for the full set of op-

tions for each of these parameter fields.  There must be exactly 4

fields.  The COM? line requires a SPACE immediately after the

COM? field.  The command line version of this command uses a

COMMA between these fields. Try not to get these confused. Of

11

course, if you have your tnc connected to one of the other com

ports(like 3), you will want to use a line that looks like:

COM3 9600,8,N,1

instead.

HELP file specification

The default HELP file name may be overridden by including the

HELP configuration line in the init file.  This is simply the key

word HELP followed by a file name.

MID screen layout

One of the feature of SimpTerm is that is has a split screen. This

allows you to have a "transmit" window that displays what you

are typing along with informative messages from the program and

a "receive" window that displays the data coming back from your

tnc. The size of the windows is determined by the mid command

line option. This value should be between 4 and 20. A value of 15

will give you a 15 line transmit window and a 9 line receive window.

If you specify a value for MID out side of this range a default will

be chosen for you.

PRELOAD file specification

There is an option available that lets you send a file to the tnc be-

fore control is turned over to you at the keyboard. The PRELOAD

option line specifies the name of this file. It follows the same form

as the HELP file spec.  This option may be used to force certain

operating parameters when starting up SimpTerm. One use of this

feature would be to keep different init files each with a different

PRELOAD file. One PRELOAD file might set the tnc to work hf,

while another might specify vhf. If you have two hams that use the

same equipment, you might want each to have his/her own custom

PRELOAD file to change, at least, the call letters.

SELFILE file specification

This line specifies the name of the file that will receive the selcal

messages.  The default is "simp.sel".  If you want to specify a

different file name and/or drive specification put the SELFILE line

in the init file.

E. g.

SELFILE a:\messages

The messages that are stored in this file are appended to any

12

previous messages, so you will not lose any previously recorded

messages but you might want to look at the size of this file occa-

sionally so you don't run yourself out of disk space.

GMT time specification line

If the program finds this line in the init file, it will attempt to

display the times in GMT. Since the time function is only used

with selective calling functions, non-selcal users can ignore this

option. In order to make these functions work right, you must set

your dos clock to your local time and set the environment variable

TZ to indicate your local time zone relative to GMT. The format

of the TZ variable is:

TZ=zzz[+/-]d[d][lll]

Where:

zzz is a 3 character string representing the name of the current

time zone. E. g. CST or EST.

[+/-  d[d]] is a optionally signed number with 1 or 2 digits.  This

number is the local time zone's difference from GMT in hours.

Positive numbers adjust westward from GMT

lllis an optional 3 character field that represents the local time

zone daylight savings time. E. g. PDT Environment variables

are either set in your autoexec.bat file, or can manually be

typed in before you execute the program. Consult your DOS

manual for more information.

Examples (DOS command line):

SET TZ=EST5EDT          (New York)

SET TZ=CST6       (Indiana, they don't used daylight time, do they?)

SET TZ=PST8PDT          (Los Angeles)

Because of the "new" rules surrounding daylight time,  you

might find that the time is an hour off during some parts of the

year.  You might want to keep your dos clock in GMT, or adjust

your TZ environment to give you the correct GMT value.

SELREPLY reply line

When a remote station sends you a specific select, i. e. a request for

you to start recording an incoming message, it will expect you to

13

reply immediately with a message so it doesn't waste time sending

you a message if your equipment isn't monitoring.  This option

specifies the reply that you will give in response to the select.

The format of this reply line can contain ascii characters, control

characters, or special formatting characters.  The ascii characters

are the normal printable characters (or subset in the case of bau-

dot or amtor). The control characters are formed by a commonly

used notation where the caret precedes a character to indicate the

control code should be sent and not the character itself.

E. g.

^m  carriage return (enter)

^j line feed

^d  Control-d

^@  Null (0)

^^  caret

Some of the control characters can be specified via the C lan-

guage backslash escape notation. These are the only ones allowed.

\t tab

\n linefeed (new line)

\r return

\\ backslash

Also included are a few time and date functions that can be

specified by a %c notation, where c is one of the following:

D  The date and time in the form dd-MMMyy hh:mm

t The time in the form hh:mm

d The date in the form dd-MMMyy

Where:

dd  is the day of the week,

MMM     is the 3 Character abbreviation of the month

yy is the 2 digit year

hh  is the hour

14

mm   is the minute

Example:

SELREPLY x\r^O^O^O^O^O^Ode K4GVO, %D\r^D

Using the PK-232, you can send the "letters" characters with

the control-O character, so to make sure that the other station

has gotten his transmitter shutoff, you might want to delay your

transmission a bit via this method.  The "x" character, followed

by the carriage return tells the PK-232 to start transmitting. The

ctrl-D at the end tells the PK-232 to stop transmitting and resume

receiving.

RSELCAL selective call string option

This line specifies the selcal string(s) that you want to react to.

You may specify up to 5 strings.  The strings can be delimited

by spaces, commas or tabs.  The strings can NOT contain any of

these characters. Since these strings should probably contain your

call or your call and some trailing characters, this shouldn't be a

problem.  These strings ARE case sensitive.  That means that if

you are running in a mode that supports upper and lower case,

you had better put your selcal in both upper and lower case and

hope that the sending station doesn't send mixed case. All strings

identified from this set of strings by the program will be replied to

with the SELREPLY string.

Examples:

RSELCAL k4gvo K4GVO k4gvo-9 kd4rqn

ALLCAL selcal option

This option specifies the set of strings that the program will

look for to start a "all call" recording. When the program finds a

string in this group, it starts recording a message without sending

a reply.  Other than that, it behaves in exactly the same manner

as the RSELCAL option.

Example: ALLCAL ANYONELISTENING TOALLOFYOU allcal

Note:  as far as I can determine the PK-232 doesn't transmit

the figures or letters characters (baudot) to the program so there

doesn't appear to be a way to specify the old (1967) all call of

LETTERS N LETTERS N LETTERS N LETTERS N, so there

must be a new convention, but I don't know what it is.

15

ENDCALL selcal option

Obviously there must be a way to tell the program to stop recording

a message and one of these ways is to send a stop string. This option

specifies a group of strings that will be used to stop the recording

of the incoming message.  This the format for these strings is the

same as the previous two.

Example:

ENDCAL NNNN nnnn ZCZC zczc

SELMAXT selcal time limit option

So that the program doesn't record reams of garbage, a time limit

can be specified. This time limit is in seconds and will only record

for "n" seconds. If it hasn't received a ENDCAL string by then, it

will close the received file. A line will be placed in the file indicating

that the recording was terminated because of a timeout.

Examples:

SELMAXT 300 (record for 5 minutes, maximum)

SELMAXT 32000 (record for virtually forever)

SELMAXT 12 (record only very short messages!)

EXTEND time limit extension string

Now it is nice to limit your messages to a specific time limit, but if

someone just has to get a long message to you, he(she) will have to

send it in chunks. He(she) might not know what you have set your

time limit to, so he might have to send it in a lot of little chunks to

be sure that he doesn't time out. In order to avoid that problem,

the EXTEND option can specify a string that the remote station

can send to you to tell you to set your time limit to a different

value. The format of the EXTEND option is:

EXTEND <string> <newtimelimit>

Where:

string is a group of characters (not including space and tab)

newtimelimit    is the absolute maximum that you want someone

to set your timelimit to.

An example is in order. If you have the following in your init file,

EXTEND NEWTIMEX 600

and a remote station sends your selcal string and then (after wait-

ing for your reply) sends a line that looks like: NEWTIMEX 300^m

16

(remember the ^m is an enter or end-of-line character) your pro-

gram will set your time limit to 300, giving the remote station the

opportunity to send you up to 5 minutes worth of data. If he(she)

had specified EXTEND NEWTX 4000, the program would limit

the time to 10 minutes, based on the absolute limit you placed on

the time of 600 seconds.

Of course, the other station should be aware of your extend

string.  If this string comes in while you are not recording a mes-

sage, it will be ignored.

FLATLOG

Use this parameter by itself to indicate that you want the station

log to be kept in a "flat" file format. In this case, flat means that

the data is in fixed columns. This might be useful for someone that

wanted to process the log from a program and didn't want to deal

with the "comma delimited" format that is the default.

SNOW

Some of the older CGA cards would produce snow on the screen

when direct video writes were done. If you see snow on your screen,

try to insert the snow directive in your simp.ini file.

BIOS

If you have a "near compatible" PC and the display isn't func-

tioning correctly, try to use the bios method of writing data to

the screen. It is fairly slow, but if that is all you have, it's better

than nothing. Insert the bios directive in your simp.ini if you are

still having trouble with snow. Don't have both active at the same

time. Include either snow or bios; or don't put anything.

NOMOUSE

If you have a mouse, you will see a scrollbar to the right of the

receiving (top) screen. If you find that having a mouse causes you

problems, insert the nomouse directive to cause the program to

treat you as if you didn't have a mouse.

QUIET

If you don't like the annoying startup screen telling you what you

are running and who's sweat gave it to you, insert the quiet direc-

tive in your startup file.

DDMMYY

17

Just in case someone wants to use this program in Europe or wher-

ever else they use the day-month-year format, instead of the month-

day-year used here, they can put this directive in their startup

file and the program will generate the date October 31,  1993,

as 31/10/93.  But I have no idea why you would want to.  (Jim

GRINS).

YYMMDD

This format is also included for completeness. It is left to the reader

to determine what it means. (Hint: read the previous paragraph)

LOGFILE

This directive specifies the name of the station log. The default is

station.log.

Example: LOGFILE k4gvo.log

4.3    Color Configuration

You can select one of the items by moving the highlighted bar up

and down with the up and down arrow keys.

4.4    Command line options

The SimpTerm command line looks like:

simp -init initfile -mid number -com comparamstring -word -V -

quiet

Where:

quiet tells the program not to display the introductory screen. See

the QUIET directive for more details.

word  causes displayed data in a window to wrap on a word boundry.

V  display the version number.

initfile is the name of a file to be used as the initialization file

containing the key definitions and other customization pa-

rameters. The default file name is "simp.ini".

number   is the line number of the middle of the screen.  This is

used to define the location of "split" between the transmit

and receive windows. Default is 12. See discussion below.

18

comparamstring     is the communications port parameter string.

This specification overrides the COM init file option.

The comparamstring parameter is in the form:

comn,baud,wordsize,parity,stopbits

Where:

comn   is the com port number, 1, 2, 3 or 4. This also can be

specified as COM1, COM2, COM3 or COM4.

baud   is the baud rate. Must be one of, 300, 1200, 2400, 4800

or 9600.

wordsize   is either 7 or 8.

parity  is one of the following: N, O, E, M, S.

stopbits  is one of: 1 or 2.

The following settings are the most common. If you don't know

for sure what to set them to, try these.

<comn>,<baud>,7,E,1

<comn>,<baud>,8,N,1

and maybe

<comn>,<baud>,7,O,1

The <comn> and <baud> depend on your computer hardware

configuration and your tnc interface speed.

You may use spaces instead of commas to separate the COM

fields. If you do, you must enclose the entire field in quotes, such

as:

simp -com "COM1 4800 8 N 1"

If you have both a COM option line in your init file and a command

line com option then the command line option will be used. This

way the command line can be used to override the init file if desired.

5    Station  logbook

The station logging screen allows you to enter a number of possibly

helpful items.  The first two fields will be filled in for you as they

are the current date and time.  The cursor will be positioned at

the third field, but if you want to change the previous fields, just

press the CTRL-TAB key. A list of the editing keys is available by

pressing the F1 key while the logging data form is displayed. The

fields and their sizes is as follows:

19

Field Name          Size           Starting position in log file

DATE                  6               1

TIME                  5               7

CALL                 11             12

NAME                 20             23

QTH                  39             43

MY RST                7             82

HIS RST               5             89

MODE                 10             94

COMMENTS            48             104

FREQUENCY            7             152

POWER                 4             159

QSL                   4             163

OTHER                12             167

ENDTIME               4             179

Which looks like about a 182 character record in the flatfile

mode. I hope all of these fields are self-explanatory. The OTHER

field is just an additional field that can be used to put things I

didn't think of.

If you elect not to use the flatfile mode your file will be in this

format

"103193","0532","k4gvo","Jim","Fayetteville, GA",

"599","599","","10.109","100","","","","0650"

Of course, it will be all on one line.  This format is called

"comma delimited" by *base and others of its ilk.  This format

may be imported to many of the database managers and spread-

sheets available on DOS. If you don't like the layout, drop me a

note.

6    Selective  calling  feature

Here is part of a typical init file that will support the selective

calling feature.

!KEY_F10 togsel

RSELCAL k4gvo K4GVO W4LYBZW k4gvozw

ENDCAL ZCZC zczc NNNN nnnn

ALLCAL ALLSTAT

SELMAXT 240 seconds

EXTEND MXLEN 600

SELREPLY $"Backslash$rx$"Backslash$r^o^o^o^o^o^ode k4gvo %D ^m^d

If, in the body of a message being recorded, the string "MXLEN"

is found, the time limit will be bumped up to 10 minutes.  Note

20

that the "seconds" field on the SELMAXT option line is really a

comment and not a specification.  The following line WON'T set

the time to 2 minutes.

SELMAXT 2 minutes

7    License

I'm not going to bother with the usual nonsense about responsibil-

ity and guarantees. If you're foolish enough to use a free package

with something that you think is critical enough that you might

be tempted to sue someone if it screws up, you deserve what you

might get.  CAVEAT EMPTOR! I hope I spelled that right.  My

last Latin course was before my first FORTRAN II course. Am I

telling my age?

I guess I didn't mention that this program is almost free. Might

even be freeware, but since freeware is a registered trademark, I

understand, I won't call it that. If you use this program and feel it

is of any value, please drop something in the next Salvation Army

kettle you see.  If that activity offends you for any reason, please

just do something nice for someone that you wouldn't normally do.

Like buy your secretary lunch the next time she screws up royally

instead of reaming her out.

You can use it, change it, abuse it, distribute it, charge for it

or most anything else you want to do with it, except if you charge

more than a reasonable copying charge for it, you must tell whoever

you sold it to that they could have gotten it for nothing if they had

just looked around. You may distribute it in any form, as long as

this file is included in its original form. You may add as many files

as you wish.

If you want source, drop me a note and we'll discuss it.  I'll

probably eventually turn it out into the public domain, but I'd be

too embarrassed to do so with it now.

8    Trouble  shooting

This section hasn't been written yet.

9    Error  messages

I hope they are few and perfectly understandable. I haven't com-

piled a complete list, but here are a few you may encounter during

the initialization phase.

21

HELP line has no file name

The HELP line must have a valid file name as the first parameter.

LOGFILE line has no file name

The LOGFILE line must have a valid file name as the first param-

eter.

PRELOAD has no file name

The PRELOAD line must have a valid file name as the first pa-

rameter.

MID has no value

The MID line must have a valid non-zero, non-blank value as the

first parameter.

Key table corrupt

This is an internal error, it should never appear unless memory

gets overwritten.

Line nn, Invalid key name <keyname>

SimpTerm can't identify the key name on the line, number nn.

Check your spelling.

Malloc failed

SimpTerm ran out of memory.

Number of lines, nnn exceeds max, mmm

The maximum number of continuation lines has been exceeded.

Port definition in error, line nnn

There is something wrong with the COM port definition on line

nnn. Be sure it looks like the examples given above.

Port init failed nn

The attempt to initialize the com port failed with an error of nn.

22

10     Final  notes

I hope that you find this program easy and enjoyable to use.  I

wrote it while I was traveling with a T1000SE (8086) laptop (no

hard drive), a PK-232 and a IC-730.  I didn't really like to use a

simple terminal program and felt the big gun programs, just were

too clumsy for portable use.  I've made a few enhancements to

that original program and hope that you will find it acceptable. I

suspect that you aren't going to find a cheaper product.

This manual was typeset with LaTEX. I will be offering a hard-

copy of this manual on 8.5 x 11, 3 hole punched paper ready for

your 3 ring binder in the near future at what amounts to repro-

duction and mailing costs. If you are interested please drop me a

note with a sase.

If you like SimpTerm. Tell others about it. If you don't like it,

please tell me why. If you don't like it and tell others that it isn't

any good, then we all lose. You lose, 'cause I just might be able to

make it what you wanted, if I only knew, others lose 'cause they

might find something in it for them that you weren't interested in

and I lose, 'cause my feelings have been hurt, so PLEASE if you

want to complain, fire it my direction. I can't promise I can make

it your personal rose garden, but I will try to fix any problems or

deficiencies that might show up.

Oh, I know the documentation isn't up to snuff, but it's better

than not getting it out at all. I sure had fun with the spell checker.

It wanted to make rtty into ratty and amtor into actor. Wonder if

it knows something I don't?

11     Bugs

I think I have fixed all that have been reported.

The enhanced keyboards aren't supported yet.  F11 and F12

don't work.

12     Futures

Clean up the code so I'm not embarrassed to distribute it.

It does not support the enhanced keyboard.

I'd like to add a more robust station logging function or maybe

multitask with the command processor so one could run their fa-

23

vorite logging program at the same time with out having to use

DV or something similar.

I plan on implementing a user-configurable log format, where the

user can describe the number, names and sizes of the log data.  I

also am thinking about supporting .dbf file structures in addition

to the flatfile and comma delimited.

Alas, there is no index in this document. /LaTex does have support

for an index, but I haven't gotten around to it yet.

24