Run emu86.exe.  This will put you in a V86 shell running DOS.  Checks

are made; it will generate an error if you don't have a 386, if XMS is

version 2.00 or greater not installed, if the dos version isn't 3.10

or better, if EMS is installed , or if the processor is already in

protected mode.  For this version you need 1MB of extendend memory

so it can play around.

This currently runs as a TSR.  To unlaoad it run 'rundown.exe'; this

will unhook it from the system and free its memory usage.

I've released this as a standalone program primarily to see if my V86

emulator works on a variety of PCs.  Eventually it will provide EMS

support.  Although I already have one person interested in modifying

the emulator to provide a virtual UART.

This program may or may not work with the FREE-DOS xms driver.  This

driver currently has bugs, I'm working with someone else to work

on compatibility issues.

Technical details follow.

The program takes up about 10K of conventional memory.  This includes

the GDT.  For addresses refer to the MAP file.

The progroam allocates 512 KB of memory through the XMS system.  It

uses an XMS lock to get the physical address of this memory.  At this

address it places the following information:

X+0:		Page directory

X+1000h:        4 Page tables (1 MB DOS + 15 MB extended RAM)

X+5000h:	TSS for V86 emulation.  This includes a 8192 byte I/O

bitmap.  Ports 84h and 85h are emulated and are used

for V86 control operations.

X+7e7ch:	Supervisor stack

X+7e80h:	IDT.  30h entries.  The first 20h are mapped to

exception vectors and the next 10h are the hardware

interrupts

x+8000h:	Page mapped as a shadow to the last page of the

ROM Bios.  We actually change the hook at FFFF:0000

to handle reboots properly.

X+8100h:	To be used as free EMS memory when EMS support is

added.

Selectors are as follows:

08h:		386 code segment

10h:		386 data segment

18h:		16 bit compatible code segment for rundown

20h:		16 bit compatible data segment for rundown

28h:		linear mem starting at 00000000h

Ports are as follows:

84h:		V86 control port

85h:		V86 data port

The V86 emulator runs at IPL 3 for speed, however it will work

albeit more slowly if you change the IPL down.

This version of the program will run itself down if it gets certain

exceptions ; other exceptions will be reflected back to DOS.

If you see the message 'emu86 unloaded' you will know that some

exceptional condition occurred.  Please contact me and let me know

if this happens.

David Lindauer gclind01@ulkyvx.louisville.edu

boot.asm

Transition from real to protected mode and back, with paging

checks.asm

Ensure the system has the minimum setup required and no mem

manager

emuinter.asm

Port 84h and 85h emulation for controlling emulator

idt.asm

IDT setup

int15.asm

int 15h support for virtualizing himem.sys accesses to

extended memory

pic.asm

PIC support

reboot.asm

handles mapping the last BIOS page, modifying the reboot

vector at FFFF:0, supporting a reboot request

rundown.asm

Seperat ASM program which runs the EMU86 system down

tss.asm

TSS setup, including I/O mapping

vm86.asm

V86 emulator support

vm386.asm

Support for 386 commands in V86 mode.  Not implemented yet.

xmsctl.asm

XMS shadowing, handles A20 line functions virtually

ems.asi

EMS header file

emuinter.asi

port emulation header file

gdt.asi

GDT definitions

idt.asi

IDT definitions

int15.asi

int 15 definitions

pic.asi

PIC definitions- also defines where interrupts will be

redirected

reboot.asi

Header file for reboot functions

segs.asi

Segment definitions and Selectors

tss.asi

TSS header file

vm86.asi

Emulator header file

vm386.asi

386 emulation header file

xmsctl.asi

XMS header file