NISTIR 4481

THE NIST "BUILDING LIFE-CYCLE COST" (BLCC) PROGRAM

(Version 3.2 for 1993)

User's Guide and Reference Manual

Stephen R. Petersen

Office of Applied Economics

Computing and Applied Mathematics Laboratory

Sponsored by

The Federal Energy Management Program

U.S. Department of Energy

Washington, D.C. 20585

January 1991

ABSTRACT

The NIST Building Life-Cycle Cost computer program (BLCC 3.2) provides

economic analysis of proposed capital investments that are expected to reduce

long-term operating costs of buildings or building systems.  It is especially

useful for evaluating the costs and benefits of energy conservation projects

in buildings.  Two or more alternative designs can be evaluated to determine

which has the lowest life-cycle cost.  Economic measures, including net

savings, savings-to-investment ratio, and adjusted internal rate of return,

can be calculated for any design alternative relative to the designated base

case.  BLCC can be used for evaluating both Federal and private-sector

projects.  It complies with ASTM standards related to building economics as

well as FEMP and OMB A-94 guidelines for economic analysis of Federal building

projects.  BLCC is designed to run on an IBM-PC or compatible microcomputer

with approximately 640K of random access memory, with or without a hard disk.

The BLCC program supersedes both the FBLCC and NBSLCC computer programs

previously released by NIST (NBS).

Note: Please see file README.1 for updated information about BLCC that is not

included in this user's guide.

iii

PREFACE

The Building Life-Cycle Cost computer program, version 3.2, has been prepared

as an aid to implementing life-cycle cost evaluations of potential energy

conservation and renewable energy projects in new and existing Federal

buildings.  It can also be used for evaluating similar projects in the private

sector and at the state and local government levels.  BLCC 3.2 supersedes and

incorporates both the Federal Building Life-Cycle Cost (FBLCC) and the

National Bureau of Standards Life-Cycle Cost (NBSLCC) programs.  For the

economic analysis of energy conservation investments in Federal buildings,

BLCC complies with NBS Handbook 135, Life-Cycle Costing Manual for the Federal

Energy Management Program.  For other investments in Federal buildings, BLCC

complies with OMB Circular A-94, "Discount Rate to Be Used in Evaluating Time-

distributed Costs and Benefits," March 27, 1972.  BLCC complies with the

American Society for Testing and Materials (ASTM) standard practices for

building economics, including E917 (Practice for Measuring Life-Cycle Costs of

Buildings and Building Systems), E964 (Practice for Measuring Benefit-to-Cost

and Savings-to-Investment Ratios for Buildings and Building Systems), E1057

(Practice for Measuring Internal Rates of Return for Investments in Buildings

and Building Systems), and E1074 (Practice for Measuring Net Benefits for

Investments in Buildings and Building Systems).

BLCC is the third of a four-volume set of reports which together provide a

comprehensive set of methods, data, and computational tools for economic

evaluation of such projects.  Included in the four-volume set are the

following:

(1) Life-Cycle Costing Manual for the Federal Energy Management Program,

National Institute of Standards and Technology, Handbook 135 (revised

1991).

Handbook 135 is a guide to understanding life-cycle costing and related

methods of economic analysis as they are applied to Federal decisions.  It

describes the required procedures and assumptions, defines and explains how to

apply and interpret economic performance measures, gives examples of Federal

decision problems and their solutions, explains how to use the energy price

indices and discount factors which are updated annually in the supplement (2),

and provides worksheets and other computational aids and instructions for

calculating the required measures.

(2) Energy Prices and Discount Factors for Life-Cycle Cost Analysis, National

Institute of Standards and Technology, (Annual Supplement to NIST

Handbook 135 and NBS Special Publication 709), NISTIR 85-3273 (updated

annually).

This report, which is updated annually, gives the energy price and discount

factor multipliers needed to estimate the present value of energy and other

future costs.  The data are based on energy price projections developed by the

Energy Information Administration of the U.S. Department of Energy.  Request

the latest edition.

iv

(3) The NIST "Building Life-Cycle Cost" (BLCC) Computer Program (version 3.2),

National Institute of Standards and Technology, 1990.

The NIST Building Life-Cycle Cost computer program (BLCC 3.2) provides

economic analysis of proposed capital investments that are expected to reduce

long-term operating costs of buildings or building systems.  It is especially

useful for evaluating the costs and benefits of energy conservation projects

in buildings.  Two or more alternative designs can be evaluated to determine

which has the lowest life-cycle cost.  Economic measures, including net

savings, savings-to-investment ratio, and adjusted internal rate of return,

can be calculated for any design alternative relative to the designated base

case.  BLCC can be used for evaluating both Federal and private-sector

projects.  It complies with ASTM standards related to building economics as

well as FEMP and OMB A-94 guidelines for economic analysis of Federal building

projects.  BLCC is designed to run on an IBM-PC or compatible microcomputer

with approximately 640K of random access memory, with or without a hard disk.

The BLCC program supersedes both the FBLCC and NBSLCC computer programs

previously released by NIST (NBS).

(4) DISCOUNT--A Program for Discounting Computations in Life-cycle Cost

Analyses (version 3.2), NISTIR 4513, National Institute of Standards and

Technology, 1991.

The DISCOUNT program computes discount factors and related present values,

future values, and periodic payment values of cash flows occurring at known

points in time.  DISCOUNT is especially useful for solving life-cycle cost

analysis problems which do not require the comprehensive summation and

reporting capabilities provided by the BLCC program.  DISCOUNT performs all of

the functions of standard discounting tables, computing present values of

future amounts, future values of present amounts, present and future values of

periodic payments, periodic payments corresponding to present and future

amounts, and corresponding discount factors.  In addition, DISCOUNT computes

the present value of periodic payments which increase at known rates over

time, and the present value of energy costs which increase at rates projected

by the U.S. Department of Energy for use in Federal life-cycle cost analyses.

DISCOUNT provides the added flexibility of accepting non-integer discount

rates, time periods, and escalation rates in its computations.  DISCOUNT runs

on most IBM PC and compatible microcomputers with no special equipment

requirements.  The user's guide and reference manual is included on the

program diskette.

Both the BLCC and DISCOUNT computer programs access DOE energy prices

projections from the same two disk files included with these programs

(currently ENCOST93.SEQ and ENCOST93.RAN).  These files are updated and

released at the same time that NISTIR 85-3273, Energy Prices and Discount

Factors for Life-Cycle Cost Analysis, is released each year.  These disk files

ensure that BLCC and DISCOUNT are compatible with computations performed using

the factors in NISTIR 85-3273.

v

For representatives of Federal government agencies and contractors to the

Federal government, these publications can be ordered from:

Advanced Sciences, Inc.

2000 North 15th Street

Suite 407

Arlington, VA 22201

(703) 243-4900

Please request the publications by name and number.

For others interested in obtaining the BLCC or DISCOUNT computer programs,

please contact one of the following organizations:

National Technical Information Service

5285 Port Royal Road

Springfield, VA 22161

(703) 487-4650

PC Software Interest Group

1030D East Duane Avenue

Sunnyvale, CA 94086

(408) 730-9291

MTS Software

5 Oak Forest Court

Saint Charles, MO 63303

(314) 441-1022

Energy Ware

PO Box 254

Littleton, NH 03561

(800) 522-4485

Workshops on the life-cycle costing method and energy analysis are conducted

at locations around the country each year.  The workshops include training and

software for both BLCC and an energy analysis computer program called "A

Simplified Energy Analysis Method" (ASEAM).  A schedule of workshops can be

obtained from the Office of Applied Economics, National Institute of Standards

and Technology, Bldg. 101, Room A415, Gaithersburg, MD 20899, (301) 975-6132.

An introduction to the workshop is provided in a video training film, "Least-

Cost Energy Decisions:  An Introduction to Life-Cycle Cost Analysis."  The

video film and companion workbook can be ordered from Video Transfer, Inc.,

5709-B Arundel Avenue, Rockville, MD 20852, (301) 881-0270.  Additional

training films in the series are in preparation.

Further information on the Federal Energy Management Program can be obtained

from the Federal Energy Management Program Staff, Office of the Assistant

Secretary for Conservation and Renewable Energy, U.S. Department of Energy.

Please direct communication to:  FEMP, CE 10.1, U.S. Department of Energy,

1000 Independence Avenue, SW, Washington, D.C. 20585.

vi

ACKNOWLEDGMENTS

The author wishes to thank Mr. Richard W. Brancato and Mr. K. Dean DeVine of

the Federal Energy Management Program of the U.S. Department of Energy (DOE)

for their continued support and direction of this work.  Appreciation is also

extended to Harold Marshall, Rosalie Ruegg and Barbara Lippiatt of the Office

of Applied Economics and Stephen Treado of the Center for Building Technology

for their careful review of this User's Guide.

vii

CONTENTS

Page

ABSTRACT. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  iii

PREFACE . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   iv

ACKNOWLEDGMENTS . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  vii

1.  INTRODUCTION. . . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

2.  INSTALLING AND RUNNING BLCC . . . . . . . . . . . . . . . . . . . . .    4

3.  CONDUCTING A BLCC ANALYSIS. . . . . . . . . . . . . . . . . . . . . .    5

3.1  The Main BLCC Menu. . . . . . . . . . . . . . . . . . . . . . .    5

3.2  Creating or Editing Input Data Files for a Project Alternative.    5

3.3  Calculating Life-Cycle Costs and Cash-Flow analyses . . . . . .   10

3.4  Lowest LCC and Comparative Economic Analysis. . . . . . . . . .   14

4.  REPORTS GENERATED BY BLCC . . . . . . . . . . . . . . . . . . . . . .   16

5.  GENERALIZED INPUT DATA FILE STRUCTURE FOR BLCC. . . . . . . . . . . .   17

6.  STANDARD DEPRECIATION TABLES. . . . . . . . . . . . . . . . . . . . .   19

7.  COMPUTATIONAL CONVENTIONS USED IN BLCC. . . . . . . . . . . . . . . .   20

8.  DEFINITIONS AND ACCEPTABLE RANGES OF INPUT VARIABLES USED IN THE

BLCC PROGRAM. . . . . . . . . . . . . . . . . . . . . . . . . . . . .   21

INDEX . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   32

viii

LIST OF TABLES

Page

Table 1.  New or Enhanced BLCC Features . . . . . . . . . . . . . . . . .   2

Table 2.  Reports Generated by BLCC . . . . . . . . . . . . . . . . . . .  16

Table 3.  Input Data File Structure . . . . . . . . . . . . . . . . . . .  18

LIST OF FIGURES

Figure 1.  Main BLCC Menu . . . . . . . . . . . . . . . . . . . . . . . .   5

Figure 2.  The "Save Data" Menu . . . . . . . . . . . . . . . . . . . . .   7

Figure 3.  Input Data File Report for DEMO1 . . . . . . . . . . . . . . .   8

Figure 4.  BLCC Summary for DEMO1 . . . . . . . . . . . . . . . . . . . .  10

Figure 5.  Detailed BLCC Report (example) . . . . . . . . . . . . . . . .  11

Figure 6.  Cash Flow Report for File DEMO1. . . . . . . . . . . . . . . .  12

Figure 7.  Lowest LCC Analysis for Files DEMO1, DEMO2, DEMO3, and DEMO4 .  14

Figure 8.  Project Data Input Form. . . . . . . . . . . . . . . . . . . .  17

Figure 9.  Example of an ASCII File of Standard Depreciation Factors. . .  19

ix

1.  INTRODUCTION

The NIST Building Life-Cycle Cost computer program (BLCC 3.2) provides

economic analysis of proposed capital investments that are expected to reduce

long-term operating costs of buildings or building systems.  It is especially

useful for evaluating the costs and benefits of energy conservation projects

in buildings.  Two or more alternative building designs that have the same

general purpose can be evaluated to determine which has the lowest life-cycle

cost.  Economic measures, including net savings, savings-to-investment ratio,

and adjusted internal rate of return, can be calculated for any design

alternative relative to the designated base case.  BLCC runs on most IBM-PC

and compatible microcomputers with approximately 640K RAM.  No graphics

capability, math coprocessor, or other special equipment are needed.

BLCC 3.2 supersedes and incorporates both the Federal Building Life-Cycle Cost

(FBLCC) and the National Bureau of Standards Life-Cycle Cost (NBSLCC)

programs.  Table 1 lists a number of BLCC features which improve upon or

differ significantly from those programs.

Five types of LCC analyses can be performed with the BLCC program:

(1) Federal Analysis:  Energy Conservation Projects

(2) Federal Analysis:  Projects subject to OMB's A-94 guidelines

(3) Private Sector:  Projects with tax analysis

(4) Private Sector:  Owner-occupied houses (tax deductions on interest and

property taxes only)

(5) Generic LCC Analysis:  No tax analysis (e.g., state & local government)

BLCC complies with the American Society for Testing and Materials (ASTM)

standard practices for building economics, including E917 (Practice for

Measuring Life-Cycle Costs of Buildings and Building Systems), E964 (Practice

for Measuring Benefit-to-Cost and Savings-to-Investment Ratios for Buildings

and Building Systems), E1057 (Practice for Measuring Internal Rates of Return

for Investments in Buildings and Building Systems), and E1074 (Practice for

Measuring Net Benefits for Investments in Buildings and Building Systems).

For the economic analysis of energy conservation investments in Federal

buildings, BLCC complies with NBS Handbook 135, Life-Cycle Costing Manual for

the Federal Energy Management Program.  For other investments in Federal

buildings, BLCC complies with OMB Circular A-94, "Discount Rate to Be Used in

Evaluating Time-Distributed Costs and Benefits," March 27, 1972.

����������������������������������������������������������������������������Ŀ

�    Note:  BLCC is a public domain program, not subject to copyright.       �

�    It can be freely copied and distributed.  However, it is requested      �

�    that attribution be given to the National Institute of Standards        �

�                            and Technology.                                 �

������������������������������������������������������������������������������

1

Table 1.  New or Enhanced BLCC Features

����������������������������������������������������������������������������Ŀ

� o  All program modules are contained in a single executable file.          �

�                                                                            �

� o  Data files and reports can now be saved to and retrieved from any drive.�

�    Use "Setup" in the main BLCC menu to specify the target drive.          �

�                                                                            �

� o  LCC computations are automatically made when the input data file is     �

�    saved.                                                                  �

�                                                                            �

� o  New cash flow report details all dollar amounts (not discounted) year-  �

�    by-year over the entire study period.                                   �

�                                                                            �

� o  Up to 100 alternative projects can be compared simultaneously to        �

�    determine which has the lowest LCC.  Simply tag the LCC files for each  �

�    alternative to be considered and press <Enter>.                         �

�                                                                            �

� o  Only questions relevant to current analysis are displayed on screen.    �

�                                                                            �

� o  Standard depreciation tables can be read from depreciation files in the �

�    current directory to minimize data entry.                               �

�                                                                            �

� o  Depreciation variables are now entered separately for replacements.     �

�    Project capital costs can be entered as aggregate investment-related    �

�    costs or divided into as many as six components.                        �

�                                                                            �

� o  Operating and maintenance (O&M) costs, replacement costs, and depre-    �

�    ciation data are specified separately for each component if capital     �

�    costs are divided into components.  (Energy and mortgage costs are      �

�    entered for the overall project and are not component specific.)        �

�                                                                            �

� o  Price escalation rates for non-annually recurring O&M costs are now     �

�    assumed to be the same as specified for annually-recurring O&M costs.   �

�                                                                            �

� o  Price escalation rates for initial capital assets, O&M, and energy      �

�    prices must include general inflation if the analysis is conducted in   �

�    current dollars (private sector and designated generic analyses only).  �

�    Previously all escalation rates were entered in real terms.             �

�                                                                            �

� o  Up to four energy types can now be included in an input data file.      �

�                                                                            �

� o  Energy usage from year to year can be assumed constant or adjusted with �

�    annual energy usage indexes.                                            �

�                                                                            �

������������������������������������������������������������������������������

2

BLCC retrieves the U.S. Department of Energy's (DOE) projections of energy

price escalation rates from the ENCOST93.SEQ and ENCOST93.RAN files on the

disk.  These DOE projections are needed for Federal energy conservation

analyses but can be used with other LCC analyses as well.

This user's guide provides an overview of BLCC capabilities, tells the user

how to install and run it, and defines all of the variables needed to conduct

an LCC analysis.  Detail regarding the computational methodology used in BLCC

can be found in Appendix H of Comprehensive Guide for Least-Cost Energy

Decisions, by Rosalie T. Ruegg and Stephen R. Petersen, NBS Special

Publication 709, National Bureau of Standards, Gaithersburg, MD, 1987.

The BLCC program was developed at the National Institute of Standards and

Technology (formerly the National Bureau of Standards).  For further

information, write to:

National Institute of Standards and Technology

Applied Economics Group

Bldg. 101, Room A415

Gaithersburg, MD 20899

3

2.  INSTALLING AND RUNNING BLCC

BLCC can be run from a hard disk or directly from the BLCC diskette.  To

install BLCC on a hard disk, use the following steps:

(1) Make a subdirectory called BLCC on the drive of your choice (say C:).

To make a subdirectory called "BLCC", type "MD \BLCC" at the ready

prompt.

(2) Type "CD \BLCC" to make this the current directory.

(3) Insert the BLCC disk into the A drive and copy all of the files from the

BLCC disk to the BLCC subdirectory by typing "COPY A:*.*".

(4) Type "BLCC" at the ready prompt to start BLCC.

To run BLCC directly from the BLCC diskette, use the following steps:

(1) Make a back-up copy of the BLCC diskette.  If you are going to run BLCC

in a high-density (1.2 MB) disk drive, copy BLCC to a high-density

diskette formatted on that drive.

IMPORTANT:  BLCC writes temporary and permanent files to the disk.  If

you use the original double-density (360K) diskette in a high-density

drive, you may destroy the original disk during the write process.

(2) Insert your new copy of the BLCC diskette into the A drive.

(3) Make the A drive the current drive by typing "A:".

(4) Type "BLCC" at the ready prompt to start BLCC.

If you run BLCC from the diskette rather than from a hard disk, space for file

storage is limited and you should therefore save your own input and output

files to a second diskette in another drive.  BLCC data entry screens and

reports will be displayed somewhat more slowly when BLCC is run directly from

the diskette.  This is because screen and report formatting files must be

accessed from the disk as data input and analysis are underway.

The executable program, BLCC.EXE, and the following supporting files must be

available in the current directory to run BLCC:

MORT.SCR         DEPDATA.SCR        COMPARE.FLD       HELP.1

CAPDATA.SCR      ARCDATA.SCR        CASHFLOW.FLD      ENCOST93.SEQ

ENDATA.SCR       ESCDATA.SCR        PRINTDAT.FLD      ENCOST93.RAN

PROJ.SCR                            PRINTLCC.FLD      SETUP.FIL

There are also several demonstration files on the diskette for use in learning

how to use the program.  These are files:

DEMO1.DAT    DEMO1.LCC    DEMO3.DAT    DEMO3.LCC  TABLE1.DEP

DEMO2.DAT    DEMO2.LCC    DEMO4.DAT    DEMO4.LCC

File README.1 provides updated information on BLCC that is not included in

this user's guide.

4

3.  CONDUCTING A BLCC ANALYSIS

3.1 The Main BLCC Menu

When you enter "BLCC" at the DOS ready prompt, the BLCC logo appears on the

screen.  Press any key to display the Main BLCC Menu, shown in figure 1.  This

menu shows the major functions of the BLCC program.  Use the up/down arrows to

highlight the desired function and press the <Enter> key.  As each function is

highlighted, a brief description of that function will be displayed at the

bottom of the screen.  Alternatively, simply press the highlighted character

for the function desired.

���������������������������������ͻ

� Create New Input Data File      �

� Edit Existing Input Data File   �

� Print Input Data File Report    �

� Print Detailed LCC Report       �

� Compute Cash-Flow Analysis      �

� Lowest LCC                      �

� Comparative Economic Analysis   �

� Information on BLCC             �

� Setup                           �

� Quit/Go To DOS                  �

���������������������������������ͼ

Figure 1.  Main BLCC menu.

The first time you run BLCC, select the "setup" option on the master menu.

This option allows you to define the type of monitor you are using, the

directory or subdirectory location of the data files and output files that are

created and saved while running BLCC, and the common filename of the DOE

energy price files on the BLCC disk (currently ENCOST93).  These responses are

saved to a file called SETUP.FIL in the BLCC subdirectory, and are retrieved

each time you run BLCC.

3.2  Creating or Editing Input Data Files for a Project Alternative

To use BLCC, you must have an input data file for each alternative building or

system design to be evaluated.  The input data file for any project

alternative contains assumptions for the LCC analysis, initial costs,

operating and maintenance costs, replacement costs, energy costs, resale

value, and (where relevant) data related to taxes and financing analysis.  The

input data file is retrieved when editing the input data file, computing the

LCC of a project alternative, computing a cash flow analysis for the

alternative, or printing an input data file report.  Read section 5 of this

manual for additional information on the input data requirements and BLCC

input data file structure.  Section 8 provides definitions of all variables

requested on the data input screens.

5

If you are going to create a new input data file, select "Create New Input

Data File" from the Main BLCC Menu.  If you are going to edit an existing

input data file, select "Edit Existing Input Data File" from the menu and a

list of the existing input data files in your current directory will be

displayed.  To designate the file to be edited, use the cursor keys to

highlight the desired filename, and then press <Enter> to begin.  All input

data files are automatically tagged with the filename extension ".DAT".

NOTE

You must make a separate input data file for each project

alternative to be evaluated.  Don't try to combine several

alternatives into a single input data file by using multiple

components.

After creating and saving an initial file for one project

alternative, it is usually easier to modify that file in the edit

mode with data for a related alternative and save it under a new

name than to create a new file from the beginning.

When creating a new input data file, you must select the type of LCC analysis

to be conducted Federal energy conservation from the following:

(1) Federal Analysis:  Energy Conservation Projects

(2) Federal Analysis:  Projects subject to OMB's A-94 guidelines

(3) Private Sector:  Projects with tax analysis

(4) Private Sector:  Owner-occupied houses

(5) Generic LCC Analysis:  No tax analysis

Once this analysis type is specified, and the first data entry screen is

completed, it cannot be changed.  (This is the only variable in BLCC that

cannot be changed, since it determines how the data file is set up.)

If the analysis is for Federal buildings, the LCC analysis will be conducted

in CONSTANT DOLLARS.  This requires that the discount rate and price

escalation rates be entered in real terms, i.e., net of general inflation.  If

the analysis is designated as private sector, the LCC analysis will be

conducted in CURRENT DOLLARS.  This requires that the discount rate, price

escalation rates, and mortgage interest rates (if any) be entered in nominal

terms, i.e., including general inflation.  If a generic LCC analysis is

selected, the user must designate whether this analysis will be performed in

constant or current dollars and enter all rates accordingly.

From most screen displays in BLCC, you can back up to the previous screen by

pressing <Esc>.  From data input screens you can move backward or forward by

pressing <PgUp> or <PgDn>, respectively.  However, if the cursor is located at

a mandatory input variable, you must fill in a valid response before moving

6

backward to a previous screen.  You cannot move forward to the next screen

until all required data fields are filled with valid responses.  If you are

editing an existing data file, you can move to the "save data" menu by

pressing F10 at any point.

After completing the data entry or review, the "save data" menu is displayed,

as shown in figure 2.  Be sure to save your input data file if you intend to

use it for BLCC analysis.  You can review the data file in the edit mode if

you select this option from the menu.  If you are editing an existing file,

you can save it under a new file name or default to the old file name; in the

latter case it will replace the old file.  Filenames must follow the DOS file-

naming convention (i.e., up to eight alpha-numeric characters).

Data entry is now complete.

�������������������������ͻ

� Save Input Data File    �

� Review Input Data File  �

� Exit without Saving     �

�������������������������ͼ

Figure 2.  The "Save Data" Menu.

Any input data file can be printed for documentation purposes by selecting

"Print Input Data Report" from the main BLCC menu.  As with all BLCC reports,

this report can be displayed to the screen, sent to the printer, or saved to

an ASCII file in a designated subdirectory.  Figure 3 shows the input data

file report for demonstration file DEMO1.DAT, which is included on the BLCC

disk.

For first-time users of BLCC, it is suggested that you select "Edit Existing

Input Data File" from the main menu, and then retrieve file DEMO1.DAT.  This

is one of four demonstration input data files on the BLCC diskette.  The data

in all of these demonstration files are for "generic" LCC analyses (without

income tax considerations).  Press the <PgDn> key after examining each of the

input data screens in order to familiarize yourself with the data entry

process.  Refer to section 8 for definitions of all input variables.  When

data review has been completed, the menu shown in figure 3 will be displayed.

Save the output file under its default name (DEMO1.LCC) and select the screen

display of the summary LCC file.

7

FILE NAME: DEMO1

FILE LAST MODIFIED ON 01-10-1991/16:29:55

PROJECT TITLE: demo1

COMMENT: This is a BLCC demonstration file

GENERAL DATA:

-------------

ANALYSIS TYPE: Generic LCC Analysis: No Tax Analysis

BASE DATE FOR LCC ANALYSIS: 1990

STUDY PERIOD:  20 YEARS

PLANNING/CONSTRUCTION PERIOD:  2 YEARS

OCCUPANCY DATE: 1992

DISCOUNT AND INTEREST RATES Real (exclusive of general inflation)

DISCOUNT RATE:  5.0%

CAPITAL ASSET COST DATA:

------------------------

INITIAL COST ($)                    10,000

EXPECTED COMPONENT LIFE(YRS)            20

RESALE VALUE FACTOR                  0.00%

AVG PRICE ESC RATE(PLAN/CONST)       0.00%

AVG PRICE ESC RATE (OCCUPANCY)       0.00%

Escalation rates do not include general inflation

NUMBER OF REPLACEMENTS                   1

COST-PHASING SCHEDULE BY YEAR OF CONSTRUCTION AND AT OCCUPANCY:

1      33.00%

2      33.00%

AT OCCUPANCY     34.00%

REPLACEMENTS TO CAPITAL ASSETS:

-------------------------------

REPLACEMENT NUMBER                    1

YEAR OF REPLACEMENT                  10

COST OF REPLACEMENT ($)           5,000

EXPECTED REPL. LIFE (YRS)            10

RESALE VALUE FACTOR               0.00%

OPERATING AND MAINTENANCE COST DATA:

------------------------------------

ANNUAL RECUR O&M COST ($):   1,000

ESCALATION RATE FOR O&M:     0.00%

Escalation rates do not include general inflation

NON-AN RECURRING O&M COSTS ($):

YR   AMOUNT

10   $1,000

20   $1,000

____________________________________________________________________

Figure 3.  Input Data File Report for DEMO1

8

ENERGY COST DATA:

-----------------

NUMBER OF ENERGY TYPES = 1

User entered energy price escalation rates used with Energy Type(s) 1

TYPE 1

ENERGY TYPE:              Electricity

AVG ANNUAL CONSUMPTION:        100,000

UNITS:                         KWH

PRICE PER UNIT ($):            0.10

ANNUAL DEMAND CHARGE ($):      0.00

ESCALATION RATES BY YEAR:   Escalation rates do not include general inflation

1990        2.00                       2000        2.00

1991        2.00                       2001        2.00

1992        2.00                       2002        2.00

1993        2.00                       2003        2.00

1994        2.00                       2004        2.00

1995        2.00                       2005        2.00

1996        2.00                       2006        2.00

1997        2.00                       2007        2.00

1998        2.00                       2008        2.00

1999        2.00                       2009        2.00

ENERGY TYPE:  Electricity

ENERGY USE INDEXES BY YEAR:

1992      100.00                       2001      100.00

1993      100.00                       2002      110.00

1994      100.00                       2003      110.00

1995      100.00                       2004      110.00

1996      100.00                       2005      110.00

1997      100.00                       2006      110.00

1998      100.00                       2007      110.00

1999      100.00                       2008      110.00

2000      100.00                       2009      110.00

________________________________________________________________

Figure 3.  Input Data File Report for DEMO1 (continued).

9

3.3  Calculating Life-Cycle Costs and Cash-Flow Analyses

At the time that a new or existing input data file is saved, the corresponding

LCC computations are automatically performed and saved in an LCC output file.

The default filename for the LCC output file is the same as that assigned to

the input data file, with the filename extension ".LCC" automatically tagged.

You can change the filename to any other valid DOS filename (up to eight

alpha-numeric characters) if you wish, but it is usually easier to keep track

of pairs of corresponding input and output files when they have the same

filename.  This LCC output file is retrieved when printing a detailed LCC

report, conducting comparative LCC analyses, or determining which of several

mutually-exclusive project alternatives has the lowest LCC.

At the time that the LCC computations are performed, a summary report of the

LCC computations is generated.  This summary can be printed to the screen,

printer, or a disk file (for later retrieval).  An example of this summary

file is shown in figure 4 for the DEMO1 example file.  To regenerate the

summary report at any time, retrieve the corresponding input data file in the

file edit mode and resave the file.

���������������������������������������������������������������������������Ŀ

�                                          PRESENT VALUE      ANNUAL VALUE  �

�  INITIAL COST (AT OCCUPANCY)                   $9,527              $764   �

�  ANNUALLY RECURRING O&M COSTS                 $10,603              $851   �

�  NON-AN. RECURRING O&M COSTS                     $557               $45   �

�  ENERGY COSTS                                $135,406           $10,865   �

�  REPLACEMENT COSTS                             $2,784              $223   �

�  LESS: REMAINING VALUE                  (          $0)    (          $0)  �

� --------------------------------------------------------------------------�

�  TOTAL LCC                                   $158,876           $12,749   �

�����������������������������������������������������������������������������

Figure 4.  BLCC Summary for DEMO1.

To generate a detailed LCC analysis for a project alternative once its input

data file and LCC output files have been saved, select "Print Detailed LCC

Report" from the main menu and use the cursor keys to highlight the LCC output

file to be used for this report.  An example of the detailed BLCC report for

the DEMO1 example file is shown in figure 5.

A year-by-year cash flow report can also be generated and printed from the

input data file by selecting "Compute Cash Flow Analysis" from the main menu.

An example of the cash flow report for file DEMO1 is shown in figure 6.  For

LCC analyses which require depreciation calculations, the depreciation

schedules used in the analyses are included at the end of the cash flow

report.

10

PART I - INITIAL ASSUMPTIONS AND COST DATA

Project name: demo1

Run date:01-10-1991 16:35:18

Comment: This is a BLCC demonstration file

Input data file: DEMO1.BCF, last modified: 01-10-1991/16:29:55

LCC output file: DEMO1.LCC, created: 01-10-1991 16:30:03

Study period:        20 years (1990 through 2009)

Plan/constr. period:  2 years (1990 through 1991)

Occupancy period:    18 years (1992 through 2009)

Discount rate:   5.0%  Real (exclusive of general inflation)

Run type: Generic LCC Analysis: No Tax Analysis

BLCC uses end-of-year discounting convention

INITIAL CAPITAL INVESTMENT COSTS (NOT DISCOUNTED)

(ADJUSTED FOR PRICE CHANGES DURING PLAN/CONST. PERIOD, IF ANY)

---------------------------------------------------------------------

YEAR    COST PHASING    YEARLY COST      TOTAL COST

----    ------------   -------------   -------------

1990        33.0%           $3,300

1991        33.0%           $3,300

AT OCCUPANCY:      1992        34.0%           $3,400

TOTAL INITIAL COST (ACTUAL)                                    $10,000

ENERGY-RELATED COSTS

---------------------------------------------------------------------

ENERGY      UNITS/     PRICE     DEMAND      TOTAL

TYPE        YEAR     ($/UNIT)    COST     P.V. COST

-----------  --------  ---------  -------    ---------

Electricity   100,000     $0.10        $0     $135,406

PART II  -  LIFE-CYCLE COST ANALYSIS

PRESENT VALUE      ANNUAL VALUE

(1990 DOLLARS)     (1990 DOLLARS)

--------------     --------------

A. CASH REQUIREMENTS AS OF OCCUPANCY

DURING CONSTRUCTION                           $6,443               $517

AT OCCUPANCY                                  $3,084               $247

------------       ------------

SUBTOTAL                                     $9,527               $764

C. OPERATING, MAINTENANCE & RELATED COSTS:

ANNUALLY RECURRING COSTS (NON-ENERGY)        $10,603               $851

NON-ANNUALLY RECURRING COSTS                    $557                $45

ENERGY COSTS                                $135,406            $10,865

------------       ------------

SUBTOTAL                                   $146,565            $11,761

D. REPLACEMENTS TO INITIAL CAPITAL ASSETS        $2,784               $223

F. REMAINING VALUE AT END OF OCCUPANCY    (          $0)     (          $0)

G. TOTAL LIFE-CYCLE PROJECT COST               $158,876            $12,749

______________________________________________________________________________

Figure 5.  Detailed BLCC report (example).

11

PROJECT NAME: demo1

COMMENT: This is a BLCC demonstration file

RUN DATE: 01-11-1991 08:50:55

INPUT DATA FILE: DEMO1.BCF, LAST MODIFIED

STUDY PERIOD:        20 YEARS (1990 THROUGH 2009)

PLAN/CONSTR. PERIOD:  2 YEARS (1990 THROUGH 1991)

OCCUPANCY PERIOD:    18 YEARS (1992 THROUGH 2009)

ANALYSIS TYPE: Generic LCC Analysis: No Tax Analysis

INITIAL CAPITAL COSTS

(AS INCURRED DURING PLANNING/CONSTRUCTION PERIOD AND AT OCCUPANCY)

TOTAL

(BY YEAR)

YEAR     ---------

1990          3300

1991          3300

1992          3400

TOTAL:     ---------

10000

CAPITAL INVESTMENT COSTS

INIT CAPITAL   CAPITAL     CAPITAL    TOTAL CAP.

YEAR    INVESTMENT  REPLACEMTS    DISPOSAL   INVESTMENT

1990         3,300           0           0        3,300

1991         3,300           0           0        3,300

1992         3,400           0           0        3,400

1993             0           0           0            0

1994             0           0           0            0

1995             0           0           0            0

1996             0           0           0            0

1997             0           0           0            0

1998             0           0           0            0

1999             0           0           0            0

2000             0           0           0            0

2001             0       5,000           0        5,000

2002             0           0           0            0

2003             0           0           0            0

2004             0           0           0            0

2005             0           0           0            0

2006             0           0           0            0

2007             0           0           0            0

2008             0           0           0            0

2009             0           0           0            0

TOTAL       10,000       5,000           0       15,000

_____________________________________________________________

Figure 6.  Cash flow report for file DEMO1.

12

OPERATING-RELATED COSTS DURING OCCUPANCY

YEAR   --OPERATING AND MAINTENANCE COSTS---      TOTAL

AN RECURRING  NON-AN REC      ENERGY   OPER. COST

----   ------------  ----------    --------   ----------

1992          1,000           0      10,612       11,612

1993          1,000           0      10,824       11,824

1994          1,000           0      11,041       12,041

1995          1,000           0      11,262       12,262

1996          1,000           0      11,487       12,487

1997          1,000           0      11,717       12,717

1998          1,000           0      11,951       12,951

1999          1,000           0      12,190       13,190

2000          1,000           0      12,434       13,434

2001          1,000       1,000      12,682       14,682

2002          1,000           0      14,230       15,230

2003          1,000           0      14,514       15,514

2004          1,000           0      14,805       15,805

2005          1,000           0      15,101       16,101

2006          1,000           0      15,403       16,403

2007          1,000           0      15,711       16,711

2008          1,000           0      16,025       17,025

2009          1,000           0      16,345       17,345

TOTAL        18,000       1,000     238,332      257,332

SUM OF ALL CASH FLOWS

CAPITAL   OPERATING        TOTAL

YEAR    INVESTMENT    COSTS           COST

----    ----------  ----------   ---------

1990         3,300           0       3,300

1991         3,300           0       3,300

1992         3,400      11,612      15,012

1993             0      11,824      11,824

1994             0      12,041      12,041

1995             0      12,262      12,262

1996             0      12,487      12,487

1997             0      12,717      12,717

1998             0      12,951      12,951

1999             0      13,190      13,190

2000             0      13,434      13,434

2001         5,000      14,682      19,682

2002             0      15,230      15,230

2003             0      15,514      15,514

2004             0      15,805      15,805

2005             0      16,101      16,101

2006             0      16,403      16,403

2007             0      16,711      16,711

2008             0      17,025      17,025

2009             0      17,345      17,345

TOTAL       15,000     257,332     272,332

______________________________________________________

Figure 6.  Cash flow report for file DEMO1 (continued)

13

3.4  Lowest LCC and Comparative Economic Analysis

After an LCC analysis has been computed for two or more mutually-exclusive

project alternatives, the life-cycle cost calculations for each alternative

can be compared to determine which has the lowest LCC.  Select "Find Lowest

LCC" from the main menu.  Highlight and tag the LCC output file for each

alternative to be included in the analysis.  Up to 100 files can be tagged

for a lowest LCC analysis.  The alternatives will be displayed in order of

increasing initial cost, and the alternative with the lowest LCC will be

flagged.  An example of the lowest LCC calculation for files DEM01, DEMO2,

DEMO3, and DEMO4 is shown in figure 7.

Note that for a comparative analysis to be performed, alternatives must have

been evaluated over the same study period and planning/construction period (if

any), using the same discount rate and the same analysis type (e.g., Federal

energy conservation projects, private sector taxable projects).  In general,

comparison of life-cycle costs is only meaningful when these costs are

representative of mutually-exclusive design alternatives in the same operating

context.  For example, the comparison of two alternative office building

designs with different shapes and orientation (and thus having different

energy consumption requirements), but with the same amount of usable space and

the same potential rental income, is a legitimate use of LCC analysis.

Comparison of the life-cycle cost of an office building with that of a

hospital is not likely to be useful in a decision-making context.

COMPARATIVE PRESENT-VALUE COSTS OF ALTERNATIVE PROJECTS

(Shown in ascending order of initial cost, * = lowest LCC)

PROJECT          LCC        INITIAL      LIFE CYCLE

NAME         FILENAME    COST (PV)      COST (PV)

-------------    --------   -----------   ------------

demo1            DEMO1           $9,527       $158,876

demo2            DEMO2          $10,479       $145,451

demo3            DEMO3          $11,432       $139,382*

demo4            DEMO4          $14,290       $139,532

_______________________________________________________________________

Figure 7.  Lowest LCC analysis for files DEMO1, DEMO2, DEMO3, and DEMO4.

A comprehensive comparative economic analysis of any two project alternatives

can be performed by selecting the "Comparative Economic Analysis" option on

the main menu.  Tag the two LCC output files corresponding to the project

alternatives to be compared.  The project alternative with the lower initial

cost is automatically designated as the base case; the project alternative

with the higher initial cost is designated as the alternative case for which

the economic measures are computed.  A summary of the major cost elements (in

14

present-value dollars) for the two alternatives will be displayed side by

side, and the difference shown.  The net savings, savings-to-investment ratio

(SIR), and adjusted internal rate of return (AIRR) are shown for the

alternative case relative to the designated base case.

In general, if a project alternative is to be considered cost effective on a

life-cycle cost basis, it must have a lower LCC than the base case, positive

net savings, an SIR greater than 1.0, and an AIRR greater than the opportunity

cost of capital (i.e., the discount rate).  These are all consistent measures

of economic analysis; that is, they will all show that a given project

alternative is either cost effective or not cost effective relative to the

specified base case.

If a single project is to be evaluated with BLCC to determine whether or not

it is economically justified, it must be compared against a "do-nothing" base

case.  An input data file must be created for the do-nothing case as well as

for the project.  This input data file will not generally include initial

costs, but will include replacement costs, O&M costs, and energy costs

reflecting the existing (before capital improvement) situation.  The same base

year, study period, and discount rate must be used for both cases.  For

example, to evaluate the cost effectiveness of storm windows added to an

existing house, a separate input data file is required for the base case (no

storm windows) without capital investment costs and with higher annual energy

costs over the period of analysis than the storm window alternative.

15

4.  REPORTS GENERATED BY BLCC

Six different type of reports can be generated by the BLCC program.  These

reports can be displayed directly to the screen, sent to the printer, or saved

to a disk file in the directory designated in the "Setup" option.  Table 2

shows the six report types and the corresponding filename extension

automatically tagged to the file when the reports are saved to a disk file.

Table 2.  Reports Generated by BLCC

______________________________________________

Filename

Report Types            Extension

______________________________________________

Input Data File report   .DTS

Summary LCC report       .SUM

Detailed LCC report      .RPT

Cashflow report          .CFR

Comparative LCC analysis .CMP

Lowest LCC report        .LOW

______________________________________________

The longer reports generated by BLCC, including the input data summary report,

detailed LCC report, cash flow report, and comparative economic analysis

report, do not have page breaks for printing purposes.  Instead, the reports

are printed continuously, assuming fan-fold paper output.  If you wish to

print these reports in presentation quality, save them to the disk and

retrieve them into a word processor as a text file.  You can then format them

for your specific printing requirements.  A form-feed control character is

sent to the printer at the end of each report to force a new page at that

point.

16

5.  GENERALIZED INPUT DATA FILE STRUCTURE FOR BLCC

When creating or modifying an input data file for life-cycle cost analysis,

data input is accomplished by choosing among options from a menu or by

entering data into a "form" displayed on the screen.  Figure 8 shows an

example of such a form for the general project data.

PROJECT DATA:

�����������������������������������������������������������ͻ

� Project title: Storm windows                              �

� Base year (1990-2000): 1991                               �

� Study period (2-60): 20 Years                             �

� Planning/Construction period (0-9): 0 Year(s)             �

� Discount rate:  7.00%                                     �

� Comment: This run is for demonstration purposes only      �

�����������������������������������������������������������ͼ

Figure 8.  Project data input form.

Input data is generally requested in the order shown in table 3.  Definitions

for each of the variables requested while creating or modifying an input data

file are provided in section 8 of this User's Guide.  If your response is

above or below the allowable range, the allowable range will be displayed at

the bottom of the screen.  If the input requested is mandatory, you will not

be allowed to continue until a valid response has been entered.

The project type and the inflation treatment can be specified only when

creating an input data file.  These two variables are fixed.  That is, they

cannot be changed in the file edit mode.

Note that percentages are entered as percentages, not as decimals (e.g., 7.0,

not 0.07).  Do not include signs such as $ or % with your entries.  Do not use

commas for marking off thousands or millions.

17

Table 3.  Input Data File Structure

_____________________________________________________________________________

Fixed Data:

Type of Analysis

Inflation Treatment (Constant dollars or Current dollars)

Project-Level Data:

General LCC Assumptions

Tax-Related Data (private sector only)

Capital-Asset Data (may be divided into six major components):

Initial Cost Data

Cost Phasing (if Planning/Construction period specified)

Depreciation Data (project with full tax analysis only)

Annually Recurring O&M Costs

Non-annually Recurring O&M Costs

Replacements:

Timing and Cost

Depreciation Data (project with full tax analysis only)

Energy-related Data:

Types, Annual Usage, and Cost

Escalation Rate Projections

Energy Use Indexes

Mortgage Data (non-Federal only):

Temporary Construction Financing (if Planning/Construction period

is specified)

Permanent Financing at Occupancy

_____________________________________________________________________________

18

6.  STANDARD DEPRECIATION TABLES

When creating or modifying an input data file that requires depreciation data,

several types of depreciation calculations can be specified, including

straight-line, sum-of-years digits, accelerated depreciation, and standard

table computations.  If the standard table option is specified, the

depreciation computations are based on depreciation factors retrieved from a

disk file previously prepared for this purpose.  This allows you to access

standard depreciation tables without having to re-enter them for each

component and each project.  (IRS publishes such tables for different classes

of depreciable assets.)  Standard depreciation tables can be used repeatedly

without typing in lengthy schedules for each component in each project.  The

same tables can be used with both initial capital components and capital

replacements.

Before running BLCC, you must create a file of depreciation factors using a

text editor (or word processor in the text mode) and save it as an ASCII file.

Files with a depreciation factors must have the filename extension ".DEP".

This file must be accessible from the BLCC directory (not the data file

subdirectory specified in the "setup" option).

Depreciation tables have the following format:

n

factor 1

factor 2

.

.

.

factor n

where:

n = number of factors in file (i.e., the number of years over

which the asset is to be depreciated), and

factor i = percent of asset cost allowed as depreciation in year i, i = 1

to n, in decimal form (e.g., 10% = .1).  The sum of these

factors must equal 1.0.

Figure 9 shows an example of such a file.  This file contains the number of

years that an asset is to be depreciated and the percent (in decimal terms) of

its depreciation basis that is allowed as depreciation in each of those years.

Separate tables must be created for each depreciation schedule to be

retrieved.  Note:  the actual depreciation schedules used in the BLCC analysis

are printed at the end of the cash flow report for any project.

5

.2

.2

.2

.2

.2

_____________________________________________________________________

Figure 9.  Example of an ASCII file of standard depreciation factors.

19

7.  COMPUTATIONAL CONVENTIONS USED IN BLCC

All time periods in BLCC are assumed to be integer (whole) years, including

the study period, planning/construction period, and occupancy period.  Initial

costs and property taxes (if any) are assumed to be incurred at the beginning

of the designated year.  All other costs are assumed to occur at the end of

the designated year and are discounted from that time, with one exception:

mortgage payments with more than one payment per year are assumed to occur at

times which correspond to the number of payments per year designated. (That

is, if 12 payments per year are designated for a mortgage loan, a mortgage

payment is assumed to be made at the end of each month in the year and each

payment is discounted from that time.)  All tax savings (if any) are assumed

to occur at the end of the designated year, regardless of when that savings

was generated.

Extensive documentation of the computational methods used in BLCC can be found

in Appendix H of Comprehensive Guide for Least-Cost Energy Decisions, by

Rosalie T. Ruegg and Stephen R. Petersen, NBS Special Publication 709,

National Bureau of Standards, Gaithersburg, MD, 1987.

20

8.  DEFINITIONS AND ACCEPTABLE RANGES OF INPUT VARIABLES

USED IN THE BLCC PROGRAM

General LCC assumptions:

o  Type of analysis--Choose the type most appropriate to your project:

(1) Federal Analysis:  FEMP (Federal Energy Management Program) projects

related to energy conservation in federal facilities.

(2) Federal Analysis:  OMB A-94 projects (most other federal government

construction projects, but not water resource projects).

(3) Private Sector:  Taxable projects such as commercial and industrial

buildings and residential rental properties.

(4) Private Sector:  Owner-occupied houses (where only mortgage interest

and property taxes are tax-deductible from federal and state income

tax).

(5) Generic LCC Analysis:  LCC projects which do not need to conform to

Federal rules and do not require income tax analysis.

o  Exclude Inflation or Include Inflation (Generic LCC analysis only)--

If the analysis is to be performed in constant dollars, the discount

rate, mortgage interest rates, and all price escalation rates must be

entered in real terms (i.e., excluding general inflation).  If the

analysis is to be performed in current dollars, these rates must be

entered in nominal terms (i.e., including general inflation).  For

Federal projects, the analysis is always performed in constant dollars

and thus these rates are entered in real terms.  For private sector

analyses (taxable and owner-occupied house), the analysis is always

performed in current dollars and thus these rates are entered in nominal

terms.  Note:  Regardless of whether the analysis is performed in

constant or current dollars, all cost data must be entered in base-year

dollars.

Project-level Data:

o   Project Title--The user may supply any name desired to identify the

project to be evaluated.  Up to 12 characters may be used.  A distinct

project title should be assigned to each project to be evaluated since

the comparative analysis will list the costs of each project under this

title.

o   Base Year--The year in which the study period begins.  This must be an

integer between 1990 and 2000.  All cost data must be entered in

constant dollars as of this date.  All costs will be discounted back to

this date.

o   Study Period (Years)--The number of years over which project-related

costs will be summed.  The Study Period includes both the planning/

construction period, if any, and the Occupancy Period.  (Integer between

2 and 60)

21

o   Planning/Construction Period--The number of years from the beginning of

the Study Period to the beginning of occupancy.  Initial costs can be

phased in during the Planning/Construction Period, but operating and

maintenance costs (including energy), replacement costs, and

appreciation (if any), are not incurred during this period.  (Integer

between 0 and 9)

o   Discount Rate (%)--The discount rate is used to convert cash flows

occurring at different points in time to present value (as of the base

date) and its equivalent annual value.  Use the nominal (i.e., inclusive

of inflation), after-tax discount rate for private sector projects or a

Generic LCC analysis designated as using current dollars.  Use a real

(i.e., exclusive of inflation) discount rate for Federal projects and

Generic LCC analyses designated as using constant dollars.  (0 to 25)

o   Comment--Additional information to identify this project can be included

in the data file and printed out with the various BLCC reports.

Optional.  (Up to 50 characters)

Tax-related Data (Private sector analyses only):

o   Marginal Federal Income Tax Rate (%)--The Federal income tax rate

applied to the last dollar of income to the investor.  (0 to 50)

o   Marginal State Income Tax Rate (%)--The state tax rate applied to the

last dollar of income to the investor.  If income taxes are imposed by

local jurisdiction, this can be included in the analysis by replacing

the state tax rate with a composite state and local rate.  Note:  BLCC

computes a composite income tax rate from the marginal Federal and state

income tax rates, based on the assumption that state and local income

taxes are deductible from Federal income tax.  All tax deductions are

assumed to be the same for both Federal and state taxes.  (0 to 20)

o     Property Tax Rate (Nominal, %)--This is the quoted property tax rate for

this type of property in this location.  Note that the property tax rate

is assumed to be the same for all capital components, but that the

percentage of each component that is subject to this property tax rate

(i.e., the assessment rate) can be specified separately for each

component.  (0 to 20)

o     Capital Gains Adjustment Factor (%)--Use 100 percent if there is no

capital gains preference.  At the end of the study period, if the resale

value of the project is greater than its initial cost, the difference is

assumed to be taxable as long-term capital gains.  The effective capital

gains tax may be lower than the ordinary income tax rate (although this

was not the case at the time of this writing.)  The Capital Gains

Adjustment Factor adjusts the ordinary income tax rate to an effective

capital gains tax rate.  (For example, in the early 1980's this factor

was 40 percent for individuals; that is, the marginal tax rate of the

individual was multiplied by 40 percent to arrive at an effective

capital gains tax rate.)  Note:  For owner-occupied houses, the capital

22

gains tax is usually deferred indefinitely if another house is purchased

within a specified amount of time.  To defer capital gains taxes

indefinitely, enter a zero for this factor.  (0 to 100)

o   Depreciation Recapture Code--At the end of the study period, if the

resale value of the project is greater than its depreciated book value

(i.e., the remaining undepreciated portion of its actual cost at

occupancy), the difference (up to the total amount of depreciation

claimed) may be subject to a depreciation recapture tax.  This

depreciation recapture tax can be computed in one of several different

ways, as designated by the following input codes:

0--no depreciation recapture

1--recapture as capital gain

2--recapture as ordinary income

3--depreciation calculated at straight-line rates, recaptured as

capital gains; any excess depreciation (due to accelerated

calculation procedures), recaptured as ordinary income

Note:  IRS rules for recapture vary with the type of property involved,

method of depreciation used, and date of asset acquisition.  If there is

no capital gains preference (i.e., the capital gains adjustment factor

is 100 percent), codes 1,2, and 3 all produce the same results.

o   Depreciation Basis Adjustment Factor (%)--If an investment (or energy)

tax credit is claimed for any component, this may reduce the original

cost basis for depreciation calculations by a specific percentage of the

tax credit.  For example, if one half of the tax credit must be deducted

from the original cost basis for depreciation, enter 50.  If no

investment tax credit is to be claimed for capital assets (as described

below), enter 0.  (0 to 100)

Capital Asset data:

o   Capital components--The user must specify whether the capital assets of

this project will be subdivided into specific components or systems

(e.g., land, building, HVAC equipment, etc.).  If the capital assets are

to be subdivided, initial costs, operating and maintenance costs (O&M),

replacement costs, and depreciation (if any) must be assigned to the

corresponding component.  In general, for analyses which include

depreciation calculations, capital assets should be differentiated by

the type and length of depreciation schedule that is to be used.  (For

example, land, which cannot be depreciated, should be entered as a

separate component.)  Price escalation rates for initial costs and O&M

costs are specified separately for each component.  Thus, if these rates

are expected to differ significantly among the various capital assets of

the project, these assets can be divided into separate components.  Up

to six capital components can be specified for any project.  (Integer

from 1 to 6)

23

(Note:  Energy and mortgage financing costs are assigned to the overall

project and not to specific components.)

o   Component names--if the capital assets are to be subdivided into

components, a name must be assigned to each component before data entry

for those components can begin.  Components can also be added or deleted

at this point.  (Names can have up to 12 characters.)

The following variables are to be specified for each component or for

the overall project if not subdivided into components:

o   Initial Cost ($)--The total installed cost of the component, unadjusted

for price escalation during construction.  If the Planning/Construction

Period is one year or more, the "Cost-Phasing Schedule" and the "Average

Price Escalation Rate During the Planning/Construction Period"

(discussed below) are used to determine the actual component cost

incurred in each year of the Planning/Construction Period and at

occupancy.  (0-99999999)

o   Expected Life (Years)--The useful life of the component (in years)

anticipated by the investor.  This is intended to be a realistic

assessment of the component life, independent of the depreciation life

assigned to the component for income tax purposes, if any. (Integer

between 2 and 100)

o   Resale Value Factor (% of initial cost)--This is the percent of the

initial component cost which remains at the end of the study period (or

at the end of the component life, if this comes first), unadjusted for

price escalation.  This factor is intended to reflect a decrease in

value due to wear, deterioration, or obsolescence, but not inflation or

price escalation.  In computing the actual resale value of the

component, BLCC uses the Average Price Escalation Rate During Occupancy

(see below) to adjust for price changes during the Study Period.  The

resale value is treated as a negative cost element in the life-cycle

cost equation even if the investor does not intend to dispose of the

property at the end of the Study Period.

o   Average Price Escalation Rate During the Planning/Construction Period

(%)--This is the average annual rate at which the cost of this component

increases during the Planning/Construction Period, if any.  No

adjustment is made to component costs phased in during the first year of

construction since component costs are assumed to be incurred on the

first day of the assigned year.  All component costs phased in after the

first year of the Planning/Construction Period are adjusted using this

rate.  If the study is performed in constant dollars (Federal analyses

and designated Generic LCC analyses), this rate should not include

general inflation.  (-5 to 20)

24

o  Average Price Escalation Rate During Occupancy (%)--This is the average

annual rate at which the cost of this type of capital component is

expected to change during the occupancy period.  This variable is used

in calculating replacement costs for this component, and in establishing

the resale value of this component after adjustments for wear,

deterioration, and obsolescence.  If the study is performed in constant

dollars (Federal analyses and designated Generic LCC analyses), this

rate should not include general inflation.  (-5 to 20)

o   Tax Credit Factor (% of initial cost)--This factor represents the

percent of the initial cost of the component (or project, if no

components are specified) which can be claimed as an income tax credit

in the first year of occupancy.  (If the tax credit is taken, the

depreciation basis may need to be reduced by some percentage of this

credit.  The percentage of the tax credit by which the depreciation

basis is to be reduced can be specified using the Depreciation Basis

Adjustment Factor, described above.)  (0 to 20)

o   Property Tax Assessment Factor (%)--This is the percentage of the

initial component cost to which the Property Tax Rate specified above

for the project is applied in order to calculate the actual property tax

obligation.  Property taxes are calculated only during occupancy years,

and are assumed to be paid at the beginning of each year, based on the

assessed value of that component at that time.  In computing the

assessed value of the component for each year of occupancy, the initial

component cost is decreased annually--in straight-line fashion--to its

estimated resale value (unadjusted for price changes) at the end of the

study period (or at the end of the component life, if this occurs before

the end of the study period).  These values are then adjusted for price

changes using the "Average Price Escalation Rate During Occupancy," and

then adjusted by the Property Tax Assessment Factor to determine the

amount to which the Property Tax Rate is applied.  Note that this is

necessarily an approximation technique; the actual assessment depends on

the taxing authority's own assessment methods.  (0 to 100)

o   Depreciation Method Code--The method to be used for depreciating each

capital component is determined by the following codes:

0--no depreciation calculated

1--straight line

2--declining balance (accelerated)

3--sum of years digits

4--depreciation table

o   Cost phasing by year--if a Planning/Construction Period is used, a

specified percentage of initial capital costs can be allocated to each

year and to the first day of occupancy.  If these factors do not add up

to 100 percent, the unallocated percentage will automatically be

assigned to the first day of occupancy.  (0 to 100 for each year, with

total for all years = 100)

25

Depreciation data when Depreciation Method Code 1, 2, or 3 is specified (See

section 6 of this report for information regarding the depreciation table

method):

o   Depreciation Life (Years)--The number of years over which the component

will be depreciated.  (2-45)

o   Depreciation Acceleration Rate (%)--If Depreciation Method Code 2, the

declining balance method, is selected, the Acceleration Rate must be

specified.  For example, for 125 percent declining balance, enter 125.

(100 to 300)

o   Salvage Value Factor (% of initial cost)--If the IRS depreciation rules

require that salvage value be deducted from the initial asset cost

before calculating depreciation, a Salvage Value Factor must be

specified.  Otherwise enter zero.  The Salvage Value Factor is specified

as a percentage of the initial component cost.  Negative values are not

allowed.  Note:  The Salvage Value Factor is specified only for

depreciation purposes here; this is not the same as the Resale Value

Factor.  (0 to 50)

o   Additional First Year Depreciation Factor--If additional first year

depreciation is allowed, enter this factor as a percent of the actual

component cost.  Additional first year depreciation is deducted from the

original depreciation basis before the depreciation schedule is set up

for this component and added to the first year of the depreciation

schedule.  (0 to 20)

Operating and Maintenance (O&M) Costs (other than energy costs) are specified

for each component if capital assets are subdivided into components.

Otherwise these costs are allocated to the overall project.  There are two

types of O&M costs, annually recurring costs (ARCs) and non-annually recurring

costs (NARCs):

(1) Annually-recurring O&M costs occur in substantially the same amount

every year or change at a relatively constant rate over the study period

(in base year dollars).  Two data entries are needed:

o     Amount (in base year dollars).

o     Average Annual Rate of Increase--This is the average annual rate

at which both annually-recurring O&M costs and non-annually-

recurring O&M costs are expected to change over the study period.

If Federal analysis or Generic LCC analysis in constant dollar

terms, enter real rate (net of general inflation); otherwise enter

nominal rate (including general inflation).  (-5 to 20)

26

(2) Non-annually-recurring O&M costs do not follow a regular pattern.  These

costs must be specified individually for each occurrence.  (More than

one occurrence per year is permitted though all occurrences within a

year are assumed to occur at the end of that year.)  For each NARC, the

following pair of data entries must be specified:

o     Year of Occurrence--The year during occupancy in which the NARC

is incurred (e.g., 5 for the fifth year).  If the year entered is

beyond the end of the study period, the NARC will be ignored.  (1

to 60)

o      Amount (in base-year dollars).

Capital Replacements:

o     Year of Replacement--The year during occupancy in which the

replacement cost is incurred (e.g., 5 for the fifth year).  If the

year entered is beyond the end of the study period, the

replacement cost will be ignored.  (2 to 60)

o     Initial Cost of Replacement--The installed cost of the replacement

(in base year dollars).  The actual replacement cost will be

calculated using the Average Price Escalation Rate During the

Planning/Construction Period (if any) and the Average Price

Escalation Rate During Occupancy assigned to the original

component.

o     Expected Life (Years)--The useful life of the replacement (in

years) anticipated by the investor.  (2 to 99)

o     Resale Value Factor (% of initial replacement cost)--This is the

percent of the initial replacement cost remaining at the end of

the study period (or at the end of the expected useful life of the

replacement, whichever is less) in base year dollars, i.e.,

unadjusted for inflation or price escalation.  The Average Price

Escalation Rate During the Planning/Construction Period (if any)

and the Average Price Escalation Rate During Occupancy are used

with this factor to compute the actual resale value.  (0-200)

o     Property Tax Assessment Rate (%)--The percent of the initial

replacement cost to which the nominal property tax rate will be

applied.  (0 to 100)

o     Depreciation Method Code--See Depreciation Codes for capital

assets above.  The same codes and depreciation tables are used for

replacements. (0 to 4)

27

Energy Costs

Annual energy costs are treated like annually recurring costs except that up

to four individual energy types may be specified and the rate of price change

can vary during the study period.  In addition, annual consumption of each

energy type can vary from year to year during occupancy, as a function of the

energy use index for that year.  (Energy costs are not calculated during the

Planning/Construction Period, if any.)  Annual energy costs consist of direct

energy costs (i.e., those costs computed directly from annual energy

consumption and corresponding unit prices) and indirect costs (e.g., demand-

related or other energy costs that cannot be computed directly from the amount

of annual energy consumption).

Energy-related variables are entered as follows:

o   Energy Type Code (designated units)--choose one:

1--Electricity (kWh)

2--Distillate Fuel Oil (gallons)

3--Residual Fuel Oil (gallons)

4--Natural Gas (therms, 100,000 Btu)

5--Liquified Petroleum Gas (pounds)

6--Coal (tons)

7--Other (unspecified)

Note:  The same energy type can be entered more than once for a given

project.  For example, electricity usage subject to winter kWh rates

might be entered separately from electricity usage subject to summer kWh

rates.

o   Annual Energy Consumption (in designated units)--The base annual energy

consumption of the specified energy type (unadjusted by the Energy Use

Indexes).

o   Energy price ($/Unit)--The price per designated unit to be used in

calculating direct energy costs for the project.  This price should be

entered in base-year dollars.

o   Annual Demand (or Other Fixed) Energy Charge ($/yr)--Annual costs

related to energy consumption but not directly accounted for in the

energy prices used to compute direct energy costs.  Example:  Charges

for peak kilowatt demand, summed over the entire year.  This cost should

be entered in base-year dollars.  This cost is assumed to increase

annually at the same escalation rate specified for the energy price.

o   Price Escalation Method--The user has two alternatives for specifying

rates of energy price increases:

(1)   Escalation rates will be entered by the user.

(2)   Escalation rates will be automatically retrieved from

ENCOSTXX.RAN, a disk file with DOE projections of energy price

28

escalation rates that is distributed with BLCC.  (XX represents

the year in which the file is released, e.g., ENCOST93.RAN was

released in fall, 1992.)  Once escalation rates have been retrieved

from ENCOSTXX.RAN, they can be modified by the user by backing up

to the energy data screen (press <PgUp>) and respecifying the Price

Escalation Method as 1.  The DOE projections will still be active

until modified.

If energy price escalation rates are to be entered by the user, up to 50

discrete escalation rates and the number of years of duration for each

rate can be specified.

o     Duration (Years)--The number of consecutive years that the

corresponding energy price escalation rate is to be used.  Note:

For each energy type, the sum of these duration periods must equal

or exceed the study period.  (0-60)

o     Energy Price Escalation Rate (%)--This is the average annual rate

of price increase for this energy type during the designated

duration period.  If the analysis is to be performed in real terms

(Federal analyses and some generic LCC analyses, this rate should

be in real terms, i.e., net of general inflation.)  (-10 to 20)

Note:  Only 50 Energy Price Escalation Rates can be displayed on the

screen.  If each rate is given a duration of one year (as in the

case when DOE rates are used), and the study period exceeds 50

years, the last years will all be given the same rate used for the

50th year.

If DOE projected Energy Price Escalation rates are to be retrieved from the

disk, the following input variables are needed to select the appropriate data

from the file:

o   Rate schedule type--The type of utility rate schedule used for the

project.  Choose one:

(1) residential    (2) commercial    (3) industrial

o   State abbreviation (2 characters)--The official U.S. Postal Service

abbreviation for the state in which the project is to be located.  This

code is used to determine in which of the (currently) four DOE regions

the project will be located.

o   General Inflation Rate--The projected rate of general price inflation

over the study period.  This is required only when the LCC analysis is

performed in current dollars (private sector and designated Generic LCC

analyses only).  This rate is used to convert DOE projections of future

energy price increases from real (excluding inflation) to nominal rates.

However, it should be consistent with the user's projection of general

inflation implicit in the discount rate, escalation rates used for

capital assets and O&M costs, and mortgage interest rates, if any.

29

Annual Energy Consumption Patterns--After energy price escalation rates are

entered (or retrieved from the disk), the pattern of annual energy consumption

can be specified as constant or variable.  If energy consumption is expected

to be approximately the same from year to year, specify "constant".  If the

amount is expected to change in some predictable manner (e.g., anticipated

switch in heating fuels in the year 2000), specify "variable".  If "variable"

is selected, up to 50 discrete energy use indexes and the number of years of

duration for each index can be specified separately for each energy type:

o   Duration (Years)--The number of consecutive years that a corresponding

Energy Use Index is to be used.  For each energy type, the sum of these

duration periods must equal or exceed the occupancy period.  Note:

These duration periods begin with occupancy (since energy consumption

calculations begin with occupancy), while the duration periods for

energy price escalation rates begin with the start of the study period

(since all prices are entered in base year dollars).  (0 to 60)

o   Energy Use Index (%)--This is the average rate of energy use during the

corresponding duration period, as a percent of the Annual Energy

Consumption specified for that energy type.  An index of 100 percent

leaves the Annual Energy Consumption unchanged for the corresponding

years.  (0 to 999)

Finance-related data (non-Federal projects only).  If borrowing costs are an

integral part of the project economics, these can be incorporated into the LCC

analysis.  If there is a Planning/Construction Period, temporary financing can

be specified.  Temporary financing is assumed to expire at occupancy.

Permanent financing begins with occupancy; one or two permanent mortgage loans

can be specified.  Data for temporary financing during Planning/Construction

Period (if any) and for permanent financing are entered on separate screens.

Note:  Adjustable-rate mortgages (ARMs) cannot be computed by BLCC.  However,

since the underlying rate of general inflation is assumed to be constant

throughout the study period, the assumption of a constant interest rate over

the same period of time is reasonable.

Data requirements for temporary and permanent mortgages are essentially the

same, except that the life of the loan does not need to be specified for

temporary financing.  (It is assumed to end at the date of occupancy.)

Financing-related variables (non-Federal projects only):

o   Percent of Initial Project Cost Borrowed--Initial project cost is the

sum of all component costs (if any), including inflation adjustments

during construction.  During the Planning/Construction Period, this

factor is assumed to be the same in each year.  For permanent financing,

the sum of these factors for two loans cannot exceed 100 percent.

30

o  Loan Type Codes (choose one):

(1) fully amortized loan with periodic payments of equal amount

(permanent financing only),

(2) interest only loan with periodic interest payments and repayment of

principal at end of loan life, and

(3) both interest payments and principal repayment deferred to end of

financing period.

o   Annual Interest Rate (%)--If the analysis is to be performed in constant

dollars (designated Generic LCC analyses only), the annual interest rate

should be entered in real terms (net of general inflation).  A nominal

interest rate should be specified, without compounding effects or loan

placement costs considered (i.e., the interest rate to be used is not

the effective interest rate or "APR").  (0 to 25)

o   Number of Payments per Year--This is the number of equal payments made

at periodic intervals through the year.  (Integer between 1 and 52)

o   Points Paid (% of loan amount)--This is a loan placement fee assessed as

a percentage of the loan amount (i.e., one "point" equals one percent of

the loan amount).  (0 to 6)

o   Life of Loan (Years)--The loan life can be longer or shorter than the

study period.  If it is longer, any remaining principal and accrued

interest at the end of the study period are assumed to be paid at the

end of the study period.  (Integer between 2 and 40)

31

Index

additional first-year depreciation, 26

annual demand energy charge, 28

annual energy consumption, 28

annual interest rate,  31

annually recurring costs,  26

average price escalation rate during planning/construction,  24

base year,  21

calculating life-cycle costs,  10

capital asset data,  23

capital components,  23

capital gains adjustment factor,  22

capital replacements,  27

comparative economic analysis,  14

component names,  24

cost phasing,  25

definitions of variables,  21

depreciation acceleration rate,  26

depreciation basis adjustment factor,  23

depreciation codes:

capital components,  25

replacements,  27

depreciation life,  26

depreciation recapture code,  23

depreciation tables,  19, 25

discount rate,  22

energy,  28

energy use index,  30

energy consumption,  28

energy consumption pattern (see 'energy use index')

energy costs,  28

energy price,  28

energy rate schedule type,  29

energy types,  28

expected life:

capital components,  24

replacements,  27

federal income taxes,  22

finance-related variables,  30

general inflation rate,  29

general LCC assumptions,  20, 21

income taxes,  22

inflation,  21, 29

initial cost:

capital components,  24

percent borrowed,  30

replacements,  27

input data file structure,  18

32

installation,  4

loan life,  31

loan type,  31

lowest LCC,  14

main menu,  5

marginal federal income tax rate, 22

marginal state income tax rate,  22

non-annually recurring costs,  27

number of payments per year (loan),  31

operating and maintenance (O&M) costs,  26

percent of initial project cost borrowed,  30

planning/construction period,  22

points (loan),  31

price escalation rates:

during occupancy,  25

during planning/construction,  24

energy,  29

O&M,  26

price escalation method (energy prices),  28

project-level data,  21

project title,  21

property tax assessment factor:

capital components,  25

replacements,  27

property taxes,  22

rate schedule type (energy),  29

replacements,  27

reports,  16

resale value:

capital components,  24

replacements,  27

salvage value factor,  26

standard depreciation tables,  19, 25

state income taxes,  22

study period,  21

tax credit factor,  25

type of analysis,  21

year of replacement,  27

33