About BBS Message Headers.

There seems to be a general mis-understanding of the function

of the message headers. Here is a little history that may clear things up.

Originally there were no message headers.

This caused some problems: it was very hard to diagnose

network failures without the audit trail of the path the mesage

took though the network. So, in mid-1984, I added the header.

In 1985, the ARRL gave a demonstration of automatic message forwarding

to the FCC. One of the main points presented to the FCC was that the message

headers allowed for FULL TRACING of who handled the message, and

when it was transmitted by each station. This automatic audit trail

was one of the major arguments in favor of automatic control of packet

BBS stations, and helped to result in Docket 85-105, and eventually

the HF STA, and thus packet radio as we know it today.

As time went on, many people discovered that they could learn a

great deal about the functioning of the network by looking at

the message headers. Some wrote software to do this automatically.

The WP server is one example. It uses the information contained

in the message headers to help build and maintain it's distributed

database system.

People are now working on software that will automatically determine

the entire network map from the information contained in message

headers. Some day, this will allow us to build our BBS routing

tables AUTOMATICALLY. As we all know, maintaining routing tables

is one of the nasty chores for a BBS sysop!

Two things must happen for this all to work:

1. It is MOST important that the information in the message header

not be altered as it passes through the various BBS on it's path.

2. The header must be in a standard format, so that the software

that decodes it can operate correctly.

3. The message headers are separated from the body of the message

by a blank line.  Anything that comes before the first blank

line is a header, whether in correct format or not.	Anything

that comes after the first blank line is the message body.

The format of these headers has evolved through many changes.

In 1987, NK6K proposed a standard to be used by all BBS authors.

Most followed this standard, or followed a slightly different

standard used originally by WA7MBL.

The following is a standard "nk6k" header:

R:920527/0507 @:W0RLI.OR.USA.NA West Linn #:6031 Z:97068

Required: The "R:" identifies the line as a standard header.

Required: Date and time the message arrived at this bbs, in GMT.

Required: BBS.

Optional: Hierarchical location. Strongly recommended.

Required: Message number.  Greater than zero, less than 65536.

Optional: QTH, which is all text after the hierarchical location

and before the message number field.

Optional: The last field is the ZIP or postal code.

Some software allows an optional time zone identifier after the time.

This is not required, and is ignored by most bbs software.

The following is a standard minimum header, as proposed by KA2BQE:

All fields are required.

R:951115/0629 3456@W0RLI.OR.USA.NA