ENLARGER.DOC                        by:  Ira F. Kavaler - May, 1987

Version 1.214 - 2/21/94

Darkroom work.

--------------

This program can be used WITH CAUTION in a darkroom while printing

negatives on photographic paper.  It is used with an enlarger,

enlarging photographic light meter (or projection print scale or

photographic test strip print), and enlarger timer to determine the

correct exposure for a print when one or more of the photographic

variable have been changed after the initial exposure measurement (or

test print) had been made.  (Read the WARNING.DOC file before using

your computer in your darkroom.  It discusses radiation from your

monitor which may inadvertently expose the photographic paper.)

Using the program.

------------------

Use whatever method you currently use to make your exposure

measurement.  I strongly suggest that you purchase an enlarging

photographic light meter.  I got a Patterson unit twenty years ago, and

it has saved me a lot of time, photographic paper and money!  The first

print I make is acceptable 99-44/100 percent of the time!

Enter the three variables for the exposure (magnification, lens

aperture, and exposure time) into the program as the measured exposure.

Magnification.

--------------

The magnification is the ratio of the print height (or width) to

the negative height (or width).  (Don't mix heights and widths;  use

one or the other, not both!)  For 35mm negatives the height is 24 mm.

(approximately one inch) and the width is 36 mm.  (35mm refers to the

film format, which is the width across the film strip, from edge to

edge.)

This measurement need not be accurate.  If you use one type of

film exclusively, you can calibrate your enlarger column and head

setting for magnification by experimentation.  For example, with a 35mm

negative in the negative carrier of the enlarger (the image measures

approximately one inch in height or width (actually vs. 25.4 mm. equals

one inch), set the enlarger head to a four inch high image;  this would

be equivalent to a four times magnification, 4" / 1" = 4x.  Remember to

focus the enlarger before measuring the image (the image size changes

during focusing).  Mark the head and column to indicate 4x

magnification.  Repeat this process for other magnifications: 3.5x, 5x,

6x, 7x, 8x, ... 16x, 17x, 18x, 19x, 20x, etc.  Keep a yard stick handy

for those off-the-baseboard, hang-the-paper-on-the-wall very large

enlargements.  The magnification setting in the program is adjustable

in 1/2 unit increments.

Due to the inverse square law in physics, when the distance from

the light source to the image is doubled, the brightness of the image

is decreased by a factor of four.

Lens aperture.

--------------

I have included lens apertures (f/stops) from f/0.7 to f/64, in

one f/stop increments.  This increment corresponds to the majority of

enlarging lens aperture detentes.  Remember that the experts say that

you should try to exposure your prints with the lens set 2 f/stops open

from the maximum (smallest) f/stop on your lens;  for example, if you

have a lens that closes down to f/16, you should try to make your

prints at f/8 for maximum sharpness.  Remember that as you close down

the lens, your depth of field will increase along with your exposure

time;  going the other way, as you open the lens your exposure time

will drop but your image focusing will be more critical.  Here again I

strongly suggest that you purchase a relatively good enlarger focusing

aid;  they're relatively inexpensive.

The f/stop is the ratio of a lens focal length (while focused at

infinity) to the widest opening of the aperture of the lens.  Foe

example, a 50mm lens having a maximum aperture of f/2 will have an

opening of 25mm.  As the lens is opened up one f/stop the brightness of

the image will be doubled.

Exposure time.

--------------

Your enlarging meter will determine the correct time for the

exposure.  Without a meter you will have to make a test print of one

form or another, a waste of time, paper, chemicals, and money.  If you

did nothing else, you don't need my program!!!

But what if you want to make a very large print and your enlarging

meter doesn't have the light sensitivity to measure the faint image.

Now you need my program.  With an initial metering made at some

measurable magnification, you enter this information into ENLARGER.

Next you set up for the desired magnification, focus and measure the

image (say) height which yields the desired magnification.  You will

also have to probably open the lens aperture to get more light on the

paper, so note the desired aperture.

Enter the desired (altered, new) information into ENLARGER, and it

will tell you the desired exposure time.  You can easily continue to

change the aperture to get either the f/stop you feel is optimum or the

exposure time you can tolerate.

The time is adjustable for the measured exposure only, in one

second increments.  The time is computed for the desired exposure.

Exposure time accumulates the brightness of the image on the

photographic paper.

A little information is dangerous.

----------------------------------

If you learned about the physics of photograph from the negative

standpoint you probably were not introduced to reciprocity.  In

photography reciprocity is the ability of the photosensitive material

(negative, positive, or paper) to follow the rules of accumulating

light purely mathematically.  (What does that mean?)

When exposing negatives, if the aperture is closed one f/stop the

brightness on the negative is halved, and thus to maintain the same

exposure, the exposure time must be doubled.  For example, if the

original exposure was 1 seconds at f/16, the new exposure will be 2

seconds at f/22.  This is an example of reciprocity, halving the light

and doubling the time to keep the same exposure..

When exposing photograph paper reciprocity doesn't usually hold

true.  If you close the aperture one f/stop the exposure time must be

increased slightly more than double to maintain the same exposure.  The

increase is approximately 2.1 times.  For example, if the original

exposure was 10 seconds at f/4, the new exposure will be 21 seconds

(not 20 seconds) at f/5.6.  Notice that the time is more than doubled.

This is an example of a lack of reciprocity.

This same affect holds true for changing the magnification of the

image.  The program incorporates a mathematical compensation for the

lack of reciprocity.  Without this little explanation you might think I

had an error in the program.  My formulas were derived from analog

calculators contained in the Eastman Kodak Darkroom Data Guide, the

analog dial on my darkroom exposure meter, and my years of photographic

experience.

How to use the program.

-----------------------

After making the measured exposure, note the values of the

magnification to the nearest 1/2 unit of magnification.  Note the lens

aperture to the nearest full f/stop, and the proper exposure time to

the nearest second.

Execute the program and when the program output screen appears (it

has the large letters) you will see that the MEASURED EXPOSURE and

Magnification headings are highlighted.  When a heading is highlighted

that item can be set.  When the program starts a set of default

settings will be displayed.  These can be changed to suit your

enlarger;  I'll discuss that later.

The space bar will alternate the highlight between the MEASURED

and DESIRED exposure headings.  The left and right arrow cursor keys

will switch highlights within the row of items.  The up and down arrow

cursor keys will change the setting of the highlighted item in the

increments (or decrements) mentioned earlier.

Again, with the MEASURED EXPOSURE and Magnification labels

highlighted, use the up and down keys to set the magnification to the

value of your measured exposure.  Use the right arrow key to advance

the highlight to the Lens Aperture label and again use the up and down

keys to set the f/stop.  Once more use the right arrow key to highlight

the Exposure Time and with the up and down keys set the time of the

measured exposure.

Now go back to the enlarger and recompose your image changing the

magnification and/or lens aperture as you desire.  Come back to the

computer and press the space bar to highlight the DESIRED EXPOSURE.

Using the four arrow keys enter the new magnification (if you changed

it) and/or new lens aperture (if you changed it).

Throughout the entire process of enter data into the program the

proper intermediate values of the desired exposure time will be

continually computed and displayed.  Your desired exposure time is read

once you have entered all your other data.  You may change any settings

at any time and again read the newly computed exposure time.

Other options.

--------------

If you press the equal (=) key, the desired exposure data will be

made the same as the measured exposure data.  This is useful for

resetting the values.

If you set all the measured exposure data and have the MEASURED

EXPOSURE heading highlighted and press the CAPITAL S, the measured

exposure data and screen color will be saved in the start-up data file.

These setting value will be initially displayed each time you start the

program.

The screen text colors can be changed by pressing the page up and

page down keys.  The page up key will cycle the color upwards from 1

through 15 (black is excluded, and after color 7, the eight bright

colors are displayed).  The page down key will cycle the colors

downwards from dim white color 7 through color 1, black and the eight

bright colors, 8 to 15, are excluded.

I have included shifted, control, and alternate  up and down

arrow inputs to more quickly set both magnification and exposure time;

however, at present they operate only of Tandy series 1000 computers;

IBM compatible implementation will occur shortly!

Item             Up/Down      Shifted        Control        Alternate

-------------    -------      -------        -------        ---------

Magnification    +/- 0.5      +/- 1          +/- 5          no change

Lens Aperture    +/- 1        no change      no change      no change

Exposure Time    +/- 1        +/- 5          +/- 10         +/- 20

Here's the small print.

-----------------------

All versions of this program including its related files are being

distributed on an "AS IS" basis.  There is absolutely no stated or

implied guarantee or warrantee of usability for any purpose or

correctness of the formulas and procedures contained in any file.

If you happen to discover an error in the program I will make

every attempt to correct the error as quickly as possible.  I am under

no obligation to replace nor make refunds for defective full versions

or demonstration/trial versions of the program.  I have to take this

posture as my cost to make even the simplest of corrections far

outweighs any monetary compensation received for the full version of

the program.

If you require any special modifications to the program I will be

happy to discuss on an individual basis the cost of supplying modified

programs and documentation.

The program was tested on a Tandy model 1000 SX using MS-DOS 3.3;

IBM PS/2 model 80, using PC-DOS 3.3;  and a Compaq Desk-Pro 486/33

using MS-DOS 6.0.

And now a word from our sponsor.

--------------------------------

You can get the latest version of this program by registering the

program.  When you register I will also include any other demonstration

/ trail programs that I have available.  Please send $10.00 for an IBM

compatible 5-1/4 or 3-1/2 inch 720 DS/DD or HD disk(s), your choice,

to:

IRA F. KAVALER

671 East 78 Street

Brooklyn, New York 11236

All inquiries that do not include the registration fee must be

accompanied by a stamped self addressed return envelope.

I reserve the right to discontinue support for, change the terms,

or withdraw any part or all of this offer including but not limited to

the programs and its associated files at any time without giving prior

notice.

No form of this program, registered or unregistered,  may be used

in commercial, educational, nor governmental applications without

written authorization or a site lease from the author;  such

authorization and/or site lease may require that a substantial fee be

paid to the author.

73's, de WA2ZIR.

----------------

I welcome your suggestions and comments about this product and

others.  I won't promise that good suggestions will be added to the

program, but they will be considered.

Thank you.

Appendix.

---------

The operating systems, programs and companies mentioned in this file:

Patterson, Eastman Kodak, PKWARE, PKZIP, PKUNZIP, MS-DOS, PC-DOS, LIST,

BROWSE, SIMCGA, Hercules, Tandy Compaq, Arche, and IBM are all

copyrights, trademarks, and/or service marks of other individuals or

other corporations.

>>>>>  End of File  <<<<<