BEAM COOKBOOK 1.0

________________________________________________________________

B E A M  C O O K B O O K

_____________________________

USER'S MANUAL

Shareware Evaluation Version 1.0

(Chapters 1, 2, and 3)

This manual is copyrighted 1994

by SoftEngineering

- All Rights Are Reserved -

BEAM COOKBOOK 1.0

________________________________________________________________

C H A P T E R  1

GETTING STARTED

This chapter  explains how  to install  and run  Beam  Cookbook.

When you have finished this chapter,  you will be ready to  work

with Beam Cookbook.  Good luck!

In this chapter:

* The Beam Cookbook package

* What you need

* Running the installation program

* Loading Beam Cookbook into your computer's memory

THE BEAM COOKBOOK PACKAGE

The Beam Cookbook package includes the following:

* Your 3 1/2 or 5 1/4 program disks

* Beam Cookbook User's Guide

* Software registration and license agreement card

WHAT YOU NEED

The next two sections cover  both necessary and optional  equip-

ment supported by Beam Cookbook.

Necessary equipment

Here's what you need to run Beam Cookbook:

# Computer.  Beam Cookbook runs  on a IBM XT, AT, PS/2,  and

fully compatible computers.

# Operating system.  DOS 3.0 or higher.

# Memory. To run Beam Cookbook, you  need at least 520K (or

532,480 bytes) of available random-access memory (RAM).

# Disk drive.  Your  computer must have  hard disk with  at

least 1 MB (1.5 MB for installation) of space required.

Optional equipment

You don't need the following equipment, but each enhances Beam

Cookbook's performance:

# Graphics card.  If you want to display a graphic repre-

sent-ation  of a beam with it's loads, and  beam diagrams

1

BEAM COOKBOOK 1.0

________________________________________________________________

(shear, moment, slope, and deflection) you need a graphics

card and a monitor capable of high-resolution graphics (a

color monitor is not necessary).  However, you can print

the graphics without a graphics card.

* IBM Color/Graphics Adapter

* Hercules (monochrome) Graphics Card

* IBM Enhanced Graphics Adapter (color or monochrome)

* IBM Video Graphics Array (color or monochrome)

* MCGA (IBM Model 30)

* IBM 8514 Graphics Adapter

* IBM 3270 PC

* AT&T 400 line

# Mouse.  Beam Cookbook works with  the Microsoft Mouse and

the Logitect Mouse.

# Printer.  You can  run Beam Cookbook  with laser, inkjet,

and dot matrix (epson compatible 9 or 24 pin) printers.

# Math  coprocessor.   Beam Cookbook  automatically detects

and uses any math coprocessor installed on your computer.

# Expanded memory card.  You can  add expanded memory  cards

to your  computer  to  increase  the  performance  of  in-

put/output windows and  printing.  Requires  a minimum  of

260k.  Beam Cookbook works with any LIM 4.0 card.

INSTALLING BEAM COOKBOOK (see README.TXT if you do not have an

install diskette)

Beam Cookbook has an Install Program to copy the necessary files

to your hard disk.

* This program creates a BCBEVAL directory on your hard  disk

and copies the contents of your Beam Cookbook into the di-

rectory.

To run the installation program:

1.  Start your computer.

2.  Insert the Installation Disk into drive A.  (You can

also substitute another drive letter depending upon

your system.)

3.  At the DOS prompt (> or C:>), type A: and press the

Enter key.

4.  At the A:> prompt, type INSTALL.  Press the Enter key.

2

BEAM COOKBOOK 1.0

________________________________________________________________

5.  Beam Cookbook will start the Install Program.  Answer

the questions and follow the directions on your com-

puter screen.

6.  When the installation is complete, store the original

Beam Cookbook disks in a safe place.  They will be

your backup.

Once you've run the installation program, you shouldn't need  to

run it again.   If you want to  use a different printer, choose

Options|Hardware|Printer in Beam  Cookbook.  To  use a  graphics

card, choose Options|Hardware|Graphics.   For more details,  see

Chapter 7, page 68.

QUICK START

If you want to start using Beam Cookbook now, then the following

steps will get you going in no time after installation.  If  you

have any trouble with starting or using Beam Cookbook, read  the

rest of the book.

1.  From your Beam Cookbook directory type BEAMEVAL.

See the following  page for a  list of the  commands you'll  use

most often.

3

BEAM COOKBOOK 1.0

________________________________________________________________

Table 1.1 Quick start commands

================================================================

Key(s)                          Description

----------------------------------------------------------------

F10               Activates the menu bar.

F10 F N           Opens a new beam file.

F10 F O           Opens an existing beam file.

F10 F S           Saves the current beam file.

F10 B             Opens Beam Configuration input window.

F10 S             Opens Supports input window.

F10 L C           Opens Concentrated Loads input window.

F10 L M           Opens Moments input window.

F10 L U           Opens Uniform Loads input window.

F10 L R           Opens Ramp Loads input window.

F10 M H           Opens Hinges input window.

F10 M M           Opens Moment Connectors input window.

F10 V             Solves the current beam problem.

F10 W Z           Zooms or unzooms the active window.

F10 W N           Cycles through all open windows.

F10 W P           Makes the previous window active.

F10 W R           Lets you change the size of the active window.

F10 W M           Lets you move the active window.

F10 W C           Close the active window.

F10 W I           Close all input windows.

F10 W O           Close all output windows.

F10 W A           Close all windows.

F1                Displays a help screen.

F2                Saves the current beam file.

F3                Opens an existing beam file.

F5                Zoom or unzooms the active window.

F6                Cycles through all open windows.

F7                Lets you change the size of the active window.

F8                Lets you move the active window.

F9                Solve the current beam problem.

Ctrl-F6           Makes the previous window active.

Alt-#             Displays a window, where # is the number of

the window you want to view.

Alt-X             Exits Beam Cookbook to DOS.

----------------------------------------------------------------

4

BEAM COOKBOOK 1.0

________________________________________________________________

C H A P T E R  2

BEAM COOKBOOK: AN OVERVIEW

This section introduces you to some of the basic procedures  in-

volved in using Beam Cookbook.  It tells you how to

* start Beam Cookbook

* use specific keys and a mouse with Beam Cookbook

* use Beam Cookbook menus to issue commands

* use "shortcuts" to execute  menu commands

* get on screen help about whatever you are doing

* use the Esc key to cancel a menu or prompt

* specify a new default directory

* enter data and make changes to it

* save and retrieve Beam Cookbook files

* switch to different display mode

* exit Beam Cookbook

STARTING BEAM COOKBOOK

To start working with Beam Cookbook  (once you've installed the

program), you must load the program into your computers memory.

1.  Get into  the Beam Cookbook directory  (the default is

C:\BCBEVAL) on your hard disk. For example, if the pro-

gram files reside in a directory called BCBEVAL, type

CD \BCBEVAL

and press the Enter key.

2.  To run BEAMEVAL.EXE type

BEAMEVAL

at the  C:\BCBEVAL> prompt and press the Enter key.

You can also start  Beam Cookbook and load a beam problem at the

same time by entering the file name:

BEAMEVAL  MYPROB (or)

BEAMEVAL  MYPROB.BCB

THE BEAM COOKBOOK MAIN SCREEN

The Beam Cookbook main screen has four distinct areas: the  menu

bar at  the top , the desktop area, function key bar, and a help

line at the bottom.

"Using the Beam Cookbook menus" on page 9 describes how the menu

bar works.  The  follow sections discuss  the desktop area,  the

function key bar, and the help line.

5

BEAM COOKBOOK 1.0

________________________________________________________________

The desktop area

The desktop area takes up most of  your screen.  It is the  area

where you open input windows (similar to a spreadsheet) and out-

put windows.  An  input windows is where  you define your  beam

problem (loads, supports, and etc).   An output window is  where

the solutions are displayed.  See  chapter 4 "Input windows"  on

page 26 and chapter 5 "Output windows" on page 35.

Function key bar and help line

The function key bar and help line  appear at the bottom of  the

screen.

The function key bar

* reminds you of basic keystrokes and shortcuts applicable

at that moment in the active window.

* lets you click the shortcuts to carry out the action

instead of choosing the command from the menu or pressing

the shortcut keystroke.

* displays the current file name at the left end.

* changes as you switch activities.

The help line

* offers one|line hints on any selected menu command,  input

window item, and etc. (There is always a hint.)

A TOUR OF YOUR KEYBOARD

Certain keys on  your keyboard have  a special  meaning in  Beam

Cookbook.  The arrow keys on the right side of your keyboard let

you move to different areas of  the screen.  The function keys ,

on the left  or on top  of your keyboard  (F1 and up),  automate

various commands.  Both of these key groups are discussed in the

following sections.  These an  other special keys are  described

in Appendix A, Beam Cookbook keys", on page 68.

Arrow keys

The Beam Cookbook arrow keys let you

* move around in input windows, and output windows

* position the cursor within an entry area during edit mode

* highlight menu items

* move around help screens

In a menu,  you can use  the arrow keys to highlight  different

menu items.  The up  arrow and down arrow  keys move you up  and

down in the menu.  The right arrow and left arrow keys take you

to adjacent menus if you are in the first level of menus  pulled

6

BEAM COOKBOOK 1.0

________________________________________________________________

down from the main menu bar.  Home takes you to  the first item

in the menu; End takes you to the last.

With help information displayed (press  F1), use the arrow  keys

to move the cursor to the  text that is highlighted on the  help

screen.  When  you press Enter,  Beam Cookbook displays  another

help screen with details about the text that is highlighted.

With the help  index displayed (press  Shift-F1), use the  arrow

keys to highlight index items.  When you press Enter, Beam Cook-

book displays another help screen  with details about the  index

item that is highlighted.

Function keys

The function keys on the left or top  your keyboard (labeled F1

and up) perform often used commands with one keystroke.  Some of

the keys have more  than one function;  to access further  func-

tions, hold down  Alt, Ctrl, or  Shift, the  press the  function

key.

Appendix A, "Beam Cookbook keys", on page 68  describes function

keys and other special keys.

The Esc key

In Beam Cookbook, the Esc key is the all purpose "uh|oh" key. It

can't reverse mistakes  you've already made,  but lets you  back

out of mistakes before it's to late.

Suppose that while editing or entering a entry field, you decide

you'd rather keep the old entry.  Press Esc and the original en-

try returns-as long as you press Esc before you press Enter.  If

you press Esc  after pressing Enter  or before  editing a  field

will close the input window or output window.

Similarly, if you choose  a command that  displays a prompt  and

then decide not to carry out  the command, Esc cancels the  com-

mand.  Within menus, you can press Esc to exit the current menu

and return to the previous menu.

If you have a mouse, the right mouse button duplicates  the Esc

key.

General rules:

Primary rule-You can interrupt any sequence that ends with Enter

by pressing Esc before you press Enter.

Secondary rule-You can also close any  menu or window by  press-

ing Esc if there are no pending edits.

7

BEAM COOKBOOK 1.0

________________________________________________________________

USING THE MOUSE

A mouse is a hand operated pointer device attached to your co m-

puter.  The mouse has up to three buttons  that send signals to

the computer when you press them,  just as pressing keys on  the

keyboard does.  You  can use a  mouse  to  perform many   tasks,

such as choosing commands or selecting entry fields.

If you  have a  mouse (and  your mouse  software is  loaded),  a

pointer appears onscreen.  The pointer is a highlighted  rectan-

gle with a diamond in the middle.  To control the pointer,  roll

the mouse across your desktop or mouse pad.

The diamond in the middle of the box pointer is what you need to

position.

Basic techniques

Table 2.1 describes the  actions you need to  know when using  a

mouse.

Table 2.1 Basic mouse techniques

================================================================

Technique       Action

----------------------------------------------------------------

Point           Move the mouse pointer to a specific area

onscreen.

Click           Quickly press and release the left button.

Double|click    Quickly press and release the left button twice

in succession.

Hold down       Hold down a button.

Drag            Press the left button down and hold it down

while you move the pointer.

Release         Let up on the left button after dragging.

Click right     Quickly press and release the right button.

----------------------------------------------------------------

Use these techniques to perform the following general  functions

with a mouse:

* Select an item or active an area.  For example, you can

click to choose a command from a menu or activate a dif-

ferent part of the screen.

* Move a boundary.  You drag to resize a window or to repo-

sition a window.

* Accept or cancel field entries and edits or entries in a

prompt box.  Click on another field or click on the "OK"

(if there is one) for accept (same as Enter key) or click

the right button for cancel (same as Esc key).

8

BEAM COOKBOOK 1.0

________________________________________________________________

More mouse shortcuts

On the function key bar you can click the  highlighted shortcut

keystroke.

In addition to the function key bar, there are boxes on the i n-

put and output windows:

* A zoom box in the  upper right hand  corner of a  window.

Click it to zoom or unzoom the window.

* A close box in the  upper left hand  corner of a  window.

Click it to close the window.

* A resize box in the lower right hand  corner of a window.

To change the size of a window, drag the resize box until

the window is the right size, then release.

* The move bar is the upper border of a window.  To reposit-

ion a window, drag it to the right location, then release.

* Scroll bars  along the bottom and right  edge of each win-

dow.   On  both vertical  and  horizontal scroll  bars,  a

scroll box indicates the position of the selector relative

to the data  onscreen.   To move  to a  different part  of

data, click on the scroll bar.

USING THE BEAM COOKBOOK MENUS

The menu bar is your  primary access to all  the menu commands.

The only time the menu bar is not visible is when you're viewing

graphical output.  You'll see a highlighted menu title when  the

menu bar is  active; this is  the currently selected  menu.   To

activate a menu press Enter.

If a menu command is followed by an  arrowhead (>), the command

leads to another menu (a pop|up menu).  A command without an ar-

row indicates that the action occurs  once you choose it.   This

action could display a dialog box or window.

To exit a menu without choosing a command, press Esc.

Here is how you choose menu commands using just the keyboard:

* Press F10. This makes the menu bar active.

* Use the arrow keys to select the menu you want to display.

Then press Enter.  To cancel an action, press Esc.

9

BEAM COOKBOOK 1.0

________________________________________________________________

As a shortcut for this step you can press the  highlighted

letter of the menu title.  For example, from the menu bar,

press F to quickly display the File menu.

At this point, Beam Cookbook  either carries out the  com-

mand, displays another menu,  displays a dialog box,  dis-

plays a input window, or displays a output window.

* If the menu command selected pops up another menu, then

use the arrow keys again to select the command you want.

Then press Enter.

Again, as a shortcut, for this  step just press the  high-

lighted  letter of a command to choose it once the menu is

displayed.

Again, at this point, Beam Cookbook either carries out the

command, displays another menu, displays a dialog box,  or

displays a window.

You can also  use a mouse  to choose commands.   The process  is

this:

* Click with the left mouse button.

* Click the desired menu title to display the menu.

* Click the desired command.  To cancel command click with

the right mouse button.

You can also drag straight from the menu title down to the  menu

command.  Release the mouse button on the command you want.  (if

you change your mind, just drag off the menu; no command will be

chosen.)

Note that some menu commands are unavailable when it would  make

no sense to choose them.

Shortcuts

Beam Cookbook offers a number of quick ways to choose menu  com-

mands. For example, mouse users can combine the two|step process

into one by dragging from the  menu title down to the menu  com-

mands and releasing the mouse button  when the command you  want

is selected.

From the keyboard,  you can use  a number of  shortcuts (or  hot

keys) to access the  menu bar and choose commands.  You can get

to, or activate, menu items by pressing hot keys that are on the

function key bar or the shortcut next to some of the menu items.

You can also click on shortcuts on the function key bar.

10

BEAM COOKBOOK 1.0

________________________________________________________________

The following tables lists the most|used Beam Cookbook hot keys.

Table 2.2  General hot keys

================================================================

Key(s)   Menu item       Function

----------------------------------------------------------------

F1       Help            Displays a help screen.

F2       File|Save       Saves the current file.

F3       File|Open       Opens a file.

F5       Window|Zoom     Zoom the active window.

F6       Window|Next     Cycles through all open windows.

F7       Window|Resize   Resize active window.

F8       Window|Move     Move active window.

F9       Solve           Solve beam problem.

F10      (none)          Takes you to the menu bar.

Alt-#    (none)          Displays a window, where # is the

number of the window you want to view.

Alt-X    File|Exit       Exits Beam Cookbook to DOS.

Ctrl-F6  Window|Previous Active previous window.

----------------------------------------------------------------

Table 2.3  Online help hot keys

================================================================

Key(s)   Menu item           Function

----------------------------------------------------------------

F1       Help                 Displays a help screen.

F1-F1    Help|Help on Help    Brings up Help on Help. Just press

F1 when your already in the help

system.

Shift-F3 Help|Index           Brings up Help index.

Alt-F1   Help|Previous Topic  Displays previous help screen

index.

----------------------------------------------------------------

Table 2.4  Window management hot keys

================================================================

Key(s)       Menu item        Function

----------------------------------------------------------------

F5 or Alt-Z  Window|Zoom      Zoom the active window.

F6           Window|Next      Cycles through all open

windows.

Ctrl-F6      Window|Previous  Returns to previously active

window.

----------------------------------------------------------------

(continued on next page)

11

BEAM COOKBOOK 1.0

________________________________________________________________

Table 2.4  Window management hot keys (continued)

================================================================

Key(s)       Menu item        Function

----------------------------------------------------------------

F7 or Alt-R  Window|Resize    Resize active window.

F8 or Alt-M  Window|Move      Move active window.

Esc          Window|Close     Closes active window.

Alt-#        (none)           Displays a window, where # is

(#=1, 2, etc)                  the number of the window you want

to view.

----------------------------------------------------------------

USING THE BEAM COOKBOOK WINDOWS

Most of what you see and does happens in a window.  A window  is

a screen area that you can  move, resize, zoom, overlap,  close,

and open.   These windows are  used to input  data, view  output

data, and to select options.

You can have all these windows open at same time (memory allo w-

ing), but only one window can be active at any time.  The active

window is the one that you're currently working in.  Any command

you choose or data you type generally applies to active window.

There are several types of windows, but most of them have  these

things in common:

* a title bar

* a close box

* a resize corner

* a zoom box

* a window number

* scroll bars

Beam Cookbook makes it easy to  spot the active window by  high-

lighting the title bar.  If your  windows are overlapping,  the

active window is always on top of the others.

The various parts of a typical window are:

* The close box of a window is the box in the upper left

corner.  You click this box to quickly close the window.

(Or choose Window|Close or press Esc.)

* The title bar, the top most  horizontal bar of a  window,

contains the name of the window and the window number. You

can drag (using the mouse) the title bar to move the win-

12

BEAM COOKBOOK 1.0

________________________________________________________________

dow around.  To  move a window  from the keyboard,  choose

Window|Move, press Alt-M, or F8.

* The window number  is in the  upper right hand  border of

each of the windows  you open in Beam  Cookbook.  You  can

make a  window active  (bringing it  to the  front of  all

other windows)  by pressing  Alt in  combination with  the

window number.  For example, if the help window is #3  but

has gotten buried under the other windows, Alt-3 brings it

to the front.

* The zoom box of a window appears in  the upper right co r-

ner.  If the icon in that  corner is an up arrow, you  can

click the arrow to enlarge the window to the largest  size

possible.  If the  icon is a double  arrow, the window  is

already at its maximum  size.  In  that case, clicking  it

returns the window to its previous size.  To zoom a window

from the keyboard, choose Window|Zoom, press Alt-Z, or F5.

* Scroll bars are horizontal or vertical bars. You use these

bars with a mouse  to scroll the  contents of the  window.

Click the arrow at either end  to scroll one at line at  a

time.    The   scroll  bar  represents   the  total   rows

(horizontal bar) or the total columns (vertical bar).  The

scroll box is positioned at the current highlighted  item.

You can click the shaded area on either side of the scroll

box to quickly move  to a spot in  the window relative  to

the position of the scroll box.

* The resize box is the lower right corner of a window.  You

drag (using  the  mouse) the  corner  to make  the  window

larger or smaller.  You can spot the resize corner by  its

symbol the plus sign  (+).  To resize by using the   ke y-

board, choose Resize from the Window menu, or press F7  or

Alt-R.

Window management

Table 2.5 gives you a quick rundown of how to handle windows in

Beam Cookbook.   Note that  you don't  need a  mouse to  perform

these actions a keyboard works fine.

13

BEAM COOKBOOK 1.0

________________________________________________________________

Table 2.5  Manipulating windows

================================================================

To accomplish this:   Use one of these methods

----------------------------------------------------------------

Close a window        Choose Window|Close, or press Esc, or

click on the close box of the window.

Activate a window     Click anywhere on the window, or press

Alt plus the window  number in the  upper

right border  of  the window,  or  Choose

Window|Next or F6 to make the next window

active  (next  in  the  order  you  first

opened them).  Or Choose  Window|Previous

or Ctrl-F6 to cycle backward.

Move the active       Drag its title bar, or Choose Window|Move

window               window or press F8  or Alt-M and use  the

arrow keys to place the window where you

want it, then press Enter.

Hold down             Hold down a button.

Resize the active     Drag the resize corner, or Choose

window               Window|Resize, or press F7 or Alt-R

and use the arrow keys to resize the

window, then press Enter.

Zoom the active       Click the zoom box in the upperright

window               corner of the window, or Choose

Window|Zoom or press F5 or Alt-Z.

-------------------------------------------------------------

GETTING HELP

Press the Help key , F1, whenever you want assistance.  A window

of information related  to what you're  doing displays over  the

current screen.   For example, if  you press F1  with a  command

highlighted, the help system displays a description of that com-

mand.

Usually, several screens of related  information are  available

for each topic. Related topics or  areas that offer further  in-

formation appear highlighted  as keywords onscreen.   Use arrow

keys, Tab key (next),  or Shift-Tab key  (previous) to move  the

cursor under  any highlighted  topic you're  interested in  then

press Enter to display the related screen.  To return to a  pre-

vious topic press Alt-F1.

To return immediately  to where  you were  before getting  help,

press Esc or click right with the mouse.

14

BEAM COOKBOOK 1.0

________________________________________________________________

To get information  about an  error message that  Beam Cookbook

displays in the middle of the screen see Appendix B, "Error mes-

sages", on page 74.

SETTING THE DEFAULT DIRECTORY

The default directory setting tells Beam Cookbook where to look

and store beam files when you haven't specified any other direc-

tory.  If you don't set this option, Beam Cookbook uses the cur-

rent drive and directory.  If  you want to keep your beam  files

in a different directory than the program files, change the  de-

fault directory:

1.  Press F10.

2.  Choose File|Change Directory.

3.  Beam Cookbook prompts you for a directory with the

current directory shown as the default.  Edit the ex-

isting directory path or type in a new path then press

Enter.

You can also type in the drive letter (such as C) then

press Enter and a Directory tree is displayed.  Choose

the directory you want and then press Enter.

4. To save the new directory as the directory default,

choose Options|Save options.

If you don't update the defaults after changing directories, the

original directories will  be in effect  the next  time you  use

Beam Cookbook.

Regardless of the default directory,  you can always access  any

directory on your disk.   Simply include  the directory path  as

part of your  file name  you specify  when saving  or opening  a

file.

ENTERING AND EDITING DATA

To enter information in a field, move the selector to the  field

you want and type  in the entry.   The characters appear in  the

field.  The  cursor, shown as  underscore, shows  you where  the

next character will appear.  If you make a mistake will  typing,

press the Backspace to erase the  character, then type the  cor-

rect one.

15

BEAM COOKBOOK 1.0

________________________________________________________________

When the entry  is complete, press  Enter or any  of Beam  Cook-

book's arrow keys (up, down, left, right, Page  Up, and so on).

If you entered the data by pressing the arrow key, the field se-

lector moves in the appropriate direction.

If Beam Cookbook  finds an error  in your entry,  it beeps,  and

displays an error message.

When editing and entry, use u and " to position the cursor  any-

where in the entry (if  the cursor is at  the either end of  the

entry field and you use these keys  you go to the next or previ-

ous field).  You can then  insert new characters, or use  Delete

to remove erroneous ones.  When you've corrected the error press

Enter to accept you're changes.

After you enter data  in a entry field,  you can always go  back

and change the entry.  Just highlight the entry field, then  re-

enter or change  the data and  then press Enter.   "Editing  en-

tries," on page 48, discusses editing entries in detail.

To highlight a entry field using  a mouse, just click the  entry

field.  Also when editing you  can position the cursor by  using

the mouse, just click where you the cursor to move too.

Saving and opening beam files

You can save the data you've entered for a beam in a disk file.

Later, you can  access the beam  file by opening  the file  it's

stored in.

To save a new file, unnamed beam file in a disk file,

1.  Press F10.

2.  Choose File|Save (or F2).  Beam  Cookbook prompts you

for the file name.

3.  Enter a file name or press Enter and choose one from a

displayed list of files in the default directory.  You

can use up to eight letters  or numbers.  The  default

file-name extension .BCB is automatically appended.

4.  Press Enter.  If you choose  or enter a name that  al-

ready exists, Beam Cookbook will prompt you to  verify

that you  want the  existing file  to be  overwritten.

You'll see a dialog box  with three options: Yes,  No,

and Cancel.  Choose Yes to  overfile, or choose No  or

Cancel to cancel the command.   See page 56 for  auto-

matic renaming of  the existing file  with .BAK file-

name extension.

16

BEAM COOKBOOK 1.0

________________________________________________________________

To save an existing, previously named beam file,

1.  Press F10.

2.  Choose File | Save (or F2).

To save the current beam file to another file name,

1.  Press F10.

2.  Choose File|Save As.  Beam  Cookbook  prompts you for

the file  name,  just  as  it  does  when  you  choose

File|Save for a unnamed beam file.

3.  Enter a file name or press Enter and choose one from a

displayed list of files in the default directory.  You

can use up to eight letters  or numbers.  The  default

file-name extension .BCB is automatically appended.

4.  Press Enter.  If you  choose or enter a name that  al-

ready exists, Beam Cookbook will prompt you to  verify

that you  want the  existing file  to be  overwritten.

You'll see a dialog box  with three options: Yes,  No,

and Cancel.  Choose Yes to  overfile, or choose No  or

Cancel  to cancel the command.  See page 56 for  auto-

matic renaming of  the existing file  with .BAK  file-

name extension.

Caution!

If you exit without saving the current beam file, all your  work

will be lost.  Beam Cookbook will prompt you to verify that  you

want the current file to be saved.  You'll see a dialog box with

three options: Yes, No, and Cancel.  Choose Yes to save changes,

choose No to discard changes and continue, or choose Cancel  the

command.

Always save before you  leave the program or  turn off the  com-

puter.

If you're entering a lot of  data, save your work  periodically.

That way if something happens to  your system memory (such as  a

power failure), you'll lose only  the data you've entered  since

your last save.

After you've saved a beam file in a file, you can bring it  back

up onscreen any time by retrieving it:

1.  Press F10.

17

BEAM COOKBOOK 1.0

________________________________________________________________

2.  Choose File|Open (or F1).  Beam Cookbook  prompts you

for the file name.

3.  You can type in  the exact name (including the  direc-

tory and file-name extension if different from the de-

fault) or use a mask and  press Enter.  If you used  a

mask or the file name cannot be found a list of  files

will be displayed.   You can  choose one  from a  dis-

played list of files in the default directory.

4.  If you haven't saved the current beam file, Beam Cook-

book will prompt you to verify that  you want the  ex-

isting file to be saved.  You'll see a dialog box with

three options: Yes,  No, and  Cancel.   Choose Yes  to

save changes, choose  No to discard  changes and  con-

tinue, or choose Cancel the command.

SETTING THE SCREEN SIZE

Normally, Beam Cookbook uses 25 lines for display.  If you  have

an EGA card you can display 43 lines.  If you have a VGA display

adapter you can display 50 lines.

1.  Press F10.

2.  Choose Options|Screen

3.  Choose 25 lines or 43/50 line.

These options are only available on hardware that supports them.

After you choose the screen size, Beam Cookbook uses it  immedi-

ately.  If you want  to use that screen  size as a new  default,

choose Options|Save options.

Using mouse drivers with graphics cards

Early versions of mouse drivers don't support screen sizes  that

have more than 25 lines.  As a result, your mouse driver may not

work correctly when you use the screen sizes of EGA 43 lines or

VGA 50 line modes.  The following table shows which mouse  driv-

ers work correctly with enhanced screen modes.

18

BEAM COOKBOOK 1.0

________________________________________________________________

Table 2.6  Mouse drivers that support enhanced screen modes

================================================================

Mouse                   Driver version supports

EGA 43 line and VGA 50 line mode

----------------------------------------------------------------

Microsoft               Version 6.11 or later

Mouse System            Version 6.01 or later

Logitech                Version 4.00 or later

PC Mouse                Version 6.01 or later

----------------------------------------------------------------

EXITING BEAM COOKBOOK

When you're finished working with Beam Cookbook,

1.  Press F10.

2.  Choose File|Exit.    If your Beam file   contains data

that you haven't yet saved, Beam Cookbook will  prompt

you to verify that  you  want the existing file to  be

saved.  You'll  see a dialog  box with three  options:

Yes, No, and Cancel.

3.  Choose  Yes to save changes  and exit  Beam  Cookbook.

Choose No to discard changes and then exit Beam  Cook-

book.  Cancel cancels the exit and returns you to Beam

Cookbook.

When you exit Beam Cookbook, the program disappears and the  DOS

prompt returns.  To return to Beam Cookbook, you can reload  the

program by typing BEAM and pressing Enter.

Alt-X is the default shortcut for the File|Exit command.

19

BEAM COOKBOOK 1.0

________________________________________________________________

C H A P T E R  3

THE PULL-DOWN MENUS

All Beam Cookbook commands are accessed through the menu bar at

the top of the screen.

Some commands on the menu bar displays another menu, which is  a

pull-down menu because you pull it down from the menu bar.  Each

pull-down menu accesses a different branch of the command tree.

This chapter briefly  describes the  pull-down menus  associated

with each menu bar  command.  Further  chapters detail the  com-

mands on these menus.

ABOUT

The F10|About command does not have a pull-down menu associated

with it.  This command  displays copyright and version  informa-

tion.

FILE

The F10|File command brings up the File menu, which you can use

to open and save files, change the default directory, get infor-

mation about your system, and exit Beam Cookbook.

+------------------+

| Open          F3 |

| New              |

| Save          F2 |

| Save As          |

+------------------+

| Change Directory |

| Get System Info  |

+------------------+

| Exit       Alt-X |

+------------------+

* Open opens an existing beam file.

* New opens a new beam file.

* Save lets you save the current beam file.

* Save As saves the current beam file under a new name.

* Change Directory lets you change the default directory.

20

BEAM COOKBOOK 1.0

________________________________________________________________

* Get System Info display information about your system.

* Exit removes Beam Cookbook from memory and returns you  to

DOS.  If your beam problem contains data that you  haven't

saved, Beam Cookbook  asks if you  want to  save the  file

first.

All File menu commands (except Exit and Get System Info) are de-

scribed in Chapter 7 on page 58, "Working with beamfiles."  Exit

is described on page 19.  Get System Info in the next paragraph.

The Get  System Info  command displays  a box  with  information

about your system.

+[-]========= Information ============+

|                                     |

|        DOS Version:        5.0      |

|        Math Coprocessor:   Yes      |

|  ---------- Dos Memory -----------  |

|    Free:   146K Max Avail: 146K     |

|  ---------- EMS Memory -----------  |

|    Total: 1024K Available: 1024K    |

|                                     |

|               [ OK ]                |

|                                     |

+-------------------------------------+

The information here is for diskplay only; you can't change  any

of the information in this box. After reviewing the  information

in the box, press Enter to put the box away.

BEAM

The F10|Beam command does not have a  pull-down menu associated

with  it.     This  command   opens  or   activates  the   "BEAM

CONFIGURATION" input window.  The  beam command is described  on

page 26.

SUPPORTS

The F10|Supports command does not have a pull-down menu  associ-

ated with it.   This command opens  or activates the  "SUPPORTS"

input window.  The supports command is described on page 28.

21

BEAM COOKBOOK 1.0

________________________________________________________________

LOADS

The F10|Loads command brings up the  loads menu , which you  can

use to open input windows for concentrated loads, moments,  uni-

form loads, and ramp loads.

+----------------------+

|  Concentrated Loads  |

|  Moments             |

|  Uniform Loads       |

|  Ramp Loads          |

+----------------------+

* Concentrated Loads  opens or activates  the "CONCENTRATED

LOADS" input window.

* Moments opens or activates the "MOMENTS" input window.

* Uniform Loads opens or activates the "UNIFORM LOADS" input

window.

* Ramp Loads opens or activates the "RAMP  LOADS" input win-

dow.

The input  windows opened  by these  commands are  described  in

Chapter 4, "Input windows."

MISCELLANEOUS

The F10|Miscellaneous command brings up the miscellaneous menu,

which you can use  to open input windows  for hinges and  moment

connectors.

+---------------------+

|  Hinges             |

|  Moment Connectors  |

+---------------------+

* Hinges opens or activates the "HINGES" input window.

* Moment Connectors opens or activates the "MOMENT CONNECT-

ORS" input window.

The input  windows opened  by these  commands are  described  in

Chapter 4, "Input windows."

22

BEAM COOKBOOK 1.0

________________________________________________________________

SOLVE

The F10|Solve command does not have a pull-down menu associated

with it.  This command solves the current beam problem and opens

or activates  the "SOLUTIONS"  output window.   The  "SOLUTIONS"

output window is described in Chapter 5, "Output windows."

OPTIONS

The F10|Options command brings up the options  menu, which co n-

tain commands that let you change default settings for the over-

all system.

+--------------------------+

|  Hardware              > |

|  Screen:      25 lines > |

|  Preferences           > |

|  StartUp               > |

+--------------------------+

|  Save Options            |

+--------------------------+

* Hardware lets you specify or change settings for your com-

puter graphics card, printer, and the device used for your

printer connection.

* Screen  lets  you specify  whether Beam  Cookbook displays

text in 25  lines or 43/50  lines. In order  to use  43/50

line mode  your system  must have  a EGA or  VGA graphics

card.

* Preferences  lets you  tailor Beam  Cookbook to  suit your

needs.  The  options are  divided into  two sections,  one

that affects Beam Cookbook when you  are using it and  the

other saves information when you leave Beam Cookbook.   In

the first section ,  auto save, you can  auto save a  con-

figuration file (same as Save|Options),default  directory,

and  window positions when exiting Beam Cookbook.  In  the

second section automatically create  a backup file  and/or

auto close windows.

* StartUp  lets you  specify whether Beam  Cookbook will use

expanded memory to speed up window display and printing.

s

* Save Options stores the all option settings defined under

the option command and  the current default directory  de-

fined under the file command.

23

BEAM COOKBOOK 1.0

________________________________________________________________

All Option commands  are described  in Chapter  7, "Setting  Op-

tions", on page 51.

WINDOW

The F10|Window command brings up the window menu, which contain

commands that let  you  close  windows and alter  the ways  Beam

Cookbook displays them.

+--------------------+

|  Zoom/Unzoom   F5  |

|  Next          F6  |

|  Previous Crtl-F6  |

|  Resize        F7  |

|  Move          F8  |

|  Close         Esc |

+--------------------+

|  Close Input       |

|  Close Output      |

|  Close All         |

+--------------------+

* Zoom/Unzoom expands the active window to its maximum size,

if already expanded if shrinks the window to its  previous

size.

* Next makes the next window active, which makes it the top

most open window.

* Previous makes the previous window active, which makes it

the topmost open window.

* Resize lets you change the size of the active window.

* Move lets you change the position of the active window.

* Close lets you close the active window.

* Close Input lets you close all the input windows currently

open.  See Chapter 4, "Input windows", page 26.

* Close Output lets you close  all the output  windows cur-

rently open.  See Chapter 5, "Output windows", page 35.

* Close All lets you close all the windows (input or output)

currently open.

All Window commands are described in Chapter 2, "Window  manage-

ment", page 13.

24

BEAM COOKBOOK 1.0

________________________________________________________________

HELP

The F10|Help command brings up the help menu, which contain com-

mands that let you   have access to  different parts of the  on-

line help, which is displayed in a special help window.

+-----------------------------+

|  Contents                   |

|  Index            Shift-F1  |

|  Previous Topic     Alt-F1  |

|  Help on help               |

+-----------------------------+

* Contents provides a summary of Help topics you can choose

from.

* Index  provides a comprehensive alphabetic index  of Help

topics you can choose from.

* Previous  Topic  lets you go  back to the  previous   Help

topic.

* Help on Help shows you how to use the Help.

All Help commands  are described in  Chapter 2, "Getting  help",

page 14.

25