Commonly used Basic Peeks, Pokes and Subroutines

Due to the lack of a comprehensive, published directory of commonly used

pokes, peeks and subroutines this list has been compiled by the SMUG

PROSIG as well as a many other hardworking PD sources. Thanks and a tip

of the hat to all contributors! Additions to the list are encouraged and

should be address to:

Don Watkins, CIS IBMSIG 76003,252

There are, of course no warrenties or guarentees that any of stuff works

and furthermore, if it blows up your machine it ain't my fault.

------------------------------------------------------------------------

By specifying a DEF SEG=&H40 in any BASIC program, it is possible to

reference the following vectors (fields) in the ROM BIOS area by using a

PEEK function and the following offsets from the current segment as

defined by the DEF SEG statement.

&H0       -  RS232 Addresses on your IBM PC.

This will allow you to tell how many (up to

four) async cards are attached, if any.

&H8       -  Printer Addresses on your IBM PC.

This will tell you what printer addresses,

and how many (up to four) exist. Each is

addressed by a two byte Hex value.

&H10      -  Equipment Flag.

This field describes the setting of the

options switches. It describes what optional

devices are attached to the system. The

following lists the bit-significance of this

field:

Bit 0 - indicates that there are diskette

drives on the system.

Bit 1 - not used.

Bit 2,3 - Planar Ram Size (00=16K 10=32K 01=48K

11=64K)

Bit 4,5 - Initial Video Mode (00=Unused

10=40x25 Color

01=80x25 Color

11=80x25 Mono)

Bit 6,7 - Number of Diskette Drives (00=1 10=2

01=3 11=4) only if bit 0 = 1.

Bit 8   - Unused

Bit 9,10,11 - Number of RS232 Cards attached

Bit 12  - Game I/O Attached

Bit 13  - Not used

Bit 14,15   - Number of printers attached

&H13      -  Memory Size in K bytes.

&H15      -  I/O RAM Size in K bytes.

&H17      -  Keyboard Flag -- the following lists the masks

set to describe current keyboard status:

Byte 1;

&H80 - Insert state active

&H40 - Caps Lock State Has been toggled

&H20 - Num Lock State has been toggled

&H10 - Scroll Lock State has been toggled

&H08 - Alternate Shift key depressed

&H04 - Control Shift key depressed

&H02 - Left Shift key depressed

&H01 - Right Shift key depressed

Byte 2;

&H80 - Insert Key is depressed

&H40 - Caps Lock Key is depressed

&H20 - Num Lock Key is depressed

&H10 - Scroll Lock key is depressed

&H08 - Suspend key has been toggled

&H49      -  Current CRT mode

&H00 - 40x25 BW

&H01 - 40x25 Color

&H02 - 80x25 BW

&H03 - 80x25 Color

&H04 - 320x200 Color

&H05 - 320x200 BW

&H06 - 640x200 BW

&H07 - 80x25 B&W Card -- specialized use, used

internal to the video routines.

&H4A      -  Number of CRT columns

&H50      -  Cursor Position (one of eight)

&H60      -  Current cursor mode

&H6C      -  Low word of Timer count

&H6E      -  High word of Timer count

&H71      -  &H07 - Break key depressed

&HFA6E    -  Beginning of character regen memory

&HFF53    -  PRTSC routine address

------------------------------------------------------------------

Real stuff--

TOGGLE NUM LOCK

DEG SEG = &H40 : POKE &H17, PEEK(&H17) OR 32   'To turn on

DEG SEG = &H40 : POKE &H17, PEEK(&H17) AND 223 'To turn off

TOGGLE CAPS LOCK

DEG SEG = &H40 : POKE &H17, PEEK(&H17) OR 64   'To turn on

DEG SEG = &H40 : POKE &H17, PEEK(&H17) AND 171 'To turn off

SET SCROLL WINDOW

10 DEF SEG : POKE 91,20 : POKE 92,25            'Sets up window on line

20 LOCATE X,20                                  'Force cursor to window

SET WINDOW WIDTH

DEF SEG : POKE 41,30                           'Sets window width to 30

RESTORE FUNCTION KEYS TO DEFAULT

10 DEF SEG = &HFACE

20 K = 1

30 I = 13

40 T$ = STRING$(13,32): J = 1

50 T1 = PEEK(I):IF T1 < 0 THEN MID$(T$,J,1) = CHR$(T1):J = J + 1:

I = I + 1 : GOTO 50

60 KEY K,LEFT$(T$,J-1):IF K <10 THEN K = K + 1: I = I + 1: GOTO 40 :

ELSE KEY ON

DETERMINE MONITOR TYPE

10 DEF SEG = 0

20 MONITOR.TYPE = PEEK(&H410) AND &H40

30 IF MONITOR.TYPE = 1 PRINT "40 X 25 COLOR"

40 IF MONITOR.TYPE = 32 PRINT "80 X 25 COLOR"

50 IF MONITOR.TYPE = 48 PRINT "MONOCHROME"

DETERMINE AMOUNT OF MEMORY INSTALLED (Only works for greater than 48k)

DEF SEG = 0: MEMORY% = PEEK(&H413)+(256*PEEK(&H414))

or, put another way:

Memory info:         DEF SEG=0

((PEEK(1040) and 12) + 4 ) * 4     -  Memory on Mother-board

PEEK(1045) + 256 * PEEK(1046)      -  Expansion memory (add on)

PEEK(1043) + 256 * PEEK(1044)      -  Total memory

READ DRIVE SWITCHES

DEF SEG = 0: NUMBER.OF.DRIVES% = PEEK(&H410) AND &HC0

CURRENT DISK INFO:  DEF SEG=64

DEF SEG=64

PEEK(69)   -  Track

PEEK(70)   -  Head

PEEK(71)   -  Sector

256^PEEK(72)   -  Bytes per sector

DETERMINE IF GAME ADAPTER EXISTS

10 DEF SEG = 0: GAME.ADAPTER% = PEEK(&H411) AND &H10

20 IF GAME.ADAPTER% = 0 THEN GAME.ADAPTER$ = "No" ELSE GAME.ADAPTER$

= "Yes --Installed"

KEYBOARD STUFF

To disable entire keyboard:  Def Seg=64: Out 97,204

To re-enable keyboard:       Def Seg=64: Out 97,76

Printer Status--- at least on Epson ---

Def Seg=64

A=peek(8)+256*peek(9)

B=(inp(a+1) and 248) xor 72

if (B and 128)<>128 then printer off line else on line

Initialize Printer:           Def Seg: Out A+2,8

Out A+2,12

Note:  the A to initialize is from Printer status routine

A short program to disable and re-enable ctrl break follows.

100 DIM OLD%(4)

110 DEF SEG=0

120 ' save the old control break address

130 FOR I=&H6C TO &H6F

140   OLD%(I-&H6C)=PEEK(I)

150 NEXT

160 ' establish new control break address  (point to IRET)

170 POKE &H6C,&H53

180 POKE &H6D,&HFF

190 POKE &H6E,&H0

200 POKE &H6F,&HF0

210 DEF SEG

220 ' reset old control break address

230 DEF SEG=0

240 FOR I=&H6C TO &H6F

250   POKE I,OLD%(I-&H6C)

260 NEXT

Save and Restore a Screen Image

1 DEF SEG = &HB800                      'Save screen image...change for

2 INPUT FILENAME$                       'monochrome.

3 BSAVE FILENAME$,0,&H4000

1000 INPUT "Filename";FILENAME$         'Restore image

1010 CLS

1020 DEF SEG = &HB800                   'Change to &HB000 to mono

1030 BLOAD FILENAME$

Nice to know

BASIC Unprotect

Enter BASICA

Type BSAVE "UN.P",1124,1

Load "MYPROG

BLOAD "UN.P",1124

The program can now be listed, edited and SAVEd as a normal file.

The list is growing but could be longer!  Any and all additions of

commonly used subroutines and peek/poke locations will be gladly added.

Address all additions to:  Don Watkins CIS 76003,252 (IBMSIG).  With a

bit of your assistance this document can become an effective tool for the

BASIC programmer....  so chip in.

# With a

bit of your assistance this docum