Serial I/O Routines for use with QuickBASIC

This library will provide you with serial I/O communications

routines for use in QuickBASIC 4.x (with or without PDQ), and the

Microsoft Basic Compiler 7.x Professional Development System. No

longer will you be forced to use the poor communications support

provided by QB.  This program will allow you to control

communications ports 1 - 4 at speeds of up to 115200 baud. You will

no longer have problems with the DTR signal. It is left in the same

state it was in when before this driver is called, or it can be

controlled by your program. The serial driver includes XON/XOFF and

CTS handshaking. Serial input is interrupt driven, with incoming

XOFF flow control (if enabled) to prevent overrunning the input

buffer. These are the same serial I/O routines used in the User

Database System Doors "Query Door" and "Upload Door".

The driver was written using Microsoft's QuickC version 2.01. This

driver can ONLY be used with QuickBASIC version 4.x and BC7.0 PDS.

The reason is that versions of QuickBASIC prior to 4.0 do not

support the extensive multi-language interface that QB4.x has (via

the DECLARE statement). It seems to work fine with BASCOM 6 also,

although I did have problems with BC6 when the entire QuickC

library (MLIBCE) was used for linking. Since only portions of the C

library are included here, the error seems to be eliminated.

Throughout this manual QBSERIAL or QB will be used to refer to both

QuickBASIC and the new Basic Compiler 7.0 PDS.

Before the driver can be used, the following DECLARE statements

must be added to the beginning of your QB program:

DECLARE SUB OpenComm CDECL ALIAS "_open_comm" (BYVAL Port%, BYVAL_

Wlen%, BYVAL Parity%, BYVAL Baud&, BYVAL HS%)

DECLARE SUB CloseComm CDECL ALIAS "_close_comm" ()

DECLARE FUNCTION WriteChar% CDECL (BYVAL Ascii%)

DECLARE FUNCTION ReadChar% CDECL ()

DECLARE SUB Transmit CDECL ALIAS "_transmit_string" (addr$)

DECLARE FUNCTION DataWaiting% CDECL ALIAS "_data_waiting" ()

DECLARE SUB ClearInputBuffer CDECL ALIAS "_clear_input_buffer" ()

DECLARE SUB CarrierDetect CDECL ALIAS "_carrier_detect_flag"_

(BYVAL OnOff%)

DECLARE SUB CDtrap CDECL ALIAS "_trap_mode" (BYVAL OnOff%)

DECLARE FUNCTION CarrierLost% CDECL ALIAS "_carrier_state" ()

DECLARE SUB DTRcontrol CDECL ALIAS "_dtr" (BYVAL OnOff%)

These declarations specify all the entry points into the serial

driver, They are in the include file QBSERIAL.DEC. DO NOT change

them or the driver may not function.

Refer to the included sample programs SIMPLE.BAS & PCBDOOR.BAS,

they use most of the calls described below.

QBSERIAL User Manual - V 1.5                               Page 1

Port Initialization

OpenComm Port%, Wordlen%, Parity%, Baudrate&, HS%

Port%     an integer value of 0, 1, 2, 3, or 4. If the specified

port is 1 to 4, the driver seizes the serial interrupt

for this port (IRQ3 or 4). You must restore this

interrupt vector before program termination (explained

later). If the port is specified as ZERO the driver

enters "LOCAL" mode. This allows you to call the driver

with data, but the driver wont actually send anything.

This is useful when working with "doors".

Wordlen%  an integer specifying the word length of the serial data.

It has a value of 7 or 8.

Parity%   an integer, 0 = NONE, 1 = ODD, 2 = EVEN.

BaudRate& is a LONG INTEGER representing the desired baud rate.

Valid numbers are 0, 300, 1200, 2400, 4800, 9600, 19200,

38400, 57600, and 115200. The value you send is NOT

checked and I assume you could generate crazy baud rates

if desired. 115200 is the absolute maximum rate that can

be generated. If you specify a baud rate of ZERO then the

serial port is used AS-IS. The rate, word length, &

parity are NOT changed. This is useful for door operation

since the port is already initialized at the proper

settings when you get control.

HS%       an integer specifying the type of handshake you wish to

use between the CPU and Modem (or destination device).

Valid numbers are: 0 = NO handshake, 1 = XON/XOFF, 2 =

CTS, 3 = XON/XOFF and CTS.

Serial Output

To send a string of data out the serial port, you use the

"Transmit" call as follows:

Temp$ = "Transmit this..."

Transmit Temp$

or

Transmit "Send this also...."

If you want to transmit single characters, you can use the

"WriteChar" function. Transmit calls WriteChar to send the actual

QBSERIAL User Manual - V 1.5                               Page 2

characters to the port. If you wish to use WriteChar, it has the

following format:

Status% = WriteChar(Char%)

Char% is the integer value of the character you want to send, it is

not a string. Status% is an integer returned by WriteChar, it is

NON-ZERO if the character was actually sent to the port, and ZERO

if the character WAS NOT SENT because of No Carrier. Please note

that WriteChar will loop indefinitely on CTS hold, XOFF hold, or

Transmit Buffer Busy. These loops will exit if carrier is lost

(unless carrier detection is disabled), and return a ZERO status%.

WriteChar gives you more low level control of the sending of

characters, but you must send each character separately and monitor

its status.

The Transmit function monitors the status of its calls to WriteChar

and triggers QuickBASIC's User Event trap (UEVENT) if a failure

occurs. The UEVENT trap is used by default. You can disable the use

of this event trap in the event you already use the UEVENT trap for

something else. Therefore, by default, carrier loss detection in

your program is done with the ON UEVENT GOSUB/UEVENT ON statements.

The example programs show the use of this method. By default UEVENT

is also used for catching carrier loss on keyboard input. this is

described below.

Serial Input

Serial input is managed with three functions: DataWaiting,

ReadChar, & ClearInputBuffer. DataWaiting is used to test the input

buffer to see if any characters need to be read from it. ReadChar

is then called to get the characters. ClearInputBuffer flushes the

input buffer of any existing characters. Both example programs

contain a subroutine called "KeyboardInput" that demonstrates the

use of these calls. It also properly handles input from the local

and remote keyboards, and can be the routine you use in your

programs if you wish. Basically the procedure for reading the

remote keyboard (serial input) is as follows:

IF DataWaiting THEN

C$ = CHR$(ReadChar)

END IF

The DataWaiting function also monitors the state of the carrier

detect signal and triggers, by default, the user event trap

(UEVENT) if carrier is lost. As you can see a singular trap routine

catches carrier loss on output as well as input. You will notice

that the "KeyboardInput" subroutine loops using DataWaiting, and as

such, carrier is constantly checked while waiting for input. If the

QBSERIAL User Manual - V 1.5                               Page 3

ReadChar function is called when no data is available (as indicated

by DataWaiting) a NULL character is returned.

If XOFF/XON or BOTH is enabled, then the serial buffer will be

protected from overfills. When the buffer reaches 75% of capacity,

an XOFF will be sent on the serial output. When the buffer empties

out to 25% of capacity, an XON will be sent. The capacity of the

input buffer is 1024 bytes.

Carrier Detect Control

Implemented in version 1.1 is a function call to allow the calling

program to control if the driver senses carrier. The driver

defaults to carrier detection ON unless you implicitly turn it off.

The following call has been added to do this:

CarrierDetect 0 [or] 1

Specifying a ZERO turns carrier detection OFF. Even if you have the

UEVENT turned ON, no trap will occur if carrier is lost. Also, any

internal loops (such as XOFF wait) will NOT exit if carrier is

lost. Specifying a ONE turns carrier detection back ON.

Examples for the use of this would be where "plain Jane" serial I/O

is required where no modem handshake signals are used (simple 3

wire EIA). Carrier detection must be disabled in this mode

otherwise no data is sent. Another use might be in a program that

dials out on a modem to initiate a connection. Obviously at the

start of the call there is no carrier, and detection must be turned

off in order to send data to the modem. After a connection is

established, then the carrier detect can turned on again by the

program. Most applications however will not even use this function

call, since carrier detection defaults to ON.

Carrier Loss Detection

As mentioned above, by default, the UEVENT trap is used to detect

when a loss of carrier occurs (assuming carrier detection is

enabled as explained above). Implemented in version 1.2 are two new

function calls that allow you to detect loss of carrier without

using the UEVENT trap. While I feel that using UEVENT is the

simplest way to catch a carrier loss in a program, it is possible

that UEVENT might need to be used by something else. The two

functions used for this purpose are:

CDtrap 0 [or] 1

and

QBSERIAL User Manual - V 1.5                               Page 4

X% = CarrierLost

CDtrap needs to be used ONLY when UEVENT is used for another

purpose. Setting CDtrap to ZERO instructs the driver not to trip

the UEVENT flag in Basic if carrier is lost. As you can see if your

program uses UEVENT for some other purpose, you wouldn't want the

driver causing false interrupts (CDtrap 1 returns driver to

normal). If you do not plan to use UEVENT for carrier loss

detection, or for ANYTHING else, then you DO NOT have to set CDtrap

to zero. As long as you do not turn UEVENT ON in your program,

Basic will not respond to the UEVENT flag even if it was tripped.

The other function call allows to you to 'poll' for the state of

carrier. the "CarrierLost" functions returns a value indicating if

carrier is present of not. CarrierLost returns a NON-ZERO value

when there is NO carrier, and a ZERO when there IS carrier.

CarrierLost is a real time function, it returns the current carrier

condition at the time of the call. This allows you to code simple

IF or DO loops to detect loss:

IF CarrierLost THEN

..process carrier loss

END IF

Do NOT confuse these two functions with the "CarrierDetect"

function described in the previous section. That function controls

whether CD is checked before sending data, and what to do if no CD

is present. CDtrap only controls whether UEVENT is to be tripped or

not, and CarrierLost only returns the current carrier state.

Program Termination

Since "OpenComm" seizes interrupt vectors, these vectors need to be

restored BEFORE your program ends. If you neglect to restore these

vectors, you are guaranteed a crash. The routine used to reset the

vectors is: CloseComm. Only use CloseComm if you specified a port

value of 1, 2, 3, or 4 on the OpenComm call. If you specified a

port value of ZERO (Local mode) the interrupt vectors are NOT

grabbed, and do not need to be reset. CloseComm also resets any

UART registers to their original value, it also resets the

programmable interrupt controller (8259 PIC) to its original

settings.

Data Terminal Ready (DTR)

Everybody that has used QuickBASIC's OPEN COMn statement is very

familiar with the problems of the DTR signal. Apparently Microsoft

QBSERIAL User Manual - V 1.5                               Page 5

felt that DTR would never be needed after a program terminates.

Anybody who has ever tried to write a door, or chain from one

program to another has cursed this decision at some time during

their programs development. This driver handles the DTR with a

different view. When you close the comm channel the DTR signal is

left in the same state it was when you opened the comm channel. For

door applications, this is a must.

Implemented at version 1.2 is a function call that allows your

program to control the DTR signal:

DTRcontrol 1 [or] 0           ' 0 = DTR OFF, 1 = DTR ON

Additionally when you use DTRcontrol to change the state of the DTR

signal it also instructs the driver to leave it this way when the

CloseComm function is called. Remember that the driver leaves DTR

the way it found it when OpenComm was called. If you used some

other method to change DTR after OpenComm was called, it would

automatically return to that original state when you called

CloseComm. Therefor only DTRcontrol should be used if you wish to

change the state of DTR.

Cresent's PDQ Library

An object module for use with PDQ is also included. This version is

intended to be used ONLY with QuickBASIC 4.x. Actually the PDQ

version is identical to the QB version. The only difference is that

the PDQ version has no calls to UEvent (since there is no UEvent

routine in PDQ). You could in fact use either version (QBSER.OBJ or

QBSERPQD.OBJ) with QB 4.x if you never wanted to use UEvent as your

carrier detect trap. If you select to use QBSERPDQ.OBJ, substitute

it for QBSER.OBJ in the following paragraphs.

Linking

Since object modules are provided for both QB and BC7, you will

need to select which one to use for your program. The additional

library (SERIAL.LIB) only contains routines from the C library that

are required by the driver. The file QBSER.OBJ should be used for

programs compiled with QuickBASIC, and BC7SER.OBJ is to be used for

programs compiled with BC7 PDS. Typical link command lines would

look like this:

Link yourprog qbser,,,serial;

or

Link yourprog bc7ser,,,serial;

QBSERIAL User Manual - V 1.5                               Page 6

An alternate way would be to add to the SERIAL.LIB the obj file you

will always use. This requires you to have the LIB program (which

comes with both versions of basic). To all the object file to the

lib, use the following command:

Lib serial +qbser;

or

Lib serial +bc7ser;

after doing this, you can link as follows:

Link yourprog,,,serial;

Remember that the library you modify can only be used with the

version of Basic whose object module you add.

Quick Libraries

If you wish to develop programs from within the environment, you

will first need to create a Quick library using one of the object

modules (QBSER.OBJ or BC7SER.OBJ) and the SERIAL.LIB file. To

create a Quick lib for QuickBASIC 4.x, use the following Link

command:

Link QBSER SERIAL.LIB,,,BQLB4x/q

Where "BQLB4x" is one of the following: BQLB40.LIB, BQLB41.LIB, or

BQLB45.LIB. Use the one that came with your compiler. If you use

the wrong BQLB file you might get an "Invalid Format" error when

attempting to start QuickBASIC. QBSER.OBJ & SERIAL.LIB may also be

added to Quick Libraries containing other routines and OBJ's as

well.

To create a Quick Library for use within QBX (BC7's environment)

use the following link command (be sure to use Link v5.05 that came

with BC7):

Link BC7SER SERIAL.LIB,,,QBXQLB/q;

Remember too, that you can also add BC7SER.OBJ & SERIAL.LIB to

Quicklibs containing other routines.

QBSERIAL User Manual - V 1.5                               Page 7

Specifications

The following port addresses and interrupts are used:

Comm      Base Address        IRQ

1          3F8 hex            4

2          2F8 hex            3

3          3E8 hex            4

4          2E8 hex            3

Support

I'm reachable in the Interlink Basic conference, or the number

below. I'll help there if you have questions. No other support is

necessary. Bugs will be tended to if required, and good suggestions

tend to be implemented.

In the spirit of programmer to programmer I am not requesting any

compensation for the use of these routines other than you

acknowledge in your documentation that you are using them. If you

feel they are worth something and you want to send me something I

won't refuse it! If you are using them in a PCBoard door, a copy of

the door for our system would be a nice gesture!

However, if you plan to use QBserial in a commercial product for

resale, please contact me first.

Source Code

The source code for QBserial is now available. The source is ONLY

available by signing a license agreement. The source may be used

for internal use only. The source may not be distributed by you to

any other person, even if you make changes to it. Nor can the

resulting object code be sold or distributed. Modifications can

only be used in your end product, and without any type of royalty.

The license fee for the QBserial source code is $75.00. Please

contact me if you are interested.

Jeff Sumberg

Sysop(2) - SailBoard BBS

Wayne, NJ, 201-831-8152

[ or ]

Box 212

Ringwood, NJ, 07456

QBSERIAL User Manual - V 1.5                               Page 8

Changes and Revisions

06/09/89  1.0       Initial Release

06/21/89  1.1       Added carrier detection control. Calling

program can turn detection off and on as

desired. Necessary if data sending/receiving

required if carrier isn't present.

09/01/89  1.2       Fixed a bug in Readchar where 'extended'

characters (ASCII 128 to 255) were causing an

Illegal Function Call in the CHR$() conversion

(because they were being returned as negative

numbers). Extended character may now be

received properly. Added three new functions:

CDtrap, CarrierLost, and DTRcontrol.

1.3

1.4       Un-released interim versions.

03/24/90  1.5       Added support for Basic Compiler 7.0 PDS and

PDQ. Added sections to manual about Linking,

Source code availability, and updated section

on Quicklibs. Separate object files now

supplied for QB, BC7, and PDQ.

QBSERIAL User Manual - V 1.5                               Page 9