PERFECT BOX Version 3.00                    7/88

Thank  you  for your support of PERFECT BOX.   I  have  spent

hundreds of hours massaging PERFECT BOX into its current version.

Initially, the program was only intended for my own purposes,  but

after  time  I  realized that a very large  hole  existed  in  the

software domain for the serious loudspeaker designer.

I have seen a few very simple routines that do the  "classic"

ideal box calculations, and concluded that there must be a way  to

make those hand calculations mean more that dB's here and  there!

That is when I though a graphic plot would make the design process

go  faster.  Thus, the old standby program, "BOXRESPONSE,"  became

the foundation on which PERFECT BOX was built.

Now,  the BOXDATA database file holds all the information  in

one  nice,  concise form, readable in seconds, for  program  use!

Gone  are the days when you can't finish (or start) a  new  design

because the T/S parameters are nowhere to be found!

Speaking  of  the  BOXDATA  FILE, I  have  added  a  few  new

routines,  external  from the main program, that will  APPEND  new

records  from another BOXDATA "like" file.  I say  "like"  because

the  NEW  file  is of the same format as  BOXDATA,  but  is  named

"DATAUP.DAT."   You  will find a few new BOXDATA  records  on  the

enclosed  disk  in  this updated file.  To  load  them  into  your

current  file, use the addata.exe routine.  It will either add  to

or  replace the records in your BOXDATA file (in the  current  DOS

default  directory) with the records in the "DATAUP.DAT" file,  by

your choice.

The  other new routine, BOXSORT.EXE,  alphabetically  sorts

the  BOXDATA by the manufacterer (1st Key) and model  number  (2nd

Key.)  To start the sort, execute the BSORT.BAT file from the  DOS

prompt (you can use the new DOS SHELL option from the opening menu

of  PERFECT  BOX.)   This routine will  also  delete  any  BOXDATA

records  with the word "DELETE" as the model or  manufacturer,  or

any records with three spaces first in both of those fields.   You

must  use EDIT to enter those spaces or the word DELETE (in  caps)

for the BOXSORT program to recognise them for deletion.  The  sort

program   itself  only  creates  a  new  temporary   file   called

BOXDATA.SRT.  Feel free to modify the batch file to your needs.

You  will  notice a few changes in the  PERFECT  BOX  program

itself including...

1.   Color is now supported!  You may change the colors  (a

temporary  change)  by typing "COLOR" as the "New  path"  for  the

BOXDATA file.

You  may  use either CGA, EGA,  or  Hercules(Tm)  graphics

modes.   The mode can be set by changing the "PERFECT.CTL"  file.

Use "CGA", "EGA", "HERCULES", or "NO", for no graphics.  (You must

use the quotes.)   If you are going to use Hercules graphics,  you

must first run the QBHERC.COM program before starting PERFECT BOX.

If  you  choose EGA mode, you will have to use  the  Toggle

Graphics Mode" function to do hard-copies of the graphics  plots.

I do all of the main work in EGA mode, then switch to CGA for  the

screen print function.  For you Apple fans, I am currently working

on converting PERFECT BOX to run on 512K Macintosh machines  (full

window operation!.)

2.   A SHELL to DOS is now supported.

3.   When you are ADDing records to the database, the  Xmax

prompt  will  accept  either inches or millimeters,  and  the  Vas

prompt will accept either cubic feet or cubic liters.  Just  press

RETURN to change back at forth at those prompts.

4.   A "Ideal box" calculation for all the drivers  in  the

BOXDATA  file can be printed easily from the "Automatic  Parameter

Entry area.

5.   The Port Table screen has been fixed up to line up  on

the CRT, and is color coded for each port size; RED-Negative  port

length,  (not usable), YELLOW-Ok, but a bit long,  and  GREEN-Good

choices.

6.  The Ideal box values are rounded to two places past the

decimal for CRT output, but not in calculations.

7.    Coprocessor support is automatic.  If you don't  have

one,  you  will  notice  a slight  loss  of  speed  from  previous

versions.  I have adjusted the calculation of data points to  skip

over the odd frequencies above 100 Hz.  If you don't use an  8087,

just go get another cup of Java!

8.  The BOXDATA file is stored in a different form than  in

previous  versions.  Your old file is NOT compatible(!) UNTIL  you

run  the conversion program included with PB.  The new  data  file

name  is  BOXDATA4.   You can Append your old  parameters  to  the

BOXDATA4  file after they are converted.  Just be careful  not  to

overwrite  the new BOXDATA4 file included with this release.   The

2BOX3.EXE conversion program takes your old BOXDATA file as input,

and  produces  the  new  BOXDATA4 file.   I  suggest  you  do  the

conversion then rename the converted file to DATAUP.DAT to be used

as input to ADDATA.EXE program.  Making (keeping) a backup copy of

all  of your data files is strongly recommended before  attempting

the changeover.

If you are doing all of this conversion stuff in a different

directory (or drive) than where the runtime module, BRUN40.EXE  is

located, you may get an error that says .."BRUN40 cannot be  found

,  enter  new path.." Just give the path to  the  directory  where

BRUN40 is located, (like C:\PERFECT.)

I  want  to comment on the BOXDATA records that I  have  been

receiving  on  your  disks.  First, I think that  many  users  are

confusing  the  definition  of the X-max  parameter.  The  typical

ranges for X-max range from 0.1 inches to 0.25 inches with 0.5  on

the JBL 18" 2245H being the largest I've seen (Have you ever  seen

one,  they  are  huge.)   If  you  don't  know  the  X-max  value,

approximate it with 0.1 to 0.15, 0.2 for large suspensions.

Note  that  X-max  is NOT the total  possible  cone  movement

allowed  by  the suspension, which on large drivers  could  be  at

least  an  inch.  I  have seen specification  sheets  from  a  few

manufacturers (Credence is one) that give an "X-max" and a "X-lin"

(for X-linear?) value. As far as I can tell from these, the  X-lin

is really the value of X-max as normally defined. And the  "X-max"

value   is  the  maximum  cone  travel  possible.    Contact   the

manufacturer  if you are confused about their  specifications.   I

hope that a standard for "X" measurements will surface soon.

The X-max measurement defines the peak (not to be  confused

with peak-to-peak) distance that the voice coil can travel without

extending  beyond  the magnetic field as determined  by  the  pole

piece  length.   The voice coil may be longer in length  than  the

pole  piece.   This  is defined as  "voice  coil  overhang."   The

program  uses  the X-max measurement in  calculating  the  Maximum

Sound  Pressure  Level (SPLmax), and thus is a key factor  in  the

Excursion Limit Plot.  Theoretically, the driver is "linear"  only

if  the cone travel is within the X-max limits!  Think about  this

the next time you watch your low-end drivers move 3/4 " peak, from

the rest position!

In general, (not always,) low Q drivers are intended for  4th

order  systems, and typically have what is coined  as  "long-throw

suspension."  Also, remember that PERFECT BOX model is valid up to

Qts of 0.7, and the Marquis 12G38BPP1H is slightly over that value

at  0.72.   Of course, each individual driver will vary  from  the

manufacturers  published specs, so you might want to measure  them

yourself to be sure.

Good  luck in your new designs, thanks again for all of  your

generous  support.   If you have any specific questions  or  would

like  chat about loudspeaker topics, drop me a letter, or call  my

new BBS, "UCF Health Topics" at (407)-281-5522, 2400 N-8-1.

You will find the latest release of Perfect Box posted here.

My  BBS  is an independent node in FidoNet, (1:18/507),  and  thus

will  accept  "Electronic  Mail" 24 Hours a day.   If  you  are  a

FidoNet  SysOp  or  know  a  friendly  one  with  a   "BinkleyTerm

compatible  Mailer", you can file request with the magic  filename

"PERFECT",  for  the  current  MS-DOS  version,  or  MACPERF   for

Macintosh  (when  available.)  Of course, you  could  always  File

Request by the current archive name "PERF300.ARC."  Also, I host a

local  echo-mail roundtable called, "Audio Topics".  You can  find

many questions regarding Loudspeakers, etc. answered there.

Warren A. Merkel

*** End of Document ***

##############################################################################