Draftsman's Helper Ver 1.0

By: Joe Thompson

8-2-92

Compuserve ID: 71321,1177

EET BBS Fido-net address 1:116/24

615-325-7800

Disclaimer and limit of liability

Joe Thompson makes no warranty of any kind, either express

or implied, including but not limited to implied warranties

of merchantability  and  fitness for a particular  purpose,

with respect to this software.

In no event shall  Joe Thompson  be  liable for any damages

(including damages for loss of business profits, business

interruption, or loss of business information) arising out of

the use of this software.

Summary

The Drartsman's Helper is an Autocad ADS program that

draws several types of geometry including ANSI geometric

tolerancing symbols, drilled holes, tapped holes, counter-

bored holes, cap screws, flathead screws, and button head

screws. The Geometric symbols are stored as blocks in a

single drawing and xrefed in as needed. The other objects

are parametricaly draw as required. This system is highly

user customizable. All parametricaly drawn objects get

their required data  from an ascii file stored in your

Autocad path. This file may be edited with new elements

added or deleted and old ones changed to any size required.

The program uses the autocad rel 12 dialog interface to

present the elements with slides and scrolling menus. The

user may add blocks to the drawing DHELPDAT.dwg and these

will automaticaly become part of the program the next time

it runs. This means that the user may add as many blocks

to the file as he needs to and the next time DHELP is run

these blocks will appear in the list box for selection and

insertion into the current drawing.

Caution! DHELP may not be ran from the drawing file DHELPDAT

because of the nature of how XREF's work.

There is also a radio button on the main DHELP dialog that

is labeled User blocks. This is a whole section of the

program alloted for only user defined blocks. The only

restrictions are that all blocks be inside DHELPDAT and that

the first three letters of the block name be USR. If you

wish to add geometric symbols the first three letter of your

symbols must be GEO.

This program uses extended data to retain its current position

in the menu system. This is completely transparent to the user

and translates to any time the program starts it takes you

back to  exactly where you were when you last left the program

this counts from drawing session to drawing session to. So if

you are somewhere deep in a list, draw the component and exit,

and then exit the drawing, when you come back if it is the next

day or the next year when you start DHELP you will be in the

same position as you were when you last left.

I have also included a utility for generating slides from

blocks it is called

compile_slides

it is inside dhelp.exp and can be invoked by typing its name.

The utility only works on blocks but seems to do a fair job

of building slide libraries. The program deletes all non-block

objects in the drawing before it begins it then freezes all

objects , copies 1  block to a non frozen layer, finds the bounds

of the block zooms in on it and makes a slide. This process is

repeated for all blocks in the drawing. This program was developed

primarily to work with DHELP but it can be used for other things.

I have included two sample blocks in the drawing DHELPDAT as a

guide to how additional blocks may be added in the USER

category.

The process is fairly simple to add blocks and their corresponding

slides to the DHELP program. The process is this:

1. Create a new block in the drawing DHELPDAT. This new block

may be anything the user wishes. (Multiple blocks may be

developed and processed at one time).

2. Make sure that SLIDELIB.EXE is in the DOS path.

3. Run the compile_slides sub-program to build a slide library

of all new and current blocks.

4. End the DHELPDAT drawing.

5.  Copy all files DHELPDAT.DWG, DHELP.SLB, DHELP.DAT, DHELP.DCL

somewhere into your Autocad path.

6. Open a new drawing or edit an existing drawing and your

new slides and blocks should be available inside the program.

Requirements

This program requires Autocad rel 12 or above.

Installation

To begin the program it must first be loaded into Autocad.

This is accomplished by typing the Autolisp command:

(xload "DHELP")

This command is required only once per Autocad drawing session.

Once the program is loaded it can be ran by typing:

DHELP

The program may be loaded automaticaly at startup by making

the following modifications to your ACAD.ADS file.

DHELP

If you do not have an ACAD.ADS file a simple ASCII text file

with the above line will do fine.