PD CADD DEMO VERSION:

PD CADD retails for $99.95 complete with user manual.

To place orders call Sundtec Software Systems at

1-404-335-7009 or

1-404-335-9084 Fax

or write to:

Sundtec Software Systems

Rt 3 Box 389A

Commerce Ga  30529

PD CADD requires a minimum of 640K RAM, a VGA graphics display, mouse

and MS DOS 3.0 or later.

A hard disk is recommended but not required.

REQUIRED:

640K RAM

VGA graphics display

1 Floppy disk drive

Microsoft compatible Mouse or Track Ball

OPTIONAL:

Epson compatible dot matrix printer

Hewlett Packard Series II compatible printer

Hewlett Packard 7470A compatible plotter or Laser printer

Math Coprocessor

Expanded Memory

Hard disk

RECOMMENDED:

Math Coprocessor

Hewlett Packard 7470A compatible plotting device

Hard disk

Expanded Memory

Extended Memory disk cache

The use of a Math Coprocessor is not required, but will improve the

speed performance.

For optimum performance use expanded memory of at least 560K and an

extended memory disk cache of at least 300K.

PD CADD requires a minimum of 480K free DOS Ram.

The first thing you should do is to read the WARRANTY NOTICE and complete

your registration card and mail it to SUNDTEC SOFTWARE SYSTEMS.

This will insure that you will be notified of any future updates.

NOTE: YOUR MOUSE DRIVER MUST BE LOADED BEFORE USING PD CADD.

INSTALLATION HARD DISK:

Step 1: Backup the original disk(s) using DISK COPY.

Refer to your DOS manual for instructions.

Step 2: Insert the installation disk in Drive A: and type

A:INSTALL and the drive letter to install it on.

For example, to install on your C: drive type,

A:INSTALL C:

Step 3: Load your mouse driver.  Refer to your mouse

documentation for the proper procedure.

After the installation is complete, you are ready to begin using PD

CADD.

After you exit the Installation program you will be in the PDC subdirectory.

Type PDC to begin using the program.

INSTALLATION FLOPPY DISK:

Step 1:         Backup the original disk(s) using DISK COPY.

Refer to your DOS manual for instructions.

If DISK COPY cannot be  used, make sure to

create \DWG and \BLK subdirectories on the

floppy, refer to your DOS manual for the

proper procedure.

Step 2:         Load your mouse driver.  Refer to your mouse

documentation for the proper procedure.

The Backup disk will now be your program disk.

Remove the original PD CADD installation disk and insert your new program

disk.

Type A: and press enter.

Type PDC to begin using the program.

The rest of the manual describes the use of all drawing commands available

with examples.

We at SUNDTEC SOFTWARE SYSTEMS are committed to providing quality software

and pride ourselves on expert technical support.

If you have any questions, problems or comments, call 404-335-7009, 8:00 AM

to 5:00 PM EST Monday thru Friday or Fax to 404-335-9084.  Have your product

serial number ready and ask for PD CADD Tech Support.

Two or three button Microsoft compatible mice can be used with PD CADD.

The left Mouse button is used to lay down points to draw objects or used to

point to objects such as telling the computer which line to erase.

The right mouse button is used to Snap (move) the mouse cursor to the

nearest point from the cursor.

On three button models, the center button is the same as pressing

Esc on your keyboard for many of the commands.

Selecting commands can be accomplished several different ways.

1)      From the top pull down menus.

2)      From the Side bar menu.

3)      By typing its two letter command.

Pressing the space bar on the keyboard repeats the last command issued.

Orthogonal (Ortho) lines, (lines that are either vertical or horizontal),

can be drawn by holding down the Control key on your keyboard while moving

the mouse cursor.  Releasing the Control key returns you to normal line

drawing mode.

Leaving your monitor on for an extended period of time with a displayed

image can cause Screen Burn.  PD CADD has a built in screen blanker to help

prevent this.  PD CADD will activate a utility which will blank the screen

display after approximately 15 minutes of inactivity.

PD CADD does not check for inactivity when the mouse cursor is in any of the

menu areas or waiting for you to complete a selected command.

When you draw,  you will probably want to draw to scale so it will

look proportional.  Drawing lines or other objects to a specified

length can be accomplished by first setting your starting point for

a line, as you normally would with the left mouse button, and typing the

distance to move.  All dimensions typed are considered to be referenced from

your last point, this is called moving relative.

For example (assuming your working in Feet and Inches (FI) units):

Suppose you want to draw a horizontal line exactly 40 feet long.

Type 40',0 and press enter.  This distance command tells the computer to

move 40 Feet in the X-Direction and 0 Feet in the Y-Direction.

If you type dimensions without the Foot (') mark, the computer assumes that

you are entering dimensions in inches, (FI units mode).

To  draw a line from the start point, 24 inches to the right and 24

inches down, type 24,-24 or 2',-2'.  Note here that - (minus) was

used to move down in the Y-Direction.  A negative X-Direction dimension will

cause the line to be drawn to the left of your reference point.

Y-Direction dimensions can also be input as Degrees or Slope.

Suppose you want to draw a line with a 144 unit horizontal projection on a

4/12 slope.  Type 144,4/12 and press enter.  This distance command tells

the computer to move horizontally 144 units and to rise 4 units for every

12 units of projection.  To draw a line with a 144 unit horizontal

projection at a 30 degree angle can be accomplished by typing 144,30d.

This tells the computer to move 144 units to the right and up at a 30

degree angle.

The file menu is used for all input and output functions.

Input meaning, retrieving drawings, blocks, etc.

Output meaning, saving drawings, blocks, etc. as well as printing drawings

to a compatible printer or plotter and entering the calculator box.

To retrieve drawings that were created with PD CADD and are saved on disk.

FILE MENU COMMANDS: =====================================================

DRAWING LOAD-DL

PURPOSE

Type DL or select DRAWING LOAD from the menu.

A "pop down" window will appear listing all the drawings catalogued

on your disk. The highlighted bar moves up and down by moving your

mouse.  When the desired drawing is highlighted, press your left

mouse button.  This will automatically load the selected drawing.

Pressing Esc or selecting Exit from the menu will terminate the

Drawing Load command.

HOW TO USE

You can also type in a drawing name to retrieve, complete with

drive and path specification.

REMARKS

See DRAWING SAVE, for related command

DRAWING SAVE-DS

PURPOSE

To save drawings on your disk.

HOW TO USE

Type DS or select DRAWING SAVE from the menu.

Type a name for your drawing (without an extension) and press enter

or press enter if the correct drawing name appears in the

command box.

Drawing files are saved with the extension .DWG

Drawing name is limited to 8 characters, A - Z, 0-9 , -- OR _.

Do not use any special characters, like comma, period, space, etc.

in the drawing name.

REMARKS

When you are creating a drawing, it's a good idea to save your work

frequently because if power is lost, your drawing is also lost.

If the drawing name already exists on disk, the command box notifies

you of this and asks if you want to overwrite it.

If you do, type Y for Yes or O for Overwrite.

See DRAWING LOAD for related command.

PLOT-PL

PURPOSE

To Plot your drawing to a compatible Plotter, Printer or disk file.

HOW TO USE

Type PL or select PLOT from the menu.

Then select the options you desire from the menu.

REMARKS

It's a good idea to save your drawing before you do a plot because

if something goes wrong, you will still be able to reload the

drawing and try again.

Make sure your plotter or printer is "on" and loaded with paper.

DXF OUT-DF

PURPOSE

To export a DXF file which is compatible with many other CAD programs.

HOW TO USE

Type DF or select DXF-OUT from the menu.

Type a name for your DXF file (without an extension) and press enter

or press enter if the correct name appears in the command box.

DXF files are saved with the extension .DXF

The file name is limited to 8 characters, A-Z, 0-9, - or _.

Do not use any special characters, like comma, period, space, etc.

in the file name.

REMARKS

If the file name already exists on your disk, the command box

notifies you of this and asks if you want to overwrite it.

If so, type Y for Yes or O for over write.

See SAVE TXT for related command.

SAVE TEXT-ST

PURPOSE

A .TXT file can be exported to other compatible software.

HOW TO USE

Type ST or select SAVE TXT from the menu.

Type a name for your TXT file (without an extension) and press enter

or press enter if the correct TXT name appears in the command box.

TXT files are saved with the extension .TXT

The file name is limited to 8 characters, A-Z, 0-9, - or _.

Do not use any special characters, like comma, period, space, etc.

in the file name.

REMARKS

If the file on your disk already exists, the command box notifies

you  and asks if you want to overwrite it.  If so, type Y for Yes or

O for Overwrite.

See DXF-OUT for related command.

LOAD PLOT-LP

PURPOSE

To load an HPGL compatible file in as a drawing.

HOW TO USE

Type LP or select LOAD PLOT from the menu.

Type the name of the plot you want to retrieve (without an extension)

and press enter.

REMARKS

Many software packages allow you to output HPGL files to disk.

This option allows you to import these files as a drawing file into

PD CADD.

The plot file must have the extension .PLT.

DRAWING RESIZE can be used to convert the drawing to any scale.

You can also type in a plot name to retrieve complete with drive

and path specification.

See DRAWING RESIZE for related command.

DATA IN-DI

PURPOSE

Data in is used to read an ASCII file containing drawing commands.

HOW TO USE

The drawing commands and syntax are listed below.

OR = Origin Set

Syntax:   OR; pointx1!, pointy1!

Function:

All subsequent drawing commands will reference this point

as the origin.

If OR is not used, PD CADD will prompt you to locate an

origin.

LI = Draw Line

Syntax:   LI; pointx1!, pointy1!, pointx2!, pointy2!

Function:

Draws a line from pointx1, pointy1 to pointx1, pointy1.

Point coordinates are referenced from the specified

Origin location.

LC = Line Color

Syntax:   LC; linecolor%

Function:

Specifies the line color for all subsequent line (LI)

commands.

Allowed range for linecolor is 1-15 inclusive.

LT = Line Type

Syntax:   LT; linetype%

Function:

Specifies the line type for all subsequent line (LI)

commands.

linetype 0= Solid Line

linetype 1= Dashed line

TA = Text Angle

Syntax:   TA;angle!

Function:

Specifies the Text Angle in degrees for all subsequent

Text Place commands.

TZ = Text Size

Syntax:   TZ;size!

Function:

Sets the text size for all subsequent Text Place

commands.

TP = Text Place

Syntax: TP;pointx1!,pointy1!,number lines of text

text string data (line 1)

text string data (line 2)

and so on....

Function:

To place text in your drawing at pointx1,pointy1.

Pointx1 and pointy1 are referenced from the

defined origin.

Number lines of text, tells PD CADD how many lines

of text to read.

DU = Drawing Units

Syntax: DU;unit$

Function:

To specify the drawing units.

i.e.: unit$ = mm for millimeters etc.

RE = Rectangle

Syntax: RE;pointx1,pointy1,pointx2,pointy2

Function:

Used to draw a rectangle starting at pointx1,pointy1,

referenced from the origin.

pointx2, pointy2 define the opposite corner of the

rectangle, referenced from pointx1,pointy1.

CI = Circle

Syntax:         CI;pointx1!,pointy1!,radius!

Function:

Used to draw circles whose center is at pointx1,pointy1

referenced from the origin.

Radius is the radius of the circle referenced from the

center, (pointx1,pointy1).

PO = Polygon

Syntax:   PO;pointx1!,pointy1!,radius!,number of sides%

Function:

Used to draw polygons whose center is at pointx1,pointy1

referenced from the origin.

Radius is the radius of the polygon referenced from the

center, (pointx1,pointy1)

Number of sides specifies the number of sides to draw.

ZA = Zoom All

Syntax: ZA;

Function:

Performs the Zoom All function.

RD = ReDraw and RG = ReGen

Syntax: RD;  or RG;

Function:

Performs the Redraw or Regen function.

READ SCRIPT-RS

PURPOSE

A script file is an ASCII file that you can create, to ask

questions while in PD CADD, and are a method used to customize

PD CADD to do certain functions that are not built in.

1)      Answers to your script questions are saved in a file so

they can be manipulated externally by another program.

2)      Your external program, (must be an .EXE, .COM or .BAT

file) can be automatically run from within PD CADD.

3)      Your external program needs to output data into a file,

following the syntax described for DI (Data In).

4)      Script questions can be smart, that is they can make

decisions depending on the answer to a question and

branch or goto another statement in your script file.

ie IF YES THEN GOTO :ENTER DATA

This script statement determines if the previous question

was answered with YES, and if so the script branches to

a label named, :ENTER DATA, and continues execution.

If the answer was not YES then the next statement in your

script file is executed.

SCRIPT FILE SYNTAX:

The first line in a Script File must be, SCRIPT QUESTIONS?

A script file command must start with a star (*).

A label must start with a colon (:)

SCRIPT FILE COMMANDS:

*GOTO   branches to a label.

*EXIT   exits or aborts the script file.

*IF     tests the last answer or a *VAR for a match.

*RP     runs the specified external program, must be an .EXE,

.COM or .BAT file.

*DI     reads a Data In file .

*OPEN   open a data file for output (data file must be preceded

with a colon)

*POINT  allows you to select points in your drawing.  Selected

x,y point locations are output to the specified data

file.  The first point located is also considered as

the origin point for all subsequent POINT and LINE

commands.

*LINE   Allows you to select lines in you drawing.  The selected

line end points, (pointx1,pointy1,pointx2,pointy2)

are output to the specified data file.  If no origin has

been set, pointx1,pointy1 becomes the origin for all

subsequent POINT and LINE commands.

Below is a sample Script file:

line 1  script questions?

line 2  :start

line 3  *open :roof.out

line 4  Common Roof pitch (4/12)

line 5  *IF 0 GOTO :LOCATE POINTS

line 6  Hip end pitch (4/12)

line 7  Overhang or Cantilever (24")

line 8  Spacing (24")

line 9  Max setback (96")

line 10 :LOCATE POINTS

line 11 *point 3

line 12 Locate first corner of hip

line 13 Locate 2nd corner of hip

line 14 Direction of hip peak

line 15 *rp pdc\script\hip.exe

line 16 *di roof.di

line 17 *goto :start

Line by line description for above Script file

line 1  1st line of script file

line 2  Label :start

line 3  Opens the data file roof.out for output

line 4  Script question, result is output to file roof.out

line 5  If 0 was input then branch to label :LOCATE POINTS

line 6  Script question, result is output to file roof.out

line 7  Script question, result is output to file roof.out

line 8  Script question, result is output to file roof.out

line 9  Script question, result is output to file roof.out

line 10 Label :LOCATE POINTS

line 11 Asks operator to locate 3 points and to use the next

three questions to prompt the operator.

line 12 first point (user selects with mouse). The x and y location

of the point is output to file roof.out

line 13 second point (user selects with mouse). The x and y

location of the point is output to file roof.out

line 14 third point (user selects with mouse). The x and y location

of the point is output to file roof.out

line 15 Runs external program named hip.exe in directory

\pdc\script

line 16 Reads data in from file roof.di, (that was created by

the program run in the previous line).

line 17 Branch to Label :START (this starts the process over again)

Pressing ESC at any time will exit the script at the current

location in the Script file.

As you can see, Script questions and programs can be created to

do practically any kind of external computation on data output

by the operator.  The data created by your external program can

then be read back in with the DI function.

RUN PROGRAM-RP

PURPOSE

To run an external program and return back to PD CADD

automatically.

HOW TO USE

Type RP or select RUN PROGRAM from the menu.

Select the desired program from the pull down menu or

type the program name including path and extension.

REMARKS

When RP is selected, a pull down menu appears listing all

.EXE programs contained in your PDC\SCRIPT directory.

To run a program not in that directory, type in the drive,

path and program name to run.

ie: to run a program named WP.EXE on your D: drive in

directory WP50, type D:\WP50\WP.EXE.  When the selected

program finishes, you will automatically be returned to

PD CADD.

BLOCK SAVE-BS

PURPOSE

To save created blocks to disk.

HOW TO USE

Type BS or select BLOCK SAVE from the menu.

Type the name of the Block you wish to save and press enter.

The Block must be in memory before you can save it to disk.

Block files are saved with the extension .BLK

REMARKS

If the block name already exists on disk, the command box will

notify you and requests if you want to overwrite it.

If so, type Y for Yes or O for Overwrite.

See BLOCK LOAD and the DRAW section for other block related commands.

BLOCK LOAD-BL

PURPOSE

To retrieve blocks from disk into memory in order to select

frequently used symbols without re-drawing them each time.

HOW TO USE

Type BL or select BLOCK LOAD from the menu.

A "pop down" window will appear listing all the blocks catalogued

on disk.  The highlighted bar moves up and down by moving your mouse.

When the desired block is highlighted press your left mouse button

which will automatically load the block into memory.

REMARKS

You can also type in a block name to retrieve complete with drive

and path specification, do not include a file extension.

A maximum of 20 Blocks can be in memory at one time.

Blocks take up precious memory, so its a good idea to unload any

blocks your finished using.

See BLOCK SAVE, BLOCK UNLOAD and the DRAW section for related

commands

SHELL-SH

PURPOSE

To shell out of PD CADD to your DOS prompt.

HOW TO USE

Type SH or select SHELL from the menu.

Type EXIT to return to PD CADD.

REMARKS

Before the Shell takes place PD CADD frees all unused

memory to make as much memory available as possible.

That is to say the amount of available memory depends on

how many lines are drawn in your current drawing.

If you are using Expanded Memory, all variable memory

will be freed up, so you get the maximum amount of memory

not dependent on how many lines you have drawn.

Shell allows you to run a DOS command or to run a specified

program.  When you are ready to return to PD CADD make sure

you are in the PDC directory and type EXIT.

CALCULATOR-CA

PURPOSE

On screen Calculator.

HOW TO USE

Type CA or select CALCULATOR from the menu.

Press ESC to exit.

REMARKS

Numbers and simple math calculations can be entered from the

keyboard, allowed keys are, 0 thru 9, /, *, -, + and =.

The Enter key performs the same function as =.

The Delete key performs the clear function.

CONFIG SAVE-CS

PURPOSE

To save the current settings as defaults in the config.fil

file.

HOW TO USE

Type CS or select CONFIG SAVE from the menu.

REMARKS

Configuration items saved are listed below,

Current Line Color

Current Test Size

Current Text Width Factor

Current Text Line Spacing Factor

Current Drawing Unit of Measure

Current Decimal Places to display

Current Dimension Extend, Away

Current Dimension Extend, Toward

Current Dimension Extend, Stretch

All of these settings are saved each time you select this command.

Configuration items relating to plotting can be changed in the

Plotting menu, select (Plot-PL) for these options.

CONFIG LOAD-CL

PURPOSE

To load the default configuration from the config.fil

file stored on disk.

HOW TO USE

Type CL or select CONFIG LOAD from the menu.

REMARKS

Configuration items loaded are listed below,

Default Line Color

Default Test Size

Default Text Width Factor

Default Text Line Spacing Factor

Default Drawing Unit of Measure

Default Decimal Places to display

Default Dimension Extend, Above

Default Dimension Extend, Below

Default Dimension Extend, Stretch

All of these settings are loaded each time you select this

command.

QUIT-QU

PURPOSE

To exit the PD CADD system and return to DOS.

HOW TO USE

Type QU or select QUIT from the menu.

You will be asked if you want to save the drawing.

If you do ,type Y for Yes. Any other letter will not save the

drawing.

You are prompted one more time if you desire to Quit or Continue.

Type Q to quit or C to continue and stay in the CADD system.

REMARKS

If you will be using your current drawing again,

MAKE SURE YOU SAVE IT.  If you don't, it will be lost forever

in never, never land!

SNAPS MENU COMMANDS: =====================================================

SNAP PARALLEL-S=

PURPOSE

To snap the second end point of the line currently being drawn,

parallel with another existing line.

HOW TO USE

Start a line by laying down the first point.

Type S= or select SNAP PARALLEL from the menu.

Point to the line which you want to draw parallel to with the mouse

cursor and press the left mouse button.

Locate the end point of your line by moving the cursor to the

desired location and press the left mouse button.

REMARKS

After your new line is drawn, you can trim it off with the EXTEND,

FILLET or TRIM command.

See SNAP PERPENDICULAR, SNAP MIDPOINT, SNAP INTERSECTION and

SNAP CENTER.

SNAP PERPENDICULAR-S+

PURPOSE

To snap the second end point of the line currently being drawn,

perpendicular with another existing line.

HOW TO USE

Start a line by laying down the first point.

Type S+ or select SNAP PERPENDICULAR from the menu.

Point to the line which you want to draw perpendicular to with the

mouse cursor and press the left mouse button.

Locate the end point of your line by moving the cursor to the

desired location and press the left mouse button.

REMARKS

After your new line is drawn, you can trim it off with the EXTEND,

FILLET or TRIM command.

See SNAP PARALLEL, SNAP MIDPOINT, SNAP INTERSECTION and SNAP CENTER.

SNAP MIDPOINT

PURPOSE

To locate the midpoint (center) of an existing line.

HOW TO USE

Type SM or select SNAP MIDPOINT from the menu.

Move the mouse cursor to the desired line and press the left mouse

button.

This command can be used to set the second point of a line being

drawn or to set the first point of a line to start.

REMARKS

See  SNAP PERPENDICULAR, SNAP PARALLEL, SNAP INTERSECTION and

SNAP CENTER.

SNAP INTERSECTION-SI

PURPOSE

To locate the intersection point of any two lines and attaches the

rubber band line to that point.

HOW TO USE

Type SI or select SNAP INTERSECTION from the menu.

Move the mouse cursor close to the desired intersection point and

press the left mouse button.

This command can be used to set the second point of a line being

drawn or to set the first point of a line to start.

REMARKS

See SNAP PERPENDICULAR, SNAP MIDPOINT, SNAP PARALLEL and SNAP CENTER.

SNAP CENTER-SC

PURPOSE

To locate the center point of a circle nearest to the mouse cursor.

HOW TO USE

Type SC or select SNAP CENTER from the menu.

Move the mouse cursor close to the center of the target circle and

press the left mouse button.

The cursor will jump to the nearest Circle center and attach a

rubber band line.

This command can be used to set the second point of a line being

drawn or to set the first point of a line to start.

REMARKS

See NEAREST POINT for a related command.

NEAREST POINT-NP

PURPOSE

To move the mouse cursor to the closest point from the mouse cursor.

HOW TO USE

Type NP or select NEAREST POINT from the menu.

Move the mouse cursor to the desired point and press the left

mouse button.

This command can be used to set the second point of a line being

drawn or to set the first point of a line to start.

Using this command from the keyboard or menu is not really necessary

because pressing the right mouse button duplicates the command

automatically.

REMARKS

See SNAP CENTER for related command.

FILLET-FL

PURPOSE

To extend or trim two non parallel lines to the intersection point

of the two selected lines.

HOW TO USE

Type FL or select FILLET from the menu.

Point to the first line and press the left mouse button.

Point to the second line and press the left mouse button.

The selected lines will be trimmed or extended to meet at a common

point.

REMARKS

See EXTEND and TRIM for related commands.

EXTEND-XT

PURPOSE

To extend one line to meet another.

Think of this as trimming a line off to meet another.

HOW TO USE

Type XT or select EXTEND from the menu.

Point to the line you want to extend and press the left mouse button.

Point to the line you want to extend to and press the left mouse

button.

The first line will be trimmed or extended to meet the second line.

REMARKS

Only one line at time can be extended or trimmed.

For multiple trims see TRIM.

See FILLET and TRIM for related commands..

TRIM-TR

PURPOSE

To extend or trim multiple lines simultaneously.

Think of this as trimming lines off to meet another line

HOW TO USE

Type TR or select TRIM from the menu.

Point to the line you want to trim to and press the left mouse button.

Point to the lines desired to trim and press the left mouse button.

After all lines have been selected, press Esc to terminate this

command.

See EXTEND and FILLET for related commands..

TEXT MENU COMMANDS: =======================================================

TEXT PLACE-TP

PURPOSE

To place text on your drawing.

Text can be placed anywhere on the drawing at any size and rotated

to any angle.

HOW TO USE

Type TP or select TEXT PLACE from the menu.

Place your mouse cursor at the desired location and press the left

mouse button.

Type the text you want, from the keyboard.

Press Esc to terminate this command.

REMARKS

See TEXT ANGLE, TEXT ERASE and TEXT SIZE for related commands.

TEXT SIZE-TZ

PURPOSE

To change text size.

HOW TO USE

Type TZ or select TEXT SIZE from the menu.

Type the desired size.

REMARKS

The Text Size is a function of the size of your drawing.

For Architectural size drawings normally size 9 (default), is

standard.

Smaller scale drawings may require a size of .25 to look proportional.

The Specified size of your text is also used for dimensioning.

See TEXT ANGLE, TEXT ERASE and TEXT PLACE for related commands.

TEXT WIDTH-TW

PURPOSE

To change the text width factor.

HOW TO USE

Type TW or select TEXT WIDTH from the menu.

Type the desired factor.

REMARKS

Example settings;

TW = .5, will give you letters half width.

TW = 2,  Will give you letters 2 times as wide.

See TEXT LINE for a related command.

TEXT LINE SPACING-TL

PURPOSE

To change the text line spacing factor.

HOW TO USE

Type TL or select TEXT LINE from the menu.

Type the desired factor.

REMARKS

Example settings;

TL = .5, will give you half line spacings.

TL = 2,  Will give you double line spacings.

See TEXT WIDTH for a related command.

TEXT ANGLE-TA

PURPOSE

To rotate text at a specified angle.

Any angle may be specified, positive or negative.

HOW TO USE

Type TA or select TEXT ANGLE from the menu.

Type the desired angle of rotation.

REMARKS

Text angles can be positive (rotated counter-clockwise) or negative

(rotated clockwise).

An angle of 90 degrees will rotate text vertically.

An angle of 180 degrees will rotate text up-side down.

An angle of -45 degrees will rotate text clockwise.

The default angle is 0 degrees.  To rotate text, select this first

before placing your text.

See  TEXT PLACE, TEXT ERASE and TEXT SIZE for related commands.

TEXT ERASE-TE

PURPOSE

To erase unwanted text from your drawing.

HOW TO USE

Type TE or select TEXT ERASE from the menu.

Draw a window around the text to be deleted.

When you lay down the second point of your window, the text will be

erased from the drawing.

REMARKS

TEXT ERASE removes text.  Other objects in the window will not be

affected.  Make sure your window completely encloses the entire

character(s), otherwise, parts of your text will be erased.

See TEXT ANGLE, TEXT PLACE, TEXT SIZE and ERASE WINDOW for related

commands.

READ TEXT-RT

PURPOSE

To place text in your drawing by reading standard ASCII files.

HOW TO USE

Type RT or select READ TEXT from the menu.

Type the name of the text file with extension, if applicable.

Point to the location of placement and press the left mouse button.

The program will read all the lines in the file and place them at the

specified location.  The text will be placed with the selected size

and angle.

REMARKS

Sometimes you may have standard notes that you use frequently.

Placing them in an ASCII file eliminates the need to retype them

each time.

ASCII files can be created with many Word Processors or the DOS

Edlin program, refer to your DOS manual for instructions.

See TEXT ANGLE and TEST SIZE for additional text options.

TEXT DATE-TD

PURPOSE

To place the current date, maintained by your computer, on your

drawing.

HOW TO USE

Type TD or select TEXT DATE from the menu.

Point to the desired placement location and press your left

mouse button.

REMARKS

The date is written as follows,

DATE 06-22-91

The date will be placed with the selected text size and angle.

See TEXT PLACE, TEXT ERASE, TEXT TIME and TEXT SIZE for additional

text options.

TEXT TIME-TT

PURPOSE

To place the current time, maintained by your computer, on your

drawing.

HOW TO USE

Type TT or select TEXT TIME from the menu.

Point to the desired placement location and press your left mouse

button.

REMARKS

The time is written as follows,

TIME 10:54

Most DOS versions maintain time in military time and that is the way

it will be placed on your drawing.

The time will be placed with the selected text size and angle.

See TEXT PLACE, TEXT ERASE, TEXT DATE and TEXT SIZE for additional

text options.

DIMENSION MENU COMMANDS: ===================================================

DIMENSION X-DX

PURPOSE

To place Dimensions on your drawing.  This command calculates the

horizontal distance between any two specified points and then places

the dimension with the dimension line, reference line, arrowheads,

and dimension calculation at the specified location in your drawing.

All dimensions will be placed as a horizontal dimension regardless

of the point locations.

HOW TO USE

Type DX or select DIMENSION-X from the menu.

Select the first reference point with the right mouse button.

Select the second reference point with the right mouse button.

Select the desired location for the dimension line and press the

left or right mouse button.

REMARKS

See DIMENSION-Y, DIMENSION ALIGNED, ANGLE DIMENSION,

DIMENSION TEXT and TEXT SIZE for related commands.

DIMENSION Y-DY

PURPOSE

To place dimensions on your drawing.  This calculates the vertical

distance between any two specified points and places the dimension

with dimension line, reference line, arrowheads, and dimension

calculation at the specified location in your drawing.

All dimensions will be placed as a vertical dimension regardless of

the point locations.

HOW TO USE

Type DY or select DIMENSION-Y from the menu.

Select the first reference point with the right mouse button.

Select the second reference point with the right mouse button.

Select the desired location for the dimension line and press the

left or right mouse button.

REMARKS

See DIMENSION-X, DIMENSION ALIGNED, ANGLE DIMENSION,

DIMENSION TEXT and TEXT SIZE for related commands.

DIMENSION ALIGNED-DA

PURPOSE

To place Dimensions on your drawing.  This command calculates the

distance between any two specified points and then places the

dimension complete with dimension line, reference line, arrowheads,

and dimension calculation at the specified location in your drawing.

All dimensions will be placed and aligned with the two points

selected.

You might think of DA as an angled dimension.

HOW TO USE

Type DA or select DIMENSION-ALIGNED from the menu.

Select the first reference point, with the right mouse button.

Select the second reference point with the right mouse button.

Select the desired location for the dimension line and press the

left or right mouse button.

REMARKS

See DIMENSION-Y, DIMENSION X, ANGLE DIMENSION, DIMENSION TEXT and

TEXT SIZE for related commands.

ANGLE DIMENSION-AD

PURPOSE

To dimension angles in degrees.

HOW TO USE

Type AD or select ANGLE DIMENSION from the menu.

Refer to MEASURE ANGLE in this section, after locating the three

points that define the angle, point to the location where you want

the Dimension arc to be placed, press the left or right mouse button.

Now point to the location you want the text to be placed, press the

left mouse button.

REMARKS

Angle dimensions will be drawn complete with the arc, arrowheads and

degrees.

Angle text is always placed horizontal.

See DIMENSION-Y, DIMENSION ALIGNED, DIMENSION-X, DIMENSION TEXT,

TEXT SIZE, DIMENSION-X and MEASURE ANGLE for related commands.

ARROW POINTER

PURPOSE

To place an arrow head pointer at the starting point of a line.

HOW TO USE

Type AP or select ARROW POINTER from the menu.

Draw a line.

The arrow head will be placed at the first point laid down.

REMARKS

This function can be used to point to an object in your drawing.

Usually TEXT PLACE is used first to define the label, then draw

the pointer.

DIMENSION TEXT-DT

PURPOSE

To override the dimension text that would normally be placed in a

dimension line by allowing you to type from the keyboard whatever

you want to print in the dimension space.

HOW TO USE

Type DT or select DIMENSION TEXT from the menu.

This command is a flip switch. The status box lets you know what

mode is selected.

REMARKS

If the DIMENSION TEXT is labeled as being ON in the status box,

the dimension text will placed normally.  If it is labeled OFF,

you can type the Dimension Text.

See DIMENSION-X, DIMENSION-Y, DIMENSION ALIGNED, ANGLE DIMENSION,

DIMENSION TEXT and TEXT SIZE for related commands.

TEXT SIZE-TZ

PURPOSE

To change the size of text.

HOW TO USE

Type TZ or select TEXT SIZE from the menu.

REMARKS

See the TEXT section for further details.

PLACE TEXT=PT

PURPOSE

To place Dimension Text at the left end, center or right

end of the dimension line.

HOW TO USE

Type PT or select PLACE TEXT from the menu.

Type the desired location, L=Left, C=Center, R=Right.

REMARKS

DIMENSION EXTEND-DE

PURPOSE

To set the Dimension Extend, (also referred to as Witness

Line), offset measurements and stretch options.

HOW TO USE

Type DW or select Dimension Extend from the menu.

The command box then displays the current settings

for A-Away, T-Toward and S-Stretch.

The Away setting is the Extension line distance

from the actual dimension line, the part of the Extend

line that extends away from the points being dimensioned.

The Toward setting is the Extension line distance

from the actual dimension line, the part of the Extend

line that extends toward the points being dimensioned.

The Stretch setting, if on, stretches the Extension line

the specified distance, (Toward), away from the points

being dimensioned. If Stretch is off, the specified Toward

distance is measured from the dimension line.

REMARKS

This command can be issued after the dimension command, which

allows you to see the result while the dimension is being

rubber banded around.

MEASURE DISTANCE-MD

PURPOSE

To check the distance between any two points.

HOW TO USE

Type MD or select MEASURE DISTANCE from the menu.

Select the two points with the right mouse button.

REMARKS

See MEASURE ANGLE for related command

MEASURE X-MX

PURPOSE

To measure the X distance between any two points,

similar to Dimension-X (DX).

HOW TO USE

Type MX or select MEASURE X from the menu.

Select the two points with the right mouse button.

REMARKS

The displayed distance in the lower right command

box, is the horizontal distance between the two

selected points.

See MEASURE DISTANCE, MEASURE Y and MEASURE ANGLE for related

commands.

MEASURE Y-MY

PURPOSE

To measure the Y distance between any two points,

similar to Dimension-Y (DY).

HOW TO USE

Type MY or select MEASURE Y from the menu.

Select the two points with the right mouse button.

REMARKS

The displayed distance in the lower right command

box, is the vertical distance between the two

selected points.

See MEASURE DISTANCE, MEASURE X and MEASURE ANGLE for related

commands.

MEASURE ANGLE-MA

PURPOSE

To check the angle between any two lines.

HOW TO USE

Type MA or select MEASURE ANGLE from the menu.

This command requires three points, the center and two other points.

Select the center point of the desired angle with the right mouse

button.

Select one of the other points of the desired angle with the right

mouse button, and the third point with the right mouse button.

REMARKS

Angles are always measured in a counter clockwise direction.

See MEASURE ANGLE for related command

WINDOW MENU COMMANDS: =====================================================

ZOOM WINDOW-ZW

PURPOSE

o view an area of a drawing at maximum size.

HOW TO USE

Type ZW or select ZOOM WINDOW from the menu.

Draw a window around the desired area.

REMARKS

See ZOOM ALL, ZOOM BACK, ZOOM UP, ZOOM LAST and PAN for

related commands.

WINDOW ERASE-WE

PURPOSE

To erase all objects completely contained in the window except for

text.

HOW TO USE

Type WE or select WINDOW ERASE from the menu.

Draw a window around the objects you wish to erase.

When placing down the second point of your window, all the objects

whose both end points are contained in the window will be erased.

REMARKS

Circles where the center point is within the window will also be

erased.

Text is not erased with this command.

If you make a mistake on the placement of your window and erase

lines that you wanted to keep, use UN-ERASE to bring then back.

See TEXT ERASE, ERASE WINDOW, UNERASE, OBJECT ERASE, ERASE LAST

and ERASE DRAWING for related commands.

WINDOW COPY-WC

PURPOSE

To copy sections of your drawings to another location a specified

number of times.

HOW TO USE

Type WC or select WINDOW COPY from the menu.

Draw a window around all the objects you want to copy or duplicate.

Select a copy from reference point with the left or right mouse

button then select the copy to point with either of the same buttons.

Type the number of copies you want to make.

REMARKS

Lines whose both end points do not lie in the window box will not be

copied, so make sure to enclose both ends of all lines that you want

to copy.

See WINDOW MIRROR and RADIAL COPY for related commands.

WINDOW MIRROR-WR

PURPOSE

To make a mirrored copy of a specified area of your drawing one at

a time.

HOW TO USE

Type WR or select WINDOW MIRROR from the menu.

Draw a window around the area to be mirrored.

Select 'Mirror From' reference point.

Select 'Mirrored To' reference point.

The axis point of the mirror will be calculated as the half-way

point of the two reference points.

See WINDOW COPY, WINDOW RESIZE, WINDOW MOVE and WINDOW ROTATE

for related commands.

WINDOW RESIZE-WZ

PURPOSE

To RESIZE (rescale) a specified area of your drawing.

HOW TO USE

Type WZ or select WINDOW RESIZE from the menu.

Draw a window around the objects you wish to resize.

Type the X-direction scale factor and press enter.

Type the Y-direction scale factor and press enter.

If you want the Y-direction factor to be the same as the X-direction

factor, press enter for the Y scale factor.

The specified area will be redrawn at the new scale factors.

REMARKS

Circles will be resized by the X-direction factor.

For an interesting effect, try using this command on text using

different X and Y factors.

See WINDOW MIRROR, WINDOW COPY, WINDOW MOVE and WINDOW ROTATE for

related command.

WINDOW MOVE-WM

PURPOSE

To move (relocate) multiple objects.

HOW TO USE

Type WM or select WINDOW MOVE from the menu.

Draw a window around the objects to be moved.

Select 'Move From' reference point. With the left or right mouse

button, select 'Move to' reference point in the same manner.

REMARKS

All lines with both end points enclosed in the window will be moved

to the desired location.

See WINDOW MIRROR, WINDOW RESIZE, WINDOW COPY, OBJECT MOVE and

WINDOW ROTATE for related commands.

WINDOW ROTATE-WO

PURPOSE

To rotate any number of objects to a specified angle about a

specified point.

HOW TO USE

Type WO or select WINDOW ROTATE from the window.

Draw a window around the objects to rotate.

Select the rotate point using the left or right mouse button,

then type the angle of rotation.

REMARKS

Positive angles rotate counter-clockwise and Negative angles rotate

clockwise.

See WINDOW MIRROR, WINDOW RESIZE, WINDOW MOVE and WINDOW COPY for

related command.

WINDOW STRETCH-WS

PURPOSE

To stretch lines similar to WINDOW MOVE, except both end points of

a line do not have to be within the window.

HOW TO USE

Type WS or select WINDOW STRETCH from the menu.

Place the window around the desired points to stretch.

Locate the reference point (stretch from location).

Locate the destination point (stretch to point).

REMARKS

All line end points contained in the window will be moved to the

specified location.  If you enclose only one end point of a line

with the window, that point will be moved thus redefining or

stretching the line whose other end point lies outside the window.

See WINDOW MOVE for related command.

WINDOW EXTEND-WX

PURPOSE

To extend (similar to extrude) objects.

HOW TO USE

Type WX or select WINDOW EXTEND from the menu.

REMARKS

Works exactly like WINDOW COPY except line end points

are connected, simulates extrusions.

Try this command on a curve, for some interesting

results.

See WINDOW COPY for related command.

WINDOW CHANGE-WG

PURPOSE

To change the line color or line type of selected existing lines.

HOW TO USE

Type WG or select WINDOW CHANGE from the menu.

Draw your window around the desired lines.

Select 1 to change line color or 2 to change line type or, press

your left mouse button for 1 or, the right button for 2.

Select either the new color or type for the lines contained in the

window.

REMARKS

Circles and Text are not affected by this command.

Only lines whose both end points lie within the window are affected.

See OBJECT CHANGE for related command.

ZOOM MENU COMMANDS: =====================================================

ZOOM ALL-ZA

PURPOSE

To view the entire drawing at maximum size.

HOW TO USE

Type ZA or select ZOOM ALL from the menu.

REMARKS

See ZOOM WINDOW, ZOOM OUT, ZOOM IN, ZOOM LAST and PAN for related

commands.

ZOOM WINDOW-ZW

PURPOSE

To view a specific area at maximum size.

HOW TO USE

Type ZW or select ZOOM WINDOW from the menu.

Draw a window around the desired area.

REMARKS

See ZOOM ALL, ZOOM OUT, ZOOM IN, ZOOM LAST and PAN for

related commands.

ZOOM OUT-ZO

PURPOSE

To reduce, (make smaller) the viewed drawing by 20% each time this

command is selected.

HOW TO USE

Select the center point for the new Zoom.  Select this point by

placing the mouse cursor at the desired location and press the left

mouse button.

The selected location will now be in the center of your screen.

REMARKS

See ZOOM WINDOW, ZOOM ALL, ZOOM IN, ZOOM LAST and PAN for related

commands.

ZOOM IN-ZI

PURPOSE

To increase (make larger) the viewed drawing by 20% each time

this command is selected.

HOW TO USE

Type ZI or select ZOOM IN from the menu.

Select the center point for the new Zoom.  Select this point by

placing the mouse cursor at the desired location and press the left

mouse button.

The selected location will now be in the center of your screen.

REMARKS

See ZOOM WINDOW, ZOOM OUT, ZOOM ALL, ZOOM LAST and PAN for related

command.

PAN-PA

PURPOSE

To pan around the drawing to view different locations in your

drawing.

HOW TO USE

Type PA or select PAN from the menu.

Then select the center point of the pan with the left mouse button.

The selected location will now be in the center of your screen.

REMARKS

See ZOOM WINDOW, ZOOM OUT, ZOOM IN, ZOOM LAST and ZOOM ALL for

related commands.

ZOOM LAST-ZL

PURPOSE

To flip the views between the last specified view and the current

view.

HOW TO USE

Type ZL or select ZOOM LAST from the menu.

REMARKS

See ZOOM WINDOW, ZOOM OUT, ZOOM IN, ZOOM ALL and PAN for related

commands.

EDITS MENU COMMANDS: =====================================================

REDRAW-RD

PURPOSE

To redraw the current view.

HOW TO USE

Type RD or select REDRAW from the menu.

REMARKS

Sometimes the screen gets cluttered with line fragments from

erasing, moving windows, etc.  This option redraws the screen at the

current view settings thus erasing any left over fragments.

See REGEN for related command.

REGEN-RG

PURPOSE

To redraw the entire screen.

HOW TO USE

Type RG or select REGEN from the menu.

REMARKS

Sometimes the screen gets cluttered with line fragments from

erasing, moving windows, etc.  This option redraws the entire

screen, (including command areas), at the current view settings

thus erasing any left over fragments.

See REDRAW for related command.

OBJECT ERASE-OE

PURPOSE

To erase a single line, one at a time.

HOW TO USE

Type OE or select OBJECT ERASE from the menu.

Place the mouse cursor on or close to the line you wish to erase,

then press the left mouse button.

REMARKS

Sometimes you might accidently erase the wrong line, if this happens

type UE to un-erase it.  See UN-ERASE for details.

It may be necessary to zoom in on the line you want to erase so you

can safely place the mouse cursor on the intended line.

See UNERASE and WINDOW ERASE for related command.

OBJECT COPY-OC

PURPOSE

To copy a single line a specified distance from the original line.

The number of copies must be specified.

HOW TO USE

Type OC or select OBJECT COPY from the menu.

Select the line to copy by placing the mouse cursor on or close to

the intended copy line, then press the left mouse button.

You will then be asked for the desired offset from this line.

Type the desired offset both X and Y direction.

Type the number of copies.

REMARKS

If too many copies are made, you can use the ERASE LAST command to

erase as many of the lines as desired.

Another way of specifying the number of copies is to move your mouse

cursor to the desired stop location and press the left mouse button.

See WINDOW COPY for related commands.

OBJECT MOVE-OM

PURPOSE

To move a single object from one location to another.

HOW TO USE

Type OM or select OBJECT MOVE from the menu.

Select the object to move by placing the mouse cursor on or close to

the desired object and press the left mouse button.

Locate the reference point to move from with the left or right mouse

button and move to location by moving the mouse cursor to the

desired location.  Press the left or right mouse button.

REMARKS

The move to location may also be specified by typing in both X and Y

dimensions.

Entering 24,24 will move the object to the right 24 units and up

24 units.

See WINDOW MOVE for related command.

OBJECT CHANGE-OG

PURPOSE

To change either the line color or line type of one line at a time.

HOW TO USE

Type OG or select OBJECT CHANGE from the menu.

Point to the desired line and press your left mouse button.

Select either 1 to change line color or, 2 to change line type.

Select either the new color or type for the selected line.

REMARKS

Circles and Text are not affected by this command.

See WINDOW CHANGE for related command.

SEGMENT LINE-SL

PURPOSE

To divide an existing line into any number of equal parts.

HOW TO USE

Type SL or select SEGMENT LINE from the menu.

Point to the line you want to segment (divide into any number of

equal parts).

Type the number of segments desired.

REMARKS

Your existing line will be broken into the number of segments

specified.

ERASE LAST-ER

PURPOSE

To erase the last line drawn.

HOW TO USE

Type ER or select ERASE LAST from the menu.

The last line drawn will be erased each time you select this option.

You can use it as many times in succession as desired to remove

several lines.

REMARKS

Has no affect on Text or Circles.

If you erase too many lines, select UN-ERASE to return them to your

drawing.

See WINDOW ERASE, OBJECT ERASE,  ERASE  DRAWING and UNERASE for

related commands.

TEXT ERASE-TE

PURPOSE

To erase unwanted text.

HOW TO USE

Type TE or select TEXT ERASE from the menu.

Draw a window around the text you want to erase.

When you lay down the second point of your window, the text will be

erased from the drawing.

REMARKS

Text erase only removes text.  Other objects in the window will not

be affected.  Make sure your window completely encloses the entire

charatcter(s), otherwise just parts of your text will be erased.

REMARKS

UN-ERASE has no effect on text, so once you erase it there is no way

to re-insert it unless you use TEXT PLACE to recreate it.

See WINDOW ERASE, ERASE WINDOW and ERASE DRAWING for related commands.

WINDOW ERASE-WE

PURPOSE

To erase all objects completely contained in the window except for

text.

HOW TO USE

Type WE or select WINDOW ERASE from the menu.

Draw a window around the objects you wish to erase.

When you lay down the second point of your window, all the objects

whose both end points are contained in the window will be erased.

REMARKS

Circles whose center point is within the window will also be erased.

Text is not erased with this command.

If you make a mistake on the placement of your window and erase

lines that you wanted to retain, you can use UN-ERASE to retrieve

them.

See TEXT ERASE, UNERASE, OBJECT ERASE, ERASE LAST, ERASE WINDOW and

ERASE DRAWING for related commands.

ERASE DRAWING-ED

PURPOSE

To erase every item from a drawing.

HOW TO USE

Type ED or select ERASE DRAWING from the menu.

As a safe guard, you are prompted, ARE YOU SURE!!!.  If so, type Y

for yes or any other key to abort this command.

REMARKS

CAUTION!  This is a very powerful command because your entire drawing

will be erased and there will be no way to retrieve it unless you

save it to disk first.

See OBJECT ERASE, ERASE LAST, TEXT ERASE, WINDOW ERASE, ERASE WINDOW

and UN-ERASE for related commands.

UNERASE-UE

PURPOSE

To un-erase or retrieve the last erased line.

HOW TO USE

Type UE or select UN-ERASE from the menu.

The last erased line will be returned.

REMARKS

This command is used mostly to re-insert lines that were

inadvertently erased with another command.

This command has no effect on Text or Circles.

See OBJECT ERASE, ERASE LAST and WINDOW ERASE for related commands.

DRAWING RESIZE-DZ

PURPOSE

To rescale an entire drawing to a specified number of units.

HOW TO USE

Type DZ or select DRAWING RESIZE from the menu.

You are prompted to select two points that are a known distance

apart. Select each point by placing the mouse cursor on or close to

the point and press the right mouse button.

Type the distance from the keyboard.

The entire drawing will be rescaled to the appropriate factor.

REMARKS

This option is used mostly with the LOAD PLOT command.

Plots loaded as drawings are never to scale so this command can be

used to reset the desired scale.

This command can also be used to convert a drawing from one unit of

measure to another.

See WINDOW RESIZE and DRAWING UNITS for related command.

DRAWING UNITS-DU

PURPOSE

To specify the data base unit of measure

HOW TO USE

Type DU or select DRAWING UNITS from the menu.

Type the unit of measure label and press enter.

Anytime you enter coordinates, measure lengths, place dimensions etc.

on your drawing, it will be done in the units label which you

specify.

REMARKS

The default unit is "FI" for Feet and Inches.

If you want to work in Millimeters, type mm and press enter.

Any label identification can be used.

A unit could be an Inch, Yard, Meter, Mile, Kilometer, etc.

If you want to convert an existing drawing to another unit of

measure, select DRAWING UNITS first, to set the units, then perform

a DRAWING RESIZE.  Remember to reset the TEXT SIZE, THICKNESS, etc.

ERASE WINDOW

PURPOSE

To erase all objects in the window.

HOW TO USE

Type EW or select ERASE WINDOW from the menu.

Draw a window around the objects you wish to erase.

When you lay down the second point of your window, all the objects

whose both end points are contained in the window will be erased.

REMARKS

Circles whose center point is within the window will also be erased.

Text will also be erased with this command.

See TEXT ERASE and WINDOW ERASE for related commands

MENU FLIP=MF

PURPOSE

To turn the right Side Bar Menu on or off.

HOW TO USE

Type MF or select MENU FLIP from the menu.

REMARKS

Sometimes it's convenient to have a larger drawing area with which

to work.

Turning the menu off provides you with additional screen length.

New users may want to keep the menu on while learning the system

because all of the two letter commands are highlighted in the

command name.

DRAW MENU COMMANDS: =====================================================

RECTANGLE-RE

PURPOSE

To draw rectangles or squares.

HOW TO USE

Type RE or select RECTANGLE from the menu.

Select the starting corner of the rectangle by moving the mouse

cursor to the desired location and press the left mouse button to

lay down the starting point or the right mouse button to snap to an

existing point.  As you move your mouse, you will see a rubber

rectangle sometimes referred to as a window.  Move the cursor to the

desired location and press the left or right mouse button to set the

opposite corner of the rectangle

REMARKS

After the first corner is laid down, dimensions can be typed to

locate the opposite corner a known distance from the starting point.

For example:  suppose you want to draw a rectangle which measures

24 units by 24 units.  After setting the first corner, type 24,24

and press enter.  The rectangle will be drawn at the specified size.

See  CIRCLE, ARC and POLYGON for related commands.

CIRCLE-CI

PURPOSE

To draw Circles.

HOW TO USE

Type CI or select CIRCLE from the menu.

Move your mouse to the desired center point and press the left

button to set the center point or press the right button to snap to

the nearest point.

Move the mouse cursor to the desired radius position and press the

left or right mouse button.

Radius can also be specified by typing the number of units.

For example: for a radius of 24 units from the center point,

type 24, enter.

REMARKS

Circles are always drawn in color 7.

A maximum of 300 circles may be drawn.

See RECTANGLE, ARC and POLYGON for related commands.

POLYGON-PG

To draw standard Polygons.

Type PO or select POLYGON from the menu.

Move the mouse cursor to the desired location and press the left or

right mouse button to select the center point of the polygon.

Type the number of sides desired.

Move the mouse cursor to the desired radius point and press either

the left or right mouse button to set the radius point.

REMARKS

A radius can be specified by typing the dimension.

The minimum number of sides allowed is 3.

The maximum number of sides allowed is limited by the amount of RAM

Memory in your computer.

Specifying a high number of sides will make the Polygon resemble a

Circle.  Using WINDOW RESIZE with different X and Y values will

change the Circle to resemble an Ellipse.

See CIRCLE, ARC and RECTANGLE for related command.

ARC-AR

PURPOSE

To draw 4 point arcs.

HOW TO USE

Type AR or select ARC from the menu.

Locate the center point of the Arc.

Locate the second point which is the starting angle of the Arc.

Locate the third point which is the ending angle of the Arc.

Locate the radius point.

REMARKS

Arcs are always drawn counter-clockwise so the order of selection of

the second and third points will give you the desired Arc.

See RECTANGLE, CIRCLE and POLYGON for related commands.

CURVE-CV

PURPOSE

To draw a 4 point curve using the cubic parametric

calculation method.

HOW TO USE

Type CV or select CURVE from the menu.

Select the starting point for the curve using the left

or right mouse button.

Select the remaining 3 points in the same manner.

The defined curve will be drawn after the fourth

point is selected.

REMARKS

The defined curve will start at point1 and end at point4.

The location of point2 and point3 define the shape of

the curve.

The curve will not pass thru point2 and point3, but will be drawn

toward those points.

DRAW DOUBLE-DD

PURPOSE

To draw two lines at a time a specified distance apart.

The distance apart is set with the THICKNESS command.

HOW TO USE

Type DD or select DOUBLE DRAW from the menu.

Start drawing lines as usual.

Press Esc or type DD again to terminate this command

REMARKS

See THICKNESS for related command.

THICKNESS-TH

PURPOSE

To specify the Double Draw line spacings.

HOW TO USE

Type TH or select THICKNESS from the menu.

Type the #1 thickness offset, press enter, then type the #2

thickness offset, press enter.

REMARKS

Example settings;

#1=0, #2=3.5.  If you now Draw Double and move clockwise, the second

line will be drawn 3.5 units inside the first line.

#1=2, #2=2.  Double lines will be drawn 2 units each side of your

reference line.

Does not work with Circles or Text.

See DOUBLE DRAW for related command.

LINE TYPE-LT

PURPOSE

Two line types are available for drawing lines.

0 = Solid line.

1 = Dashed line.

This option lets you select either one.

HOW TO USE

Type LT or select LINE TYPE from the menu.

Type either a 0 or 1 to select the desired line type.

REMARKS

Line Types do not affect Circles or Text.

LINE COLOR-LC

PURPOSE

To select a color to draw lines.

HOW TO USE

Type LC or select LINE COLOR from the menu.

A "pop down" menu will appear showing all the available line colors.

Select the desired color by moving the highlighted bar up or down by

moving the mouse and stopping on the desired color.

Press the left mouse button to initiate the selection.

REMARKS

Line colors can not be selected for Circles, Text or Dimension lines.

These objects will always be drawn in color #7.

RADIAL COPY-RC

PURPOSE

To copy objects in a circular direction about a center point.

HOW TO USE

Type RC or select RADIAL COPY from the menu.

Draw a window around the objects to copy.

Locate the center radius point (rotate about point) with the left or

right mouse button.

Type number of copies.

Type number of degrees to copy the objects.  The default is 360

degrees by pressing enter.

REMARKS

Both negative and positive degrees can be specified.

Positive angles copy objects counter-clockwise.

Negative angles copy objects clockwise.

See WINDOW COPY for related command.

BLOCK CREATE-BC

PURPOSE

To create symbols or blocks which can be saved to disk or placed

elsewhere in the drawing.

HOW TO USE

Type BC or select BLOCK CREATE from the menu.

Draw a window around all the objects which are to be part of

the block.

Type an identification name for your block.

REMARKS

Identification name is limited to 8 characters, A-Z, 0-9, - or _

Do not use any special characters, like comma, period, space, etc.

in the block name.

A maximum of 20 Blocks can be in memory at one time.

See  BLOCK PLACE, BLOCK ANGLE, BLOCK SAVE , BLOCK LOAD and

BLOCK UN-LOAD for related commands.

BLOCK PLACE-BP

PURPOSE

To place previously defined blocks in your drawing.

HOW TO USE

Type BP or select BLOCK PLACE from the menu.

A "pop down" window will appear listing all the blocks in memory.

Select the block from the window.

Move the mouse cursor to the desired location and press the left or

right mouse button.

REMARKS

Only blocks in memory can be accessed by this command.  If you have

a block on disk that you wish to use, load it into memory with

BLOCK LOAD, then select BLOCK PLACE.

See BLOCK CREATE, BLOCK ANGLE, BLOCK SAVE , BLOCK LOAD and

BLOCK UN-LOAD for related commands.

BLOCK ANGLE-BA

PURPOSE

To rotate blocks in memory to any angle.

To place a block at an angle different from the default, use this

option.

HOW TO USE

Type BA or select BLOCK ANGLE from the menu.

Type the angle in degrees, + to rotate counter-clockwise or

negative(-) to rotate clockwise.  Any block placed will be at the

specified angle of rotation.

REMARKS

The default Block angle is 0 degrees.

See BLOCK PLACE, BLOCK CREATE, BLOCK SAVE , BLOCK LOAD and

BLOCK UN-LOAD for related commands.

BLOCK UNLOAD-BU

PURPOSE

All blocks in memory take up valuable memory.  Any block in memory

can be un-loaded to gain additional drawing memory.

HOW TO USE

Type BU or select BLOCK UN-LOAD from the menu.

A "pop down" menu will appear.  Select the name of the block to

clear from memory and press the left mouse button.

REMARKS

Once a block is removed from memory, there is no way to recall it,

therefore, you might want to save it to disk with BLOCK SAVE before

deleting it.

Blocks are also removed from memory when plotting is done,

so if you need the blocks, save them first before plotting.

See BLOCK PLACE, BLOCK ANGLE, BLOCK SAVE , BLOCK LOAD and

BLOCK CREATE for related commands.

LAYER SELECT-LS

PURPOSE

To select the current layer to draw on.

HOW TO USE

Type LS or select LAYER SELECT from the menu.

Then type the current layer number you want to draw on.

REMARKS

Layer range is 1 thru 99.

You cannot select a layer that is hidden as your current layer.

See LAYER HIDE for related command.

LAYER HIDE-LH

PURPOSE

To hide or un-hide specified layers.

HOW TO USE

Type "LH" or select LAYER HIDE from the menu.

Then type the layer number to hide.

REMARKS

Layer range is 1 to 99.

Selecting layer 100 will un-hide all hidden layers.

You cannot hide the layer you have selected as the current layer.

Once a layer is hidden, it can be unhidden (viewed) by selecting

that layer number again.

See LAYER SELECT for related command.

DECIMAL PLACES-DP

PURPOSE

To set the number of decimal places to display when measuring angles

or working in units other than Feet and Inches (FI).

HOW TO USE

Type DP or select DECIMAL PLACES from the menu.

Then type the desired number of decimal places to display.

REMARKS

The default is 3 decimal places.

The allowed range is 0 to 6 decimal places.

FILLET RADIUS-FR

PURPOSE

To specify a radius for Filleted corners.  This command

works only on lines where one line is vertical and one

line is horizontal.

HOW TO USE

Type FR or select FILLET RADIUS from the menu.

Type the radius, press enter.

REMARKS

See CHAMFER for a related command

CHAMFER-CH

PURPOSE

To specify a Chamfer distance for corners.  This command

works only on lines where one line is vertical and one

line is horizontal.

HOW TO USE

Type CH or select CHAMFER from the menu.

Type the chamfer distance, press enter.

REMARKS

See FILLET RADIUS for a related command

APPENDIX:

Configuration file (CONFIG.FIL)

This file contains default configuration settings on the way your

PD CADD works.

This is the default settings for PD CADD as it is shipped.

Line 1.. 5      Line Color

Line 2.. 9      Text Size

Line 3.. 1      Text Width Factor

Line 4.. 1      Text Line Spacing

Line 5.. FI     Drawing unit of measure

Line 6.. 3      Number of decimal places to display

Line 7.. 26     Output device number (1-27)

Line 8.. 7.8    Plotting Paper Width (inches)

Line 9.. 10     Plotting Paper Length (inches)

Line 10. 6      Number of plotter pens to use (1 thru 15 inclusive)

Line 11. 12     Dimension Extend, away Dim.. line

Line 12. 3      Dimension Extend, toward Dim.. line

Line 13.-1      Dimension Extend, stretch, (-1 = YES), (0 = NO)

Line 14. NO     Rotate plot 90 degrees

Line 15. \PDC\DWG Work directory for plotting or printing

Line 16. lpt1: Device output port

If you are outputting to a COM port, use MODE.COM, (supplied with your DOS),

to set the required communication parameters before running PD CADD.

A typical setting for HP Plotters is:

MODE COM1: 96,E,7,1,B

The configuration file can be changed using any text editor such

as EDLIN, supplied with your DOS, or using the Config Save, (CS)

command.

Line colors can be selected from the top right menu bar as well

as from the Draw Menu (Line Color, LC command).

Pressing and holding the Alt key, displays a full screen

cursor that is helpful in lining up objects from one side

of the screen to the other.

Plotted drawings can be output to any of the following or compatible

devices;

EPSON (FX-MX-LX) 9-PIN NARROW LOW RES

EPSON (FX-MX-LX) 9-PIN NARROW HIGH RES

EPSON (FX-MX-LX) 9-PIN WIDE LOW RES

EPSON (FX-MX-LX) 9-PIN WIDE HIGH RES

EPSON (LQ) 24-PIN NARROW LOW RES

EPSON (LQ) 24-PIN NARROW HIGH RES

EPSON (LQ) 24-PIN WIDE LOW RES

EPSON (LQ) 24-PIN WIDE HIGH RES

NEC PINWRITER 24-PIN NARROW LOW RES

NEC PINWRITER 24-PIN NARROW HIGH RES

NEC PINWRITER 24-PIN WIDE  RES

NEC PINWRITER 24-PIN WIDE HIGH RES

OKIDATA MICROLINE 9-PIN NARROW LOW RES

OKIDATA MICROLINE 9-PIN NARROW HIGH RES

HP LASERJET OR DESKJET 75 DPI

HP LASERJET OR DESKJET 100 DPI

HP LASERJET OR DESKJET 150 DPI

HP LASERJET OR DESKJET 300 DPI

HP LASERJET III (HPGL/2)

HP PAINTJET 2 COLOR 75 DPI

HP PAINTJET 4 COLOR 100 DPI

HP PAINTJET 8 COLOR 150 DPI

HP PAINTJET 2 COLOR 300 DPI

HP PAINTJET 16 COLOR 300 DPI

POSTSCRIPT 300 DPI

HP PLOTTER HP-GL

HOUSTON INST. PLOTTER DM/PL