Cport

C Language Serial Communications Library

Version 1.10

Copyright (c) 1991  Bri Productions

1

Table of Contents

Introduction

Acknowledgments..........................5

General Description......................5

Disclaimer...............................5

Registration.............................5

Contacting the Author....................6

A typedef................................6

"Remote Device"..........................6

Control Functions

Description..............................7

ComOpen..................................8

ComOpenS................................11

ComReopen...............................14

ComClose................................14

ComParam................................15

ComBaud.................................16

ComMode.................................17

ComHandshake............................19

ComTx...................................20

Input Functions

Description.............................22

ComGetc.................................23

ComGets.................................24

ComIn...................................25

ComFlushRx..............................26

2

ComLenRx................................27

ComPeek.................................28

Output Functions

Description.............................30

ComPutc.................................31

ComPuts.................................32

ComOut..................................33

ComFlushTx..............................34

ComLenTx................................35

Status Functions

Description.............................36

ComError................................37

ComStatus...............................38

ComRts..................................39

ComDtr..................................40

ComOut1.................................41

Data Integrity

Description.............................43

ComChecksum.............................44

ComCrc16................................45

Miscellaneous Functions

Description.............................47

ComSetBreak.............................48

ComClrBreak.............................48

ComPutScrtch............................49

ComGetScrtch............................50

3

Appendix A - Things to Look Out For.............52

Appendix B - Handshaking Schemes................53

Appendix C - The UART Chip......................55

Appendix D - The RS-232 Standard ?!!............56

Appendix E - Registration Form..................57

4

Acknowledgments.

I would like to thank Earl Jensen and Steve Hill for technical

review, Diana Hill for proofreading the documentation, Chip Curry

and Luxtron Corp for beta testing, John Kerwin for technical advice,

and Ton Kneist and Nol Gaskell for facilitating development.

General Description.

Cport is a C language interrupt-driven serial communication library

for IBM and compatible computers. Cport is compatible with most if

not all DOS based C compilers.

Some of Cport's features are:

Baud rates from 50 to 115200 baud.

Both transmitter and receiver are interrupt-driven.

Queues as large as 65534 bytes each.

Written in assembly language for optimum speed and efficiency.

DISCLAIMER.

Cport is provided AS IS. Bri Productions specifically disclaims

any and all warranties, expressed or implied, including fitness

for a particular purpose. Use this product at your own risk.

Registration.

Cport is a user supported software product. Distribution of

this product, unaltered and without charge, is encouraged. If

you find this product useful, you are encouraged to register

your copy. This allows Bri Productions to continue to provide

support low cost, high quality shareware.

The registration fee is $25.00 US dollars and includes libraries

for small, medium, compact and large memory models and complete

source code. A registration form is provided in Appendix E.

5

Contacting the author.

Bri Productions may be contacted by any of the following means:

CompuServe      -       76635,2246

U.S. mail       -       Bri Productions

P.O.Box 7121

Fremont, CA 94537-7121

CompuServe is a trademark of CompuServe Inc.

A typedef.

You will see the data type 'byte' throughout the Cport library.

This is a typedef defined in cport.h. Byte is an unsigned

character used for byte size values.

"Remote device"

In this documentation, the expression "remote device" is used

to describe the device that the computer is communicating with.

The actual device may be a modem, serial printer or other

communications device.

6

Control Functions

Control functions manage the parameters and the modes in

which the serial port will operate. There are two functions

which are mandatory: ComOpen or ComOpenS, and ComClose.

ComOpen and/or ComOpenS open a serial port and sets the

parameters and modes. ComClose closes the opened serial port.

Failing to call ComClose prior to a program's termination will

cause the operating system crash.

ComBaud and ComMode modify one or more of the parameters/modes

of the serial port. ComParam reports on the current parameters

and handshaking under which the currently opened serial port

are operating.

ComHandshake determines if and what type of handshaking

scheme will be enabled and sets the receiver threshold. ComTx

turns the transmitter on and off, allowing you to implement

your own handshaking scheme. See Appendix B for a description

of handshaking.

7

----------------------------------------------------------------

ComOpen

----------------------------------------------------------------

Function

Opens or reopens a serial port.

Syntax

#include "cport.h"

int ComOpen(unsigned com, int baud, byte mode,

unsigned rxQ, unsigned txQ);

Parameters

com   - Constant that defines the serial port to be

opened. Possibilities are:

COM1  (0xC3F8)

COM2  (0xB2F8)

COM3  (0xC3E8)

COM4  (0xB2E8)

baud  - Constant defining the baud rate to which the

serial port will be initialized. Possibilities

are:

B115200    (1)

B57600     (2)

B38400     (3)

B19200     (6)

B9600     (12)

B7200     (16)

B4800     (24)

B3600     (32)

B2400     (48)

B2000     (58)

B1800     (64)

B1200     (96)

B600     (192)

B300     (384)

B150     (768)

B110    (1047)

B75     (1536)

B50     (2304)

mode  - Bit ORed constant defining the word

length, number of stop bits and the

parity that the serial port will use.

Possibilities are:

8

W8  (0x03)  -  8 bit words

W7  (0x02)  -  7 bit words

W6  (0x01)  -  6 bit words

W5  (0x00)  -  5 bit words

S1  (0x00)  -  1 stop bit

S2  (0x04)  -  2 stop bits

NONE   (0x00) -  No parity

ODD    (0x08) -  Odd parity

EVEN   (0x18) -  Even parity

MARK   (0x28) -  Mark parity

SPACE  (0x38) -  Space parity

rxQ  - Size, in bytes, of the receive queue.

txQ   - Size in bytes, of the transmit queue.

* Limited to 32K in small and medium

memory models.

Remarks

ComOpen opens the serial port 'com' for communications.

If a serial port has been previously opened, it will

be close prior to the new serial port being opened.

The new serial port can be a different port or the

same port with new parameters. This feature allows

the programmer to reopen serial ports without first

closing the previous one.

The 4 most significant bits in 'com' represent the

interrupt vector, while the lower 12 bits represent

the port address. Constants are provided in cport.h

which will be satisfactory in most cases.

The parameter 'baud' is the baud rate divisor which

determines the baud rate. Constants are provided in

cport.h for the most common baud rates. The 'baud'

rate divisor is defined as f / (16 * baud rate) where

f is 1.8432e+6.

Achieving baud rates above 19200 is largely dependent

on many variables such as machine speed, cable length,

handshaking activity, degree bidirectional transfers,

the type UART (see Appendix C) used and activity of

other hardware interrupts.

The parameter 'mode' determines the word length,

number of stop bits and parity of the exchanged data.

Bits 0 & 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables

parity and bits 4 & 5 determine the type of parity

9

check. Constants which can be ORed together are

provided in cport.h.

'RxQ' and 'txQ' set the size of the receive and transmit

queues respectively. 'RxQ' should be at least 4 times

larger than the largest string or block of data the

programmer intends to fetch from the queue at once. '

TxQ' can be set as small as 1, but is not recommended.

In the small and medium memory models, where all data

is limited to 64K, the size queues are limited. In

the compact and large memory models, each queue can

be as large as 64K -1 (65534 bytes).

Return value

If successful, ComOpen returns 0. In the event of an

error, it returns one of the following error codes:

BAD_COM   (2) -  Bad 'com' parameter.

NO_UART   (3) -  No UART chip detected.

RCV_ALC   (4) -  Receive queue allocation error.

TX_ALC    (5) -  Transmit queue allocation error.

See also

ComOpenS ComClose ComBaud ComMode

Example

Open COM1 at 9600 baud with 8 bit words, 1 stop bit,

no parity, a 1024 byte receiver and a 512 byte

transmitter.

{

int rv;

rv = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(rv)

{

...error handler

}

...more code

}

10

----------------------------------------------------------------

ComOpenS

----------------------------------------------------------------

Function

Opens or reopens a serial port.

Syntax

#include "cport.h"

int ComOpenS(const C_param *param);

Parameters

param   - Pointer to the structure of type 'C_param'.

The structure 'C_param' is defined as follows:

struct C_param{

unsigned com;

int      baud;

byte     mode;

unsigned rxQ;

unsigned txQ;

byte     htype;

unsigned thresh;

};

com   - Constant that defines the serial port to be

opened. Possibilities are:

COM1  (0xC3F8)

COM2  (0xB2F8)

COM3  (0xC3E8)

COM4  (0xB2E8)

baud  - Constant defining the baud rate to which

the serial port will be initialized.

Possibilities are:

B115200    (1)

B57600     (2)

B38400     (3)

B19200     (6)

B9600     (12)

B7200     (16)

B4800     (24)

B3600     (32)

B2400     (48)

B2000     (58)

B1800     (64)

B1200     (96)

11

B600     (192)

B300     (384)

B150     (768)

B110    (1047)

B75     (1536)

B50     (2304)

mode  - Bit ORed constant defining the word

length, number of stop bits and the

parity that the serial port will use.

Possibilities are:

W8  (0x03)  -  8 bit words

W7  (0x02)  -  7 bit words

W6  (0x01)  -  6 bit words

W5  (0x00)  -  5 bit words

S1  (0x00)  -  1 stop bit

S2  (0x04)  -  2 stop bits

NONE   (0x00) -  No parity)

ODD    (0x08) -  Odd parity)

EVEN   (0x18) -  Even parity)

MARK   (0x28) -  Mark parity)

SPACE  (0x38) -  Space parity)

rxQ  - Size, in bytes, of the receive queue.

txQ   - Size in bytes, of the transmit queue.

* Limited to 32K in small and medium

memory models.

htype - Type of handshaking to be used. The two types

are:

SOFT  (1) - Uses XON and XOFF characters.

HARD  (2) - Uses RTS and CTS.

thresh - Receive queue threshold.

Remarks

ComOpenS is an alternative to ComOpen for opening or

a reopening serial port. It takes a pointer to a

structure of parameters instead of taking discrete

arguments. It also can set the handshaking without

making a separate call to ComHandshake.

The 4 most significant bits in 'com' represent the

12

interrupt vector, while the lower 12 bits represent

the port address. Constants are provided in cport.h

which will be satisfactory in most cases.

The parameter 'baud' is the baud rate divisor which

determines the baud rate. Constants are provided in

cport.h for the most common baud rates. The 'baud'

rate divisor is defined as f / (16 * baud rate) where

f is 1.8432e+6.

Achieving baud rates above 19200 is largely dependent

on many variables such as machine speed, cable length,

handshaking activity, degree bidirectional transfers,

the type UART (see Appendix C) used and activity of

other hardware interrupts.

The parameter 'mode' determines the word length,

number of stop bits and parity of the exchanged data.

Bits 0 & 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables

parity and bits 4 & 5 determine the type of parity

check. Constants which can be ORed together are

provided in cport.h.

'RxQ' and 'txQ' set the size of the receive and transmit

queues respectively. 'RxQ' should be at least 4 times

larger than the largest string or block of data the

programmer intends to fetch from the queue at once. '

TxQ' can be set as small as 1, but is not recommended.

In the small and medium memory models, where all data

is limited to 64K, the size queues are limited. In

the compact and large memory models, each queue can

be as large as 64K -1 (65534).

The parameter 'htype' defines the type of flow

control (handshaking) to be used by the transmitter

and receiver. 'Threshold' determines when the

receiver will assert of disassert the selected

handshaking.

Return value

If successful, ComOpenS returns 0. In the event of an

error, it returns one of the following error codes:

BAD_COM   (2) -  Bad 'com' parameter.

NO_UART   (3) -  No UART chip detected.

RCV_ALC   (4) -  Receive queue allocation error.

TX_ALC    (5) -  Transmit queue allocation error.

See also

ComOpen ComParam ComHandshake ComClose

13

Example

Switch to COM2 with the same parameters.

{

struct C_param param;

ComParam(&param);

param.com = COM2;

ComOpenS(&param);

}

----------------------------------------------------------------

ComReopen

----------------------------------------------------------------

This function has become obsolete, however, the

function name 'ComReopen' has been retained as a

function macro to maintain backwards compatibility.

NOTE: See ComOpen and ComOpenS

----------------------------------------------------------------

ComClose

----------------------------------------------------------------

Function

Closes the currently opened serial port.

Syntax

#include "cport.h"

void ComClose(void);

Remarks

ComClose closes the currently opened serial port.

Failing to call ComClose before a program is terminated

will cause the operating system crash. It need only be

called once prior to exiting, regardless of how many

times serial ports have been reopened.

See also

ComOpen ComOpenS

14

Example

Close the serial port prior to terminating.

{

...closing code

ComClose();

exit(0)

}

----------------------------------------------------------------

ComParam

----------------------------------------------------------------

Function

Retrieves the parameters of the currently opened serial

port.

Syntax

#include "cport.h"

void ComParam(const struct C_param *param);

Remarks

ComParam retrieves the parameters and handshaking

status of the currently opened serial port and stores

them in a structure of type 'C_param'. The structure

'C_param' is defined as follows:

struct C_param{

unsigned com;

int      baud;

byte     mode;

unsigned rxQ;

unsigned txQ;

byte     htype;

unsigned thresh;

};

See ComOpenS for a discussion of the individual

elements.

See also

ComOpen ComOpenS ComMode ComBaud

15

Example

Switch to COM2 with the same parameters.

{

struct C_param param;

ComParam(&param);

param.com = COM2;

ComOpenS(&param);

}

----------------------------------------------------------------

ComBaud

----------------------------------------------------------------

Function

Changes the baud rate of the currently opened serial port.

Syntax

#include "cport.h"

void ComBaud(int baud);

Parameters

baud  - Constant defining the baud rate to which

the serial port will be initialized.

Possibilities are:

B115200    (1)

B57600     (2)

B38400     (3)

B19200     (6)

B9600     (12)

B7200     (16)

B4800     (24)

B3600     (32)

B2400     (48)

B2000     (58)

B1800     (64)

B1200     (96)

B600     (192)

B300     (384)

B150     (768)

B110    (1047)

B75     (1536)

B50     (2304)

16

Remarks

The parameter 'baud' is the baud rate divisor which

determines the baud rate. Constants are provided in

cport.h for the most common baud rates. The 'baud'

rate divisor is defined as f / (16 * baud rate) where

f is 1.8432e+6.

Achieving baud rates above 19200 are largely dependent

on many variables such as machine speed, cable length,

handshaking activity, degree bidirectional transfers,

the type of UART used and activity of other hardware

interrupts.

See also

ComOpen ComOpenS ComParam ComMode

Example

Set the currently opened serial port to the next baud

rate in the table 'baud'.

{

static int indx = 0;

int baud[4] = { B9600,

B2400,

B1200,

B300

};

indx++;

indx %= 4;

ComBaud(baud[indx]);

}

----------------------------------------------------------------

ComMode

----------------------------------------------------------------

Function

Changes the word length, number of stop bits and parity

of the opened serial port.

Syntax

#include "cport.h"

void ComMode(int mode);

17

Parameters

mode  - Bit ORed constant defining the word length,

number of stop bits and the parity that the

serial port will use. Possibilities are:

W8  (0x03)  -  8 bit words

W7  (0x02)  -  7 bit words

W6  (0x01)  -  6 bit words

W5  (0x00)  -  5 bit words

S1  (0x00)  -  1 stop bit

S2  (0x04)  -  2 stop bits

NONE   (0x00) -  No parity)

ODD    (0x08) -  Odd parity)

EVEN   (0x18) -  Even parity)

MARK   (0x28) -  Mark parity)

SPACE  (0x38) -  Space parity)

Remarks

The parameter 'mode' determines the word length,

number of stop bits and parity of the exchanged data.

Bits 0 & 1 determine the word length, bit 2

determines the number of stop bits, bit 3 enables

parity and bits 4 & 5 determine the type of parity

check. Constants that can be ORed together are

provided in cport.h.

See also

ComOpen ComOpenS ComParam ComBaud

Example

Toggle the currently opened serial port to the next mode

in the table 'mode'.

{

static int indx = 0;

int mode[2] = { W8|S1|NONE,

W7|S1|EVEN,

};

indx ^= 1;

ComMode(mode[indx]);

}

18

----------------------------------------------------------------

ComHandshake

----------------------------------------------------------------

Function

Enables or disables flow control (handshaking).

Syntax

#include "cport.h"

void ComHandshake(byte htype, unsigned thresh);

Parameters

type - Type of handshaking to be used. The two types

are:

SOFT  (1) - Uses XON and XOFF characters.

HARD  (2) - Uses RTS and CTS.

thresh - Receive queue threshold.

Remarks

ComHandshake enables or disables software and/or

hardware handshaking. In software handshaking, XON

and XOFF characters are transmitted and received to

suspend and resume communications. In hardware

handshaking uses the RTS and CTS lines to suspend

and resume communications. 'Thresh' determines when

the receiver will assert and disassert.

See also

ComParam ComOpenS ComTx

Example

If COM1 is successfully opened, turn on software

handshaking and set the receiver threshold to 3/4 of

the receiver's full size.

{

int rv;

rv = ComOpen(COM1, B9600, W8|S1|NONE, 1024, 512);

if(rv)

{

19

...error handler

}

ComHandshake(SOFT, RCVQ_SIZE * 3 / 4);

...more code

}

----------------------------------------------------------------

ComTx

----------------------------------------------------------------

Function

Turns the transmitter on or off

Syntax

#include "cport.h"

void ComTx(byte on_off);

Parameters

on_off - Determines whether the transmitter is to be

turned on or off. Constants are provided in

cport.h. Possibilities are:

ON  (1) - Turns the transmitter on

OFF (0) - Turns the transmitter off

Remarks

ComTx allows you to turn the transmitter on and off

to suspend and resume transmissions. This function

can be very useful when implementing your own

handshaking scheme.

See also

ComHandshake ComOpenS ComParam

Example

If the transmitter is off ('off') and needs to be

turned on ('turn_on'), turn it on. If the transmitter

is on and needs to be turned off, turn it off.

20

{

if(off && turn_on)

ComTx(ON);

else if(!off && turn_off)

ComTx(OFF);

}

21

Input Functions

Input fuctions interface with the receiver and the internal

receive queue.

There are four functions for fetching from the receive

queue. ComGetc and ComGets can be used to fetch control codes

as well as normal ascii characters, while ComIn is intended

for fetching binary data. ComPeek returns a copy of the head

character without removing it from the queue.

ComFlushRx flushes the receiver while ComLenRx finds

the number of characters in the queue.

22

----------------------------------------------------------------

ComGetc

----------------------------------------------------------------

Function

Fetches a character from the receive queue.

Syntax

#include "cport.h"

char ComGetc(void);

Remarks

ComGetc fetches the head character from the receive

queue. If there are no characters in the receive

queue, ComGetc returns a null character ('\0').

Return value

On Success, ComGetc returns the character from the

receive queue. If it fails it returns a '\0'.

See also

ComPutc ComGets ComIn ComPeek

Example

Get a character from the serial port and test if it

is a start of header control character.

#define SOH 1

{

char c;

c = ComGetc();

if(c == SOH)

{

...SOH code

}

else

return c;

}

23

----------------------------------------------------------------

ComGets

----------------------------------------------------------------

Function

Fetches a string of characters from the receive queue.

Syntax

#include "cport.h"

char *ComGets(char *str, int max_c, char term_c);

Parameters

*str  - Pointer to the buffer where the string of

characters will be stored.

max_c - Maximum number of characters to fetch.

term_c - Termination character. ComGets will return

if this character is encountered.

Remarks

ComGets fetches a string of characters from the

receive queue. The string will be stored in 'str'.

'str' must be at least 'max_c' + 1 long to allow

for the null terminator.

ComGets will return on the first occurrence of one of

three conditions: 1) the character 'term_c' is

encountered; 2) 'max_c' characters are reached; or 3)

No more characters are available in the receive queue.

Return value

ComGets returns a pointer to the null terminated

string.

See also

ComPuts ComGetc ComIn ComLenRx

Example

Get a line of max 80 characters from the receive queue

and store it in 'str'. The string will be echoed to the

display.

24

{

char str[81];

if(ComLenRx() > 80)

puts(ComGets(str, 80, '\n'));

}

----------------------------------------------------------------

ComIn

----------------------------------------------------------------

Function

Fetches a block of bytes from the receive queue.

Syntax

#include "cport.h"

unsigned ComIn(void *abyte, unsigned num_byte);

Parameters

*abyte - Pointer to the buffer where the block of bytes

will be stored.

num_byte - Number of bytes to get from the receive queue.

Remarks

ComIn fetches a block of bytes from the receive queue

and stores it in 'abyte'. ComIn will always fetch

'num_byte' bytes unless not enough bytes are available.

ComIn is well suited for receiving binary data.

Return value

ComIn returns the number of bytes fetched.

See also

ComOut ComGetc ComGets ComLenRx

Example

Get a block of 1024 bytes from the serial port.

25

#define BLK_SZ 1024

{

byte block[BLK_SZ];

unsigned rs;

if(ComLenRx() > BLK_SZ)

{

rs = ComIn(block, BLK_SZ);

if(rs < BLK_SZ)

{

...error handler

}

}

}

----------------------------------------------------------------

ComFlushRx

----------------------------------------------------------------

Function

Flushes the receiver and receive queue.

Syntax

#include "cport.h"

void ComFlushRx(void);

Remarks

ComFlushRx flushes the receive queue and the UART of

any existing characters.

See also

ComLenRx ComFlushTx

Example

If a receive flush command is in order, flush the

receiver.

#define ...

#define RCV_FLSH 2

#define ...

{

int command;

...fetch command

26

switch(command)

{

case ...:

...case code

break;

case RCV_FLSH:

ComFlushRx();

break;

case ...:

...case code

break;

}

}

----------------------------------------------------------------

ComLenRx

----------------------------------------------------------------

Function

Calculates the number of characters in the receive queue.

Syntax

#include "cport.h"

unsigned ComLenRx(void);

Remarks

ComLenRx calculates the number of characters in the

receive queue. Valid numbers are 0 to 65534.

Return value

ComLenRx returns the number of characters presently

residing in the receive queue.

See also

ComFlushRx ComLenTx

27

Example

If there are more than 80 characters available, get a

line.

{

char str[81];

if(ComLenRx() > 80)

puts(ComGets(str, 80, '\n'));

}

----------------------------------------------------------------

ComPeek

----------------------------------------------------------------

Function

Fetches a copy of the head character in the receive

queue.

Syntax

#include "cport.h"

char ComPeek(void);

Remarks

ComPeek fetches a copy of the head character in the

receive queue. ComPeek will not remove the character

from the queue.

Return value

ComPeek returns the copy of the head character.

See also

ComGetc

Example

Determine if the head character is a control character.

#define CTRL_A 1

#define CTRL_Z 26

{

28

char peek_c;

char str[81];

peek_c = ComPeek();

if(peek_c >= CTRL_A && peek_c <= CTRL_Z)

{

switch(ComGetc())

{

...command processor

}

}

else

puts(ComGets(str, 80, '\n');

}

29

Output Functions

Output functions interface with the transmitter and the

internal transmit queue.

There are three functions for putting characters/bytes into

the transmit queue, all of which return the actual number of

characters/bytes successfully put. ComPutc and ComPuts can be

used to put control characters as well as normal ascii characters

into the transmit queue. ComOut is intended for putting binary

data into the transmit queue.

ComFlushTx flushes the transmitter while ComLenTx finds the

number of characters in the queue.

30

----------------------------------------------------------------

ComPutc

----------------------------------------------------------------

Function

Puts a character into the transmit queue.

Syntax

#include "cport.h"

int ComPutc(char c);

Parameters

c - Character to be put into the transmit queue.

Remarks

ComPutc puts a character into the transmit queue

to be transmitted. If the transmit queue is full,

the character is not put and ComPutc returns a 0.

Return value

On success, ComPutc returns a 1. If it fails, it

returns 0.

See also

ComGetc ComPuts ComOut

Example

Send an XOFF to the transmitter.

#define XOFF 13

{

if(!ComPutc(XOFF))

{

...error handler

}

31

----------------------------------------------------------------

ComPuts

----------------------------------------------------------------

Function

Puts a string of characters into the transmit queue.

Syntax

#include "cport.h"

int ComPuts(const char *str);

Parameters

*str  - Pointer to the string to be put into the

transmit queue.

Remarks

ComPuts puts the string 'str' into the transmit queue

to be transmitted. If the transmit queue becomes full,

ComPuts will not attempt to put any more characters in

the queue.

Return value

ComPuts returns the actual number of characters put

into the transmit queue.

See also

ComGets ComPutc ComOut

Example

Send the string "hello world" to the transmitter.

{

char *str = "hello world";

if(ComPuts(str) < strlen(str))

{

...error handler

}

}

32

----------------------------------------------------------------

ComOut

----------------------------------------------------------------

Function

Puts a block of bytes into the transmit queue.

Syntax

#include "cport.h"

unsigned ComOut(const void *abyte, unsigned num_byte);

Parameters

*abyte - Pointer to the buffer of the block to be put

into the transmit queue.

num_byte - Number of bytes to be put into the transmit

queue.

Remarks

ComOut puts the block of bytes pointed to by 'abyte'

into the transmit queue to be transmitted.

Return value

ComOutQ returns the number of bytes actually put into

the transmit queue.

See also

ComIn ComPutc ComPuts

Example

Send a block of 128 bytes to the transmitter.

#define BLK_SZ 128

{

byte block[BLK_SZ];

if(ComOut(block, BLK_SZ) < BLK_SZ)

{

...error handler

}

33

}

}

----------------------------------------------------------------

ComFlushTx

----------------------------------------------------------------

Function

Flushes the transmit queue.

Syntax

#include "cport.h"

void ComFlushTx(void);

Remarks

ComFlushTx flushes any existing characters in the

transmit queue.

See also

ComFlushRx ComLenTx

Example

If a transmit flush command is in order, flush the

transmitter.

#define ...

#define TX_FLSH 3

#define ...

{

int command;

...fetch command

switch(command)

{

case ...:

...case code

break;

case TX_FLSH:

ComFlushTx();

break;

case ...:

34

...case code

break;

}

}

----------------------------------------------------------------

ComLenTx

----------------------------------------------------------------

Function

Calculates the number of characters in the transmit

queue.

Syntax

#include "cport.h"

unsigned ComLenTx(void);

Remarks

ComLenTx calculates the number of characters in the

receive queue. Valid numbers are 0 to 65534.

Return value

ComLenTx returns the number of characters presently

in the transmit queue.

See also

ComLenRx ComFlushTx

Example

Check if the transmitter has reached 3/4 of its

capacity.

{

if(ComLenTx() > (TX_SIZE * 3 / 4))

{

...pause

}

}

35

Status Functions

Status functions are for obtaining or setting various status.

There are two functions for obtaining status: ComError detects

line errors in the serial port; and ComStatus returns the status

of the input hardware lines.

ComDtr, ComRts and ComOut1 manipulate the output hardware

lines

36

----------------------------------------------------------------

ComError

----------------------------------------------------------------

Function

Determines if any errors have occurred since the last

call to ComError.

Syntax

#include "cport.h"

unsigned ComError(void);

Remarks

ComError determines if any errors have occurred since

the last call to ComError or since the start of the

program on the first call to ComError. ComError returns

an error word in which the bits are defined as follows:

OVERUN  (0x002)  -  overrun error

PARITY  (0x004)  -  parity error

FRAMING (0x008)  -  framing error

BREAK   (0x010)  -  break detect

TIMOUT  (0x080)  -  timeout

TXFULL  (0x100)  -  transmit queue overflow

RXFULL  (0x200)  -  receive queue overflow

Return value

If an error has occurred, ComError returns an error

word. If no errors have occurred it returns a 0.

See also

ComStatus

Example

Determine the source of the error if one has occurred.

{

unsigned com_err;

com_err = ComError();

if(com_err)

{

switch(com_err)

37

{

case OVERUN:

...over run handler

break;

case PARITY:

...parity error handler

break;

case ...

}

}

----------------------------------------------------------------

ComStatus

----------------------------------------------------------------

Function

Retrieves the status of the hardware lines.

Syntax

#include "cport.h"

byte ComStatus(void);

Remarks

ComStatus retrieves the status of the serial port's

hardware lines. ComStatus returns a status byte in

which the bits are defined as follows:

DCTS (0x01) - change in clear to send.

DDSR (0x02) - change in data set ready.

TERI (0x04) - trailing edge ring indicator.

DDCD (0x08) - change in data carrier detect.

CTS  (0x10) - clear to send.

DSR  (0x20) - data set ready.

RI   (0x40) - ring indicator.

DCD  (0x80) - data carrier detect.

Return value

ComStatus returns the status byte.

38

See also

ComError ComRts ComDtr

Example

Test if the modem is powered up and ready.

{

ComDtr(ON);

delay(40);

if(!(ComStatus() & DSR))

{

puts("modem not responding");

...what to do code

}

}

----------------------------------------------------------------

ComRts

----------------------------------------------------------------

Function

Sets or clears the Request to Send line.

Syntax

#include "cport.h"

void ComRts(byte on_off);

Parameters

on_off - Determines whether RTS is to be set or

cleared. Constants are provided in cport.h.

Possibilities are:

ON  (1) - Sets RTS.

OFF (0) - Clears RTS.

Remarks

ComRts sets or clears the RTS line depending on the

value of 'on_off'. In many cases, this signal will be

looked at by the remote device to determine if it

is safe to transmit. This gives the programmer the

ability to turn off the remote device's transmitter.

39

NOTE: ComRts should never be called when hardware

handshaking is enabled.

See also

ComDtr ComOut1 ComStatus

Example

If the receiver is off ('rx_off') but ready to

receive, assert RTS. If it is on but not ready, turn

off RTS.

{

if(rx_off && rx_rdy)

ComRts(ON);

else if(!rx_off && !rx_rdy)

ComRts(OFF);

}

----------------------------------------------------------------

ComDtr

----------------------------------------------------------------

Function

Sets or clears the Data Terminal Ready line.

Syntax

#include "cport.h"

void ComDtr(byte on_off);

Parameters

on_off - Determines whether DTR is to be set or

cleared. Constants are provided in cport.h.

Possibilities are:

ON  (1) - Sets DTR.

OFF (0) - Clears DTR.

Remarks

ComDtr set or clears the DTR line depending on the

40

value of 'on_off'. Setting DTR informs the remote

equipment that the computer is ready to establish

communications.

See also

ComRts ComOut1 ComStatus

Example

If an break is detected, turn off DTR.

{

unsigned com_err;

int brk_flg = 0;

com_err = ComError();

if(com_err)

{

switch(com_err)

{

case ...

case BREAK:

ComDtr(OFF);

brk_flg = 1;

break;

case ...

}

}

}

----------------------------------------------------------------

ComOut1

----------------------------------------------------------------

Function

Sets or clears the Out 1 line.

Syntax

#include "cport.h"

void ComOut1(byte on_off);

41

Parameters

on_off - Determines whether OUT1 is to be set or

cleared. Constants are provided in cport.H.

Possibilities are:

ON  (1) - Sets OUT1.

OFF (0) - Clears OUT1.

Remarks

ComOut1 sets or clears the general purpose OUT1 line

depending on the value of 'on_off'. OUT1 is a general

purpose output sometimes utilized by device for a

special feature or reset.

See also

ComRts ComDtr ComStatus

Example

Reset a smart modem.

#define ComSMSet() ComOut1(ON)

#define ComSMClr() ComOut1(OFF)

{

ComSMRes();

delay(50);

ComSMClr();

}

42

Data Integrity

Two functions are currently provided for checking the integrity

of data. They are: ComChecksum and ComCrc16. ComChecksum and

ComCrc16 perform a checksum calculation and a 16 bit cyclic

redundancy check respectively, on a block of bytes. Notice also

that these functions take the same arguments as the functions

ComOut and ComIn.

43

----------------------------------------------------------------

ComChecksum

----------------------------------------------------------------

Function

Performs a checksum calculation on a block of bytes.

Syntax

#include "cport.h"

byte ComChecksum(const void *abyte, unsigned n_byte);

Parameters

*abyte - Pointer to the block on which the checksum

is to be performed

num_byte - Number of bytes in the block.

Remarks

ComChecksum performs a one byte checksum on the block

of bytes pointed to by 'abyte'. The checksum is

calculated by summing all the bytes in the block and

ignoring the carry outs.

Return value

ComChecksum returns the one byte checksum.

See also

ComCrc16 ComIn ComOut

Example

Verify the actual checksum matches the received

checksum.

{

struct Xmodem{

byte num;

byte _num;

byte data[128];

byte checksum;

}block;

44

ComIn(&block, sizeof(struct Xmodem));

if(ComChecksum(block.data, 128) != block.checksum)

{

...bad block handler

}

else...

}

----------------------------------------------------------------

ComCrc16

----------------------------------------------------------------

Function

Performs a 16 bit cyclic redundancy check on a block of

bytes.

Syntax

#include "cport.h"

unsigned ComCrc16(const void *abyte, unsigned n_byte);

Parameters

*abyte - Pointer to the block on which the 16 bit

cyclic redundancy check is to be performed.

num_byte - Number of bytes in the block.

Remarks

ComCrc16 performs a 16 bit cyclic redundancy check on

the block of bytes pointed to by 'abyte'. The 16 bit

CRC is calculated using a table of precalculated

values.

Return value

ComCrc16 returns the two byte CRC value.

See also

ComChecksum ComIn ComOut

45

Example

Verify the actual CRC matches the received CRC.

{

struct Xmodem{

byte num;

byte _num;

byte data[128];

unsigned crc16;

}block;

ComIn(&block, sizeof(struct Xmodem));

if(ComCrc16(block.data, 128) != block.crc16)

{

...bad block handler

}

else...

}

46

Misc Functions

There are two pair of misc functions: ComSetBreak and

ComClrBreak set and clear a break condition; ComPutScrtch

and ComGetScrtch write to and read from the scratch register.

47

----------------------------------------------------------------

ComSetBreak

----------------------------------------------------------------

Function

Set a break condition.

Syntax

#include "cport.h"

void ComSetBreak(void);

Remarks

ComSetBreak sets a break condition. The break condition

will stay in effect until a subsequent call to

ComClrBreak is made.

See also

ComClrBreak

Example

If the an abort condition has been declared, set a

break condition. When it clears, release the break

condition.

{

int abrt_flg;

int brk_flg;

if(abrt_flg && !brk_flg)

ComSetBreak();

else if(!abrt_flg && brk_flg)

ComClrBreak();

}

----------------------------------------------------------------

ComClrBreak

----------------------------------------------------------------

Function

Clears a previously set break condition.

48

Syntax

#include "cport.h"

void ClrBreak(void);

Remarks

ComClrBreak clears a previously set break condition.

See also

ComSetBreak

Example

If the an abort condition has been declared, set a

break condition. When it clears, release the break

condition.

{

int abrt_flg;

int brk_flg;

if(abrt_flg && !brk_flg)

ComSetBreak();

else if(!abrt_flg && brk_flg)

ComClrBreak();

}

----------------------------------------------------------------

ComPutScrtch

----------------------------------------------------------------

Function

Writes a byte to the serial port's scratch register.

Syntax

#include "cport.h"

void ComPutScrtch(byte abyte);

Parameters

abyte - The byte to be written to the scratch register.

49

Remarks

ComPutScrtch writes the byte value 'abyte' to the

UART scratch register. Not all UARTs have a scratch

register.

See also

ComGetScrtch

Example

Use the scratch register to store a status byte.

#define ComSavStatus() ComPutScrtch(ComStatus())

#define ComOldStatus() ComGetScrtch()

{

unsigned old_stat;

ComSavStatus();

...later on

old_stat = ComOldStatus();

}

----------------------------------------------------------------

ComGetScrtch

----------------------------------------------------------------

Function

Fetches the byte stored in the serial port scratch

register.

Syntax

#include "cport.h"

byte ComGetScrtch(void);

Remarks

ComGetScratch reads the byte in the UART scratch

register. Not all UARTs have a scratch register.

Return value

ComGetScrtch returns the byte read from the UART

scratch register.

50

See also

ComPutScrtch

Example

Use the scratch register to store a status byte.

#define ComSavStatus() ComPutScrtch(ComStatus())

#define ComOldStatus() ComGetScrtch()

{

unsigned old_stat;

ComSavStatus();

...later on

old_stat = ComOldStatus();

}

51

Appendix A

Things to look out for:

1) Since Cport is driven by hardware interrupts, input and

output functions will work unpredictably inside a debugger.

If a break point is set and the program is run, everything

fuctions until the break point is reached. At that point the

interrupts will probably cease and the transmitter and

receiver will freeze. Needless to say, single-stepping is out

of the question.

2) Baud rates above 19200 require special attention because they

are sensitive to many factors. Some of these factors are:

Machine speed

Activity of handshaking

Other interrupt activity

The last factor, interrupt activity, includes keyboard

activity which can be a major impedance. During critical high

speed transfers, some interrupts can be temporarily masked

out (disabled) at the PIC to preclude them from impeding the

communications interrupt handler.

3) In the small and medium memory models, the near heap where

the queue buffers are allocated is limited. The data segment

(globals etc.) + the stack + allocated memory (queue buffers)

must not exceed 64K bytes. If extremely large queue(s) are

required, the compact or large model where each queue can be

as large as 65534 bytes must be used.

4) The functions ComOpen, ComOpenS and ComClose make calls to

the C functions malloc() and free().

52

Appendix B

Handshaking schemes

There are three options available for implementing handshaking.

The first two options are built-in and are enabled by calling

the function ComHandshake. The third option is external. The

three options are described as follows:

Hardware handshaking

In hardware handshaking , the hardware signals RTS and CTS

are used to achieve handshaking. RTS is used by the receiver

to signal remote device to suspend or resume transmitting.

Similarly, CTS is used by the remote device receiver to signal

the transmitter to suspend or resume transmitting.

Software handshaking

In software handshaking, the control characters XON (ctrl Q)

and XOFF (ctrl S) are used to achieve handshaking. When

either the remote device or the computer receive an XOFF

character, transmitting is suspended. When an XON character

is received transmitting is resumed.

The receiver

When software and/or hardware handshaking are enabled the

receiver monitors the length of the receive queue. If the

length exceeds the threshold set by the function

ComHandshake, the receiver will take the appropriate action

to signal the remote device to suspend transmitting. When

the length of the receive queue subsequently falls below half

of the threshold, the appropriate action is taken to signal the

remote device to resume transmitting.

The transmitter

If the computer is signaled to suspend transmitting and the

corresponding handshaking is enabled, the transmitter will

stop transmitting. Subsequently, when the computer is

signaled to resume transmitting, the transmitter will pick up

where it left off.

User handshaking

The third option for implementing handshaking is to implement

the programmer's own handshaking scheme. To accomplish this,

Cport provides the function ComTx. ComTx enables the programmer

53

to turn the transmitter on and off. When the transmitter is

turned off, transmitting halts immediately. If any characters

still reside in the transmit queue, they will remain there until

the transmitter is turned back on or the transmit queue is

flushed. More characters may be put into the transmit queue

while the transmitter is turned off.

Cooperation

In any handshaking scheme it is imperative that the computer and

the remote device recognize the handshaking scheme. For instance,

if the remote device recognizes hardware handshaking and the

computer does not, the computer will ignore any requests by the

remote device to suspend transmitting. If the remote device

recognized software handshaking and the computer did not, bogus

XON and XOFF characters might be found to be corrupting the

received data.

54

Appendix C

The UART (Universal Asynchronous Receiver/ Transmitter) chip

The UART, a single integrated circuit chip, is the backbone

of serial communications in computers today. Serial

communications in IBM and compatible computers is based on the

National Semiconductor family of UARTs. These chips include:

INS8250

INS8250-B

INS8250A

INS82C50A

NS16450

NS16C450

NS16550A

The latest and greatest is the NS16550A which features

transmitter and receiver FIFO's. This feature greatly

improves performance. Cport will take advantage of the

NS16550 chip in future versions.

55

Appendix D

The RS-232 Standard ?!!

Voltages, logic levels and pin assignments of the RS-232

standard are more or less adhered to. However, the way the

RS-232 standard prescribes data transfers to be controlled,

more commonly know as handshaking, does not work for computers.

This can lead to much confusion and frustration when trying

to connect two serial communications devices from different

manufacturers.

The book "The RS-232 Solution" by Joe Campbell addresses

this problem and includes a number of case studies. It is

recommended reading for both the novice and expert programmer.

56

Appendix E

Cport Registration Form

Name: ______________________________________________________

Company: ___________________________________________________

Address: ___________________________________________________

City: ____________________________  State: _________________

Zip Code: _________________  Phone # : _____________________

CompuServe # : _____________________________________________

How did you acquire Cport ? _________________________________

____________________________________________________________

What functions do you find most useful ?____________________

____________________________________________________________

What functions do you find least useful ?___________________

___________________________________________________________

What functions, not in Cport, would you like to see ?_______

____________________________________________________________

____________________________________________________________

Other comments ?____________________________________________

____________________________________________________________

Registration                                      $ 25.00

California residents add sales tax ( $ 1.75 )     $________

Shipping inside continental U.S.   ( $ 2.00 )

Shipping outside continental U.S.  ( $ 5.00 )     $________

Total:   $________

All Payments Must be in U.S. Dollars

Make check or money order payable to:    Bri Productions

P.O. Box 7121

Fremont, CA 94537-7121

57