CTask Manual          Version 1.1  88-07-01                Page 1

CTask

A Multitasking Kernel for C

Version 1.1  Released 88-07-01

Public Domain Software written by

Thomas Wagner

Patschkauer Weg 31

D-1000 Berlin 33

West Germany

BIXmail: twagner

Introduction

============

CTask  is a set of routines that allow your C program  to  execute

functions  in parallel, without you having to build  in  sophisti-

cated  polling and switching schemes. CTask handles the  switching

of processor time with a priority based, preemptive scheduler, and

provides a fairly complete set of routines for inter-task communi-

cation,  event signalling, and task interlocking.  CTask also  in-

cludes  a  number of drivers for MS-DOS that build  on  the  basic

functions  to allow you to include serial I/O, printer  buffering,

and  concurrent  access to DOS functions into your  programs  with

little programming effort.

An Example

To  illustrate one possible use of CTask, let me elaborate on  the

following  example. Say you just finished your nifty  telecommuni-

cations  program, complete with download protocols,  scripts,  and

everything.  But wouldn't it be nice to be able to print the  file

you  just downloaded while receiving the next, and edit a  comment

to some previous message without interrupting the transmission? So

you take those editor routines from a previous project, plug  them

in, and - oops, how do you switch back and forth between  editing,

communication  and  printing? The answer to this is  CTask.  CTask

allows your C program to do many different things at the same time

by switching the processor between the tasks you define. And since

most  of  the time your program is waiting for  some  slow  device

(like the human hand) to provide feedback, this switching is  com-

pletely  transparent,  and will not noticeably slow  your  program

down.

Switching the Context

So  what is needed to allow the user to edit a file while  at  the

same  time  downloading another and printing a third?  First,  you

have to have some form of *context switching*. This means that you

have  to be able to interrupt the processing of the download  when

CTask Manual          Version 1.1  88-07-01                Page 2

the  user presses a key, process the key, and return to the  down-

load  *task*  at  the exact same point  it  was  interrupted.  One

solution  to this would be to include a poll for the  keyboard  at

several points in the download routine, and call the editor *task*

when a key is available.  But apart from cluttering your code with

lots  of unrelated calls, there is another problem.  What  if  the

operation  the user requested is more involved than  just  putting

the character on the screen, like writing the file to disk?   This

might  take so long that your download times out. There must be  a

way  to  pass control back and forth between the two  tasks,  such

that  no task is delayed for an extended period of time, and  also

to  activate  the print spooler task at some defined  interval  to

output  the data to the printer. This context switching is  called

*scheduling* in CTask. The *scheduler* is invoked on every  system

timer  tick,  and will save the context of the current  task.  The

scheduler  then  takes the first element from the queue  of  tasks

that  are  eligible to be run, and restores the  context  of  this

task, returning to the point where the task was interrupted.  This

switching  is completely automatic, and requires no  special  pro-

gramming in the tasks itself. All you have to do is to tell  CTask

that  there are three tasks, the download task, the spooler  task,

and the editor task.

You have Mail

All  you have to do for context switching, that is. There's a  bit

more  to  multitasking  than meets the eye. How do  you  tell  the

spooler task what files to spool, and the download task what files

to  download? You can't call a task like an ordinary function,  so

what  you need for this is *inter-task communication*. There  must

be  a way to pass a message containing the filename to be  printed

to  the spooler task, and there are several in CTask, one of  them

the  *mailbox*.  The spooler can use a CTask call,  wait_mail,  to

wait  for a message to arrive at its mailbox. As long  as  nothing

arrives, the spooler task will no longer be scheduled, so if there

is nothing to print, it will not use any processor time. When  you

send  a  message with send_mail to the mailbox, the  spooler  will

wake  up, and process the file. You can also send more  file  name

messages to the spooler while it still prints a file, leaving  the

messages in the mailbox until the spooler is ready to process  the

next file.

Reentrancy and Resources

This  last  example  seems  innocent enough,  but  there's  a  big

stumbling block hidden in it. You allocate the file name  messages

in  the  controlling task with malloc, and you free  them  in  the

spooler  with  free,  no problem, right? Wrong,  there  is  a  big

problem,  *reentrancy*.  Reentrancy means that you can re-enter  a

routine while another task is already using it, and that this will

not disturb the operation of the interrupted task. But malloc  and

free  share  and  modify global data, the  chain  of  free  memory

blocks.  Imagine  the following: You just called malloc  from  the

controlling task. Malloc has loaded the address of a free  element

CTask Manual          Version 1.1  88-07-01                Page 3

into  a local variable, and is about to write back the pointer  to

the  next free element into the last. At exactly this moment,  the

timer  ticks, and the spooler is activated.  It has just  finished

printing,  so it calls free. Free steps through the chain of  free

blocks  to find the right place to insert the block. According  to

Murphy's law, it will find just the place where malloc is about to

write back the pointer. Free coerces the elements, points the next

pointer  to the element malloc just wants to take off  the  chain,

and returns. Malloc writes its next pointer into the middle of the

newly coerced block, and now returns an element which is still  in

the  free list. Compared to the job of finding this kind  of  bug,

stepping  in for Tantalus may feel like a vacation. This  kind  of

problem  code is called a *critical region*.  There must be a  way

to make sure that no two tasks simultaneously enter such a region,

and, you guessed it, CTask provides one, the *resource*. When  you

call  request_resource  in  one task, all other  tasks  trying  to

request  the same resource after that are put to sleep  until  you

call  release_resource. Only then will the highest  priority  task

that waits for the resource wake up, and get access to the protec-

ted region. So you would have to substitute malloc and free  calls

in  your  routines with calls to functions that  first  request  a

resource, execute the function, and then release the resource.

DOS Access

But, you might ask, isn't there another reentrancy problem in this

example, since both the spooler and the download task might simul-

taneously call DOS to do their file-I/O, and DOS is not reentrant?

Do I have to substitute all my calls to fread and fwrite, too? The

answer  to this, luckily, is no. CTask traps all your  DOS  calls,

and automatically encloses them in the necessary resource  request

and release calls, so you don't have to worry about trashing  your

disk by simultaneous DOS requests. The limited multitasking  capa-

bilities of DOS are exploited to allow some parallel processing in

DOS,  and CTask will also detect and handle DOS calls by  resident

background programs like the DOS PRINT utility.

Piping the Keyboard

CTask  also  allows you to circumvent DOS for keyboard  input,  so

that  waiting  for the keyboard will not block  other  tasks  from

access  to  DOS functions. CTask uses another form  of  inter-task

communication  for  storing  keys entered  on  the  keyboard,  the

*pipe*.  The pipe is similar to the mailbox in that you  can  wait

for  items to be sent to a pipe. But unlike mailboxes,  pipes  use

their  own buffer to store the items (which are limited  to  bytes

and  words),  so you don't have to allocate mail blocks  for  each

item. When waiting on the pipe, your task is put to sleep, so  the

processor  is  free  to do more interesting things  than  to  loop

waiting for the user to press a key.  When a key is pressed, it is

written to the pipe, waking your task.

CTask Manual          Version 1.1  88-07-01                Page 4

Serial I/O and Timeouts

Pipes are also used for the serial I/O handler included with CTask

that  makes some of the work you've put into  your  communications

package obsolete. When outputting data to the serial port via  the

CTask routines, the data is buffered in a pipe, and incoming  data

is  also  placed in a pipe. All interrupt handling, and  the  pro-

cessing of modem status and XON/XOFF protocols, is done by  CTask,

so you can concentrate on implementing the higher level protocols.

Since CTask allows all calls that wait for pipes, mail, and  other

events, to specify a timeout that is based on the system tick, you

do  not have to resort to timed waiting loops to  detect  communi-

cation line faults.

Priorities

If the protocol you implement requires fast responses to  incoming

blocks,  you  can  influence the response of CTask  to  your  comm

task's  needs by giving this task a higher priority. CTask  allows

65535 different priority levels, and tasks having higher  priority

are  scheduled  before  tasks  with  lower  priority.  Also,  high

priority  tasks  will get access to mail,  pipes,  and  resources,

before  other tasks. It might even be sensible to split  the  comm

task into two separate tasks, one of high priority that  assembles

the  incoming  bytes into blocks and handles the protocol,  and  a

lower priority task that reads the received blocks from a  mailbox

and stores them on the disk. In extremely time critical applicati-

ons, you can even turn off task preemption, so the timer tick will

no longer cause a task switch.

Change to your liking

CTask provides all basic building blocks for implementing  concur-

rent  programs in an easy and comprehensible way. Since  CTask  is

mainly  implemented  in  C, it may not  be  the  fastest  possible

system,  but  due  to the straightforward design  which  uses  few

shortcuts, modifying the sources to suit your needs and taste  can

be  done  without weeks of studying assembler code  that  squeezes

every microsecond from the processor. CTask is public domain code,

and  there  are no restrictions on its use. It is  distributed  in

source form, so you are free to change all aspects of the package.

Multitasking  programs, especially in embedded applications,  tend

to  be  very diverse in their needs for  specific  constructs.  So

although CTask is ready to run under DOS, and is easily  adaptable

for embedded applications, you should see CTask more as a starting

point  for your own thoughts, and as a toolbox from which you  can

pick the instruments you need, than as a finished and fixed  block

of code you simply plug into your application.

CTask Manual          Version 1.1  88-07-01                Page 5

How does CTask work

===================

Queues

CTask  uses priority queues for most of its functions. A  priority

queue differs from the usual FIFO (first in, first out) queues  in

the insertion of elements into the queue. Rather than just placing

new elements after the last queue member, the priority of the task

determines  its place in the queue. A task is enqueued  after  all

queue members with higher or equal priority, but before any member

with  lower  priority. Removal from the queue takes place  at  the

first element. These queues are used with all operations in  which

a  task is made waiting, be it waiting to get run, or waiting  for

an event. The advantage of this is the simplicity of  implementing

priorities. Other schemes might rely on arrays of queues, or sear-

ching  through  lists, to determine the highest  priority  waiting

task. With priority queues, the most time critical function, sche-

duling, is very fast, since it can simply take the first task from

the  queue  of eligible functions. The disadvantage  is  the  time

required to step through all greater or equal priority members  of

a queue to find the right place to insert the task. But since  the

number of tasks simultaneously enqueued in the same queue is  nor-

mally relatively small with most multitasking systems, this disad-

vantage is more than offset by the simple, and thus easy to imple-

ment  efficiently, scheme. The operation of removing a  task  from

somewhere in the middle of a queue is rare (it will only occur  on

task kill, task wake, or timeout), and thus the overhead of  mana-

ging a doubly linked list is not justified.

The Scheduler

In  a  CTask system, there is at least one  essential  queue:  the

eligible queue, which contains all tasks waiting to run. Each time

the scheduler is invoked, it will first save all processor  regis-

ters  on the current stack, save the stack pointer in the  current

tasks  control block, and then enqueue the current task  into  the

appropriate  queue.   Normally, this will be the  eligible  queue.

Then  the first element in the eligible queue is removed from  the

queue,  the stack is switched to the stack of this task, and  pro-

cessor registers are restored from the new stack, returning to the

point where the new task was interrupted. If the eligible queue is

empty  the scheduler will loop with interrupts enabled. The  queue

head pointer of the task control block of the current running task

determines  what  will happen to the task when  the  scheduler  is

invoked. If it points to an event control block, the task will  be

enqueued  as waiting for this event. If it is NULL, the task  will

not  be  enqueued  in any queue, effectively stopping  it.  If  it

points to the eligible queue, the task will be enqueued there.  In

any case, the scheduler does not care what queue the task is to be

enqueued  in, since all queues use the same priority based  order-

ing. If a task is no longer in the eligible queue, it can only  be

returned to this queue by an external event.

CTask Manual          Version 1.1  88-07-01                Page 6

Events

External  events  can  take several forms,  which  are  relatively

similar on the inside. All CTask events use a control block,  with

at  least one queue for tasks waiting for the event.  Although  it

would  have been possible to summarize all events under  a  global

structure,  this approach was not used in CTask, the main  reasons

being execution speed and ease of use. So if you scan through  the

source  files  for CTask events, you will see  many  very  similar

routines, which only differ in the types of data and the names  of

queues they process. In a class based language like C++, one would

certainly  define  all events as instances or derivations  of  one

class.  In plain C, the necessary type casting, and the  different

handling of certain cases, would clobber the code to the point  of

illegibility.

Resources

The  event types "resource", "flag", and "counter" differ only  in

the  kind of wait operations and the handling of the  state  vari-

able.  The  resource is mainly for use  in  interlocking  critical

regions  of  code, to protect access to  non-reentrant  resources.

Only one task can "own" a resource, all other tasks requesting the

resource are delayed until the owning task releases it. For  added

protection,  CTask  stores the task control block address  of  the

current owner of the resource in the resource control block, so no

other task can erroneously release it.

Flags

Flags, on the other hand, can be set and cleared by any task,  and

also by interrupt handlers. Tasks can wait on either state of  the

flag,  and all tasks waiting for a state are simultaneously  acti-

vated  when  the flag is changed to this state. This  makes  flags

suitable for use as a global signalling mechanism.

Counters

Counters  are  a variation on the flag concept. A counter  can  be

incremented  and cleared by any task, and by  interrupt  handlers.

Tasks  can wait for a counter to be zero or nonzero.  Like  flags,

all tasks waiting for the zero state are activated simultaneously.

But  unlike  flags, only the first task waiting  for  the  nonzero

state of a counter will be activated on incrementing the  counter,

and  the  counter will be automatically decremented  by  one.  The

counter is used inside CTask to handle timer interrupts. The timer

counter  will  be incremented on each timer tick,  activating  the

timer task. If for any reason the timer task is unable to complete

its run until the next tick, this tick will not be lost, since the

counter  is incremented, and the timer task will continue  to  run

the next time it calls the counter wait request.

CTask Manual          Version 1.1  88-07-01                Page 7

Mailboxes and Pipes

The  "mailbox"  and  "pipe"  events can  be  used  for  inter-task

communication,   and  for  the  communication  between   interrupt

handlers and tasks.

Mailboxes

Mailboxes can hold an unlimited number of mail blocks. Mail blocks

have  no fixed structure, but the first doubleword in  each  block

passed  to  a  mailbox routine is used as a  chain  pointer.  Mail

blocks  are chained into a mailbox in FIFO order. Tasks  can  wait

for  mail to arrive, or can conditionally read mail if a block  is

available.  Tasks  and interrupt handlers can write  blocks  to  a

mailbox. Since mailboxes don't need any copying of data, they  are

suited  for high speed exchange of larger amounts of data  between

tasks. The disadvantage of sending mail this way is that you  have

to  make sure that a mail block is not re-used before it has  been

read  out  of the box and processed by the  receiving  task.  When

exchanging fixed length messages, you can build a free mail  block

chain by using a mailbox to hold the available blocks.

Pipes and Buffers

Buffered message exchange is possible using pipes. When creating a

pipe,  you  specify  a buffer area and its length,  and  the  pipe

routines  will buffer all data written to the pipe in  this  area.

This implies that writing to a pipe may cause a task to be delayed

until  there  is space available in the pipe. To  allow  interrupt

handlers to write to pipes, there is a conditional write  request,

which  will simply return if the pipe is full. Tasks can wait  for

data  to  arrive  in a pipe, and for the pipe  to  be  emptied.  A

conditional  read  request is also provided. The  disadvantage  of

pipes  is that they are slightly slower than mailboxes due to  the

necessary copying, and that you can only place word or byte  sized

items  in  a pipe. When there is more than one  reader  or  writer

task,  you  can  not  rely on the bytes in a  pipe  being  in  any

specific  order.  A "buffer" construct is provided in  CTask  that

expands pipes to allow arbitrary length messages to be written and

read. This is implemented using a resource for reading and writing

to  the pipe associated with the buffer, so the message is  always

guaranteed  to  be  written  and read  in  one  piece.  But  since

resources  can  not  be used in  interrupt  handlers,  using  such

buffers is not allowed from interrupts.

Applications

There  are a number of routines included in the CTask package  for

PC-specific tasks. Although they will be of limited use for embed-

ded  applications, studying the serial I/O and  printer  interface

routines  will give you some hints on how to use the CTask  kernel

for  implementing  your own device drivers. For  PC  based  appli-

cations,  the routines should be usable with no or little  changes

for implementing complete communications packages.

CTask Manual          Version 1.1  88-07-01                Page 8

The  serial I/O handler uses interrupts for both input and  output

of data, and supports both XON/XOFF and RTS/CTS handshake methods.

The modem inputs can selectively be enabled to control data trans-

mission.   Pipes are used for transmit and receive data, with  the

buffer  and  its size specified on initialization. Both  COM1  and

COM2 can be active simultaneously, and other ports can be  suppor-

ted by editing a table in the source code. Since CTask allows  the

access  to  all events in interrupt  handlers,  writing  interrupt

based I/O drivers is relatively simple.  On receiving a character,

the character and any associated errors are placed in the  receive

pipe,  and the transmit pipe is read on transmit ready  interrupt.

Note  that  conditional reads and writes have to be  used  in  the

interrupt  handler,  since  you can't safely  delay  an  interrupt

handler.

The  printer  output driver supports both  polling  and  interrupt

output.  Again, a pipe is used for the output characters. However,

since polling is supported, and because of the somewhat unreliable

interrupt  structure  of the printer ports, a driver task  is  re-

quired.   This  task is automatically created  on  installing  the

driver.  The  task first waits on the pipe for a character  to  be

written  to  the printer.  When polling is enabled,  it  tries  to

output the character directly, using a short busy-waiting loop. So

as  not  to load the system too heavily by the polling,  the  task

will delay itself for a defined amount of timer ticks if it  can't

output  the character after a small number of loops.  When  inter-

rupts  are  enabled, the process is essentially the  same  at  the

start,  but after the character is written to the port,  the  task

will wait on a flag to be set. The interrupt handler will set  the

flag  on  interrupt, enabling the task to get the  next  character

from the pipe. Since interrupts are unreliable with most  printers

due to the usually very short pulses on the acknowledge line,  the

task  uses a timeout on the flag wait, so it does not hang  if  an

interrupt  is missed. Studying the printer driver will  also  give

you  an idea what the optional parameter on task creation  can  be

used  for. In the printer driver, this parameter is used  to  pass

the address of the printer control block, which contains the pipe,

the  flag,  and the hardware info, to the task.  This  allows  the

printer  driver to be simultaneously installed for any  number  of

printers without having to write separate printer tasks.

CTask Manual          Version 1.1  88-07-01                Page 9

General Notes

=============

What can CTask NOT be used for?

CTask  is not intended to provide for multitasking on the  command

level of MS-DOS.  Although CTask includes a module for  channeling

simultaneous  DOS requests inside a program,  the strategy used in

this module is not sufficient for the functionality required  when

switching between programs. Adding this functionality would not be

trivial (although certainly worthwhile).

Also, there is no warranty that CTask does perform without errors,

or  does exactly what you or I intended.  So CTask should  not  be

used  in  applications  in which malfunction of routines  of  this

package would result in damage to property or health of any person

without  *very*  extensive testing under all kinds  of  loads.  In

using CTask, you do so at your own risk.

What is required to use CTask?

To  compile  CTask,  Microsoft C 5.0 or later,  or Turbo C 1.0  or

later  are required.  Microsoft MASM 5.0 or later is required  for

the assembler parts.  Conversion to other compilers is possible if

they  conform to the new ANSI (draft) standard. Conversion of  the

assembler parts to other Assembler versions requires  substitution

of  the  simplified  model directives by  explicit  segment  defi-

nitions, and adding the DGROUP to offsets referencing data.

CTask will add 8-12k of code to your program, depending on options

and  installed drivers. The minimum static data used by  CTask  is

approximately 4k.

Converting  CTask for stand-alone operation requires few  changes.

Mainly,  the  timer interrupt handler (in "tsktim.asm") has to  be

rewritten,  and the initialization code (in "tskmain.c") may  have

to  be  changed. Changes to other modules  (naturally  except  the

optional hardware drivers) should not be necessary.

Another requirement is a good debugger.  If you never before wrote

multitasking  applications,  you're  in for  some  surprises.  The

normal debugging tools (Symdeb, Codeview) are of only limited use,

since they use DOS calls for their I/O, and thus may conflict with

your background tasks.  One safety measure is to first  thoroughly

test  your  program with preemption disabled,  possibly  inserting

some schedule() calls, and only allow task preemption if you found

most major bugs.  I personally recommend Periscope for  debugging,

since  it  can be made resident and so is  always  available,  and

because it does not use DOS.  Periscope III is the most  expensive

solution,  and the best tool you can imagine (except for Periscope

IV,  announced for April),  but the less costly versions will also

help a lot.

CTask Manual          Version 1.1  88-07-01                Page 10

Do I have to pay for using CTask?

No. One reason for writing CTask was to provide a free, no strings

attached,  utility, instead of the usual "for personal  use  only"

restriction.  Writing a multitasking application for personal  use

only doesn't seem too interesting to me. CTask is completely free,

and there is no restriction on its use. You may incorporate all or

parts of CTask in your programs, and redistribute it in source  or

binary form by any means. I also do not restrict the use of  CTask

in  commercial  applications.  Since  trying  to  distribute   the

unmodified CTask for money will only give you a bad name, you  may

even do that if you find someone dumb enough to buy it. Naturally,

if  you make a bundle from it, or simply like CTask, I  would  not

reject a donation. However, this is not required, and it will  not

give you any additional support.

What support can I expect?

I  will try my best to eliminate any bugs reported to me,  and  to

incorporate suggested enhancements and changes.  However, my spare

time  is limited,  so I can not guarantee continued or  individual

support.  (But since I am a free-lance consultant,  you can always

hire me to do it...)

At the time of this writing,  I'm connecting to BIX almost  daily.

Problems  of limited general interest can be reported via BIXmail,

suggested enhancements and changes,  plus bug reports,  should  be

posted  in the c.language/tools conference on BIX (until responses

warrant a new conference,  or I can't afford BIX any  longer),  so

they can be discussed among all interested (I hope there will be a

few).  Normal  mail  can  be used,  too,  but  don't  expect  fast

responses.

About this Release

Since  the  Beta release of CTask in March, CTask has  been  down-

loaded  from BIX more than 90 times. Of all downloaders, just  six

commented  on  CTasks  features, or sent bug  reports.  The  ideas

presented by the commentors are mainly integrated in this version,

as  are the fixes for the reported bugs. But since  comments  were

sparse,  this is not a revolutionary new release. Many  areas  are

completely unchanged, or just slightly modified to accommodate new

flags  and  options. See the accompanying file  "changes.doc"  for

a summary of the changes.

Although I currently have no new ideas that would warrant  another

release,  I  again  would like to invite you to  voice  your  com-

plaints, suggest enhancements, supply your own code for  inclusion

in  the package (note that any code submitted for  inclusion  must

not be copyrighted or usage restricted, but I'll naturally respect

copyrights  in  code  submitted for  demo  purposes),  or  suggest

additions to this manual.

CTask Manual          Version 1.1  88-07-01                Page 11

Multitasking Basics

===================

Tasks

In CTask, a "task" is defined as a (far) C function. The number of

tasks  is  not limited,  and one function may be used for  several

tasks.  There  is little difference between a task function and  a

normal function. The usual form of a task function is

void far my_task (farptr arg)

{

one-time initialization code

while (TRUE)

{

processing code

}

}

A task function is (usually) never called directly.  Rather, it is

specified  in the call to the create_task routine,  and started by

start_task.  It will then continue to run,  sharing the  processor

time  with  all other tasks,  until it is "killed"  by  kill_task.

Returning  from  the routine will have the same effect as a  kill.

The sharing of processor time is accomplished by "preempting"  the

tasks. Preemption means that the task is interrupted in the middle

of some statement by a hardware interrupt (usually the timer), and

is *not* immediately restarted when the interrupt handler returns.

Instead,  the  next  task that is able to run is activated by  the

"scheduler", with the interrupted task continuing its duty at some

(normally  unpredictable)  later time.  You can also  have  multi-

tasking without preemption, and CTask supports this, too, but this

requires full cooperation of all tasks in the system, such that no

task  continues  to  run for an extended period  of  time  without

passing control to other tasks by an explicit scheduling  request,

or by waiting for an event.

The  optional  argument  to the task function may be used  if  one

function is to be used for more than one task, for example to pass

a  pointer to a static data area for use by this specific instance

of the function.

CTask Manual          Version 1.1  88-07-01                Page 12

Events

Tasks alone would be of limited use.  If you have several routines

which  just do number crunching or sorting or  such,  making  them

into  parallel  tasks would be sensible only on  a  multiprocessor

system.  Normally,  at least some of your tasks will wait for some

outside  "event" to happen,  be it the user pressing a key,  or  a

character  arriving from the modem.  Then there may be tasks which

have to wait until another task finishes processing on some  piece

of data before they can continue.  For this synchronization, there

are  a number of constructs in CTask,  which I summarize under the

name "event".  Common to all events are the operations of  waiting

for  an  event  to  happen,  and signalling  that  the  event  has

happened.  Using a CTask event is much more efficient than looping

while  waiting on some shared data location to change  state,  and

also  eliminates  concurrency problems inherent in such  a  simple

approach.  Tasks  waiting for an event are taken off the scheduler

queue, so they no longer use processor time.

Reentrancy

One of the biggest problem with multitasking in general,  and C on

the PC in particular, is reentrancy. Reentrancy means that you can

use  a routine,  be it you own,  or one of the C run-time library,

from different tasks at the same time. When writing your own code,

you can easily avoid problems, but when using the run-time library

routines,  you often can only guess if the routines are  reentrant

or not.

A routine is NOT reentrant if it modifies static data. This can be

illustrated by the following nonsense example:

int non_reentrant (int val)

{    static int temp;

temp = val;

return temp * 2;

}

Now take two tasks,  which call this routine.  Task1 calls it with

val=3,  Task2  with val=7.  What will be the return value for both

tasks? You never know. There are three possible outcomes:

1) The tasks execute sequentially.  Task1 will get 6,  and Task2

14 as a result. This is what one normally expects.

2) Task1  runs  up to "temp = val",  then is interrupted by  the

timer.  Task2  executes,  and gets 14 as result.  Then  Task1

continues. Return for Task1 is 14.

3) Task2  runs up to "temp = val",  then is interrupted  by  the

timer.  Task1  executes,  and  gets 6 as result.  Then  Task2

continues. Return for Task2 is 6.

CTask Manual          Version 1.1  88-07-01                Page 13

add  to  this the effects of optimization,  and a  loop,  and  the

outcome is completely random.

Most  routines in the C library will not explicitly  do  something

like this, but all functions that

- do file I/O (read, write, printf, scanf, etc.) or

- change memory allocation (malloc, free, etc.)

have  to  use some static data to do buffering,  or to  store  the

chain of memory blocks in. Interrupting such an operation may have

disastrous effects.  The most devilish aspect of non-reentrancy is

that  the  effects are unpredictable.  Your program may  run  1000

times  without  any error,  and then on the 1001th time crash  the

system so completely that only the big red switch will help.

So  what can you do about it?  A lot.  There are several  ways  to

protect  "critical  regions" from being entered in  parallel.  The

most simple method is to disable interrupts. This, however, should

be  used only for *very* short periods of time,  and not for  rou-

tines  that  might themselves re-enable them  (like  file-I/O).  A

better  method  is to temporarily  disable  task  preemption.  The

routines  tsk_dis_preempt  and tsk_ena_preempt allow this form  of

short-term task switch disable. Interrupts may still be processed,

but  tasks  will  not  be preempted.  The best way  is  to  use  a

"resource".  A resource is a special kind of event, which only one

task  can  possess  at  a time.  Requesting  the  resource  before

entering the routine,  and releasing it afterwards,  will  protect

you  from  any  other task simultaneously  entering  the  critical

region (assuming that this task also requests the resource).

It  is also reasonably safe to use file-I/O without protection  if

it  goes  to  different  files.  The C  file  control  blocks  for

different files are distinct, so there will be no conflict between

tasks.  Since  DOS  access is automatically  protected  by  CTask,

concurrent file I/O is possible.

What  you  may NEVER do is to use a non-reentrant routine that  is

not  protected by an interrupt disable from an interrupt  handler.

An interrupt handler is not a task,  and so can not safely request

a resource or disable task preemption.  This is the reason why the

CTask  routines  generally disable interrupts before  manipulating

the internal queues rather than only disabling task preemption.

Deadlocks

One  thing to watch out for when using resources or similar  event

mechanisms is not to get into a situation where Task 1 waits for a

resource that Task 2 has requested,  while at the same time Task 2

waits  for a resource that Task 1 already has.  This situation  is

called a deadlock (or,  more picturesque,  deadly embrace), and it

can only be resolved with outside help (e.g.  waking up one of the

tasks forcibly). To illustrate, consider the following example:

CTask Manual          Version 1.1  88-07-01                Page 14

void far task_1 ()

{

...

request_resource (&rsc1, 0L);

request_resource (&rsc2, 0L);

...

}

void far task_2 ()

{

...

request_resource (&rsc2, 0L);

request_resource (&rsc1, 0L);

...

}

Since interrupts are always enabled on return from a task  switch,

even if the statements are enclosed in a critical region, there is

no  guarantee  that  the request_resource calls will  be  executed

without interruption. In this example, the problem is obvious, but

in  a more complex application,  where resource requests or  other

waits might be buried in some nested routine, you should watch out

for similar situations. One way to avoid problems would be in this

example to change task_2 to

void far task_2 ()

{

int again;

...

do {

request_resource (&rsc2, 0L);

if (again = c_request_resource (&rsc1))

{

release_resource (&rsc2);

delay (2L);

}

} while (again);

...

}

Note that this is only one of many possible approaches,  and  that

this approach favors task_1 over task_2.

You  should also take care not to kill tasks that currently own  a

resource. CTask will not detect this, and the resource will  never

be freed.

CTask Manual          Version 1.1  88-07-01                Page 15

Multitasking and DOS

CTask includes (and automatically installs) a routine which  traps

all  DOS  calls,  and makes sure that no two tasks  simultaneously

enter DOS. This is accomplished using the resource mechanism, with

special  provisions  for  the  limited  multitasking  capabilities

provided by DOS. There are a few calls, namely those with function

codes  <=  0x0c,  which allow functions with codes >  0x0c  to  be

executed  while  DOS is waiting for an external device  (generally

the keyboard) to get ready.  This, however, limits the use of some

C library functions,  namely scanf and fread,  for console  input.

Both  these  functions  use  handle input,  and thus  can  not  be

interrupted.  When  writing  routines  for  handling  user  input,

keyboard  read  functions should either use  the  low-level  calls

getch  and  gets,  or,  better yet,  the direct entries  into  the

keyboard  handler,  t_read_key and t_keyhit,  and then process the

string with sscanf if desired.  The keyboard handler (contained in

tskkbd.asm) traps all keyboard interrupts, storing entered keys in

a  pipe.  If a task reads from the keyboard,  it is  automatically

waiting  for  this pipe.  Using getch and gets is  less  desirable

since they use polling instead of waiting, and thus degrade system

performance.  Also,  the t_read_key and t_keyhit functions do  not

use DOS, so DOS functions <= 0C can be executed concurrently.

You  should  NEVER  circumvent DOS by  calling  the  BIOS-disk-I/O

function (INT 13) directly.  This entry is not protected by CTask,

and  using it in parallel to DOS file-I/O may send your  hard-disk

FAT and directory into never-never land. If you really should need

this  interrupt,  you should consider adding support for it in the

DOS-handler module "tskdos.asm".  Using the direct sector read and

write interrupts 25 and 26 is supported, however. Using other BIOS

interrupts  directly  should  also  be  avoided,  unless  you  are

absolutely  sure they will not be used by other routines via  DOS,

or you provide your own critical region handling.  Using interrupt

16, the keyboard interrupt, is safe, since it is redirected to the

keyboard handler pipe.

The DOS access module has been tested to work with DOS  3.30,  and

with the DOS 3.30 PRINT routine running in the background. Special

provisions  are built into the DOS module to detect background DOS

calls.  Using Sidekick also hasn't lead to any problems,  although

you  may trash Sidekick's screen display by writing to the  screen

while  Sidekick is active (note that your application  *continues*

to  run  while Sidekick or other pop-ups are active).  I  can  not

guarantee  complete compatibility with all background  and  pop-up

programs  under all versions of DOS.  Specifically,  DOS  versions

prior   to  3.1  have  significant  problems  with   multitasking.

Upgrading  to a newer DOS version is recommended if you are  still

using DOS < 3.2 (DOS 3.1 has some bugs in other areas).

Critical  errors  and  Control C occurring  while  concurrent  DOS

access  takes  place may also be fatal.  Using the  ctrlbrk()  and

dosexterr() functions of Turbo C, or their equivalents in MS C, to

trap critical errors and Control C is highly recommended.

CTask Manual          Version 1.1  88-07-01                Page 16

Using CTask

===========

CTask  comes  archived with both source and binaries.  The  binary

version is compiled in the large model, but since the pre-compiled

kernel  routines don't use any functions from the C  library,  you

can  use  all functions in small or other model  programs  (except

Tiny). The include files provided specify all model  dependencies,

so you don't have to use the large model for your application, but

always remember to include "tsk.h" for the type and routine proto-

types.  The  C  source files will work without  changes  for  both

Microsoft  and Turbo C. The library files are not  compatible,  so

use "ctaskms.lib" for Microsoft, and "ctasktc.lib" for Turbo C. In

the distributed configuration (i.e. dynamic allocation of  control

blocks enabled), the file TSKALLOC.C must be added to the  library

or separately linked after compiling it *in the same model as  the

main  program*. This file uses C-library routines, and  thus  must

match  the  main  program's  memory  model.  The  same  goes   for

TSKSNAP.C, an optional snapshot-dump utility.

Configuration Options

The  file TSKCONF.H contains a number of #define's that allow  you

to configure some CTask features. The entries are

TSK_DYNAMIC

If  1,  you can let CTask dynamically create task  and  event

control  blocks,  task stacks, and pipe buffers,  by  passing

NULL as the block address. Since this requires the C  runtime

allocation calls, it is not suitable for non-DOS applications

(except if you provide your own memory allocation routines).

This option is normally enabled (1).

Changing  this  option requires  changing  the  corresponding

option in the Assembler macro file TSK.MAC.

TSK_NAMEPAR

If  1, all create_xxx calls accept an  additional  parameter,

the name of the created control block. This name should  con-

tain  only  printeable characters, and should not  be  longer

than  8 characters plus the zero terminator. This  option  is

used together with TSK_NAMED (see below).

This option is normally enabled (1).

Changing  this  option requires  changing  the  corresponding

option in the Assembler macro file TSK.MAC.

TSK_NAMED

If  1, all control blocks (except timer control  blocks)  are

named  and linked on creation. This allows the snapshot  dump

CTask Manual          Version 1.1  88-07-01                Page 17

routine  to  display the system state.  TSK_NAMEPAR  must  be

defined for this option to work. Since it may be desirable to

switch  off  the handling of the names  after  the  debugging

phase, without having to change all create_xxx calls to  omit

the  name  parameter,  the  name  parameter  is  ignored   if

TSK_NAMED is undefined, but TSK_NAMEPAR is defined.

This option is normally enabled (1).

Changing  this  option requires  changing  the  corresponding

option in the Assembler macro file TSK.MAC.

CLOCK_MSEC

If  1, all timeouts are specified in milliseconds instead  of

timer  ticks. This allows programming of delays and  timeouts

independent of the speedup-parameter or the system tick rate.

Since  timeout calculations use floating point operations  if

this  option is enabled, a floating point library is  needed.

This precludes model independence of the CTask kernel.

This option is normally disabled (0).

PRI_TIMER

Specifies the priority of the timer task. Normally the  timer

task should have a higher priority than any other task in the

system.

The value is normally 0xf000.

PRI_STD

This  value  may  be used in your programs  as  the  standard

priority  of  user tasks. Its value is arbitrary. It  is  not

used in the kernel except for the definition of PRI_INT9 (see

below).

The value is normally 100 (decimal).

PRI_INT9

Determines the priority of the "int9"-task. This task  chains

to  the previous interrupt vector for the sytem  timer  tick,

and  thus may activate resident TSR's. Since  TSR's  normally

use  polling when accessing the keyboard and  other  devices,

the  priority  of the int9-task should be equal to  or  lower

than normal user-defined tasks to allow your program to  con-

tinue to run while the TSR is active. You can tune this value

so  that some tasks are blocked by the TSR to avoid  trashing

the screen.

The value is normally  PRI_STD - 1.

CTask Manual          Version 1.1  88-07-01                Page 18

IBM

DOS

Both IBM and DOS are more or less of informative value  only,

to  point out those areas in the kernel that  need  attention

when converting to non-IBM or non-DOS environments. Disabling

one or both of this options requires the substitution of your

own routines for installation and keyboard handling.

Both options must normally be enabled (1).

AT_BIOS

If enabled, the AT BIOS wait/post handler is installed.

May be disabled when compatibility problems arise.

This option is normally enabled (1).

Memory Allocation

TSKALLOC.C  is  needed if TSK_DYNAMIC is enabled in  tskconf.h  to

handle  the allocation and free calls. If you want to use  dynamic

allocation  in your own tasks, you should also use  the  functions

tsk_alloc and tsk_free to avoid the reentrancy problems  mentioned

in the introduction. If you should need other forms of alloc (like

calloc)  the  recommended  way  is  to  add  those  functions   to

TSKALLOC.C, requesting the resource alloc_resource before  calling

the C memory-allocation function.

Snapshot

TSKSNAP.C is only needed if you want to include the snapshot  dump

into  your  program. Note that you can *not* use snapshot  if  you

disable TSK_NAMED in tskconf.h.

Task Stacks

When  compiling your application,  turn stack  checking  off.  The

standard  stack check is of little use with task stacks,  and  may

interfere with CTask's operation.  The stack area for tasks should

be  allocated  on the main program's stack,  not in the static  or

heap data space. The reason for this is that some of the C library

routines check for stack overflow regardless of your  compile-time

switches,  and  will  crash  your application if  you  use  stacks

outside the normal stack. The stack allocation parameter with LINK

(MS-C),  or the _stacksize variable (Turbo C) have to be increased

to reflect the additional stack space. When calculating task stack

sizes, keep in mind that library routines (esp. printf) allocate a

lot of space on the stack for temporary variables. A minimum of 1k

for  tasks using library routines is recommended,  2k puts you  on

the  safe  side.  Tasks  not using C library routines  may  use  a

smaller  stack,  about 256 bytes at a minimum,  plus space for any

CTask Manual          Version 1.1  88-07-01                Page 19

local variables and nested routines. Then add up all task  stacks,

and   add   space  for  the  main  task  (the   function   calling

install_tasker), with this size also dependent on what you will do

in  the main task while CTask is active. Stacks for tasks that  do

not use C library routines may be allocated anywhere.

Drivers

The  keyboard  and DOS handlers are always installed  with  CTask.

Using the serial I/O and printer drivers is optional,  so you have

to  install them separately,  but only *after*  installing  CTask.

When  using  the serial driver,  include "sio.h" in your  modules,

when using the printer driver, include "prt.h".

Another  driver  that  is  automatically  installed  is  the  BIOS

wait/post  handler  for  the IBM AT. The  AT  BIOS  contains  some

multitasking  hooks  to  avoid busy waiting in the  BIOS.  If  you

experience  problems with disk accesses or printer output  through

BIOS  (not through the CTask printer driver), you  should  disable

installation  of  this  driver  by  setting  AT_BIOS  to  zero  in

tskconf.h.  Normally,  no problems should arise with  this  driver

even in XT type machines.

Things to remember

Remember that tasks are not automatically started after  creation.

Use start_task to allow a created task to run.

Always use create_xxx on resources,  pipes,  etc.  Using the event

routines without doing so will have unpredictable results.

Before exiting the program, all installed drivers and CTask should

be  explicitly  removed.  Although  the  DOS  handler  traps   the

terminate  call and automatically calls remove_tasker, you  should

make  sure  that  all  tasks  are  completed  properly,  and  call

remove_tasker yourself.

Deleting events before exiting the program is not  mandatory,  but

recommended to kill all tasks waiting for the event. You should be

careful  not  to  kill tasks while they are  active  in  DOS.  The

kill_task routine should be reserved for fatal error handling. The

best  way  is to let the tasks kill themselves depending  on  some

global  variable or event.  If a task is killed while waiting  for

input  in DOS, DOS operation may be severely impaired. If you  use

the C console input routines, make sure that the task returns from

DOS  before it is killed, if necessary by requesting the  user  to

press a key.

CTask Manual          Version 1.1  88-07-01                Page 20

Priority Handling

=================

CTask   provides   for  prioritized  task  execution   and   event

processing. This means that a task that has a higher priority will

be  run  before any other tasks having  lower  priority.  Also,  a

higher  priority  task will gain access  to  resources,  counters,

pipes,   and  mail,   before  lower  priority  tasks.  With  fixed

priorities,  this  means that a high priority task can  monopolize

CPU  time,  even  if  it  calls  schedule()  explicitly.  Variable

priority  increases each eligible task's priority by one  on  each

scheduler  call,  so that lower priority tasks will slowly rise to

the head of the queue until they get executed.  The priority  will

be reset to the initial priority when a task is run.

Since  variable priority increases processing time in the critical

region  of  the scheduler,  it is not recommended for  systems  in

which  a  larger  number  of tasks  is  expected  to  be  eligible

simultaneously.

Usually,  all  tasks  in a system should have the  same  priority,

with   only  very  few  exceptions  for  non-critical   background

processing  (low  priority)  or  very  time-critical  tasks  (high

priority).  High  priority  tasks should be written in such a  way

that  they  either  reduce  their  priority  when  processing   is

completed,  or that they wait for an event. Busy waiting in a high

priority  task will severely impair system operation with variable

priority  enabled,  and  will stop the system until  the  task  is

placed in a waiting state with fixed priority.

The  (automatically created) main task is started with the highest

possible priority below the timer task, so that it can process all

initialisations  before other tasks start running.  To  allow  the

system to operate,  the priority of the main task must be reduced,

or the main task must wait for an event or a timeout.

CTask Manual          Version 1.1  88-07-01                Page 21

CTask Data Types

================

Note  that you do not have to know the innards of the  structures.

All  structure fields are filled by the "create_xxx" routines  and

modified  by the CTask functions.  You should NEVER modify a field

in  one of the structures directly.  The structures are  explained

here shortly only for those wanting to modify the routines.

If  you only want to use the routines,  you should simply  include

the file "tsk.h" in your source, and define variables of the types

tcb         - for task control blocks

tcbptr      - for far pointers to tcbs

flag        - for flag events

flagptr     - for far pointers to flags

resource    - for resource events

resourceptr - for far pointers to resources

counter     - for counter events

counterptr  - for far pointers to counters

mailbox     - for mailbox events

mailboxptr  - for far pointers to mailboxes

pipe        - for pipe events

pipeptr     - for far pointers to pipes

wpipe       - for word pipe events

wpipeptr    - for far pointers to word pipes

buffer      - for buffer events

bufferptr   - for far pointers to buffers

tlink       - for timeout control blocks

tlinkptr    - for far pointers to timeout control blocks

namerec     - for control block names

nameptr     - for name pointers

without caring what's behind them.

Additionally, you may use the types

byte        - for unsigned characters

word        - for unsigned short integers

dword       - for unsigned long integers

funcptr     - for pointers to task functions

farptr      - for far pointers to anything

byteptr     - for far pointers to byte arrays

wordptr     - for far pointers to word arrays

CTask Manual          Version 1.1  88-07-01                Page 22

in  defining  or typecasting items to be passed as  parameters  to

CTask functions.

Typedefs used for simplified type specifications

typedef unsigned char byte;

typedef unsigned short word;

typedef unsigned long dword;

typedef void (cdecl far *funcptr)();

typedef void far *farptr;

typedef byte far *byteptr;

typedef word far *wordptr;

Error return values for mailbox functions

#define TTIMEOUT ((farptr) -1L)

This value is returned if a timeout occurred during a mailbox

wait.

#define TWAKE    ((farptr) -2L)

This  value  is returned if the task was waked  up  during  a

mailbox wait.

The timer control block

The  timer control block is included in every task control  block.

It  may  also be created as a separate entity to  specify  special

actions  to be executed after or every n timer ticks.  The  "next"

field links the active structures into the timer queue.

"timeout"  gives the number of timer ticks, "reload" is  used  for

repetitive  timeout actions to reload the original timeout  value.

"strucp"  points  to  the  structure to  be  acted  upon,  "tkind"

specifies the kind of structure:

#define  TKIND_TASK        1

Strucp  points  to the task control block of  which  the

element is a member. The task will be awakened when  the

timeout expires.

#define  TKIND_WAKE        2

Strucp points to a task control block. Otherwise same as

TKIND_TASK.

#define  TKIND_PROC        3

Strucp  contains the address of a function to be  called

on timeout.

#define  TKIND_FLAG        4

Strucp points to a flag that should be set on timeout.

CTask Manual          Version 1.1  88-07-01                Page 23

#define  TKIND_COUNTER     5

Strucp  points to a counter that should be increased  on

timeout.

#define  TKIND_TEMP        0x80

This  is  a modifying flag. If set,  the  timer  control

block was allocated dynamically, and should be freed  on

timeout.

"tstate" contains the state of the element:

#define  TSTAT_REMOVE      -1

The  element should be taken off the queue on  the  next

pass (if the TKIND_TEMP-flag is set, it will be freed).

#define  TSTAT_IDLE        0

The element is not enqueued.

#define  TSTAT_COUNTDOWN   1

The  timeout counter is counted down. If the timeout  is

reached, the element is removed from the queue.

#define  TSTAT_REPEAT      2

The  timeout counter is counted down. If the timeout  is

reached, the timeout count is reloaded.

typedef struct tlink_rec far *tlinkptr;

struct tlink_rec {

tlinkptr next;

dword    timeout;

dword    reload;

farptr   strucp;

byte     tstate;

byte     tkind;

};

typedef struct tlink_rec tlink;

The name link structure

If  TSK_NAMED is enabled, all structures except the timer  control

block contain a name link. All control blocks are linked and named

via  this  element.  To facilitate removal from the  list,  it  is

doubly  linked  via  the pointers "follow"  and  "prev".

"strucp"  points to the head of the structure the name link is  an

element of, with "nkind" specifying the type of structure:

CTask Manual          Version 1.1  88-07-01                Page 24

#define  TYP_TCB        1     task control block

#define  TYP_FLAG       2     flag event

#define  TYP_RESOURCE   3     resource event

#define  TYP_COUNTER    4     counter event

#define  TYP_MAILBOX    5     mailbox event

#define  TYP_PIPE       6     byte pipe

#define  TYP_WPIPE      7     word pipe

#define  TYP_BUFFER     8     buffer

The head element of the name list has its nkind field set to zero.

The  "name" field contains an up to 8-character name plus  a  zero

terminator.

#define  NAMELENGTH     9

typedef struct name_rec far *nameptr;

struct name_rec {

nameptr    follow;

nameptr    prev;

farptr     strucp;

byte       nkind;

char       name [NAMELENGTH];

};

typedef struct name_rec namerec;

The task control block structure

The  "next" field points to the next tcb in a queue.  The  "queue"

pointer  points to the head of the queue the task is enqueued  in,

or will be enqueued in on the next schedule request in the case of

the current running task.

"stack" contains the saved task stack pointer (offset and segment)

if the task is not running. The field "stkbot" contains the bottom

address  of  the task stack.  It is set  by  create_task,  but  is

currently  not used anywhere else.  Stack checking routines  might

use this value to test for stack overflow and/or stack usage.

"prior"  contains  the tasks current  priority,  with  0xffff  the

highest possible priority, and 0 the lowest. "initprior" initially

contains the same value.  If variable priority is enabled, "prior"

is  incremented on each scheduler call,  and reset to  "initprior"

when the task is activated.

"state" and "flags" contain the tasks state and flags.

"timerq" is a tlink structure used to chain the tcb into the timer

queue,  if  the  task is waiting for a timeout. See  above  for  a

description of the tlink structure.

CTask Manual          Version 1.1  88-07-01                Page 25

The fields "retptr" and "retsize" are used in event handling. They

are  used  when a task is waiting for an event by the  task  acti-

vating the event,  and also by timeout and wake to indicate  error

returns.   The  use of these pointers eliminates the need to  loop

for  an  event,  which  requires slightly more code in  the  event

handling routines, but reduces the need for task switching.

The  "name"  namerec  structure is present only  if  TSK_NAMED  is

enabled.  It  may  be  used in debugging  (see  tsksnap.c  for  an

example),  and in applications where the address of the  structure

can not be passed directly.

typedef struct tcb_rec far *tcbptr;

typedef tcbptr far *tqueptr;

struct tcb_rec {

tcbptr   next;

tqueptr  queue;

byteptr  stack;

byteptr  stkbot;

word     prior;

word     initprior;

byte     state;

byte     flags;

dlink    timerq;

farptr   retptr;

int      retsize;

};

typedef struct tcb_rec tcb;

Task states

#define  ST_KILLED   0

#define  ST_STOPPED  1

#define  ST_DELAYED  2

#define  ST_WAITING  3

#define  ST_ELIGIBLE 4

#define  ST_RUNNING  5

CTask Manual          Version 1.1  88-07-01                Page 26

Possible task states and queue association

ST_KILLED      The  task has been killed.  Restarting the task  is

not possible. Queue pointers are invalid.

ST_STOPPED     The  task is not enqueued in any queue.  To be  in-

cluded  in  scheduling,  it  has to  be  explicitly

started. The queue head pointer is NULL.

ST_DELAYED     The task is enqueued in the timer queue only.  When

the  timer expires,  it is placed in  the  eligible

queue.  The queue head pointer is NULL.

ST_ELIGIBLE    The  task  is  enqueued in the queue  of  processes

eligible for running.  It can not be chained in the

timer queue.  The queue head pointer points to  the

eligible queue.

ST_RUNNING     The task is the current running process.   Although

it  is  not enqueued in any queue,  the queue  head

pointer in its control block is valid and points to

the  queue the process will be enqueued in  on  the

next schedule request.

ST_WAITING     The task is waiting for an event to happen.  It can

also  be chained into the timer queue if a  timeout

was specified in the call.   The queue head pointer

points to the queue head in the event control block

for the event the process is waiting on.

Possible state transitions and their reasons

stopped  -> eligible    by start_task ()

delayed  -> killed      by kill_task ()

-> eligible    by timer task, or wake_task ()

eligible -> killed      by kill_task ()

-> running     by scheduler

running  -> killed      by kill_task ()

-> stopped     by delay (0)

-> delayed     by delay (n != 0)

-> eligible    by scheduler

-> waiting     by wait_xxx ()

waiting  -> killed      by kill_task ()

-> eligible    by event happening, timeout,

or wake_task()

CTask Manual          Version 1.1  88-07-01                Page 27

Task flags

System flags:

#define  F_TEMP   0x80

This tcb was allocated automatically, and must be free'd

on task kill.

#define  F_STTEMP 0x40

The tasks stack was allocated automatically, and must be

free'd on task kill.

User changeable flags:

#define  F_CRIT   0x01

This  task  may not be preempted. It will run  until  it

clears  this  flag, delays itself, calls  the  scheduler

explicitly, or waits for an event.

The event control blocks

All event control blocks have two optional fields:

"name"  is only present if TSK_NAMED is enabled. It is  a  namerec

structure as explained above.

"flags" is only present if TSK_DYNAMIC is enabled. It contains

F_TEMP    If this control block was allocated  automatically,

and must be free'd on delete.

F_STTEMP  If  the buffer for this event was  allocated  auto-

matically, and must be free'd on delete (pipes  and

buffers only).

CTask Manual          Version 1.1  88-07-01                Page 28

The flag event structure

Contains  two queues for processes waiting on a flag state  (clear

or set), plus the flag state (0 = clear, 1 = set).

typedef struct {

tcbptr  wait_set;

tcbptr  wait_clear;

int     state;

byte    flags;   (if TSK_DYNAMIC)

namerec name;    (if TSK_NAMED)

} flag;

typedef flag far *flagptr;

The counter event structure

Similar to a flag, but contains a doubleword state counter.

typedef struct {

tcbptr  wait_set;

tcbptr  wait_clear;

dword   state;

byte    flags;   (if TSK_DYNAMIC)

namerec name;    (if TSK_NAMED)

} counter;

typedef counter far *counterptr;

The resource event structure

Contains a queue for the tasks waiting for access to the resource,

a  pointer  to  the current owner of the resource  (to  check  for

illegal "release_resource" calls),  and the resource state (0 = in

use, 1 = free).

typedef struct {

tcbptr   waiting;

tcbptr   owner;

int      state;

byte     flags;   (if TSK_DYNAMIC)

namerec  name;    (if TSK_NAMED)

} resource;

typedef resource far *resourceptr;

CTask Manual          Version 1.1  88-07-01                Page 29

The mailbox event structure

The msgptr type is only used internally to chain mail blocks  into

the  mailbox.  The  mailbox  type contains a queue  of  the  tasks

waiting  for mail,  and a first and last pointer for the chain  of

mail blocks.

struct msg_header {

struct msg_header far *next;

};

typedef struct msg_header far *msgptr;

typedef struct {

tcbptr  waiting;

msgptr  mail_first;

msgptr  mail_last;

byte    flags;   (if TSK_DYNAMIC)

namerec name;    (if TSK_NAMED)

} mailbox;

typedef mailbox far *mailboxptr;

The pipe and word pipe event structure

Contains queues of the tasks waiting to read or write to the pipe,

indices  for reading (outptr) and writing (inptr) into the buffer,

the  buffer size,  the number of bytes or words currently  in  the

pipe  ("filled"),  and the pointer to the buffer.  A word pipe  is

handled exactly the same as a byte pipe, the only difference being

the element size placed in the buffer.  With normal pipes, charac-

ters are buffered, with word pipes, words.

typedef struct {

tcbptr   wait_read;

tcbptr   wait_write;

tcbptr   wait_clear;

word     bufsize;

word     filled;

word     inptr;

word     outptr;

byteptr  contents;

byte    flags;   (if TSK_DYNAMIC)

namerec name;    (if TSK_NAMED)

} pipe;

typedef pipe far *pipeptr;

CTask Manual          Version 1.1  88-07-01                Page 30

typedef struct {

tcbptr   wait_read;

tcbptr   wait_write;

tcbptr   wait_clear;

word     bufsize;

word     filled;

word     inptr;

word     outptr;

wordptr  wcontents;

byte    flags;   (if TSK_DYNAMIC)

namerec name;    (if TSK_NAMED)

} wpipe;

typedef wpipe far *wpipeptr;

The buffer event structure

Contains  resources for read and write access, the word pipe  used

for buffering, and a message counter.

typedef struct {

resource    buf_write;

resource    buf_read;

wpipe       pip;

word        msgcnt;

byte        flags;   (if TSK_DYNAMIC)

namerec     name;    (if TSK_NAMED)

} buffer;

typedef buffer far *bufferptr;

NOTE:  When  modifying CTask structures,  take care to modify  the

equivalent  definitions  in the assembler include file  "tsk.mac".

Some  of  the assembler routines have to use  field  offsets  into

pointers,  so  having  different offsets in C and  assembler  will

crash the system.

CTask Manual          Version 1.1  88-07-01                Page 31

CTask Routines

==============

Installation and Removal

void install_tasker (int varpri, int speedup);

Installs  the multitasker.  Must be called prior to any other

routine. The calling routine is defined as the main task, and

assigned  the  highest priority.   To allow  other  tasks  to

execute,  the  main task must have its priority  reduced,  be

delayed,  or  wait  on an event.

The "varpri" parameter enables variable priority if nonzero.

The  "speedup" parameter is defined for the IBM  PC/XT/AT  as

the clock tick speedup factor. The timer tick frequency  will

be set to

speedup  ticks/sec   msecs/tick

0        18.2        54.9     (normal clock)

1        36.4        27.5

2        72.8        13.7

3       145.6         6.9

4       291.3         3.4

Note  that  all  timeouts are specified  in  tick  units,  so

changing  the speedup parameter will influence  timeouts  and

delays.  You  can enable CLOCK_MSECS in  tskconf.h  to  allow

timeouts to be specified in milliseconds.

The system clock will not be disturbed by changing the speed.

Using values above 2 can lead to interrupt overruns on slower

machines and is not recommended.

void remove_tasker (void);

Uninstalls the multitasker. Must only be called from the main

task, and should be called before exiting the program.

Miscellaneous

void preempt_off (void);

Disables task preemption. This is the default after installa-

tion.  With preemption turned off,  only delays, event waits,

and explicit scheduler calls will cause a task switch.

CTask Manual          Version 1.1  88-07-01                Page 32

void preempt_on (void);

Enables  task preemption.  Task switches will occur on  timer

ticks.

void far schedule (void);

Explicit  scheduling request.  The highest priority  eligible

task will be made running.

void far c_schedule (void);

Conditional  scheduling request.  Scheduling will take  place

only if preemption is allowed.

void tsk_dis_preempt (void)

Temporarily  disable task preemption.  Preemption will be re-

enabled  by  tsk_ena_preempt or  an  unconditional  scheduler

call.

void tsk_ena_preempt (void)

Re-enable  task  preemption.  Note that  tsk_dis_preempt  and

tsk_ena_preempt do not change the global preemption state set

by preempt_off and preempt_on.

int tsk_dis_int (void)

Disable  interrupts.  Returns the state of the interrupt flag

prior to this call (1 if interrupts were enabled).

void tsk_ena_int (int state)

Enables  interrupts if "state" is nonzero.  Normally used  in

conjunction with tsk_dis_int.

The routines tsk_dis_int and tsk_ena_int may be used in a  simpli-

fied scheme with the defines

CRITICAL;      Declares "int crit_intsav;".

C_ENTER;       Expands to "crit_intsav = tsk_dis_int ();"

C_LEAVE;       Expands to "tsk_ena_int (crit_intsav);".

void tsk_cli (void)

Disables interrupts (intrinsic function).

CTask Manual          Version 1.1  88-07-01                Page 33

void tsk_sti (void)

Unconditionally enables interrupts (intrinsic function).

void tsk_outp (int port, byte b)

Outputs  the  value "b" to  hardware-port  "port"  (intrinsic

function).

byte tsk_inp (int port)

Returns the value read from port "port" (intrinsic function).

The following entry points may be used from assembler routines:

extrn scheduler: far

Direct entry into the scheduler.  The stack must be set up as

for an interrupt handler, e.g.

pushf

cli

call  scheduler

extrn _sched_int: far

Conditional scheduling call.  The stack must be set up as for

an interrupt handler.

CTask Manual          Version 1.1  88-07-01                Page 34

Task Operations

tcbptr create_task (tcbptr task, funcptr func, byteptr stack,

word stksz, word prior, farptr arg

[, byteptr name]);

Initialises a task.  Must be called prior to any other opera-

tions  on this task.  The task is in the stopped state  after

creation. It must be started to be able to run.

"task"    is  a  pointer to a tcb (NULL for  automatic  allo-

cation).

"func"    is a pointer to a void far function, with a single

dword sized parameter.

"stack"   is a pointer to a stack area for the task (NULL for

automatic allocation).

"stksz"   is the size of the stack area in bytes.

"prior"   is the tasks priority (0 lowest, 0xffff highest).

"arg"     is an argument to the created task. It may be used

to differentiate between tasks when using the same

function in different tasks.

"name"    is  the name of the task,  up to eight  characters,

plus  zero-terminator.  This  parameter is  defined

only if TSK_NAMEPAR is enabled in tskconf.h.

void kill_task (tcbptr task);

Kills a task. The task can no longer be used.

int start_task (tcbptr task);

Starts a task, i.e. makes it eligible for running.

Returns  0  if  task  was started,  -1 if the  task  was  not

stopped.

A value of NULL for "task" will start the "main task".

int wake_task (tcbptr task);

Prematurely wakes a delayed or waiting task.

If the task was waiting for an event, it will be removed from

the  waiting queue,  with the operation terminating  with  an

error return value.

Returns  0 if task was waked,  -1 if the task was not delayed

or waiting.

A value of NULL for "task" will wake the "main task".

CTask Manual          Version 1.1  88-07-01                Page 35

void set_priority (tcbptr task, word prior)

Sets the priority of the specified task to "prior".

Note that you should NOT modify the priority field in the tcb

structure directly.

A value of NULL for "task" will set the priority of the "main

task".

void set_task_flags (tcbptr task, byte flags)

Sets the flags of the task to "flags".  Currently,  the  only

flag that can be changed is

F_CRIT   the task can not be preempted if set.

Note  that  you  may  NOT modify the flag field  in  the  tcb

structure directly.

A  value of NULL for "task" will set the flags of  the  "main

task".

Timer Operations

Timeouts are normally specified in timer ticks. If CLOCK_MSECS  is

enabled, timeouts will be converted to the nearest number of timer

ticks  automatically.  The global  word  variable  "ticks_per_sec"

contains  a rough estimate of the number of ticks per second  even

if CLOCK_MSECS is not enabled.

Event wait Timeouts

When  waiting  for  an event,  a timeout may  be  specified.   The

operation will terminate with an error return value if the timeout

is reached before the event occurs.

NOTE  that the timeout parameter is not optional.  To  specify  no

timeout, use the value 0.

Delays

int t_delay (dword ticks);

Delay the current task for "ticks" clock ticks/milliseconds.

If ticks is 0, the task is stopped.

Timed Events

You can create timer control blocks to

- set a flag

- increment a counter

CTask Manual          Version 1.1  88-07-01                Page 36

- wake up a task

- call a function

after  a specified timeout interval.  The operation may optionally

be  repeated.  For calling functions on a timeout,  the  following

must be noted:

Timeout functions should be as short as possible, since they

run at the highest priority.

The  stack and data area is the area of the  timer  task.  Be

careful when referencing variables.

Timeout  functions may not use any functions that could cause

the timer task to be made waiting.

tlinkptr create_timer (tlinkptr elem, dword tout, farptr strucp,

byte kind, byte rept);

Create a timer control block.

Returns the address of the control block, NULL on error.

"elem"    is the control block to initialise (NULL for auto-

matic allocation).

"tout"    specifies the timeout interval.

"strucp"  points  to  the structure to be  used.  This  is  a

flagptr  for  setting  a  flag,  a  counterptr  for

increasing counters, a tcbptr for waking a task, or

a funcptr for calling a function.

"kind"    gives the kind of structure "strucp" points to.  It

must be one of

TKIND_WAKE     for task-wakeup

TKIND_PROC     for function call

TKIND_FLAG     for flag set

TKIND_COUNTER  for counter increment.

"rept"    if  nonzero,  the action will be repeated on  every

"tout" timeout interval.

NOTE: Timer control blocks can not be named.

void delete_timer (tlinkptr elem);

Deletes  a  timer  control block,  and removes  it  from  the

timeout queue.

void change_timer (tlinkptr elem, dword tout, byte rept);

Changes  the  timeout  value and/or the repeat  flag  for  an

existing timer control block. If "tout" is zero, the call has

the same effect as delete_timer.

NOTE:  delete_timer  and change_timer should not be used on  auto-

CTask Manual          Version 1.1  88-07-01                Page 37

matically  allocated timer control blocks.  Since such blocks  are

deallocated   once   the  timeout  expires  (except   for   repeat

operations), the validity of the pointer is not guaranteed.

Event Operations

================

Resources

A Resource is either in use or free, the default state is free.

If a task has requested a resource, its state is in use.

Tasks  requesting  a  resource  while it is in use  will  be  made

waiting. If the resource is released, the highest priority waiting

task is made eligible,  and is assigned the  resource.   Interrupt

handlers    may   not   use   resource   functions   other    than

"check_resource".

resourceptr create_resource (resourceptr rsc [, byteptr name]);

This initialises a resource.  Must be used prior to any other

operations on a resource.

Returns the address of the control block, NULL on error.

"rsc"     is  the resource control block (NULL for  automatic

allocation).

"name"    is   the  name  of  the  resource,   up  to   eight

characters, plus zero-terminator. This parameter is

defined only if TSK_NAMEPAR is enabled in tskconf.h.

void delete_resource (resourceptr rsc);

Calling  this  routine is optional.  It will kill  all  tasks

waiting for the resource.

void release_resource (resourceptr rsc);

Release the resource.  If the task does not own the resource,

the call is ignored.

If  tasks  are waiting,  the highest priority task will  gain

access to the resource.

int request_resource (resourceptr rsc, dword timeout);

Requests the resource.  If it is not available,  the task  is

suspended. A timeout may be specified.

Returns  0  if  resource  was  allocated,  -1  if  a  timeout

occurred, -2 on wake.

CTask Manual          Version 1.1  88-07-01                Page 38

This  call  is  ignored (returns a 0)  if  the  calling  task

already owns the resource.

int c_request_resource (resourceptr rsc);

Requests the resource only if it is available.

Returns 0 if resource was allocated, -1 if unavailable.

int check_resource (resourceptr rsc);

Returns 0 if resource is allocated, 1 if free.

Flags

A Flag can be either on or off, the default state is off (0).

Tasks  can  wait  on either state of the flag.  If  the  state  is

changed,  all  tasks  waiting  for the state  are  made  eligible.

Interrupt  handlers  may use  the  "set_flag",  "clear_flag",  and

"check_flag" functions.

flagptr create_flag (flagptr flg [, byteptr name]);

This  initialises  a flag.  Must be used prior to  any  other

operations on a flag. The state is set to 0.

Returns the address of the control block, NULL on error.

"flg"     is the flag control block (NULL for automatic allo-

cation).

"name"    is  the name of the flag,  up to eight  characters,

plus  zero-terminator.  This  parameter is  defined

only if TSK_NAMEPAR is enabled in tskconf.h.

void delete_flag (flagptr flg);

Calling  this  routine is optional.  It will kill  all  tasks

waiting for the flag.

void set_flag (flagptr flg);

This sets the flag.  All tasks waiting for the set state will

be made eligible for running.

CTask Manual          Version 1.1  88-07-01                Page 39

void clear_flag (flagptr flg);

This  clears the flag.  All tasks waiting for the clear state

will be made eligible for running.

int wait_flag_set (flagptr flg, dword timeout);

Waits for the set state of the flag.  If the flag is not set,

the task is suspended. A timeout may be specified.

Returns 0 if the flag was set, -1 on timeout, -2 on wake.

int wait_flag_clear (flagptr flg, dword timeout);

Waits  for the clear state of the flag.  If the flag  is  not

clear, the task is suspended. A timeout may be specified.

Returns 0 if the flag was cleared, -1 on timeout, -2 on wake.

int clear_flag_wait_set (flagptr flg, dword timeout)

Combines the operations clear_flag and wait_flag_set.

Returns 0 if the flag was set, -1 on timeout, -2 on wake.

int check_flag (flagptr flg);

Returns 0 if flag clear, 1 if set.

Counters

A  Counter can have any value from 0L to 0xffffffffL,  the default

value is 0. Tasks can wait for a counter being zero or non-zero.

If the counter is cleared or decremented to zero,  all tasks  wai-

ting for the zero condition are made eligible.

If  the counter is incremented,  the highest priority task waiting

for non-zero is made eligible,  and the counter is decremented  by

one.

Interrupt handlers may use the "clear_counter", "inc_counter", and

"check_counter" functions.

counterptr create_counter (counterptr cnt [, byteptr name]);

This  initialises a counter.  Must be used prior to any other

operations on a flag. The value is set to 0.

Returns the address of the control block, NULL on error.

"cnt"     is  the  counter control block (NULL for  automatic

allocation).

CTask Manual          Version 1.1  88-07-01                Page 40

"name"    is the name of the counter, up to eight characters,

plus  zero-terminator.  This parameter  is  defined

only if TSK_NAMEPAR is enabled in tskconf.h.

void delete_counter (counterptr cnt);

Calling  this  routine is optional.  It will kill  all  tasks

waiting for the counter.

void clear_counter (counterptr cnt);

Clears  the counter to zero.  All tasks waiting for the  zero

state will be made eligible for running.

int wait_counter_set (counterptr cnt, dword timeout);

Waits for the counter having a nonzero value. If the value is

zero,  the task is suspended.  The value is decremented  when

the task gets access to the counter.  A timeout may be speci-

fied.

Returns  0 if the counter was nonzero,  -1 on timeout,  -2 on

wake.

int wait_counter_clear (counterptr cnt, dword timeout);

Waits  for the counter having a zero value.  If the value  is

nonzero, the task is suspended. A timeout may be specified.

Returns 0 if the counter was zero, -1 on timeout, -2 on wake.

void inc_counter (counterptr cnt);

Increments the counter.  If tasks are waiting for the nonzero

state,  the  highest  priority task is given  access  to  the

counter.

dword check_counter (counterptr cnt);

Returns the current value of the counter.

CTask Manual          Version 1.1  88-07-01                Page 41

Mailboxes

A Mailbox can hold any number of mail blocks.

Tasks  can send mail to a mailbox and wait for mail to  arrive.  A

mail block is assigned to the highest priority waiting task.  Care

must  be  exercised not to re-use a mail block until it  has  been

processed  by the receiving task.   The mail block format is  user

defineable,  with the first doubleword in a block reserved for the

tasking  system.   Interrupt  handlers  may use  the  "send_mail",

"c_wait_mail", and "check_mailbox" functions.

Note  that mailboxes are well suited to provide the mechanism  for

managing a chain of (equally sized) free mail blocks. On initiali-

sation,  all  free  blocks  would be "sent" to  the  manager  box.

Requesting a free block would use "wait_mail",  and freeing a used

block would again "send" it to the manager mailbox.

mailboxptr create_mailbox (mailboxptr box [, byteptr name]);

This initialises a mailbox.  Must be used prior to any  other

operations on a mailbox.

Returns the address of the control block, NULL on error.

"box"     is  the mailbox control block (NULL  for  automatic

allocation).

"name"    is the name of the mailbox, up to eight characters,

plus  zero-terminator.  This  parameter is  defined

only if TSK_NAMEPAR is enabled in tskconf.h.

void delete_mailbox (mailboxptr box);

Calling  this  routine is optional.  It will kill  all  tasks

waiting for mail.

void send_mail (mailboxptr box, farptr msg);

Sends  a message to the specified mailbox.  If tasks are wai-

ting for mail, the highest priority task will get it.

farptr wait_mail (mailboxptr box, dword timeout);

Waits for mail.  If no mail is available, the task is suspen-

ded. A timeout may be specified.

Returns the pointer to the received mail block, TTIMEOUT

(-1) on timeout, TWAKE (-2) on wake.

CTask Manual          Version 1.1  88-07-01                Page 42

farptr c_wait_mail (mailboxptr box);

Reads mail only if mail is available.

Returns  NULL  if there is no mail,  else a  pointer  to  the

received message.

int check_mailbox (mailboxptr box);

Returns 0 if mailbox is empty, 1 otherwise.

Pipes

A Pipe has a buffer to hold character or word sized items.

An  item may be written to a pipe if there is space in the buffer.

Otherwise,  the writing task will be made waiting.  A reading task

will be made waiting if the buffer is empty.   If an item has been

read,  the highest priority task waiting to write to the pipe will

be allowed to write.

Interrupt  handlers  may  only use  pipe  functions  "check_pipe",

"c_write_pipe", and "c_read_pipe".

Note  that  the  values -1 and -2 (0xffff and  0xfffe)  should  be

avoided when writing to word pipes.  These values are used to mark

timeout and wake when reading,  or pipe empty when checking a word

pipe, and thus may lead to erroneous operation of your routines.

pipeptr create_pipe (pipeptr pip, farptr buf, word bufsize

[, byteptr name]);

wpipeptr create_wpipe (wpipeptr pip, farptr buf, word bufsize

[, byteptr name]);

This  initialises  a pipe.  Must be used prior to  any  other

operations on a pipe.  "bufsize" specifies the buffer size in

bytes.  With word pipes,  the buffer size should be divisible

by two.

Returns the address of the control block, NULL on error.

"pip"     is the pipe/wpipe control block (NULL for automatic

allocation).

"name"    is  the name of the pipe,  up to eight  characters,

plus  zero-terminator.  This  parameter is  defined

only if TSK_NAMEPAR is enabled in tskconf.h.

CTask Manual          Version 1.1  88-07-01                Page 43

void delete_pipe (pipeptr pip);

void delete_wpipe (wpipeptr pip);

Calling  this  routine is optional.  It will kill  all  tasks

waiting to read or write messages.

int read_pipe (pipeptr pip, dword timeout);

word read_wpipe (wpipeptr pip, dword timeout);

Read an item from a pipe.  If no item is available,  the task

is suspended. A timeout may be specified.

Returns the item, or -1 on timeout, -2 on wake.

int c_read_pipe (pipeptr pip);

word c_read_wpipe (wpipeptr pip);

Reads an item from a pipe only if one is available.

Returns the received item, or -1 if none is available.

int write_pipe (pipeptr pip, byte ch, dword timeout);

int write_wpipe (wpipeptr pip, word ch, dword timeout);

Writes an item to a pipe.  If the buffer is full, the task is

suspended. A timeout may be specified.

If  tasks are waiting to read,  the item will be assigned  to

the highest priority waiting task.

Returns 0 on success, or -1 on timeout, -2 on wake.

int c_write_pipe (pipeptr pip, byte ch);

int c_write_wpipe (wpipeptr pip, word ch);

Writes an item to a pipe only if enough space is available.

Returns 0 on success, or -1 if no space available.

int wait_pipe_empty (pipeptr pip, dword timeout);

int wait_wpipe_empty (wpipeptr pip, dword timeout);

Waits  for  the pipe to be emptied.  If the pipe  is  already

empty  on  entry,  the task continues to run.  Returns  0  on

empty, -1 on timeout, -2 on wake.

int check_pipe (pipeptr pip);

word check_wpipe (pipeptr pip);

Returns  -1 if the pipe is empty,  else the first item in the

pipe. The item is not removed from the pipe.

CTask Manual          Version 1.1  88-07-01                Page 44

word pipe_free (pipeptr pip);

word wpipe_free (wpipeptr pip);

Returns the number of free items available in the pipe.

Buffers

A Buffer has a buffer to hold message strings.

A  message may be written to a buffer if it fits into the  buffer.

Otherwise,  the writing task will be made waiting.  A reading task

will  be made waiting if the buffer is empty.   If a  message  has

been  read,  the  highest  priority task waiting to write  to  the

buffer  will  be  allowed to write if its message  fits  into  the

available space.

Interrupt  handlers  may  not  use  buffer  functions  other  than

"check_buffer".

The buffer routines are implemented using resources and pipes, and

thus are not part of the true "kernel" routines.

bufferptr create_buffer (bufferptr buf, farptr pbuf, word bufsize

[, byteptr name]);

This  initialises a buffer.  Must be used prior to any  other

operations on a buffer.

The minimum buffer size is the length of the longest expected

message plus two.

Returns the address of the control block, NULL on error.

"buf"     is  the  buffer control block (NULL  for  automatic

allocation).

"name"    is the name of the buffer,  up to eight characters,

plus  zero-terminator.  This  parameter is  defined

only if TSK_NAMEPAR is enabled in tskconf.h.

void delete_buffer (bufferptr buf);

Calling  this  routine is optional.  It will kill  all  tasks

waiting to read or write messages.

CTask Manual          Version 1.1  88-07-01                Page 45

int read_buffer (bufferptr buf, farptr msg, int size,

dword timeout);

Read a message from a buffer. If no message is available, the

task is suspended. A timeout may be specified.

The  message  will  be copied to the  buffer  "buf",  with  a

maximum length of "size" bytes.

Returns the length of the received message, -1 on timeout, -2

on wake.

int c_read_buffer (bufferptr buf, farptr msg, int size);

Reads  a  message  from a buffer only if  one  is  available.

Returns the length of the received message,  or -1 if none is

available.

int write_buffer (bufferptr buf, farptr msg, int size,

dword timeout);

Writes  a message from "msg" with length "size" to a  buffer.

If not enough space for the message is available, the task is

suspended. A timeout may be specified.

If tasks are waiting for messages,  the highest priority task

will get it.

Returns the length of the message, -1 on timeout, -2 on wake,

-3 on error (length < 0 or length > buffer buffer size).

int c_write_buffer (bufferptr buf, farptr msg, int size);

Writes  a  message  to  a buffer  only  if  enough  space  is

available.

Returns the length of the message,  -1 if no space available,

or -3 on error (length < 0 or length > buffer buffer size).

word check_buffer (bufferptr buf);

Returns  the  current number of bytes (not messages)  in  the

buffer, including the length words.

CTask Manual          Version 1.1  88-07-01                Page 46

The Keyboard Handler

====================

word t_read_key (void)

Waits for a key to be entered.  Returns the ASCII-code in the

lower  byte,  and  the  scan-code in the upper  byte,  or  -2

(0xfffe) on wake.

word t_wait_key (dword timeout)

Waits for a key to be entered.  Returns the ASCII-code in the

lower  byte,  and  the scan-code in the  upper  byte,  or  -1

(0xffff) on timeout, -2 (0xfffe) on wake.

word t_keyhit (void)

Returns -1 (0xffff) if no key was entered,  else the value of

the key. The key is not removed.

The Serial I/O handler

======================

The  serial I/O handler provides full duplex interrupt driven  I/O

on the serial ports. Support for COM1 and COM2 is included, adding

other  ports  is possible by changing the source  and/or  defining

ports on-line.

int v24_define_port (int base, byte irq, byte vector)

Defines  a  new COM-port. This routine can only  be  used  if

TSK_DYNAMIC is enabled.

"base" is the port base I/O address.

"irq" is the IRQ-line number (0-7 for XT, 0-15 for AT) the

port uses for interrupts.

"vector" is the interrupt vector number (0-0xff).

The  return  value is the internal port number for  use  with

v24_install. -1 is returned on error (memory full).

CTask Manual          Version 1.1  88-07-01                Page 47

sioptr v24_install (int port, int init,

farptr rcvbuf, word rcvsize,

farptr xmitbuf, word xmitsize);

Installs the handler for the specified port. Currently, ports

0  (COM1) and 1 (COM2) are supported. Both ports may be  used

simultaneously,  the buffer areas for the ports must  not  be

shared.

"rcvbuf" is a word pipe buffer area for received  characters,

with "rcvsize" specifying the buffer size in bytes. Note that

the  buffer  will  hold  rcvsize  /  2  received  characters.

"rcvbuf" may be NULL for automatic allocation.

"xmitbuf"  is  a byte pipe buffer for characters  waiting  for

transmissions, "xmitsize" gives its size in bytes.

"xmitbuf" may be NULL  for automatic allocation.

If  the  port number for v24_install is ORed with  0x80,  the

port  is  *relative*. This means that the entry in  the  BIOS

table for COM-Ports is used to search the tables internal  to

the  driver  for the port information, instead of  using  the

table  entry directly. If the port address cannot  be  found,

the  driver returns with an error code. Note that  ports  are

numbered from 0, so to specify COM1, pass 0x80 as parameter.

If the "init" parameter is non-zero, the port is  initialised

with  the  default  values specified in the  source.  If  the

parameter  is zero, the control registers and baud  rates  on

entry are not modified.

The  return value is a pointer to the sio control  block  for

use with the other driver routines. NULL is returned on error

(invalid   port   number,  bad  buffer   sizes,   nonexistent

hardware, out of memory).

void v24_remove (sioptr sio, int restore);

Removes the driver for the specified control block.

Should  be called for all ports installed before exiting  the

program.

If the "restore" parameter is nonzero, the control  registers

and  the baud rate that were set before installation  of  the

driver  are  restored. If the parameter is zero,  the  values

are not changed.

CTask Manual          Version 1.1  88-07-01                Page 48

void v24_remove_all (void)

Removes  all  installed serial i/o drivers. This  routine  is

automatically   called  on  remove_tasker  if  drivers   were

installed. Note that you can not specify a restore parameter,

the default restore parameter is used (constant in tsksio.c).

void v24_change_rts (sioptr sio, int on);

Changes the state of the RTS output line. A nonzero value for

"on" will turn the output on.

void v24_change_dtr (sioptr sio, int on);

Changes the state of the DTR output line. A nonzero value for

"on" will turn the output on.

extern void far v24_change_baud (sioptr sio, long rate);

Changes  the baud rate.  The following baud rates are suppor-

ted:

50,  75, 110, 134, 150, 300, 600, 1200, 1800, 2000,

2400, 3600, 4800, 7200, 9600, 19200L, 38400L.

Note  that baud rates above 9600 may cause problems  on  slow

machines.

extern void far v24_change_parity (sioptr sio, int par);

Changes  parity.  The parameter must be one of the  following

values defined in "sio.h":

PAR_NONE  no parity checks

PAR_EVEN  even parity

PAR_ODD   odd parity

PAR_MARK  mark parity

PAR_SPACE space parity

extern void far v24_change_wordlength (sioptr sio, int len);

Changes word length. Values 5, 6, 7, 8 may be given.

extern void far v24_change_stopbits (sioptr sio, int n);

Changes Stopbits. Values 1 and 2 are allowed.

CTask Manual          Version 1.1  88-07-01                Page 49

extern void far v24_watch_modem (sioptr sio, byte flags);

The  modem  status  input  lines  specified  in  the  "flags"

parameter must be active to allow transmission.  Transmission

will  stop if one of the lines goes inactive.  The  parameter

may be combined from the following values defined in "sio.h":

CTS  to watch clear to send

DSR  to watch data set ready

RI   to watch ring indicator

CD   to watch carrier detect

A  value  of  zero  (the  default)  will  allow  transmission

regardless of modem status.

extern void far v24_protocol (sioptr sio, int prot,

word offthresh, word onthresh);

Sets the handshake protocol to use.  The "prot" parameter may

be combined from the following values:

XONXOFF   to enable XON/XOFF (DC1/DC3) handshake

RTSCTS    to enable RTS/CTS handshake

The  "offthresh"  value specifies the minimum number of  free

items in the receive buffer. If this threshold is reached, an

XOFF is transmitted and/or the RTS line is inactivated.

The  "onthresh"  value specifies the minimum number of  items

that  must be free before XON is transmitted and/or  the  RTS

line is re-activated.

Enabling XONXOFF will remove all XON and XOFF characters from

the input stream.  Transmission will be disabled when XOFF is

received, and re-enabled when XON is received.

Enabling RTSCTS will stop transmission when the CTS modem

input line is inactive.

int v24_send (sioptr sio, byte ch, dword timeout);

Transmits the character "ch". A timeout may be specified.

Returns -1 on timeout, -2 on wake, else 0.

CTask Manual          Version 1.1  88-07-01                Page 50

int v24_receive (sioptr sio, dword timeout);

Waits  for  a  received  character.   A  timeout  may  be

specified.  Returns -1 on timeout,  -2 on wake,  else the

character  in the lower byte,  plus an error code in  the

upper byte. The error code is the combination of

0x02      overrun error

0x04      parity error

0x08      framing error

0x10      break interrupt.

int v24_check (sioptr sio);

Returns -1 if no receive character is available, else the

next  character  from  the pipe.  The  character  is  not

removed.

int v24_overrun (sioptr sio);

Checks for receive pipe overrun.  Returns 1 if an overrun

occurred, 0 otherwise. Clears the overrun flag.

int v24_modem_status (sioptr sio);

Returns the current modem status word.  The CTS, DSR, RI,

and  CD  defines may be used to extract the  modem  input

line status.

int v24_complete (sioptr sio);

Returns  1 if all characters in the transmit pipe  have  been

sent, else 0.

int v24_wait_complete (sioptr sio, dword timeout);

Waits  for  the  transmit pipe to be  empty.  Returns  -1  on

timeout, -2 on wake, else 0.

CTask Manual          Version 1.1  88-07-01                Page 51

The Printer Output Driver

=========================

The  printer output driver provides for buffered output to  up  to

three  printer  ports (more can be added by editing  the  source).

Interrupt  or polling may be selected. Due to the  usual  hardware

implementation  of  printers  and  the  printer  interface,  using

polling  is  recommended. When using interrupts,  you  should  not

simultaneously  install  both  port 0 and port  1  with  interrupt

enabled, since both ports share the same interrupt line.

int prt_install (int port, byte polling, word prior,

farptr xmitbuf, word xmitsize);

Installs the printer driver for the specified port.  Ports  0

(LPT1), 1 (LPT2), and 2 (LPT3) are supported.

The   "polling"  parameter  specifies  polling  output   when

nonzero, interrupt output when zero.

The "prior" parameter sets the priority of the printer output

task.

"xmitbuf"  is  a buffer area for the printer  output  buffer,

"xmitsize" specifies its size in bytes.

The  buffer  pointer  may be specified as  NULL  for  dynamic

allocation if TSK_DYNAMIC is enabled.

If  the  port number for prt_install is ORed with  0x80,  the

port  is  *relative*. This means that the entry in  the  BIOS

table for LPT-Ports is used to search the tables internal  to

the  driver  for the port information, instead of  using  the

table  entry directly. If the port address cannot  be  found,

the  driver returns with an error code. Note that  ports  are

numbered from 0, so to specify LPT1, pass 0x80 as parameter.

The return value is the internal port number for use with the

other driver routines. -1 is returned on error (invalid  port

number, bad buffer sizes, nonexistent hardware).

void prt_remove (int port);

Removes  the  printer  driver for  the  specified  port.

void prt_remove_all (void)

Removes  all installed printer ports. This routine  is  auto-

matically called on remove_tasker if ports were installed.

CTask Manual          Version 1.1  88-07-01                Page 52

void prt_change_control (int port, byte control);

Changes  the  printer control output lines of the  port.  The

value for "control" may be combined from the following values

defined in "prt.h":

AUTOFEED  will enable printer auto feed if set

INIT      will initialise (prime) the printer if clear

SELECT    will select the printer if set

int prt_write (int port, byte ch, dword timeout);

Write a byte to the printer.  A timeout may be given. Returns

0 on success, -1 on timeout, -2 on wake.

int prt_status (int port);

Returns the current printer status lines,  combined from  the

values

BUSY      Printer is busy when 0

ACK       Acknowledge (pulsed 0)

PEND      Paper End detected when 0

SELIN     Printer is selected (on line) when 1

ERROR     Printer error when 0

int prt_complete (int port);

Returns 1 if the printer buffer has been completely transmit-

ted, 0 otherwise.

int prt_wait_complete (int port, dword timeout);

Waits for printer output to complete.  Returns 0 on  success,

else -1 on timeout, -2 on wake.

#                                                                                                                               ###################