DiskPark by Simcha Lerner

Version 1.00, November 1984

This program is a utility that will  locate all hard disks in the system

and move  their heads to the  inner-most cylinder.  It will  give status

messages for  each drive encountered.   This program uses the  hard disk

ROM BIOS calls  as documented in the  IBM Tech Ref for the  XT. This has

been tested on a  Compaq Plus without any problems.  It  will work on PC

compatible  machines and  non-IBM  hard  disks only  if  they are  fully

compatible at the BIOS level.  All other  I/O is through DOS so that you

can use this via a re-directed console.

The ability to exit back to DOS is  to allow you to park the disk if you

are going to leave the machine for  a while.  EXITING TO DOS CAN UN-PARK

THE HARD DISK, SO DO NOT EXIT TO  DOS IF YOU ARE ABOUT TO POWER OFF YOUR

MACHINE.   In order  to exit  back to  DOS, hit  three carriage  returns

slowly.  The program  flushes the buffer before each  character to avoid

accidental activation.

****************************************************************************

*                                                                          *

* IMPORTANT:                                                               *

*                                                                          *

* This  program does  NOT lock  the  heads.  If  you intend  to move  your *

* system, run the utility that came with it for that purpose.              *

*                                                                          *

****************************************************************************

#                                              *

*****************************************************************