NCBUSY.EXE

Jeffrey S. Morley

Released to the PUBLIC DOMAIN

September 29, 1992

A simple little program to place the modem "off-hook" or "busy" if the

CARRIER DETECT signal is low (no carrier detected) when the program is

run.  This program has application for BBS systems which require that

the modem be set in "auto-answer" mode (ATS0=1) and want to have a

method for ensuring that a user cannot inadvertantly be connected to

the BBS as the previous user (such as if a DOOR or other external program

is running and the user hangs up), and really for anyone who wants a

command line method of placing the modem in the busy state.

This program will take NO action if carrier is detected, and will not

change any port parameters or UART bits if the carrier signal is high.

Released to the PUBLIC DOMAIN.

Usage:

NCBUSY port speed

Where port is any comm port from 1 to 4, and speed is any baud rate from

110 to 115000.  If your modem is set to "lock" the DTE rate to a specific

speed, then that speed should be used.