Diamond SpeedSTAR 8514/A Emulation Driver

The instructions that follow explain how to install the appropriate

8514/A emulation driver for your needs. On this disk you should find

these files:

RIXAI4.EXE

RIXAI8.EXE

STAN0715.FNT

STAN0814.FNT

STAN1220.FNT

README.DOC (this file)

Two emulators are provided. Choose either emulator depending on your video

memory configuration and number of colors desired. The following table

illustrates the resolutions available by driver name according to the video

memory configuration on your board.

MINIMUM DISPLAY

RESOLUTION/COLORS       MODE (hex)      MEMORY REQUIRED         DRIVER

640x480     16         12                      512KB           RIXAI4

1024x768     16         37                      512KB           RIXAI4

640x480    256         2E                      512KB           RIXAI8

1024x768    256         38                      1MB             RIXAI8

INSTALLATION

1) Create a subdirectory on the hard disk called RIXAI or another name

of your choosing.

2) Copy the contents of the floppy diskette to that newly-created

subdirectory.

Decide which version of the emulator is needed based on the amount of

video memory on your video adapter and the number of colors you wish

and proceed.

4) Type RIXAIx at the DOS prompt, where x is either 4 or 8.

5) You can now run your application software under 8514/A emulation.

Remember to load the appropriate emulation driver before running

application software that requires 8514/A. You might want to create a

simple batch file that loads the emulation driver and your software

automatically to simplify the process.

EXAMPLE (acadai.bat)

RIXAI8

ACAD

You may need to reconfigure some software to run under an 8514/A

driver. Check the user manuals of your applications to determine if

such a driver is supported.

#