PACFAST for Amateur Radio

Copyright 1992 By Van Flynn, N5ARU

Documentation for Ver 1.1

by Minnie, KB5JM

PACFAST is a high  speed packet terminal program. It is  designed

to  work at baud rates up to 115200.  It was written in  "C"  for

fast  execution.  It  can utilize the 16550 UART  with  a  screen

display  indicator for this high speed UART chip.  It is able  to

maintain  log files, and will shell to  DOS.  It will work  under

windows  V3.0. This program will work on most MS-DOS or IBM  com-

patible   machine.  It is a very simple and easy program to  work

with and relies on the operators knowledge of the TNC's  Firmware

rather than a complicated and unwieldy software program.  For the

packet experimenter it offers the benefit of being one of the few

programs for packet radio that is designed for high speed as well

as  the utilization of the 16550 UART Chip.  As an added  feature

in  this version, it will look up Amateur Radio Call  Signs  from

within the program if you have RT Systems' SAM Program  installed

on  your computer.  This program will also indicate if you  don't

have SAM loaded in your machine.  You do not have to have SAM  in

order  to use this program; it will operate as well  without  it.

With  SAM, however, you can get callsign information  fast  while

working packet contacts.

There  are  many  other more sophisticated  programs  for  packet

radio.   However, the simplicity of this program will  appeal  to

the average packet user who just does not have time to engage  in

a  higher learning curve type of program.  Some  packet  programs

take months to learn; this one only takes minutes.

To  use  PACFAST insert the original disk in the drive  slot  and

make  a backup first.  Then if you wish to have the  program  in-

stalled  on your hard drive, copy  all of the files over  to  the

hard  drive.   To  run the program merely  type  PACFAST  at  the

prompt.   The program will come up under its default settings  of

COM1 1200N81.  If you need different parameters, for instance COM

2 at 9600 Baud, with no parity, 8 data bits, and 1 stop bit;  You

would then type PACFAST COM2 9600N81.  On the original disk there

is a file called SAMPLE.BAT.  You can change it and rename it  to

reflect your own set of parameters if they are different from the

default.  You can also change the parameters from within  PACFAST

by  using ALT P.  You cannot change the com port from within  the

program.

Once  PACFAST is running F1 gets the help screen which is  listed

below.   From  then on to receive and transmit  packet,  use  the

normal TNC commands like C for connect, etc.  If you have a 16550

high  speed UART Chip in the Com Port you are using on your  com-

puter,  the digits "16550" will appear on the bottom line of  the

screen (towards the right side).  If you don't have a 16550 UART,

then nothing will appear there.

PACFAST works very nicely under Windows v 3.0.  While in  Windows

use  the Program Manager go to the drive and directory with  PAC-

FAST  and  click on PACFAST.EXE or PACFAST.PIF.   Note  that  the

files  PACFAST.EXE and PACFAST.PIF must be on the same drive  and

directory  for the program to function under  Windows.   PACFAST,

while  not a windows program, will work as a window and will  run

as part of a multi-tasking environment.  This means that you  can

have PACFAST running in the background while you are doing  some-

thing else in Windows.

The following commands are available in PACFAST:

Alt C: Clears the screen

Alt E: Toggle Echo - If you type something and don't see

it appear on the screen then turn echo on by typing

ALT E.

Alt L: Toggle Line Feed - If each line of text you receive

in packet seems to be overwriting the previous line

then turn on Line Feed by typing ALT L.

Please note that both line feed and echo can be

toggled with commands to the TNC also.

Alt P: Changes Parameters - This command allows you

to change the baud, parity, data, and stop bit

settings from within the program.  Normally the

parity, data, and stop bit would be left at

N81.

Alt S: Shell to DOS - This command enables you to leave

PACFAST in a "status quo"  setting and go to the

DOS prompt and perform some DOS command such as

DIR, TYPE, etc.

F2   : Open a Log File - F2 will toggle a Log File named

PACFAST.LOG.  If the file is open anything received

via packet will be saved to disk.

F1   : Help Screen - A help screen will be display

the above commands.

F10  : This will open a small window and prompt you for

a callsign.  Type in the callsign, hit return,  and

the  information will appear in this  same  window.

You  must  have the SAM program installed  on  your

system   and   the  SAMAPI   loaded   resident   in

your systems memory.  A message will appear in  the

window  indicating if SAMAPI is not loaded.   Refer

to SAM Documentation for details.

Alt X: Exit -  This will exit you from the program.

This program was written using Borland C Compiler and the  MCOMM5

Library  and is copyrighted February 1992.  As such it is  share-

ware and not in the public domain. The author makes no  represen-

tations  or warranties with respect to the contents here  of  and

specifically disclaims any implied warranties of fitness for  any

particular  purpose. The SAM Callsign Program is a product of  RT

Systems, Laceys Spring, Alabama.

If you like the program,  and would like a  future update;  there

is  a  $14.50  registration fee which will entitle  you  to   one

upgrade  (if there is one - this is the second  update).   Please

make  checks  or  money orders payable to: Van  H.  Flynn,  N5ARU

P.O.Box  24816  New Orleans, Louisiana 70184.  Also  any  sugges-

tions,  reports of bugs, etc.  will be graciously  received.   If

you  like this documentation, and would like to let me  know  how

much  you  enjoyed reading it, please send E-Mail  via  Packet---

kb5jm@n5uxt.la.usa

de Minnie, KB5JM  2/28/92

####################