���������������������������������������������

�

�                                            ��

�        Documentation for KACK v1.0         ��

�                                            ��

�                                            ��

���������������������������������������������

���������������������������������������������

������������������

�                 �

�   Quick start   �

�                 �

������������������

KACK is short for Key Accelerate & Click Key.   It is a  program

which speeds up the typematic rate of the keyboard  and/or gives

a click each time pressing a key.  Entering  only `KACK'  at the

command line gives you this message:

Usage: KACK /<C n Px U>

/C for key Clicks

/n (1..4) keyspeed

/P x = tone period (100..10000)

/U unload from memory

Case of the argument is irrelevant. Some examples of usage:

KACK /1       gives lowest repeat rate (about 7.5cps).

KACK /4       gives fastest repeat rate (about 30cps).

KACK /C /4    as above plus making clicks.

KACK /C4p200  as above with higher tone

KACK /U       remove from memory if resident.

The program  goes resident if the click function (/C) is chosen.

No click is heard when a key is released, neither when  pressing

a shift-key (Alt, Ctrl, LeftShift or RightShift). Changing type-

matic rate is only possible on an AT or better. The PC/XT hasn't

the 8042 keyboard processor which does this possible.

������������������

�                 �

� Technical notes �

�                 �

������������������

As a TSR, KACK will occupy only 160 bytes.  This exceptional low

memory usage is  accomplished by using  assembly language and by

disposing most of the PSP. Most of this PSP area is of no use to

this program. Many textbooks warns agains modifying values below

5Ch,  but the word at 5Ch (DOS  function  dispatched) is a left-

over  from the CP/M  days.  I have found no program that rely on

this value. Certainly not this program.  (BTW, PkLite 1.14  puts

the word 4B50h ('PK') at this location).

If you wish to use a similar technique when making TSRs, be sure

to leave the start of the PSP unchanged as this area is used for

restoring  interrupt vectors when  exiting the program.  Consult

your technical literature for information on the PSP. The inter-

rupt list  edited by  Ralf Brown is an indispensable tool.  Look

for INTER30?.ZIP or later on BBSes or at a shareware supplier.

The program also releases its environment block  when going TSR.

Thus  making the program invisible to memory  mapping  utilities

like  MapMem or  Manifest when running MS-DOS prior to MS-DOS 5.

To make the  memory  hole left by the environment block as small

as possible,  reduce  your  environment as much as  possible. If

installed in your  AUTOEXEC.BAT place the call  before your PATH

or PROMPT statement.  If your are  using 4DOS, a simple trick is

to put a `@' before `KACK'.  This will  reduce some  environment

space.

To unload  the  program enter `KACK /u'  at the DOS prompt. This

frees the memory used (even if loaded high).  If any TSR  loaded

after KACK grabs the keyboard interrupt, unloading becomes impos-

sible.  KACK will have no way of knowing the last owner of INT 9.

This is the only interrupt  KACK is hooking. If the memory block

associated with KACK has  become  destroyed, execution halt with

an error message. See the source code in KACK.8 for details.

This program is released as FreeWare. Use the accompanied source

code with caution. You are free to modify it as you pleases, but

leave my name in as the originator of the code.  The program has

undergone tests on several PCs with MS-DOS 3.2,  MS-DOS 5.0  and

DR-DOS 6.0. No problems has been noted. If you have any problems

using KACK, or find any errors or  incompatabilities please drop

me a message through one of the following channels:

InterNet:     giva@oscar.bbb.no

Fido:         2:502/503

PolarNet:     GISLE VANEM @YRO         (for Norwegians mostly)

RIME conf:    Software or shareware

ILink conf:   Shareware & Reviews

Snail-Mail:   Gisle Vanem, Breimyra 56

5091 Flaktveit, Bergen, NORWAY

tel. +47-5-188249

Enjoy!

9. July 1992