SERVER AND BBS SOFTWARE FOR THE PACKET RADIO

MULTIPORTS, MULTICONNECTION, MULTILINGUAL

F6FBB, Jean Paul ROUBELAT

TABLE OF CONTENTS

-----------------                                         Page

1 ACKNOWLEDGMENTS ........................................ 4

2 COPYRIGHT .............................................. 5

3 FEATURES ............................................... 6

4 CONTENT OF THE DISTRIBUTION DISK ....................... 8

5 FIRST-TIME INSTALLATION AND SETUP ...................... 12

5.1  First-time setup of different files ........ 15

5.2  APPEL.BAT .................................. 16

5.3  INIT.SRV ................................... 17

5.4  EPURMESS.INI ............................... 25

5.5  PORT.SYS ................................... 27

5.6  SWAPP.SYS .................................. 29

5.7  INITTNCx.SYS ............................... 30

5.8  MAINTx.SYS ................................. 31

5.9  BALISEx.SYS ................................ 32

5.10 BBS.SYS .................................... 33

5.11 FORWARD.SYS ................................ 34

5.12 Start the BBS for the first time ........... 37

5.13 TNC. ....................................... 38

5.14 PK-232 in host-mode. ....................... 39

5.15 Kantronics KAM in host-mode. ............... 40

5.16 DRSI-card. ................................. 41

5.17 G8BPQ-node (uses TNCs in KISS-mode). ....... 43

6 BBS COMMANDS ........................................... 45

7 SERVER COMMANDS ........................................ 51

7.1  Main menu .................................. 51

7.2  Connections-Stats .......................... 52

7.3  Documentations ............................. 53

7.4  Directory/Callbook ......................... 54

7.5  QTH-Locator ................................ 55

7.6  Satellite Orbital Computation .............. 56

8 SYSOP COMMANDS ......................................... 57

8.1  BBS menu commands .......................... 57

8.2  Other commands ............................. 61

9 FBBDOS COMMANDS ........................................ 62

10 CONSOLE COMMANDS ...................................... 65

11 TEXT VARIABLES ........................................ 67

11.1  Variables definitions ..................... 67

11.2  Variables list ............................ 68

12 FULL SCREEN TINY EDIT ................................. 70

13 EMS and XMS ........................................... 72

13.1  EMS ....................................... 72

13.2  XMS ....................................... 73

13.3  FBB and additional memory ................. 74

13.4  Choice of add-on memory ................... 74

FBB 5.15                          Feb 06 1993                         Page - 1

14 ASYNC PORT DRIVERS AND TELEPHONE MODEM ................ 75

14.1  ESS.COM ................................... 75

14.2  FBBIOS.COM ................................ 76

14.3  TELEPHONE MODEM ........................... 77

15 MISCELLANEOUS TOOLS ................................... 81

15.1  INSTAFBB.COM .............................. 82

15.2  EPURMESS.COM .............................. 83

15.3  EPURWP.COM ................................ 85

15.4  LOGSTAT.EXE ............................... 86

15.5  SATUPDAT.EXE .............................. 87

15.6  SLEEP.COM ................................. 92

15.7  TLABEL.COM ................................ 93

15.8  MAKEPG.COM ................................ 94

15.9  SETUSER.COM ............................... 95

15.10 CLEANUP.COM ............................... 96

15.11 MAINTINF.COM .............................. 97

15.12 MAINTREJ.COM .............................. 98

15.13 CUT.COM ................................... 99

16 WHITE PAGES DATABASE AND SERVER ....................... 100

16.1  DESCRIPTION ............................... 100

16.2  UPDATE REQUESTS ........................... 101

16.3  DATABASE DESCRIPTION ...................... 101

16.4  DATABASE MANAGER .......................... 102

16.5  EPURWP AND UPDATE MESSAGES ................ 102

16.6  WP SERVER REQUESTS ........................ 103

17 SERVERS ............................................... 104

17.1  REQDIR .................................... 105

17.2  REQFIL .................................... 106

17.3  NEWDOC .................................... 107

17.4  REQCFG .................................... 108

18 OTHER FILE-formats (continued from section 5).......... 110

18.1  PORT.SYS .................................. 111

18.2  CRON.SYS .................................. 112

18.3  REJET.SYS ................................. 115

18.4  PASSWD.SYS ................................ 116

18.5  PROTECT.SYS ............................... 117

18.6  LANGUE.SYS ................................ 118

18.7  FORWARD.SYS ............................... 120

18.8  x.TXT, x.HLP, x.ENT, x.INF ................ 128

18.9  FBBLOG.nn ................................. 129

18.10 DIRMES.SYS ................................ 131

18.11 STATIS.DAT ................................ 132

18.12 WFBID.SYS ................................. 133

18.13 INF.SYS ................................... 134

18.14 HROUTE.SYS ................................ 135

18.15 YAPLBL.SYS ................................ 136

18.16 MEMO.SYS .................................. 137

18.17 SATEL.DAT ................................. 138

18.18 nnnnn.SAT (characteristics) ............... 139

18.19 WP.SYS .................................... 140

18.20 MESS.WP ................................... 141

FBB 5.15                          Feb 06 1993                         Page - 2

APPENDIX ................................................. 142

APPENDIX 1 : EDIT Commands ...................... 142

APPENDIX 2 : PK232 Host-Mode .................... 144

APPENDIX 3 : PROCESS NUMBERS AND HELP ........... 145

APPENDIX 4 : RECORDING A MESSAGE ................ 146

APPENDIX 5 : TRICKS AND TIPS .................... 147

APPENDIX 6 : FBB FORWARD PROTOCOLE .............. 148

APPENDIX 7 : COMPRESSED FORWARDING .............. 151

APPENDIX 8 : FORMAT OF ACK MESSAGES ............. 153

APPENDIX 9 : REPLACEMENT CHARACTERS OR WILDCARDS. 154

APPENDIX 10: PROGRAMMING TECHNICS FOR SERVERS ... 155

APPENDIX 11: CONTENTS OF THE DIRECTORIES ........ 159

APPENDIX 12: CONNECTION FILTERING ............... 161

APPENDIX 13: MESSAGE FILTERING .................. 163

APPENDIX 14: PG PROGRAMS DEVELOPMENT ............ 165

APPENDIX 15: UNPROTO MESSAGE LISTS .............. 167

APPENDIX 16: EXTENSIONS TO THE YAPP PROTOCOLE ... 169

FBB 5.15                          Feb 06 1993                         Page - 3

1 ACKNOWLEDGMENTS:

------------------

I would like to particularly thank Patrick FC1GHV and Per LA6CU, who have

helped me to achieve and set up this software, debugging and documentation,

as well as Claude FC1CDC, the "dumb individual", who, with his advices and

criticism (often vitriolic!), has given his support over more than four years

in the achievement of this task.

I would like to thank, as well, all the programmers for the development of

applications and complementary tools for this software. Such a support made

me free of these tasks, very often ingrate.

I thank again all the SYSOPS and users who are confident in the product, and

cooperate, thanks to their comments and suggestions, in improving this BBS

software package.

I would like, at the end, to thank WA8DED for the excellent piece of

software, the Host-Mode for the TNC, and also NORD><LINK for the additions

made to it.

FBB 5.15                          Feb 06 1993                         Page - 4

2 COPYRIGHT:

------------

This software is delivered at no cost and is free of all charges. It can be

freely copied or installed for any HAM-RADIO application. Any other usage for

commercial, professional, or industrial purposes is forbidden.

Despite the free sharing nature of this product, a participation to the

investment and to the mailing cost at the level of 100 FF or 20 US Dollars

(or more) would be most welcome.

The author can be contacted by mail at the following address:

Jean-Paul ROUBELAT

6, rue George SAND

31120 ROQUETTES

FRANCE

FBB 5.15                          Feb 06 1993                         Page - 5

3 FEATURES :

------------

The usage of this software is very close to the one made by WA7MBL, whose

commands have been kept. It has also a set of specific and original

supplementary commands.

- Works on any 100 % compatible XT or AT PC fitted with a hard disk and 640 k

RAM, monochrome CGA or EGA VDU, 1 up to 8 serial ports. This software allows

user-customized  colours  but  works  also  without  any  modificati

monochrome adapter.

- Multiconnections: up to 50 simultaneous channels on 8 TNC (4 or 8 channels

per TNC depending on the software used).

- Multiplexing: this software supports usage of an external MUX and supports

also extension boards in the case of a hardware configuration having more

than 2 ports. The MUX connects 4 TNC on 1 serial port, either com1 or com2.

Printed board is available from the ATEPRA association, (ATEPRA, 23 rue de

Provins, 77520 MONS EN MONTOIS - FRANCE).

Operates:

- with any TNC2 or clone fitted with the special 27256 eprom containing the

WA8DED firmware, or TF4/TF8, with a Z80 clock of 2.5 or, better, 4.9 MHz. -

- with a PK232 in Host-Mode (but TFPK software, WA8DED hostmode for PK232

works greater !).

- with a G8BPQ node.

- With a KAM hostmode (I seem to have still some problems).

- Standard BBS: it is a BBS identical to the WA7MBL or W0RLI. It has also

server functions (computation of satellite orbits, callbook, and operator

customizable chapters, gateway to an other channel, the users being able to

chat together in real time on the same or different channels, etc...).

- Forwarding: with the other BBS WA7MBL type or compatible, BIDS management

(up to 2000 of them, or more depending on a parameter, are saved in a

separate file), a BID is automatically generated should the user doesnt

provide if. Private message works with the management of MID. The messages

are suppressed automatically after a delay which can be user defined. This is

true for bulletins and private mail.

- The forwarding is optimized between BBS using FBB protocole type. It is

more efficient on a VHF/UHF network. This kind of forwarding can be disabled

by configuration.

- The compressed forwarding allows the reduction by a factor of about 40-50%

the quantity of data to exchange in big messages. The messages are protected

by checksums, then the transfer is made error-free.

- The forwarding is simultaneous on the various ports regardless they are

incoming or outgoing. There might be several forwardings outgoing per port

which number is set by parameter. The number of incoming forwarding is a

function of the available channels. The time and the period of forward can be

set separately on each port.

- The hierarchical routing is supported.

- The ping-pong phenomenon is automatically detected and an information is

given to the sysop by a system message.

- Messages and bulletins for SYSOP are duplicated to a destination call which

can be defined by configuration.

FBB 5.15                          Feb 06 1993                         Page - 6

- A detailed log of the BBS activity is maintained and a statistic analysis

program, written by FC1MVP, is also available.

- Binary transfer is supported with the usage of the YAPP protocol of WA7MBL.

An extension to this protocol has been made, including the automatic restart

and the checksum, should a stop has occurred or a disconnection took place

during the transfer. This extension to the protocol works with the TPK,

packet terminal program written by FC1EBN.

- Gateway between connected stations or with another port.

- Conference within the limits of the available ports and channels.

- Language: at the first connection, the connect language is attributed to

the user, depending of the callsign.

- Remote SYSOP operation is supported and the housekeeping of the BBS

messages, mail and old mail is done each and every night at low activity

period which can be parametered, and is done automatically.

- Works under DesqView.

FBB 5.15                          Feb 06 1993                         Page - 7

4 CONTENTS OF THE DISTRIBUTION DISK.

------------------------------------

Directory of C:\515\DISK1

INSTA515 BAT       120 28.03.92   20:27

FBB515   ZIP      2454 06.03.93   14:27

514515   BAT       120 18.08.91   12:40

BIN515_2 ZIP    235835 06.03.93   14:27

READ     ME        528 13.02.93   15:46

FBBMUX            2190 05.03.93   20:17

PKUNZJR  COM      2750 28.12.92    2:04

FWD515   ZIP       626 06.03.93   14:27

STAT515  ZIP     39800 06.03.93   14:27

SYS515   ZIP      5844 06.03.93   14:28

SUITE1   BAT      4491 05.03.93   22:08

SUITE2   BAT      7814 05.03.93   22:10

Directory of C:\515\DISK2

DOC515   ZIP    127982 06.03.93   14:28

BIN515_1 ZIP    197029 06.03.93   14:28

SAT515   ZIP     18420 06.03.93   14:28

Directory of C:\515\DISK3

LANG515  ZIP    199993 06.03.93   14:28

FIRMWARE ZIP    138645 06.03.93   14:28

Directory of C:\515\DISK1\BIN515_2

SERV     BIN    562941 06.03.93   14:26

Directory of C:\515\DISK1\STAT515

LOGSTATF EXE     39152 23.01.93   18:31

LOGSTATU EXE     39136 23.01.93   18:39

LISEZMOI DOC      8212 23.01.93   18:51

LOG      DOC       918 23.01.93   18:53

LOGCALL  EXE     11200 13.09.92   17:24

LOGFBB   CNF       358 23.01.93   18:57

FBB 5.15                          Feb 06 1993                         Page - 8

Directory of C:\515\DISK1\SYS515

BALISE1  SYS        89 22.03.92    2:05

BALISE2  SYS        81 22.03.92    2:05

INITTNC  DED        47 15.08.90   11:49

INITTNC  PK         44 08.12.90   16:50

INITTNC1 SYS        45 16.12.91   23:03

INITTNC2 SYS        34 13.10.91   20:02

LANGUE   SYS       807 05.03.93   20:48

CRON     SYS      1009 22.03.92    0:33

MAINT1   SYS        61 22.03.92    0:49

MAINT2   SYS        61 22.03.92    0:49

SWAPP    SYS       473 05.03.93   20:12

PROTECT  SYS       159 26.08.90   12:49

MEMO     SYS       143 11.12.91   22:24

FORWARD  SYS       215 05.03.93   20:01

BBS      SYS       584 05.03.93   20:01

PASSWD   SYS       766 05.03.93   20:03

PORT     SYS      2604 05.03.93   20:08

REJET    SYS       515 05.03.93   20:11

Directory of C:\515\DISK1\FBB515

INIT     SRV      3502 05.03.93   19:52

NEWDOC   SYS        14 10.02.91   12:54

EPURMESS INI      1406 05.03.93   19:53

Directory of C:\515\DISK1\FWD515

AMSAT               86 27.05.91   19:02

F6ABJ              491 13.10.91   20:00

LA6CU              244 13.10.91    9:26

Directory of C:\515\DISK2\BIN515_1

APPEL    BAT      1231 22.03.92    1:44

EPURMESS BIN     27269 05.03.93   19:25

MAINTINF BIN     14401 05.03.93   19:25

BIPPER   COM     12142 10.02.91   17:48

MAKEPG   COM      9034 06.03.93    8:29

ICOM     COM      6998 27.02.93   17:51

INSTAFBB COM     44758 05.03.93   19:52

REQDIR   COM     13810 03.06.92   18:42

REQFIL   COM     11940 03.06.92   18:50

TLABEL   COM       780 31.08.91   11:35

ESS      COM      3128 20.01.93   15:55

CUT      COM      7984 08.03.92   14:40

NEWDOC   COM     13060 10.02.91   13:09

CLEANUP  COM     58064 18.01.93   18:50

CTRLBK   COM       174 22.02.92    9:51

FBBIOS   COM      5775 10.02.93    1:30

HR2WP    COM     17152 27.12.92   17:20

SLEEP    COM      8778 10.02.91   16:39

SETUSER  COM      8114 04.10.90   18:08

FV       COM      8396 10.01.93    1:42

MULTI    EXE     15568 10.10.92    7:27

SATUPDAT EXE     47458 22.11.91    6:52

MAINTREJ COM     11500 04.06.92   20:36

EPURWP   BIN     19140 05.03.93   19:25

FBB 5.15                          Feb 06 1993                         Page - 9

Directory of C:\515\DISK2\DOC515

MULTI    DOC      5813 22.11.92   19:22

CHANGES  DOC     25841 02.02.93   15:56

515-GB   DOC    279165 07.02.93   15:42

WA8DED21 DOC     33778 29.10.87    0:24

WA8V21B  DOC     21331 20.01.88   22:39

WA8V21C  DOC     12483 15.03.88   21:15

FV       DOC      7388 10.01.93    1:42

TFPK     DOC      9700 02.05.92   15:51

Directory of C:\515\DISK2\SAT515

SATEL    DAT      4968 05.03.93    2:39

14129    SAT       964 03.05.92    2:33

14781    SAT       653 09.02.91   11:13

18129    SAT      2981 03.05.92    2:33

19216    SAT      1691 03.05.92    2:33

20480    SAT      1205 04.05.92    2:33

20437    SAT       860 03.05.92    2:33

20440    SAT       356 09.02.91   12:37

20441    SAT       495 04.05.92    2:33

20442    SAT      1293 04.05.92    2:33

20439    SAT      1479 03.05.92    2:33

21087    SAT      5956 03.05.92    2:33

20826    SAT       300 03.05.92    2:33

19336    SAT       306 03.05.92    2:33

20305    SAT       306 03.05.92    2:33

21232    SAT       325 03.05.92    2:33

16969    SAT       298 03.05.92    2:33

20788    SAT       331 04.05.92    2:33

20580    SAT       276 04.05.92    2:33

18820    SAT       305 20.05.91   21:03

19851    SAT       316 03.05.92    2:33

20670    SAT       313 04.05.92    2:33

16609    SAT      1112 03.05.92    2:33

19531    SAT       284 03.05.92    2:33

21089    SAT       538 03.05.92    2:33

21578    SAT       716 05.09.91   17:02

21575    SAT      1400 03.05.92    2:33

21577    SAT       848 03.05.92    2:33

FBB 5.15                          Feb 06 1993                        Page - 10

Directory of C:\515\DISK3\LANG515

FRANC8   HLP     52571 01.08.92   21:33

FRANCAIS HLP     52578 07.04.92   19:52

FRANCAIS TXT     11280 31.10.92   12:06

ITALIANO HLP     35632 04.08.92   20:42

FRANC8   TXT     14799 15.02.93   22:14

SVENSK   TXT     11312 09.08.92   17:05

PORTUGUE TXT     11375 09.08.92   17:05

NORSK    TXT     18897 09.08.92   17:05

NEDERLAN TXT     12058 09.08.92   17:05

ITALIANO TXT     13343 09.08.92   17:05

CATALA   HLP     18310 12.04.92   10:27

DANSK    HLP     31267 13.01.91    5:55

DEUTSCH  HLP     52368 20.03.92    2:31

ENGLISH  HLP     55909 22.08.92    7:06

ESPAGNOL HLP     18509 12.04.92   10:28

HRVATSKI TXT     10831 09.08.92   17:05

NEDERLAN HLP     16113 18.05.90   16:49

NORSK    HLP     62807 14.06.92   12:42

PORTUGUE HLP     19685 01.11.90   19:57

SVENSK   HLP     14271 07.03.91   23:29

ESPAGNOL TXT     12430 07.02.93   15:54

HRVATSKI HLP     39541 01.01.80    3:28

ENGLISH  TXT     14221 03.02.93   15:20

DEUTSCH  TXT     14122 07.02.93   15:53

DANSK    TXT     13977 09.08.92   17:05

CATALA   TXT     11423 09.08.92   17:05

MODEM    ENT       479 17.09.92    2:33

Directory of C:\515\DISK3\FIRMWARE

TF4      ROM     32768 17.03.88   20:23

TF18     ROM     32768 14.11.88   10:34

TF24     ROM     32768 14.09.90    9:14

TF8      ROM     32768 23.03.88   12:26

PK88U16K ROM     25508 18.06.90   16:59

PK88U32K ROM     25511 18.06.90   16:59

PK87U16K ROM     25417 06.03.90   21:08

PK87U32K ROM     25420 06.03.90   21:06

TFPK     ROM     32768 02.05.92   15:48

TFPKM    ROM     32768 02.05.92   15:48

FBB 5.15                          Feb 06 1993                        Page - 11

5 FIRST-TIME INSTALLATION AND SETUP.

------------------------------------

This part of the manual is intended for the sysop that has never used FBB

software before. I hope this part of the manual will help you to get "on the

air" with FBB without too many problems.

Normally you will install FBB from diskettes. Normally your diskette-drive

will be called A, and normally you will install the FBB-program on hard-disk

called C. But this is no absolute need, you can use other drives as well.

More about this a little later.

Your hard-disk is organised in several directories. If you are not familiar

with this, you should read in your DOS-manual about how the hard-disk is

organised.

FBB will normally be installed on your hard-disk in a directory named FBB.

This directory should be placed directly under root  (C:\FBB). This directory

will be automatically created for you. The BBS will also need a lot of other

directories, and they can be made directly from root, or they can be made

under the FBB-directory. In the procedure I will describe here, only the FBB-

directory will be at the root (C:\FBB) of the hard-disk, while all the other

directories will be placed under FBB (like  C:\FBB\USERS etc). But you can

choose this yourself. More about this a little later.

You should now be ready to start the installation procedure. Put the first

of the diskettes (if you use such a format that more than one diskette is

needed) in your disk-drive, and type A: if that drive is named A, or B: if

that drive is named B. Press ENTER to execute the command (you must always

press ENTER after a command to the PC). Your PC will now answer with

something like A:\> or B:\>

If you now are ready to install FBB 5.14 for the first time on your PC, you

type INSTA514 (For version 5.15 you type INSTA515 and so on..)

You will now see at your screen something like this:

---- QUOTE: ----

**** PREMIERE INSTALLATION DE LA FBB 5.14 ***

**** FIRST INSTALL OF FBB 5.14 SOFTWARE ***

Installation - depuis le disque A: - from disk A:

- Programmes du serveur -    BBS programs -     C:\FBB

- Fichiers systeme -         System files -     C:\FBB\SYSTEM

- Messages -                 Messages -         C:\FBB\MAIL

- Messages binaires -        Binary mail -      C:\FBB\BINMAIL

- Repertoires utilisateurs - User's directory - C:\FBB\USERS

- Fichiers Yapp -            Yapp files -       C:\FBB\USERS\YAPP

- Archive messages -         Message archive -  C:\FBB\OLDMAIL

- Statistiques -             Statistics -       C:\FBB\STATS

Si cette configuration ne convient pas,

modifier les chemins par defaut dans le fichier A:SUITE1.BAT

If this configuration is not ok,

please change it in the A:SUITE1.BAT file.

Tapez Ctrl-C si une version precedente existe deja

Et utilisez 513514.BAT pour une mise a niveau.

Type Ctrl-C if a precedent version already exists

FBB 5.15                          Feb 06 1993                        Page - 12

and use 513514.BAT instead.

----- UNQUOTE. -----

This screen may vary a bit from version to version, but the work you have to

do, will still be the same. First of all, you will see that the PC says that

you will install from disk A. Is this correct ? If you are installing from

disk-drive B, you must stop the installation now. The same applies if you

want to use other directory-names or paths for some of the other directories

that are listed on your screen now.

If you DO want to change any of these things, you MUST do it now. The

changes are decribed in this paragraph; if you do NOT want any changes, you

can go on to the next paragraph. You stop the installation-batch by pressing

CTRL-C. Now you will see the standard A:\> prompt again. Now you must use a

normal text-editor and edit the file SUITE1.BAT on your diskette. I presume

you have such an editor... Make sure that it is able to save the file again

in ASCII (DOS) format ! So if you use for example Word Perfect, you MUST

remember to save the file again in DOS-format !

At the beginning of that file, you will see a line saying SET disk=A: If you

want to install from B-drive, you edit this line like this:

SET disk=B:

You change only the letter after '=', nothing else ! You can also change all

directory-names or directory-paths in this same way. If you do not want to

use C:\FBB\SYSTEM, but want it to be C:\FICHIERS, you just change the line

saying

SET fichiers=C:\FBB\SYSTEM

to

SET fichiers=C:\FICHIERS

and so on. You must ONLY change the name behind the '=' sign, you must NOT

change anything before that in that line. You can do this with any (or all)

the directory-names and paths. But remember that you will need to do some

extra work with other config-files later if you do not use the default

installation.

When you have finished doing the changes you want in the lines starting with

SET, you must save the file again to disk. When that is done, you start over

again from the beginning, by typing INSTA514 etc.

OK. That is about all. Now the BBS-program and all its files will be

installed on your hard-disk. You only have to change diskettes each time you

are asked to (if you install from more than one diskette). When the

installation is complete, you will be asked to modify some of the files. We

will have a look at this now.

If you have a printer, it is a good idea to make a hard-copy of the doc-

files now. Go to the DOC-directory and type

COPY *.DOC PRN.

You will need a lot of paper....

This  is  the  recommended  organization  for  your  directories

directories will be made like this if you made no changes :

FBB               INIT.SRV and EPURMESS.INI (configuration) files.

|

|\__BACKUP       Backup important files.

|

FBB 5.15                          Feb 06 1993                        Page - 13

|\__BIN          All the BAT, COM and EXE files.

|

|\__BINMAIL

|    |

|    |\_MAIL0    Compressed mail files (number ending with 0).

|    |

|    :\_MAIL1

|    :

|    :

|    :

|    |\_MAIL8

|    |

|     \_MAIL9    Compressed mail files (number ending with 9).

|

|\__DOCS         Files of the DOCS command.

|

|\__MAIL

|    |

|    |\_MAIL0    Ascii mail files (number ending with 0).

|    |

|    :\_MAIL1

|    :

|    :

|    :

|    |\_MAIL8

|    |

|     \_MAIL9    Ascii mail files (number ending with 9).

|

|\_OLDMAIL       Optional archive for messages.

|

|\_PG            COM and EXE files for the PG command.

|

|\_USERS         Root of the FbbDos directories.

|    |

|     \_YAPP

|

\_SYSTEM        System files.

|

|\_FWD      Includes of FORWARD.SYS file.

|

|\_LANG     Texts and help files.

|

|\_LOG      FBBLOG.nn files.

|

|\_SAT      Satellite files.

|

\_WP       White Pages files.

The path FBB\BIN must be included in your PATH in AUTOEXEC.BAT. Do this by

adding to the PATH line in AUTOEXEC.BAT ";C:\FBB\BIN" like :

PATH C:\;C:\FBB\BIN

A list of the files of each directory is given in appendix 11.

FBB 5.15                          Feb 06 1993                        Page - 14

5.1 FIRST-TIME SETUP OF DIFFERENT FILES.

-----------------------------------------

So far so good... Now we must modify some files, to make the system work. I

will describe all the changes you MUST do, and some changes that you CAN do,

if you like. Read this section very carefully. For most of the files it is

VERY important that the number of non-comment lines must NOT be changed.

Later in the sysop-manual you will find more details on some of these files,

and also detailes on other files that are not described here in this startup-

section.

FBB 5.15                          Feb 06 1993                        Page - 15

5.2 APPEL.BAT.

---------------

This is the very first file that FBB will use. You will find it in the main

FBB-directory. You don't really need to do any changes in this file, unless

you did some changes in SUITE1.BAT during installation. If you did, you must

change the pathnames in APPEL.BAT exactly the same as you did in SUITE1.BAT.

This is very important ! So look for the lines that I have marked with an <--

- here :

set fbb=C:\FBB        <-- Defines the base of the FBB file system.

echo off

cd %fbb%                                                      <---

break off

echo Program is started, use ^C to interrupt within 3 seconds

sleep 3

serv %1

if errorlevel 2 goto direct

if errorlevel 1 goto suite

goto fin

:suite

cls

echo

echo Sorting HROUTE.SYS...

copy %fbb%\system\hroute.sys temp.$$$                         <---

type temp.$$$ | sort | find /v "# " > %fbb%\system\hroute.sys <---

del temp.$$$

echo

echo Backup of important files...

copy %fbb%\system\dirmes.sys %fbb%\backup\dirmes.s1           <---

copy %fbb%\system\inf.sys %fbb%\backup\inf.s1                 <---

echo Cleaning up message-file...

del epurmess.res

epurmess

echo Cleaning up white pages database...

del epurwp.res

epurwp

:direct

if not exist c:%fbb%\system\sat\amsat.txt goto appel          <---

cd %fbb%\system                                               <---

tlabel

cd %fbb%\system\sat

satupdat amsat.txt /n > satupdat.res

satupdat amsat.txt >> satupdat.res

del amsat.txt

cd %fbb%                                                      <---

:appel

echo Program is started, use ^C to interrupt within 3 seconds

sleep 3

serv -t

if errorlevel 2 goto direct

if errorlevel 1 goto suite

:fin

echo Program is terminated..

break on

Echo on

FBB 5.15                          Feb 06 1993                        Page - 16

5.3 INIT.SRV.

--------------

This file is the next file that FBB looks into during startup. This file

will be found in the main FBB-directory. All basic data is collected from

this file. Use your standard editor (as explained earlier) to edit the

following lines: (all lines starting with # are comments)

INIT.SRV is now created or updated with a little program named INSTAFBB.COM.

This program is organized as pages of menues, and you can edit or change any

topic in your configuration. It is no longer necessary to edit the INIT.SRV

file.

The version of the software is now indicated in the first comment line. This

will allow further automatic updates.

# FBB5.15

#

# Callsign and H-adress (Do not include SSID !)

LA1B.#BRG.NOR.EU

#

# SSID for the BBS

0

#

# Qra locator

JP20RH

#

# Qth (variable $c)

Bergen

#

In the above lines you MUST replace the callsign and H-adress with your own

BBS-call and H-adress. You also must replace the SSID, that is the number

after the callsign. In LA1B-6, the 6 is the SSID. If you do not want any

SSID, type 0 here. You also must replace the QRA-locator with your own

locator (ask some other ham for help on this, if you don't know the BBSs QRA-

locator. Do NOT use the value from this example-file ! You also must replace

the QTH (don't make it too long...). If the QTH-name is short, you might add

the QRA-locator here too, like I have done.

Next we have some filenames and path-names. These filenames and paths are

the same as used in the installation-procedure. If you did no changes in this

during installation, you will not need to change any of these now. But if you

DID make changes during installation (in SUITE1.BAT) you MUST do the exact

same changes here ! This is VERY important ! Note the syntax, and note that

directory-names MUST end with a \

# Directory for files used by the BBS

C:\fbb\system\

#

# Directory for messages

C:\fbb\MAIL\

#

# Directory for binary-messages

C:\FBB\BINMAIL\

#

# Directory for users DOS

C:\fbb\users\

#

# Directory for Yapp-files

C:\fbb\users\yapp\

#

FBB 5.15                          Feb 06 1993                        Page - 17

# Directory for DOCS

C:\FBB\DOCS\

#

That was not so difficult ?

Next you MUST replace the first-name of sysop, and the callsign of sysop.

# First-name of sysop

Per

#

# Sysops callsign

LA6CU

#

In the following line you CAN (not necessary) replace the ***** with a

callsign. If you do, that callsign will receive a copy of all SYSOP-messages

that arrives. You might want to put your own callsign there.

# Callsign (and route if needed) that shall have copy of messages

# adressed to SYSOP

# (*** for no copy, FD1CDC@FD1CDC.FMLR.FRA.EU if route)

*****

#

No need to change this:

# Path and filname for import-file

\FBB\MAIL.in

#

Here you must first write the code for the kind of monitor you use, and then

you must have a number for how many minutes you want before the screen should

go blank, after no activity on the keyboard. In this example I have a CGA-

monitor, and my screen will go blank after 5 minutes since I last touched the

keyboard.

# Type video:0=CGA, 1=EGA, 2=VGA Time before screen-blank.

#

0 5

#

Log complet gives you a log-file for every week. If you write NO here, no

log-file will be made. The log-file will appear in the directory named LOG.

Direct video uses fast BIOS-calls if you answer OK. If you plan to use

Desqview or Windows you must change it to NO. Indication of channels will

normally be OK (that means that monitored information on screen will be more

complete, with callsign of user before every frame from each user, etc).

# Log complet (OK or NO)

OK

#

# Direct video (OK or NO)

OK

#

# Indication of channels

OK

#

FBB 5.15                          Feb 06 1993                        Page - 18

Next we have Mode Test (NON - OK). If you type OK, the BBS will start, but

will not access the TNC(s). Use this the first times, to make sure that

everything works fine. When you are sure that all is well, connect the TNC(s)

and change this to NO. Restart the FBB, and now the TNCs will be used, as

normal.

# Mode Test (NO - OK)

OK

#

Forward type FBB is faster and more efficient than standard MBL-type

forward. Use this whenever possible (OK). If you type NO, FBB-type forward

will not be allowed. The same goes for compressed forward. Compressed forward

is very much faster, and should always be used. So, leave these lines un-

changed, you can change them later, if you want to :

# Use (when possible) forward type FBB

OK

#

# Use (when possible) compressed forward

OK

#

Hroute update validation. If you type OK, the HROUTE look-up will be

validated. This takes some tens KB of memory. A route without hierarchical

extension will be changed to hierarchical route if found in the HROUTE.SYS

database. This  database  is  automaticly  updated  while  receiving

headers.

# HROUTE look-up and database update validation

OK

#

Informations query. This ensures that the user will give his informations.

This will be helpfull for the update of the white pages database. If "OK",

the user will prompted until he answers to the queries.

# User MUST give Name, HomeBBS, Qth and ZIPcode

OK

#

Next we have 'Masque'. This is a default value that is given to all new

users when they log on the BBS for the FIRST TIME. Select the things you want

for all new users, and the values for them. If you f.ex. want all new users

to get Paging, and be in Guest mode, add 32 and 64, and put the result 96 in

the file. If you want none of the things listed here, put in a 0. The "see-

all_mail" mask will be active only if the user has the "read-all-messages"

security code (see behind).

# First connection mask.

# 0   : disable.

# 1   : Excluded.

# 2   : Local.

# 4   : Expert.

# 8   : Sysop.

# 16  : BBS.

# 32  : Pagination.

# 64  : Guest.

# 128 : Modem.

FBB 5.15                          Feb 06 1993                        Page - 19

# 256 : See-all-messages.

# 512 : Unproto-list asking is allowed.

# 1024: List of new messages given at logon.

# Add up the values for your choises.

1568

#

Then we have 'Security-codes'. Here we give 3 values. The first

says what ALL users are allowed to do. The next says what sysops

are allowed to do when they connect the BBS, and the third says

what a sysop is allowed to do after he has successfully performed

the SYS-command. The same applies here as above; choose the

values you want and add them up.

# Security-codes.

# All users can:

# 1  : Read all messages, including all personal messages.

# 2  : Kill all messages.

# 4  : Send SYS-command.

# 8  : Use remote-sysop commands (edit, forward etc).

# 16 : Edit labels in YAPP, FBBDOS, DOC.

# 32 : Can delete files in YAPP and FBBDOS.

# 64 : Have access to all gateways.

# 128: Run dos-programs (command DOS in FBBDOS).

# 256: Have access to the entire hard-disk.

# 512: Have access to command /A (stop BBS) and /R (reboot PC).

#

# All: Sysop: Sysop after successful SYS-command:

0    127    1023

#

Next we have the time for cleanup of messages. This should be done in a low-

traffic hour, as the BBS is completely shut down during cleanup. Type also

the timeout-values you want, the default values here should be OK. Put also

in the number of hours (+ or -) between the PCs local-time and UTC.

# Time (hour) for cleanup of messages

02

#

#

# Time-out for normal use (minutes)

10

#

# Time-out during forward (minutes)

15

#

# Hours +/- in relation to UTC

0

#

Type how many callsigns you want (maximum) in each mail-beacon (variable

$Q). If you precede the number by a B, also bulletins to local users (type BN

or BY) will be brodcast in the mail-beacon. Like this:

# Number of callsigns in mail-beacon

B20

#

FBB 5.15                          Feb 06 1993                        Page - 20

Number of lines in scroll-buffer may be changed. Just remember that they use

a lot of memory (each line takes 160 bytes of memory, but can be in high

memory), so I advice you to use these values:

# Number of lines in scroll-buffer

# User Console Monitoring

200    100     100

#

Leave this one alone:

# Text for forward-header (variables OK), appears LAST in the R:line.

# Please do not change the contents and the order of the fields except

# the FBB$E field which is only an information and can be removed.

# $c is the QTH as declared before in this file.

[$c] #:%M Z:$z FBB$E

#

Next you must decide how many BIDs to keep. 3000 is a good value. If you are

using EMS or XMS-memory (with a special driver loaded for this..) you should

answer OK for BID in EMS/XMS, if not, you must answer NO.

# Number of BIDs that are saved (32000 max)

# No more than 3000 when EMS/XMS is not used !

3000

#

Leave this one alone. This one means that bulletins that are older than (in

this case) 30 days, will not be forwarded again from my BBS, and will go

directly to X-status. Be aware: a too low value here will make problems for

other BBSs that you forward to..

# Lifetime for bulletins. Number of days from message is written

30

#

Here you must say if you use any EMS or XMS memory, or if you have no such

memory available. If you DO have such memory available, you should answer 1

or 2 here followed by the list of topics you want to load in high memory. The

BBS will do these operations very much faster.

#

# Use EMS or XMS memory ? 0=No, 1=EMS, 2=XMS.

# and optional list of topics to put in high memory

#

# Topics : BID = Bulletins or messages identifiers

#          MSG = Message lists

#          HIE = Hierarchical information

#          OVR = Software overlays

#          SCR = Screen buffers

#          WPG = White Pages database

#          REJ = Reject/hold informations

#          FWD = Forwarding informations

#

2 BID MSG WPG

#

FBB 5.15                          Feb 06 1993                        Page - 21

List of routes to send WP update messages. First, keep this line blank until

you have a WP network available, then give the route(s) to send your updates.

# List of routes to send WP messages

#

#

Zip code of the BBS. Give the post code area where is the BBS.

# ZipCode of the BBS

#

31120

#

Here you will give the paramaters allowing the list of messages in unproto

mode. This list can be interpreted by some terminal softwares (as TPK) and

greatly reduce the traffic on the local frequency.

The first parameter is the maximum backward number. If a unproto user asks a

list which is lower than this number backward, he will be limited to this

parameter (IE: if the current message number is 20000 and a user asks the

list of the message 15000, he will be then limited to the 19500 if the

backward number is 500).

The second parameter is the speed of the unproto updates given in seconds.

The frames will be sent with this period.

# Back number and unproto lists period

#

500 10

#

Leave this lines alone, unless you are absolutely sure you want to change

it:

# Intern interrupt (hexadecimal, default 7C)

7C

#

Then you have two optional programs of batches which are called at the end

of the initialization, and just before shutdown. If you don't need them, keep

these lines empty.

#

# BBS-UP program (default empty)

#

# BBS-DW program (default empty)

#

You can change these colours if you like:

# Colour on texts :

#

# Black      : 0      Dark grey     : 8

# Blue       : 1      Light blue    : 9

# Green      : 2      Light green   : 10

# Cyan       : 3      Light cyan    : 11

FBB 5.15                          Feb 06 1993                        Page - 22

# Red        : 4      Light red     : 12

# Magenta    : 5      Light magenta : 13

# Brown      : 6      Yellow        : 14

# Light grey : 7      White         : 15

#

#

# COULEUR FONDS

#

# Status (top)

1

# Bandeau (middle)

7

# Tekst (bottom)

0

# Menues

4

#

# Colour of characters

#

# Status (top)

7

# Bandeau (middle)

1

# Menues

14

# Text sent

10

# Text received

12

# UI (Headers)

15

# UI (traffic)

7

# Console

14

# Beacon

6

# Channel-marking

11

#

------

#

You may change these lines if you like. These are the servers that are

normally available in most FBB-BBSs, and they are automatically installed for

you from the diskettes. So you can leave them alone. Or you can disable them

with a # in front of each of these lines.

Two built-in servers already exist, but you MUST declare them in this file

to make them available. As they are built-in, the program-name MUST be

replaced with stars.

# List of "servers" :

#

# To Program-name Description

#

REQCFG ****** Request configuration

WP     ****** WP server

REQDIR REQDIR Request directory-listings from other BBSs.

REQFIL REQFIL Request files from other BBSs.

NEWDOC NEWDOC Upgrade files under DOCS.

------

FBB 5.15                          Feb 06 1993                        Page - 23

#

# End of file

#

Ok. That was the INIT.SRV file. This file is VERY important, so check and

double-check to make sure that you have it all OK.

FBB 5.15                          Feb 06 1993                        Page - 24

5.4 EPURMESS.INI.

-----------------

This file is also in the main FBB-directory. This is the file that controls

the lifetime of messages. Each night EPURMESS will be activated at the time

specified in INIT.SRV. Normally you will not need to change this, but if you

did any changes in SUITE1.BAT during installation, you must also change the

same pathnames here. That goes for these lines:

#

# File for deciding messages' lifetime..

#

# Directory for messages

\FBB\MAIL\

#

# Directory for binary-messages

\FBB\BINMAIL\

#

# Directory for "killed" messages

\FBB\OLDMAIL\

#

# The file DIRMES.SYS (database-file)

\FBB\SYSTEM\DIRMES.SYS

#

# Old database-file (backup)

\FBB\SYSTEM\DIRMES.OLD

#

# New database-file (after EPURMESS (cleanup))

\FBB\SYSTEM\DIRMES.NEW

#

# Result of cleanup is pu into this file:

\FBB\EPURMESS.RES

#

Next, you decide if you want to save "killed" messages in the OLDMAIL-

directory. If you do, write an 1 first, if you want to save old personal

messages, and another 1 if you want to save old bulletins.

# Personal Bulletins (0=kill 1=archive in oldmail)

0 0

#

Next we have the actual parametres (in days) for the lifetimes of bulletins

and personal messages. Let me just explain the letters in use here:

P=Personal message

B=Bulletin

N=Not read

Y=Had been read

F=Forwarded (forwarded to all BBSs that should receive this msg)

X=Expired (still readable, but will not be forwarded to other BBSs)

K=Killed (readable only by sysop)

A=Archived (same as killed, but will be deleted at once, or moved to

OLDMAIL)

I think this is self-explainatory:

# Parametres in days:

#

# PARAMETRES FOR PERSONAL MESSAGES:

#

# PN -> PX (days after message is written)

FBB 5.15                          Feb 06 1993                        Page - 25

30

#

# PY -> PX (days after changed to Y)

7

#

# PF -> PK (days after changed to F)

7

#

# PX -> PK (days after changed to X)

0

#

# PK -> PA (days after changed to K)

0

#

# PARAMETRES FOR BULLETINS

#

# BN -> BX (days after the message is written)

7

#

# B$ -> BX (days after the message is written)

7

#

# BY -> BX (days after changed to Y)

7

#

# BX -> BK (days after changed to X)

14

#

# BF -> BX (days after message is written)

14

#

# BK -> BA (days after changed to K)

0

#

------

#

At the end of the file, you can have special lifetimes for special messages.

In the example below, messages that are TO LA6CU will be 365 days old,

messages FROM system will be 0 days, messages @LA will be 365 days. I have an

# in front of them, this means they are disabled right now.

# List of lifetime-tests

#

# Type To Days

@   VEST   365

@   LA     365

>   LA6CU  365

<   SYSTEM 0

#

# End of this file.

#

FBB 5.15                          Feb 06 1993                        Page - 26

5.5 PORT.SYS.

--------------

This file is found in the directory called SYSTEM (if you used my default

directory-names). In this file we decide what kind of TNCs to use, how many

available channels etc. You must do several changes here, and be very careful

that you do all changes correctly. If you don't, the BBS cannot operate

properly.  Remember that a line starting with # is a comment-line, and the

BBS ignores these lines.

The BBS has built-in drivers for a maximum of 8 serial-ports, named COM1-

COM8. It can also use a special port-multiplexer so that you can have up to 4

TNCs per COM-port. First you must type how many ports and TNCs you use, like

this:

# File for programming of channels and TNCs.

#

# Ports : How many ports (COM1, COM2, Etc...)

# TNCs : How many TNCs and modems in use. With multiplexer

# there can be up to 4 TNCs per port.

#

#Ports TNCs

2      2

#

Next you must type one complete line of parametres for each COM-port you

use. You must be careful to use the correct values here. If you use external

drivers like COMBIOS, BPQ or DRSI-card, you must be careful to give the

correct values here. If you use COMBIOS, FBBIOS, BPQ etc, they must always be

loaded before you start the BBS-program.

In this example I use 2 TNCs, one on COM1 and one on COM3, and I use the

internal driver :

# Com      : COM-number (1,2,...8)

# Interface: 1 = External ESS driver

#            2 = BPQ-node V 4.05 or up.

#            3 = Telephone-modem with FBBIOS

#            4 = DRSI card with driver

#            5 = TFPCR/TFPCX interface

# Adress   : Adress of port in hexadecimal (Needed for multiplexor).

# Baud     : Ports baud rate. Ignored by BPQ.

#

# Use same number of lines as number of ports.

#

#Com Interface Adress (Hex) Baud

1    0         3F8          4800

3    0         338          4800

#

Once more, the number of lines below must be the same as number of TNCs in

use. One line for each TNC. I think most of it is self-explainatory. MultCh

will normally be 1. If you use DRSI-card, you can use a value from 0 to 7. If

you have a KAM in host-mode, you must use 1 for VHF and 2 for HF. MxBloc

decides how many kb will be forwarded one way to another BBS before the

forward is reversed. Leave this value at 10 for VHF/UHF and a little smaller

for HF. Type host-mode depends on your TNC, and if you use BPQ :

# TNC     : Number on TNC in use.

# NbCh    : Number of channels I want to use in the TNC.

#           Maximum available channels depend on firmware.

FBB 5.15                          Feb 06 1993                        Page - 27

# Com     : Number of the COM-port. Com1, Com2 etc.

# MultCh  : Number of channel if port-multiplexer is used, otherwise 1.

#           In DRSI use values from 0 to 7, by KAM use 1/VHF and 2/HF.

# Paclen  : PACLEN on this TNC.

# Maxframe: The maximum nb of frames the TNC will send at a time.

# NbFwd   : Number of channels for OUTGOING forward at same time.

# MxBloc  : Size of forward-block in kb.

# M/P-Fwd : Minute of the hour for start of forward, and period

#           (how many minutes between each forward-start).

# Port mode, one of these:

#           B : BBS-mode.

#           G : "Guest"-mode.

#           U : Normal-mode.

# Type host-mode, one of these:

#           D : WA8DED

#           K : KAM hostmode.

#           P : PK-232

#           Q : BPQ v 4.x

# Addition: One or more of these letters can be used too:

#           L : Send unproto beacon after each arriving mail.

#           M : Telephone-modem.

#           Y : Yapp allowed on this QRG.

#           W : Gateway allowed TO this QRG.

#           R : Modem port allowed in Read-only mode.

# Freq.   : Text to describe this port (max 9 characters, no space)

# Same number of lines as number of TNCs.

#

#TNC NbCh Com MultCh Pacln Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    7    1   1      230   4     1     10     30/60   UDYW 433.650

2    1    3   1      80    2     1     5      17/30   GDW  15/20m

#

At the end of the file, you can specify one (or more) callsigns and SSIDs

for some channels. You might use this for special callsigns on forward-

channels. If you don't need this (you probably don't, at least not the first

times..), place an # in front of the line. This only works with WA8DED

interface.

# Special callsigns and modes for some channels.

#

#TNC Nbs Callsign-SSID Mode

1    2   LA1B-1        B

#

# End of file.

#

FBB 5.15                          Feb 06 1993                        Page - 28

5.6 SWAPP.SYS.

---------------

This file will also stay in SYSTEM-directory. This file is used for

automatically re-route messages. This file normally changes the @BBS-adress,

but can also change the TO-field. You might not need so many callsigns etc.

here, but you should make one line with only your callsign after the @. In

that case the @BBS will be removed from messages to users in your own BBS.

For the first tries, it is enough to have one line starting with @, and then

your BBS-callsign. You can fill in the rest later.

#

# File SWAPP.SYS is used to change @BBS-adresses.

# For example replace @EUR to @EU

#

# Change TO-adress... Use > instead of @

# Change @BBS for a specific callsign, ex; > LA8AK @ OZ2PAC

#

@ LA8AK  @ LA9K

@ LA5IV  @ LA9K

@ LA5F   @ LA9H

@ EUR    @ EU

@ HF     @ EU

@ WWW    @ WW

> LA1B   @ LA6CU

@ LA6TAA @ LA1Z

@ LA9WO  @ LA9H

> F6FBB  @ F6FBB.FMLR.FRA.EU

FBB 5.15                          Feb 06 1993                        Page - 29

5.7 INITTNCx.SYS

-----------------

This file is also in the SYSTEM-directory. When the BBS starts, it sends

some standard parametres to the TNC, like PACLEN, MAXFRAME etc. These

parametres are in PORT.SYS. But in addition to this we may send som more

parametres to each TNC. We send parametres to TNC 1 with the INITTNC1.SYS, to

TNC 2 with INITTNC2.SYS etc. Here is just one example from LA1B BBS : (you do

not need to use the same parametres)

C MAIL v LA7QR

U 0

N 10

M IU

P 64

T 40

C MAIL V LA7QR means that unproto mail-beacon will be sent to MAIL via the

digipeater with callsign LA7QR. You may use just C MAIL if you do not want to

send via digipeaters. U 0 means that the TNC will not send any TNC-message to

user when he connects to the BBS. You should use this parametre.

N 10 means that RETRY will be set to 10.

M IU means that that all I and UI frames will be shown in monitor-windows. P

64 means a p-persistance value of 64. This is normal.

T 40 sets a TXDELAY of 40. This value depends on Transceiver in use. So, you

can use any suitable parametres in this file. You need one file for each TNC.

The commands sent to the TNC, are the commands described in the

documentation for the firmware you use, or in the documentation for PK-232,

DRSI, BPQ etc.

If you are using BPQ, DRSI, PK-232 or KAM, look in section 14 for special

commands for this files.

FBB 5.15                          Feb 06 1993                        Page - 30

5.8 MAINTx.SYS

---------------

This file is in the SYSTEM directory. This works exacly like INITTNCx.SYS,

but the parametres are sent to the TNC when the BBS is shut down. The file

can look like this:

Y 1

U 1 BBS ($c) was shut down for service $d $T.

Y 1 means that only 1 station can connect my TNC now.

U 1 means that the TNC will send this message to any user that connects

after the BBS has shut down.

If you use a PK-232, the same file would look like this:

UR1

CTBBS ($c) was shut down for service $d $T.

The  commands  sent  to  the  TNC,  are  the  commands  describ

documentation for the firmware you use, or in the documentation for PK-232,

DRSI, BPQ etc.

FBB 5.15                          Feb 06 1993                        Page - 31

5.9 BALISEx.SYS

----------------

This file is in the SYSTEM directory. This file makes the text that will be

broadcast as mail-beacon from the BBS. (Balise in french means beacon). The

first line is a parametre that says how often to send the beacon. %15 means

"send one beacon every 15 minutes". The next line says to what unproto-

address the beacon will be sent. ! MAIL v LA7QR means send beacon to MAIL via

LA7QR digipeater.

The next lines are the actual beacon-text. Here we can use variables like $d

$T etc. We cannot use variables that are specific to one channel alone. You

do not need to change anything else then the !-line in this file. But you

will need one file named BALISE1.SYS for TNC1, one calles BALISE2.SYS for

TNC2 etc.

%15

! MAIL v LA7QR

$d $T <<< Mailbox $O $c >>> $N active messages.

Messages for$W$Q

FBB 5.15                          Feb 06 1993                        Page - 32

5.10 BBS.SYS

-------------

This file is in SYSTEM directory. In this file you MUST put the callsign of

every BBS you will forward directly to. There must be 80 (from 1 to 80)

lines, even if you do not use them all. The format is very critical, so do

NOT change line-arrangement !

All the BBSs you forward to (the callsigns after A in FORWARD.SYS) must also

be in BBS.SYS. Before you start your BBS for the first time, you should

remove all not-used callsigns in BBS.SYS, and put in the file only the BBSs

that you forward to in your FORWARD.SYS. Later it is recommended that you do

NOT replace one callsign with another, as messages then will go to the wrong

BBS.

This file should speak for itself.

FBB 5.15                          Feb 06 1993                        Page - 33

5.11 FORWARD.SYS

-----------------

This file is in SYSTEM directory. In this file all necessary data for

forwarding of messages is held. First I will give an example of a file that

contains the minimum of data that MUST be in FORWARD.SYS if you have forward

to only 1 other BBS, called LA2D in this example.

The file is organised in blocks, one block for each BBS we forward directly

to. A block starts with A (callsign) and ends with -------- We can have as

many blocks as we want in FORWARD.SYS.

A LA2D

#

P C

#

C C LA2D

#

B LA2D

F LA2D

#

---------

All lines starting with a '#' are comments-lines, and are ignored by the

BBS. The first line must always start with the letter A, and next the

callsign of the BBS we want to forward to. Here: A LA2D

Then there must be line telling on which port to start the forward. This

line must be there even if we only have 1 port.

In this example I use port C, which is the same as TNC 3 in my PORT.SYS.

Command: P C

So I will send the connect-command to the TNC. That line must start with a C

before the actual connect-command. So if I call LA2D directly, I need to

write C C LA2D. (Only 1 C will not work). And lastly I must write which

messages I shall forward to that BBS. In this case I type only B LA2D, which

means all messages with @ LA2D, and F LA2D, which means all messages TO LA2D.

The forward-file must end with a string of -------. If you have more than one

BBS to forward to, you write all data for that BBS in a similar block in this

same FORWARD-SYS-file (starting with A, and ending with ----) right after

this block for LA2D.

You can also use one include-file for every BBS, but we will look at that in

section 14 of this sysop-documantation.

Well. I expect you do not want to forward only those messages that are TO

LA2D or @ LA2D (in this example), I expect you want to forward bulletins

also. Just include some lines starting with G, like G EU, G ALL, G WW etc in

the file. G EU means all bulletins adressed to @ EU. So may want to add this

in each file, like this:

A LA2D

#

P C

#

C C LA2D

#

B LA2D

F LA2D

#

G EU

G ALL

FBB 5.15                          Feb 06 1993                        Page - 34

G WW

---------

OK. Next you may want to forward messages through LA2D, that is not really

for LA2D, but that LA2D can forward on to this other BBS. These other BBSs

can be included in lines starting with a B, like B LA1G, B LA4O, B LA5G etc.

like this:

A LA2D

#

P C

#

C C LA2D

#

B LA2D

F LA2D

#

B LA1G

B LA4O

B LA5G

#

G EU

G ALL

G WW

---------

Also, if you want to forward ALL messages for other SM-BBSs this way, you

can use wildcards (as in DOS) like B SM*, B SK*, B OH* etc. like this:

A LA2D

#

P C

#

C C LA2D

#

B LA2D

F LA2D

#

B LA1G

B LA4O

B LA5G

#

B SM*

B SK*

B OH*

#

G EU

G ALL

G WW

---------

Well. Now the file is almost complete for most needs. We may want to add the

possibillity to forward (on VHF or UHF) via several nodes. This is very easy,

as we just add another C C (callsign) line for each node we have to call. If

i must call first my local node LA7QR, next the local node of LA2D (LA1EAX-

7), and lastly LA2D, the complete file for LA2D will now look like this:

A LA2D

#

P C

#

C C LA7QR

C C LA1EAX-7

FBB 5.15                          Feb 06 1993                        Page - 35

C C LA2D

#

B LA2D

F LA2D

#

B LA1G

B LA4O

B LA5G

#

B SM*

B SK*

B OH*

#

G EU

G ALL

G WW

#

R

#

---------

If you want to forward via KA-nodes etc., look in section 14.7.

I put in an R at the end. This forces my BBS to call LA2D BBS and ask him

for forward from him to me (revers forward), even if I have nothing to

forward to him.

Well, this was a simple forward-file. There is a lot more we can do with

this file, so you should read section 14 of the sysop-manual very carefully,

I think you will find all you need there.

FBB 5.15                          Feb 06 1993                        Page - 36

5.12 Start the BBS for the first time.

---------------------------------------

Now you should be ready to start the BBS for the first time. Before you do,

make sure that the internal clock of the BBS is correct. If not, you must

correct it now with the DOS-command DATE and TIME. In most newer PCs today,

the clock is automatically set at each power-up. But in some older PCs you

may need to use a special clock/calendar card in your PC, and use a special

program to set the date/time at each startup. For this you must look in the

manual for that clock/calendar card and include the appropriate command in

your AUTOEXEC.BAT file.

When this is settled, go to the FBB directory and type APPEL and press

ENTER. The first time you start the BBS like this, it will ask you if you

want to create some files. Just answer Y (for Yes) every time. Then those

files are automatically created for you.

The BBS should now be ready for use. You will find a short help-list for all

the function-keys by pressing F1.

Try a connect to the BBS. Do this by pressing F2. The BBS will ask you for

your name and QRA-locator. Next you must define all the BBSs you want to

forward to. You do this with the command EU. If you want to have forward to

for example LA2D, you type EU LA2D.

The BBS will say that this callsign is not known, and ask if you want to

create it. You answer Y. Next you will see a line of different options for

that user. You only have to type B. Now that callsign (LA2D) will be defined

as a BBS. You must press ENTER once more to stop editing the user. Repeat

this procedure with the callsign of all the BBSs that you will forward to.

You terminate the keyboard-session with the command B (Bye).

Well, if all is ok, and you have not encountered any big problems so far,

you can stop the BBS (you do this with Alt-F10) now. You can connect the

TNCs, and in INIT.SRV you must change from No to Ok in the line that asks

'Mode test ?', or use INSTAFBB to change the field "TEST MODE" to "N".

FBB 5.15                          Feb 06 1993                        Page - 37

5.13 TNC.

----------

This software will work with different kinds of TNCs or TNC-interfaces:

- TNC-2 or close clones. The EPROM of type 27256 must be replaced by a

special EPROM with the WA8DED host-mode software, or by the german TF4, TF8

or TF18.

- PK-232 in host-mode.

- Kantronics KAM in host-mode. You need a special driver called ESSKAM.

- DRSI-card.

- G8BPQ-node (uses TNCs in KISS-mode).

The software for the TNC-2 host-mode EPROMs TF4 and TF8, are on the

distribution-disks. If you cannot burn new EPROMs yourself, there are a lot

of hams around who can. You can also send a new EPROM to F6FBB or FC1GHV

(with return-postage and packing material, of course) and they will do the

work for you.

The TNC should work on 4.9 MHz. The standard is 2.5 MHz, but a modification

for this is normally quite simple. In some cases it might be necessary to

replace the Z80 and the Z80 SIO by A-versions.

If you use standard TNC-2 with one of the mentioned host-EPROMs, you can

start again now, and all should work fine. Make sure that the TNCs have the

same baud-rate as you wrote in PORT.SYS, and that they really are connected

to the COM-ports that you said in PORT.SYS.

If you do NOT use a standard TNC-2 with DED-type host, but PK-232,

Kantronics KAM, DRSI-card or BPQ-node, you must read about your special TNC-

setup in one (or more) of the chapters 5.14 to 5.17 before you can start the

BBS again.

If the TNCs don't work ok with the software, you will see this when you

start the BBS. You will have a lot of error-messages on the screen, and the

TNCs may start to resync. Stop the program and go throug all the setup once

more.

FBB 5.15                          Feb 06 1993                        Page - 38

5.14 PK-232 in host-mode.

--------------------------

If you plan to use a PK-232 with this sofware, you must make some changes.

First, check again PORT.SYS. You must have a P in "Type host mode". For

example:

#

#TNC NbCh Com MultCh Pacln Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    7    1   1      230   4     1     10     30/60   UPYW 433.650

2    1    3   1      80    2     1     5      17/30   GPW  15/20m

#

Example of INITTNCx.SYS for PK-232:

UNMAIL v LA7QR   means Unproto MAIL v LA7QR

RY10               "   Retry 10

MN6                "   Monitor 6

MC6                "   Mcon 6

Example of MAINTx.SYS for PK-232:

UR1

CTBBS ($c) was shut down for service $d $T.

PK-232 host-mode commands (from F6AIW) :

8B 8BITCONV AU AAB      AB ABAUD    AG ACHG     AA ACRDISP

AK ACRPACK  AT ACRRTTY  AE ADDRESS  AD ADELAY   AI ALFDISP

AP ALFPACK  AR ALFRTTY  AL ALIST    AM AMTOR    AC ARQ

AO ARQTMO   AS ASCII    AY ASPECT   AW AWLEN    AV AX25L2V2

AX AXDELAY  AH AXHANG   BA BAUDOT   BE BEACON   BI BITINV

BK BKONDEL  BT BTEXT    CL CANLINE  CP CANPAC   CX CASEDISP

CU CBELL    CC CCITT    CF CFROM    CB CHCALL   CD CHDOUBLE

CH CHSWITCH CK CHECK    CQ CMDTIME  CM CMSG     CI CODE

CN COMMAND  CE CONMODE  CO CONNECT  CY CONPERM  CG CONSTAMP

CI CPACTIME CR CRADD    CT CTEXT    CW CWID     DS DAYSTAMP

DA DAYTIME  DC DCDCONN  DL DELETE   DF DFROM    DI DISCONNE

DW DWAIT    EA EAS      EC ECHO     ES ESCAPE   FA FAX

FN FAXNEG   FE FEC      FL FLOW     FR FRACK    FS FSPEED

FU FULLDUP  GR GRAPHICS HB HBAUD    HD HEADERLN HI HID

HO HOST     HP HPOLL    ID ID       IL ILFPACK  IO IO

JU JUSTIFY  KI KISS     LR LEFTRITE LO LOCK     MX MAXFRAME

MB MBX      MC MCON     MD MDIGI    MM MEMORY   MI MFILTER

MF MFROM    MH MHEARD   MN MONITOR  MO MORSE    MP MSPEED

MR MRPT     MS MSTAMP   MT MTO      MA MYALIAS  ML MYCALL

MG MYSELCAL MK MYALTCAL NE NEWMODE  NO NOMODE   NR NUCR

NF NULF     NU NULLS    OK OK       OP OPMODE   PA PACKET

PL PACLEN   PT PACTIME  PR PARITY   PS PASS     PX PASSALL

PE PERSIST  PP PPERSIST PC PRCON    PF PRFAX    PO PROUT

PY PRTYPE   RW RAWHDLC  RB RBAUD    RC RCVE     RE RECEIVE

RX RXREV    RD REDISPLA RL RELINK   RS RESET    RP RESPTIME

RT RESTART  RY RETRY    RF RFEC     SE SELFEC   SP SENDPAC

SI SIGNAL   SL SLOTTIME SQ SQUELCH  SR SRXALL   ST START

SO STOP     TB TBAUD    TC TCLEAR   TM TIME     TR TRACE

TW TRFLOW   TI TRIES    TD TXDELAY  TF TXFLOW   TX TXREV

UN UNPROTO  UR USERS    US USOS     VH VHF      WI WIDESHFT

WO WORDOUT  WR WRU      XW XFLOW    XM XMIT     XO XMITOK

XF XOFF     XN XON

FBB 5.15                          Feb 06 1993                        Page - 39

5.15 Kantronics KAM in host-mode.

----------------------------------

If you plan to use a KAM with this sofware, you must make some changes.

Before you start the BBS, you must load the RS232 driver (ESS). The driver

is now working with KAM from the version 1.10. Do not use another driver such

as COMBIOS, MBBIOS or ESSKAM.

Run it like this:

ESS 1             (Com1, Adress and IRQ = defaults)

ESS 1 03F8 4      (Com1, Adress=03F8Hex, IRQ4)

The KAM must be in the right baudrate, and in hostmode. You put it into

hostmode with these commands:

INTF HOST

PERM

There is a special format for INITTNCx.SYS. For HF, use this:

UNPROTO MAIL/

MONITOR ON/

MCOM ON/

MCON ON/

MRESP ON/

PID ON/

RETRY 15/

RESPTIM 0/

FRACK 2/

CHECK 30/

HID OFF/

For VHF use this:

UNPROTO /MAIL

MONITOR /ON

MCOM /OFF

MCON /ON

MRESP /ON

PID /ON

RETRY /8

CHECK /30

HID /OFF

As you can see, the position of the "/" is very important for HF/VHF !

In PORT.SYS you must use 1 for VHF and 2 for HF in MultCh. You must also use

K in "Type host-mode".

#

# Same number of lines as number of TNCs.

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    4    1   1      230  4     1     10     30/60   UKYW 144.675

2    1    1   2      80   1     1     6      15/60   UKYW HF

#

FBB 5.15                          Feb 06 1993                        Page - 40

5.16 DRSI-card.

----------------

If you plan to use DRSI-card with this sofware, you must make some changes.

Before you start FBB, you must load the DRSI-driver that comes with the DRSI-

card. This must be TNCTSR-R or TNCTSR-L with a version-number higher or equal

to 2.1.

The first DRSI-card (MultCh 0 and 1) must be at adress 300.

The second DRSI-card (MultCh 2 and 3) must be at adress 310.

The third DRSI-card (MultCh 4 and 5) must be at adress 308.

The fourth DRSI-card (MultCh 6 and 7) must be at adress 318.

Before installing cards in the PC, use the program CHKADDR to verify that

the adresses corresponding to the cards have a value of FF. If there is a

problem, check your configuration to find what peripheral is already using

this address. Install just one card at the time, and verify each cards

presence with CHKADDR.

When all cards are ok, configure the driver by means of the TAILORnn program

(depending of the version). TNCTSR-S will be ok if you use only a few

channels, while TNCTSR-L should be used for big configurations, up to 32

channels.

One problem: What DRSI calls DRSI-PORT, is what we call MultCh in FBB ! What

DRSI calls PORT, is what FBB calls port or TNC ! We always use FBB-names

here...

Each DRSI-card works like 2 radio-ports. The first DRSI-card is always

MultCh 0 and 1, and the forth card is MultCh 6 and 7. If one radio-port is

used on HF, that port must be the second port on the card.

All the DRSI-cards together, work like only one COM. And this COM-port does

not need to exist in the PC. You may call that port COM 7 or COM 8 and keep

free the already existing COMS.

Baud-rate does not matter, but should be set to a standard value, to avoid

error-messages.

Here is an example of PORT.SYS with 2 DRSI-cards (4 radios) :

#

#Ports TNCs

1      4

#

#

#Com Interface Adress (Hex) Baud

7    4         0            4800

#

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    8    7   0      230  4     1     10     30/60   UDYW 433.650

2    1    7   1      80   2     1     5      12/30   GDW  15/20m

3    8    7   2      230  4     1     10     36/60   UDYW 433.650

4    8    7   3      230  4     1     5      10/30   GDW  145.300

#

If you are using DRSI-card, you also must change the INITTNC1.SYS. You will

need only ONE file for all the ports.

FBB 5.15                          Feb 06 1993                        Page - 41

Example of INITTNC1.SYS that initializes 4 ports:

MUIS

U0

p0 1 64 10 4 4 10 100 18000 30 2 0

p1 1 64 10 4 1 16 100 18000 30 1 0

p2 1 64 10 4 4 10 100 18000 30 2 0

p3 1 64 10 4 4 10 100 18000 30 2 0

The same goes for MAINT1.SYS, only 1 file is needed for all 4 ports, like

this:

Y 1

U 1 BBS ($c) was shut down for service $d $T.

In FORWARD.SYS there is no changes. Use standard syntax.

Well, I think that should be all (for DRSI). Oh, bye the way: DRSI cannot

limit the number of connections per radio-port. But in FBB we still can have

NbCh set to for example 1. When a station then connects to the BBS on that

radio-port, and there already is one user connected there, the BBS will send

a message to him: "Sorry, no more channels available" and disconnect him.

FBB 5.15                          Feb 06 1993                        Page - 42

5.17 G8BPQ-node (uses TNCs in KISS-mode, DRSI cards, etc...).

-------------------------------------------------------------

If you plan to use BPQ-node with this sofware, you must make some changes.

Before you start FBB, you must load the BPQ-program.

From version 4.05 of BPQcode, separate ports can be declared. Level 2

connections issue to the MultCh in the same order as PORTs declared in the

BPQCFG.TXT (G8BPQ distribution) file. First port corresponds to MultCh 0.

Level 4 connections (from the network) always issue to MultCh 0.

In PORT.SYS you must use 2 in INTERFACE, and Q in type host mode. Version

4.05 or up is recommended. COM can be from 1 to 8, but will exclude existing

COMs. Chosing 8 is a good solution to keep existing COMs.

Only the INITTNCx.SYS for the 1st port must exist. The parameters for other

ports will be taken by default, and then no conflict should happen between

ports. If you want to define the streams differently on each port, you can do

it with the INITTNCx.SYS file, but be sure to give the right configuration.

If paclen is equal to 0, then the default BPQ paclen of the port will be

taken, otherwise the specified paclen will overwrite the default BPQ paclen.

If you have validated the gateway, you must configure ENABLE_LINKED=A in

BPQCFG.TXT to give to the stream the callsign of the user.

Here is an example of PORT.SYS with BPQ-node and 2 TNCs:

#

#Ports TNCs

1      2

#

#

#Com Interface Adress (Hex) Baud

8    2         0            4800

#

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    8    8   0      230  4     1     10     30/60   UQYW VHF

2    1    8   1      80   1     1     5      30/60   UQYW HF

#

Example of INITTNCx.SYS for BPQ 4.05 (Only ONE INITTNCx.SYS) :

A1      Number of application

R0      Returns to node (0=No,1=Yes)

UMAIL   Unproto MAIL

M1      Monitor (0=No,1=YES)

N1      Number of first stream

The parameters for other ports will be taken by default, For instance, as the

first stream of the first port is 1, the first stream of the second port will

be 9. (8 channels defined in PORT.SYS for port 1). This will be done

automatically, and the second INITTNCx.SYS should not exist.

In FORWARD.SYS there are a few changes, here is a short example:

A LA2D

#

P B

#

C C SWITCH       Connect first BPQ switch.

FBB 5.15                          Feb 06 1993                        Page - 43

C C 2 LA2D       Connect LA2D on port 2 of BPQ.

#

B LA2D

F LA2D

#

---------

FBB 5.15                          Feb 06 1993                        Page - 44

6 BBS COMMANDS:

---------------

? :

Reading the help file. Type in "?[COMMAND]" to obtain help on a basic

command.

> :

"Break" between channels. Type in > [CALLSIGN] [TEXT] to send a text to a

connected callsign on another channel. You will get back a message from the

machine telling you that the text has been delivered. This text cannot be

longer than one line.

= :

Connection between two channels. Type in = [CALLSIGN] to get connected to a

callsign on another channel. The callsign can be connected only if it is not

busy in a command; the BBS should be idle (waiting for a command). If the

callsign is not available, either you wait or you cancel your request. The

callsign currently busy in forwarding tasks cannot be connected. A Ctrl-Z is

used to leave this mode.

! :

Provides a short and minimal information on the BBS usage.

A :

Aborts the BBS output at any time.

B :

Disconnects the BBS.

C :

Enters the conference mode, multiconnection. The command CW yields the list

of the callsigns involved in the current conference.

Any text typed in while in the conference is sent out to all the parties

involved. A header before the text shows its origin. The header is not

repeated if the next text has the same origin.

In the conference, the commands should begin with a semi-colon in the first

column, so as to be interpreted.

Commands in the Conference mode:

.C[PORT] CALLSIGN : connection to a callsign on a port. The port indication

is optional; the port of the connected user is then choosen as a default.

.D CALLSIGN : Disconnects the callsign from the conference.

.H : Help

.W : List of the participants in the conference.

.Q : Exit of the conference.

D :

Invokes the FBBDOS or Receive a file from the BBS if the command is followed

by a file name.

F :

Switches to Server mode, and access to special commands.

G :

Access to the Gateway.

FBB 5.15                          Feb 06 1993                        Page - 45

Gateway commands:

B : Exit the Gateway

K : Enters Converse mode.

C : Connect.

D : Disconnect.

H : Help

J : Lists the 20 last stations heard on the port.

P : Link from one port to another.

Once the selection of an authorized port has been made, a channel will be

granted,  depending  upon  availability.  This  channel  will  use

callsign. Switching to converse mode is automatic when the corresponding

station is connected; switching back to command mode takes place on a

disconnection, or if the Esc character or a > in the first column is sent

out.

Switching to converse mode can also be made with the command K. The C command

must be followed by the destination callsign, and optionnaly by the letter V

followed by a string of digipeater callsigns.

C Destination call V repeat1 repeat2 ...

H :

Same meaning as the ? command.

I :

Informations about this BBS. This command displays the file FRANCAIS.INF (or

another language depending upon the configuration and the language attributed

to the involved channel).

I callsign :

Gives the informations on callsign taken from the white pages database.

Callsign can include wildcards.

I@ bbs :

Gives the users of BBS found in the WP database.

IH route :

Gives the users of the hierarchical route area from WP database (wildcards

are allowed).

IZ Zip :

Gives the users of the ZIP code area from WP database (wildcards are

allowed).

J :

Lists the callsigns heard by or connected on the BBS.

JK : Shows the last 20 connected callsigns.

JA : Connected callsigns on port A.

JB : Connected callsigns on port B, etc...

J1 : Heard callsigns on port A.

J2 : Heard callsigns on port B, etc...

K :

Deletes the messages sent by you or to you.

K msg# :

Deletes a message identified by its number.

KM :

Deletes all the messages addressed to you, that you don't yet read. The

messages not read will not be deleted.

L :

Lists the new messages since the last usage of this command.

FBB 5.15                          Feb 06 1993                        Page - 46

LB :

Lists the Bulletins.

LC [mask]

Lists the Messages which "TO" filed matches with mask. * (star) allows to

list all the messages and should be the default value. Type "LC *" to see all

messages. LC alone gives the current mask. Mask is recorded for each user

while disconnecting.

LD> YYMMDD :

Lists the Messages received after the specified date.

LD< YYMMDD :

Lists the Messages received before the specified date.

LM :

Lists the TO YOU mesages.

LN :

Lists the NEW messages TO YOU.

LL 10:

Lists the last 10 messages.

LR :

is equivalent to L but the messages are listed in reverse order.

LS text :

Searchs for a text in the messages titles. Usage of wildcards is allowed.

L< Callsign :

Lists the message originated from [Callsign]. The wildcards are allowed.

L> Callsign :

Lists the messages destinated to [Callsign]. The wildcards are allowed.

L@ [BBS] :

Lists the messages VIA BBS if specified. L@ alone lists the message without

route. The wildcards are allowed.

L msg#- :

Lists all messages, starting from msg#

L msg#-msg# :

List of messages between the given numbers.

M msg# File :

transfer the text of the message msg# to the file File. This file will be

placed in the directory USERS or from the root in case of a SYSOP user.

MH msg# File :

Identical, but the format is the R command.

MV msg# File :

Identical, but the format is the V command.

MA msg# File :

transfer the text of the message msg# to the file File. This file will be

placed in the directory USERS or from the root in case of a SYSOP user. The

message is appended to the file.

MHA msg# File :

FBB 5.15                          Feb 06 1993                        Page - 47

Identical, but the format is the R command. The message is appended to the

file.

MVA msg# File :

Identical, but the format is the V command. The message is appended to the

file.

N :

Entry or modification of the name (max 12 characters allowed).

NH :

Entry or modification of the Home BBS. A point deletes the current Home BBS.

NL :

Entry or modification of the QRA locator.

NP :

Modification of the password for the access via telephone modem.

NQ :

Entry or modification of the city (Qth).

NZ :

Entry or modification of the Zip Code.

O :

Display or setting of various options.

OL :

Lists the available languages.

OL number :

to select a language.

ON number :

defines the number of the thousand base for the messages. Example: ON 37

defines number 37000 as being the base for numbering the messages. Then the

command R 25 will cause the message 37025 to be read, although R 36025 will

actually read the message nr 36025.

OP :

validates or disables the scrolling of pages.

OP nbr of lines :

Initialization of the page scrolling

OR :

Validates or disables the list of all messages if the user has access to the

whole list of messages (See security code 1 of INIT.SRV).

PG [program]:

PG alone gives the list of PG programs available in the BBS. If followed by

a program name gives the control to this program. The PG program can also be

called by its name. If the name is the same that an existing command, the

existing command will be replaced with the PG program.

PS :

Gives the list of servers available in the BBS.

R :

Reading of messages.

R msg# msg# :

FBB 5.15                          Feb 06 1993                        Page - 48

reads the messages by their numbers.

RM :

reads all the messages TO YOU.

RN :

reads all the NEW messages TO YOU.

S[type] callsign :

will send a message or a bulletin. If the type is not specified, it will be

processed as a private, provided the destination callsign is a valid one,

otherwise it will be considered as a message.

SP callsign :

sends a private message.

S callsign @ BBSCALLSIGN :

sends  a  message  to  a  station  AT  another  BBS  through  "fo

BBSCALLSIGN  can  be  an  address  of  hierarchical  type,  for

F6FBB.FRA.EU. Type in "SB ALL" to send a bulletin addressed to ALL. The

message must be terminated by a Ctl Z or /EX followed by a return. Warning:

the /EX must be in the first column.

SC Msg# Callsign @ BBSCALLSIGN :

Copy of a message or bulletin to the specified callsign. Such a copy will

always be of the "personnal" type.

SR [Msg# [title]] :

Reply to a message already read or formerly suppressed, or to a message

matching the specified number. If the title is not stated, it will be the

title of the read message, with the mention "Re:" placed ahead.

T :

Paging the SysOp. Should the SysOp be not available within a minute, you

will be advised accordingly and automatically returned to the normal BBS

menu.

U :

Sends a file from the user to the server of the BBS. The file will be

written in the root directory of the FBBDOS. The U command is to be typed in

at the BBS prompt, it is not necessary to go to FBBDOS before.

V :

Version number of this BBS software. It gives also the number of active

messages, and the next message number.

V msg# msg# :

reads the messages by their numbers, including the headers.

VM :

reads all the messages TO you, including the headers.

VN :

reads all the NEW messages TO you, including the headers.

W :

Lists the files which are within the user directory. This command can be

followed by a mask specifying search criteria or a directory name; in the

latter case, the directory content will be listed.

X :

Toggles "normal" or "expert" status. "Normal" grants the extended menus,

"expert" provides only minimal informations.

FBB 5.15                          Feb 06 1993                        Page - 49

Y :

Binary transfer program using the YAPP protocol. As a user, you should also

have the YAPP protocol available your end, in order to be able to transfer

binary data.

YD File :

Reception of a file FROM the BBS TO you.

YI [mask] :

lists the available binary files, and their description.

YN [mask] :

lists the NEW binary files since your last connection.

YU File :

transmission of a binary file from you TO the BBS. You cannot replace or

modify an already existing file.

YW [mask] :

lists the available binary files.

YZ Filename :

deletes the specified filename in the YAPP directory.

Z Filename :

deletes the specified filename in the user directory.

FBB 5.15                          Feb 06 1993                        Page - 50

7 SERVER COMMANDS :

-------------------

7.1 Main menu :

---------------

The server commands are gathered in groups.

C : Connections - Statistics.

D : Documentation.

I : Informations, documentation.

Q : QTH locator.

N : Ham Callsigns Directory ("Call-Book").

T : Satellite orbits computation.

F : Switches back to BBS mode.

B : Quit and disconnects

FBB 5.15                          Feb 06 1993                        Page - 51

7.2 Connections - Stats:

------------------------

Statistical data about the operation of the Server and the Traffic Log.

O :

Activity of the different groups.

G :

Global statistics.

L :

List of connections.

H :

Hourly usage.

J :

Daily usage.

I :

Users Callsign list.

O :

Percentage of usage for the different activities.

G :

General informations on the server:

- Number of active messages.

- Number of connections.

- Times of connections.

- Peak traffic periods.

L :

Lists the connections (Traffic Log). For each and every connection, the

channel - callsign - date and time - as well as the connection duration are

indicated. These informations are listed in reverse chronological order.

H :

Histogramm by hours.

J :

Histogramm by days.

I :

List of the callsigns that have connected the server. This list is given in

alphanumeric order.

FBB 5.15                          Feb 06 1993                        Page - 52

7.3 Documentations.

-------------------

You will find under these comments the informations or documentations given

by various hams.

The documentation is organized like the image of a tree structure whose root

is defined in INIT.SRV (C:\DOCS). The directories represent the topics and

the files represent the documentation.

The selection of the topics or the documentation is made through numeral

command. The menu is presented with the form of one or several numbers

followed with their description (topics or documentations).

In the case of topics, the description corresponds to the name of the MsDos

directory. This name is eleven characters long, the dot between the name and

the extension will not appear. It is possible also to simulate the space

character with an underscore character.

In the case of documentation files, a label of 30 characters is associated

to the name of the MsDos file. The label only will be presented to the user.

To optimize the fill up of the screen, each line will contain two labels.

In the case of a user, given the right of SYSOP, the display will be

different. One line will contain only the name of the MsDos file followed by

its label.

L gives the list of the documentation on the current chapter.

Labellizing of the files is made by the command D, reserved to SYSOP, or

automated through the NEWDOC server (See paragraph 15 : SERVERS).

Example : D BBS.DOC Documentation of the BBS.

FBB 5.15                          Feb 06 1993                        Page - 53

7.4 Callsigns Directory.

------------------------

You will find in this module the informations that the users will have

accepted to leave there: name, first name, address, phone number, etc...

You will be able, as well, to update your own various informations.

N :

to modify your name and address.

R :

to search for informations about a user.

I :

to list the connected callsigns.

N :

Modifies your name and address. If these informations in the file are

erroneous, or should they have changed, you can modify them with this

command. A space or a return as an answer to the question will not modify the

content. If  the  content  of  the  file  is  correct,  you  can

modification by replying N to the modification request.

R : Search for information about a user. If the user is known in the file,

you will get informations for him. You can concatenate the callsign of the

user who is searched for behind the command.

Ex.: R F6FBB will give informations about F6FBB.

I : lists the callsigns of the users who connected the server. This list is

sorted in an alphanumeric order.

FBB 5.15                          Feb 06 1993                        Page - 54

7.5 QTH locator.

----------------

The QTH locator is made of 2 letters, 2 figures, and 2 letters yielding the

geographical coordinates of a station. You can translate the QTH locator into

Longitude/Latidude or vice-versa, compute the distance and azimut between two

QTH  locator  or  compute  your  Log-Sheets  for  contests  (summi

distances).

Q :

Converts QTH Locator into Longitude/Latitude.

L :

Converts Longitude/Latitude into QTH Locator.

D :

Distance and Azimut computation between two QTH Locators.

C :

Sum up all distances between one QTH Locator and several other

QTH Locators.

F :

Switches back to the Server main menu.

B :

Quit and Disconnect.

Q :

Converts one QTH Locator into Longitude and Latitude. The West longitude and

North latitude are given in degrees minutes, or grades.

L :

Converts the Longitude and Latitude into QTH Locator. The west longitude and

north latitude can be given in centesimal degrees, degrees minutes, or

grades.

D :

Computation of the distance and azimut between two QTH Locators. The

distance is given in Kilometres and the azimut in degrees.

C :

Sum up the distances. After having entered the QTH Locator which will be

used as the origin for the computation, you will enter the QTH Locator of the

destination. The computed distance can then be summed up with a second or

several other computations, thus allowing you to get the total number of

kilometres.

FBB 5.15                          Feb 06 1993                        Page - 55

7.6 Satellite orbital computation.

-----------------------------------

Satellite orbital parameters computation, and satellite characteristics.

P :

Display of orbital parameters.

C :

Amateur Satellites characteristics.

F :

Switches back to Server main menu.

B :

Quit and disconnects.

T :

Satellite orbital computation.

To select a satellite, type the number (a list of the satellites can be

obtained with the command L instead of the number). Once the satellite has

been selected, you will have to supply the date and the time to be taken into

account for the computation. If you only answer with a return character, the

current date (or time) will be taken. The geographical position is the QTH

Locator of the connected user, if it is known in the file. Otherwise, the QTH

Locator of the server will be used as a default. Depending on the satellites,

the computation step can be variable, and one line of parameters will be

displayed per step. This line corresponds to a minimal elevation of -5 degree

s, that means the satellite 5 degrees below the horizon.

P :

Selected satellite orbital parameters display. These parameters will be used

for the computation.

C :

Characteristics and informations regarding the selected satellite.

FBB 5.15                          Feb 06 1993                        Page - 56

8 SYSOP COMMANDS :

------------------

8.1 Commands available under the BBS menu.

------------------------------------------

DU callsign :

Displays the informations related to the given callsign.

DB :

Lists the BBS callsigns.

DE :

Lists the EXCLUDED callsigns.

DS :

Lists the SYSOPS callsigns.

DX :

Lists the EXPERT callsigns.

DP :

Lists callsigns which are using the page scroll.

E msg# :

Edits the message # header. The message header is first displayed, then a

string of options will allow you to edit the various fields of this header. A

"return" terminates the edit.

EU callsign :

Edits the main parameters of a user. The user's parameters are first

displayed, then a string of options will allow you to edit the various

parameters. A "return" terminates the edit. Most of fields can be cleared

with a point. A user can be edited while connected and then action is

immediate. Flags are :

R : All mail (included personnal) can be read

P : Paging validated

B : BBS

S : Sysop

X : Expert

L : Local (Allowed to all commands on guest access).

E : Excluded (cannot connect to the BBS)

M : Modem access

U : Unproto (Allows unproto list requests).

Other commands must be followed with the information :

G number   : selected language

N name     : name of the user

W password : password of the user

V directory: private directory of the user (12 characters max)

H home-bbs : home-bbs of the user

Z zip-code : zip-code of the user

FA msg# BBS :

Specifies that the message defined by its number will be forwarded to the

specified adjacent BBS.

FB BBS :

Gives the list of messages which are in the waiting list for this BBS.

FC Route :

FBB 5.15                          Feb 06 1993                        Page - 57

Gives the list of BBS routing the specified route. If the route is a

possible callsign, will look for BBS routing, if composed with digits will

look for NTS routing, otherwise will look for bulletins (group) routing.

FD msg# BBS :

Removes the message # from the forward to the specified adjacent BBS.

FH BBS :

Gives the hierarchical extension for BBS.

FI BID command:

BID maintenance.

Command = ? : Gives message number and the message type of the BID/MID.

+ : Adds the BID/MID to the list.

- : Deletes the BID/MID from the list.

FL [BBS] :

Lists the messages in the queue (for BBS if specified).

FN Msg# :

List of the BBS which are in the waiting list for this message number, and

the BBS already forwarded or named in the preamble.

FP command callsign:

Swap test.

Command = @ : Gives the swap (if existing) for the via callsign.

> : Gives the swap (if existing) for the to callsign.

< : Gives the swap (if existing) for the from callsign.

FR BBS :

Forces reverse forwarding to BBS, even if there is no pending mail.

FS Argument :

Stops the forwarding. If the argument is a callsign of known BBS, the

forwarding to this BBS will be stopped. Although the argument can be a port

number, in this case the forwarding will be stopped on that port, if 9 is

specified then the forwarding will be stopped for all the ports.

FT [BBS] :

Lists the volume of the messages which are waiting for the BBS if it is

specified.

FU Channel :

Immediately disconnects the specified channel.

FV :

Recompiles FORWARD.SYS and its include files, SWAPP.SYS and BBS.SYS and then

starts scanning the messages to update the forwarding list. This can be

needed after a FORWARD file change.

FW Argument :

Initiates the forwarding if the argument is a callsign of a known BBS. The

forwarding throught this BBS is started and the parameters of FORWARD.SYS

file are taken into account. The parameter can also be the number of a port,

in which case the forwarding will be started for this port, if 9 is specified

the forwarding will be initiated for all the ports.

IE Callsign :

Gives first the WP information using IL format (see below), and then prompts

the deletion of the record. If the answer is Y, the record will be deleted,

else you enter the edition mode. Type return to exit edition mode. The

commands are :

U          : Updates primary part with the temporary part (2 -> 1).

FBB 5.15                          Feb 06 1993                        Page - 58

N  name    : Changes the name of the user.

1H Route   : Changes the primary route.

1Q Qth     : Changes the primary Qth.

1Z Zipcode : Changes the primary ZipCode.

2H Route   : Changes the temporary route.

2Q Qth     : Changes the temporary Qth.

2Z Zipcode : Changes the temporary ZipCode.

IL Callsign :

Gives all the information from the white pages database.

Kx : The fact that a leading K is entered before a kill command with the

letter K (I.E. KK, KK<, ...) triggers immediate killing of the specified

message. The archiving will be done at the next epurmess process according to

epurmess parameters. The status of the message becomes "A". The syntax is

identical to the command "K".

K> callsign : Deletion of any message to this callsign.

K< callsign : Deletion of any message sent from this callsign.

K@ route : Suppresses all the messages which have the specified route.

KF : Suppresses all the messages already forwarded.

$ msg# : Informations about msg# forwarding: lists the BBS which have not yet

received the message #.

LU (RU) : Lists (or displays) all the private messages which have not yet

been read by their addressee.

Mx : Same as the command "L", with the same syntax, but the BID is inserted

at the beginning of the field "SUBJECT". This command allows you to do some

search or to visualize BIDs in the message lists (commands MS, ML, etc...).

PR : Enables or disables the printer from the console. The printer is

automatically disabled on disconnect.

PR[V] #Msg : Sends [verbose] message to the printer.

PR> filename : validates and redirects the printing to a filename.

PR> -        : validates and redirects the printing to the standard printer.

SR [#msg] [+filename] [subject]

Reply to a message already read or formerly suppressed, or to a message

matching the specified number and appends the contents of filename. If the

subject is not stated, it will be the title of the read message, with the

mention "Re:" placed ahead.

SYS : Enables remote Sysop mode. A callsign validated as SYSOP is permitted

to access any message in any circumstance, but he cannot delete or modify

data which do not belong to him. The SYS command will grant him these

privileges, together with the ability to enter the root of the DOS directory

tree  in  FBBDOS.  The  SYS  command  will  return  a  string  of

corresponding to the position of a letter in a sentence located in the

PASSWD.SYS file. If you send back the 5 letters (without any space) matching

the 5 numbers, an OK will be sent out to confirm that the SYSOP mode is

granted.

YL file : Modifies the description of an existing YAPP file.

FBB 5.15                          Feb 06 1993                        Page - 59

8.2 Other commands.

-------------------

M :

In the SATELLITE ORBITAL COMPUTATION menu, manual modification of the

orbital parameters of the satellites.

N Callsign :

In  the  NOMENCLATURE  menu,  this  command  allows  to  edit/mo

nomenclature informations of a user.

S file :

In the gateway and in command mode, send an ascii file.

W file :

In the gateway, opens a capture file. The file is closed with another W

command.

YU Filename :

In gateway mode and only from the console send a binary file to a remote

system under the YAPP protocole.

YD Filename :

In gateway mode and only from the console receive a binary file from a

remote system under the YAPP protocole.

FBB 5.15                          Feb 06 1993                        Page - 60

9 FBBDOS COMMANDS :

-------------------

The syntax of the various FBBDOS commands is very close to that of MsDos

(R).

Several specific commands have been added: PUT and GET for ASCII file

transfers. XPUT, YPUT and XGET, YGET for binary file transfers.

EDIT for ASCII files edition.

When entering FBBDOS, the prompt shows the virtual directory in which the

user is located. There is no physical drive concept (A, C, etc...), the user

is situated in a directory defined in the INIT.SRV file; this directory

becomes the root of the available directory tree. Should the user be a SYSOP,

and if he successfully validated his privileges using the SYS command, the

root of FBBDOS will be the root of the current drive.

A file created by a user belongs to this user.

Only the owner of the file, or a SYSOP granted writing privileges (SYS

command) are permitted to delete or modify this file.

CD directory :

Changes the active directory. This command allows you to move within the

directory tree by changing the active directory. "CD.." changes back by one

directory "upstream" towards the root.

COPY Origin Destination :

Copies a file into another file. If the destination file already exists, it

must belong to the user.

DEL file :

Deletes a file. A file cannot be deleted if it does not belong to the user.

DIR [mask] :

Displays all the files contained in the directory or these corresponding to

the mask if it is specified. If the mask matches a directory name, the

content of this directory will be listed.

DOS Program Arguments :

Allows a remote SYSOP (after SYS command) to run a DOS program. This program

can not expect keyboard input. A list of arguments can be specified. The

output to the screen will be send to the remote SYSOP at the completion of

the program. If the program expects inputs, after a time-out of 2'45, it will

be aborted (if possible) and the normal operation will resume.

DU [mask] :

Gives the size of the tree, and local used disk.

EDIT file :

Edition of a text file, using the line editor. If the file already exists,

it must belong to the user. The instructions to use the line editor can be

found in the Appendix 1.

EXIT :

Exits FBBDOS and brings back to the BBS or server.

FBB 5.15                          Feb 06 1993                        Page - 61

GET file :

Displays the content of an ASCII file. Such a display is never scrolled by

pages, and ends with a Ctrl-Z, thus allowing ASCII file transfers from the

BBS to the user.

LIST mask :

Displays all the files contained in the directory or these matching the mask

specified. if the mask matches a directory name, the content of this

directory will be listed. Each line displays the filename and a descriptive

comment (cf YI command).

MD directory :

Creates a new directory. The new directory is appended next to the current

directory.

O :

Display or setting of various options.

OL :

Lists the available languages.

OL number :

to select a language.

OP :

validates or disables the scrolling of pages.

OP nbr of lines :

Initialization of the page scrolling

PRIV :

Access to a private directory (or return to Fbb-Dos directories). Each user

can have access to a private directory provided it has been defined by the

SYSOP (I.E. EU command). Several users can share a private directory.

PUT file :

Accepts an ASCII file from a user. The file must end with a Ctrl-Z to be

validated. If the file already exists, it should belong to the user.

RD directory :

Deletes a directory. A directory cannot be deleted as long as it is not

empty.

TYPE file :

Displays the content of an ASCII file. Eventually, this list can be scrolled

by pages.

VIEW mask :

Displays the content of all the archive files contained in the directory and

corresponding to the specified mask. This function uses the FV.COM shareware.

Thanks to the authors of this last one.

XGET file [offset]:

A binary file is sent by the BBS. The Xmodem protocol is used. This command

works only on a PSTN modem port. If offset is specified, download will start

from offset.

XPUT file :

The BBS receives a binary file. The Xmodem protocol is used. This command

works only on a PSTN modem port.

YGET file :

FBB 5.15                          Feb 06 1993                        Page - 62

Transmission of a binary file by the BBS. Both YAPP and YAPC protocols are

used.

YPUT file :

Reception of a binary file by the BBS. Both YAPP and YAPC protocols are

used.

FBB 5.15                          Feb 06 1993                        Page - 63

10 CONSOLE COMMANDS :

---------------------

Most of these commands will open a window. The server operations is

suspended while this windows stays open, but the user requests are queued and

will not be lost. If no reply is given, the window will close down after a

few seconds, and the default reply will not be taken into account. Do not

forget to validate your replies with "Enter".

F1 General help related to commands attribution.

F2 Connection to the server in local mode with the callsign specified after

"Console:"

F3 Manual disconnection of one channel.

F4 Manual forwarding request, or cancellation of current forwarding. If a

number up to 8 is specified, the corresponding port will be scanned for

forwarding. If the number 9 is specified, all ports will be scanned. If a

callsign is specified, it will be scanned for forwarding. Preceeding the

callsign with + will force reverse even if there is no waiting mail,

preceeding the callsign with - will stop the forwarding and disconnect.

F5 Split screen. The upper part is for internal server traffic purpose, the

lower part shows the traffic flow on the various channels. The up and down

arrow keys are used to move the separation border between the two windows.

Pressing F5 again toggles back to a single window display.

F6 Used to change the console callsign. Thus, you can simulate a call from a

user.

F7 Direct commands to the TNC. This allows a dynamic modification of the

parameters which will be affected as default to the channels of the related

port. These parameters will not be taken into account for the channels

already connected, and will be transferred to the channel

upon disconnection. The commands are specific to the TNC in use: WA8DED with

a TNC2, HostMode PK232 with a PK232, or the BPQ node.

F8 Server reboot. The cleaning and the housekeeping of the mail is not done.

The reboot can take place at once (immediate disconnect of channels in use),

or delayed (awaiting the normal disconnect of all the active

channels).

F9 Traffic with the console callsing (Gateway mode).

F10 Switches to chat mode with the user who requested this mode (using the T

command), or end of chatting (user is returned to normal BBS usage).

ALT-F1 Displays the volume of the forwarding queues sorted per BBS.

ALT-F2 Displays the list of the stations connected at the time this command

is keyed in.

ALT-F3 Starts or stops the messages scanning after the modification of the

FORWARD.SYS file. Before starting scanning, FORWARD.SYS and its include

files, BBS.SYS and SWAPP.SYS are recompiled.

ALT-F4 List the last connections.

ALT-F5 Call the full screnn tiny-edit (cf para. 12).

ALT-F6 Miscellaneous options:

FBB 5.15                          Feb 06 1993                        Page - 64

- Valid call : Enables the T command.

- Connection Bip : Beeps on every connection.

- Display calls : Displays the channel and the callsign.

- Gateway access : Gatewaying between ports is enabled.

- Justification : The texts typed in at the console are justified.

ALT-F7 Forward imports out of an external file (MBL compatible).

ALT-F8 Forward exports to an external file (MBL compatible).

ALT-F9 Jump to DOS (EXIT to return). WARNING: the BBS operation is suspended

all this time long. The users will not be disconnected, and their commands

will be queued.

F10 Complete server shutdown. The shutdown can be immediate (disconnection of

the active channels), or delayed (awaiting the normal disconnect of all the

active channels).

ALT-B  Forces screenblank.

ALT-D  Writes the current screen buffer to a file

ALT-R  Redisplays the screen

ALT-X  Server shutdown (idem ALT-F10)

ALT-N  Allows reprogramming a telephone modem

ALT-S  Swap to DOS screen

Arrow -> Display next channel.

Arrow <- Display previous channel.

Ctrl -> ou Ctrl <- Return to the display of all channels.

PG-UP Display the previous page if it exists

PG-DW Display next page.

HOME Display the first page.

END Display the last page.

FBB 5.15                          Feb 06 1993                        Page - 65

11 TEXT VARIABLES :

-------------------

11.1 Variables definition.

--------------------------

Most of the text files support usage of variables. These variables enable

text or values to be placed, depending upon the context or the channels in

use.

These variables are defined with the $ sign, followed by a letter. The

meaning will be different if the letter is in capital or in lower case. A

second set of variables is defined with the % sign, followed by a letter.

They can also be followed by a figure and in this case, this is a parameter

whose value is function of the context. These particular variables are used

in texts, usually as computation results. Do not use them in other places

than text files (*.TXT).

The $ sign will have to be written $$, if it is not to be considered as a

variable. The % sign will have to be written %%.

FBB 5.15                          Feb 06 1993                        Page - 66

11.2 List of the variables.

---------------------------

$0 - $9 : Temporary variables.

$A : @ BBS of the message header.

$a : Year of the current date.

$B : Inserts a Ctrl-G (Bell).

$b : Zip code of the user.

$C : Number of the next message.

$c : City where the BBS is located.

$D : Current date (Format : YYMMDD).

$d : Current date (Format : DD-Dec).

$E : Version number.

$F : Minute of the hour for forwarding.

$f : File appended to the message header.

$G : Destination of the message header.

$g : Number of gateway ports available.

$H : Current time (Format : HH:MM).

$h : Home BBS of the user.

$I : First name of the connected user.

$i : Date and time of the message header (Format MMDD/HHMM).

$J : Date of the message header (Format YYMMDD).

$j : Date of the message header (Format 29-Dec).

$K : Time of the message header (Format : HH:MM).

$k : List of the users involved in the conference.

$L : Number of the last used message.

$l : Filter value of LC command.

$M : Number of the message header.

$m : Frequency of the port in use.

$N : Number of active messages.

$n : Size of the message header.

$O : BBS callsign.

$o : SSID of the BBS.

$P : Origin of the message header.

$p : Number of lines of scroll page option, for the connected user

$Q : List of the 8 callsigns which have new messages.

$q : Base messages number of the user.

$R : BID of the message header.

$r : L or space character. L means that the message was locally created.

$S : Subject of the message header.

$s : Status of the message header.

$T : Time of message header (Format : HH:MM).

$t : Type of message header.

$U : Callsign of the connected user.

$u : Current drive in FBBDOS.

$V : SysOp first name.

$v : Complete route (with the hierarchical structure).

$W : Inserts a carriage return.

$w : Inserts an escape character.

$X : Date of the last connection (Format : 881229).

$x : Date of the last connection (Format : 29-Dec).

$Y : Time of the last connection (Format : 12:30).

$y : Year of message.

$Z : Last message read by the user (L command).

$z : Zip Code of the BBS.

$* : Number of active channels.

$= : User's current channel.

$! : User's current port.

$^ : Number of allowed ports.

$? : BBS QTH-Locator.

$% : List of the connected users.

$: : Connection duration (Format : 12mn 05s).

$. : Machine operation time (Format : 12mn 05s).

FBB 5.15                          Feb 06 1993                        Page - 67

$$ : Character $

Second set of variables :

(unformatted means that text is not padded with spaces) :

%A : @ BBS of the message header, unformatted.

%G : Destination of the message header, unformatted.

%i : Date and time of the message creation (Format MMDD/HHMM).

%J : Date of the message creation (format YYMMDD).

%j : Date of the message creation (format 29-Dec).

%K : Time of the message creation (format HH:MM).

%M : Numer of the message, unformatted.

%n : Size of the message header, unformatted.

%P : Origin of the message header, unformatted.

%y : Year of the message creation.

%% : Character %

FBB 5.15                          Feb 06 1993                        Page - 68

12 FULL SCREEN TINY-EDIT :

--------------------------

The Tiny-Edit is started with the Alt-F5 command. Normal operation of the

BBS is maintained, the editing takes place in the console window, above the

monitoring area of the display.

Most of the text files can be edited with this editor despite of its

simplicity. The most concerning drawback is the fact that the line length can

not be longer than 79 characters. The language files (xxxx.TXT) therefore

cannot be edited.

Tabs are not generated and are presentend on the display with a special

character.

During the editing, the usual BBS command set are no longer available, they

are replaced by the tyni-edit commands.

Upon activation of the editor, a dialog window ask for the filename to edit.

The default directory is the directory "SYSTEM" as defined in INIT.SRV.

Examples: FORWARD.SYS will edit SYSTEM\FORWARD.SYS

FWD\F6FBB will edit SYSTEM\FWD\F6FBB

It will be necessary to specify the full path for a file outside of the

default directory root or located on another disk drive. If only the drive

name is specified, the current directory on this drive is searched for the

filename.

Example: C:INIT.SRV will edit C:\FBB\INIT.SRV (while the current directory

is c:\FBB).

To edit a message, one should give the message number with a leading pound

sign ('#').

Example : #42645 will edit the message number 42645.

In case the edited file is a compiled one (I.E. FORWARD.SYS) it will be

compiled again after the termination of tiny-edit

Main Commands :

---------------

F1 : Help windows.

F2 : The edited file is appended to the incoming mail file of the BBS

(MAIL.IN).

F8 : Save current file.

F9 : Save current file and exit.

F10 : Exit Tiny-Edit without saving ! Caution : acted upon ! no confirmation

requested before closure of the current edit session.

Edit Commands :

---------------

Almost all commands are available from the numeric key-pad.

up-arrow : the cursor goes up one line

FBB 5.15                          Feb 06 1993                        Page - 69

down-arrow : the cursor goes to next line

right-arrow : the cursor moves forward one character. At the end of the line

it goes to the beginning of the next line.

left-arrow : the cursor moves backward one character position. At the

beginning of the line it goes back to the end of the former line.

page-down : the cursor moves toward the end of the current page. If it is

already at the end of page position, then the next page will be displayed.

page-up : the cursor goes back to the first line of the current

page. If it is already at the first line, the former page will be displayed.

home : the cursor moves to the first character of the current line.

end : the cursor goes to the character position next after the

last character of the line.

insert : toggle the insert editing mode. The cursor is an

underscore while in insert mode. It is a cursor block in

overstrike mode.

delete : erase the character at cursor position, the cursor is not

moved.

back-space : the character preceding is deleted and the cursor

moves backward one position.

Ctl right-arrow : the cursor moves to the first character of the

next word.

Ctl left-arrow : the cursor moves to the first character of the

previous word.

Ctl home : the first page of the file is displayed and the cursor

goes to the first line.

Ctl end : the last page is displayed and the cursor goes to the

last line of the file.

Ctl Y : the current line is deleted.

Ctl Q : the current line is deleted from the cursor position up to

the end.

FBB 5.15                          Feb 06 1993                        Page - 70

13 EMS and XMS :

----------------

With the PC, DOS is drastically limited to 1 Mo. This limitation has been

overcomed, thanks to some tricks of different flavours. Several standards had

seen the day light, the most welknown in use to day are LIM-EMS and XMS. Both

EMS and XMS are trademarks.

13.1 EMS (Expanded memory) :

----------------------------

EMS stands for Expanded Memory Specification.

EMS works on all PC's and compatable, with a paging system for the memory

(all the 386 family and some of the 286). Some add-on memory board on XT's or

AT's support this feature, using software activated memory bank switches.

Several drivers for expanded memory exists. As for examples, QEMM386.SYS

(Quaterdeck) and EMM386.SYS (Microsoft) for 386 based machines. These drivers

are to  adhere  to  the  LIM-EMS  (Lotus-Intel-Microsoft).  The  most

versions are 3.2 and 4.0. Several extension were made within the 4.0,

nevertheless the FBB software accomodates both.

The way it works is quite straightforward. The expanded memory is cut in 16K

pages. A window of 4 pages (64K) is set in the standard addressing area of

the PC and it allows the access to 4 pages of the extended memory. This

window is usualy in the segment D000 or E000 outside of the 640 K.

The size of the expanded memory depends on the total memory installed in the

PC. The parameters given to the driver will allow the user to set the amount

of the available memory which will be used as expanded (paged) memory.

A command sent from the application software to the driver will specify the

pages to be seen through the window and given access. It is then simple to

write and read in the paged memory.

This explanation is simplified, the LIM-EMS standard has the provision for

many other extended features

FBB 5.15                          Feb 06 1993                        Page - 71

13.2 XMS (Extended memory) :

----------------------------

XMS means eXtended Memory Specification.

XMS works on most of the PC's and compatable 286 or 386 with extended memory

above 1Mo.

One of the drivers based on this standard is the HIMEM.SYS from Microsoft

and being part of some software package or operating systems. (I.E. Windows).

Other drivers are available on the market which support the same standard.

This driver allows also the load and the activation of TSR modules in high

memory (HMA).

One trick in the behaviour of the 286 and 386, is such that in real mode,

(similar to 8086 8088) the addressing does not cover 1Mo only, it covers

actualy 1Mo plus 64K. In real mode the addressed segments are of 65536, each

segment can be defined on 16 bytes boundaries.

Example : Segment 1000 Offset 4000 (1000:4000) is actualy the 14000 address.

Should we take the last segment FFFF (FFFF:0000), the address is FFFF0, and

this corresponds to the MegaByte minus 16 bytes (each segment can be defined

at each 16 bytes boundaries). In theory this last segment would normaly be

limited to 16 bytes. In fact we go past these 16 bytes and thus gain access

to the first 64Ko of the extended memory. This so called HMA (High Memory

Area) is of 65520 bytes, from FFFF:0010 to FFFF:FFFF. Resident program can be

loaded in this area, and executed.

Beyond the HMA zone, the only way to access the memory is by the virtual

mode (extended addressing). This addressing is transparent for the driver

HIMEM.SYS and allows depending on the type of processor in use (286 or 386)

to access a large amount of memory.

This memory zone is called EMB (Extended Memory Blocks). It will be used to

store and retrieve data. Access is given through HIMEM.SYS which works by

copy, the processor being set in the driver in virtual mode (extended

addressing). The copy process can be lengthy for large amount tranfers. The

driver is stopped by high level interrupts such as the system clock and the

prioritized I/O routines.

FBB 5.15                          Feb 06 1993                        Page - 72

13.3 FBB and the additional memory :

------------------------------------

The FBB knows about the usage of both memory types (EMS or XMS).

Messages lists are stored in this memory. Other data will be stored there

latter-on. The increase in throughput for search is well noticed, response

time for L>, L<, LS ... commands is ten to twenty times better, about 1

second for a 386.

XMS release shoud be greater than 2.0.

The type of handling is at first depending on the processor, XMS on standard

286, EMS or XMS on 386. The EMS can require an add-on memory board, the

system is then rendered very efficient.

Some of the 286 machines incorporate the hardware chips to carry on with the

bank switching on the mother board. Others support this feature at the

expense of heavy! software spoiling sometimes more than 64K of memory.

The EMS on 386 machines calls only for a small driver like EMM386, QEMM386

(few Kbytes).

13.4 Choice of the add-on memory :

----------------------------------

The choice of the type of the add-on memory is conditioned by the harware of

the machine to be upgraded.

To summarize :

XT : nothing but mandatory add-on card.

AT 286 : always XMS sometimes EMS.

AT 386 : all modes possible, EMS is the more efficient.

FBB 5.15                          Feb 06 1993                        Page - 73

14 ASYNC PORT DRIVERS AND TELEPHONE MODEM.

------------------------------------------

14.1 ESS.COM (BIN).

-------------------

ESS is an async port driver optimized for FBB software. It can drive either

TNC2/PK232/clones nor KAM.

This small device driver is loaded as resident at the beginning of the

machine initialization (TSR).

One ESS handles one serial line and makes use of one IRQ.

The COM number, the address of the serial interface and the IRQ number are

given as paremeters appended to the command line. The address should be given

as a FOUR digit hex number.

Example : Install ESS for COM1 at hex 03F8 and using IRQ 4 :

ESS 1 03F8 4

If only the COM number is specified, the address and the IRQ will default to

the standard DOS value. Thus ESS 1 would have had the same effect than the

example.

Any address or IRQ number can be specified. Carefully check that the address

and the IRQ number match the board you intend to use, special attention

should be given to the IRQ number which should not already be used by another

peripheral device in the machine. On a PC the IRQ's can not usually be

shared.

Should there be more than one port to handle, several ESS can be cascaded.

The IRQ should be different for each ESS, except for the case in which the

interrupt sharing is supported by the serial card used.

Example :

ESS 1 03F8 4

ESS 2 02F8 3

This sequence will load ESS's for COM1 and COM2.

ESS accommodates also the muxing arrangement of serial ports.

FBB 5.15                          Feb 06 1993                        Page - 74

14.2 FBBIOS.COM (BIN).

----------------------

FBBIOS is a serial line driver dedicated to the usage of a PSTN modem.

Binary as well as ASCII file tranfer are supported with the XMODEM protocole.

FBBIOS handles one serial line, one IRQ is required. The COM number, the

address of the serial interface and the IRQ number are given as parameters

appended to the command line, the address should be specified in FOUR hex

digits.

Example : Install FBBIOS for COM1 at hex 03F8 and using IRQ 4 :

FBBIOS 1 03F8 4

If only the COM number is specified, the address and the IRQ will default to

the standard DOS value. Thus FBBIOS 1 would have had the same effect than the

example.

Any address or IRQ number can be specified. Carefully check that the address

and the IRQ number match the board you intend to use, special attention

should be given to the IRQ number which should not already be used by another

peripheral device.

On a PC the IRQ's can not usually be shared.

If the modem in use is at the HAYES standard, FBBIOS can modify the line

speed according to the one of the incoming call connection. In this case the

parameter 'A' should be appended to the command line to activate this

feature. This is not true with some buffered modems using a junction with a

settled speed.

Example : FBBIOS 1 03F8 4 A

Upon receipt of the string CONNECT 1200 or CONNECT 2400, the driver will

adjust its speed to the incoming call.

From version 1.30, FBBIOS.COM can run up to 56 KBauds. Be sure that your

computer is fast enought !

FBB 5.15                          Feb 06 1993                        Page - 75

14.3 TELEPHONE MODEM :

----------------------

The FBB software allows the connection of a telephone modem on a serial

port.

The modem must be declared in PORT.SYS. It is necessary to use the

FBBIOS.COM program. This driver performs BBS to Modem interfacing.

It is therefore necessary to load FBBIOS prior to BBS start up. A line

FBBIOS will be added ahead of APPEL.BAT file. See below an example of the

APPEL.BAT file.

Of course, the FBBIOS used will have to match the serial port on which the

modem is to be connected (address and IRQ).

FBBIOS will modify the RS232 speed according to the CONNECT info received

from the modem :

CONNECT 1200 set the driver to 1200 Baud.

CONNECT 2400 set the driver to 2400 Baud.

The driver is reset to the default speed rate (specified in PORT.SYS) at

disconnect.

At connect, the text ; "xxxx BBS. Phone Access" is sent by default. This can

be modified by creating a file MODEM.ENT in the SYSTEM directory.

You can specify in the PORT.SYS file that you accept connections, even if

the user is not registered. In this case, the connection will be "read-only",

the user has only a read only-access, he cannot download files or write

messages, except to the sysop.

When a non-registred user is connected, he will be signified that he only

has a "read only access", and will be prompted to go on. If he answers "Y",

he will go to the BBS menu, else he will be asked again for a callsign.

Type of modem.

--------------

Various experiments have been carried out with several different kinds of

V22 (external, or internal cards) modems. If possible, modems at 1200 or

better 2400 bps should be used; the modem speed is to be declared in

PORT.SYS. Some modems can work either 1200 or 2400 bps, with an internal

speed conversion between the modem and the computer. For example, the modem

can transmit and receive on the telephone line at 2400 bps and talk to the PC

at 1200 bps.

If the modem uses the HAYES protocole, the INITTNCx.SYS file must hold the

correct HAYES init lines.

Protocol:

---------

The configuration used is 1 start bit, 8 data bits, 0 parity, 1 stop bit,

that is to say 10 bits words during the transmission.

The Minitel standard is not supported (7 bits, even parity).

The software being parametered by default in 8 bits, it was not possible to

modify this protocol, as the binary file transfers are carried out on 8 bits.

FBB 5.15                          Feb 06 1993                        Page - 76

The transfer of binary files is made with the Xmodem protocol. The frame

size should be specified at 128 bytes.

NB.: FBBIOS handles XON/XOFF protocol during the transmissions.

Wiring for external modems.

---------------------------

The modem must be able to tell the BBS that a call has been detected with

the remote modem carrier detection. This is the reason why all the RS232

signals are used by the BBS. The RS232 cable has to be wired up pin to pin

with no inversion: pins 2, 3, 4, 5, 6, 7, 8, 20, and 22 of the DB25

connector.

Usage.

------

BBS users are not authorized to connect the BBS through a modem as per the

default settings.

The SysOp must declare the modem users. To do it, perform a user edition (EU

call) and put M (for Modem) then W pass_word. A password made of the three

last letters of the callsign can be attributed for the first connect. Then

the user will be able to change it through the NP command. The authorized

callsigns can be listed by the DM command.

When a modem call occurs, once the modulations have been detected, the modem

advises the BBS that a connection is effective on the telephone line. Then

the BBS asks for the caller's call sign and password.

The user is granted three tries before being disconnected. If everything is

all right, the user can access the same commands like connected on a radio

channel, he will have to disconnect using the B command.

The log file is maintained for the connections through the modem channel.

Keep in mind to add a line in the LOGFBB.CNF file for the modem calls to be

processed in the stats analysis by LOGSTAT.

Example of APPEL.BAT with FBBIOS for a modem connected to COM3/IRQ4 :

echo off

break off

timer/s

echo Appel du serveur ^C pour interrompre

sleep 3

fbbios 3 03E8 4

serv %1

if errorlevel 2 goto direct

if errorlevel 1 goto suite

goto fin

:suite

etc........

Example of PORT.SYS file with 3 serial port used:

COM1 for a TNC, COM2 for a TNC, COM3 for the modem.

FBBIOS will have been previously configured for COM3 (3E8, IRQ 4).

# Programmation and channel setup file

#

# number of Coms : COM1, COM2, Etc...

# number of TNCs : number of frequencies used.

FBB 5.15                          Feb 06 1993                        Page - 77

#

#Ports TNCs

3      3

#

# Com : COM number(1,2 .. 8)

# Interface: 0 : Mode Direct

# 1 : Mode Combios

# 2 : Mode BPQ (BPQ in AA4RE mode)

# 3 : Mode Modem

# Address : in hexadecimal. Ignored if Combios/BPQ/Modem.

# Speed : Baud rate of port. Ignored if BPQ.

#

# There must be as many lines as declared ports

#

#Com Interface Address (Hex) Speed

1    0         3F8           9600

2    0         2F8           9600

3    3         3E8           1200

#

# TNC : TNC number, in increasing order.

# Nbvoies : Number of channels dedicated to one TNC, depends on

# the firmware used, the DED or the TF4 allowing a max of 4

# channels management, 18 with the TF18.

# Com : Number of the Com to which the TNC is connected.

# Canal : Number of channel if a MUX is used, or 1.

# Paclen : Default TNC paclen value.

# Maxframe: Default TNC maxframe value.

# M/P-Fwd : start of forward: minute of the hour, and forward

# period for corresponding frequency.

# Mode : Access - One lettre out of:

# B : Mode BBS.

# G : Mode invite.

# U : Mode Normal

# Emulation - One lettre out of:

# D : Mode DED.

# P : PK232.

# Options - Optionally, one or more letters out of:

# M : Modem access.

# Y : Yapp authorised on this frequency.

# W : Gateway authorised to this frequency.

# Frequence: Text giving the TNC frequency (9 characters max).

#

# There must be as many lines as TNC's declared.

#

#TNC NbCh Com MultCh Pacl MxBloc Fwd M/P-Fwd Mode Freq

1    5    1   1      230  2      1   10/60   UDW  144.675

2    4    2   1      250  3      2   12/30   UDYW 430.675

3    1    3   1      128  2      0   00/60   UM   MODEM

Example of INITTNCx.SYS file for HAYES modem :

#

ATZ

ATS0=1

Example of  FORWARD.SYS  file  to  forward  via  modem.  A  protoco

correction (like MNP) must be used to avoid any modifications of texts. The

HAYES command ATDT diales the line, and when the connection is established,

the callsign (preceded with a "." to avoid echoing) is sent twice and then

the password. The callsign is sent twice in case of a problem the first time.

^M is the <return> variable.

A LA1B

FBB 5.15                          Feb 06 1993                        Page - 78

#

P C                                <-- Modem port.

#

C C F6FBB ATDT19~~~12345678^M      <-- Connects F6FBB using HAYES.

V .F6FBB^M~~~.F6FBB^M~~~PASSWORD^M <--sends callsigns and password.

#

B LA1B

F LA6CU

#

------

In the PORT.SYS file, the number of forward channels for the TNC (modem)

must be 1.

#TNC NbCh Com MultCh Pacl MxBloc Fwd M/P-Fwd Mode Freq

1    5    1   1      230  2      1   10/60   UDW  144.675

2    4    2   1      250  3      2   12/30   UDYW 430.675

3    1    3   1      128  2      1   00/60   UM   MODEM

FBB 5.15                          Feb 06 1993                        Page - 79

15 MISCELLANEOUS TOOLS :

------------------------

A set of tools is available to the SysOp. These tools are for manual or

automatic maintenance. In most of cases, these tools should be handled with

caution, the process can be dangerous for the files.

Avoid using those tools while the server is in operation (the F9 command, as

for an  example),  because  the  modification  of  some  files  duri

operation can yield unpredictable corruptions.

FBB 5.15                          Feb 06 1993                        Page - 80

15.1 INSTAFBB.COM (BIN).

------------------------

This program is a fullscreen configuration of INIT.SRV. Most of commands are

documented. You must only follow the instructions.

If the INIT.SRV file does not exist, you will be prompted to create a new

one. Be sure that you are in the FBB directory when running INSTAFBB.COM.

If the INIT.SRV is not of the right version, you will also be prompted, but

be carefull, as the results could be hasardous.

The comments of the original file are not changed.

FBB 5.15                          Feb 06 1993                        Page - 81

15.2 EPURMESS.COM (BIN).

------------------------

EPURMESS is a DOS tool which allows for modification of messages: status,

removal, or archiving. This tool is called every night by the BBS in order to

maintain the message list as a function of the time.

The EPURMESS configuration is made by the text file EPURMESS.INI. The

parameters which are contained are given as a suggestion, and can be edited

later if needed, to suit the targeted archiving and the mass of messages and

bulletins received.

EPURMESS searchs the current directory for its EPURMESS.INI initialization

file. In that file, it will find out all the parameters for processing, as

well as the file name for the report EPURMESS.RES (this file name can be

changed in the EPURMESS.INI file).

At the beginning of the process, EPURMESS first copies DIRMES.SYS into

DIRMES.OLD to keep an archive trace of the previous file, then creates

DIRMES.NEW, in which the processing will take place. At the end of the

process,  DIRMES.NEW  will  be  copied  into  DIRMES.SYS.  A  repor

processing will be written in the EPURMESS.RES file.

If the last update of the DIRMES.SYS file has been carried out more than 24

hours ago, EPURMESS will not make its process, in order to avoid any loss of

data due to a possible clock error in the system. It will report the error in

the EPURMESS.RES file. In such a case, after the reason of the error has been

detected, it will be necessary to suppress the EPURMESS.RES report file, in

order to re-enable the processing.

The ARCHIVE line has two digits, the first one applies to private messages

and the second to bulletins. The '1' digit validate the archiving while the

'0' will suppress the messages once and for all.

It is possible to specify origins, destinations or routing for which the

obsolete time (X status) are different from the default values. These

particular cases are specified at the end of the file with the form of

supplementary lines. Each line specifies a particular case.

The line should begin with the character which specifies the field to be

tested, then the content of the field and then the number of days before the

status X is given.

Ex :

> ALL 1

The messages for ALL will be valide only one day.

Example of EPURMESS.INI file:

# message file directory

\FBB\MAIL\

#

# message archive directory

\FBB\OLDMAIL\

#

# binary messages directory

\FBB\BINMAIL\

#

# Fichier DIRMES.SYS

\FBB\SYSTEM\DIRMES.SYS

#

# previous file backup

FBB 5.15                          Feb 06 1993                        Page - 82

\FBB\SYSTEM\DIRMES.OLD

#

# new file backup

\FBB\SYSTEM\DIRMES.NEW

#

# report file

\FBB\EPURMESS.RES

#

# Private and Bulletins : 0=destruction 1=archive

1 0

#

# PRIVATE MESSAGES EPURATION PARAMETRES

# PN -> PX (days after creation)

30

#

# PY -> PX (days after change into Y)

5

#

# PF -> PK (days after change into F)

1

#

# PX -> PK (days after change into X)

2

#

# PK -> PA (days after change into K)

2

#

# BULLETINS EPURATION PARAMETRES

# BN -> BX (days after creation)

7

#

# B$ -> BX (days after creation)

4

#

# BY -> BX (days after creation)

4

#

# BX -> BK (days after change into X)

4

#

# BF -> BX (days after creation)

4

#

# BK -> BA (days after change into K)

1

#

--------

#

# Special cases

#

> ALL 1

@ FRA 10

#

FBB 5.15                          Feb 06 1993                        Page - 83

15.3 EPURWP.COM (BIN).

----------------------

EPURWP is a DOS tool which allows updates and modifications of the white

pages database.

An optional parameter can follow and specify the number of days for a data

to be valid. If the temporary part of the record has not changed during this

number of days, then it will be considered as stable and transfered to the

primary (used) part.

FBB 5.15                          Feb 06 1993                        Page - 84

15.4 LOGSTAT.EXE (BIN).

-----------------------

Statistical analysis software for the FBBLOG file written by FC1MVP.

The configuration file for this software is in the text file LOGFBB.CNF,

which should be located in the same directory.

LOGFBB.CNF file layout :

------------------------

This file is made of several lines indicating:

- BBS Callsign

- the name of the SysOp port, a comment.

- The abbreviated name of the 1st port, a comment.

- The abbreviated name of the 2nd port, a comment.

- and so on until the last port (8 ports maximum).

- Three stars (***) at the beginning of the line means the end of the ports.

- The BIOS screen access option.

The BBS callsign must have 6 characters maximum. It will be truncated should

it be longer.

The abbreviated port name may have up to 10 characters, it is recommended to

limit at 4 (ex VHF1 or HF2).

A comma separates the abbreviated name from the comment which follows it,

without any space. The comment may have 10 characters too (frequency).

At the end of the comment, it is possible to add a comment which is internal

to the CNF file, and which will not be used elsewhere, using the ":" as a

separation.

If the BIOS line has a NON at the beginning, the screen access will be

direct. The word OUI means that the screen access is made via the BIOS. The

direct access improves the processing speed by 50 percent but is careless

with DesqView.

Example of LOGFBB.CNF file:

---------------------------

F6FBB : BBS name.

SYSOP,at keyboard : port name @ , comment.

VHF, 144.675 : Port A , comment.

VHF, 145.275 : Port B , comment.

UHF, 430.675 : Port C , comment.

HF, 21.107 : Port D , comment.

SHF, 1299,675 : Port E , comment.

MODEM, Telefone : Port F , comment.

*** End of ports definition.

NON : Screen access through BIOS (OUI) or

Direct (NON).

Software limitations :

---------------------

The software is able to handle a log file within the following limits:

- Maximum number of days : 35, or 5 weeks if you chain 5 log files.

- Maximum number of channels: 50

FBB 5.15                          Feb 06 1993                        Page - 85

- Maximum number of ports : 8+1, corresponding to ports A to H handled by

the BBS and the SysOp keyboard "port".

- Maximum nr of callsigns : 300 (This is not the number of the call signs

known by the BBS, but the number of calls which were used the BBS during the

log period.

In case an overflow occurs, the mention AUTRES for the callsign and 99 for

the day will be displayed in the corresponding areas.

Regarding the forward efficiency, only the stations which have actually

forwarded will be given a value, the others will be mentioned "nc".

Usage :

-------

On the same diskette (or the same disk) there should be at least the program

LOGSTAT.EXE, the config file LOGFBB.CNF and enough disk free space for the

result files to be created. The log file may stay on another diskette.

For the startup you should type in LOGSTAT FBBLOG.nn (nn is the number of

the week) or A:FBBLOG.nn (if the file is located on A:)

The results are recorded into the files LOG.@, LOG.A ... LOG.T being the

total of the logs.

Archiving :

-----------

A suggestion: you should better archive your FBBLOG.nn file rather than the

LOG.x files. You should ZIP them (or ARC or LZH) with the name LOG990nn.ZIP

(.ARC or .LZH) with 990 standing for 1990 (000 will be for 2000...) and nn is

the number of the week.

Log structure :

---------------

The number of resulting files created is equal to the number of ports of the

BBS plus 2. The 2 supplementary files correspond to the console port and the

sum of the various ports as being the total result.

The first log, named LOG.@ corresponds to the SysOp log (console), the last

one, named LOG.T is the summation of the ports. The files LOG.A to LOG.H are

the logs of the various ports.

Obviously, if a port has not been used, the corresponding file will not be

created.

For each LOG.x file, you find in the following order:

- Diary of the connections per day (y-axix) and the time (x-axix) with the

daily total at the end of the line and the hourly total at the bottom of each

column.

- The total connection time: it is the sum of the connection times of each

user. For the LOG.T file, the total connection time per channel will also be

given.

- Mean time per connection : this is the total connection time divided by

the number of connections.

- Mean time per user : total connection time divided by the number of users.

- Number of killed messages : self explanatory.

FBB 5.15                          Feb 06 1993                        Page - 86

- Number of read messages : idem.

- Number of users.

- A matrix of forwarded messages per day (y-axix) and per hour (x-axix) is

made like before. This matrix will not appear if there has not been at least

one forward on this port.

- Number of forwarded messages: number of messages sent to another BBS or

PMS.

- Number of messages received in forward: number of messages transmitted by

other BBS in forward.

- Number of messages received in back-forward: number of messages received

from another BBS when forwarding towards it.

- Number of messages refused as already received.

- An array giving the number and the size of the messages received from or

sent to another BBS and the efficiency in Bits/s (Max 1200 !).

The BBS's are sorted by alphanumeric order, and the efficiency indication

appears for callsigns having conducted only forwarding operations. This

matrix will not appear unless there has been at least one BBS forwarded.

- A matrix of the main functions utilized, with on the y-axix the callsigns

sorted by alphabetical order, and on the x-axis the total time of connections

in minutes, the number of connections, and the various functions used (see

the comment following the array).

In the LOG.T file, the X.F column (forced disconnection entered by the

SysOp) is replaced by the number of killing commands. At the end of each

column is the compound sum.

Log of a specific station.

--------------------------

- The LOGCALL program yields generation of a log for a specific station

The principle of operation is the same as for LOGSTAT, except that a

supplementary calling parameter is required to define the station.

LOGCALL FBBLOG.33 FE1ZZZ

The generated file will be of the same format as the FBBLOG, but it will

enclose only the lines specific to the callsign. The name of the file will

match the format FE1ZZZ.LOG, if the requested callsign is FE1ZZZ.

While executing LOGSTAT FE1ZZZ.LOG, you will have the statistics related to

that station (in the LOG.A, LOG.B,...).

If you specify the SSID, you will create a log corresponding only to the

callsign and the specified SSID. Ex:

LOGCALL FBBLOG.33 FE1ZZZ-2

Editing the file FE1ZZZ.LOG will allow you to display all the commands sent

by that station.

FBB 5.15                          Feb 06 1993                        Page - 87

15.5 SATUPDAT.EXE (BIN).

--------------------------

SATUPDAT.EXE will read messages from AMSAT copied via packet radio. This is

the english version of AJOURSAT.EXE. This software was developped by F6BVP.

It extracts keplerian elements both in NASA and AMSAT formats and updates

the results into the satellites data base of the F6FBB's BBS (SATEL.DAT)

Loading SATUPDAT without arguments will print the following message on your

screen (between lines -----):

------------------------------------------------------------------

Automatic update of satellites orbital parameters for F6FBB's bbs Version

1.77e - June 1991 - Bernard Pidoux, F6BVP

Usage: SATUPDAT [file name<.txt>] </option> </option>

Reading option:

/a AMSAT format (default)

/n NASA format

Update option:

/f merging new and old data (default)

/u update only satellites present in the

data base

/s keep only satellites present in the input file

<<767 satellites maximum>>

------------------------------------------------------------------

This version 1.77e verifies checksum both for AMSAT files and NASA files.

For automatic update of keplerian elements into F6FBB's bbs just create a

virtual BBS into BBS.SYS file. Its name could be AMSAT for example.

Then declare the following forward in the file FORWARD.SYS :

A AMSAT

*

G AMSAT

P @

C C:\FBB\SYSTEM\SAT\AMSAT.TXT

*

------

CAUTION: The name of the disk unit and the path name should be the exact one

of your configuration.

In order to run automatically SATUPDAT when your computer boots, you must

modify the batch file like the following example:

:direct

if not exist c:\fbb\system\sat\amsat.txt goto appel

cd \fbb\system\sat

satupdat amsat /n > satupdat.res

satupdat amsat >> satupdat.res

del amsat.txt

cd \fbb

:appel

echo Running the BBS ^C for stopping

sleep 3

serv -t

FBB 5.15                          Feb 06 1993                        Page - 88

if errorlevel 2 goto direct

if errorlevel 1 goto suite

:suite

(Once again be carefull with the disk name and directories !)

SATUPDAT OPTIONS

================

The default extention for input file is .TXT, however you can specify

another extention.

Both arguments after the filename in the command line are optional: the

first one indicates the reading format, either the AMSAT format or the NASA

"2-line" format. The first one is selected by default.

The second option is for the choice of update mode. By default the program

will merge the old list with the new one (same if you select option /f). Old

data of satellites which are not present in the input file are kept as is and

the other one are updated.

Option /u can be selected if you dont want to add new satellites to the one

already present. Only keplerian elements from known satellites are updated.

This prevents the list to grow with incoming satellites that you are not

eventually interested in.

Option /s, on the contrary, gives you the possibility to keep only in the

data base the data from the satellites whose name is in the input file.

This can help you to suppress old satellites from the data base.

In all cases the program takes care of the permanent data of satellites such

as the frequency (for doppler) and the step for computing the tracking (you

must provide both manually only the first time).

If the keplerian elements of the input file are older than the one already

present in the BBS data base, the old one are kept.

The list is sorted by name in order to help the finding of the satellite

name when someone is connected to the BBS.

If elements for one satellite are duplicated in the input file, the program

will look again for the newest data.

Of course, if CRC computed by the program is not the same as the one in the

input file, the program informs you that the CRC is BAD and it does not take

the keplerian elements.

In the batch file example given above you have certainly noticed that I call

twice the program SATUPDAT with two different format arguments. The first

time I ask the program to read the NASA format and the second time to read

the default format AMSAT. The reason is that the probability for an error to

occure in both formats for the same satellite is very small. Thus if an error

occurs in one format the program will complete the update when he will found

unaffected keplerian elements in the other format. Thanks to the recently

adopted checksum by AMSAT!

FBB 5.15                          Feb 06 1993                        Page - 89

15.6 SLEEP.COM (BIN).

---------------------

Utility which spends the time (in seconds specified as argument) while doing

nothing. It can be interrupted by a Ctrl-C, and allows interruption of a

Batch file running.

The syntax to call it is of the type :

SLEEP nn

nn is the number of seconds to wait.

FBB 5.15                          Feb 06 1993                        Page - 90

15.7 TLABEL.COM (BIN).

----------------------

Utility to update the file YAPPLDL.SYS which contains the various labels

(informations) of the files received while the YAPP protocol was used. TLABEL

renames YAPPLBL.SYS into YAPPLBL.OLD and creates a new YAPPLBL.SYS containing

only the labels of the existing YAPP files. This program was written by

FC1EBN.

The execution of this utility should be done only in the directory SYSTEM.

There is no argument for this command.

FBB 5.15                          Feb 06 1993                        Page - 91

15.8 MAKEPG.COM (BIN).

----------------------

Utility used to generate .EXE (or .COM) program file out of the binary file

(xxx.BIN) using English or French language.

The syntax is of the type:

MAKEPG program FR

To generate a program in French

MAKEPG program GB

To generate a program in English.

MAKEPG can transform the following programs :

SERV.BIN     -> SERV.EXE

MAINTINF.BIN -> MAINTINF.COM

EPURMESS.BIN -> EPURMESS.COM

EPURWP.BIN   -> EPURWP.COM

FBB 5.15                          Feb 06 1993                        Page - 92

15.9 SETUSER.COM (BIN).

-----------------------

Utility giving the property of a file to a user for further use by FBBDOS.

The syntax is of the type:

SETUSER callsign filename.

The command "SETUSER F6FBB TOTO" will allow F6FBB, and him only, to modify

or delete the file TOTO.

FBB 5.15                          Feb 06 1993                        Page - 93

15.10 CLEANUP.COM (BIN).

------------------------

Utility killing all messages files unused and not defined in DIRMES.SYS.

This program should be run in the SYSTEM directory.

The syntax is of the type :

CLEANUP mail_directory

Be carefull when using this utility. If another directory than MAIL or

BINMAIL is specified, it should delete all the files of the directory.

FBB 5.15                          Feb 06 1993                        Page - 94

15.11 MAINTINF.COM (BIN).

-------------------------

Utility maintaining the INF.SYS file. The INF.SYS file holds the users'

database. In case of problem in INF.SYS file, goto the SYSTEM directory and

run MAINTINF followed by a number of monthes. The records of  users not

connected within this number of monthes will be deleted and these users will

be asked as for a new connection. The number 0 will not delete any record and

only check the INF.SYS file.

FBB 5.15                          Feb 06 1993                        Page - 95

15.12 MAINTREJ.COM (BIN).

-------------------------

Utility maintaining the REJET.SYS file. The REJET.SYS file describes the

Reject, Hold and Local hold informations.

The program reformats the lines and verify the number of parameters.

FBB 5.15                          Feb 06 1993                        Page - 96

15.13 CUT.COM (BIN).

--------------------

Utility allowing to peak a part of a file and copy or append it to another

file. The format is :

CUT From-file To-file length offset [/A]

To-file will have added up to length bytes of From-file, starting at offset.

If the switch /A is specified, then datas will be appended to To-file,

otherwise To-file will be replaced.

This little utility is usefull to take parts or file, or to do copy/paste

with either binary nor ascii files.

Together with the specification of the offset in the XGET command, in case

of crash, first note the amount of datas already received (ie : 50300), save

the part of the file already received, and then ask "XGET filename 50000".

The transfert will start at offset 50000 and then you will be able to receive

the end of the file. The paste the two parts, either with COPY command or

with the CUT command. It is more secure to cut the first received part before

as the last bytes should be garbaged.

FBB 5.15                          Feb 06 1993                        Page - 97

16 WHITE PAGES DATABASE AND SERVER

----------------------------------

16.1 DESCRIPTION.

-----------------

The White Pages implementation in FBB software has been based upon the W0RLI

model (many thanks to Hank for his work). I've tried to maintain a high

degree of comptibility whilst making further development to my own criteria.

I shall try to explain how FBB White Pages works.

I have probably mis-understood some features of W0RLI's specifications but I

hope that this will not greatly  affect the compatibility.

First of all, why do we need White Pages?

White pages has some interesting features. Not least :

- A dynamic database containing users Name, zip code, HomeBBS and QTH (as

well as other fields).

- Automatic addressing/routing of mail to the HomeBBS of the destination

callsign.

- A White Pages server for remote interrogation of the database.

The database information is updated, firstly from the information given by

users when they exercise the N, NH, NQ and NZ features at their home (or

another WP equipped) BBS; and secondly, from information contained within the

messages headers as they traverse the Network.

The database is dynamic, it is changing constantly, and it updates itself in

real time. Either as soon as a line of a message header is received when in

ASCII forwarding mode, or when a complete message is decoded in compressed

forwarding mode; or else when a user disconnects from the BBS (this is to

prevent multiple updates being generated during a session).

So, the database can hold many callsigns. In fact it maintains a list of all

the callsigns seen from all individuals sending messages as well has all of

the BBS's seen in the forwarding paths. More than 10,000 valid records is not

impossible today, and this will surely increase as the number of packet radio

users grows with each day. This will allow user to send messages to other

users around the world without necessarily having to be concerned to find

their full Heirarchical Address, the old principle of the user typing:

BBS PROMPT >

SP K6VAZ @ KM6WU.#CENCA.CA.USA.NOAM

should now be replaced by the user entering:

BBS PROMPT >

SP KM6VAZ

The BBS will add the HA and send the response:

BBS PROMPT >

SP K6VAZ

WP ROUTING @KM6WU.#CENCA.CA.USA.NOAM ADDED

TITLE ?

If the routing destination HA is not recorded in the database then the user

will be advised and prompted to enter the address manually.

FBB 5.15                          Feb 06 1993                        Page - 98

Another capability of FBB White Pages is the automatic sending of update

messages to other BBS's. These messages are generated every night during

House-Keeping and are a listing of the additions and modifications made to

the database during thhat day. These messages are sent addressed both to and

from WP.

When passing through or terminating at another White Pages equipped BBS, the

message will automatically update the 'local' WP database at that BBS. This

feature MUST BE USED WITH CARE, as updates can generate a lot of traffic and

the Network must be able to support it.

*** It's not be a good idea to send these update messages on HF ! ***

A built-in White Pages server (WP) will provide information from the

database in response to a remote request. This server is described in

paragraph xx.

All files used by White Pages are in the FBB\SYSTEM\WP subdirectory.

16.2 UPDATE REQUESTS.

---------------------

The  database  receives  information  from  three  sources.  The  s

indicated on each line of the update message as a suffix to the callsign:-

- The /U suffix denotes that the information in this line of the   update is

User-Generated as is therefore assumed to be CORRECT. This information is

collected by the BBS whenever the User responds to the N, NH, NQ or NZ

commands. The date associated with the information is the date when the User

disconnects that session.

- The /G suffix denotes that the information in this line has been gathered

by examining the header of a message to GUESS at which BBS the sender is

registered. The HomeBBS of the User is assumed to be the BBS shown in the

first R: header line. The date associated with this information is the date

shown on this R: header line.

- The /I suffix denotes information about forwarding BBS's taken from the R:

header lines. This information can consist of the HA (the Heirarchical

Address), the QTH (within brackets) and the zip code (following the Z:). The

date of this information is again taken from the R: header line of the BBS in

question.

When the BBS is idle the Database Manager is called and the update

information detailed above is processed.

16.3 DATABASE DESCRIPTION.

--------------------------

The database  is  composed  of  individual  records.  Each  record

following components :

- Callsign and Name.

- Active information.

- Temporary information.

The active and temporary information components are identical and each

includes the following fields:

FBB 5.15                          Feb 06 1993                        Page - 99

- Date of the information

- Hierarchical Address   (one word)

- Zip code          (one word)

- Qth               (one or more words)

Only the Active information is used for addressing/routing and database

requests.

16.4 DATABASE MANAGER.

----------------------

This process freshens the database, following receipt of the new or changed

information detailed above.

The update subroutine will first look for an entry in the database for the

callsign which matches the received information. If it does not exist then a

completely new record will be created in the database and the information be

used to fill what fields it can, in both the active and the temporary

components. The date will be then changed to the one associated with the

update information.

If the record does already exist, then the unknown fields of both the

temporary and active fields will be filled in, and those fields already known

in the temporary part will be replaced by the new information if the date new

information is younger than that already on file. The date will then be

adjusted such that it is consistent with the updated information.

If the new information is of the /U category, then the current fields will

be replaced by the new information in both the primary and secondary (Active

and Temporary) parts of the record, as this information has been input

directly from the user. If the information was of another category then only

the secondary (Temporary) part of the record will be updated, so the Active

or primary record will remain unchanged at this time.

If a field is changed, a flag giving the update request type is then

validated. If the /U flag is already validated, it will not be replaced. This

flag will be used in case the WP update messages are validated.

16.5 EPURWP AND UPDATE MESSAGES.

--------------------------------

EPURWP is a maintenance program for the White Pages database which should be

run during each House-Keeping cycle.

The program conducts a validity check on each of the entries, and discards

any "unwanted" records (in the case of an invalid callsign for example).

The program also checks the date of the last update of the temporary part of

each record. If this date is older than a pre-defined number of days (given

as a parameter, default 40 days) then the temporary part is considered as

stable, and then the known fields will be transferred to the Primary or

Active part, which is then used to answer all addressing/server requests.

This process ensures that the database is tolerant of users sending messages

from mailboxes other than their normal HomeBBS. Once the Active or primary

part of the record is set, then the temporary (or secondary) part can be

updated/changed many times. Only once this temporary field has remained

unchanged for 40 days, or the user excercises any of the "Nx" options at his

new HomeBBs will the Active or Primary record be changed.

FBB 5.15                          Feb 06 1993                       Page - 100

If the changes to the database are validated, then the record is marked with

an update flag and a line will be appended to the file MESS.WP

Each line of the outgoing WP update messages looks like :

On 930123 FD1CDC/U @ F6FBB.FMLR.FRA.EU zip 31240 Claude Saint Jean

Any unknown fields are replaced by "?" like :

On 930123 FD1CDC/U @ F6FBB.FMLR.FRA.EU zip ? ? Saint Jean

The U character is the update type.

16.6 WP SERVER REQUESTS.

------------------------

FBB software has an internal built-in WP server.

The format of the WP server requests are as shown below :

BBS PROMPT >

SP WP @ F6FBB

Title of message

WP Request (does not matter)

Text of message

F6FBB ?

EA3* ?

^Z (or /EX)

The server will answer to the request with a private message, addressed to

the sender, and routed to the BBS according to the first R: header line of

the incoing request.

The reply message is restricted to a maximum of 100 lines, as the use of

wildcards in the request could generate a unacceptably long replies.

FBB 5.15                          Feb 06 1993                       Page - 101

17 SERVERS

----------

The BBS software offers various servers which are triggered when machinable

messages are received. The number of offered servers is is without limit.

The servers are activated when receiving a message whose destination had

been given at the end of the INIT.SRV file.

Declaration of servers at the end of INIT.SRV :

-----------------------------------------------

#

# Indicateur de voies

11

#

------

#

# Liste des services :

#

# Desti Nom du programme

#

REQDIR REQDIR.COM

REQFIL REQFIL.COM

NEWDOC NEWDOC.EXE

#

# Fin du fichier d'initialisation

#

Servers are exec programs whose behaviour is explained in appendix 9. They

should be found in the directory containing the exec program SERV.EXE and the

configuration file INIT.SRV.

When a private message is received, whose destination is defined in the

INIT.SRV file, the corresponding program to the destination is called with

the name of a temporary file on the command line. This temporary file

contains a message function of the requested server (see description of

servers below).

At the completion of the server program, the incoming mail file is checked

to import an answer, should there be one. The message addressed to the server

and the temporary file are erased after this processing.

Various services can be developped, time available has its limits, but the

imagination has not !

FBB 5.15                          Feb 06 1993                       Page - 102

17.1 REQDIR

-----------

The REQDIR server sends back to origin of the machinable message the list of

files (total or partial) of a directory.

The standard command line of the message is used. The message should be of

the private type and addressed to the BBS that we want to ask :

SP REQDIR @ bbs-destination

The title of the message contains two informations, the mask of the

selection (*.EXE, *.*, etc...) and the @ sign followed the return bbs, that

is to say yours. The return message will be addressed to the sender of the

request via the return BBS. The mask can not be specified. In this case, all

the files will be listed.

Mask @ bbs-return <- mask of the selection

The text of the message is not necessary, its content will be ignored by the

server.

Example :

F6FBB BBS >

SP REQDIR @ F6ABJ.FRA.EU

Title of message :

YAPP\*.ZIP @ F6FBB.FMLR.FRA.EU

Text of message :

/EX

We assume with this example that YAPP is a sub-directory of the USERS

directory (this a suggested configuration).

FBB 5.15                          Feb 06 1993                       Page - 103

17.2 REQFIL.

------------

REQFIL server allows to send back the ascii file requested to the originator

of the message.

The message sent to REQFIL has a well defined syntax. The standard command

line of message is utilized. The message should be of the private type and

addressed to the BBS which we want to ask.

SP REQFIL @ bbs-destination

The title of the message contains two informations, the name of the

requested file and the @ sign followed by the return BBS, that is to say

yours. The return message will be addressed to the originator of the request

to the return bbs.

filename @ bbs-return

The text of the message is not necessary, its content will not be taken into

account by the service.

Example :

F6FBB BBS >

SP REQFIL @ F6ABJ.FRA.EU

Title of message :

DEMOS\ESSAI.TXT @ F6FBB.FMLR.FRA.EU

Text of message :

/EX

This example makes the assumption that DEMO is a sub-directory of the USERS

directory.

The answer of the remote mailbox will be of the form :

SP F6FBB @ F6FBB.FMLR.FRA.EU < F6ABJ

Req File: DEMOS/ESSAI.TXT

Contents of ESSAI.TXT file

...

/EX

FBB 5.15                          Feb 06 1993                       Page - 104

17.3 NEWDOC.

------------

The NEWDOC server allows a simple and fast update of the documentation

database (topic DOCS of the server). This server will create the file and

will give it a label. It will not create a directory if it doesn't exist.

Therefore it is necessary to prepare the directory tree structure before.

A message will give a summary to the originator.

The standard command line for messages is used. The message should be of

type private and addressed to the BBS which will be updated.

SP NEWDOC @ bbs-destination

The message title contains two informations , the name of the file requested

followed by the label to give to the file (see the description of the topics

DOCS).

Filename Label eventually on several words.

The text of the message will be transferred in the documentation file.

Example :

F6FBB BBS >

SP NEWDOC @ F6ABJ.FRA.EU

Title of message :

BBS\DOC_1.DOC This is the label of the documentation.

Text of message :

This is the content of the documentation.

....

End of documentation.

/EX

This example assumes that the BBS is a subdirectory of DOCS, thus one of the

topics of the documentation.

If the NEWDOC.SYS file is specified, the server will be limited to the

callsigns declared in this file, one callsign in each line.

Example of NEWDOC.SYS file :

F6FBB

FD1CDC

FBB 5.15                          Feb 06 1993                       Page - 105

17.4 REQCFG.

------------

REQCFG server allows to send back the current configuration of the software.

The message sent to REQCFG has a well defined syntax. The text of the

message is not used. The message should be of the private type and addressed

to the BBS which we want to ask.

SP REQCFG @ bbs-destination

The title of the message contains the @ sign followed by the return BBS,

that is to say yours. The return message will be addressed to the originator

of the request to the return bbs.

@ bbs-return

The text of the message is not necessary, its content will not be taken into

account by the service.

Example :

F6FBB BBS >

SP REQCFG @ FD1CDC.FMLR.FRA.EU

Title of message :

@ F6FBB.FMLR.FRA.EU

Text of message :

/EX

The answer of the remote mailbox will be of the form :

SP F6FBB @ F6FBB.FMLR.FRA.EU < F6ABJ

ReqCfg V 1.2 (C) F6FBB 1992 - BBS F6FBB

Software F6FBB Version 5.15 compiled on Feb 07 1993

Mem Us:14620  Mem Ok:148288  Bid:30000  Ports:4  Ch:12  FBB Ok  BIN Ok

Available volumes : C: D: E:

EMS V4.0

MSG : 29 pages (464 KB)

BID : 17 pages (272 KB)

HIE : No

FWD :  1 pages ( 16 KB)

REJ :  1 pages ( 16 KB)

OVR : 11 pages (176 KB)

SCR : 45 pages (720 KB)

WPG :  5 pages ( 80 KB)

Total : 109 pages (1744 KB)

Languages

1 : ENGLISH

2 : FRANCAIS

3 : ESPAGNOL

4 : ITALIANO

5 : NEDERLAN

6 : DEUTSCH

7 : CATALA

FBB 5.15                          Feb 06 1993                       Page - 106

8 : NORSK

9 : PORTUGUE

10 : DANSK

11 : SVENSK

12 : HRVATSKI

13 : FRANC8

Servers

REQDIR : Demande de repertoire

REQFIL : Demande de fichier ascii

NEWDOC : Ajout de documentation

REQCFG : Demande de configuration

AUTO7P : Serveur 7plus

7PSERV : Serveur de fichiers 7PLUS

SWFMLR : Distribution Infos Reseau

WP : Serveur W.P.

MULTI : Multi-addresses

Port Interface Emulat. Ch Mode  Frequency

1    DRSI      WA8DED  9  UYWL  NODAL

2    ESS/COM.  KAM     1  GYW   15M/20M

3    FBBIOS    FBBIOS  1  UYM   MODEM

4    ESS/COM.  KAM     1  GYW   HF-PSK

/EX

FBB 5.15                          Feb 06 1993                       Page - 107

18 FILE FORMAT :

----------------

The miscellaneous files used for the configuration and the operation of this

BBS software have a very accurate syntax. The good behaviour of your software

depends upon how these files have been configured.

The first file to be configured is INIT.SRV. This file contains the

informations on the BBS and about your hardware.

The system holds more than 10 text files which can be configured. Do not

forget to save the previous content of the file before modification. This

will ease the restauration which might be often necessary. Never delete a

file, you might loose major data. Never modify a binary file. Some files have

a preset number of lines, and adding or deleting lines might generate an

error message. This is the case of the configuration files and text files.

In most of the configuration files and within the text files, the character

# in the FIRST COLUMN means that the line is a comment, and will not be taken

into account for the number of lines.

Files may stay in different directories, FBB, BIN, SYSTEM, etc... The

default directory is indicated after the file name inside parentheses.

FBB 5.15                          Feb 06 1993                       Page - 108

18.1 PORT.SYS (SYSTEM).

-----------------------

This file is found in the directory called SYSTEM (if you used my default

directory-names). Remember that a line starting with # is a comment-line, and

the BBS ignores these lines. The BBS has built-in drivers for a maximum of 8

serial-ports, named COM1-COM8. It can also use a special port-multiplexer so

that you can have up to 4 TNCs per COM-port. In addition to the built-in

port-drivers, you can use other drivers like COMBIOS, MBBIOS, FBBIOS etc.

These drivers must be loaded BEFORE the BBS-program. You can include the

drivers in APPEL.BAT for convenience. It is possible to use telephone-modem

with the BBS. In that case the number of TNCs include the modem. The FBBIOS-

driver must be loaded.

Standard PORT.SYS with 1 TNC (COM1) with WA8DED-hostmode, no com-driver, 4

channels, 1 channel for outgoing forward, 1 channel reserved for BBSs with

callsign LA1B-1 :

# File for programming of channels and TNCs.

#

# Ports : How many ports (COM1, COM2, Etc...)

# TNCs : How many TNCs and modems in use. With multiplexer

# there can be up to 4 TNCs per port.

#

#Ports TNCs

1 1

#

# Com : COM-number (1,2,...8)

# Interface :

# 1 = ESS COM driver (external)

# 2 = BPQ-node (version 4.05 and up)

# 3 = Telephone-modem with FBBIOS

# 4 = DRSI card with driver

# 5 = TFPCR/TFPCX interface

#

# Adress : Adress of port in hexadecimal. Needed for multiplexor.

#

# Baud : Ports baud rate (up to 56 KBds). Ignored by BPQ.

#

# Use same number of lines as number of ports.

#

#Com Interface Adress (Hex) Baud

1 0 3F8 4800

#

# TNC : Number on TNC in use.

# NbCh : Number of channels I want to use in the TNC.

# Maximum available channels depend on firmware.

# Com : Number of the COM-port. Com1, Com2 etc.

# MultCh : Number of channel if port-multiplexer is used,

# otherwise 1. In DRSI use values from 0 to 7, by KAM

# use 1 for VHF and 2 for HF.

# Paclen : PACLEN on this TNC.

# Maxframe: The maximum number of frames the TNC will send at one

# time.

# NbFwd : Number of channels for OUTGOING forward at same time.

# MxBloc : Size of forward-block in kb.

# M/P-Fwd : Minute of the hour for start of forward, and period

# (how many minutes between each forward-start).

# Mode : One of these:

# B : BBS-mode.

# G : "Guest"-mode.

# U : Normal-mode.

# Type host-mode, one of these:

FBB 5.15                          Feb 06 1993                       Page - 109

# D : WA8DED

# K : KAM hostmode.

# P : PK-232

# Q : BPQ v 4.x

# Addition: One (or more) of these letters can be used too:

# L : Send unproto beacon after each arriving mail.

# M : Telephone-modem.

# Y : Yapp allowed on this QRG.

# W : Gateway allowed TO this QRG.

# R : Read-only mode allowed on this port.

# Freq. : Text to describe this port (max 9 letters, no

# space)

# Same number of lines as number of TNCs.

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    4    1   1      230  4     1     10     30/60   UD   433.650

#

# Special callsigns and modes for some channels.

#

#TNC Nbs Callsign-SSID Mode  (only with WA8DED host mode)

1    1   LA1B-1        B

#

# End of file.

#

In the next examples, all comment-lines are removed, for clarity.

Next an example of a file with 4 ports, 13 channels, no com-driver, with a

port-multiplexer on COM 1. Port 1 has limited access (guest-user), port 2 and

3 allows YAPP-transfer and port 4 uses a PK-232.

#Ports TNCs

1      4

#

#

#Com Interface Adress (Hex) Baud

1    1         3F8          4800

#

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    4    1   1      230  4     0     1      00/60   GD   433.650

2    4    1   2      230  4     1     5      10/30   UDY  144.675

3    4    1   3      230  3     4     10     20/60   UDYW 433.650

4    1    1   4      80   1     1     5      15/30   BP   15/20m

#

Next an example of a file with 4 ports, 13 channels, with ESS/COMBIOS.

#Ports TNCs

4      4

#

#

#Com Interface Adress (Hex) Baud

1    1         0            4800

2    1         0            4800

3    1         0            4800

4    1         0            4800

#

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    4    1   1      230  4     0     1      00/60   GD   433.650

FBB 5.15                          Feb 06 1993                       Page - 110

2    4    2   1      230  4     1     5      10/30   UDY  144.675

3    4    3   1      230  3     4     10     20/60   UDYW 433.650

4    1    4   1      80   1     1     5      15/30   BP   15/20m

#

Next an example of a file with 1 port with BPQ-node and 4 channels.

#Ports TNCs

1      1

#

#

#Com Interface Adress (Hex) Baud

1    2         0            4800

#

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    4    1   1      230  4     1     1      00/60   UQY  433.650

#

Next an example of a file with 1 DRSI-card and 8 channels, max 1 channel on

HF.

#Ports TNCs

1      2

#

#

#Com Interface Adress (Hex) Baud

7    4         0            4800

#

#

#TNC NbCh Com MultCh Pacl Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1    8    7   0      230  4     1     10     00/60   UDY  433.650

2    1    7   1      80   2     1     10     30/60   GDW  HF

#

FBB 5.15                          Feb 06 1993                       Page - 111

18.2 CRON.SYS (SYSTEM).

-----------------------

This file is in the SYSTEM directory. The BBS looks in this file once every

hour. If the hour and date is right, the BBS will execute the commands in

this file.

DOS-programs are run with X or D command, like X DOTHIS.EXE or D DOTHAT.EXE.

When running BAT files, a COMMAND.COM is automaticly first sent. That needs

some more memory.

With D the screen is saved, and reappears after the program has terminated.

With X the program will not write to the scren.

#

# File CRON.SYS. Here DOS-commands can be executed to fixed

# hours, also Gateway, Yapp, Bip and Talk can be turned on or

# off.

# Format: TALK YES (NO), BIP YES (NO), GATE 1 YES (NO), YAPP 2

# YES (NO).

# DOS-program is run like in FORWARD.SYS with D or X command.

#

# As in FORWARD.SYS, you can use H for hour and D for day.

# 0=sunday, 1=monday,.....6=saturday.

# Also IF-ENDIF and IF-ELSE-ENDIF is allowed

#

# Possible formats for time:

# IF H 0-9 All hours between 00 and 09.

# IF H 0,9 Only at 00 and 09.

# IF H 0,9-11 At 00, and all other hours between 09 and 11.

# IF D 0-2 Sunday, monday tuesday.

#

TALK NO

BIP NO

GATE 1 YES

GATE 2 YES

YAPP 1 YES

YAPP 2 YES

IF D 1-5

IF H 15-23

GATE 3 YES

YAPP 3 YES

ELSE

GATE 3 NO

YAPP 3 NO

ENDIF

ELSE

IF H 8-23

GATE 3 YES

YAPP 3 YES

ELSE

GATE 3 NO

YAPP 3 NO

ENDIF

ENDIF

X DOTHIS ARG1 ARG2 ARG3

D DOTHAT ARG1 ARG2

FBB 5.15                          Feb 06 1993                       Page - 112

18.3 REJET.SYS (SYSTEM).

------------------------

This file is in SYSTEM-directory. With this file it is possible to reject or

hold certain types or sizes of messages.

The first letter of each valid line specifies the action :

R = Reject : The message will not be received.

H = Hold   : The message will be received but held untill the sysop reviews.

L = Local Hold : Only messages created on this BBS will be held.

# File for rejecting messages. They are rejected with N-BID:

#

# Type, from, @BBS, to, BID, maximum size:

#

# * and ? can be used as wildcards (as in MS-DOS)

#

R B TOTO  ALL    TATA  * 0

R B *     *      VENTE * 0

R B *     VENTE  *     * 0

H * P1RAT *      *     * 0

L B *     *      *     * 0

FBB 5.15                          Feb 06 1993                       Page - 113

18.4 PASSWD.SYS (SYSTEM).

-------------------------

This file is in SYSTEM directory.

The first line in this file is the generic password. That is, this password

will be used for all sysops that do NOT have a special password of his own.

Be adviced to change this to your own, secret password !

The next line is a special password for one special sysop. In this case

LA6CU. The two first numbers correspond to the same "security-numbers" in

INIT.SRV (check this). So in this case of LA6CU, I will have the security-

number 69 before a successful SYS-command, and 1023 after. In the next line

the numbers for LA6IM is 123 and 1023.

jkshdfkjasdfuweiuryiusdfashdfabsmfzmxncvbiouweqyriuyweruyasjhf

LA6CU 69 1023 jkasdhfiuweyiruyasidkasfuiweriuoasdfkjlahsdfjlha

LA6IM 123 1023 uiwyejhaskjdfhasdfiuyaiuowyeruakjsdfhkjahfdlkja

Each line in PASSWD.SYS can have a maximum of 255 characters.  For the first

tests with the BBS-program, you will only need the first line. Write whatever

you like in it, and have a copy of it an paper. And let nobody else see that

paper (hi).

FBB 5.15                          Feb 06 1993                       Page - 114

18.5 PROTECT.SYS (SYSTEM).

--------------------------

This file is in the SYSTEM directory.

In this file you can specify some directories in which you do NOT want any

sysop to be able to delete any files, without a special syntax. To delete

files in  protected  directories,  the  command  must  look  like  t

+FORWARD.SYS In an un-protected directory the command looks like this: DEL

FORWARD.SYS. So you can see that you need an extra '+' sign in protected

directories.

Example of PROTECT.SYS:

\

\FBB

\TOOLS

FBB 5.15                          Feb 06 1993                       Page - 115

18.6 LANGUE.SYS (SYSTEM).

-------------------------

This file is in the SYSTEM-directory. You do not need to change this file

for the first startups.

First there are 3 numbers. The text in the file explains how to use them.

Next there is a list of all available languages. These names must be the

same names as used in the .TXT and .HLP files in the directory called LANG.

So, there must be an ENGLISH.TXT and ENGLISH.HLP there, if ENGLISH is in

LANGUE.SYS file.

After some comment-lines, there is a long list of prefixes, and the

corresponding number of language that each user will be presented with for

his first connect. The user himself, can change language later with OL-

command in the BBS.

Remember, only the "Number in PC at the time" decides how much memory the

languages will use. All languages will be available anyway, but if more then

(in this case) 4 are in use at the same time, the BBS will swap the less used

language to disk, and load a new one from disk.

#

# File that decides what language each user will use in the first

# connect. He can change language himself later...

# All prefixes not listed here, will receive language no 1 (english).

# 3 parametres :

# Number of languages - Number in PC at the time - Console-language.

#

17 4 1

#

ENGLISH

NORSK

NORSK8

FRANCAIS

SVENSKA

SVENSKA8

DANSK

DANSK8

SUOMI

PORTUGUE

ITALIANO

DEUTSCH

CATALA

ESPAGNOL

NEDERLAN

RUSSE

HRVATSKI

#

# 1 : English

# 2 : Norwegian (7-bits)

# 3 : Norwegian 8-bit

# 4 : French

# 5 : Swedish (7-bits)

# 6 : Swedish 8-bit

# 7 : Danish

# 8 : Danish 8-bit

# 9 : Finnish

# 10: Portugues

# 11: Italian

# 12: German

# 13: Catalan

FBB 5.15                          Feb 06 1993                       Page - 116

# 14: Spanish

# 15: Dutch

# 16: Russian

# 17: Jugoslavian

#

CT* 10

CU* 10

C3* 13

DU* 1

D* 12

EI* 1

E* 14

F* 4

etc. etc.

FBB 5.15                          Feb 06 1993                       Page - 117

18.7 FORWARD.SYS File (SYSTEM).

-------------------------------

The FORWARD.SYS file defines the setup of the various adjacent BBS's to

which we will route the mail. The file is organized in blocks, each block

being specific to a BBS. Only the adjacent BBS's should be defined.

This file and its includes are compiled at the BBS start-up and after

editing with tiny-edit or return from the DOS Shell (AltF9) if one of the

files has been modified.

If the FORWARD.SYS file or one of its includes has been modified all the

active messages are scanned, and new routings will be set according to the

updated file(s). The scanning can be stopped with AltF3. (Can be convenient

if the modification does not impact the routing.)

Every line of the file begins with a letter which defines the function of

the line. Any line beginning with a # is regarded as a comment and is

ignored.

Line types :

A (callsign)

All blocks must start with a line like this. The callsign is the callsign of

the BBS we will forward to.

B (callsign)

All blocks must include at least one line like this. Personal messages

adressed to somebody with adress '@ (callsign)' will be forwarded.

C C (callsign1) [V DIGI1 DIGI2 ETC...]

The complete line of connection (C calsign V digi1 digi2) must follow the

letter C. In most cases, there will be two letters C successively, with a

space between them. The letter V, specifying a string of repeaters is

optional if PROMs TF4 (or equivalent) are used - see WA8DED.DOC. There may be

several lines C, in the case of cascaded connections (use of THENET, as per

an example). A line of conditional connection (enclosed between an IF and an

ENDIF) allows calls on preset times. Up to 8 lines are allowed, each line can

held up to 80 characters.

In case of KA-node and X-connect, we can send C X (callsign). When connected

to a node, all command after the first C is actually sent to that node. Like

here, the X is sent too.

D (command)

Command to be performed by the DOS (transceiver frequency set, for example).

The screen is saved before the command, and restored when done, thus enabling

screen access for this command. When running BAT files, a COMMAND.COM is

automaticly first sent. That needs some more memory.

ELSE

Complementary conditional lines to the previous IF.

ENDIF

End of conditional lines defined by an IF.

F (callsign)

Callsign which will be automatically routed (except if a routing has been

specified) to the BBS which has been previously stated by a B line (must

follow a B line). There may be several F lines following a B line. If this

line is not preceeded with a B line, then the bulletins will be routed on

their destination.

FBB 5.15                          Feb 06 1993                       Page - 118

G (route)

Group routing  indicator  for  bulletins  (EU  for  example).  This

indicator  applies  only  to  bulletins.  The  private  messages  ar

according to the BBS indicator (see line B). There may be several routing

indication lines per adjacent BBS.

H (route)

Route definition using the hierarchical addressing mode. The wildcards (* or

?) must be used to replace the missing part of the address as a function of

the destination.

IF C1 ... IF C9

Conditional test of the route selection. The value for "n" is a figure, from

1 to 9, depending upon the selection. The first selection is 1, and the

maximum is 9. The lines located between the IF Cn and the ENDIF will be taken

into account only if the current selection is n. If the connection has not

been successful (remote BBS prompt not received), the selection number will

be incremented, and the block will be tested again if the port is still the

same. If the port is different, the block will be tested during the tests

related to the new port. If the connection is successful, the selection

number is reset at 1 for the next call, otherwise the selection number is

incremented. If all the selections failed, the selection number is reset at

1, the call is given up, and the system manages the next block.

IF D xx

Conditional test of the days of the week. This line defines a set of lines

which will be taken into account if the expression which defines the day(s)

following the IF is true. 0 corresponds to Sunday and six to Saturday. The

expression 'IF D 0,2,4-6' is true for Sunday, Tuesday, and Thursday to

Saturday. There can be several embedded IF's (indentation recommended), they

shall never expand outside a block. The ENDIF defines the end of the

conditional block.

IF H xx

Conditional test of the hour (IF). This line defines a set of lines which

will be taken into account only if the hour period following the IF is TRUE.

The hour period written in the format 0,4,12-23 is true at 0h, 4h, and for

the hours from 12h to 23h. There may be several IF conditions embedded

(indentation should be used), but never outside of the blocks.

IF FREE

Conditional test for the availabilite of a port. The test will be true if

none of the channels for the port is in use. This test can be a necessity

before the autoqsy of a transceiver as for an example. There can be several

embedded IF's (indentation is recommended), they shall never expand outside a

block. The ENDIF, defines the end of the conditional block.

IF MAN

Conditional test only true if forward has been asked manually by F4, FR or

FW commands.

K Channel

Forces forwarding to start on the specified channel. If the channel is busy,

forwarding will not start.

L (command) [value]

Parametres programmation for the TNC. The parameters will be assigned to the

related channel. The syntax is the one of the TNC hostmode. Only the command

B (Paclen = packet size) has been supplied in all cases. B parameter must be

used with all TNCs (including KAM) to specify a paclen. The paclen is always

managed by the BBS software, as the different host modes are transparent to

the paclen. The PACLEN of the KAM should be 0/0 (256) to make the BBS paclen

work in all circumstances.

FBB 5.15                          Feb 06 1993                       Page - 119

Those parameters will also be used for "reverse forwarding".

N

Assigns the standard forward protocol (MBL/RLI) for the specified BBS.

O (minutes)

Defines a timeout for the specified BBS. Timeout is expressed in minutes.

P (port)

Selects the port on which the connection is to take place. The port is

defined by a letter from A to H for the ports 1 to 8.

Q

Prevents the reverse forwarding from my BBS to take place.

R

Specifies that a call should be made to the adjacent BBS even if there is no

mail in queue, so as to trigger the reverse forwarding.

S Connect Failure Busy

It is a set of supplementary test in the case of sequential connections

(TheNet network for example). The 3 tests should be defined. The tests

"Connect" "Failure" and "Busy" are already defined in the software. Take care

to the capital and lowercase letters, the tests take them into account. Ex.:

S Conne Echec Occup. The software will interpret "Conne" like Connect,

"Echec" like Failure, and "Occup" like Busy. This allows to comply with nodes

messages which are not those commonly used by TheNet.

Up to 4 S lines (set of tests) can be defined in a forwarding block.

T options

Definition of the maximum size for a file authorized to be forwarded, type

of messages and strategy. This line can be made conditional (between an IF

and an ENDIF).

T P : means only private

T S : means smaller first (instead of older first)

T 2 : means only less or equal to 2 KB.

It is possible to combine one or more of these option like :

T P S 2 : means only private less or equal to 2 KB, smallest first.

In all cases, personnal mail is forwarded first, and then bulletins.

U ROUTE1 ROUTE2 ... (up to eight routes)

Give the priority when forwarding bulletins. Bulletins @ROUTE1 will be

forwarded first, then bulletins @ROUTE2, then other bulletins. It may be up

to eight specified routings. Private mail (including P, T and A messages) are

always forwarded first.

V (text)

Text sent on connection of the called BBS. (Mandatory on some TCPIP BBS to

declare the protocol in use). Allows sending callsign and password when

connecting to a phone BBS.

X (command)

Command to be executed by DOS (Selection of a frequency, as for an example).

The screen is not saved before the command, thus, the latter should not

access the screen. This command is identical to the D command, but it is

transparent and faster. When running BAT files, a COMMAND.COM is automaticly

first sent. That needs some more memory.

FBB 5.15                          Feb 06 1993                       Page - 120

Y

Automatic time update of PMS having the clock ability. (if SID looks like

[PMS-3.0-C$]). PMS is set with the local time.

Z route

NTS routing indicator for NTS messages. This routing indicator applies only

to messages of "T" type. Wildcards can be used to define masks. There may be

several routing indication lines per adjacent BBS.

------ (string of dashes)

Defines the end of the block for the adjacent BBS. The block begins with an

A line.

! Line

Defines an exception. Will be used with lines of types B, F, G, H. The

exception should always be set before the true condition statement. An

exception is always from the same type statement (you cannot have a !F

exception to a B line).

Ex:

! B F6FBB Except for F6FBB BBS

B F* Forward to all the BBS which begin with an F

The number of the exceptions is not limited.

< File

Include the file specified. The name of the file should be in the directory

SYSTEM or in one of its subdirectories.

#

Comment line.

Definition of the forward file :

A forward file is made of a main file including a set of blocks, each block

corresponds to an adjacent BBS, that is to say that can be connected.

Definition of the minimum block :

A block will be an included file in the FORWARD.SYS file. It is always made

with a first line beginning with an A, defining the name of the adjacent BBS,

and of a last line made of a string of dashes. Inside this block, there will

be at least one P line (port used) a C line (connection request), and a B

line (destination BBS). An F line with the SYSOP callsign may follow the B,

enabling the automatic routing for his mail.

A FC1CDC

P A                      Port A

C C FC1CDC-1 F6FBB-5     connection line

B FC1CDC BBS             desserved by FC1CDC

F FC1CDC                 callsign autimatically forwarded to FC1CDC

--------

This type of block is well suited for PMS of BBS which do not support

automatic mail forwarding.

Example of a more complex block :

A FC1HAQ

#

G EU                     FC1HAQ will receive

G FRA                    the bulletins for those

FBB 5.15                          Feb 06 1993                       Page - 121

G WW                     various routings.

G AMSAT

F AMSAT                  bulletins to AMSAT are also marked.

#

H *.NA                   All the H. routes which end

H *.SA                   with .NA, .SA

H *.EU                   and .EU

#

O 5                      5 minutes timeout.

#

IF C1                    1st choice parameters

P C

L B 250                Paclen is 250 bytes

L N 4                  Retry 4

C C FC1HAQ-1 F6FBB-7 019301

ENDIF

IF C2                    2nd choice parameters

P C

IF 8-17                from 8h to 17h

T 5                  files of 5 kb Max.

ENDIF

L B 250                Paclen is 250 bytes

L N 4                  Maxframe is 4

C C FC1HAQ-1 F6FBB-7 019002

ENDIF

IF C3                    3rd choice parameters

P A                    port A is used

T P                    private mail only

C C F6FBB-2            Connection of the 1st Thenet.

C C FF6KUB-2           Connection of the 2nd Thenet.

C C FC1HAQ-1           Final connection.

ENDIF

#

B FC1HAQ                 Definition of the various

F FC1HAQ                 BBS routed by FC1HAQ and

F F6CQU                  of its SYSOPs and /or direct

#                        specified users.

B F6ABJ

F F6ABJ                  Other BBS and miscellaneous

#                        callsigns forwarded by FC1HAQ.

B F5LO

F F5LO

#

B G*                     other BBSs.

B ON*

B D*

B HB*

#

------                     End of the FC1HAQ block

It is now possible to use include-files with FBB. In this way we can have a

very small FORWARD.SYS, with only the callsign of the BBSs to forward to, and

then we have another directory with all the forward-blocks. One file for each

BBS, for example. Just transfer one whole block (starting with A (callsign)

and ending with ------) to file, and pu the filename in FORWARD.SYS with a

'<' in front of it, like  this:

Example of FORWARD.SYS file with include-files.

FBB 5.15                          Feb 06 1993                       Page - 122

#

# FORWARD FILE F6FBB

# ------------------------

#

# UHF

#

< FWD\FE6BIG

< FWD\FC1GJC

< FWD\FF6KNI

< FWD\FC1HAQ

< FWD\F1EBV

< FWD\FD1JGK

< FWD\EA3BKZ

#

# PMS

#

< FWD\FD1CDC

< FWD\FD1MEU

< FWD\FD1OAC

< FWD\F8PD

< FWD\F6HED

< FWD\FE1JYH

< FWD\F1LMI

#

# HF

#

< FWD\LA1B

< FWD\LA6CU

< FWD\SV1IW

< FWD\4X1RU

< FWD\RS3A

< FWD\I7XGL

< FWD\EA8AML

< FWD\EA8RT

#

# VHF

#

< FWD\F6CDD

#

# FILES

#

< FWD\AMSAT

< FWD\MAIL

#

# Back to the park frequency.

#

P B

IF 0-3,21-23

X ICOM A2E8 M14

ELSE

X ICOM A2E8 M4

ENDIF

Example of an include file FWD\EA8RT

------------------------------------

A EA8RT

#

G EU

G ALL

FBB 5.15                          Feb 06 1993                       Page - 123

G AMSAT

G WW

G EA

G EANET

G LATNET

#

P B

#

O 2

#

L T 20

L B 80

L O 2

L P 100

L F 2

L @T2 20

#

#

IF 7-22

X ICOM A2E8 M6

C C EA8RT-2

ENDIF

#

B EA8RT

F EA8RT

F E?1*

F E?4*

F E?7*

F E?8*

#

B E?1*

#

B E?4*

#

B E?7*

#

B E?8*

#

----

Automatic forward to a file.

----------------------------

It is interesting to allow automatic routing of messages or bulletins to a

file, for further processing, archive, or communication between BBSs which

co-exist in the same computer. The communication can be made through files.

The incoming mail file (see INIT.SRV) is tested once every minute; in case of

existence, the mail it contains is entered into the BBS.

On the other hand, a file can be defined in a forward block and updated once

every 15 minutes. This file is defined in a block of forward identical to the

forward between BBSs, the port being @ and the connection line given the name

of the file created or updated. The name of the BBS (possibly dummy) used

will be declared in BBS.SYS, of course.

Example of a forward to a file (forward to the file B:MAIL.FBB of my own

private mail) :

A MAIL

P @

C B:\MAIL.FBB

FBB 5.15                          Feb 06 1993                       Page - 124

B F6FBB

F F6FBB

------

End of file DOS command specific to a port.

-------------------------------------------

Each DOS command (D or X line) defined out of a forward block will be run

only if the forwarding port matches with the last port selection.

Example :

You use the autoqsy capabilities of your HF tranceiver with a special

programm named "ICOM". To be sure to return to your standby frequency, you

must set your frequency at the end of the forward file, only if the forward

applies to the HF port. You only have to select the port before running the

DOS command.

B xxxxxx

#

-------                   End of last forward block

#

P D                       Select HF port (port D)

X ICOM A3E8 M12           Send the DOS command ICOM with its

#                         arguments if the file is scanned for

#                         D port. IF and ENDIF are allowed.

FBB 5.15                          Feb 06 1993                       Page - 125

18.8 Specific language files x.TXT, x.HLP, x.ENT, x.INF (SYSTEM\LANG).

----------------------------------------------------------------------

These various files have the names of the languages followed by the suffixes

TXT, HLP, ENT, or INF. They may content variables and are searched for in the

directory holding the files (SYSTEM\LANG).

TXT : file which contains text used during the operation of the software.

Each line is associated to a text used by the software. The number of lines

must not be altered, except for the comment lines which are not taken into

account (lines beginning with a # in the first column).

HLP : Help file called by the ? or H command. The help is reffered to a word

following the command, (ex: ? HELP). Each help block should begin with a line

of a special format:

@@ 14 S

The two @@ characters, without space, in the first column mark the beginning

of a help block. They are followed by an identification number for the type

of handling (see appendix 3), and of the word which identifies the help, in

capital. A block ends at the line beginning with @@ (next block identifier).

ENT : Optional file displayed on the user connection, between the header and

the list of commands. The FRANCAIS.ENT file will be displayed if it exists

and if the caller uses french language.

INF : File displayed when the command "I" is used. If the file does not

exist, the display of the ! command will take place.

FBB 5.15                          Feb 06 1993                       Page - 126

18.9 FBBLOG.nn File (SYSTEM\LOG).

---------------------------------

Most of the executed commands are recorded in the file FBBLOG.nn - n is the

number of the week. This yields a weekly archive which is not of a too big

size.

This file will be updated if the line "Complete Log" of the file INIT.SRV is

"OK".

It is an ASCII file which is automatically created, and does not request any

special care. This single file keeps the activity record for all ports

together.

All the lines begin with a string of figures, corresponding to the month,

day, hour, minute and second of the trace, followed by the channel number,

then the executed command.

A stats analysis software, LOGSTAT.EXE allows the processing of the log file

(see 13.xxx).

FBBLOG file sample :

043004460500SI *** BBS Initialize

043004483200SA *** BBS Online

043004483305CB FC1NOU-0 VIA F6FNL-5

043004483905XD

043004484705CB FC1NOU-0 VIA F6FNL-5

043004500905MR 64072

043004511405XB

043005003201CA F1EBV-14

043005012816CC F1EBV-1 VIA FE6BEX-8 {FC1BPS-7,F1EBV-7,FF6KQK-2}

043005020701MW 64081P F:FC1GQL T:FE1JNY@F6DEG [281] S:F6CQP

043005022816MN B:19226_FF6KNL V:F1EBV

043005424401M> 62893 V:F1EBV [2881]

043005463616MN B:4759_EA6RCM V:F1EBV

043005463616MF 62896 V:F1EBV [393]

043005470813CC FC1HAQ-1 VIA F6FBB-7,019101

043005481701M> 62897 V:F1EBV [872]

043005485816MF 62897 V:F1EBV [872]

043005510701M> 62898 V:F1EBV [815]

043005514516MF 62898 V:F1EBV [815]

043005522901M> 62901 V:F1EBV [398]

043005530516MN B:26051_FC1HAQ V:F1EBV

043005530616MF 62901 V:F1EBV [398]

043005533313MW 64084B F:DF9WM T:ALL@EU [1161] S:Address Needed

043005572701M> 62931 V:F1EBV [2728]

043005580113XD

043005584516MF 62931 V:F1EBV [2728]

043006003012CB F6CDD-1

043006211401XT

043006303216CC FC1HAQ-1 VIA F6FBB-8,019101

043006323016MF 64081 V:FC1HAQ [281]

043006340716MF 64082 V:FC1HAQ [830]

043006354116M< 64086B F:DK3GI T:DXNEWS@EU [953] S:BY4AA in RTTY

043006443216M< 64089B F:OZ2FAR T:ALL@WW [1895] S:re qrt

043006452112CB F6CDD-1

043006465112MF 64086 V:F6CDD [953]

043006465212MF 64087 V:F6CDD [2632]

043006465312MF 64088 V:F6CDD [1626]

043006465312MF 64089 V:F6CDD [1895]

043006465412XB

043006474016M< 64090B F:VS6XQF T:ALL@WW [1748] S:CRY FOR TR-9500

FBB 5.15                          Feb 06 1993                       Page - 127

043006490316M< 64091B F:DJ0AW T:ALL@WW [972] S:mfj-1278

043006534816M< 64092B F:DB2OS T:KEPLER@WW [3684] S:Elements 118.

043007002612CB F6CDD-1

043007005516M< 64095B F:VK5CGB T:ALL@WW [1166] S:RNARS PACKET

043007022116M< 64096B F:DF7MX T:COMMOD@EU [913] S:SUPPORT

043007034316M< 64097B F:I1ZEU T:ALL@EU [1096] S:ID1V

043007065416SQ *** BBS Quit

FBB 5.15                          Feb 06 1993                       Page - 128

18.10 DIRMES.SYS (SYSTEM).

--------------------------

This binary file holds the informations on all active messages. This file is

very important and must not be edited or changed otherwise the list of the

messages could be lost. This file can only be opened for reading.

It is organized with records, each record containing information on a

message. The first record of the file only contains the number of the last

message. Never modify this file while the BBS is running.

Structure of one record (C language) :

#define NBBBS 80

#define NBMASK NBBBS/8

typedef struct {

char type         ;  /* 1   Type of message (A,B,P,T)       */

char status       ;  /* 1   Status of message ($,A,F,K,N,Y) */

long numero       ;  /* 4   Number of the message           */

long taille       ;  /* 4   Size of message in characters   */

long date         ;  /* 4   Date of message                 */

char bbsf[7]      ;  /* 7   Adjacent BBS giving the message */

char bbsv[41]     ;  /* 41  Route                           */

char exped[7]     ;  /* 7   Destination of the message      */

char desti[7]     ;  /* 7   To field                        */

char bid[13]      ;  /* 13  BID or MID                      */

char titre[61]    ;  /* 61  Title of message                */

char free[16]     ;  /* 16  Reserved bytes                  */

long datesd       ;  /* 4   Date of the message creation    */

long datech       ;  /* 4   Date of last status change      */

char fbbs[NBMASK] ;  /* 10  Mask of BBSes to forward to     */

char forw[NBMASK] ;  /* 10  Mask of BBSes already forwarded */

} bullist            ;  /* 194 bytes = length of one record    */

A null type of message (00 hex) invalidates the record.

Dates are given as the number of seconds since january 1st, 1970 00:00.

Masks are bit fields, number of bit corresponds to the number of the BBS in

the BBS.SYS file.

In C language, all strings are ended with a NULL (00 hex) character.

FBB 5.15                          Feb 06 1993                       Page - 129

18.11 STATIS.DAT (SYSTEM).

--------------------------

This binary file holds the informations on all connections. This file is

very important and must not be edited or changed otherwise the list of the

connections could be lost. This file can only be opened for reading.

It is organized with records, each record containing information on a

connection. Never modify this file while the BBS is running.

Structure of one record (C language) :

typedef struct {

char  indcnx[7]    ;  /* 7   Callsign                        */

char  port         ;  /* 1   (Port x 32) + channel           */

long  datcnx       ;  /* 4   Date of the connection          */

int   tpscnx       ;  /* 2   duration of the connection      */

} statis             ;  /* 14  bytes = length of one record    */

Date is given as the number of seconds since january 1st, 1970 00:00.

In C language, all strings are ended with a NULL (00 hex) character.

FBB 5.15                          Feb 06 1993                       Page - 130

18.12 WFBID.SYS (SYSTEM).

-------------------------

This binary file holds the last received BIDs. The number of records of this

file is defined in the INIT.SRV file. This file is very important and must

not be edited or changed otherwise the list of the messages could be lost.

This file can only be opened for reading.

It is organized with records, each record containing information on a BID.

This is a circular file and the first record of the file contains the pointer

of the last overwritten record in the msg_number field. Never modify this

file while the BBS is running.

Structure of one record (C language) :

typedef struct {

char  mode         ;  /* 1   Type of message (A,B,P,T)       */

char  fbid[13]     ;  /* 13  BID of the message              */

long  msg_number   ;  /* 4   Number of the message           */

} bidfwd             ;  /* 18  bytes = length of one record    */

In C language, all strings are ended with a NULL (00 hex) character.

FBB 5.15                          Feb 06 1993                       Page - 131

18.13 INF.SYS file (SYSTEM).

----------------------------

This binary file holds the informations on all users of the BBS. This file

is very important and must not be edited or changed otherwise the list of the

users could be lost. This file can only be opened for reading.

It is organized with records, each record containing information on a user.

Never modify this file while the BBS is running.

Structure of one record (C language) :

#define uchar unsigned char

typedef struct {        /* Callsign structure used in info  */

char callsign[7]   ;

char ssid          ;

} indicat            ;

typedef struct {

indicat  indic     ;  /* 8   Callsign                     */

indicat  relai[8]  ;  /* 64  Digis path                   */

long     lastmes   ;  /* 4   Last L number                */

long     nbcon     ;  /* 4   Number of connexions         */

long     hcon      ;  /* 4   Last connexion date          */

long     lastyap   ;  /* 4   Last YN date                 */

unsigned flags     ;  /* 2   Flags                        */

unsigned on_base   ;  /* 2   ON Base number               */

uchar    nbl       ;  /* 1   Lines paging                 */

uchar    lang      ;  /* 1   Language                     */

char     free[27]  ;  /* 27  Reserved                     */

char     nom[18]   ;  /* 18  1st Name                     */

char     prenom[13];  /* 13  Christian name               */

char     adres[61] ;  /* 61  Address                      */

char     ville[31] ;  /* 31  City                         */

char     teld[13]  ;  /* 13  home phone                   */

char     telp[13]  ;  /* 13  job phone                    */

char     home[41]  ;  /* 41  home BBS                     */

char     qra[7]    ;  /* 7   Qth Locator                  */

char     priv[13]  ;  /* 13  PRIV directory               */

char     filtre[7] ;  /* 7   LC choice filter             */

char     pass[13]  ;  /* 13  Password                     */

char     zip[9]    ;  /* 9   Zipcode                      */

} info               ;  /* 360 bytes = lenght of one record */

Dates are given as the number of seconds since january 1st, 1970 00:00.

In C language, all strings are ended with a NULL (00 hex) character.

FBB 5.15                          Feb 06 1993                       Page - 132

18.14 HROUTE.SYS (SYSTEM).

--------------------------

This ascii file holds the information on BBS hierarchical routing. Each time

a "R:" header is received in a message or bulletin, the hierarchical routing

found in the line is added to a list and updates the HROUTE.SYS file.

This is an example of HROUTE.SYS :

# SM2LZK.GSTAD.BD.SWE.EU

# VE3JJV.NWON.CAN.NA

# W2NRE.NY.USA

# W6MCV.AZ.USA.NA

# W6MCV.AZ.USA.NOAM

V 3B8FP.MRU.AF

P 4X1RU.ISR.MDLE

V 4Z4SV.ISR.MDLE

V 7J1AAA.10.JNET1.JPN.AS

V 7K1DJL.11.JNET1.JPN.AS

V 7Z1AB.SAU.MDLE

The character in first column can be :

# The line is ignored (comment, or line disabled).

P The line is persistent and will not change.

V The line is volatile and will change if a new route exists for this BBS.

When a new route is found for a BBS, the old one is commented by changing

the V character with a #, and a new line will be appended at the end of the

file.

HROUTE.SYS is automaticly updated by the software and does not need any

maintenance.

This file can be sorted and comments deleted with a sequence like :

COPY HROUTE.SYS TEMP.$$$

TYPE TEMP.$$$ | SORT | FIND /v "# " > HROUTE.SYS

DEL TEMP.$$$

This sequence can be included in the APPEL.BAT loop.

This file will be no longer used and updated if the White Pages service is

validated, as the WP service helds the same informations.

FBB 5.15                          Feb 06 1993                       Page - 133

18.15 YAPLBL.SYS file (SYSTEM).

-------------------------------

This binary file holds the descriptions of the FbbDos files.

It is organized with records, each record containing the filename in upcase

characters (with the full path including disk name) and the description of

this file.

Structure of one record (C language) :

typedef struct {

char  filename[40];  /* Complete path of the filename */

char  label[80]   ;  /* Description of the file       */

long  creation    ;  /* Date of creation of the file  */

long  unused      ;  /* Not used should be 0          */

} YapLbl;

Date is given as the number of seconds since january 1st, 1970 00:00.

In C language, all strings are ended with a NULL (00 hex) character.

FBB 5.15                          Feb 06 1993                       Page - 134

18.16 MEMO.SYS (SYSTEM).

------------------------

This ascii file will substitute a text when typing CtrL F1 to Ctrl F10. This

text can hold variables and is limited to 255 characters. It can include $W

variable (CR) and then will be formatted as more than one line.

This file will have 10 lines, the first line corresponding to Ctrl F1, and

the last one to Ctrl F10. If a line is not used, it must have a dot in first

column.

Example of MEMO.SYS file :

$W$O BBS (FBB$E) in $c ($?).$W

.

.

.

.

.

.

.

$W  Vy 73. Jean-Paul, F6FBB @ F6FBB.FMLR.FRA.EU$W$W/EX$W

$W  73 Qro. Jean-Paul, F6FBB@F6FBB.FMLR.FRA.EU$W$W/EX$W

FBB 5.15                          Feb 06 1993                       Page - 135

18.17 SATEL.DAT file (SAT).

---------------------------

This binary file holds the informations on the satellite data base of the

BBS. This file is very important and must not be edited or changed otherwise

the list of the satellites could be lost. This file can only be opened for

reading.

It is organized with records, each record containing information on a

satellite. Never modify this file while the BBS is running.

Structure of one record (C language) :

typedef struct {

char   sat_name[18];  /* 18  Name of the satellite           */

int    year        ;  /* 2   Year of reference               */

double day         ;  /* 8   Day of reference                */

int    month       ;  /* 2   Month of reference              */

int    hour        ;  /* 2   Hour of reference               */

int    minute      ;  /* 2   Minute of reference             */

int    second      ;  /* 2   Second of reference             */

double inclination ;  /* 8   Inclination                     */

double raan        ;  /* 8   R.A.A.N.                        */

double excentricity;  /* 8   Excentricity                    */

double a_perigee   ;  /* 8   Argument of perigee             */

double mean_anomaly;  /* 8   Mean anomaly                    */

double a           ;  /* 8   must be 0.0                     */

double mean_motion ;  /* 8   mean motion                     */

double dec_rate    ;  /* 8   Decay rate                      */

long   revolution  ;  /* 4   Epoch revolution                */

double frequency   ;  /* 8   Frequency for doppler computing */

double v           ;  /* 8   must be 0.0                     */

int    step        ;  /* 2   Step of processing (minutes)    */

long   last_upd    ;  /* 4   Date of last update             */

long   catalog     ;  /* 4   NASA Catalog Number             */

int    free[4]     ;  /* 8   Unused                          */

} satel              ;  /* 138 bytes : lenght of one record    */

In C language, all strings are ended with a NULL (00 hex) character.

FBB 5.15                          Feb 06 1993                       Page - 136

18.18 nnnnn.SAT (SAT) (Satellite characteristics).

--------------------------------------------------

The nnnnn.SAT  files  (where  nnnnn  is  the  NASA  catalog  numbe

satellite) will be displayed when the command C will be typed from the

satellite menu. They are text files and are located in the SAT subdirectory.

FBB 5.15                          Feb 06 1993                       Page - 137

18.19 WP.SYS file format.

-------------------------

The WP.SYS file is composed of records. Each record is 194 bytes long.

typedef struct {

char    callsign[7]      ;  // Callsign of the record

char    name[13]         ;  // Name of the user

char    free             ;  // Unused

char    changed          ;  // Flag (U, G, I) of modification

unsigned     seen        ;  // Number of updates

long    activ_date       ;  // Date of active part

long    temp_date        ;  // Date of temp part

char    activ_homebbs[41];  // Home BBS in active part

char    temp_homebbs[41] ;  // Home BBS in temp part

char    activ_zip[9]     ;  // Zip code in active part

char    temp_zip[9]      ;  // Zip code in temp part

char    activ_qth[31]    ;  // Qth in active part

char    temp_qth[31]     ;  // Qth in temp part

} Wps;

In C language, all strings are ended with a NULL (00 hex) character.

This file should NOT be modified while the BBS software is running !

FBB 5.15                          Feb 06 1993                       Page - 138

18.20 MESS.WP file format.

--------------------------

The MESS.WP file is an ascii file composed of standard WP update lines. It

has no size limit and will be sent as messages to the different routes

defined for WP updates in INIT.SRV. The outgoing messages will be truncated

to 5Kb sections if required.

This file is read at the end of the BBS start-up.

FBB 5.15                          Feb 06 1993                       Page - 139

APPENDIX -1- Commands of the line editor EDIT (FBBDOS)

------------------------------------------------------

The FBBDOS offers a small line editor allowing the process of ASCII files.

It can help the distant SysOp maintening the system files of the software.

The commands are straightforward, and for a better efficiency, they can be

chained on a single line.

List of the commands:

? : Gives the list of the editor's commands.

A : Adds one line after the current line. The text of the line follows the

command, and must be terminated with the character /. The character / can be

imcluded into the character string provided it is preceded by a character \.

To include a \ character into the string, this character must be sent twice

(\\). The line pointer is on the inserted line.

B : places the line pointer at the beginning of the file.

E : places the line pointer at the end of the file.

F : Search for the first occurence of the character string as specified

after the command, starting from the current line. The character string

should be terminated by the character /. The character / can be inserted into

a string under the condition that a \ character is leading. The \ character

must be sent twice if it is to be considered as a valid character. The line

pointer stands on the line where the string has been found.

I : Inserts a line ahead of the current line. The text of the line follows

the command and must be terminated by the character /. The character / can be

included into the string provided it is preceded by a character \. The

character \ must be sent twice if it is to be considered as a valid

character. The line pointer stands on the inserted line.

K : Suppresses the number of lines specified before the command, starting

from the current line. If the number is not stated, the value 1 is taken as a

default.

L : moves the pointer of the number of lines specified before the command.

This number can be positive or negative.

N : Enables or disables the numbering of the lines.

P : Displays the number of lines specified before the command, starting at

the current line.

R : Search for the first occurrence of the character string specified after

the command, and replace it by the second character string. The character

strings must be terminated by a character /. The / character can be included

provided it is preceded by a \ character. The \ character must be sent twice

if it is to be taken into account as a valid character. The line pointer

stays on the modified line.

S : Saves the modified file.

Q : Exits the editor. Take care, the file is not automatically saved. Think

about doing it before you quit.

Examples :

EDIT>B5L10P

FBB 5.15                          Feb 06 1993                       Page - 140

places the pointer at the beginning of the file, advances 5 lines, displays

10 lines.

EDIT>B4L6K-2L10P

Places the pointer at the beginning of the file, advances 4 lines, deletes 6

lines, steps back 2 lines and displays 10 lines.

EDIT>BFBonjour/K-2L5P

Sets the pointer at the beginning of the file, searches for "Bonjour",

suppresses the line (containing "Bonjour"), steps back

2 lines, and displays 5 lines.

EDIT>RBonjour/Au revoir/-2L5P

Searches for "Bonjour", replaces by "Au revoir", steps back 2 lines and

displays 5 lines.

EDIT>BFBonjour/IC'est une new line with a \/inside/-1L3P

Sets the pointer at the beginning of the file, searches for "Bonjour",

inserts a new line, steps back 1 line, and displays 3 lines. This lines

includes a character /. A leading \ indicates that it is to be conside red as

a valid character and not as a delimiter.

EDIT>SQ

Saves, and quits the editor.

FBB 5.15                          Feb 06 1993                       Page - 141

APPENDIX -2- Commands corresponding to the PK232 host-mode (as per F6AIW)

-------------------------------------------------------------------------

8B 8BITCONV AU AAB      AB ABAUD    AG ACHG     AA ACRDISP

AK ACRPACK  AT ACRRTTY  AE ADDRESS  AD ADELAY   AI ALFDISP

AP ALFPACK  AR ALFRTTY  AL ALIST    AM AMTOR    AC ARQ

AO ARQTMO   AS ASCII    AY ASPECT   AW AWLEN    AV AX25L2V2

AX AXDELAY  AH AXHANG   BA BAUDOT   BE BEACON   BI BITINV

BK BKONDEL  BT BTEXT    CL CANLINE  CP CANPAC   CX CASEDISP

CU CBELL    CC CCITT    CF CFROM    CB CHCALL   CD CHDOUBLE

CH CHSWITCH CK CHECK    CQ CMDTIME  CM CMSG     CI CODE

CN COMMAND  CE CONMODE  CO CONNECT  CY CONPERM  CG CONSTAMP

CI CPACTIME CR CRADD    CT CTEXT    CW CWID     DS DAYSTAMP

DA DAYTIME  DC DCDCONN  DL DELETE   DF DFROM    DI DISCONNE

DW DWAIT    EA EAS      EC ECHO     ES ESCAPE   FA FAX

FN FAXNEG   FE FEC      FL FLOW     FR FRACK    FS FSPEED

FU FULLDUP  GR GRAPHICS HB HBAUD    HD HEADERLN HI HID

HO HOST     HP HPOLL    ID ID       IL ILFPACK  IO IO

JU JUSTIFY  KI KISS     LR LEFTRITE LO LOCK     MX MAXFRAME

MB MBX      MC MCON     MD MDIGI    MM MEMORY   MI MFILTER

MF MFROM    MH MHEARD   MN MONITOR  MO MORSE    MP MSPEED

MR MRPT     MS MSTAMP   MT MTO      MA MYALIAS  ML MYCALL

MG MYSELCAL MK MYALTCAL NE NEWMODE  NO NOMODE   NR NUCR

NF NULF     NU NULLS    OK OK       OP OPMODE   PA PACKET

PL PACLEN   PT PACTIME  PR PARITY   PS PASS     PX PASSALL

PE PERSIST  PP PPERSIST PC PRCON    PF PRFAX    PO PROUT

PY PRTYPE   RW RAWHDLC  RB RBAUD    RC RCVE     RE RECEIVE

RX RXREV    RD REDISPLA RL RELINK   RS RESET    RP RESPTIME

RT RESTART  RY RETRY    RF RFEC     SE SELFEC   SP SENDPAC

SI SIGNAL   SL SLOTTIME SQ SQUELCH  SR SRXALL   ST START

SO STOP     TB TBAUD    TC TCLEAR   TM TIME     TR TRACE

TW TRFLOW   TI TRIES    TD TXDELAY  TF TXFLOW   TX TXREV

UN UNPROTO  UR USERS    US USOS     VH VHF      WI WIDESHFT

WO WORDOUT  WR WRU      XW XFLOW    XM XMIT     XO XMITOK

XF XOFF     XN XON

FBB 5.15                          Feb 06 1993                       Page - 142

APPENDIX -3- Process number and on-line help identification.

------------------------------------------------------------

The various processing functions contained in the software are identified by

three numbers displayed in the status banner. The first of those three

numbers  is  the  main  process  system  (BBS,  FBBDOS,  Satellite

Computation, etc...), the second number is the process function (in the BBS,

list, message sending, etc..) and the third number is a sub-function (record

of the message title, message, etc..)

A complete description of these numbers would be useless and time consuming.

They are mainly used for debugging purpose. The first number is also used to

identify the help block out of the x.HLP file. Upon receipt of the "?" or the

"H", the  software  searches  the  x.HLP  file  for  a  line  of

corresponding to the language in use, and of the format @@ number word in

which "number" stands for the current processing level, and "word" stands for

the word following the command "?" or "H".

Example : you are inside FBBDOS, and you type in the command "? EDIT", the

help block searched for must begin with the line:

@@ 9 EDIT

It may happen that a block matches several search keywords. It is enough to

specify the various words separated by the character "|" (vertical bar), WITH

NO SPACE.

@@ 9 EDIT|EDITEUR

List of the processing levels :

0 Connection.

2 Qra-Locator.

3 Statistics.

4 Informations.

5 Nomenclature.

6 Satellite Orbital Computation.

9 FbbDos.

11 Telefone Modem

14 BBS.

15 Forward.

16 GatewaySysop page.

17 YAPP.

18 Conference.

FBB 5.15                          Feb 06 1993                       Page - 143

APPENDIX -4- Recording a message.

---------------------------------

A message can be left by a user or within a forwarding connection. The

recording mechanism is always the same.

The recording command is always like :

Sx desti @ bbs < exped $ ident + filename

Only recipient field is mandatory, all other fields are optional.

Appending a filename is a possibility reserved to the sysop. The name must

be complete, including logic unit and complete path (C:\FBB\SYSTEM\TEST.TXT).

When receiving the command line, a first test checks if a route exists when

a route has been specified, or if the message must be automatically routed

when no route was specified.

The title of the message is then asked to the user.

If the title is a missing, the message is canceled and the user returns to

the main menu.

The text of the message is then asked to the user.

The software checks possible preamble lines. These lines give informations

on the previous BBS having routed this message. They all begin by R: on first

column. The BBS callsign is given behind the @ character within the preamble

line. All adjacent BBS mentioned in this preamble will be included in the

"already forwarded" list, and will not be concerned by this message. This

list specific to each message can be displayed with the $ or FN command

followed by the message number.

When receiving a /EX in first column, or a Ctrl Z, a message number is then

assigned, The BID (or MID if private) and the list of adjacent BBS concerned

by this message are created. All these informations are sent to the user when

acknoledging the message.

In case of disconnection before the /EX or Ctrl Z, the whole message will be

lost, and the texts already stored are deleted.

All informations about the message (sender, recipient, route, MID, title,

etc...) are stored in the DIRMES.SYS file. The text of the message is stored

in a sub-directory of the MAIL directory. The sub-directory is MAILn where n

is the last digit of the message number. The name of the file corresponds to

the message number 123 is M_000123.MES, the number is 6 digits wide, in this

case it is in the sub-directory MAIL3.

The message number uses a long integer (32 bits), the number boundary is

very far (more than 4 billions !).

FBB 5.15                          Feb 06 1993                       Page - 144

APPENDIX -5- Tricks and tips.

-----------------------------

This rubric is yours, more than mine. I'll try to insert there all tricks

you will tell me.

A/ Using DesqView (c).

------------------------

There is no particular problem when using DesqView. The minimum window size

is 500 KB. You MUST use communication drivers, like ESS, COMBIOS or MBBIOS,

as the software does not dispose of the whole process time.

B/ Communication errors displaying.

-----------------------------------

An error counter can be displayed just right of the date, on the first line

of the screen. If you are using TNC2 with WA8DED software, these errors can

be minor, but with PK232, error recovery is more difficult, and the system

may reboot.

With a correct operation of your system, this counter will not appear, or

exceptionnaly. If errors are displayed, they can result from :

- Using DOS 4.0 or 5.0 : The keyboard driver of these versions is very slow.

You must use the ESS driver for RS232 (or COMBIOS).

- A too hight baudrate, or RS232 defective cables. The baudrate can be

selected down to 4800 Bds. It is not a good idea to go down 4800 Bds, as the

performance of the software should be lower.

- Change the LM324 fitting out some TNC RS232 line drivers by a TL074 or

TL084.

- HF detection in the TNC. Errors and resynchronizations will appear when

the transmitter is running. There is no real cure, you must investigate.

You can also use communication drivers like ESS, COMBIOS or MBBIOS if you

are not still using them.

C/ Repetition of the last message number.

-----------------------------------------

The last message number displayed, read, killed, etc... can be utilised

again with the # (pound) character. This short-cut allows as for an example

to read a message after a list or to suppress it just after its reading.

Example :

F6FBB BBS > R 12351

The message is displayed ...

F6FBB BBS > K #

Message #12351 killed.

F6FBB BBS >

FBB 5.15                          Feb 06 1993                       Page - 145

APPENDIX -6- FBB Forward Protocole.

-----------------------------------

FBB software includes two forward protocoles. The first one is standard with

MBL/RLI  protocole.  The  second  one  was  developped  to  allow

efficiency, particularly on long links where propagation time of data are

long. The exchange of commands is reduced to a minimum, and not acknoledged

to get time. The data transfer direction is changed every block of data, a

block of data holding up to five messages. This uses the "pipeline" effect of

long links (Nodes and digipeaters), and gain some time over short links

(HF...).

FBB protocole is very simple in its principle. It is based on MID/BID usage.

The identification is made by the F letter in the SID (system type identifier

contained in square brackets). All command lines must start in first collumn

with the 'F' character. All command lines are ended by a return (CR)

character.

Suppose I call another BBS to forward some mail. When I connect another BBS

using FBB protocole, I will receive the SID followed by a text and the prompt

(">"). If the SID contains the F flag, I will send immediately my SID and the

first proposal.

Proposals looks like :

FB P F6FBB FC1GHV FC1MVP 24657_F6FBB 1345

F> HH

FB : Identifies the type of the command (proposal)

P : Type of message (P = Private, B = Bulletin).

F6FBB : Sender (from field).

FC1GHV : BBS of recipient (@field).

FC1MVP : Recipient (to field).

24657_F6FBB : BID ou MID.

1345 : Size of message in bytes.

F> : End of proposal.

HH is optional. It is the checksum of the whole proposal in hexadecimal.

ALL the fields are necessary. This kind of command must hold seven fields.

If a field is missing upon receiving, an error message will be send

immediately followed by a disconnection.

A proposal can handle up to five FB command lines. If the total size of

messages seems to be too important, the proposal can handle less lines. In

FBB software, a parameter is defined in INIT.SRV file to tell the maximum

size of the message block. It is set by default to 10KB.

Exemple of proposal :

FB P F6FBB FC1GHV.FFPC.FRA.EU FC1MVP 24657_F6FBB 1345

FB P FC1CDC F6ABJ F6AXV 24643_F6FBB 5346

FB B F6FBB FRA FBB 22_456_F6FBB 8548

F> HH

This proposal is limited to three FB lines, as the amount of messages

overran the 10KB limit.

When receiving the proposal, the other BBS will reject, accept or defer the

message. This command is made by a FS line :

FS -+=

This means :

FBB 5.15                          Feb 06 1993                       Page - 146

- I don't want the first message (-).

- I need the second message (+).

- I defer the third message, as I'm still receiving it.

It should interesting to defer a message if you are still receiving it on a

other channel, or if you think that the size is to big, or for another

reason. The message should be proposed again at the next connection.

FS line MUST have as many +,-,= signs as lines in the proposal.

When receiving the FS lines, I can send the block of messages. Each message

is made with the title on the first line, the text, and a Ctrl Z in the last

line. The is no blank line between the messages.

Title of 2nd message

Text of 2nd message

.....

^Z

When the other BBS has received all the asked messages, it acknoledges by

sending its proposal, and the system is reversed.

If it has no message to send, it only sends a line :

FF

This line must not to be followed by a F>.

If the other hand has no message, it sends a line :

FQ

and asks for the disconnection.

Example :

---------

F6FBB                          FC1GHV

----------------------------------------------------------------

Connects FC1GHV

Connected

[FBB-5.11-FHM$]

Bienvenue a Poitiers, Jean-Paul.

>

[FBB-5.11-FHM$]     (F6FBB has the F flag in the SID)

FB P F6FBB FC1GHV.FFPC.FRA.EU FC1MVP 24657_F6FBB 1345

FB P FC1CDC F6ABJ F6AXV 24643_F6FBB 5346

FB B F6FBB FRA FBB 22_456_F6FBB 8548

F> HH

FS +-+ (accepts the 1st and the 3rd).

Title 1st message

Text 1st message

......

^Z

Title 3rd message

FBB 5.15                          Feb 06 1993                       Page - 147

Text 3rd message

......

^Z

FB P FC1GHV F6FBB F6FBB 2734_FC1GHV 234

FB B FC1GHV F6FBB FC1CDC 2745_FC1GHV 3524

F> HH

FS -- (Don't need them, and send immediately the proposal).

FB P FC1CDC F6ABJ F6AXV 24754_F6FBB 345

F> HH

FS + (Accepts the message)

Title message

Text message

......

^Z

FF (no more message)

FB B F6FBB TEST FRA 24654_F6FBB 145

F> HH

FS + (Accepts the message)

Title message

Text message

......

^Z

FF (still no message)

FQ (No more message)

Disconnection of the link.

In this example, FBB protocole is used as the two BBS were identified by the

F flag in the SID. If F6FBB had sent the SID [FBB-5.11-MH$] when answering

FC1GHV, the protocole should be the standard MBL/RLI.

All callsigns are only examples !

FBB 5.15                          Feb 06 1993                       Page - 148

APPENDIX -7- Extension to the protocole. Compressed forward FBB.

----------------------------------------------------------------

The protocole utilized for the transfer of ascii files compressed is an

extension to the existing protocole. The compressed forward is validated by

the presence of the letter B in the SID [FBB-5.12-BFHM$]. The transfer of

compressed files can only take place under FBB protocole. The presence of the

letter B in the SID without the F letter will remain without effect.

The only difference as regard to the standard protocol is the submit line.

It can specify the type of data contained in the compressed message. FA means

that the transfer will be an ascii compressed message. FB means that the

message will be a binary compressed file (this last possibility is not yet

implemented in the version 5.12).

The submission of an ascii message will be in the form :

FA P FC1CDC F6ABJ F6AXV 24754_F6FBB 345

The submission of a binary file will be in the form :

FB P FC1CDC F6ABJ F6AXV 24754_F6FBB 345

The transfered data are of a specific format. The transfer will be done in

binary mode. This last one is derived of the YAPP protocol which is very

reliable. All transfer is made of a header, a block of data, an end of

message and a checksum. Each transfer is equivalent to the transfer of one

message of the standard protocol and shall not be followed by a control Z,

the end of file specifier is defined in another way.

Format of header for an ascii compressed message (submission FA) :

<SOH> 1 byte = 01 hex

Length of the header 1 byte = Length from the title,

including the two <NUL> characters.

Title of the message 1 to 80 bytes

<NUL> 1 byte = 00 hex

Offset 1 to 6 bytes

<NUL> 1 byte = 00 hex

Format of header for a binary compressed file (submission FB) :

<SOH> 1 byte = 01 hex

Length of the header 1 byte = Length from the filename,

including the two <NUL> characters.

Name of the file 1 to 80 bytes

<NUL> 1 byte = 00 hex

Offset 1 to 6 bytes

<NUL> 1 byte = 00 hex

As to follow the french regulation, the title of the message or the file

name are transmitted in ascii, not compressed.

The offset is also transmitted in ascii and specifies the offset at which

the data should be inserted in the file (in case of a fragmented file). In

the version 5.12, this parameter is not utilized and is always equal to zero.

A data block contains from one to 256 bytes. It begins by two bytes which

specify the format.

Data block format :

<STX> 1 byte = 02 hex

Number of data 1 byte = 00 to ff hex. (00 if length = 256 bytes).

FBB 5.15                          Feb 06 1993                       Page - 149

Data bytes 1 to 256 bytes

The last data block is followed by the end of file specifier and the

checksum.

End of file specifier format :

<EOT> 1 byte = 04 hex

Checksum 1 byte = 00 a ff hex

The checksum is equal to the sum of all the data bytes of the transmitted

file, modulo 256 (8 bits) and then two's complemented.

The checking of the checksum is very simple :

The sum of the datas from the file and the checksum received modulo 256

(anded with FF) shall be equal to zero.

In case of a checksum error, the message or the file is not taken to account

and the system issues a disconnect request after having sent the comment :

*** Checksum error

FBB 5.15                          Feb 06 1993                       Page - 150

APPENDIX -8- Format of the ACK messages.

----------------------------------------

The ACK messages on receiving have a simple and compact format. The aim is

to have a message as short as possible in order to avoid an unnecessary usage

of the network.

The title of the message is the title of the original message with a leading

"ACK:". Example :

ACK:Title of the original message.

These ACK messages are true messages strickly speaking. They carry the

origin, the destination, the route and the MID but they are of a particular

type, the type A (private are of type P, bulletins of type B, etc...). This

difference allows the routing of these messages without the lines "R:". This

is done again with the aim of avoiding an excessive load by data which are of

no use in this case.

To keep the compatibility with the existing forwarding protocol, the type of

these messages is changed to P (private) if the receiving BBS of the

forwarding does not know the type of ACK messages (specified in the SID [FBB-

5.12-ABFHM$] by the letter A). In this case, the ack message will continue on

its route as a private message.

The ACK messages are of the following form :

ACK:Message test. <-- Title of message

Msg FD1CDC@F6FBB - 22-dec 17:28z <-- Text of message

It tells that the message that you had sent to FD1CDC at F6FBB and whose

title is "Message test" has been received in the BBS F6FBB on 22 dec at 12:28

GMT.

FBB 5.15                          Feb 06 1993                       Page - 151

APPENDIX -9- Replacement characters or wildcards.

-------------------------------------------------

Most of the search commands or list commands and some configuration files as

well, accept replacement characters or wildcards.

Character Replaces

-----------------------------------------------------------------

@         a letter

?         an alphanum character (letter or number)

=         a printable character

#         a numeral character or the # character

*         a string of printable characters.

&         a dot followed by printable characters. (equiv to .* )

-----------------------------------------------------------------

FBB 5.15                          Feb 06 1993                       Page - 152

APPENDIX -10- Programming technics for servers.

-----------------------------------------------

The servers are exec programs (.COM or .EXE). They are compact and fast.

They will work as the function of the messages which are addressed to them.

They should be compact because the available memory to run their application

is limited (check the information Ok:nnnn in the status window). They should

be fast because they are executed in the MsDos environment which is not

multi-task.

The programming language can be of any kind provided that it could be

compiled and that it is able to read parameters which are given appended in

the command line.

I wrote three servers in TurboC but I have no equivalent in TurboPascal or

in TurboBasic, since I usually don't write in these languages. The working

principle remains always the same whatever language is utilized.

The program is called with the following manner from the MsDos (Example for

the REQDIR.COM command) :

C> REQDIR.COM TEMP.$$$

TEMP.$$$ is the name of the file in which the message addressed to REQDIR is

located. It is necessary to read the name of this file in the command line,

as the one can change from one call to another.

The file TEMP.$$$ contains the message with the following format :

SP REQDIR < F6FBB

Title of message

Text of message line 1

Text of message line 2

...

Text of message last line.

/EX

The server should then eventually work as a function of the contents of this

message.

The server can read and make use of the configuration file of the BBS

software (in particular INIT.SRV) to execute its process.

If the server generates a return message, it should be APPENDED to the

incoming mail file to the BBS. The name of this file can be found in

INIT.SRV. Take care : it is necessary to open the incoming mail file in

APPEND as to add the answer at the end of the file. If it is not done this

way, the messages which could be waiting in this file are destroyed.

The incoming mail file is tested each and every minute, except in the case

of the usage of a service, where it is tested right after.

The format of the messages in the incoming mail file is identical to the

format of the file given to the server. Several messages can be written

sequentially in the file. There should not be blank lines or separations

between the messages. The routing fields (@ field), and the originator (<

field) should mandatory be specified. The originator field is the callsign of

the BBS which is taken from the INIT.SRV file.

FBB 5.15                          Feb 06 1993                       Page - 153

Example of server REQFIL written in C language.

/*

* REQFIL.C Server example.

*

* This server is called with a command line like this :

*

* REQFIL.COM FILE

*

* FILE is the filename of the message to be answered.

*

*

* This server answers to a message like this :

*

* SP REQFIL < FC1EBN

* TEST.TXT @ F6ABJ

* Text is not necessary

* /EX

*

* by a message like this

*

* # <- This is a local message

* SP FC1EBN @ F6ABJ < F6FBB <- command line

* Req File : TEST.TXT <- subject

* Contents of the file <- text

* etc.....

* /EX <- end of text (must be in 1st column)

*

* Appent to mail in bbs file.

*

*

* The server receives from FBB software 1 argument :

*

* argv[1] = Name of the file including the message received from

* FBB software.

*

* ============================================

* The server must APPEND its answer to MAIL.IN

* file to avoid destroying existing mail.

* ============================================

*

* As this server opens the INIT.SRV file, it must be in the same

* directory.

*

*/

#include <stdio.h>

#include <fcntl.h>

#include <sys/stat.h>

/* Offsets of parameters from INIT.SRV */

#define BBS_CALL 1

#define USER_DIR 8

#define MAIL_IN 14

main(int argc, char **argv)

{

#define LINE 80

int end = 0;

int index = 0;

FILE *fptr;

FBB 5.15                          Feb 06 1993                       Page - 154

char buffer[LINE];

char sender[LINE];

char route[LINE];

char file[LINE];

char bbs_call[LINE];

char base_dir[LINE];

char mail_in[LINE];

if (argc != 2) exit(1);            /* Check the number of arguments */

/* The first task is to open and then read the message */

fptr = fopen(argv[1], "rt") ;      /* Open the received message */

if (fptr == NULL) exit(1);

fgets(buffer, LINE, fptr);         /* Read the command line */

sscanf(buffer, "%*s %*s %*s %s\n", sender);

*file = *route = '\0';

fgets(buffer, LINE, fptr);         /* Read the subject */

strupr(buffer);                    /* Capitalize */

/* Scan dir and route */

sscanf(buffer, "%[^@\n]%[^\n]", file, route);

fclose(fptr);                   /* All needed is read in the message */

/* We must get some informations from the INIT.SRV file */

fptr = fopen("INIT.SRV", "rt"); /* Open the file */

if (fptr == NULL) exit(1);

/* Scan the file to get the requested lines. */

while (!end) {

fgets(buffer, LINE, fptr) ;

if (*buffer == '#') continue; /* Comments ignored */

switch (++index) {

case BBS_CALL:

sscanf(buffer,"%[0-9A-Za-z]", bbs_call);

break; /* Callsign */

case USER_DIR:

sscanf(buffer,"%s\n", base_dir);

break; /* Users directory */

case MAIL_IN :

sscanf(buffer,"%s\n", mail_in);

end = 1; /* Mail in file */

break;

}

}

fclose(fptr);

/* Append the answer to mail in file */

/* Mail in file is opened in appent text mode */

if (fptr = fopen(mail_in, "at")) {

/* Tell that this is a message from this BBS */

FBB 5.15                          Feb 06 1993                       Page - 155

fprintf(fptr, "#\n");

/* Send command line */

fprintf(fptr, "SP %s %s < %s\n",

sender, route, bbs_call);

/* Send subject and requested file */

send_file(fptr, base_dir, file);

/* Send end of message */

fprintf(fptr, "/EX\n");

/* That's all ! */

fclose(fptr);

}

exit(0);                                /* Tell BBS all is correct */

}

int points(char *ptr)   /* Looks for a ".." sequence in the path */

{

while (*ptr) {

if ((*ptr == '.') && (*(ptr+1) == '.')) return(1);

++ptr;

}

return(0);          /* ".." not fond ! */

}

send_file(FILE *fptr, char *base_dir, char *filename)

{

#define BUF_SIZE 1000

int fd;

int nb;

char path[256];

char buffer[BUF_SIZE];

char last_char;

sprintf(path, "%s%s", base_dir, filename);       /* Complete path */

fprintf(fptr, "ReqFil 1.1 : %s\n", filename);       /* Subject */

if ((!points(path)) && ((fd = open(path, O_RDONLY | O_TEXT)) != -1)) {

while (nb = read(fd, buffer, BUF_SIZE)) {

fwrite(buffer, nb, 1, fptr);

last_char = buffer[nb-1];

}

close(fd);

/* Be sure /EX will be in first column */

if (last_char != '\n') fputc('\n', fptr);

}

else fprintf(fptr, "File not found !\n");

}

FBB 5.15                          Feb 06 1993                       Page - 156

APPENDIX -11- Contents of directories.

--------------------------------------

These files and directories are mandatory. Other files may reside but will be

created by the system or by the sysop.

Directory FBB :

[BIN]         BAT, COM and EXE files.

[BINMAIL]     Compressed messages.

[DOCS]        Files used for DOCS command.

[OLDMAIL]     Archives of messages.

[MAIL]        Ascii messages.

[PG]          COM and EXE files for PG command.

[SYSTEM]      System files.

EPURMESS INI  Init file for EPURMESS.

INIT     SRV  Init file for SERV.

NEWDOC   SYS  Configuration for NEWDOC server.

Directory BIN :

APPEL    BAT  Batch file running the software.

CLEANUP  COM  Program to clean MAIL and BINMAIL directory.

CUT      COM  Copy/paste utility.

EPURMESS COM  Message processing.

EPURWP   COM  White Pages processing.

ESS      COM  RS232 driver.

ESSKAM   COM  KAM RS232 driver.

FBBIOS   COM  MODEM driver.

FV       COM  FV shareware used for LIST command in FbbDos.

LOGSTAT  EXE  Log statistics.

MAINTINF COM  INF.SYS maintenance program.

MAINTREJ COM  REJET.SYS maintenance program.

MKINF514 COM  Change INF.SYS for 5.13 to 5.14

NEWDOC   COM  NEWDOC server.

REQDIR   COM  REQDIR server.

REQFIL   COM  REQFIL server.

SATUPDAT EXE  Automatic update of satellite database.

SERV     EXE  FBB software EXE file.

SETUSER  COM  set a user owner of a FbbDos file.

SLEEP    EXE  waits for some seconds.

TLABEL   COM  processes label file of FbbDos.

Directory BINMAIL :

[MAIL0]        [MAIL1]        [MAIL2]       [MAIL3]         [MAIL4]

[MAIL5]        [MAIL6]        [MAIL7]       [MAIL8]         [MAIL9]

Directory MAIL :

[MAIL0]        [MAIL1]        [MAIL2]       [MAIL3]         [MAIL4]

[MAIL5]        [MAIL6]        [MAIL7]       [MAIL8]         [MAIL9]

Directory SYSTEM :

[FWD]         Directory for FORWARD.SYS includes

[LOG]         Directory for FBBLOG.nn files.

[LANG]        Directory for .TXT .HLP .ENT language files.

[SAT]         Directory for satellite files

[WP]          Directory for White pages database

BALISEn  SYS  Beacon file.

BBS      SYS  BBS callsigns

CRON     SYS  Hour work.

INITTNCn SYS  Initialization of TNC.

FORWARD  SYS  Forwarding description.

LANGUE   SYS  Language description

MAINTn   SYS  Housekeeping text.

FBB 5.15                          Feb 06 1993                       Page - 157

MEMO     SYS  CtrlF0 to CtrlF9 aliases

PASSWD   SYS  Password file.

PORT     SYS  Port description.

PROTECT  SYS  Directories protection.

REJET    SYS  Messages reject.

SWAPP    SYS  Swap file.

FBB 5.15                          Feb 06 1993                       Page - 158

APPENDIX -12- CONNECTION FILTERING.

-----------------------------------

FBB software allows filtering on connection. Filtering is not done by the

BBS software but by external programs developped by users.

Connection filter may be interactive and allows to incorporate some features

like dedicated information for predifined callsigns, password filtering,

etc...

I did not develop such programs, but this is an open door to many

applications.

The C_FILTER program must be found by the PATH of MsDos. Its extension can

be COM or EXE, and it must be little and fast as multitasking is stopped

during the activity of this program. If this program is not found, it will

not be called until the BBS is rebooted.

When receiving the connection, the C_FILTER program (if found) is called

with some arguments including a level number. This number is incremented each

time the program is called in the same connection session. The first time the

level number will be 0.

The line arguments given to the C_FILTER program are :

- Callsign (format as F6FBB-8).

- Level number (0 is the first time, up to 99).

- Flags of the user (binary number as user's mask of INIT.SRV).

- New : Flag indicating if the user is unknown in the BBS database.

- Record number of the user in INF.SYS.

- Received data (each word is a new argument).

The C_FILTER program ends with an exit value. This value is very important

and tells the BBS what to do :

0 : end of session and return to the BBS menu.

1 : the program will be called again and the level number is incremented.

2 : the user will be disconnected.

/*

* C_FILTER.C

*

* Connection filter for FBB BBS software. (C) F6FBB 1991.

*

*/

#include <stdio.h>

/*

* Connexion filter called for each connection.

* All datas sent to stdout will be sent to the user.

*

* Filter is called with some arguments on the command line :

* C_FILTER CallSign Level Flags New Record ReceivedData....

*

* The return value tells the BBS if C_FILTER must be called again or not

* 0 if the BBS can go on,

* 1 if the C_FILTER must be called again

* 2 if the user must be disconnected.

*

* Callsign is in the FORM CALLSIGN-SSID (F6FBB-0).

FBB 5.15                          Feb 06 1993                       Page - 159

*

* The first time C_FILTER is called Level=0, and then will be incremented

* each time it is called for the same connection.

*

* Flags give the flags of the user (MGPBSXLE) in a bit field. as defined

* in the INIT.SRV user's mask. (0x80=Modem, 0x40=Guest, etc...)

*

* If New=1, then this is the first connection of the user on the BBS.

* Record is the record number in the INF.SYS file.

*

* All other arguments are the words sent by the user

* (password for instance).

*

* The number of arguments is variable and depends of the number of words

* in the answer of the user.

*

*/

/*

* This is only a little example to test the system. It will be called

* four times and will give the list of arguments.

*

* The fourth time, the hand will be given back to the BBS.

*/

main(int ac, char **av)

{

int i;

int level = atoi(av[2]);            /* Get level from argument list  */

/* and transform it to integer   */

if (level == 0) {                   /* Is level equal to 0 ?         */

printf("Connection line :\n");    /* This is the first call        */

for (i = 0 ; i < ac ; i++)        /* List line arguments           */

printf("%s ", av[i]);

putchar('\n');

return(1);                        /* C_FILTER must be called again */

}

else {

printf("Following line  :\n");    /* These are other lines         */

for (i = 0 ; i < ac ; i++)        /* List line arguments           */

printf("%s ", av[i]);

putchar('\n');

if (level == 4)                   /* Is it the last time ?         */

return(0);                      /* Yes, go on BBS                */

else

return(1);                      /* No, call once more C_FILTER   */

}

}

FBB 5.15                          Feb 06 1993                       Page - 160

APPENDIX -13- MESSAGE FILTERING.

--------------------------------

FBB software allows filtering messages. Filtering is not done by the BBS

software but by external programs developped by users.

may be interactive and allows to incorporate some features like dedicated

information for predifined callsigns, password filtering, etc...

I did not develop such programs, but this is an open door to many

applications.

The M_FILTER program must be found by the PATH of MsDos. Its extension can

be COM or EXE, and it must be little and fast as multitasking is stopped

during the activity of this program. If this program is not found, it will

not be called until the BBS is rebooted.

The message filter is called (if found) each time a message is ready to be

recorded (when Ctrl Z or /EX is received). The decision to validate or not

the message is function of the exit value of the M_FILTER program.

The M_FILTER program (if found) is called with some arguments including a

level number. This number is incremented each time the program is called in

the same connection session. The first time the level number will be 0.

The line arguments given to the M_FILTER program are :

- File name including the text of the message.

- Type of the message (P, B, T).

- Sender.

- "To" field.

- Record number of DIRMES.SYS file.

The M_FILTER program ends with an exit value. This value is very important

and tells the BBS what to do :

0 : Message is recorded.

1 : Message is killed   (status = K).

2 : Message is archived (status = A).

3 : Message is held     (status = H).

/*

* M_FILTER.C

*

* The message filter MUST be named M_FILTER (COM or EXE).

*

* This example only writes its call arguments in the TEST.MES file.

*

* It is called with 5 arguments :

*   File name of the message.

*   Type .

*   Sender.

*   To.

*   Number of the record in the DIRMES.SYS file.

*

* If it returns 0 : The message is accepted.

*               1 : The message is killed (status K).

*               2 : The message is archived (status A).

*

*/

#include <stdio.h>

FBB 5.15                          Feb 06 1993                       Page - 161

main(argc, argv)

int  argc;

char **argv;

{

int  i;

FILE * fptr = fopen("TEST.MES", "at");

for (i = 0 ; i < argc ; fprintf(fptr, "%s ", argv[i++]));

fputc('\n', fptr);

fclose(fptr);

return(0);

}

FBB 5.15                          Feb 06 1993                       Page - 162

APPENDIX -14- PG programs development.

--------------------------------------

PG programs are in the PG subdirectory. They are little programs allowing

interactivity with the user.

COM or EXE programs can be called.

PG programs must be little as the amount of memory is limited and fast

because the multitasking is stopped during its activity.

To run a PG program and start a session, the user must type the command PG

followed by the name of the program. The PG command alone gives the content

of the PG subdirectory. The PG program is particularly developped for FBB

software but can be an interface to a standard program.

Each time a complete line (up to the return character) is received, the PG

program is called with some arguments including a level number. This number

is incremented each time the program is called in the same PG session. The

first time the level number will be 0.

The line arguments given to the PG program are :

- Callsign (format as F6FBB-8).

- Level number (0 is the first time, up to 99).

- Flags of the user (binary number as user's mask of INIT.SRV).

- Record number of the user in INF.SYS.

- Received data (each word is a new argument).

The PG program ends with an exit value. This value is very important and

tells the BBS what to do :

0 : end of session and return to the BBS menu.

1 : the program will be called again and the level number is incremented.

2 : the user will be disconnected.

3 : the receive datas will be sent as a BBS command and return to BBS.

4 : the receive datas will be sent as a BBS command, level incremented.

5 : the program will be called again, but the level is not incremented.

The datas sent by the PG program to the standard output will be sent to the

user. This allows a real interactivity between the user and the PG program.

FBB 5.15                          Feb 06 1993                       Page - 163

Here is an example of a little program :

/*

* TST_PG.C

*

* Little test program of "PG" command for FBB BBS software.

*

* (C) F6FBB 1991.

*

* FBB software 5.14 and upper.

*

*

* This program echoes to the user what he types

* or executes a BBS command preceeded by "CMD"

* until "BYE" is received

*/

#include <stdio.h>

main(int argc, char **argv)

{

int  i;

int  level = atoi(argv[2]);        /* Get level from argument list   */

/* and transform it to integer    */

if (level == 0) {                  /* Is level equal to 0 ?          */

/* This is the first call         */

printf("Hello %s, type BYE when you want to stop !\n", argv[1]);

return(1);                       /* program will be called again   */

}

else {

strupr(argv[5]);                 /* Capitalise the first word      */

if (strcmp(argv[5], "BYE") == 0) {       /* is BYE received ?      */

printf("Ok, bye-bye\n");

return(0);                     /* Yes, go on BBS                 */

}

else if (strcmp(argv[5], "CMD") == 0) {  /* is CMD received ?      */

for (i = 6 ; i < argc ; i++)   /* List line arguments            */

printf("%s ", argv[i]);        /* sent by user                   */

putchar('\n');

for (i = 6 ; i < argc ; i++)   /* List line arguments            */

printf("%s ", argv[i]);      /* sent by user                   */

putchar('\n');

return(4);                     /* Yes, send command              */

}

else {

printf("You told me : ");      /* These are other lines          */

for (i = 5 ; i < argc ; i++)   /* List line arguments            */

printf("%s ", argv[i]);        /* sent by user                   */

putchar('\n');

return(1);                     /* No, call again program         */

}

}

}

FBB 5.15                          Feb 06 1993                       Page - 164

APPENDIX -15- UNPROTO LISTS.

----------------------------

FBB software allows sending unproto lists of message. This is validated

separately on each port (letter L in PORT.SYS). Unproto address is FBB with

the following header :

fm F6FBB-1 to FBB ctl UI

An unproto list line is sent on every validated port each time a message is

recorded. The line is in the form :

12345 B 2053 TEST@ALL F6FBB 920325 This is the subject

If a message number is missing or does not exist the line will be :

12346 #

This allows a system listening to the UI packets on a frequency to create a

list identical to the one of the BBS, and then the user will not have to

connect the BBS to know the list of messages and bulletins.

A control can be done on the number of the messages to check if a line is

missing.

If the remote system receives a new line, and a line is missing, it only has

to send an unproto frame addressed to the BBS callsign like this :

fm FC1EBN-3 to F6FBB-1 ctl UI

? 00002EE00E

This will be taken in account only if the user has the U flag validated (EU

command).

If the user has not his flag validated in the BBS, he will receive a line

like :

fm F6FBB-1 to FBB ctl UI

12200 / callsign

In this case, the the remote software MUST stop asking unprotoes.

The first 8 digits are the hexadecimal number of the requested start of the

list (here 00002EE0 -> 12000) and the last two digits are the sum of the four

bytes anded with FF (0E).

The BBS will then starts sending lines from the requested number up to the

last message number.

If the number requested seems to be too far from the current line, the BBS

can reajust the request of "callsign" while sending :

fm F6FBB-1 to FBB ctl UI

12200 ! CALLSIGN

12201 B 2040    TEST@FRA    F6FBB  920325 This is a bulletin

12202 #

12203 P 206    F6ABJ@F6ABJ  F6FBB  920325 Hello Remy.

etc...

and then starts sending lines from 12201. The remote system must change its

base number to 12201.

FBB 5.15                          Feb 06 1993                       Page - 165

If the number requested is greater than the last message received in the

BBS, the BBS will send a line like :

12300 !!

This indicates that the list in the remote system is up to date. The last

received message in the BBS is 12300.

The remote system can also connect the BBS and ask for messages in binary

compressed mode using the following sequence :

BBS                                    Remote system

--------------------------------------------------------------------------

Connection request

Connection.

[FBB-5.14-ABFHM$]

Welcome in Toulouse, Gerard.

F6FBB BBS>

[TPK-1.80-$]

1>

F< #Msg

Binary compressed message #Msg

is sent using format described

in appendix 7

1>

Disconnect.

From TPK version 1.80 (packet communication program developped by FC1EBN),

this protocole is implemented.

FBB 5.15                          Feb 06 1993                       Page - 166

APPENDIX -16- EXTENSIONS TO THE YAPP PROTOCOLE.

-----------------------------------------------

These extensions are used in TPK 1.65a (and up) and FBB 5.14 and up.

HEADER extension.

The header now carries the DATE and TIME of the file being transmitted.

[SOH] [Len] [Filename] [NUL] [File Size] [NUL] [Date] [Time] [NUL]

The Date and Time are the values reported by DOS, coded in 4 hexadecimal

digits and are sent in ASCII (8 characters).

The receiver has the choice of using either extended Yapp with checksum or

normal Yapp.

The normal Yapp reply is RF, as before and the receiver can keep the date

and time information .

The extended Yapp reply is : RT  Receive_TPK  and is coded : [ACK] [ACK]

If the receiver reply is RT the protocol used will be what I have called

YappC for Yapp with checksum. When the sender gets this packet he MUST use

YappC.

Data Packets extension.

If the receivers reply is RT the protocol used will be YappC. The checksum

allows detection of packets corrupted along the link, particularly on the

RS232 lines where there is no error control or correction (or it's very

poor!)

Data packets : [STX] [Len] [Datas] [Checksum]

Checksum is the sum of all datas bytes anded with FF in 8 bits like Xmodem.

If the checksum is bad then the receiver must send a Cancel and enters CW

state.

Crash Recovery.

A new field has been added to the resume reply to tell the sender if the

receiver can use YappC or not. Resume is sent instead of RF (or RT).

Resume reply for Yapp: (as used before by TPK and FBB)

[NAK] [Len] [R] [NUL] [Received Length] [NUL]

I    I                I

I    I                +-- in ASCII as in the header

I    +-- as Resume !

I

+------- len of the following bytes

Resume reply for YappC:

[NAK] [Len] [R] [NUL] [Received Length] [NUL] [C] [NUL]

I

Tells sender I can use YappC -----+

When the sender gets this packet then he must also use YappC.

FBB 5.15                          Feb 06 1993                       Page - 167