======================================================================

Microsoft Product Support Services Application Note (Text File)

GA0363: MICROSOFT DIAGNOSTICS UTILITY (MSD)

======================================================================

Revision Date: 04/92

Disk Included

--------------------------------------------------------------------

| INFORMATION PROVIDED IN THIS DOCUMENT AND ANY SOFTWARE THAT MAY    |

| ACCOMPANY THIS DOCUMENT (collectively referred to as an            |

| Application Note) IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY      |

| KIND, EITHER EXPRESSED OR IMPLIED, INCLUDING BUT NOT LIMITED TO    |

| THE IMPLIED WARRANTIES OF MERCHANTABILITY AND/OR FITNESS FOR A     |

| PARTICULAR PURPOSE. The user assumes the entire risk as to the     |

| accuracy and the use of this Application Note. This Application    |

| Note may be copied and distributed subject to the following        |

| conditions: 1) All text must be copied without modification and    |

| all pages must be included; 2) If software is included, all files  |

| on the disk(s) must be copied without modification [the MS-DOS(R)  |

| utility DISKCOPY is appropriate for this purpose]; 3) All          |

| components of this Application Note must be distributed together;  |

| and 4) This Application Note may not be distributed for profit.    |

|                                                                    |

| Copyright 1990-1992 Microsoft Corporation. All Rights Reserved.    |

| Microsoft and MS-DOS are registered trademarks of Microsoft        |

| Corporation.                                                       |

--------------------------------------------------------------------

INTRODUCTION

============

Microsoft Diagnostics (MSD) is an MS-DOS-based utility designed to

provide Microsoft customers and Microsoft Product Support Services

(PSS) personnel with detailed technical information about a computer's

hardware and software components. This information can be helpful when

trying to diagnose and solve problems with Microsoft products.

INSTALLATION

============

Follow these steps to install MSD on your hard disk. (If you do not

want to install MSD on your hard disk, or if your computer does not

have a hard disk, you can run MSD from your floppy disk drive.)

1. Place the enclosed GA0363 disk in drive A.

2. Copy MSD.EXE and MSD.INI to your hard disk. At the command prompt,

type the following and press ENTER:

copy a:\msd.* c:\

NOTE: If you are copying MSD from a drive other than A, substitute

the appropriate drive letter for "a" in the command above. The

above command copies MSD to your root directory; you can specify a

different destination if you prefer. Please refer to your "MS-DOS

User's Guide and Reference" for more information on the COPY

command.

USING MICROSOFT DIAGNOSTICS

===========================

MSD is an MS-DOS-based application that should be run from the MS-DOS

command prompt. The syntax for MSD is:

msd [/b] [/i] [/f <filename> | /p <filename> | /s [<filename>]] [/?]

Where:

Option 		Description

-----		-----------

/b  		Starts MSD in black-and-white mode. Use this option if your

computer has a monochrome or LCD monitor.

/i 		Specifies that no initial hardware detection be performed.

Use this option if MSD fails to start correctly.

/f <filename> 	Writes a complete MSD report to the specified file.

Requests information such as name, company, address,

telephone number, and comments.

/p <filename> 	Writes a complete MSD report to the specified file

without requesting any information.

/s [<filename>] Writes a summary MSD report to the specified file.

If no filename is specified, the report is displayed

on the screen.

/? 		Displays the MSD command options.

To start MSD, do the following:

1. Change to the drive and directory where you installed MSD.EXE.

2. At the command prompt, type the following and press ENTER:

msd

3. As MSD loads, an introduction screen is displayed with a message

indicating that MSD is examining your system.

Once MSD has loaded, you will see a menu bar at the top of your screen

along with 13 buttons for viewing information on various components of

your system. To choose a button, press the key that corresponds to the

highlighted letter in the button's description. For example, press V

to display information about your video configuration. To choose a

menu from the menu bar, hold down the ALT key and then press the first

letter of the desired menu. For example, to choose the File menu, hold

down the ALT key and then press F.

If you have a mouse installed on your computer, you can point to and

click the desired button or menu.

When you choose a button, a window containing the requested

information will appear. If there is more information than can be

displayed in one screen, use the up and down arrow keys or the PAGE UP

and PAGE DOWN keys to scroll through the additional information. If

you have a mouse, you can use the scroll bars on the right side of the

window to scroll through the information. After you have finished

viewing the information, press ENTER or choose the OK button to close

the window and return to MSD's main screen. For windows that have

multiple buttons (such as an OK button and a Cancel button), use the

TAB key to move from button to button or use your mouse.

When you are finished using MSD, choose Exit from the File menu or

press the F3 function key to exit.

MSD Button Options

------------------

The following is a brief description of the buttons that appear on

MSD's main screen:

Button 			Description

------ 			-----------

Computer		Displays computer manufacturer, processor type,

and bus type; ROM BIOS manufacturer, version,

and date; keyboard type; DMA controller

configuration; and math coprocessor status

Memory 			Displays a map of the upper memory area (UMA)

--the memory region from 640K to 1024K

Video			Shows your video card manufacturer, model and type;

video BIOS version and date; and current video mode

Network 		Displays network-specific configuration information

Operating System	Displays the operating system version, location

of MS-DOS in memory (MS-DOS 5.0 only), the drive

the system was booted from (MS-DOS 4.0 or later),

the current environment settings, and the path

from which MSD was run

Mouse 			Shows the MS-DOS mouse driver version, mouse type,

mouse interrupt request line (IRQ) number, and

other information specific to the configuration

of the mouse

Other Ports 		Dynamically displays the game card status

for up to two game devices or joysticks

Disk Drives 		Displays the size of and number of bytes free on

local and remote drives

LPT Ports 		Displays the port addresses of all installed

parallel ports and dynamically displays the status

of each port

COM Ports 		Displays the port addresses and current

communications parameters of all installed

serial ports and dynamically displays the status

of each port

IRQ Status 		Displays the configuration of the hardware IRQs

TSR Programs 		Displays the name, location in memory, and size

of each	program loaded in memory at the time MSD

was run

Device Drivers 		Displays the names of all device drivers installed

at the time MSD was run

MSD Menu Options

----------------

The following is a brief description of the menu options that appear

on MSD's main screen:

File Option 		Description

------------		-------------

Find File		Searches for a specific file and allows you to view

the contents of the file

Print Report 		Prints a "faxable" report to a normal TTY printer,

PostScript printer, or file

Exit 			Exit the Microsoft Diagnostics utility

AUTOEXEC.BAT 		Lists the contents of your AUTOEXEC.BAT file

CONFIG.SYS 		Lists the contents of your CONFIG.SYS file

SYSTEM.INI 		Lists the contents of your Microsoft Windows

SYSTEM.INI file

WIN.INI 		Lists the contents of your Microsoft Windows WIN.INI file

Utility Option 		Description

---------------- 	------------

Memory Block Display	Shows memory allocation--the starting address

of each block and its size in bytes

Memory Browser		Searches selected ROM and RAM areas for strings

such as "copyright" and "version"

Insert Command		Allows for quick insertion of a command in the

appropriate file; the list of commands you can

choose from is stored in the [commands] section

of the MSD.INI file

Test Printer 		Prints a page to test the connection between the

computer and printer

Help Option 		Description

-----------		-----------

About			Displays Microsoft Diagnostics version and

copyright information