Program MAK2LINE

Make 2-Line Elements using Keplerian Elements

(C) Copyright 1992, All Rights Reserved

by David H. Ransom, Jr.

Rancho Palos Verdes, CA

Version  9208

February 16, 1992

BBS: (310) 541-7299

MAK2LINE Program Documentation                                    Page 1

INTRODUCTION

------------

This program provides a means to individuals without modems to receive

the so-called "Keplerian Orbital Elements" and reliably prepare "2-Line

Orbital Elements" as required by satellite tracking programs such as my own

STSORBIT and STSORBIT PLUS as well as Paul Traufler's excellent TRAKSAT and

other similar programs. However, more information is included in the 2-line

orbital element set than is usually included in the Keplerian orbital

elements; this means that the "missing" information must either be obtained

from other sources or be set to a specified value or zero. These

instructions and the example and sample form which follows will help the

unskilled user to do these tasks.

The current version of MAK2LINE is always available on my RPV

ASTRONOMY BBS at (310) 541-7299 in Rancho Palos Verdes, CA near Los

Angeles. It is a very busy system, so please be patient (especially during

Space Shuttle missions).

Program MAK2LINE is copyrighted software. Use it if you like it,

discard it if you don't. There are no warranties of any kind. Program

registration is not required; however, if you use and enjoy Program

MAK2LINE, a donation of US$10.00 will be much appreciated and will encourage

me to continue supporting the amateur radio and satellite tracking

communities with software big and small. A donation of US$10.00 will also

bring you the latest version of the program on disk. Please specify disk

size, 5-1/4" 360K or 3-1/2" 720K.

David H. Ransom, Jr.

7130 Avenida Altisima

Rancho Palos Verdes, CA  90274

MAK2LINE Program Documentation                                    Page 2

PROGRAM NOTES

-------------

Program MAK2LINE has but a single purpose: to prepare 2-line orbital

elements from Keplerian orbital elements by means of user keyboard input.

Since the program is designed only to transcribe valid data from one format

to another, no error checking is performed and the program makes no tests

of the "reasonableness" of the various data and parameters. The user is

therefore cautioned to check his data before using this program, or to use

the resulting data with caution until it has been verified.

The information shown below is usually included in the Keplerian

orbital elements as received via modem or via voice (amateur radio or

telephone). This sample is an actual file from Space Shuttle Flight STS-42

in January of 1992 and received via modem direct from the NASA Johnson

Space Center:

Satellite: STS-42

Catalog number: 21846

Epoch time:      92024.87908918        (24 JAN 92   21:05:53.31 UTC)

Element set:     JSC-005

Inclination:       56.9720 deg

RA of node:       233.8707 deg          Space Shuttle Flight STS-42

Eccentricity:     .0009197                SGP4 Keplerian Elements

Arg of perigee:   264.4174 deg         from NASA flight Day 3 vector

Mean anomaly:      95.5875 deg

Mean motion:   15.91353638 rev/day               G. L. Carman

Decay rate:    2.10000e-03 rev/day^2      NASA Johnson Space Center

Epoch rev:              37

If you compare the information requested by Program MAK2LINE (as shown

on the example on the following page) with that above, several items are

missing or may be in a slightly different format. Here are some

suggestions.

1.   A sample form is included in this documentation which will assist you

when receiving Keplerian orbital information by voice line or when it

is transmitted by amateur radio link. Note that the form includes

lines for "IAU Designation" and "BSTAR Drag"; these items are

frequently omitted from voice transmissions.

2.   The NORAD Number is assigned by the U. S. Space Command at Cheyenne

Mountain after a satellite has been successfully launched. Initial or

pre-launch Keplerian elements may use the mission number or some other

convenient number for this item. Most tracking programs will accept

any number here.

3.   The IAU (International Astronomical Union) Designation is often not

assigned until well after a flight is in progress; press ENTER to use

all spaces for this item.

4.   For the 2-line orbital element set, the element set number (shown as

"JSC-005" in the example above) MUST be numeric. For this example,

enter "5" or "005". Most tracking programs will accept any number for

this item, up to a maximum of FOUR DIGITS.

MAK2LINE Program Documentation                                    Page 3

5.   The "Epoch Rev/Orbit" is the orbit number at the epoch (the instant

when the data is taken). Up to FOUR DIGITS may be entered here but

except for the orbit number on displays, this number has no effect on

the propagated orbit.

6.   When MAK2LINE asks for "NDOT2 Drag/Decay", the information being

requested is the "Decay rate" in the Keplerian elements. It may be

entered in the form shown in the example or as a decimal fraction such

as ".0021" (which is identical to the "2.10000e-03" in scientific

notation as used in the example).

7.   When MAK2LINE asks for "NDDOT6 Drag", press ENTER to insert a value of

zero. This should be satisfactory for most tracking programs.

8.   When MAK2LINE asks for "BSTAR", you may press ENTER to insert a value

of zero. This will be satisfactory for times very close to the Epoch

Time for the elements. However, as time progresses the satellite may

lag behind the propagated (projected) ground track if no value is

avaialble for BSTAR. It may be satisfactory to use a value from a

similar satellite or a prior set of 2-line elements in order to

achieve more accurate long term results.

9.   After the last data item has been entered, MAK2LINE will request the

filename to which the 2-line orbital data is to be written. Enter the

full file name such as "TESTSAT.TXT". The file will be written to the

current drive and directory by default. A drive and/or path may

optionally be included in the filename entered. Any prior file by the

same name will be overwritten.

10.  If you make a mistake during the data entry process, you may use the

BACKSPACE key until ENTER has been pressed. If you wish to cancel the

program without writing the data to a file, press CTRL-C.

Finally, a word about the NDOT2 and BSTAR parameters. Both the U. S.

Space Command (formerly NORAD) and Johnson Space Center have been known to

"tweak" these values for a variety of reasons. Usually it is because the

orbit is in a state of change due, perhaps, to maneuvers or excessive drag.

A positive value for BSTAR, which indicates positive acceleration and is

somewhat unusual (and often also an indication of "tweaking"), is NOT

handled correctly by MAK2LINE at this point and must be manually edited. If

this proves troublesome, it will be corrected in a future release of the

program.

MAK2LINE Program Documentation                                    Page 4

PROGRAM MAK2LINE EXAMPLE

------------------------

Enter 2-LINE ELEMENTS

Enter the required data as prompted. Most essential data is

so-called Keplerian Orbital Data available to amateur radio

enthusiasts. Other data may be inserted if available or may

be omitted if not. Accuracy may be affected, see documentation.

Enter Satellite Name (15 chars max):      sts-42

Enter NORAD Number (NNNNN):               21846

Enter IAU Designation (8 chars max):      92  2  A

Enter Epoch Time (YYDDD.TTTTTTTT):        92024.87908918

Enter Element Set Number (NNN):           5

Enter Inclination (DDD.DDDD):             56.972

Enter RA of Ascend Node (DDD.DDDD):       233.8707

Enter Eccentricity (.NNNNNNN):            .0009197

Enter Arg of Perigee (DDD.DDDD):          264.4174

Enter Mean Anomaly (DDD.DDDD):            95.5875

Enter Mean Motion (NN.NNNNNNNN):          15.91353638

Enter Epoch Rev/Orbit (NNN):              37

Enter NDOT2 Drag/Decay (.NNNNNNNN):       .0021

Enter NDDOT6 Drag (NNNNN-N):

Enter BSTAR (NNNNN-N):                    61700-3

Satellite Name (15 chars max):            Sts-42

NORAD Number (NNNNN):                     21846

IAU Designation (8 chars max):            92  2  A

Epoch Time (YYDDD.TTTTTTTT):              92024.87908918

Element Set Number (NNN):                  5

Inclination (DDD.DDDD):                    56.9720

RA of Ascend Node (DDD.DDDD):             233.8707

Eccentricity (.NNNNNNN):                  .0009197

Arg of Perigee (DDD.DDDD):                264.4174

Mean Anomaly (DDD.DDDD):                   95.5875

Mean Motion (NN.NNNNNNNN):                 15.91353638

Epoch Rev/Orbit (NNN):                     37

NDOT2 Drag/Decay (.NNNNNNNN):             .00210000

NDDOT6 Drag (NNNNN-N):                    00000-0

BSTAR (NNNNN-N):                          61700-3

Sts-42

1 21846U 92  2  A 92024.87908918  .00210000  00000-0  61700-3 0    59

2 21846  56.9720 233.8707 0009197 264.4174  95.5875 15.91353638   379

MAK2LINE Program Documentation                                    Page 5

RECEIVED KEPLERIAN ORBITAL DATA

-------------------------------

This form is provided as a convenience to users receiving Keplerian

orbital data via voice link (amateur radio or telephone). Each set of

underline characters indicates an expected character; the decimal point is

shown where expected (if appropriate for the item). Make multiple copies of

this form prior to a mission.

Satellite:          __ __ __ __ __ __ __ __ __ __ __ __ __ __ __

Catalog Number:     __ __ __ __ __

IAU Designation:    __ __ __ __ __ __ __ __

Epoch Time:         __ __ __ __ __ . __ __ __ __ __ __ __ __

Element Set:        __ __ __ __

Inclination:        __ __ __ . __ __ __ __

RA of Node:         __ __ __ . __ __ __ __

Eccentricity:       . __ __ __ __ __ __ __

Arg of Perigee:     __ __ __ . __ __ __ __

Mean Anomaly:       __ __ __ . __ __ __ __

Mean Motion:        __ __ . __ __ __ __ __ __ __ __

Decay Rate:         . __ __ __ __ __ __ __ __

BSTAR Drag:         __ __ __ __ __ - __

Epoch Rev:          __ __ __ __

MAK2LINE Program Documentation                                    Page 6

MAK2LINE REVISION HISTORY

-------------------------

Each released version of MAK2LINE uses a four digit revision code such

as 9208. The first two digits indicate the year and the second two digits

indicate the week of the year. In some cases, an additional letter suffix

is added to distinguish changes occuring within the same week. A partial

week at the beginning or end of the year is counted as a full week. Using

this method, a year will typically have 53 weeks although it is possible to

have 54 weeks in a leap year (1972 is an example). The current year-week

revision code is shown on the Julian Date display, Display Mode 7, using my

program ASTROCLK.

This section records the revision history of program MAK2LINE through

all of the minor twists and turns that usually accompany the evolution of

such a program. It illustrates the tortuous process of maintaining and

refining a program as ideas and problems are reported from every quarter.

It may also be helpful to users who are upgrading from one version to

another to find out what has changed. This section may also describe recent

changes not included in the main documentation.

Version 9208  02/16/92

----------------------

-Initial public release of the program.

-User comments and suggestions are welcome.

############################################