Terminal Program PAC4

PAC4 is a simple terminal program, which uses the BPQ Host Mode to talk

to the switch. The program provides a buffered input line, which allows use

of the editing keys ins, del, cul, cur, etc. It allows logging to disk

(CTRL/L toggles this function on and off), and maintains a simple 'Response

Time Monitor' function - useful for seeing how badly the network is

performing! A scroll-back buffer is also provided - use PageUp and PageDown

The program has one required paramter (port number), and a second optional

one (M for mono mode). So possible command lines are:

PAC4 63			; to use stream 63

PAC4 1 M		; to use stream 1, if you have a mono monitor.

Connection is made to the switch when a message is input. The ESC key

disconnects you from the switch.

Monitoring is toggled on/off using F1. F2 toggles MCOM mode (monitoring of

control frames as well as info frames). F3 allows you to set MMASK. Press

F3, then key up to 4 hex digits, terminated with a newline. Each bit of MMASK

enables the monitoring of the corresponding radio port. Bit 0 enables port

1, bit 15 port 16. Any combination of bits may be set. The current monitor

settings are dislayed on the status line.

To exit from PAC4, enter CTRL/Y

John Wiseman, G8BPQ

11/4/92

################################################################################################