G8BPQ Host Mode

From Version 4.00, a new application interface to the switch will be provided.

This will replace the existing interfaces (TNC2, PK Host and KISS). New

software should if possible use the new interface - interface routines

supporting the old modes, and WA8DED Host mode, will be provided as TSRs

to support existing application software.

The interface is normally called using INT 7Fh. From version 4.03 the interrupt

number may be changed - The value is specified in BPQCFG.TXT as INTERRUPT=nnn,

where nnn is in decimal. The function required is in AH, and the stream in AL.

The system supports 64 streams, or sessions, numbered from 1 - 64.

Changes in Version 4.05.

1.	Additional option on funtion 6 - allows connects using BBS Call, even

if APPLMASK is not set (say to prevent connects).

2.	Additional info returned by function 8.

3.	Format of frame returned by function 11 added to documentation.

4.	Function 15 added. Simplifies access to timing info.

5. 	Broadcast option added. If function 2 is called with AL=0, the

data will be sent as a UI frame to all ports which have an UNPROTO

address set.

6. 	Function 12 added to update Beacon text.

COMMANDS SUPPORTED ARE:

Note that in all cases where a string is returned, ES:DI must point to

a buffer supplied by the user, and the switch will copy the data to

this buffer. The code does NOT return pointers to its own data.

AH = 1	Set application mask to value in DL.

Set application flags to value in CL.

If top bit of CL is set, monitored frames will be available.

Use function 11 to receive them.

AH = 2	Send frame in ES:SI (length CX)

AH = 3	Receive frame into buffer at ES:DI, length of frame returned

in CX.  BX returns the number of outstanding frames still to

be received (ie. after this one) or zero if no more frames

(ie. this is last one).

Note that CX is NOT an input param - the supplied buffer

must be big enough to receive a full AX.25 frame, including

up to 8 digi callsigns. 340 should be enough.

AH = 4	Get stream status.  Returns:

CX = 0 if stream disconnected or CX = 1 if stream connected

DX = 0 if no change of state, or DX = 1 if

the connected/disconnected state has changed.

AH = 5	ACK stream status change. Must be called to allow another

status change to be reported.

AH = 6	Session control.

CX = 0 Connect to Node. Session uses BBS callsign if bottom

bit of DL=1, otherwise uses Node Call .

CX = 1 Connect to Node. Session uses BBS Call if APPLMASK=1,

otherwise Node Call.

CX = 2 Disconnect

CX = 3 Return user to node

AH = 7	Get buffer counts for stream.  Returns:

BX = number of frames queued for receive

CX = number of un-acked frames to be sent

DX = number of buffers left in node

AH = 8	Port control/information.  Called with a stream number

in AL returns:

ES:DI = CALLSIGN (10 bytes space padded)

The following are now supported (4.05 onwards)

AL = Radio port on which channel is connected (if level 2)

AH = Session Type (If Connected). Session type bits are:

L2LINK	EQU	1

SESSION	EQU	10B

UPLINK	EQU	100B

DOWNLINK	EQU	1000B

BPQHOST	EQU	100000B

BX = L2 paclen for the session

CX = Maxframe. Taken from PORT maxframe if L2 session,

Switch maxframe if L4.

DX = L4 window size (if L4 circuit, or zero)

AH = 10	RAW (KISS) transmit frame.  Data pointed to by ES:SI, of

length CX, is transmitted as a HDLC frame on the radio

port (not stream) in AL.

AH = 11 Get Trace (RAW Data) Frame into ES:DI, Length to CX. Data

is as received off air (ie with calls in AX.25 format, etc)

See note on CX under function 3.

The returned buffer includes a 5 byte header:

Bytes 0-1  - Internal control info (ignore)

Byte  2    - Port number. Top bit set if transmitted frame.

Bytes 3-4  - Frame length (including this header)

AH = 12 Update Switch Info. This will be used for a number of

operations. At the moment the only one implemented is to

update the Beacon Text field.

DX = Function Number.

DX = 1  Update Beacon Text.

ES:SI Data to send in Beacons

CX    Length of Data

AH = 15 Get time marker in AX. Value is number of timer interrupts

since switch was loaded, modulo 65536. Timer interrupts

normally occur at approx 18.2 per second.

The following additions have been proposed. Similar functions may

be provided, but not necessarily in this format.

AH = 0	Get node/switch version number and description.  On return

AH = major version number and AL = minor version number,

and user's buffer pointed to by ES:SI is set to the text

string normally output by the USERS command. CX is set to the

length of the text string.

AH = 9	Fetch node/application callsign & alias or application names

AL = application

BL = 0 to get Callsign, 1 to get Application Name

number:

0 = node

1 = BBS

2 = HOST

3 = SYSOP etc. etc.

Returns string with alias & callsign or application name in

user's buffer pointed to by ES:SI length CX.  For example:

"WORCS:G8TIC"  or "TICPMS:G8TIC-10".

PACLEN.

Frames are transmitted as received from the application, so the application

is responsible for the transmitted PACLEN. Function 8 will return the

recommended value for the current session. I think the value is always OK,

but I suggest you treat a value of zero as 'undefined', and use your own

default. The value may change during a session, if for instance, you connect

to the switch, and then out on a port with a different MAXFRAME. Also at

some time in the future I hope to adjust PACLEN to suit conditions on the

channel. So I recommend that the value is re-read from time to time during

a session.

The following sample of code shows how to find the Interrupt Number to use

to call the switch, and check that the switch is loaded.

NOTBPQERROR	DB	'Switch not found$'

VERSERROR	DB	'Version not compatible with Node Software$'

G8BPQ		DB	'G8BPQ'

BADCONFIGMSG	DB	'Configuration file read error',0DH,0AH,'$'

CONFIGFILENAME	DB	'BPQCFG.BIN',0

NOCONFIGMSG	DB	'Configuration file BPQCFG.BIN not found',0DH,0AH,'$'

;

;	BPQCFG FIRST 128 BYTES

;

CONFIGTABLE	LABEL	BYTE

;

;	CONFIGURATION DATA STRUCTURE

;

;	DEFINES LAYOUT OF CONFIG RECORD PRODUCED BY CONFIGURATION PROG

;

;	LAYOUT MUST MATCH THAT IN CONFIG.C SOURCE

;

C_NODECALL	DB	10 DUP (0)	 ; OFFSET = 0

C_NODEALIAS	DB	10 DUP (0)	 ; OFFSET = 10

C_BBSCALL	DB	10 DUP (0)	 ; OFFSET = 20

C_BBSALIAS	DB	10 DUP (0)	 ; OFFSET = 30

;

C_OBSINIT	DW	0		 ; OFFSET = 40

C_OBSMIN	DW	0		 ; OFFSET = 42

C_NODESINTERVAL	DW	0		 ; OFFSET = 44

C_L3TIMETOLIVE	DW	0		 ; OFFSET = 46

C_L4RETRIES	DW	0		 ; OFFSET = 48

C_L4TIMEOUT	DW	0		 ; OFFSET = 50

C_BUFFERS	DW	0		 ; OFFSET = 52

C_PACLEN	DW	0		 ; OFFSET = 54

C_TRANSDELAY	DW	0		 ; OFFSET = 56

C_T3		DW	0		 ; OFFSET = 58

DW	0		 ; OFFSET = 60

DW	0		 ; OFFSET = 62

C_IDLETIME	DW	0		 ; OFFSET = 64

C_EMSFLAG	DB	0		 ; OFFSET = 66

DB	0

C_BBS		DB	0		 ; OFFSET = 68

C_NODE		DB	0		 ; OFFSET = 69

C_HOSTINTERRUPT	DB	0		 ; OFFSET = 70

C_DESQVIEW	DB	0		 ; OFFSET = 71

C_MAXLINKS	DW	0		 ; OFFSET = 72

C_MAXDESTS	DW	0

C_MAXNEIGHBOURS	DW	0

C_MAXCIRCUITS	DW	0		; 78

C_TNCPORTLISTO	DB	16 DUP (0)	 ; OFFSET = 80

C_IDINTERVAL	DW	0		; 96

C_XXXXXXXX	DW	0		; 98    ; SPARE (WAS DIGIFLAG)

C_MINQUAL	DW	0		; 100

C_HIDENODES	DB	0		; 102

C_L4DELAY	DW	0		; 103

C_L4WINDOW	DW	0		; 105

C_BTINTERVAL	DW	0		; 106

X_UNPROTO	DB	9 DUP (0)	; 108	; NOW SPARE

C_BBSQUAL	DW	0		; 117

DB	(CONFIGTABLE+128-$) DUP (0)

CONFIGHANDLE	DW	0

NODE:

INT	7FH

RET

INIT:

;

;	GET NODE INTERRUPT NUMBER FROM CONFIG FILE

;

MOV	DX,OFFSET CONFIGFILENAME

MOV	AH,3DH

MOV	AL,0			; READ ONLY

INT	21H			; OPEN IT

JC	NOCONFIGFILE

MOV	CONFIGHANDLE,AX

MOV	BX,AX

MOV	DX,OFFSET CONFIGTABLE

MOV	CX,128

MOV	AH,3FH

INT	21H			; READ

CMP	AX,CX

JNE	SHORTCONFIG

JMP SHORT PROCESSCONFIG

NOCONFIGFILE:

MOV	DX,OFFSET NOCONFIGMSG

JMP SHORT CONFIGERR

SHORTCONFIG:

MOV	DX,OFFSET BADCONFIGMSG

CONFIGERR:

MOV	AH,9

INT	21H

MOV	AX,4C01H

INT	21H			; EXIT

PROCESSCONFIG:

MOV	BX,CONFIGHANDLE

MOV	AH,3EH

INT	21H			; CLOSE IT

MOV	AL,C_HOSTINTERRUPT	; INTERRUPT

MOV	BYTE PTR NODE+1,AL	; PATCH NODE CALL INSTRUCTION

;

;	GET NODE VERSION

;

PUSH	DS

MOV	AH,0

ADD	AX,AX			; VECTORS ARE 4 BYTES LONG

ADD	AX,AX

MOV	SI,AX

XOR	AX,AX

MOV	DS,AX

LDS	SI,DS:[SI]		; GET POINTER TO HOST SUPPORT CODE

SUB	SI,7

MOV	DI,OFFSET G8BPQ

MOV	CX,5

REP CMPSB			; MAKE SURE SWITCH IS LOADED

JE	SWITCHOK

POP	DS

MOV	DX,OFFSET NOTBPQERROR

MOV	AH,9

INT	21H

JMP SHORT EXIT

SWITCHOK:

CMP	BYTE PTR DS:[SI],MAJORVERSION

JNE	BADVERSION

CMP	BYTE PTR DS:1[SI],MINORVERSION

JE	VERSIONOK

BADVERSION:

POP	DS

MOV	DX,OFFSET VERSERROR

MOV	AH,9

INT	21H

EXIT:

MOV	AX,4C00H

INT	21H

BADPORTVAL:

MOV	AH,9

MOV	DX,OFFSET ERRORMSG

INT	21H

RET

VERSIONOK:

POP	DS

;

..........

The release disk also contains a demonstration terminal program, TERM4,

written by G8IMB (in PASCAL) to demonstate the use of the new interface from

a high level language. Note, however, that it was written for version 4.02,

and therefore only works with the Host Mode interrupt configred at 7Fh

John Wiseman, G8BPQ @ GB7BAD

20/12/91

###########