Brief nodes on SWITCH.EXE

This is a foreground version of BPQCODE. It is intended to run only as

a packet switch, and not to support an application system (eg BBS). A

simple terminal program (equivalent to PAC4) is built in, as is the

SYSOP handler from the Data Engine version of the switch.

BPQCFG.TXT

This can be the same as for the normal version. The code will assume

NODE=1 and BBS=0, whatever is entered.

Terminal support.

See PAC4.DOC for details. Ctrl/Y shuts down the whole switch.

SYSOP Commands.

The SYSOP feature is similar to SYSOPH, but the password protection works

rather differently. To enter SYSOP mode, enter PASSWORD. If you are using

the console, the switch will respond Ok, and enter SYSOP mode. If you are

accessing the system remotely, it will repond with 5 random numbers. You

must reenter PASSWORD, this time followed by the corresponding characters

from the password string contained in the file PASSWORD.BPQ. If these

are correct the switch responds OK, othewise it ignores the message.

So, if the password string is ABCDEFGHIJKLM the following is a typical

sequence:

PASSWORD

BPQ:G8BPQ} 04 02 05 06 03

PASSWORD DBEFC

BPQ:GBBPQ} Ok

Note that the password is converted to upper case before testing, so

PASSWORD.BPQ should contain upper case letters or digits. It should

not contain spaces or other special characters.

There are a number of extra SYSOP commands. These were added primarily

for the Data Engine version, where experimenting with parameters is

rather mode difficult, but I've left them in this version as well.

Command			Corresponding BPQCFG parameter

OBSINIT			OBSINIT

OBSMIN			OBSMIN

NODESINT		NODESINTERVAL

L3TTL			L3TIMETOLIVE

L4RETRIES		L4RETRIES

L4TIMEOUT		L4TIMEOUT

T3			T3

IDLETIME		IDLETIME

LINKEDFLAG		ENABLE_LINKED

IDINTERVAL		IDINTERVAL

MINQUAL			MINQUAL

FULLCTEXT		FULL_CTEXT

HIDENODES		HIDENODES

L4DELAY			L4DELAY

L4WINDOW		L4WINDOW

BTINTERVAL		BTINTERVAL

There is also one commend which isnt in the DE version - SAVENODES

#########################################################################################