Routes and Nodes tables for the G8BPQ Switch System.

Starting from version 3.20, the BPQ switch software reads an additional

file, BPQNODES, after processing the configuration information in BPQCFG.BIN.

The file contains Route and Node information to be added to relevant tables.

The file may be built by hand, but will normally be created by running

program BPQNODES.COM while the switch is running. This creates file BPQNODES

containing the current ROUTES and NODES lists in the required format.

There are various ways in which the program could be used. If you run a BBS

which allows programs to be run from the forwarding system, then you could

include it in that. Or perhaps just run it when you notice a significant

change in the network.

The file format is: ( {} indicates optional entries - don't include the{})

ROUTE ADD CALL P QUAL {!} {VIA DIGI1 {DIGI2}}

where P is the port, qual the Quality, and the optional ! indicates a locked

route. Up to 2 DIGIS may be specified.

NODE ADD ALIAS:CALL CALL-1 P QUAL {!} {CALL-2 P QUAL {!} } {CALL-3 ....}

where CALL-1, CALL-2, CALL-3 are up to 3 alternate routs, which must be

defined (by ROUTES config entries or ROUTE ADD) before being used.

Although locked nodes are allowed, I strongly recommend  that they are used

only in exceptional circumstances (eg when using a digi'ed route). Note also

that a locked route to a node will be removed if the system finds 3 higher

quality routes to it.

Lines beginning with ; are treated as comments, and fields should be separated

by one (and only one) space.

#######################################################