CW.COM

07-26-87

-----------------------------------------------------------------

The Original Comprehensive Morse Code Training Program for the PC

-----------------------------------------------------------------

***************************************************************

*PLEASE UPLOAD TO YOUR LOCAL COMPUTER BULLETIN BOARD SYSTEMS! *

***************************************************************

I.    Background

A.  Version

This documentation is updated to version 3.11 of the program.

B.  Purpose

CW was written to help prospective hams overcome what many perceive

to be the biggest obstacle to obtaining an amateur radio license--learning

the Morse code.  There are numerous programs available in the public domain

which send Morse code via a PC's speaker, but most are very limited and are

written in BASIC, which requires a clumsy interpreter.  CW seeks to provide

a comprehensive program which provides several modes of learning and

practicing the code. The best features of other programs are included and

many new features have been added to those previously available.

C.  The Author

The author is a registered patent attorney working with Rockwell

International Corporation in Cedar Rapids, IA (the old Collins Radio Company).

He can be contacted on Compuserve (ID No.: 71016,1355), or GEnie (Address:

L.MURRAH), or by mail at the following address:

10 Cottage Grove Woods, S.E.

Cedar Rapids, IA 52403

The home telephone number is (319) 365-6530.

D.  The Software

CW is written in Turbo Pascal.  The development of CW began in early

1986 and has gone through a number of versions.  New versions will be released

as features are added and corrections are made to previously released versions.

CWnnn.ARC contains the following files:

Name           Description                            Needed to Run?

----           -----------                            --------------

ANTENNAS.CW  - Names of antennas for QSO feature           Yes

CITIES.CW    - Names of cities for QSO feature             Yes

CW.COM       - the main executable file                    Yes

CW.CFG       - the CW configuration file                   Yes

CW.DOC       - this documentation file                     No

CWGUIDE.TXT  - A guide to CW prepared by a user            No

FEEDBACK.FRM - Form for user feedback to author (Please!)  No

GROUPS.CW    - Code groups for Learn feature               Yes

GROUP.1      - Alternate code group for Learn feature      No

GROUP.2      - Alternate code group for Learn feature      No

HAMRADIO.INF - A brief file on ham radio                   No

JOBS.CW      - Name of occupations for QSO feature         Yes

MYGROUP.CW   - User defined group for Learn feature        Yes

NAMES.CW     - Names of operators for QSO feature          Yes

PAIRS.CW     - Character pairings for Learn feature        Yes

PROBLEM.RPT  - Problem report form for special problems    No

README.1ST   - Important information about calibration     No

RIGS.CW      - Names of radio equipment for QSO feature    Yes

WORDS.CW     - Word list for Receive feature               Yes

E.  Latest Version

The latest version of CW can usually be found on GEnie or Compuserve.

The latest version can also be obtained by sending a FORMATTED diskette and a

stamped, self-addressed diskette mailer to the author at the above address.

Although I do not operate an active bulletin board system, I do have

RBBS software which I can load to permit downloading CW from my computer.  If

you do not have access to GEnie and would like to obtain the latest version of

the program this way, please give me a call on weekends or during early evening

hours, and I will set it up for you.

II.   Equipment and Program Configuration

A.  General

CW can be run on the IBM-PC and compatibles having 192K of memory and

one floppy disk drive.  A hard disk is not required, although you will find

that a hard disk will enhance operation of this and any other program.

Your CONFIG.SYS file should authorize at least 20 files.  This can be

done with the following statement:

FILES = 20

CONFIG.SYS may be edited using a word processor or program editor.

B.  Computer Clock Speed

The clock speed for your computer will affect the code speed.

Therefore,  you should calibrate CW code speed for your computer.  If you

have a multiple speed system,  use CW only in the calibrated clock speed

unless you wish to recalibrate.

C.  Video Display Card

The program was originally written for the Color Graphics Adapter.

However,  I have added an option to configure CW for a monochrome video card.

Since I do not have such a card, I am not certain that the option will operate

correctly.  Feedback from users with monochrome cards would be very helpful.

D.  Memory Resident Programs

Since the code speed for CW is determined by a software loop in

to provide greater code speed accuracy,  you will need to recalibrate the

code speed if memory resident programs are used or changed.  Such programs

affect the clock operation upon which the loop is based.

III.  Your Right to Use CW

A.  License

As you may have noticed I have retained the copyright in CW, primarily in

order to control its orderly development.  However, I have granted the right

to anyone to use the program for private, non-commercial use without further

permission from me.  That is, you can copy it, use it, and share it so long as

you do not charge any money for it.

B.  Contributions

Several users have asked me whether I require payment of a "shareware"

fee in order to use the program.  The answer is "No"; but if you would like

to support CW with a small contribution (say $5), we certainly would not

refuse your generosity.  It would certainly help in continuing to improve

the program.

IV.   Tips on Learning the Morse Code

1.  Learn the characters by sound, not by sight.  Each of the characters

has its own characteristic length and rhythm.  Do not memorize the characters

using the visual dot and dash symbols.  That will slow you down!

2.  Set the character speed to 13 words per minute and the send speed

at whatever lower speed you desire.  Since your ultimate goal is 13 words

per minute, it is much easier to learn how the characters sound at that

speed even though they are sent at a lower speed.  This helps to eliminate

the "hump" that most people experience around 10 words per minute.

3.  Study every day, but if you feel you are stuck and not making

progress, take a few days off.  It's amazing how this will sometimes clear

your head.

4.  Try to copy code in you head as much as possible.

5.  Get access to a ham receiver and try to copy off real transmissions

off the air.

6.  As soon as possible learn the sounds of whole words.

7.  Turn on the CW <G>roups function and let it send in the background

while you are doing something else, such as reading the paper or working on a

construction project.  Try to identify characters while doing the other

activity.  This will help your brain shift to "automatic" and will help the

code sounds become a natural "part of the wallpaper."

8.  Although the code test no longer requires that you demonstrate an

ability to send code,  you should get a code practice oscillator and practice

your sending.

9.  As soon as you pass the novice test, get a rig on the air and operate

on the novice bands.  Nothing will increase your speed and concentration as

well as the pressure of an on-the-air contact.  Don't worry about being

perfect--the other guy is probably in the same boat as you.

V.  Using CW

A.  The Character Set

Since several Morse code procedure signs are not printable as ASCII

characters since they are a combination of two characters, special characters

are used to represent them as follows as follows:

Prosign      ASCII Representation    Meaning

-------      --------------------    -------

<AR>                 @              Over

<SK>                 $              End of contact, end of message

<AS>                 !              Wait, stand-by

<KN>                 #              Over-specified station only

B.  Setting Up CW

1.  The Configuration File

The default values for code timing, code speed, oscillator tone,

screen graphics type, and screen colors are contained in the CW.CFG file.  If

this file is not found when the CW is first run, it will be created.  The code

speed and oscillator tone can be changed using the function keys as defined

hereinafter any time code is being sent.  The user is given the opportunity to

save the new settings when the program is exited using the <X> command.  The

default values for code timing, screen graphics type, and screen colors can

be changed from within CW using the <U>tilities command in the main menu.

2.  Code Timing

The timing of the code in CW is initially set assuming a computer

clock speed of 4.77 MHz.  If your computer runs at a different speed or uses

background programs which steal clock cycles, you will need to use the

<T>iming subcommand in the <U>tilities menu to change the timing factor.  To

decrease code speed increase the value of the timing factor and vice versa.

Use the <S>peed Test option and a clock with a sweep second hand to test the

speed resulting from a new setting.  Keep changing the factor until the proper

code speed in attained.  With a clock speed of 4.77 HMz. and with no memory

resident programs installed, the author has found that a timing factor of about

1075 is necessary.

You should ALWAYS check the code speed for your computer before

relying upon CW.

If users who need to change the timing factor will relay the

results to me,  I will include a table for various computers and clock speeds

in future editions of this file.

3.  The Code Speed

The code speed can be set at any time code is being sent using the

function keys as follows:

F1 - Decrease SEND Speed      F2 - Increase SEND Speed

F3 - Decrease CHAR Speed      F4 - Increase CHAR Speed

F5 - Decrease WORD Speed      F6 - Increase WORD Speed

F9 - Set SEND, CHAR, and WORD to Same Speed

You can set the speed at which the character is sent separately

from the overall send speed (the Farnsworth method).  For example, you can

set the send speed at a slow speed but set the characters to be sent using

shorter dits and dahs equivalent to a much faster send speed.  This makes it

easier to increase your code speed since you will already know the sound of

the characters at the higher speed.

You can also set the word speed separately from send and character

speed.  This permits you to learn the sounds of whole words individually sent

at high speeds but spaced at a much slower speed.

The minimum code speed is 3 WPM, and the maximum is 99 WPM (good

luck with that setting!).  The send speed and character speeds are coupled so

that the send speed will never exceed the character speed.  Also in the speed

test mode, the word speed will automatically be set to the send speed so that

the test is valid.

4.  The Oscillator Tone

The oscillator tone can be set to suit your tastes at any time

code is being sent with the function keys as follows:

F7 - Decrease Speaker Tone    F8 - Increase Speaker Tone

The minimum tone is 400 Hz., and the maximum is 1200 Hz.  The author has

found that about 500 Hz. is optimum for sound and volume.

5.  Video Type

At program invocation CW will determine the color video type.

However, this can be changed if necessary (e.g., you have two video cards

installed) with the <V>ideo Type command.  Screen colors will automatically

be set for the selected video card.

6.  Screen Colors

The colors of the screen, the windows, and the highlighting can

be set to suit your taste using the <U>tilities command and the <C>olors

subcommand when the utilities menu window appears.  The colors are

initialized for a black on white when CW is first run.

7.  Hiding the Text

If you would like to hide the text so that you are not tempted to

peek, this can be done with the F10 key at any time code is being sent.  This

can be reversed using the same key. The current status is displayed on the

status line.  With some functions the text will be hidden regardless of the

setting of the hide subcommand.  The hide function also suppresses the display

of 'DIT' and 'DAH' in the <L>earn mode.

8.  Speed Test

You can check the speed at which CW sends by using the

<U>tilities command and the <S>peed Test subcommand.  Simply count the number

of time that the word PARIS is sent in one minute, and that will be the code

speed.  You will probably need to use this command in conjunction with the

<T>iming command to calibrate code speed if your computer runs at other than

4.77 MHz. or if you are running background programs which steal clock cycles.

IT IS ALWAYS A GOOD IDEA TO CHECK THE CODE SPEED PRIOR TO RELYING

UPON CW TO PREPARE YOU FOR A CODE TEST.

8.  Altering the Data Files

Certain of the data files may be customized to suit your desires.

This is described more fully in connection with the <Q>SO and <W>ords function

under the <R>eceive command.

C.  The CW Status Lines

1.  The Message Line (Top)

At all times while CW is running, the top line of the screen is

used to identify the current function and the key necessary to abort the

current function.

2.  The Parameters Line (Bottom)

Likewise, the bottom line at the bottom of the screen will display

from left to right the code send speed in words per minute, the code character

speed in words per minute, the oscillator tone frequency in cycles per second,

and whether or not the text is displayed.

D.  The ESC Key

At any time that code is being sent, the current operation can be

aborted by pressing the ESC key.  A positive indication is given by a beep

sound and a large right pointer is printed on the screen.  In most functions

it is necessary to press another key to continue as indicated on the message

line.

E.  The Function Keys

The function keys are active any time code is being sent.  They are

defined as follows:

F1  - Decrease SEND Speed      F2 - Increase SEND Speed

F3  - Decrease CHAR Speed      F4 - Increase CHAR Speed

F5  - Decrease WORD Speed      F6 - Increase WORD Speed

F7  - Decrease Speaker Tone    F8 - Increase Speaker Tone

F9  - Set SEND, CHAR, and WORD to Same Speed

F10 - Hide/Unhide Text

F.  Starting Out--The <L>earn Command

To help you learn the Morse code, CW provides a <L>earn function, which

presents the characters in sound-related groups.

It is recommended that the <L>earn subcommands be used in the same

as the selections are numbered.

1.  The Basic Code Sounds--Dits and Dahs

The <1> selection teaches you the basic DIT (short) and DAH (long)

sounds.  After sending DITS and DAHS in various formats, you are asked to

identify randomly presented DITS and DAHS using the '1' key for DITS and the

'2' key for DAHS.

2.  Character Length

The <2> selection sends a character and asks you how many total

sound elements (DITS and DAHS) are included in the character.  This forces you

to listen for the LENGTH of a character, which is a very important character

attribute.

3.  Character Pattern

The <3> selection sends a character and asks you how many DAHS are

included in the character.  Since DAHS are the dominant factor in determining

the overall sound pattern or rhythm of a character, this forces you to

concentrate on this character attribute.

4.  The Character Sounds

The <4> selection combines the previously learned DITS and DAHS

into complete character sounds.  The characters are paired with

similar sounds in a series of increasingly harder steps, and the user is asked

to identify the characters in each step randomly chosen.  The user should

start with step 1 and learn to distinguish the characters in each step before

continuing on to the next higher step.  After all 59 steps have been mastered,

the user should be ready to start with the <G>roups and <L>evels commands.

5.  Character Groups

The <5> selection lets you study each character group separately.

The characters from the selected group are presented randomly, and you are

asked to identify them.

The groups used in CW are those used in the ARRL publication "Tune

in the World with Ham Radio".  If you are using materials having a different

grouping, you can change the groups by altering the contents of the file named

GROUPS.CW using a text editor or word processor (you must use the ASCII store

mode so that nothing but printable ASCII characters are stored in the file.

The groups which are initially installed are included in the file GROUPS.1,

and a second grouping used in older ARRL code booklets are included in

GROUPS.2.  To use either of these groups, just delete the current GROUPS.CW

file and copy either GROUPS.1 or GROUPS.2 to the filename GROUPS.CW.  Please

note that only the letters and numbers can be changed. Punctuation and prosigns

are automatically added as the last two group.

If the display of 'DIT' and 'DAH' prove to be distracting in

either this or the <L>evels command which follows, they can be suppressed by

using the  F10 key any time code is being sent.

6.  Character Learning Levels

The <6> selection permits you to study the various groups in a

progressive manner starting with the simple Level 1 sounds and adding

characters with each level.  Each level is simply a combination of groups 1

as defined above through the group whose number is the same as the level

number. After the new characters for each level are presented,  you are asked

to identify randomly presented characters from the level selected.

7.  User Defined Group

If you are having a particularly hard time with several characters,

you can create your own special character group and study it using the

<M>y Group subcommand.  The special group can be created by editing the file

MYGROUP.CW with a word processor in the ASCII output mode or an editor.  The

file can have only ONE line and is limited to 10 characters!

6.  Viewing the Character Steps and Groups

The character steps and groups used in the preceding subcommands

may be displayed using the <V>iew subcommand.

G.  Receiving Code

Once you have mastered the character sounds, you can work on increasing

your speed.  CW provides several ways to do this:

1.  The <G>roups Command

One of the classic ways of practicing code reception is sending

five letter groups of random characters.  The <G>roups command does this.

2.  The <W>ords Command

If you would rather work with real words, use this command.  In

order to promote recognition of whole words, the words are not printed on the

screen until the entire word has been sent in code.

The words sent by this command can be changed by editing the file

WORDS.CW with a word processor.  Make sure each word is on a separate line

followed by a carriage return and that there are no blank lines after the last

entry.  The maximum number of words which may be entered into the file is 500.

Each word may be up to 10 characters in length.

3.  The <Q>SO Command

Use this command if you would like to copy a simulated QSO (a

radio contact).  A standard pattern is used, but the call signs, names, etc.,

are randomly selected from a list.  The call signs are randomly selected from

all the possible call signs.  This means that some calls signs which have not

yet been issued will be used.

The names, antenna types, city names, and rig types can be changed

to reflect your desires by editing the files ANTENNAS.CW, NAMES.CW, CITIES.CW,

RIGS.CW, and JOBS.CW with a word processor.  Simply place your entry on a

single line followed by a carriage return.  Make sure there are no blank lines

in the middle of the file or after the last entry. The maximum number and

length of the entries are as follows:

File             Max. No. Entries     Max. Length Each Entry

----             ----------------     ----------------------

ANTENNAS.CW            50                      15

CITIES.CW             200                      15

JOBS.CW               100                      20

NAMES.CW              200                      10

RIGS.CW               100                      15

If the QSO is sent in the text hide mode, the user is given the

opportunity to review the QSO before exiting.

4.  The <E>nter Command

With this command you can have a friend enter test messages via the

keyboard, and you can attempt to copy it without looking at the screen.  The

maximum length of the message is 255 characters.  This command differs from

the <K>eyboard Send command in that nothing is sent until RETURN is pressed.

5.  The <D>isk Command

This command sends the contents of a disk text file in Morse code.

You can practice by having someone create a text file of unknown content.

Another idea is to exchange disks with a friend who is also studying the code

and carry on a 'disk QSO' between the two of you.

The disk file required for this command can be any ASCII text file

created with a text editor or a word processor.  If a word processor is used,

make sure it is in the ASCII store mode so that only ASCII characters are

stored in the file.  Unprintable characters are ignored, but a time delay for

the missing character is inserted, which slows down the send speed.  To

familiarize yourself with the use of this command, you can enter the name of

the words file (WORDS.CW) or any of the other data files having the CW

extension, although this will not provide very useful code practice.

6.  The <Z>oom Command

This command lets the user receive 5 character code groups at

increasing speeds.  The user is asked to enter the start speed, the end speed,

and the number of groups to be sent at each speed.  After the specified number

of groups is sent, the code speed is increased by 1 WPM until the end speed is

reached.

H.  Sending Code

1.  The <K>eyboard Send Command

You may wish simply to select enter individual keys to hear how

they sound.  This is the purpose of the <K>eyboard send function.  You can

type ahead of the CW sender by 80 characters maximum.  Unlike the <E>nter

command, you do not have to press RETURN to start sending characters.

2.  The <M>anual Key Command

You may also wish to practice your "fist", i.e., your manual

sending ability.  This can be done by using the ALT key as a telegraph

key.  You may not wish to overuse this command due to the possible wear on

the ALT key.

I.  The <T>est Command

1.  The <F>CC Test Command

This command permits you to take a simulated code test much

like the one you would take when you sit for your license exam.  The test

is based upon the simulated contact used for the <Q>SO command discussed

above.  The number of questions asked does not necessarily correspond to

the actual number on the test.  Recommendations for increasing the realism of

this function are solicited from users.

After the test has been taken, the user is given the opportunity

review the text of the test QSO.

2.  <K>eyboard Receive Test

This command lets you test your approximate receive speed.  The

computer will send a character and wait for you to type in the correct key.

When you exit this function using the ESC key, the computer will print out

your the approximate send speed, your approximate receive speed, and your

accuracy rate.  This command can also be used to learn to copy code on a

keyboard.  Although you will not be tested on a keyboard, it is the fastest

way to copy code and is the methos used by all the military services.

VI.  We Need Your Feedback and Problem Reports

A.  Feedback

As I continue to improve CW, it is helpful for me to have you feedback

about what you like about CW, what you don't like about it, any bugs or

cosmetic defects you have discovered, what you would like to see changed or

added, etc.  Please take the time to fill out and mail the form in the file

FEEDBACK.FRM to me.  It would be much appreciated.

If you would like to have the latest version of CW, please send a

formatted diskette and a self-addressed, stamped diskette mailer along with

your feedback form.

I particularly need feedback from those who use CW to train novices.

If I know what your needs are, I will endeavor to add those features to CW.

B.  Problems

If you encounter severe problems in running CW, please complete and

return PROBLEM.RPT to me.

VI.  Timing Calculations

A.  Element Lengths

1.  One bit is defined as one dot time.

2.  Thus the code elements are defined as follows:

dot                           = 1 bit

dash                          = 3 bits

space between dots and dashes = 1 bit

space between characters      = 3 bits

space between words           = 7 bits

B.  Speed Calculations in General

1.  The word PARIS is used as the standard word, i.e. the code

speed is the number of times PARIS is sent in one minute.

2.  PARIS has 31 bits, or an average of 6.2 bits per character.

C.  Speed Calculations for keyboard test function.

1.  The average number of bits per character in all the numbers and

letters is 9.83.

2.  The average character takes 9.83/6.2=1.58 times as long to send

as the average character in the standard word.

3.  A word is assumed to be 5 characters in length.

4.  A 7 bit wordspace is added for every 5 characters --

0.71 character lengths.

5.  Thus, wpm can be estimated as recv_speed*60/5*1.58.

8.  Spaces between characters are ignored.

VII.  Join the ARRL

If you are a prospective ham, you may wish to join the American Radio

League, which is the official organization representing amateur radio

in the United States.  The ARRL publishes a monthly magazine called QST and

provides many helpful services and publications.  The address is:

American Radio Relay League

225 Main Street

Newington, CT 06111

VIII.  Troubleshooting

A.  Code Speed Is Too Fast or Too Slow

Calibrate the program as described hereinabove and in the README.1ST

file.  For best results you should disable memory resident programs since they

steal clock cycles which CW depends upon for proper timing.

B.  'File Not Found '

1.  Make sure all the required files listed early in this

documentation are on the program disk.

2.  Make sure the program disk is in the currently active drive

when run.

3.  Make sure that CW.CFG is present and is not shown with 0 bytes.

The latter is caused if the program is rebooted while setting program

parameters or using certain utilities. If this happens, erase CW.CFG and start

the program again.  You will be forced to reconfigure CW unless you have saved

your current configuration in a backup file.

C.  Program Works, but No Sound

1.  Try another program which makes sounds or plays music.  If that

program does not work, then the problem is in your system.  If you do not have

such a program, check your distribution disk for a BASIC music program which

you can run using BASICA.

2.  Apparently some IBM clones do not fully implement the speaker

output and will make bell sounds (beep) only.  If you have such a machine, CW

will not work on this type of system.