This file describes details involved with operating KAMterm under

certain ``special'' environments.  The majority of the input for this

file will ultimately come from KAMterm users operating under these

environments.

Any users with input to this file are invited to contact me with

whatever information they feel might help others in similar situations.

i) General System Requirements:

*) PC or clone (XT/286/386/486/....) running dos

*) AT LEAST 256 k of free memory after dos and any utils loaded

(256 k will be a very minimal configuration)

*) Serial port connected to TNC (see notes below on serial ports!)

*) For Host Mode operations, you must have a Kantronics TNC

*) For certain Host Mode features, you must have revision 5.0 or

greater firmware in the TNC

CONTENTS:

i) General System Requirements

1) Serial ports with the NS16550AN or NS16550AFN UART

2) DESQview

3) Non-Kantronics TNCs

4) Very old computers (circa 1982)

5) Computers with non-standard serial ports

1) Serial ports with the NS16550AN or NS16550AFN UART:

In short, KAMterm provides full support for these advanced UARTs.

It will automatically detect them if they exist, and will make full

use of the extended buffering, etc., to improve performance.  KAMterm

will notify you while loading if either of these UARTs were detected.

For more information, see the main documentation.

There is one thing, however, that must be considered when using the

enhanced capabilities of the 16550.  If a program which utilizes the

16550's enhanced features exits without resetting the chip to normal

(e.g., 8250-style) operation, some older programs without support for

the 16550 will not be able to use the serial port until a hard reset

is performed (CTRL-ALT-DEL is not enough).  This is the result of

slight differences between how the older 8250 UART could, in some

cases, be handled, and how the 16550 is handled.

KAMterm does reset the 16550 when it exits, so this is not normally

an issue.  HOWEVER, if KAMterm does not terminate normally, e.g., if

you do a soft-reboot (CTRL-ALT-DEL), close a DESQview window while

KAMterm is still running in it, etc., the 16550 will not be reset.

After this, some older communications software may not be able to

access the port until a full hardware reset (using the RESET switch

or powering down the computer) is performed.

In other words, always exit KAMterm properly when using a 16550-based

serial port.  This will avoid such problems.

2) DESQview:

When using KAMterm under DESQview, it is important that you provide

DESQview with the appropriate configuration information.  I am not a

DESQview user, myself, so I can't promise that this list is

comprehensive, however, here are the important setup parameters I

determined in a short bit of experimenting on a friend's computer:

*) Memory Size:  *AT LEAST* 256k, for an absolute minimum

environment (NO scrollback buffers used, only 1 or 2

windows allowed, etc.).  Best to set at 300k or better.

*) Uses its own colors:  Yes

*) Writes directly to screen:  Yes

*) Uses serial ports:  Yes

*) Can be swapped out:  No

*) Runs in background:  Yes

3) Non-Kantronics TNCs:

Check out KTterm, a non-Host Mode terminal program from the makers

of KAMterm.  It offers those KAMterm features which do not require

Host Mode to work, and costs less than KAMterm.  :-)

If you use another brand of TNC that does have a Host Mode interface

protocol, and would like KAMterm to be expanded to support that TNC,

please contact me --- I would like to eventually expand KAMterm to

support multiple comm ports, and as part of that, to also support

multiple host protocols.  But I need the protocol specs for the other

TNCs to do this.

4) Very old computers (circa 1982):

KAMterm has been observed to have problems with very, very old

computers, or computers whose BIOS dates back this far.  This has

only appeared once or twice to my knowledge, but it has happened.

In one case, KAMterm was transferred to another, newer computer,

with almost the exact same configuration as the older one, and

worked perfectly.

I would appreciate it if anyone with such problems would let me

know as many details as possible so I can attempt to determine

a pattern, and eventually, a fix.

5) Computers with non-standard serial ports:

At this time, KAMterm supports the following serial port

configurations:

COM1:  IRQ4 / Base address 0x3f8

COM2:  IRQ3 / Base address 0x2f8

COM3:  IRQ4 / Base address 0x3e8

COM4:  IRQ3 / Base address 0x2e8

Work is in progress to expand the async library used in KAMterm to

support non-standard serial port configurations (to be defined

directly by interrupt line and base address), but this has not yet

been completed.