THENET-6.DOC

NODE CHECKOUT

*****************

Before placing the  node in service, it is a good idea to verify modem tone

frequency  accuracy.  This can be done by using a frequency counter on the  TNC

audio output together with the alignment  tone  commands.  Or, by following the

modem  calibration  procedure  using the firmware chip as described in the  TNC

manual.  Carefully save the original TNC firmware chip for possible future use.

NOTE:   Sometimes  the  contents  previously  stored  in  RAM  memory  will

interfere  with  node  initialization when the TNC is first powered  ON.   This

situation  has been observed after replacing the TNC firmware chip with the new

nodechip.  The solution is to turn the TNC off and pull the battery jumper (JMP

5 on MFJ's) for about one minute.   The power down process deletes the contents

previously held in RAM memory.  Reinstall the battery jumper and the TNC should

initialize properly.

Using  a  standard  RS-232  cable, such  as  the one  between the  TNC  and

terminal,  connect  the  terminal  to the noded TNC.   Make  sure  the  TNC  to

terminal data rate is set to agree with each other!  Power on the TNC.  The STA

and CON  LED's should go on and then off.  A TNPlus sign-on message will appear

on the  terminal's  screen.   Approximately 6  seconds after initial TNC power-

on, the PTT LED will flash.   Connect  to the  new node by pushing  the  ESC  C

<enter> keys.  The screen should now read: "CONN to <callsign>".

At  this  time  verify the node is working by checking  out  its  commands.

Type:   "I"  <return>, "H" <return>, "N" <return>, "P" <return>, "R"  <return>,

"S" <return>,  "U"  <return>, an invalid command such as "W" <return>, and  "B"

<return>.   You should get an appropriate response back from each of these node

commands.   Operating the node in this manner is called "Host Interface" and is

similar  to  using  a  regular TNC in the NON-MONITOR mode.   You will  NOT  be

able to monitor unconnected packets.

Reconnect  to  the  node  and  if a nearby  station or node is operational,

connect  to it by issuing the standard  "C <callsign>" command.  (The "C alias"

command  will not work until the node has become aware of its neighbor.)    All

of the normal packet  activities can be  conducted while connected as the Host.

After the TNC has been powered on for six seconds  or so, the node will issue a

NODES broadcast  which will be recognized by nearby line-of-sight nodes.  NOTE:

If one doesn't want knowledge of the node to be injected into the network, pull

the TNC-to-radio connector until after the 6 - 10 second initial turn-on period

has elapsed.   When satisfied the node is performing satisfactorily, disconnect

with either the B command, or the ESC D command.

A unique password string has been hard burned into the  "TNPlus" node chip.

Remember this password string since from time to time, you'll need to enter new

material into the INFO section.  To view the node's password string, it will be

necessary to enter the ROM image for that specific node into the SET210 utility

and view menu selection 36.  An  important  note:  The password  string is CASE

sensitive,  which means it will  not accept a  lower case letter when a capital

letter is required.

To  remotely  SYSOP the  node, connect to it  by  issuing a "C <nodealias>"

command  and  then type "S".  A random  series of  5 numbers  will be returned.

Enter the password string that corresponds to these numbers.  (Review the SYSOP

COMMAND  LIST  files for more details).  The node will ignore you.  To test the

success  of  the SYSOP command, type "P <return>".   This will return  a string

of numbers,  representing  the default values for the various node  parameters.

Note the value of the first number (typically 86).  Type "P 1 100".  If correct

the first number will be the new value "100".   Type "P 1 <original number>" to

restore the original first number.

Now that you have been accepted as a SYSOP, new information can be put into

the INFO section.   One may want  to place information about the node's QTH and

operating  frequency.  Following that, one might want to mention the next radio

club  meeting,  callsign of the local BBS, or other  information of interest to

local and distant users.  For example, as  SYSOP,

enter:

"I CARA elections coming up on the Sept 3rd meeting, 7PM" <enter>

The node will  echo this line.   For specific information on how to  write into

the  INFO  section,  review  the  material presented in the SYSOP COMMAND LIST.

As  SYSOP, one can also change the parameters.  In fact, one was changed in

the example above.   The default  node parameters  have been  carefully set for

proper operation.   (IT IS  NOT RECOMMENDED  THEY BE CHANGED  UNLESS YOU HAVE A

STRONG REASON  FOR DOING SO!)   The node parameters modify the surrounding node

network  performance  in  subtle,  and  sometimes  not  so  subtle,  ways.   An

improperly  set  node  parameter can harm the overall network thruput.   Before

making  any  changes,  read over this manual carefully and  consult  with  your

neighbor NodeOps to get a feel for what the change may do.