********************************************************************************

SET210.EXE

A Utility for setting up TNPLUS version 2.10

software prior to programming

By William A. Beech, NJ7P

NJ7P@NJ7P.AZ.USA.NA

*************************************

This utility is specifically designed to assist NodeOps in setting up TNPlus

version 2.10 prior to programming.    It is menu driven and simple to use.  Be-

fore  invoking SET210.EXE, it is assumed the TN210xx.ROM files are available in

the current directory.  The commands for bringing up SET210.EXE are:

(For monochrome monitors)  SET210 -b

(For CGA color monitors)   SET210

SET210.EXE will then load and show the main menu:

IMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM

;    SETPLUS 2.10 (C) by Bill Beech, NJ7P   ;

;                                           ;

;         (1)   Load Binary ROM image       ;

;         (2)   Load HEX ROM image          ;

;         (3)   Edit Parameters             ;

;         (4)   Save Binary ROM Image       ;

;         (5)   Save HEX ROM Image          ;

;                                           ;

; Enter Desired Operation  (0 to Quit):     ;

HMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM<

Ordinarily the binary TN210xx.ROM file will be treated.  Following selection

of number 1 from the window, SET210 will ask the name of the file to be loaded.

SET210 will only accept the file name WITHOUT the extension.    For instance if

TN210ST is listed, it will be loaded.   However, it won't accept TN210ST.ROM as

a valid filename.

After  the  file  is loaded, the  main menu will appear on the screen again.

This time select number 3, which allows the custom features to be setup.  Other

than installing  the  callsign/SSID, node alias and password, there is not much

else to do unless parameters other than those displayed, are desired.  Normally

the defaulted  parameters  are  adequate for simplex multi-user node operation.

If uncertain  about values on any of the first 23 parameters, don't worry about

it  as  they  can be  remotely set by  SYSOP action.   The parameters following

number 23 cannot be changed after the node chip is programmed.     A discussion

on parameter settings are contained in files TN210_11.DOC through TN210_16.DOC.

These should be consulted before proceeding.

It should be noted  the values listed on the  parameter menu are the same as

associated  with  the specific  TN210xx.ROM image file being dealt with.  Since

this  is subject to variation (for instance, the file passed on to you may have

had the parameters changed somewhere along the line),  the recommended defaults

are listed  in the following menu.    Suggest these be printed out and compared

with the ROM image you are working with.

PARAMETER MENU

with

Recommended node default parameters

IMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM;

; 1.  Minimum Quality for Update 86       2.       HDLC Channel Quality 100   ;

; 3.      RS-232 Channel Quality 255      4.    Obs Count Initial Value 6     ;

; 5.  Obs Count Minimum for Bcst 5        6.  Nodes Bcst Interval (sec) 1800  ;

; 7.                 FRACK (sec) 6        8.                   MAXFRAME 7     ;

; 9.          Link Maximum Tries 10      10.         Validate Callsigns On    ;

;11.          Host Mode Connects Off     12.             TxDELAY (10ms) 30    ;

;13.          Broadcast via Port Both    14.                # Propagate Off   ;

;15.         Conn Command Enable On      16.    Destination List Length 100   ;

;17.    Time-to-live Initializer 12      18.    Transport Timeout (sec) 240   ;

;19.     Transport Maximum tries 2       20.  Transport ACK Delay (sec) 3     ;

;21.  Transport Busy Delay (sec) 100     22.      Transport Window size 4     ;

;23.   Congestion Control Thresh 4

NOTE:  The above 23 parameters are remotely SYSOPable.

The following ones are fixed at time of programming:

24.  No-Activity Timeout (sec) 900   ;

;25.        P-Persistance Thresh 64      26.           Slot Time (10ms) 10    ;

;27.      Link T2 Timeout (10ms) 100     28.     Link T3 Timeout (10ms) 0     ;

;29           Station ID Beacons Used    30.              CQ Broadcasts On    ;

;31.                 Full Duplex Off     32.             Port Direction A0 B0 ;

;33.                 Multipliers Off     34.        Node Callsign   NJ7P  -4  ;

;35.            Node Alias      SVATST   36.              Node Password       ;

;                                                                             ;

;Enter Number of Parameter to Change (0 to exit):                             ;

HMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM<

The  Password string  requires a minimum of 5 characters and no more than 40

characters.

When  finished  with  the  parameter  menu,  select 0 and the main menu will

reappear.    Here, choices  4 or 5  are  the ones to zero in on.    This choice

should  be based  on whether your EPROM programmer likes a binary or a hex file

to  work  with.     SET210 will assign the appropriate extension, so it is only

necessary  to  select  a  suitable  filename,  such  as the node alias when the

prompt for a filename appears.

Parameters 32 and 33 are peculiar to only the TN210TM.ROM image.  Therefore

entries are not required of them for the TN210ST.ROM and TN210DX.ROM images.

If any questions or problems develop, drop me a msg via my BBS address.

73 and happy NodeOping!

de Bill, NJ7P@NJ7P.AZ.USA.NA

June 28, 1992