STAC ELECTRONICS

TECHNICAL SUPPORT BULLETIN

NUMBER:     008

SUBJECT:    COPROCESSOR

DATE:       July 9, 1991

TIME:       12:47 PM

SOURCE:         Tech Support, Engineering

SUPERCEDES/

SUPERCEDED BY:

RELEASE STATUS: Public

TITLE:

Checklist for coprocessor board installation

OVERVIEW:

The STACKER hardware installation routine may fail at all addresses for

the coprocessor card.  Often this is due to a conflict with a 16-bit VGA

card.

ISSUES/COMMENTS:

The STACKER.COM file (versions 1.12 and greater) has been modified to

resolve this conflict for most systems.

It is possible that incompatibility with the 16-bit VGA card may persist

on some systems despite the modifications to STACKER.COM.

RECOMMENDED ACTIONS:

1) Examine the date of \STACKER\STACKER.COM of the boot drive.  If dated

prior to 6-27-91, it does NOT include the VGA fix.  You may download the

new driver and instructions (VGAFIX.EXE, a self-extracting ZIP file) from

the Main Board of the BBS, or contact STAC Technical Support.

In case the VGA fix does not solve the conflict, try steps (2) through

(6).

2) Attempt to install at all seven addresses as specified in manual (Re-

enabling Memory Managers, p.14 or 17).  Note that any change to the

/B=addr switch MUST be followed by a power cycle (or cold boot) to reset

the addresses.  A warm boot does not suffice.

3) Remove (or type REM in front of) all load-high lines in CONFIG.SYS and

AUTOEXEC.BAT.  Alternatively, exclude addresses C400-DFFF (all possible

ranges for the STACKER card) from the memory manager.  Then attempt to

install.

4) If possible, reset the VGA card from 16- to 8-bit mode.  Different

cards do this in different ways: via software, via CMOS (system setup at

boot time), on the card itself (DIP switches, jumper).  It can also be

forced to 8-bit mode when moved to an 8-bit slot.

5) Trident's VGA card has a jumper (J2) which, when reset, will often

allow the card to locate an address.

6) If applicable, disable shadow (sometimes called "scratch") RAM, shadow

BIOS and shadow video.  Then attempt to install.