STAC ELECTRONICS

TECHNICAL SUPPORT BULLETIN

NUMBER:     010

SUBJECT:    COPROCESSOR

DATE:       June 27, 1991

TIME:       10:06 AM

SOURCE:         Tech Support, Engineering

SUPERCEDES/

SUPERCEDED BY:

RELEASE STATUS: Public

TITLE:

Performance difference between STACKER software and coprocessor versions.

OVERVIEW:

1) STACKER's software version is supplemented and enhanced by the

coprocessor card.  The card allows higher compression of data, and the

hardware support offers greater speed in compression and decompression.

2) In the coprocessor version:

a) compression offers as much as 12% more disk space;

b) disk-reads and -writes are 5-15% faster;

c) the higher performance is especially apparent on slower machines;

d) the device driver takes up 9KB less base memory.

3) In the software version, 16KB of upper memory needed for the card is

left free.

ISSUES/COMMENTS:

1) If the coprocessor card does not install correctly or load at boot

time, STACKER will still run ("Coprocessor card not functioning--invoking

software driver") and data will still be both compressed and accessible.

2) Subsequent upgrade from the software to the coprocessor version does

not require re-installing STACKER.  However, only those files written to

the volume with the coprocessor benefit from the higher compression.

3) Use of the coprocessor card may require excluding its address range

from use by other applications.  Consult the User's Manual for exclude

statements in memory managers and Microsoft Windows ("Re-enabling Memory

Manager Software".)

RECOMMENDED ACTIONS:

1) Verify addresses, syntax, typing and memory management in CONFIG.SYS.

2) Back up, SREMOVE and re-install and restore the STACKER volume to

benefit from the increased compression of the coprocessor card.