Application Notes for use of All Chargecard on a PS/2 Model 30-

286, 50, 50Z, or 60 running 3Com 3+ or 3+Open DOS workstation

software.

Configuration 1 -   3+ DOS workstation OR

(see also config.w3p and autoexec.w3p)

3+Open DOS workstation - XNS protocols

(see also config.w3o and autoexec.w3o)

Full NetBIOS or partial (not shown)

Windows/286 (No EMS, no Smartdrive)

VGA

1MB of motherboard memory

3Com EtherLink/MC network adapter

Configuration 2 -   3+ DOS workstation OR

(see also config.3p and autoexec.3p)

3+Open DOS workstation - XNS protocols

Full NetBIOS or partial (not shown)

VGA

1MB of motherboard memory

3Com EtherLink/MC network adapter

*** CAVEATS:

1) On 3Com's 3+ network operating system, if full NetBIOS must be

loaded high, you must use NetBIOS 1.2.1H, available

for 3+ customers from 3Com by dialing XXX-XXX-XXXX.

(This module is included with 3Com 3+Open systems.)

2) In a Model 50Z, both the LCC and PLCC adapters must be used.

Both are available separately from All Computer.  You lose the

use of two expansion slots.

3) In a Model 30-286, the PLCC adapter must be used.  This

adapter is available separately from All Computer.

INSTALLATION AND CONFIGURATION PROCEDURE - Configuration 1

1) Install the All ChargeCard hardware per instructions.  Follow

the instructions VERY CAREFULLY.

2) Use the IBM PS/2 Model 30-286 or 50/60 Reference diskette move

the Packet Buffer RAM of the EtherLink/MC card to address

C000-C5FF.

3) Add

device=<path>\allemm4.sys /RAM /FR=NONE /X=C000-C5FF /EXT=64

line to config.sys.  Do not load any network drivers high

at this time.

/RAM - tells allemm4 to find any available logical RAM between

640K and 1MB.  On a '286 PS/2, VGA memory (A000-AFFF,

and B800-BFFF) is not available, neither is extra ROM

BIOS memory (in which there are no ROMS, thanks PS/2)

(E000-EFFF) is not available either.  /RAM figures this

out all by itself.

/X=C000-C5FF - eXcludes the 24K RAM Packet Buffer area of the

EtherLink/MC card.This leaves the largest

contiguous area (104K) available (C600-DFFF).

**This parameter MUST match the RAM Packet

Buffer area assigned using the PS/2 Reference

Diskette.

/EXT=64 - Reserves the first 64K of extended memory (above

1MB) for use by himem.sys.

4) Install Windows/286, telling it that extended and expanded

memory are available.

5) Add remaining lines to config.sys and autoexec.bat per the

appropriate config and autoexec files.  The first line of

config.sys MUST be "device=himem.sys".  The second line of

config.sys MUST be the "device=<path>\allemm4.sys..." line.

The remaining "device =..." lines are in autoexec.bat.  All

operating software includes its own "device," "files,"

"buffers," and "lastdriv" 'commands.'  That is why these are

in autoexec.bat rather than config.sys.

The "alload" command loads drivers high.

3+Open only:  In the \3open\doswksta\lanman\lanman.ini file,

find the line containing the "redir" command.  Add to the

front of it the full path to the alload command:

i.e.  \<path>\alload redir .....

6) At this point you may want to experiment with loading drivers

in different spots to obtain the best fit; you may also use

the PS/2 Reference Diskette to move the location of the

Packet Buffer RAM.  Don't forget to change the /X:...

parameter of the "device=<path>\allemm4.sys..." line of

config.sys to reflect the new location.  Use the alload,

device, or xexamem commands interactively with no parameters

to look at how memory is used.

Useful parameters to load drivers in different places are:

device /LM - loads driver into Low Memory (below 640K)

device /AF - Assigns (loads) driver into First available

segment

device /AT - Assigns (loads) driver into starting from the

end (Top) of available memory

alload (includes above parameters)

alload /SG:XX00 - loads driver into specified memory segment

(e.g. /SG:B000)

***Experiments have shown that spp.sys wants to be loaded

first using the /AF command.  The order of the other device

drivers does not matter, but the included autoexec.bat were

designed to obtain the best fit.

INSTALLATION AND CONFIGURATION PROCEDURE - Configuration 2

1) Install the All ChargeCard hardware per instructions.  Follow

the instructions VERY CAREFULLY.

2) Use the IBM PS/2 Model 30-286 or 50/60 Reference diskette move

the Packet Buffer RAM of the EtherLink/MC card to address C000-

C5FF.  This leaves the largest contiguous area (104K)

available (C600-DFFF).

3) Add

device=<path>\allemm4.sys /RAM /FR=NONE /X=C000-C5FF

line to config.sys.

/RAM - tells allemm4 to find any available logical RAM between

640K and 1MB.  On a '286 PS/2, VGA memory (A000-AFFF,

and B800-BFFF) is not available, neither is extra ROM

BIOS memory (in which there are no ROMS, thanks PS/2)

(E000-EFFF) is not available either.  /RAM figures this

out all by itself.

/X=C000-C5FF - eXcludes the 24K RAM Packet Buffer area of the

EtherLink/MC card.

**This parameter MUST match the RAM Packet

Buffer area assigned using the PS/2 Reference

Diskette.

4) Add remaining lines to config.sys and autoexec.bat per

attached files.  The first line of config.sys MUST be the

"device=<path>\allemm4.sys..." line.  The remaining "device

=..." lines are in autoexec.bat.  All operating software

includes its own "device," "files," "buffers," and "lastdriv"

'commands.'  That is why these are in autoexec.bat rather than

config.sys.

The "alload" command loads drivers high.

3+Open only:  In the \3open\doswksta\lanman\lanman.ini file,

find the line containing the "redir" command.  Add to the

front of it the full path to the alload command:

i.e.  \<path>\alload redir .....

5) At this point you may want to experiment with loading drivers

in different spots to obtain the best fit; you may also use

the PS/2 Reference Diskette to move the location of the

Packet Buffer RAM.  Don't forget to change the /X:...

parameter of the "device=<path>\allemm4.sys..." line of

config.sys to reflect the new location.  Use the alload,

device, or xexamem commands interactively with no parameters

to look at how memory is used.

Useful parameters to load drivers in different places are:

device /LM - loads driver into Low Memory (below 640K)

device /AF - Assigns (loads) driver into First available

segment

device /AT - Assigns (loads) driver into starting from the

end (Top) of available memory

alload (includes above parameters)

alload /SG:XX00 - loads driver into specified memory segment

(e.g. /SG:B000)

***Experiments have shown that spp.sys wants to be loaded

first using the /AF command.  The order of the other device

drivers does not matter, but the included autoexec.bats were

designed to obtain the best fit.