TINYHOST

V 3.31

10 August 1993

Copyright (C) 1991-93 by Bruce A. Krobusek

A small, personal, Host / BBS program for

IBM PC's and compatibles

Created by : Bruce A. Krobusek

5950 King Hill Drive

Farmington, NY  14425

TinyHost BBS : 716-924-4193

CompuServe : 74106,1335

1.0   INTRODUCTION                                              1

1.1   What is TinyHost?                                       1

1.2   Why TinyHost?                                           2

1.3   What is a SysOp?                                        2

1.4   Quick Start                                             3

2.0   INSTALLATION and SETUP                                    3

2.1   Host System Requirements                                3

2.2   Connecting System Requirements                          3

2.3   Installation of TinyHost                                4

2.4   Running TinyHost                                        4

2.5   Modem Requirements                                      4

2.6   Modifying the TINYHOST.CFG File                         5

2.6.1   Descriptor Summary                                  6

2.6.2   Communications Specific Descriptors                 6

BAUD - Beginning baud (bps) rate                  6

CARRIER - Monitor Carrier Detect select           7

HANGUP - Modem hangup string                      7

INIT - Modem initialization string                7

IRQ - COM port IRQ select                         8

ISDN - ISDN support select                        8

PORT - COM port to be used                        8

RESET - Modem reset string                        8

RINGBACK - Ringback mode select                   8

2.6.3   Security Level Descriptors                          8

ACCESS - Directory access security level          8

BOOT - Warm boot security level                   9

CHDIR - Change directory security level           9

DOWNLOAD - Download security level                9

OVERWRITE - File overwrite security level         9

PRIVATE - Private message security level          9

PUBLIC - Public message security level            9

SHELL - DOS Shell security level                  9

SYSOP - SysOp security level                     10

TYPE - Type (display file) security level        10

UPLOAD - Upload security level                   10

UPLOAD_DIR - Default Upload directory            10

USER - User account information                  10

VIEW - View User List security level             11

2.6.4   General Descriptors                                11

CHAT - Chat mode default                         11

DBUG - Debug information enable                  11

EVENT - Event time                               11

EXTERNAL - External protocol definition          11

LOG - Log file enable                            11

SCRNSAVER - Screen saver timeout                 11

SOUND - Local sounds On/Off                      12

START - TinyHost start-up directory              12

2.7   Setting up Access Levels                               12

2.8   Setting up a Default Upload Directory                  14

2.9   Custom Display Files - Welcome, Bulletin, and Logoff   14

3.0   OPERATION                                                15

3.1   Ringback Operation                                     15

3.2   TinyHost Operation                                     15

3.3   File Oriented Commands                                 17

3.3.1   File Transfers                                     17

3.3.2   Changing Directories                               17

3.3.3   List Directory                                     18

3.3.4   Typing out a File                                  18

3.4   Message Commands                                       18

3.4.1   Reading Messages                                   18

3.4.2   Entering a Message                                 19

3.4.3   Editing a Message                                  19

3.4.4   Killing (Deleting) Messages                        20

3.4.5   Viewing User List                                  20

3.5   Miscellaneous Commands                                 20

3.5.1   Expert Mode                                        20

3.5.2   Operator Page                                      21

3.5.3   Shelling-to-DOS                                    21

3.5.2   Executing a Warm Boot                              21

3.6   Command Chaining                                       21

3.7   User Inactivity and Disconnects                        22

3.8   Quitting a TinyHost Session                            22

4.0   ADVANCED TOPICS                                          22

4.1   General Notes                                          22

4.2   ISDN Support                                           22

4.3   External Protocols                                     23

4.4   Shelling-to-DOS                                        24

4.5   High-Speed Modems                                      25

5.0   TROUBLESHOOTING TIPS                                     25

5.1   General Comments                                       25

5.2   System Hang (Lockup)                                   26

5.3   Error in User Account                                  26

5.4   Modem Fails Reset                                      26

5.5   Modem Fails Initialization                             26

5.6   Phone Call Not Answered                                27

5.7   Garbage After Connect                                  27

5.8   Immediate Connect and Logoff                           27

5.9   Garbage During Logon                                   27

5.10  Added Message Not Displayed                            28

5.11  THREMOTE.BAT Not Found                                 28

5.12  TinyHost Doesn't Hangup                                28

6.0   MISCELLANEOUS INFORMATION                                28

6.1   What is Shareware?                                     28

6.2   Distributing TinyHost                                  29

6.3   License Information                                    29

6.4   Warranty Information                                   29

6.5   Program Support                                        30

6.6   New User Logons                                        31

6.7   Message File Format                                    31

6.8   Logging User Activity                                  32

6.9   SysOp Specific Notes                                   32

6.10  Using TinyHost with DesqView                           32

6.11  Known Program Limits                                   32

6.12  TinyHost Plus                                          33

Brand and product names used throughout this document are

trademarks or registered trademarks of their respective holders.

1.0   INTRODUCTION

1.1   What is TinyHost?

TinyHost is a small, personal, Host / BBS program I have

developed for IBM PC's and compatibles. It permits the transfer

of files (using modems) between two computers without the

requirements or overhead of other programs, particularly the

full-fledged BBS programs.

TinyHost has the following features:

*  Operation can be via COM1, COM2, COM3, or COM4 using a

Hayes compatible modem, and at rates of 300 to 115.2K bps.

*  Selectable IRQs for non-standard COM port interrupts.

Supports IRQs 2, 3, 4, 5, and 7.

*  Supports XModem, XModem/CRC, and 1K-XModem (sometimes

called YModem) as internal file transfer protocols.

*  Up to 6 external file transfer protocols can be

implemented.

*  Supports up to 100 users (registered version) with 11

security levels. The Shareware version supports up to 10

users.

*  Supports Ringback mode, which permits easy use of a voice

phone line for data communications.

*  A text file that resides on the host system can be viewed

without downloading.

*  Supports Personal and Public mail, and SysOp Comments.

Entry of Personal and Public mail can be set to a minimum

security level.

*  The actions of a remote user are echoed to the local

screen.

*  Unauthorized users are not able to logon to TinyHost.

*  Using the security level feature, access can be restricted

to one directory or the entire system can be available.

*  An Operator Chat function exists to permit the local

operator to chat on-line with the remote user.

*  Shelling-to-DOS while connected remotely can be permitted.

*  Support has been added for ISDN phone systems.

*  Uploads and/or downloads can be restricted to a minimum

security level.

*  Depending upon a user's security setting, all uploads can

be directed to a default Upload directory.

Page 1

*  User activity can be logged to a file.

*  TinyHost performs a self-check on execution to ensure

that it has not been tampered with by, for instance, a

virus.

*  TinyHost is just that - tiny! The executable file is less

than 100K, and the program only requires approximately

150K of system RAM to run in.

Your version of TinyHost should include the following files:

THOSTxxx.EXE - TinyHost's executable. The xxx

refer to the version number of

the program

TINYHOST.DOC - The TinyHost manual in printer

ready form (PRINT TINYHOST.DOC)

TINYHOST.WEL - A sample welcome file

TINYHOST.CFG - A sample configuration file

THREMOTE.BAT - Sample batch file for Shell-to-DOS

ZMXMIT.BAT   - Batch file for DSZ ZModem downloads

ZMRCV.BAT    - Batch file for DSZ ZModem uploads

REGISTER.FRM - Ready-to-print registration

form (PRINT REGISTER.FRM)

HISTORY.TXT  - A file describing the release

history of TinyHost

READ.ME      - A short text file describing

TinyHost

IN-NODE.LST  - A text file with a list of all

current Intelec nodes (BBSes)

1.2   Why TinyHost?

Why did I create TinyHost? I needed a method to transfer

files with friends without hassle. I also found that, since I use

computers at work and at home, I was constantly leaving needed

files at one location or another, which resulted in duplicated

(or lost) effort.

TinyHost is also ideal for the company that needs to keep in

touch with remote employees, such as a sales force. With the file

transfer and message capability, it is easy to provide a single

transfer point for up-to-date information.

TinyHost was written using Borland's C++, and consumes

approximately 150K bytes of memory space when running. The small

memory requirements, along with the small .EXE and other

associated files, permits even floppy-disk based systems to be

used for TinyHost operation.

1.3   What is a SysOp?

When the term SysOp is used in this manual it refers to the

person who installs TinyHost on his computer (you), decides who

can logon to his system, and what directories they are able to

access.

Page 2

1.4   Quick Start

It is recommended that the manual be read before trying to

use TinyHost. However, it is always tempting to fire up a new

program to see how it works. To print out the manual and

registration form, simply enter PRINT TINYHOST.DOC and PRINT

REGISTER.FRM at your DOS command prompt.

Here is a summary of what must be changed (at a minimum) to

get TinyHost to work.

First, the following comments assume that the target system

has a hard disk (Drive C), and that TinyHost will have its own

directory. Create a directory called TINYHOST in the root (main)

directory of the hard disk (C:\TINYHOST). Then create a sub-

directory called BIN within the TINYHOST directory

(C:\TINYHOST\BIN). Copy the THOSTxxx.EXE, TINYHOST.CFG, and

TINYHOST.WEL files to the C:\TINYHOST\BIN subdirectory. Then move

to this directory.

Using an ASCII editor (even EDLIN will do), edit the

TINYHOST.CFG file as follows: set the PORT descriptor to the

serial port that the modem is connected to (1, 2, 3, or 4). Set

the BAUD descriptor to the initial data rate of the modem,

usually the fastest speed of the modem (valid values are 300,

1200, 2400, 4800, 9600, 14400, 19200, 38400, 57600, and 115200).

Finally, edit one of the default USER accounts (or create a new

one) and create an account with your name, a password, and your

security level (8 or 9 is adequate). Exit your editor and save

all of these changes.

To run TinyHost, simply enter THOSTxxx.EXE at the DOS prompt

and, after a few seconds of setup, TinyHost should respond with

its standby screen while it waits for a call.

2.0   INSTALLATION and SETUP

2.1   Host System Requirements

TinyHost does not need an elaborate system in order to be

used. A minimal system would consist of a single floppy drive,

256K RAM, any type of display, a serial port, a Hayes compatible

modem, and MS-DOS 3.1 or later. TinyHost supports COM1-4 using,

by default, the normal hardware interrupts for each port (IRQ4

for COM1 and COM3; IRQ3 for COM2 and COM4). Other IRQs are also

supported, with a choice between 2, 3, 4, 5, or 7. ANSI.SYS must

also be loaded or local information may not be displayed

properly.

2.2   Connecting System Requirements

Any type of system and communications package should work

with TinyHost. Communications requirements are 8 bits, no parity,

and 1 stop bit.

Page 3

2.3   Installation of TinyHost

It is recommended that TinyHost be installed in its own sub-

directory. The directory that holds THOSTxxx.EXE is called the

"execution directory", and it is here that the .CFG and other

TinyHost specific files should be kept. Only two files are needed

for operation - THOST.EXE and TINYHOST.CFG. Specific information

as to the expected config file format follows in section 2.6.

Please remember that user accounts MUST be created prior to using

TinyHost - the program does not permit new user logon!

For security reasons, it is STRONGLY suggested that the

TinyHost execution directory be different from the start-up

directory!

2.4   Running TinyHost

To start TinyHost, move to the execution directory and type

"THOSTxxx", where the 'xxx' refers to the version number of

TinyHost.

2.5   Modem Requirements

TinyHost expects a modem that supports the Hayes compatible

command set and ASCII return codes. Because of the many different

modem configurations (switch settings) there would be no point in

attempting to provide a definitive setup for each. However, I can

describe what TinyHost expects - refer to the manual for your

modem to see how to make it fit these needs.

Most modems support two types of result codes - numeric

values (non-verbose) and ASCII strings (verbose). TinyHost

expects to receive verbose result codes of the following types:

OK - Informs the host that the modem has executed the

command string successfully.

RING - Informs the host that an incoming call has been

detected.

CONNECT - Denotes that the modem has made a 300 bps

connection.

CONNECT 1200 - A 1200 bps connection.

CONNECT 2400 - A 2400 bps connection.

CONNECT 4800 - A 4800 bps connection.

* CONNECT 7200 - A 7200 bps connection.

CONNECT 9600 - A 9600 bps connection.

* CONNECT 14400 - A 14.4K bps connection.

CONNECT 19200 - A 19.2K bps connection.

Page 4

CONNECT 38400 - A 38.4K bps connection

CONNECT 57600 - A 57.6K bps connection

CONNECT 115200 - A 115.2K bps connection

* Please refer to Section 4.5 on High Speed modems for

further information on supporting this CONNECT response.

Most modems support a switch setting that controls the state

of the RS-232 Data Terminal Ready (DTR) handshake line. The

switch setting can either force DTR TRUE at all times at the

modem, or it can be set to permit the host computer to control

DTR. If DTR is supported with the selected host computer and

modem, then a modem hangup string will be unnecessary as TinyHost

will toggle DTR when it wishes to drop carrier. However, if DTR

is unavailable, then the modem hangup string must be defined (see

HANGUP below) or TinyHost will be unable to terminate a call and

will never drop carrier.

2.6   Modifying the TINYHOST.CFG File

When TinyHost is run it looks for TINYHOST.CFG in its

execution directory. If it doesn't find it the program aborts. If

the file is located, TinyHost reads in the information that it

needs to run. The config file can be created or modified using a

simple ASCII editor. If the editor used places an EOF character

($1A) at the end of the file, make sure that at least one blank

or commented line is at the end of the file.

Each line in the config file can be up to 80 characters in

length. At any point in the line a comment can be added by

placing a pound sign (#) in the line - everything following it

will be ignored.

TinyHost requires specific information to run. To this end I

have implemented what I call 'descriptors' which denote what type

of information is specified on the line. The descriptor must

begin in the first column of the line. The descriptor can be in

either upper or lower case, it must be followed by a colon (:),

and the descriptor must be spelled properly. If a required

descriptor does not exist TinyHost will abort and display an

error message. Descriptors can be defined in any order.

The descriptors fall into three categories: communications,

security, and general. The following table summarizes each

descriptor, with more detailed information in the following

sections.

Embedded spaces must be eliminated from any descriptor

command line - especially from the USER account fields. For

instance, a USER descriptor account such as:

USER:Joe,De User,Password,8

is invalid because of the embedded space in the user's last name.

Page 5

The valid way to set up this account would be:

USER:Joe,DeUser,Password,8

2.6.1   Descriptor Summary

*BAUD       - Beginning baud (bps) rate

CARRIER    - Monitor Carrier Detect select

HANGUP     - Modem hangup string

*INIT       - Modem initialization string

IRQ        - COM port IRQ setting

+ISDN       - ISDN support select

*PORT       - COM port to be used

RESET      - Modem reset string

RINGBACK   - Ringback mode select

ACCESS     - Directory access security level

BOOT       - Warm boot security level

CHDIR      - Change directory security level

DOWNLOAD   - Download security level

OVERWRITE  - File overwrite security level

PRIVATE    - Private message security level

PUBLIC     - Public message security level

+SHELL      - DOS Shell security level

SYSOP      - SysOp security level

TYPE       - Type (display file) security level

UPLOAD     - Upload security level

UPLOAD_DIR - Default upload directory

*USER       - User account information

VIEW       - View User List security level

CHAT       - Chat mode default

DBUG       - Debug information enable

EVENT      - Event time

+EXTERNAL   - External Protocol definition

LOG        - Log file enable

#SCRNSAVER  - Screen saver timeout

#SOUND      - Local sounds On/Off

*START      - TinyHost start-up directory

* Required Descriptor

+ Explained in Advanced Topics section

# Available in Registered versions only

2.6.2   Communications Specific Descriptors

The following are the descriptors used in setting up TinyHost

communications. Sample uses are given for each command.

BAUD - This specifies the baud (bps) rate that the TinyHost

serial port is to operate at. Legal values are 300, 1200, 2400,

4800, 9600, 14400, 19200, 38400, 57600, and 115200. BAUD is

required. In addition, the COM port baud rate may be locked at

Page 6

the specified value by adding ",LOCK" after the baud rate:

BAUD:19200,LOCK

This has been added for use with error-correcting modems. When

using this mode the proper command(s) must be added to the modem

init string to lock the modem's serial port communications rate

as well.

For further information on the use of high-speed modems refer to

Section 4.5.

CARRIER - Specifies whether or not the modem's Carrier Detect

(CD) handshake should be monitored. A 'Y' or 'N' is expected.

Monitoring the CD line permits TinyHost to see if a caller has

been disconnected unexpectedly. CARRIER is optional.

CARRIER:Y       # Monitor CD for disconnect

HANGUP - Specifies the command string necessary to tell the

modem to hang up the phone (go on hook). Refer to the section on

modem requirements (2.5) to see if this string is necessary.

HANGUP is Optional.

HANGUP:ATH0     # Sample hangup (on hook) command

INIT - This is the initialization string used to configure

the modem. INIT is required. The string included with TinyHost

includes some modem commands that are necessary for proper

operation:

INIT:ATE0M0Q0V1X2S0=0S2=43       # Sample init string

E0 - Controls echo from the modem. Must be

set to 0, which is for no echo. Required.

M0 - Controls the modem speaker. A value of

0 turns the speaker off. Optional.

Q0 - Determines the kind of result codes sent

by the modem. Must be set for 0 (result

codes sent). Required.

V1 - Determines the type of result codes sent.

Must be set to 1 which specifies English

(ASCII) codes are sent. Required.

X2 - Selects extended modem result codes. Must

set to 2 or greater. Required.

S0 - Determines the number of rings on which

the modem will answer the phone. Must be

set to 0. Required.

S2 - Sets the value of the escape code for the

modem. Must be set to 43 ("+"). Required.

Note that, unlike some other communications programs, a  ^M

combination should not be added to the end of the string.

Page 7

IRQ - Permits setting a non-standard IRQ for the COM port.

Valid IRQs are 2, 3, 4, 5, and 7. If IRQ is not present then the

standard interrupt for the specified COM port is assumed.

IRQ:5            # Denotes IRQ5 is interrupt for the COM port

ISDN - Specifies whether or not ISDN support should be used.

A 'Y' or 'N' setting is expected. Refer to section 4.2 for

further information regarding ISDN support.

ISDN:Y           # Enable ISDN support

PORT - This specifies the COM port that is to be used. Valid

values are 1, 2, 3, or 4. PORT is required.

PORT:1           # Use COM port 1

RESET - Specifies the command string necessary to reset the

modem. If RESET is specified then the modem must default to

verbose text responses for proper TinyHost operation. RESET is

optional.

RESET:ATZ        # Modem reset string

RINGBACK - Specifies whether or not TinyHost will operate in

ringback mode. RINGBACK is not required - TinyHost will default

to Ringback Enabled Mode.

RINGBACK:Y       # Enable Ringback mode

2.6.3   Security Level Descriptors

ACCESS - Can be used to specify a security level needed to

access a directory. Up to 25 Access restrictions can be defined.

If no access restrictions are specified then anyone with CHDIR

permission can move to any system drive or directory. This could

be troublesome when floppy drives are specified and they contain

no diskettes. ACCESS expects a valid directory path (up to the

DOS limit of 63 characters), followed by a comma, followed by a

numeric value between 0 and 10 indicating a security level. A

typical ACCESS descriptor would be:

ACCESS:C:\TINYHOST\BIN,9

which would limit access to the C:\TINYHOST\BIN directory to

users with a security level of 9 or greater.

A default access value can also be specified as follows:

ACCESS:DEFAULT,9

This means that any unspecified directories can only be accessed

by users with a security level of 9 or greater. This will make

Page 8

securing a typical computer system much easier. Access settings

are reloaded with the Warm Boot command. See section 2.7 for

further information.

BOOT - Specifies the security level needed to perform a warm

boot of the host system. This command can be used with Overwrite

to update TinyHost's config file from a remote location. If Boot

security is not specified then no user will have Boot permission.

BOOT expects a numeric value between 0 and 10.

BOOT:9           # Security level to execute a Warm Boot

CHDIR - Specifies the minimum security level necessary to be

able to move between directories of the system. If not specified

then no security level will be permitted to change directories.

CHDIR expects a numeric value between 0 and 10.

CHDIR:3          # Sets Change Directory security to 3

DOWNLOAD - Specifies the minimum security needed to download

a file from TinyHost. DOWNLOAD expects a numeric value between 0

and 10. If it is not specified then anyone can download a file.

DOWNLOAD:3       # Sets Download security level to 3 or greater

OVERWRITE - Specifies the security level needed for file

overwrite permission. This can be used to update TinyHost's

config file remotely when used with the Warm Boot command. If

Overwrite security is not specified then no user will have

Overwrite permission. OVERWRITE expects a numeric value between 0

and 10.

OVERWRITE:9      # Sets Overwrite security level to 9 or greater

PRIVATE - Specifies the minimum security level needed to

enter a private message. PRIVATE expects a numeric value between

0 and 10. If it is not specified than anyone can leave a private

message.

PRIVATE:3        # Sets Private message entry to 3 or greater

PUBLIC - Specifies the minimum security level needed to

enter a public message. PUBLIC expects a numeric value between

0 and 10. If it is not specified than anyone can leave a public

message.

PUBLIC:3         # Sets Public message entry to 3 or greater

SHELL - Used to set the security level for executing the

Shell-to-DOS function. Refer to Section 4.4 for further

Page 9

information. SHELL is optional.

SHELL:9          # Sets DOS Shell security to 9 or greater

SYSOP -  Specifies the minimum security level necessary to

perform SysOp level functions. SysOp functions include the

ability to delete Public mail, and read and delete SysOp

Comments. SYSOP expects a numeric value between 0 and 10. If

it is not specified then remote SysOp privileges will be

disabled.

SYSOP:9          # Sets SysOp security level at 9

TYPE - Specifies the minimum security needed to execute the

[T]ype command to display a file. TYPE expects a numeric value

between 0 and 10. If it is not specified then anyone can display

a file.

TYPE:3           # Sets [T]ype minimum security level at 3

UPLOAD - Specifies the minimum security needed to upload a

file to TinyHost. UPLOAD expects a numeric value between 0 and

10. If it is not specified then anyone can upload a file.

UPLOAD:3         # Sets Upload security level to 3 or greater

UPLOAD_DIR - Specifies a path and security level for uploads

to TinyHost. UPLOAD_DIR expects a path to a directory to store

the files, along with the minimum security level needed to

override this feature.

UPLOAD_DIR:C:\TINYHOST\UPLOAD,7

For users with a security level less than 7, this would place

their uploaded files in the C:\TINYHOST\UPLOAD directory. See

section 2.8 for further information.

USER - Defines a user account. The line should contain (in

order) the users first name (15 characters maximum), last name

(30 characters), password (8 characters), and security level (2

characters). Items must be separated by a

comma (,). The security level should be specified by a numerical

string ranging in value from 0 to 10, with 10 being the highest

permissible security level. At least one, and up to 100 (10 for

the Shareware version), user accounts can be created. User

accounts are reloaded when the Warm Boot command is executed.

The following USER descriptor:

USER:Joe,Shmoe,JOEUSER,7         # User with security of 7

creates a user account for Joe Shmoe with the password of

JOEUSER. Joe has a security level of 7.

Page 10

VIEW - Minimum security necessary to perform a View User List

function. It may be desired to prevent lower level users from

viewing the names of the TinyHost user base, and setting VIEW

high enough will do this. View expects a numeric value between 0

and 10. Not specifying a VIEW security level will result in all

users being able to view the User List.

VIEW:3           # Sets View Users security level at 3

2.6.4   General Descriptors

CHAT - Specifies the default setting for Chat Mode on

TinyHost execution. A 'Y' enables Operator Chat; a 'N' disables

it. CHAT is optional, and defaults to enabled.

CHAT:Y           # Enable Operator Chat on start-up

DBUG - Set to 'Y' will direct TinyHost to display modem

response information on the local monitor. This can be very

useful when trying to troubleshoot a modem initialization

problem. DBUG is optional, and defaults to disabled.

DBUG:Y           # Enables display of modem responses

EVENT - Used to set a time for TinyHost to quit and return to

DOS. The time must be entered in 24-hour (military) format (0000-

2359, 0000 = midnight). All four digits must be entered. If a

user is logged on when the Event time occurs, TinyHost will not

exit until the user has logged off. The Event time is reset when

executing a Warm Boot.

EXTERNAL - Used to add an external protocol (such as ZModem)

for use with TinyHost file transfers. Refer to Section 4.3 for

further information. EXTERNAL is optional.

LOG - Enables the logging of user activity to a file. LOG

expects a 'Y' or 'N' setting. LOG is optional.

LOG:Y            # Logs user activity to TINYHOST.LOG

SCRNSAVER - Enables the resident TinyHost screen saver, and

can also specify an inactivity timeout (in seconds).

SCRNSAVER:Y,300          # Screen saver on with 5 minute wait

If a timeout value is not specified, a default of 2 minutes will

be used. After the screen is blanked, an incoming call or local

keyboard activity will bring the display back. Note that whatever

key is pressed locally WILL be passed to the TinyHost program.

This function is only available in the registered versions of

TinyHost and TinyHost Plus.

Page 11

SOUND - Specifies whether or not the embedded Tinyhost sound

effects (bells or beeps) are echoed to the local system. This is

handy for those that have a system running at home and dislike

hearing the beeps at all hours of the day and night. This does

*not* affect the beeps associated with an Operator Chat page. A

'Y' lets all sounds echo to the local system; a 'N' turns them

off.

SOUND:N                  # Disables local beeps

This function does not intercept any embedded Control-G

characters (ASCII "bells") in any user created files such as the

Welcome file. SOUND is also only available in the registered

versions of TinyHost and TinyHost Plus.

START - Specifies the start-up directory for TinyHost. This

is the directory that TinyHost moves to after its initialization

is complete. This is also the directory that is available to the

remote user that logs onto TinyHost so care should be taken that

no sensitive files are available for download from this

directory. START is required. A terminating backslash should not

be used except when specifying a root directory that is being

used for the start-up directory.

START:C:\TINYHOST        # Directory at initial logon

2.7   Setting up Access Levels

The first thing to remember is that if a user's account

hasn't been created in the TINYHOST.CFG file he will never be

able to log on! (See Section 6.6 for a method to permit new users

to log on).

From there, things get a little more complicated (but not

*too* complicated!). The SysOp will have to make decisions as to

how much system access to give to the user accounts he creates.

It is recommended that security level 10 be used to deny access

to items such as the floppy drives, etc. when a hard disk based

system is used. Trying to remotely access a floppy drive that

does not have a diskette in it will cause TinyHost to hang.

As noted above, a remote user can be restricted to the Start-

Up directory for his uploads and downloads. This is accomplished

by setting the CHDIR access to a higher level than that of the

remote user.

To limit access to other areas of the system, the ACCESS

descriptor should be used. For instance, it is recommended that

only the SysOp have remote access to the sub-directory that

contains the TinyHost program, config file, and message files.

Since there is no specific file upload or download restrictions,

permitting access to these files (particularly the config file)

would then give access to the entire user log.

Entire disk drives (floppy, hard, and RAM) can be locked out

Page 12

by simply setting up the following ACCESS descriptor:

ACCESS:X:,10

This would force a security level of 10 or greater to access

drive X. As long as no user (SysOp included) has a security of

10, then no access of drive X is possible. To lock out the root

directory of a drive the descriptor would be changed to:

ACCESS:X:\,10

Note the trailing backslash after the drive designation. This is

the only time that the backslash is necessary. To lock out a

particular directory:

ACCESS:C:\TINYHOST\BIN,9

would permit access to the TINYHOST\BIN subdirectory by people

with security levels of 9 or greater. Note that a trailing

backslash should not be used when specifying a complete directory

path.

For example, suppose that one wanted to have two separate

subdirectories, each with its own access level, within the

TINYHOST directory. Assuming that one directory is called LEVEL7

and the other is LEVEL8, the directory structure would look like

this:

C:\ --- TINYHOST --+--- LEVEL7

|

+--- LEVEL8

|

+--- BIN

Also, assume that the access levels within TINYHOST.CFG are set

as follows:

ACCESS:C:\TINYHOST\BIN,9

ACCESS:C:\TINYHOST\LEVEL7,7

ACCESS:C:\TINYHOST\LEVEL8,8

Users with a security level of 7 or higher would be permitted

access to the LEVEL7 directory. A security level of 8 or more

would be required to be able to move to the LEVEL8 directory. And

a level of 9 or 10 (probably a SysOp) would have access to the

BIN directory where all of the important TinyHost files are kept.

By adding the following line:

ACCESS:DEFAULT,9

the system is also further protected by setting a default access.

This means that any directory not explicitly mentioned in an

ACCESS descriptor can only be accessed by a user with a security

level of 9 or greater. Specifying a default access level does not

use one of the 25 standard ACCESS directory resources.

Page 13

The config file that has been included with the TinyHost

package is set up to lockout floppy drives A and B. Overwrite,

boot permission, default directory access, and SysOp security

levels are set to that of the "SysOp", Sam Sysop.

2.8   Setting up a Default Upload Directory

To give a SysOp better control over what files are available

to his/her users, TinyHost can place any or all uploads into a

default directory for review at a later time. The descriptor

format to enable this function is:

UPLOAD_DIR:<directory path>,<security level>

where <directory path> is the complete path to the directory

where the uploads are to be stored, and <security level> is the

minimum security level needed to override this function. For

example:

UPLOAD_DIR:C:\TINYHOST\UPLOAD,7

would place all files uploaded by users with security levels of

0-6 in a directory named C:\TINYHOST\UPLOAD. Users with a

security level of 7 or higher would have their uploads left in

the TinyHost system directory they are in at the time.

TinyHost will verify on start-up that the Default Upload

directory exists. If a valid security level is not given then a

default of 0 (zero) is assumed. This would disable the Default

Upload directory option.

Any UPLOAD security level also specified in the config file

is still in force. For instance, if an UPLOAD security level of 2

is specified, then users with a security level of 0 or 1 will not

be able to upload any files at all. Users with a security of 3-6

would have their uploads re-directed to the UPLOAD_DIR directory.

A security level of 7 or higher would then be needed to override

this default upload directory setting.

2.9   Custom Display Files - Welcome, Bulletin, and Logoff

TinyHost permits the creation of three display files by the

SysOp that are shown at different points within the program.

TINYHOST.WEL is displayed after a connection has been made and

before a user has logged on. TINYHOST.BUL is displayed after the

logon process has been successfully completed. TINYHOST.BYE is

shown after the user has logged off. A sample TINYHOST.WEL has

been included on the TinyHost disk.

Each file can have any number of lines, but each line should

be no more than 80 characters long, and should be terminated with

a carriage return and line feed. If the file is longer than 20

lines TinyHost will pause at the 21st line (and every 21st line

thereafter).

Page 14

These files should be placed in the same directory that

THOSTxxx.EXE and TINYHOST.CFG reside (the execution directory).

If any of the display files do not exist TinyHost will continue

on its merry way!

3.0   OPERATION

3.1   Ringback Operation

Ringback mode has been implemented to permit using the same

phone line for both voice and data communications. To access

TinyHost when ringback is implemented the caller must first dial

into the computer and permit the phone to ring at least once (or

permit the modem to detect at least one ring). Then the caller

should hang up and wait at least 8 seconds before redialing the

computer. As long as the redial is done within 1 minute of the

first call, TinyHost will answer the phone and data

communications can begin. If more than 1 minute elapses then

TinyHost resets and begins waiting for the first ring.

3.2   TinyHost Operation

TinyHost is a very easy program to use - if for no other

reason than there aren't too many options with which to become

confused! After the program is started a status screen is

displayed on the local monitor. This information includes whether

or not TinyHost is in ringback mode, and, if so, whether or not

the program is waiting for the ringback. A local operator is also

able to view the progress of a remote connection since all

activity is echoed to the computer's monitor.

After TinyHost has been started, and while it is waiting for

a phone call, a status screen is displayed. Information such as

Ringback status (Enabled or Disabled), Chat Enabled or Disabled,

etc. is displayed. Pressing the Escape key ( <ESC> ) at this time

will exit TinyHost. A local user can logon by pressing <ENTER>

(or <RETURN>). The user will then be presented with the same

prompts that a remote user gets. Pressing <RETURN> again

(entering nothing for a First Name) will also cause an exit from

TinyHost.

Whether logging on locally or remotely, if logon is desired,

the user should enter his first name, last name, and password as

requested. It is permissible to string together any and all user

information (for instance, entering first and last name,

separated by a space, at the First Name prompt). It is not

recommended to enter ones password in this fashion as the

characters would be echoed to both the local and remote screens.

The names and password inputs are not case sensitive. TinyHost

does not permit retries when entering this information - if it is

entered improperly TinyHost will deny access and will hang up.

After the user successfully logs on, TinyHost checks to see

if any personal messages exist for the user. If so, TinyHost

beeps once, and displays a message that Personal Mail is waiting.

A check is also made for Public mail and SysOp Comments (security

Page 15

level permitting), and notices are displayed if either forms of

messages exist.

The user will then be presented with the Main Menu, which

displays the following command options. Either upper or lower

case characters can be used:

*D - Download a file from TinyHost to the user (if

permitted).

*U - Upload a file to TinyHost from the user (if

permitted). If the user's security level

permits, file overwrite is possible with or

without backup.

*C - If permitted, allows the user to move

between drives and directories of the system.

L - List the contents of the present directory.

*T - Types out (displays) a text file that resides

on the host system (if permitted).

*R - Read Personal or Public messages, or SysOp

comments (security level dependent).

*E - Enter a Personal or Public message or SysOp

comment. (Security level dependent)

*K - Kill Personal messages, or Private messages

or SysOp comments (security level dependent).

V - View User List (security level dependent).

S - Shell-to-DOS (security level dependent).

I - Displays an informative screen about TinyHost.

H - Display a Help screen ('?' can also be used).

X - Toggles TinyHost's Expert mode on and off.

When Expert mode is ON the longer menu and

informative messages are suppressed.

B - Re-display the logon Bulletin.

G - Goodbye. Ends the TinyHost connection.

O - Page the Operator (SysOp) for chatting.

W - Log off and reload the access, user, and event

info from the config file. Subject to permission.

This option is *not* displayed on the screen

- it is a SysOp secret!!

* Command Chaining possible

When using TinyHost, and reading this manual, text surrounded by

Page 16

angle brackets refer to names of keys, such as <F1> and <RETURN>.

Characters with square brackets around them ( [T] or [C] ) refer

to characters that, when entered, will execute that appropriate

action. Characters with curly braces ( {N} or {Y} ) denote

default actions (executed on pressing the <RETURN> key with no

other keys being pressed).

3.3   File Oriented Commands

3.3.1   File Transfers

TinyHost provides internal XModem, XModem/CRC, and 1K-Xmodem

upload and download capability (1K-XModem protocol is sometimes

known as YModem). All uploads and downloads take place from the

current directory that the user is in. If CHDIR access has not

been given, then file transfers take place from the Start-Up

directory.

Through the use of External Protocols TinyHost is able to

support other protocols such as ZModem. Refer to Section 4.3 for

further information on how to implement these.

Xmodem and XModem/CRC transfers appear as a single selection.

TinyHost will attempt to begin transfers using CRC as this is a

more reliable protocol than checksum. If CRC attempts are ignored

then the system will fall back to checksum.

When uploading a file to the TinyHost system, if the file

already exists the system will typically prevent overwriting the

existing file. The exception to this is if the OVERWRITE

descriptor has been set and the caller's security level matches

or is greater than that value. If this is the case then the user

will have the option of overwriting the existing file. If

overwrite is selected then the user will also be asked if a

backup of the current file is desired. If a backup already exists

then the current backup will be overwritten by the new backup (no

backups of backups are made!).

The overwrite option was added to permit updating TinyHost's

config file from a remote location. Since new users cannot

normally logon to TinyHost it is necessary to create an

appropriate account. By using both the overwrite and warm boot

capability the SysOp can download the existing config file,

modify it, upload it and overwrite the existing config file, then

tell TinyHost to load the new information.

Note that if a user logs on locally file transfers are not

permitted (for obvious reasons). However, the user can still go

through the motions to test the installation of any external

protocols. To aid in troubleshooting, TinyHost will display the

batch file command that would normally be sent to DOS.

3.3.2   Changing Directories

The Change Directory function of TinyHost acts much the same

as the chdir (or cd) function from DOS. When Change Directory has

Page 17

been chosen the program will prompt for a new directory. The same

type of input that is acceptable from a DOS command level is

suitable for TinyHost - entering two dots (..) will take TinyHost

to the next directory up the tree. Entering a complete path will

take the user to the specified directory. All of this is, of

course, assuming that the user has the necessary access to permit

moving around the host system. To ensure security of the host

system, TinyHost will return to the designated Start-Up directory

after a user has logged off.

3.3.3   List Directory

This function displays the files that reside in the currently

selected sub-directory of the system. First, the current

directory path is displayed. Then, any additional sub-directories

that exist are shown. Finally, the individual files in the

current directory are displayed along with their sizes.

3.3.4   Typing out a File

This function permits typing out (listing) a file that

resides on the TinyHost computer. It is strongly advised that

this function be used only to view text (ASCII) files. Viewing

binary files (those with .EXE, .COM, .ZIP, etc. extensions) can

cause VERY unpredictable results!

If the user has Expert Mode turned off when this feature is

selected, a cautionary message similar to the above paragraph

will be displayed, then the user will be prompted for a filename.

If Expert Mode is enabled, then the user will be immediately

prompted for the filename. If the file exists in the currently

selected directory, then it will be displayed. TinyHost will

pause every 21 lines and ask if it is to continue displaying the

file. TinyHost will also automatically wrap the screen display if

the line length exceeds 80 characters. TinyHost will keep track

of added line feeds to be sure that the display is paused

properly.

The maximum line length that TinyHost will support (without a

terminating carriage return or line feed) is 256 characters.

3.4   Message Commands

3.4.1   Reading Messages

When a user logs onto TinyHost a notification is given if any

Personal or Public mail is waiting, beeping once for Personal

mail. If the user's security level is high enough, a notification

of waiting SysOp Comments is also given. To read the messages,

enter 'R' at the Main Menu prompt. A choice will then be given to

read Personal ('P'), Public ('U'), or SysOp Comments ('C'). After

the second choice is made the message(s) are then displayed. If

more then one message is waiting, TinyHost pauses before

continuing on to the next message. The very first line of the

Page 18

message notes who the message is from, and the time and date that

the message was entered.

If no mail is waiting of the selected type, a message

reporting this is displayed. If the user does not have sufficient

security to read SysOp Comments a notification is given and the

Comments are not displayed.

3.4.2   Entering a Message

To Enter a message or SysOp Comment, enter an 'E' at the Main

Menu prompt. The user will be prompted for the type of message to

be left (Personal, Public, or SysOp Comment). After the selection

is made (taking into account any Personal or Public security

restrictions), a few notes regarding message input are displayed

along with a ruler line showing the length of a message line.

If entering a Personal (Private) message, the user will be

asked for the name of the person to send the message to. TinyHost

will then check to be sure that such a person exists in its user

base, so it is imperative that the user's name be spelled

correctly. To find out who exists in the user base, and to find

the proper spelling of names, a View User list can be done

(within security limits).

Up to 20 lines of 70 characters each may be entered. TinyHost

will automatically wrap input to the next line.  When entering

the message, do not put a period in the first (leftmost) column.

This is how TinyHost distinguishes the end of a particular

message (see Section 6.7). To end the message input, enter a

blank line. At this point, the message can be edited, saved, or

aborted.

If the message is saved, TinyHost will automatically add the

sending individual's name, and the time and date the message was

entered, to the beginning of the message. The entire message will

then be saved into the appropriate message file.

3.4.3   Editing a Message

After entering a message, and before saving it, the message

can be edited using the following editing commands:

[L]ist - List the entire message

[I]nsert - inserts a line after the line number specified.

Entering a blank line will abort the insert process.

If the message length is already 20 lines then an

insert will not be permitted.

[D]elete - deletes the specified line

[E]dit - permits editing the specified line. After entering

the line number the user is asked to enter the source

string to be changed. TinyHost will do an exact, case

sensitive search and if a match is found then the user

Page 19

is prompted for the string to be substituted. If the

resulting line exceeds 70 characters then the end of

the line will be truncated to fit.

When editing the message is complete, the message can either

be [S]aved or [A]borted.

3.4.4   Killing (Deleting) Messages

TinyHost will retain all messages until they are deleted.

This is accomplished by entering a 'K' at the Main Menu prompt.

The user will then be prompted for the type of mail to delete -

Personal, Public, or SysOp Comments. The user is then asked if

all messages are truly to be deleted, and if the response is Yes,

then the entire mail file is deleted.

If the user's security is not high enough to delete Public

Mail or SysOp Comments, an appropriate message will be displayed

and the user will be returned to the Main Menu.

It must be stressed that when Killing Messages, ALL such

messages are deleted! There is presently no way, for instance, to

just delete one Private message out of the group. However, since

a DOS file delete is done, the file should be recoverable using

an easily available file recovery utility.

Non-deleted messages will hang around forever (or until the

next disk crash!). Personal messages, though, will make a minor

nuisance of themselves by beeping every time the user logs on.

This should help remind users to purge their old messages.

See Section 6.7 for information on editing the message files

off-line using an ASCII editor.

3.4.5   Viewing User List

To help users when entering Personal mail, a View User List

function has been added. Entering a 'V' at the Main Menu prompt

will display the list of users currently defined in TinyHost's

user base. Both the first and last name of the individuals are

displayed.

The View User List function can be restricted to higher

security levels simply by setting the VIEW descriptor value high

enough. This may be desired when TinyHost is open to new callers

(see Section 6.6), or when different security levels are

implemented. If the VIEW descriptor is not found in the

TINYHOST.CFG file, any users will be able to see the user list.

3.5   Miscellaneous Commands

3.5.1   Expert Mode

Entering an 'X' at TinyHost's Main Menu will toggle the

Expert User Mode on and off. When OFF, full menus and informative

Page 20

displays are given throughout TinyHost. When ON, these displays

are suppressed and only a line showing the valid character inputs

is displayed.

3.5.2   Operator Page

Entering an 'O' will page the operator for online chat.

This is assuming, of course, that the SysOp has enabled this

function. If Chatting is enabled, TinyHost will beep every two

seconds for 30 seconds to get the SysOp's attention. Any key

pressed during this time by the remote user will cancel the page.

On the local screen, TinyHost will display the name of the

user who initiated the page. If the SysOp wishes to chat then the

<F1> key should be pressed on the local keyboard. The remote user

and SysOp can then chat back and forth simply by typing their

messages. TinyHost will automatically wrap the input to the

following line. To end the Chat, the SysOp must press <F1> again.

While TinyHost is waiting for a call, the SysOp may enable or

disable the Chat function by pressing <F1>. The status of Chat is

shown on the local screen.

3.5.3   Shelling-to-DOS

Entering an 'S' will select the Shell-to-DOS function.

Depending upon the implementation the remote user will then be

taken to the DOS prompt of the TinyHost machine. To return to

TinyHost, the remote user must type 'EXIT' and hit <RETURN>.

Whether or not a remote user can execute this command is set

(in part) by the SHELL descriptor security level.

Because of the complexity of implementing this command at the

TinyHost level, further information can be found in Section 4.4

under Advanced Topics.

3.5.2   Executing a Warm Boot

As described above, the warm boot option has been included to

permit the SysOp to modify some settings remotely by updating and

reloading the config file. TinyHost normally only reads its

config file on initial start-up. Entering a 'W' will result in

TinyHost logging off (same as if Goodbye had been selected). The

program will then proceed to re-load its config file and retrieve

ACCESS, USER, and EVENT info - everything else in its

configuration (INIT, RESET, etc.) will remain untouched.

3.6   Command Chaining

Many commands can be used with Command Chaining. This is the

process of entering two pieces of information at the Main Menu

prompt, with a space separating the items. For instance, to type

Page 21

out the file FOOBAR.TXT from the current directory, the command

line would be:

T FOOBAR.TXT <return>

Command chaining can be used in or out of Expert Mode, and

currently works with file and message commands. Section 3.2 lists

the available TinyHost commands, and the ones where command

chaining can be used are marked with asterisks ('*').

3.7   User Inactivity and Disconnects

TinyHost monitors the COM port for keyboard activity to be

sure that a user is still out there and awake. If no keyboard

activity is seen for an extended period of time (at least 2

minutes) TinyHost will hang up and recycle itself. The total

length of time that TinyHost will wait depend upon what menu the

user is in when the inactive period begins. A 5 minute delay is

set when Viewing a file; a 2 minute delay exists at the Main Menu

prompt. This activity monitor can also handle disconnects that

occur in the middle of a logon session.

In addition, if the CARRIER descriptor has been set to 'Y',

TinyHost will monitor the status of the Carrier Detect (CD)

handshake from the modem. If at any time the CD handshake is

removed (denoting a broken connection), TinyHost will recycle

itself.

3.8   Quitting a TinyHost Session

For those users with a security level greater than or equal

to that set by the SYSOP descriptor, TinyHost offers a QUIT

command. When executed, this will take the user through the

standard logoff procedure. However, instead of recycling back to

the normal mode of waiting for an incoming call, TinyHost will

cease running and quit to DOS.

4.0   ADVANCED TOPICS

4.1   General Notes

The features covered in this section are NOT for the faint of

heart!! In order to implement these properly a good knowledge of

DOS, and of the system and environment that TinyHost is running

in, is essential!

4.2   ISDN Support

ISDN is a digital only phone system that is starting to

become more widespread. With ISDN there is no modem, no init

string, no result codes, and a fixed baud rate. All activity is

controlled using the serial ports handshake lines. To enable ISDN

Page 22

support, the following descriptor must be added to TinyHost's

config file:

ISDN:Y                   # Enable ISDN support

When ISDN is used, RINGBACK cannot be set to Yes, and a

modem HANGUP string cannot be defined. It is also suggested that

baud rate locking (for example,  BAUD:9600,LOCK ) be used.

When in ISDN mode, the DTR line is not asserted, as it

is when in a modem application. TinyHost waits until the RING

handshake becomes active and then asserts DTR. TinyHost then waits

for DCD to be asserted to signify that a connection has been

established. To break the connection, TinyHost simply drops the

DTR handshake.

4.3   External Protocols

For those that want to use protocols other than XModem (such

as ZModem), TinyHost supports the use of external protocols. For

instance, the author uses DSZ (by Chuck Forsberg) to implement

ZModem file transfers. Up to 6 external protocols may be defined.

The descriptor format for defining an External protocol is:

EXTERNAL:<menu name>,<transmit batch>,<receive batch>

for example::

EXTERNAL:Zmodem,ZMXMIT,ZMRCV     # Sample for ZModem

<menu name> - The name of the protocol. This string may be up

to 30 characters in length, and may include embedded spaces. This

string will be displayed to the user at the Protocol Select menu

within TinyHost - in this case, "ZModem".

<transmit batch> - The name of the batch file used to invoke

the external protocol when sending a file from TinyHost to the

User (TinyHost Download). This batch file must reside in the

TinyHost execution directory. In the above example, ZMXMIT will

have TinyHost look for a batch file named ZMXMIT.BAT in the

execution directory for downloads.

<receive batch> - The name of the batch file used to invoke

the external protocol when receiving a file from the User

(TinyHost Upload). This batch file must reside in the TinyHost

execution directory. In the above example, the name of the upload

batch file is ZMRCV.BAT.

The actual external protocol executable file must be in a

directory pointed to by the DOS path command. The .BAT file

extension should not be added when putting the filenames in the

descriptor field.

When an external protocol is selected, TinyHost starts a

secondary command processor (usually COMMAND.COM). TinyHost also

passes some necessary parameters to the batch file. These are, in

Page 23

order: 1) COM port number, 2) Port speed (baud rate), and 3) the

filename. The COM port and baud rate are passed as ASCII

characters.

Two sample batch files have been included. These can be used

with DSZ to implement ZModem transfers within TinyHost. For

instance, the batch file for a TinyHost Upload would be:

DSZ port %1 speed %2 rz %3

The batch file for a ZModem Download would be:

DSZ port %1 speed %2 sz %3

4.4   Shelling-to-DOS

The SHELL descriptor permits setting the minimum security

level necessary for accessing this command remotely. However, a

lot of behind-the-scenes effort has to be done to make this work.

In addition to the security level requirement, there must be

a THREMOTE.BAT file in the *exact* directory the user is in when

trying to shell out. There is a reason for this: Some SysOps may

not want their users dropping to DOS in their machine, but they

might want to make custom menus, programs, etc. available for

their use. By customizing a THREMOTE batch file for the area the

user is in, different functions can be implemented using the same

command.

When executing the Shell command, TinyHost passes two pieces

of data to the batch file (much like it does when using an

external protocol). First, the COM port number is passed, then

the Baud rate. Both are passed as ASCII character strings.

A sample THREMOTE.BAT file has been included with TinyHost.

This file shows how a shell-to-DOS function can be implemented

using the DOS CTTY command. When returning from calling this

batch file, TinyHost will automatically reset the CTTY value to

CON (the standard console).

Caution should be taken when using the CTTY command to

implement a DOS Shell. Depending upon circumstances that are

unclear, when at the DOS level via remote a backspace character

can sometimes cause the host system to go berserk. Unlimited

space characters appear to be sent, and a Control-C may or may

not work in halting them. The only cure appears to be to break

connection (hang-up).

When using CTTY, it is recommend that caution be used when

executing other programs remotely. Direct video writes cannot be

supported using this function, and system lockup may be

experienced. When finished with DOS operations, entering "EXIT"

at the DOS prompt will return to TinyHost. It is suggested that

the user return to the directory the Shell command was called

from. Failing to do so could cause the system to lock up.

Page 24

Also, if one were to hang up or suffer a broken connection

while in DOS, there is no way for TinyHost to recover and the

system would stay that way unless manually reset. A good solution

here is to use a utility called WATCHDOG (or something similar)

which automatically reboots the computer should the carrier be

lost.

4.5   High-Speed Modems

With the advent of the higher-speed modems (faster than 2400

baud) the life of the communications programmer became much more

difficult! The many different modes, speeds, and lack of command

and programming standards has made any attempt at universal modem

support incredibly difficult. The important things to remember

when dealing with these buggers is that the modem connect rate

can differ from the computer-to-modem serial port rate.

Even though TinyHost supports computer-to-modem serial rates

(sometimes referred to as Fixed DTE, modem speed conversion,

etc.) of 7200 and 14400 baud, many (most?) modems do not. Please

check your modem manual to see if these rates are supported

before setting them with the BAUD descriptor. When in doubt, use

the faster rates of 9600 or 19200.

If your modem should respond with a CONNECT 14400 message and

baud rate locking is not enabled, TinyHost will attempt to set

the serial port baud rate to 14400. If, as mentioned above, your

modem doesn't support that serial port rate, all information to

and from the remote user will be garbled. NOTE: there will be no

indication of trouble on the TinyHost local screen! All menus

will appear normal!

When using baud rate locking, both the TinyHost BAUD

descriptor and the modem INIT string must be configured properly.

With TinyHost this is easily done by adding   ,LOCK   following

the baud rate value in the BAUD descriptor. For example:

BAUD:19200,LOCK

will lock the computer-to-modem baud rate at 19200. For this to

work properly, the necessary modem command must have been

included in the modem INIT string to prevent the modem from

changing speeds.

5.0   TROUBLESHOOTING TIPS

5.1   General Comments

Although I have done my best to make TinyHost an easy-to-use

program, it is difficult (if not impossible) to take all of the

different port and modem combinations into account. Here are some

suggestions as to causes and cures of some common problems in

getting TinyHost to work.

Page 25

A helpful debug feature has been incorporated into TinyHost.

By adding the DBUG descriptor to a line in the TINYHOST.CFG file,

modem response information will be displayed on the local

monitor's screen. This can sometimes help pinpoint the problem

area.

5.2   System Hang (Lockup)

This failure is when the computer that is to be used locks up

as soon as TinyHost is executed. Chances are, the last sign of

life was the "Initialization..." message being displayed on the

monitor screen.

This is usually due to a problem with the selected serial

port. It is possible that the specified serial port does not

exist in the computer. When trying to use COM3 or COM4, it is

also possible that an non-standard COM interrupt is being used.

TinyHost defaults to IRQ4 for COM3; IRQ3 for COM4. If a different

interrupt is needed, use the IRQ descriptor to set the proper

one.

TinyHost can also hang the system if a remote user attempts

to access a floppy drive that does not have a diskette in it.

5.3   Error in User Account

This message will be displayed during the initialization

process. This will be caused by an embedded space somewhere

within the user account information. It could also be caused by

an invalid security level.

5.4   Modem Fails Reset

This will occur as TinyHost is being initialized. The program

attempts to reset the modem, and has received no response. The

expected response is an "OK" string.

A number of causes are possible. Make sure that the modem is

connected to the specified serial port, and that the default bps

rate being specified with the BAUD descriptor is one that the

modem's serial port can support. The modem could also be set (via

DIP switches or non-volatile RAM) to default to no result codes

or to numeric instead of verbose. The simplest solution is to

remove the RESET descriptor from the TINYHOST.CFG file.

5.5   Modem Fails Initialization

TinyHost displays a "Modem Initialization Failure" string

when this happens.

Again, a number of causes are possible. Make sure that the

modem is connected to the specified serial port, and that a valid

BAUD rate setting is being used. The initialization string also

must have the Q0, V1, and E0 modem commands set as shown (Q0

Page 26

forces the display of result codes; V1 specifies that result

codes as character strings rather than numbers; E0 turns off

command echo).

Make sure that the modem supports the serial port connect

rate that has been chosen. Many (most?) modems do not support

computer-to-modem connect rates of 7200 and 14400 baud. See

Section 4.5 for further high-speed modem information.

5.6   Phone Call Not Answered

If TinyHost does not answer the incoming call, make sure that

the phone line is properly plugged into the modem. Also, make

sure that the RINGBACK descriptor is set to 'N' (No).

5.7   Garbage After Connect

This problem will only be seen by the remote caller, and will

appear as all garbage characters being received instead of the

expected Welcome file (if used) or First Name prompt.

This is probably caused by different bps rate settings

between the TinyHost system and the remote caller. This can be

due to the Data Rate of the modem being locked to a particular

value by an initialization command (usually available only with

error correcting or MNP modems). Try adding (or removing) the

LOCK command of the BAUD descriptor.

If occasional incorrect characters are received, the problem

is probably due to a noisy phone connection between the two

systems.

5.8   Immediate Connect and Logoff

Although difficult to describe, this is when TinyHost answers

the phone, displays the requests for First Name, Last Name, and

Password, then logs off without the remote user doing anything.

The number of times each prompt is displayed can vary.

Essentially, the remote user is never given the chance to logon.

This is probably caused by an improper echo command setting

in the Modem Init string. Command Echo must be set to off by

using the E0 setting in the init string.

5.9   Garbage During Logon

Here, TinyHost answered the call, established communications,

but the remote user sees occasional garbage echoed during the

logon process, and he is not permitted access to the system.

The most likely cause is the remote users communications

package is incorrectly set. TinyHost expects the connecting

caller to be using 8-bits, no parity, and 1 stop bit.

Page 27

5.10   Added Message Not Displayed

This type of error usually occurs when a message is created

off-line, using an ASCII text editor. What happens is that a

message added via TinyHost does not appear to be added (appended)

to the original message file.

This is caused by the ASCII editor used to create the initial

text file adding an End-Of-Text (EOT or EOF, $1A) to the file

when it is saved. When adding a message via TinyHost, the new

message is simply added on to the existing file. The EOT

character is never erased or overwritten, and when it is seen

TinyHost assumes that the entire message file has been read. This

character can be removed from the message file using a hex

editor, and this problem can be avoided by ensuring that the

ASCII editor used will not add the EOT character.

5.11   THREMOTE.BAT Not Found

This error message will appear when a [S]hell-to-DOS has been

attempted and is unsuccessful. When a [S]hell is attempted

remotely, TinyHost looks in the current user directory for a copy

of this batch file. When it's found it is then executed. A batch

file of this name *must* be in any directory that a user can

attempt a [S]hell-to-DOS from.

5.12   TinyHost Doesn't Hangup

If TinyHost doesn't hangup (disconnect) after a call is

complete, the problem is probably due to a lack of a modem hangup

string specification in the TINYHOST.CFG file. Refer to Section

2.5 (Modem Requirements) for further information.

6.0   MISCELLANEOUS INFORMATION

6.1   What is Shareware?

TinyHost is being released as Shareware, which means that you

can try the program out *before* you buy it and make sure that it

will do what you want. If you find it of use, you are requested

to mail a contribution ($25 suggested) to the author to support

the continued development of this and other products. Please send

a check or money order (U.S. funds only) to:

Bruce A. Krobusek

5950 King Hill Drive

Farmington, NY  14425

For your $25 you will receive the latest registered version

of TinyHost on either 5 1/4 (360K) or 3 1/2 (720K) media (please

specify). The registered version of TinyHost has basically the

same functionality as the Shareware version, but:

Page 28

*  Supports up to 100 users

*  Includes the Screenblanker and Sound On/Off functionality

*  Has exit delay removed

*  Has registration reminder text removed

You will also receive a registered version of the next major

release of TinyHost, and notifications of future upgrades.

A ready-to-print registration form has been included with

TinyHost. Simply print out REGISTER.FRM using the DOS PRINT

command, fill it out, and return it with your check or money

order.

For those interested, quantity discounts are available.

Custom versions of TinyHost can also be created. Contact the

author for more information.

6.2   Distributing TinyHost

Even though TinyHost is Shareware it remains a

copyrighted program. As such, it may be freely copied and

distributed under the following guidelines:

* The author retains the sole right to sell the program

for a profit.

* The author grants users the right to copy and distribute

the Shareware (Unregistered) version of TinyHost so long

as all program and documentation files remain unaltered

and are distributed as a group.

* The program may not be bundled with any other hardware

or software without the permission of the author.

* Recognized users groups may charge a maximum of $5 for

duplication and/or distribution of TinyHost.

Registered users are requested to distribute only the

Shareware (Unregistered) version of TinyHost. There is no major

difference in functionality.

6.3   License Information

Unregistered users are granted a 60-day evaluation period.

After that time, they are expected to either register TinyHost or

cease using it.

Registered users may use TinyHost on more than one machine,

as long as there is no possibility of more than one copy being in

use at a time. Site licenses are available.

6.4   Warranty Information

The author (Bruce A. Krobusek) disclaims all warranties

expressed or implied as to the quality and performance of this

program (TinyHost). The author will not be held liable for any

lost profits, lost savings, or any other direct, indirect,

incidental, or consequential damages resulting from the use of

Page 29

this program. Your use of this program constitutes your agreement

to this disclaimer and your release of the author from any form

of liability or litigation.

6.5   Program Support

Now that the legal technicalities are out of the way, let me

state that I have done my best to ensure that TinyHost is error

free. However, that is not to imply that one might not find

errors that have been missed. If a problem should be found,

contact me through one of the avenues mentioned below and I will

do my best to correct it. Naturally, the more information that

can be supplied as to the nature of the bug the better chance I

will have of finding and squashing it!

I am also interested in receiving any comments or suggestions

regarding TinyHost. Although I have a few ideas for upcoming

enhancements, I am always interested in hearing what other people

think of TinyHost.

Here are some methods that can be used in contacting me,

starting with the most preferable:

TinyHost Support BBS - (716) 924-4193  USR Courier V.32bis

This is my personal BBS that is running TinyHost,

and it is available 24 hours a day. (Unless I happen

to be in the process of debugging a new release.)

Logon as NEW USER with the password GUEST. (This

may change as TinyHost Plus grows. Read the opening

screen for the latest logon and version info.)

Intelec Network - This is a national BBS network

There are nodes all over the U.S. and Canada,

with a few starting to appear in other nations.

I will monitor the Communications and Shareware

conferences for any messages addressed to me.

The latest Intelec node (BBS) list has been

included as IN-NODE.LST. There is probably a BBS

that carries Intelec near you.

CompuServe Informational Services - My account number

is 74106,1335. I usually logon daily.

Internet - via CompuServe:

74106.1335@compuserve.com

Micro Science BBS - (716) 594-1804  USR Dual Standard /

V.32bis. This Bulletin Board is located in

Rochester, NY, and is run by Larry Helber. The

latest version of TinyHost will always be available

as a free download. Do a file search (Z) and scan

for TINYHOST, or download THOST*.ZIP. There is also

a TinyHost support conference where messages can be

left and received. No BBS registration is required

for TinyHost download or support.

Page 30

Home Address - 5950 King Hill Drive

Farmington, NY  14425

6.6   New User Logons

Although TinyHost does not permit a new user to register,

there is a way to permit new users to logon. To do this, a user

account can be created, with a password, and this information can

be displayed as part of the TinyHost Welcome screen. For

instance, an account could be created with the name New User, the

password GUEST, and a security level of 0 (zero). A message could

be left as part of the TINYHOST.WEL screen stating that new users

should use this account to logon. These users would then be able

to access the files in the Start-Up directory, read Public mail,

and could leave any type of mail message. If the security levels

were set properly (the settings in the included TINYHOST.CFG file

would be fine), then the new user would not be able to do

anything else. This would be an easy way of leaving the TinyHost

system partially accessible to, for instance, the customers of a

company.

6.7   Message File Format

The TinyHost message files are standard ASCII text files.

Each file has the extension ".TH". Files for Personal mail

consist of filename made up of the user's first and last names.

The user's first name is used, with characters from the last name

being added to pad the filename for up to 8 characters (the DOS

limit). For instance, if the user's name is Joe Fabeets, then his

personal mail file will have the filename JOEFABEE.TH. Public

mail is stored in PUBLIC.TH; SysOp Comments are stored in

COMMENTS.TH.

Although TinyHost limits message input to 70 characters per

line and 20 lines, this is strictly a program limit. A message

file created off-line does not have to adhere to these

restrictions (although it is suggested that line length be kept

under 80 characters). When creating a message off-line, each line

should be terminated in a carriage return / line feed pair. The

end of the message should be marked with a period ('.') in the

first column of the terminating line, followed again by a

carriage return / line feed. This terminating period is not

displayed to the reader, but is used by TinyHost to determine the

end of the message.

By editing the Public Message file offline, the SysOp is able

to keep desired messages in the file while leaving room for new

messages.

It is important to note that the text editor used should

*not* add an End-of Text character (EOT or EOF, $1A) to the

message file. If it does then messages entered from within

TinyHost will not be added to that file.

Page 31

6.8   Logging User Activity

When the LOG feature is enable, TinyHost will keep track of

user activity in a text file called TINYHOST.LOG. This file will

be stored in the execution directory (with THOSTxxx.EXE, the

config file, etc.). Monitored activity includes all user logins

(including failures and wrong passwords), all files uploaded and

downloaded, and the times of logon and logoff. While the call is

in progress, information is stored in a file called TINYHOST.TMP.

After the user has logged off, this file is copied to the normal

log file. The file format is such that the most recent activity

is available at the beginning of the file.

6.9   SysOp Specific Notes

TinyHost offers some additional security protection that is

invisible to the SysOp. The TINYHOST.CFG, TINYHOST.LOG, and

THREMOTE.BAT files cannot be [T]yped or [D]ownloaded by users

with security levels less than that set by the SYSOP descriptor.

This adds some system protection in case a remote user gains

unexpected access to the directory containing those files.

6.10   Using TinyHost with DesqView

The author has had great success using TinyHost within a

DesqView window. Although the normal memory requirements are 150K

or so, more should be made available depending upon the TinyHost

features used. External protocols, another copy of COMMAND.COM

(when shelling to DOS), etc. all take up extra memory. Adjust the

DV memory allocation accordingly.

Also, when trying to shell to DOS from within a DV window, it

may be necessary to set the "Manage Printer Contention" option to

YES. I have no idea why - but it works.

6.11   Known Program Limits

A few areas exist where there are some known limitations:

First, I am somewhat confused as to the flexibility of

1K-XModem file transfers. The implementation within TinyHost

supports both CRC and checksum error detection, yet in the

testing that I have done I have never been able to implement a

1K-XModem checksum file transfer because the communications

package I was using did not support it. As TinyHost is currently

implemented it will attempt to start a CRC upload, and will retry

for a total of 30 seconds. If the upload does not begin in that

time it will fall back to attempting a checksum upload. However,

after TinyHost falls back to checksum it will not return to a CRC

mode of transfer. The upshot of all of this is that if it takes

more than 30 seconds to initiate a 1K-XModem upload using CRC

error checking then errors will occur and the upload will never

begin (assuming that your communications package does not support

1K checksum). If this happens simply abort the upload and begin

again.

Page 32

Secondly, file transfer errors have been observed when using

the 1K-XModem protocol at 1200 bps with Windows 3.0 and having

TinyHost running as a background task. The host machine was a

16MHz 386SX with 4Mb of RAM. These errors have *not* been seen

running TinyHost within DesqView at rates of up to 2400 bps. It

is entirely possible that by judicious time-slice selection

within Windows that this problem could be eliminated.

6.12  TinyHost Plus

An extended version of TinyHost (called TinyHost Plus) is now

available. TinyHost Plus has been created to permit adding many

advanced features that may be of interest, yet keep the original

TinyHost program the way it is - small and simple. This is not to

say that TinyHost will not be improved - just that it will not be

burdened with specialty functions.

TinyHost Plus (V1.10) presently has the following additional

features:

*  The ability to dial another TinyHost/Plus system and

transfer up to 20 files (registered version; 5 Shareware),

all at a preset time, without operator assistance

*  A Personal Directory option, where users of a specified

security level get their own directory that only they and

the SysOp can access remotely

*  Call statistics since program execution or SysOp reset.

Number of calls (not counting the SysOp), number of

uploads and downloads, and number of [C]omments left are

displayed locally or available remotely. (Registered

version only.)

TinyHost Plus has also been released as Shareware, and is

available on the author's TinyHost Support BBS, CompuServe

(IBMBBS), and Micro Science BBS.

Page 33